Chapter 3

Connectors and jumpers

This chapter defines the connectors and jumpers on the motherboard. The figure below shows the connector and jumper positions.

# n  Connector Definition

| Connector | Function |
| --- | --- |
| J3 | Keyboard |
| J4 | Power |
| J5 | External PS/2 mouse |
| J6 | USB |
| J7 | FDD |
| J8 | 2nd IDE |
| J10 | SIO2 |
| J11 | SIO1 |
| J12 | PIO |
| J13 | 1st IDE |
| J14 | CPU fan power (12V) |
| J15 | Reset SW |
| J16 | Keylock |
| J17 | IDE HDD LED |
| J18 | Suspend SW |
| J19 | IrDA |
| J20 | Speaker |
| J21 | Power LED |
| CN1~CN4 | PCI slots |
| CN5~CN10 | ISA slots |
| SI1~SI4 | SIMM sockets |
| SI5, SI6 | DIMM sockets |
| SI7 | PB SRAM socket |

# n  Jumper Definition

Jumpers are used to select options for certain features. To set a jumper to "close" means covering the jumper pins with the jumper caps, and "open" means not to cover the pins. Jumpers with more than two pins have numbers on the motherboard identifying the pins.

## CPU Jumper Settings

| Name | Type | Ratio | Freq. | JP5 | JP8 | JP7 | JP10 |  | JP11 |  |  | JP12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intel |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 75 | open | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  | 1.5X | 90 | 3-4 | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  |  | 100 | 1-2 | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  | STD | 2X | 120 | 3-4 | 1-2 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  | (3.3V) |  | 133 | 1-2 | 1-2 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  | 2.5X | 150 | 3-4 | 1-2,3-4 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  |  | 166 | 1-2 | 1-2,3-4 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
| P54C |  | 3X | 200 | 1-2 | 3-4 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  |  | 75 | open | open | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  |  | 1.5X | 90 | 3-4 | open | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  |  |  | 100 | 1-2 | open | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  | VRE | 2X | 120 | 3-4 | 1-2 | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  | (3.52V) |  | 133 | 1-2 | 1-2 | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  |  | 2.5X | 150 | 3-4 | 1-2,3-4 | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  |  |  | 166 | 1-2 | 1-2,3-4 | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
|  |  | 3X | 200 | 1-2 | 3-4 | 1-2,3-4,5-6 |  | 1-2 |  | 1-2 | 1-3 |  |
| P55C | (2.8V) | 2.5X | 166 | 3-4 | 3-4 | open |  | 2-3 |  | 2-3 | 1-3 |  |
|  |  | 3X | 200 | 1-2 | 3-4 | open |  | 2-3 |  | 2-3 | 1-3 |  |
| AMD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PR75 | open | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  |  | PR90 | 3-4 | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  | 1.5X | PR100 | 1-2 | open | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
| K5 |  |  | PR120 | 3-4 | 1-2 |  |  |  |  |  |  |  |
|  |  |  | PR133 | 1-2 | 1-2 | See the AMD voltage code table. |  |  |  |  |  |  |
|  |  | 2X | PR150 | 3-4 | 1-2,3-4 |  |  |  |  |  |  |  |
|  |  |  | PR166 | 1-2 | 1-2,3-4 |  |  |  |  |  |  |  |
| Cyrix |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | P133+ | 1-2,3-4 | 1-2 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
| 6X86 |  | 2X | P150+ | 3-4 | 1-2 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |
|  |  |  | P166+ | 1-2 | 1-2 | 1-2,3-4,5-6 |  | 2-3 |  | 1-2 | 1-3 |  |

AMD Voltage Code Table

| Voltage Code | JP7 | JP10 | JP11 | JP12 |
| --- | --- | --- | --- | --- |
| B | 1-2,3-4,5-6 | 1-2 | 1-2 | 1-3 |
| C | 1-2,3-4,5-6 | 2-3 | 1-2 | 1-3 |
| F | 1-2,3-4,5-6 | 2-3 | 1-2 | 1-3 |
| H | open | 2-3 | 2-3 | 4-6 |
| J | open | 2-3 | 2-3 | 2-4 |
| K | open | 2-3 | 2-3 | 1-3 |

Note:
The voltage code can be found in the CPU model name which
is printed on the CPU itself. For example,

## Other Jumper Settings

| JP | Definition |
| --- | --- |
| JP1 | RTC 1-2 : CMOS cleared 2-3 : normal |
| JP3 JP4 | Cache size select 0K/256K 512K 2-3 1-2 2-3 1-2 |