AWS3274: 3274 Control Unit Emulator

===================================

AWS3274 allows TCP/IP connectable 3270 emulators to be attached as 3274 control

unit display devices such as 3277, 3278, and 3279.

AWS3274 uses a TCP/IP telnet server to connect 3270 emulators such as

xhcon, x3270, PC3270, etc.  This allows 3270 sessions to be connected from

any TCP/IP client on the LAN.  The port number 3046 is used to connect these

sessions.

Example: x3270 -ext -sk -graphics -port 3046 'hostname' &

DEVICE TYPE:

============

Generally, the 3270 emulator characteristics determine the true device type.

Valid device types are 3277, 3278, and 3279.

MAX DEVICES SUPPORTED

=====================

Prior to 07/16/97, maximum number of AWS3274 devices supported was 48.  This was

increased to 64 with a patch.  See the available fixes for AWS3274 (aka dm3270).

Do not attempt to configure more than the maximum number of AWS3274 devices.

CONFIGURING a 3274 CONTROL UNIT

===============================

To configure a 3274, go to the Configurator F2 "UPDATE SYSTEM DEVICES" menu

and enter the real address of a 3277, 3278, or 3279 device in your system I/O

configuration.  For VM as an example, this would be those addresses with a DEVTYPE

of 3277, 3278, or 3279 and a CUTYPE of 3274, 3174, or 3272.

Miscellaneous Information

==========================

Device Addresses managed by AWS3274 are assigned to 3270 emulators

sequentially, starting with the lowest address first.  Thus if you have AWS3274

device address 200 and 201 defined, the first session started will get

address 200 and the next 201.