WAN3172 : XCA SDLC Emulator

===========================

The WAN3172 emulator enables VTAM running on operating systems

(which do not support ICA) to communicate over SDLC links using the

XCA types of configurations.  VTAM thinks that it is talking

over a token ring LAN but actually the communications are over SDLC line.

Both Leased Line as well as Switched Line (Dial In Only as of now)

are supported. Multidrop lines are not supported at this point of time.

A maximum of 8 ports (2 cards, 4 ports/card) are supported at this time.

IMPORTANT: For RS/6000 Machines with MCA Architecture, 4 Port Selectable

Portmaster Card is used while for Machines with PCI/ISA

Bus Architecture, the ISA 4 Port Selectable Multiport Model 2

Adapter is used for communications.

IMPORTANT: For ISA/PCI supported machines, install the fileset

'devices.isa.mm2'.  This is the Multiport Model 2 Adapter

software and can be found on AIX ver 4.x installation media.

IMPORTANT: For ISA cards, make sure to set the proper interrupt level

for the card so that it does not clash with any other

card in the system. This can be set by setting the option

switches 1, 2 and 3 on the card. For more information, see

Realtime Interface Co-Processor Multiport Adapter, Model 2

Guide to Operations.

Also make sure that the base I/O address of the card doesn't

clash with that of any other card. If required, base I/O

address can also be changed by option switches on the card.

Refer to Realtime Interface Co-Processor Multiport Adapter, Model 2

Guide to Operations for more information.

IMPORTANT: ISA cards are not recognized automatically by the system after

reboot. After rebooting, go to the SMIT. Choose 'Devices', then

choose 'Communication' from the devices menu, choose

'Multiport Model 2  Adapter' from communication menu, choose

'Adapter' from the next menu, choose 'Add a Multiport Model 2

Adapter' from the next menu to configure the card. After adding the

card, choose 'List All Multiport Model 2 Adapters' from the same

menu to ensure that the card is in Available state.

NOTE : MAXIMUM PACKET SIZE THAT CAN BE USED WITH THIS DEVICE MANAGER IS

4000 BYTES. IF AN ATTEMPT IS MADE TO TRANSFER HIGHER SIZE PACKETS,

IT WILL GIVE CLOSE STATION INDICATION TO VTAM AND DROP THE LINK.

DEVICE TYPE: 3088

=================

CONFIGURING A WAN3172

=====================

To configure a WAN3172, go to the P/390 Configuration "Update System Devices"

(F2) screen and configure ONE 3088 DEVICE ADDRESS FOR EACH PORT YOU WISH TO

USE ON THE ADAPTER.  Use the manager code for WAN3172 in the Mgr field.

In the FN/P field, put the parameters as follows :

Par1,[Par2],[Par3],[Par4],[Par5],[Par6],[Par7],[Par8],[Par9]

where:

Par1 -- Port Name of the mpqp device, e.g. mpq0 (required)

mpq0 (port 0)

mpq1 (port 1)

mpq2 (port 2)

mpq3 (port 3)

Par2 -- Node Type

PRI if this station will be Primary Node (default if omitted)

SEC if this station will be Secondary Node

Par3 -- Self SDLC Station Address

0         if Par2 is PRI (default if omitted and Par2 is PRI)

1 to 254  if Par2 is SEC (must be decimal)

193       (default if omitted and Par2 is SEC)

Par4 -- Destination SDLC Station Address

0         if Par2 is SEC (default if omitted and Par2 is SEC)

1 to 254  if Par2 is PRI (must be decimal)

193       (default if omitted and Par2 is PRI)

Par5 -- Line Type

LE for Leased Line (default if omitted)

SW for Switched Line

Par6 -- Type of XCA Node

SUB	for the Subarea type of node (default if omitted)

PER	for the Peripheral type of node

Par7 -- NRZ Convention

NRZ   (default if omitted)

NRZI

Par8 -- Electrical Interface

232	for RS232D (default if omitted)

X21	for X.21   **LEASED LINE ONLY**

V25	for V.35   **NOT TESTED YET**

422	for RS422A **NOT TESTED YET**

Par9 -- Phone number for Dialing out **NOT OFFICALLY SUPPORTED**

Note 1: FOR THE LEASED LINE, THE XCA NODE WITH HIGHER VTAM SUBAREA NUMBER MUST

BE CONFIGURED AS THE PRIMARY NODE e.g. if communication is between VTAM

Subarea 1 and VTAM Subarea 2, the node with subarea 2 has to be primary node

and the node with subarea 1 has to be secondary node.

Note 2: For the SWITCHED DIAL-IN SUBAREA type of connection, this side should

be configured as SECONDARY node and the side dialing out as the

Primary node.  If the calling side is a VTAM integrated communication

adapter (ICA), it will naturally be a Primary node.  If the calling

side is a NCP, you must insure that it will only operate as a

Primary node.  This is done by specifying only the primary

(first) suboperand on the NCP SDLCST keyword of the LINE macro.

Note 3: For the SWITCHED DIAL-IN PERIPHERAL type of connection, it should be

configured as the Primary node and the PU 2 (the terminal dialing out)

as the secondary node.

Note 4: The destination secondary address on primary node should be same as

self address on the secondary node. NORMALLY, A SECONDARY ADDRESS OF

193 (0xC1) is used by PU 2,PU 4 and PU 5 type of connections.

Note 5: Although in case of DIAL-IN PERIPHERAL type of connection, both

PU2 and PU2.1 are supported, the functionality of PU2.1 will

be limited to equivalent of PU2 i.e. 3270 emulation.

Example:

Suppose 9 is the manager code for WAN3172, then

1. For a Primary Node over Switched line, using portmaster/multiport port mpq0,

with destination secondary address 193, configured as Subarea type

of node i.e. to talk with PU4 or PU5 type of nodes,it would be configured

as :

______________________________________________________________________________

|Addr Device Label  Atype  Size     Mgr  FN/P                 Path:>         |

|-------------------------------------- -------------------------------------|

| 648 >3088 >      >     >          9    mpq0,PRI,0,193,SW,SUB,NRZI        |

|____________________________________________________________________________|

2. For a Primary Node over Switched line, using portmaster/multiport port mpq1,

with destination secondary address 193, configured as Peripheral node and

to

act as dial in only to the PU2 terminals, it will be configured as :

______________________________________________________________________________

|Addr Device Label  Atype  Size     Mgr  FN/P                 Path:>         |

|-------------------------------------- -------------------------------------|

| 644 >3088 >      >     >          9    mpq1,PRI,0,193,SW,PER,NRZ           |

|____________________________________________________________________________|

3. For a Secondary node over Leased line, using Portmaster/Multiport port mpq2,

with self address as 193, acting as Subarea node will be configured as:

______________________________________________________________________________

|Addr Device Label  Atype  Size     Mgr  FN/P                 Path:>         |

|-------------------------------------- -------------------------------------|

| 646 >3088 >      >     >          9    mpq2,SEC,193,0,LE,SUB,NRZI          |

|____________________________________________________________________________|

INSTALLATION NOTES:

==================

WAN3172 device manager uses two services provided by AIX.

1) DLC device manager

dlcsdlc - SDLC Data Link Control device manager

Make sure dlcsdlc is in Available state. If not, configure is using

SMIT as given in SMIT Configuration below.

2) The ports from the MultiPorotocol portmaster card/Multiport Model 2 Adapter

"mpq0"

"mpq1"

"mpq2" .....etc

List all the ports available to you using the SMIT path described

below :

For 4-port Selectable Multiprotocol Portmaster Card,

"smit"

"Devices",   "Communications",     "PortMaster Adapter/A",

"Adapter",   "Manage Device Drivers for Portmaster Adapter/A",

"Manage 4-port Multiprotocol Device Driver", "Manage Ports",

"List all defined Multiprotocol Ports"

For 4 port Selectable Multiport Model 2 Adapter,

"smit"

"Devices",   "Communications",     "PortMaster Adapter/A",

"Adapter",   "Manage Device Drivers for Multiport Model 2 Adapter",

"Manage 4-port Multiprotocol Device Driver", "Manage Ports",

"List all defined Multiprotocol Ports"

Make sure that the ports you are using are in available state. If not,

configure using SMIT as given in SMIT configurations below.

CONFIGURATIONS ... :

====================

The RS6000/AIX related configurations are to be done through "SMIT"

interface provided by AIX. Refer to appropriate manuals of AIX for more

detail.

SMIT CONFIGURATION:

==================

To configure LAN parameters necessary for WAN3172 device manager,

1) Configuring DLCSDLC device manager.

-----------------------------------

use smit options in the following order to configure DLCSDLC.

"smit"

"Devices",   "Communications",    ("Portmaster Adapter/A" for MCA),

("Multiport Model 2 Adapter" for ISA)

"Services",  "Data Link Controls", "Add a SDLC Data Link Control"

and select the following from the list,

"dlc sdlc dlc SDLC Data Link Control"

2a) Configuring the MultiPorotocol Portmaster Ports (For MCA Architecture)

----------------------------------------------------------------------------

Follow the following path to configure the portmaster adapter and ports:

"smit"

"Devices",   "Communications",     "PortMaster Adapter/A",

"Adapter"

After this, use

"List All Portmaster Adapter/A Adapters" option to make sure your

adapter is in Available state.

Next, select "Manage Device Drivers for Portmaster Adapter/A"

select "Manage 4-port Multiprotocol Device Driver"

Configure the driver using "Configure a Defined Device Driver" if

the driver is not already configured.

Configure the ports using "Manage Ports" option if they are not

in Available state.

Under "manage ports" option, you can use the "List all Defined

Multiprotocol ports" to see which ports are available to you.

Only ports in Available state can be used. Note down the

available port names as you have to give port name under FN/P field

in the configurator.

2b) Configuring the Multiport Model 2 Adapter Card (For ISA Architecture)

-------------------------------------------------------------------------

Follow the following path to configure the Multiport Model 2 adapter and

ports:

"smit"

"Devices",   "Communications",     "Multiport Model 2 Adapter",

"Adapter"

After this, use

"List All Multiport Model 2 Adapters" option to make sure your

adapter is in Available state.

Next, select "Manage Device Drivers for Multiport Model 2 Adapters"

select "Manage 4-port Multiprotocol Device Driver"

Configure the driver using "Configure a Defined Device Driver" if

the driver is not already configured.

Configure the ports using "Manage Ports" option if they are not

in Available state.

Under "manage ports" option, you can use the "List all Defined

Multiprotocol ports" to see which ports are available to you.

Only ports in Available state can be used. Note down the

available port names as you have to give port name under FN/P field

in the configurator.

Invocation of the WAN3172 device manager (dmxcasdlc)

====================================================

NOTE: At r390.obj 4.2.1.5 and up, modifications to ipl390 are NOT needed.

WAN3172 is started by changing the "nostart" keyword to "noparms" on the

"Start Device Manager Parameter Input/Display" panel of the Configurator

(press F12 from the Update System Devices panel).  If you have this

panel, skip this section.

Invoke the WAN3172 device manager (dmxcasdlc) from ipl390 script like,

dmxcasdlc &

where,

dmxcasdlc     -->  is the executable file for WAN3172 emulator.

NOTE: To turn the debug trace ON, invoke the WAN3172 device manager as :

dmxcasdlc -d &

where -d option turns the debug trace on. The trace will be logged in the

/var/r390/xca_log0 and /var/r390/xca_log1.

VTAM CONFIGURATION - SAMPLE PEER TO PEER XCA SDLC CONNECTION (SUBAREA TYPE)

==================

_______________                                        _______________

|             |    _____                     _____     |             |

| SYS1 (SA=1) |___|Modem|___________________|Modem|____| SYS2 (SA=2) |

|             |   |_____|    SDLC Line      |_____|    |             |

|_____________|                                        |_____________|

Secondary SDLC Node                                      Primary SDLC Node

** Since SYS2 has higher Subarea number, it will be configured as the

Primary node in the FN/P field in configurator entry. This is true for

LEASED line. For switched dial-in for Subarea, our side i.e. the side being

dialed in will always be secondary.

SYS1: XCA VTAMLST

-----------------

SYS1XCA VBUILD TYPE=XCA

SYS1648 PORT CUADDR=648,  * SYS1 WAN3172 DEVMAP 370 Address

ADAPNO=0,    * NOT USED

MEDIUM=RING, * NOT USED

SAPADDR=04,  * NOT USED

TIMER=20

SYS1XCA1 GROUP DIAL=NO   * For both Leased and Switched SUBAREA Nodes.

SYS1XLS1 LINE USER=SNA,

ISTATUS=ACTIVE

SYS1XP1 PU ISTATUS=ACTIVE,

MACADDR=400074700002, *Any addr of form 40007470000x. NOT USED.

PUTYPE=5,

SAPADDR=04,

SUBAREA=02,

TGN=11

SYS2: XCA VTAMLST

-----------------

SYS2XCA VBUILD TYPE=XCA

SYS2648 PORT CUADDR=648,  * SYS1 WAN3172 DEVMAP 370 Address

ADAPNO=0,    * NOT USED

MEDIUM=RING, * NOT USED

SAPADDR=04,  * NOT USED

TIMER=20

SYS2XCA1 GROUP DIAL=NO   * For both Leased and Switched SUBAREA Nodes.

SYS2XLS1 LINE USER=SNA,

ISTATUS=ACTIVE

SYS2XP1 PU ISTATUS=ACTIVE,

MACADDR=400074700002, *Any addr of form 40007470000x. NOT USED.

PUTYPE=5,

SAPADDR=04,

SUBAREA=01,

TGN=11

VTAM CONFIGURATION - PERIPHERAL XCA NODE TO PU2 CONNECTION (PERIPHERAL TYPE)

==================

_______________                                        _______________

|             |    _____                     _____     |             |

|   XCA Node  |___|Modem|___________________|Modem|____| PU 2 Node   |

|(Peripheral) |   |_____|    SDLC Line      |_____|    |             |

|_____________|                                        |_____________|

Primary SDLC Node  Dial-In                   Dial-Out   Secondary SDLC Node

*** A Peripheral XCA Node acting as a Dial in to PU 2 type of nodes should

always be configured as Primary in the configurator.

XCA VTAMLST (Peripheral)

-------------------------

********

SYS1XCA VBUILD TYPE=XCA

*

SYS1648 PORT CUADDR=648,  * SYS1 WAN3172 DEVMAP 370 Address

ADAPNO=0,    * NOT USED

MEDIUM=RING, * NOT USED

SAPADDR=04,  * NOT USED

TIMER=20

*

SYS1XCA1 GROUP DIAL=YES,  * For PERIPHERAL Nodes.

CALL=IN,

ANSWER=ON

*

SYS1XLS1 LINE

SYS1XP1  PU

********

NOTE: CODE ONLY 1 LINE,PU PAIR. MULTIDROP CONNECTIONS ARE NOT SUPPORTED

AS OF NOW.

SWNET VTAM DEFINITION :

---------------------

********

SWNDIAL	VBUILD	TYPE=SWNET

*

PUDIAL		PU		IDBLK=050,

IDNUM=00100,

ADDR=C1,

MAXDATA=265,

USSTAB=NGUSSS,

DLOGMOD=HTPCMD2E,

MODETAB=KGNMODE

*

LUDIAL		LU		LOCADDR=1,

LOCADDR=2,

LOCADDR=3,

LOCADDR=4,

LOCADDR=5,

LOCADDR=6,

LOCADDR=7,

LOCADDR=8

NOTE: XCA NODE WILL BE ABLE TO TALK TO ONLY 1 PU , SO CODE ONLY 1 PU

FOR THE XCA NODE in THIS DEFINITION.