AWS2821: PRINTER MANAGER

========================

AWS2821 emulates a 1403 or 3211 printer using an AIX printer, print queue,

or file as output.  A maximum of 3 AWS2821 devices are supported.

DEVICE TYPE: 1403, 3211

=======================

1403 = No Forms Control Buffer (FCB) support.

3211 = Forms Control Buffer emulation supported.

CONFIGURING AN AWS2821 DEVICE

=============================

To configure a 2821, go to the Configurator F2 "UPDATE SYSTEM DEVICES" menu and

enter the real address for the 1403 or 3211 printer in the Addr field, the MGR

code for AWS2821 in the Mgr field, and one of the following parameters in the

FN/P field:

- Fully qualified file name for file output (eg. /tmp/print.00e)

- Device name of a real serial or parallel attached printer (eg. /dev/lp0)

- 'c=' followed by a fully qualified control file name for customized

operations, including use of print queues (eg. c=/tmp/print.cntl).

To use this option, please read "PRINTER CONTROL FILE" below.

AIX PRINTER SET-UP FOR ALL REAL PRINTERS

========================================

The following parameters are required for a real printer from the

smit panel "Change/Show Characteristics of a Printer/Plotter":

Printer TIME OUT period          = 6 or 7

Columns to INDENT                = 0

Send all chars to prt UNMODIFIED = YES

Return on ERROR                  = YES

The following AIX command can be used instead (Where xxx = printer

name, example: lp0):

chdev -l xxx -w p -a ptop=1 -a ind=0 -a plot=yes -a mode=yes

In most cases, all other parameters can be left to the default values.

AIX PRINTER SET-UP FOR 3211 EMULATION

=====================================

To use a real workstation printer, configure the physical printer with the SMIT

PRINTER/PLOTTER DEVICES menu.  Use the configuration parameters listed above.

If you want to direct your output to a print queue, you must use the control

file option (see "PRINTER CONTROL FILE" below.)

Only printers that support the following control and escape sequences

can be used as a type 3211 with FCB support.

Line Feed        = 0x0A

Form Feed        = 0x0C

Carriage Return  = 0x0D

ESC 3 n (where n = lines per inch)

ESC C n (where n = lines per page)

PRINTER CONTROL FILE

====================

A printer control file allows a certain amount of flexibility and custom

control, especially with VSE where special features where added to exploit

AWS2821.

The following text serves as an example of a printer control file.

// You can comment out a line by putting // at the start of the line.

//

// This file is used by AWS2821 for customized printer operations.  A

// control file is specified by proceeding the file name with 'c=' in

// the FN/P field of the configurator UPDATE SYSTEM DEVICES menu

// Example: C=/tmp/print.cntl

//

// The entries are defined as follows (case is ignored except for EOJ):

//

// PC=string  Where string is a series of printer control characters or codes

// that will be sent to the printer to place it in a particular mode

// (eg. landscape.)  The user must consult the printer reference manual to find

// the proper codes and sequences.  If invalid codes or sequences are used,

// unpredictable results may occur.  AWS2821 will NOT check codes for validity.

// Each code MUST be specified in hex and all codes MUST be separated by a

// blank space.

// Example: PC=ab 3e f1 11 22 33 01 10

//

// EOJ=string  Where string is information that AWS2821 can use to determine

// the end of a print job.  Upon reaching the EOJ condition, AWS2821 may close

// a print file and pass it to user exit routine, or close a print file and

// send it to a queue, or simply perform a page eject on the target workstation

// printer.  These actions depend other options specified in the control file.

// EOJ consists of a line number, column number and search string.  The first

// number is the line number on each page to search, the second is the column

// number or offset of the line to start the search, and the string to search

// for is in single quotes.  The syntax is EOJ=line,col,' this is a string '.

// THERE MUST NOT BE ANY BLANKS USED EXCEPT WITHIN THE SINGLE QUOTES.

// If column 0 is specified, the pattern is searched in that entire line.

// Line number 0 is INVALID prior to the printer.Ebin service level

// (July 9, 1998).  At version printer.Ebin, line number 0 causes all lines

// on each page to be searched.

// The first occurrence found will cause the EOJ condition to be recognized.

// Example: EOJ=11,25,'*****END****'

// Example: EOJ=24,0,'job completed'

//

// LL=n  Where n is a number for the maximum printed line length of any line.

// Lines longer than the max will be truncated.  The default is 132.

// Example: LL=120

//

// FILE=name  Where name is the name of a file where the printed output will be

// placed. It can be a real printer device like /dev/lp0 or a file such as

// /tmp/printer.00e.  Either a printer device or an AIX file can be specified.

// Example: FILE=/dev/lp0

// Example: FILE=/tmp/printer.00e

//

// UE_EXIT=program  Where program is a user executable which will be called

// when AWS2821 detects the end of job (EOJ) condition.  AWS2821 will save the

// current print file and the name to the user exit program.  AWS2821 may take

// other actions depending on the control file parameters specified pass (see

// USER EXIT DETAILS below).  Real printers and printer queues can not be used.

// User must write the program to take desired action on a user exit.

// Example: UE_EXIT=/usr/bin1/printit

//

// QUEUE=name  Where name is a valid print queue where the print file will be

// printed on either getting 'FF' CCW or on end of job (see PRINTING TO QUEUE

// below.)

// Example: QUEUE=lp0

//

// LINE NUMBERS  This activates a tool to help you determine EOJ values.

// It will print a line number at the beginning of each printed line so

// that you can see which line on a page contains the desired EOJ string.

// Example: LINE NUMBERS

//

// TR=old_code1 old_code2 old_code3,new_code1 new_code2 new_code3

// This allows the user to change specific entries in the EBCDIC-to-ASCII

// translate table.  Each code is represented by a 2 digit hex value.  Blanks

// are required between codes.  Old_code is the "old" hex EBCDIC code value to

// be translated, new_code is it's corresponding NEW hex ASCII replacement

// value.  All old codes are on the left, all corresponding new codes are on

// the right side of the comma.  Up to 256 translations are of allowed.  There

// can be multiple TR= lines to accommodate many translations.  There must be

// the same number of hex values on the left as corresponding values on the

// right.  Mixed case is allowed.  The following example converts blanks (40)

// to underscores (5f) and dollar signs (5b) to pound signs (23).

// Example: TR=40 5b,5f 23

//

// Here is a more or less practical example:

//

// Number lines and page eject when finding EE on line 60, column 3 of any page.

//

line numbers

eoj=60,3,'EE'

//

// The next line demonstrates placing the IBM Laser Printer into:

// HP Emulation mode, Courier 16.67 (small font), landscape mode,12 lines/inch

//

pc=1b 5b 4b 05 00 05 31 02 00 0a 1b 26 6c 31 4f 1b 26 6c 31 32 44

ll=80

//

// Send output to a real printer but translate some characters

//

file=/dev/lp0

tr=c4 C5 d6,b0 DB db

tr=c1 c2 c3 c4 c5 c6, a0 62 87 64 82 66

//

// Not using user exit or queue

//

//ue_path=/tmp

//ue_exit=/usr/bin1/printit

//queue=lp0

EOJ and QUEUE ENHANCEMENTS MADE NOV 4, 1998

===========================================

At maintenance level printer.Jbin, multiple EOJ and QUEUE entries are

supported to allow better separation of different jobs and to allow

jobs to be sent to unique queues.

The rules:

1) If more than 1 QUEUE is specified, then the number of EOJs must equal

the number of QUEUEs.

If only 1 QUEUE is specified, or if fewer QUEUEs then EOJs are found,

then output is sent to the first QUEUE on all EOJ conditions.

2) The first EOJ is paired with the first QUEUE, second EOJ with the second

QUEUE, etc. Up to 4 pairs are supported.

EOJs and QUEUEs found after the limit of 4 are ignored.

3) If the number of EOJs matches the number of QUEUEs (up to 4), the

output is sent to the QUEUE that corresponds to the EOJ string found.

4) If the same EOJ string appears more than once, only the first one is used.

If the same QUEUE name appears more than once, no problem. You can send

multiple EOJ conditions to the same QUEUE this way, and still have another      unique EOJ/QUEUE pair.

// Example 1:

// Send all jobs to a single queue

FILE=/tmp/printer.00e

EOJ=0,0,'end of COBOL job'

EOJ=0,0,'end of JES2 job'

EOJ=0,0,'end of C job'

EOJ=0,0,'end of PLI job'

QUEUE=prt1

// Example 2:

// Send all to unique QUEUEs

FILE=/tmp/printer.00e

EOJ=0,0,'end of COBOL job'

QUEUE=prt1

EOJ=0,0,'end of JES2 job'

QUEUE=prt2

EOJ=0,0,'end of C job'

QUEUE=prt3

EOJ=0,0,'end of PLI job'

QUEUE=prt4

// Example 3:

// Send JES2 to unique QUEUEs, rest to same QUEUE

FILE=/tmp/printer.00e

EOJ=0,0,'end of COBOL job'

QUEUE=prt1

EOJ=0,0,'end of JES2 job'

QUEUE=prt2

EOJ=0,0,'end of C job'

QUEUE=prt1

EOJ=0,0,'end of PLI job'

QUEUE=prt1

USER EXIT DETAILS

=================

Note: Invoking a user exit is dependent on information

in the printer control file.

1) A User Exit is invoked by the printer device manager only

when ALL of these conditions at met:

- Control file specifies an AIX file for output.

Real printers and printer spools/queues can not be used.

file=/tmp/printer.00e

- Control file has valid path name (for temp files to be created)

ue_path=/tmp

- Control file specifies an existing user exit program name.

ue_exit=/usr/bin1/printit

- a valid FF command is received with the data chain flag OFF and

data length > 0.

OR

an end of job is detected as defined by 'eoj' entry in the printer

control file.

2) If the FF command is received, but the user exit can not be

invoked (for any reason), the command executes as if the

User Exit was not requested.  This means we return Channel End,

Device End, and residual count = requested count.

3) When the FF command meets the requirements in 1) above or an end of

job is detected with conditions in 1) satisfied:

- The data associated with the command is written into a file

located in ue_path with the name "datafile".  The file extension

starts at 000 and is incremented for each successful FF

command.  When 999 is reached, it starts over at 000 even if

this causes an existing file to be overwritten. This does not

hold true when the user exit is called on detecting eoj pattern.

- The current printer file is closed and moved/renamed into

the ue_path with file name "prnfile".  The file extension

starts at 000 and is incremented for each successful FF

command.  When 999 is reached, it starts over at 000 even if

this causes an existing file to be overwritten.

- The ue_exit program is called and passed the fully qualified

FF data file and the printer file names (in this order).

/usr/bin1/printit    /tmp/datafile.000   /tmp/prnfile.000 &

- In the case where user exit is called on detecting end of job as

defined by 'eoj' command, the ue_exit program is called as:

/usr/bin1/printit	  INVALID	/tmp/prnfile.000 &

- Successful completion is indicated with Channel End, Device End,

and residual count = 0.

PRINTING TO QUEUE

=================

The Print file is sent to queue for printing only if all of the

following conditions are true :

- Control file specifies an AIX file for output.

FILE=/tmp/printer.00e

- A valid queue is specified in the printer control file.

QUEUE=lp0

- No user exit is specified in the printer control file.

- a valid FF command is received

OR

an end of job is detected as defined by 'eoj' entry in the printer

control file.