AWS34XX SCSI Tape Drive Manager

============================================

AWS34XX emulates various 370 tape drives on SCSI attached drives.

Use the following table to determine the appropriate usage.

NOTE:  Other compatible devices may work.  Check with your service/suppport representative

before purchasing devices not in this list.

+------------------------------------------+-----+------+------+------+-----+

| TAPE DRIVE TO DEVICE TYPE USAGE          | 3490| 3480 | 3422 | 3420 | 9348|

+------------------------------------------+-----+------+------+------+-----+

| IBM 4MM (tested, supported)              | NO  | YES  | YES  | YES  | NO  |

+------------------------------------------+-----+------+------+------+-----+

| IBM/Exabyte 8MM (tested, supported)      | NO  | NO   | YES  | YES  | NO  |

+------------------------------------------+-----+------+------+------+-----+

| Qualstar 9trk NRZI (tested, in-use)      | NO  | NO   | YES  | YES  | NO  |

+------------------------------------------+-----+------+------+------+-----+

| Overland 9trk NRZI (tested, in-use)      | NO  | NO   | YES  | YES  | NO  |

+------------------------------------------+-----+------+------+------+-----+

| M4 9trk NRZI (tested, in-use)            | NO  | NO   | YES  | YES  | NO  |

+------------------------------------------+-----+------+------+------+-----+

| IBM 3490E Exx (tested, supported)        | NO  | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| Overland/Cipher 480,490 (tested, in-use) | NO  | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| Fujitsu 248x (tested, in-use)            | NO  | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| IBM 3490E Fxx (tested, supported)        | YES | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| Overland 490E (in-use)                   | YES | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| IBM 3570 (in-use)                        | NO  | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

| IBM 9348 (in-use)                        | NO  | NO   | NO   | NO   | YES |

+------------------------------------------+-----+------+------+------+-----+

| QUANTUM DLT7000 IBM-3503 (in-use) Note 2.| NO  | YES  | NO   | NO   | NO  |

+------------------------------------------+-----+------+------+------+-----+

Note: "in-use" means that the tape drive is known to be in use with the device

type(s) indicated but support for AWS34XX related problems is limited.

Note 2: DLT7000 was configured with "Other SCSI Tape" (with settings listed

below) and briefly tested on VM/ESA using VMFPCL2 commands and Ditto.

3503 was the enclosure box used.

Ditto "Tape Map" problem fixed in dm34xx.Rbin (11/11/98).

User Notes/Restrictions:

========================

1) Before using this in production, you should thoroughly test it with your

application.

IMPORTANT: For most tape drives, especially 4MM and 8MM drives, it will

be necessary to set the Missing Interrupt Handler timeout value of the S/390

Operating System to AT LEAST 180 seconds for AWS34XX devices.

2) 4MM and 8MM drives generally do not support the READ BACKWARDS command.

However, AWS34XX now emulates Read Backwards on all drives. This emulation

is transparent to the S/390 software.

3) As of AWS34XX service level "F" (dm34xx.Fbin), for versions 4.2 and 4.1 of

AIX, a LOAD DISPLAY CCW's message data is sent to the R/390 display if the

target drive does not support this function, or if the target drive is

unreachable.  These messages that are normally displayed on drive's LEDs.

If AWS34XX sends this message to the R/390 display, it will appear in this

format:

AWS34XX Load Display message for device xxxx - MSG1=xxxxxxxx MSG2=xxxxxxxx

To enable this feature, -msg must be specified as a start parameter.

4) It is very important that the specified Device in the P/390 Configuration

match the operating system definition and that the device type used is

supported as stated above.  There is no checking by the R/390 for this.

Example: target drive = 8MM EXABYTE, The device type can be 3420 or 3422.

If 3420 is used in the P/390 Configuration (say address 580), then

the 390 Operating System must have address 580 defined as a 3420.

A mismatch can cause unpredictible results.

DEVICE TYPE: 3420, 3422, 3480, 3490, 9348

=========================================

Depending on which type of drive you are using, you

must select the appropriate device type (above).

CONFIGURATION:

=============

To configure a 34xx tape drive:

1.) Prior to service level "E" (dm34xx.Ebin), a tape MUST be present on

the tape drive before IPLing or the device would not be configured.  This

restriction was removed at service level dm34xx.Ebin.  At this level, the

ipl390 script must be changed to start dm34xx as a user process (consult

with your service provider for more information).

2.) The RS/6000 must be configured for the drive via smit(ty) if it is

not already there. The AIX device path name (eg. /dev/rmt0) will be used by

AWS34XX.

3.) There are some required settings on the SMIT "Change/Show Characteristics

of a Tape Drive" panel.  If these are present, they must be:

BLOCK size (0=variable length)            [0]

Use Extended File Marks                   [no]

Set delay after a FAILED command          [2]

Set timeout for the READ or WRITE command [30]

All other settings are at the user's preference but should normally default.

4.) Select "P/390 Configuration" on the R390 Main Menu and locate the

UPDATE SYSTEM DEVICES (F2) screen.  Fill in the Addr, Device, Mgr, and FN/P

fields appropriately.  Specify the AIX device path name of the target SCSI

drive in the parameter/filename field (eg. /dev/rmt0, /dev/rmt1, etc.).

Example:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>580   >3420                         >C   >/dev/rmt1

MGR Codes: C=AWS34XX

Sharing the tape device with AIX

================================

Normally, AWS34XX prevents AIX from accessing the device.  For example,

the 'tar' command may fail with 'resource temporarily unavailable'.

To give the device back to AIX:

1) VARY the tape offline,

2) Issue a Rewind-Unload command if this was not done by the VARY,

3) remove and replace the tape cartridge.

The tape drive is now free for AIX use.

Once the tape is free for AIX use, the next command from the channel causes

AWS34XX to own it again and you have to repeat the above steps to make it

available to AIX again.