AWSCKD: Count, Key, Data DASD emulator

======================================

CKD DASD is designed to emulate most Count Key Data dasds

connected to an appropriate control unit.

DEVICE TYPE:  3330, 3350, 3375, 3380, 3390, or 9345

===========

Configuring a CKD device

========================

1. The CKD DASD volumes need to be allocated as files.

See "CKD DASD Device Allocation", below.

2. The CKD DASD volumes then need to be formatted or initialized.

See "Initializing CKD DASD Volumes", below.

CKD DASD Device Allocation

==========================

Allocation of CKD DASD volumes can be done using

a. The P/390 Configurator F2 "UPDATE SYSTEM DEVICES" menu

b. The P/390 Configurator F3 "UPDATE USER MINIDISKS" menu

Use method a for VM system volumes, and all VSE and non-VM volumes.

Use method b for VM user minidisks.

Initializing CKD DASD Volumes

=============================

Depending on the operating system being used or how the data will actually be

written to a DASD volume, it may have to be initialized.  Initialization must

be done using standalone ICKDSF (Device Support Facilities program number

5747-DS1).  ICKDSF under VSE or MVS will not work on an emulated volume due to

the fact that there is no special service cylinder available.  However, the

standalone version of ICKDSF will work using the MIMIC(MINI) parameter.

Here is an example of an INIT command for standalone ICKDSF:

INIT UNIT(u) NOVFY DEVTYP(3380) MIMIC(MINI(s)) VOLID(v) VTOC(c,t,e)

where:

u = unit address

s = size of volume in cylinders

v = volume label

the VTOC parameter is optional but if used:

c = cylinder where VTOC is to be placed

t = track where VTOC is to be placed

e = size of VTOC in tracks

A standalone version of DDR is supplied with the P/390 VM Starter System as

/vm/ipldsf.580.

An alternative to initializing the volume is to do a full restore from an

existing pack using DDR (DASD Dump Restore).  DDR can be used to fully dump and

restore a pack including the VTOC and all data.  A standalone version

of DDR is supplied with the P/390 Starter System as /vm/iplddr.580.

3380 CKD Option

===============

By default, 3380s will support ECKD (Extended Count Key Data) architecture.

Some of the older 370 systems may have a problem with this, so we have

provided an option that allows all 3380s to appear to the system as CKD

devices that do not emulate the architecture extensions.

If you are on an older 370 system that is out of service, you may want to

enable this option.  To do so, edit the /usr/bin/r390/ipl390 file.  Add

the parameter -ckd3380 after the statement that starts dmckd:

$DMDIR/AWSStart dmckd -ckd3380

Actually, only '-c' is needed because the rest of the parameter is ignored.

The	parameter can be either upper or lower case.

AWSCKD Enhancement

==================

By default, a single instance of AWSCKD claims all ckd devices in the

configuration.  With large configurations, this can be a performance

bottleneck.  Also, a single instance of AWSCKD will cause the loss of

all ckd devices in the event of an unrecoverrable error on any one device.

This new option allows multiple copies of AWSCKD, each claiming

a specified number of devices.  This may provide better performance when

accessing multiple devices simultaniously and in the event of an

unrecoverable error, loss of only those devices claimed, thereby allowing

other instances of AWSCKD to keep running unaffected.  There are 8

values allowed:

$DMDIR/AWSStart dmckd -1   (1 device per AWSCKD instance)

$DMDIR/AWSStart dmckd -2   (2 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -3   (3 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -4   (4 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -5   (5 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -6   (6 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -7   (7 devices per AWSCKD instance)

$DMDIR/AWSStart dmckd -8   (8 devices per AWSCKD instance)

The first option (-1) will provide the best performance in most

configurations.

RECOMMENDED USAGE:  number of AWSCKD devices     use

------------------------    -----

1-32                 -1

33-64                 -2

65-96                 -3

97-128                -4

>128                 -6

NOTE:  With later versions of r390.obj, optional parameters should be

specified on the "Start Device Manager Parameter Input/Display" panel

of the configurator (hit F12 from the "Update System Devices" panel to

see if you have this support).  Here, only the parameter(s) are

specified, not the entire start statement:

Example:

Start Device Manager Parameter Input/Display

1   AWSFBA    >noparms

2   AWSCKD    >-ckd3380 -1

3   AWS3274   >noparms

4   AWS3215   >noparms

5   AWS2821   >noparms

6   AWS2540   >noparms

7   LAN3172   >ent1

8   LCS3172   >noparms

9   WAN3172   >nostart

A   AWSPBS    >nostart

B   AWSTAPE   >noparms

C   AWS34XX   >noparms

D   AWSOMA    >noparms

E   AWSICE    >noparms

F1=Help  F7=Up  F8=Down  F10=Accept/Return  ENTER=Accept Data  ESC=Cancel