CABLETRON SYSTEMS INC.

4/16 Mbps Token Ring NDIS Driver

for MS OS/2 1.2X

and MS DOS

The following is an example of the line to be entered into the

config.sys file.  Refer to the User Guide for your particular

protocol to determine the point in the config.sys file where

the line should be located.

For MS OS/2 1.2X

----------------

DEVICE=C:\<path>\T20NDIS.OS2

For MS DOS

----------

DEVICE=C:\<path>\T20NDIS.DOS

The following is an example of what a protocol.ini file should

look like once the correct parameters are loaded for the driver

T20NDIS.OS2 or T20NDIS.DOS.

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[NETBEUI_XIF]

Drivername = netbeui$

BINDINGS = "T20"

[T20]

DRIVERNAME=T20NDIS$

IOADDRESS=0XA20

DMACHANNEL=5

INTERRUPT=3

BANDWIDTH=0

MEDIA=PRI

TIMING=DEF

RECEIVEBUFSIZE=4096

RECEIVEBUFFERS=6

MAXREQUESTS=6

MAXMULTICAST=10

MAXTRANSMITS=6

RECEIVECHAINS=6

NOTE:  The values specified are default values.  It is required to list

the keyword in the table only if you choose to use something other

than the default value.  There are two exceptions, the keywords

DRIVERNAME and BOARDTYPE must always be specified.

Possible choices for hardware parameters:

IOADDRESS  ==>  0XA20, 0X0A80, 0X1A20, 0X1A80

DMACHANNEL ==>  5, 6, 7

INTERRUPT  ==>  3, 4, 5, 6, 7, 9, 10, 11, 12, 14, 15

BANDWIDTH  ==>  0-16 Mbps, 1-4 Mbps

MEDIA      ==>  PRI - Shielded TP, SEC - Unshielded TP

TIMING     ==>  DEF - Default Timing, ALT - IBM PS/2 Model 30