Newsgroups: alt.cd-rom

From: gloster@INFERENCE.COM ("Vance M. Gloster")

Subject: MPC Spec

Message-ID: <9211302218.AA23663@quaestor>

Sender: CD-ROM <CDROM-L@uccvma.ucop.edu>

Organization: FUNET-NGW

Date: Mon, 30 Nov 1992 22:18:07 GMT

Lines: 450

I have had several requests for the MPC (Multimedia PC) Spec.  Here it

is:

-Vance Gloster

gloster@inference.com

Multimedia PC Specification Version 1.0

Created: May 2, 1992

SYSTEM HARDWARE

Minimum Multimedia PC Full System Configuration

CPU

IntelR 386SXT or compatible

RAM

2 MB of RAM

Magnetic Storage

3.5-inch floppy disk drive, 1.44-MB capacity

30-MB hard disk drive

Optical Storage

CD-ROM with CD-DA outputs

Audio

8-bit digital-to-analog converter (DAC), linear PCM sampling,

22.05 kHz and 11.025 kHz rate, DMA/FIFO with interrupt

8-bit analog-to-digital converter (ADC), linear PCM sampling,

11.025 kHz rate, microphone level input

Music synthesizer

On-board analog audio mixing capabilities

Display Adapter

VGA

Input

101-key keyboard

Two-button mouse

I/O

Serial port

Parallel port

MIDI I/O port

Joystick port

Minimum Multimedia PC Hardware Upgrade Kit Configuration

Optical Storage

CD-ROM with CD-DA outputs

Audio

8-bit DAC, linear PCM sampling, 22.05 kHz and 11.025 kHz rate,

DMA/FIFO with interrupt

8-bit ADC, linear PCM sampling, 11.025 kHz rate, microphone

level input

Music synthesizer

On-board analog audio mixing capabilities

I/O

MIDI I/O port

Joystick port

Hardware Specifications

CPU

Intel 386SX or compatible (minimum)

RAM

2 MB of extended (linear address space) memory

Magnetic Storage

3.5-inch floppy disk drive, 1.44-MB capacity

30-MB hard disk drive (minimum)

Optical Storage

CD-ROM drive with:

Sustained 150 KB/sec transfer rate

Average seek time of one second or less

10,000 hours MTBF

Mode 1 capability (mode 2 and forms 1 and 2 optional)

Subchannel Q (subchannels P and R through W optional)

If R through W subchannel support is provided,

additional APIs must be implemented in the MSCDEX

driver. Specifications for these additional APIs are

available from Microsoft Corporation.

MSCDEX version 2.2 driver that implements the extended

audio APIs

The drive must sustain a transfer rate of 150 KB/sec without

consuming more than 40 percent of the CPU bandwidth. This

requirement is for read block sizes no less than 16K and a lead

time no longer than is required to load the CD-ROM buffer with

one read block of data. We recommend that the drive have

on-board buffers of 64 KB and implement read-ahead buffering.

Read-ahead buffering is described in a specification available

from Microsoft Corporation.

Audio

CD-ROM drive with CD-DA (Red Book) outputs and a front-panel

volume control.

CD ROM/XA audio may be provided as an option.

8-bit (16-bit recommended) DAC with:

Linear PCM sampling

DMA or FIFO buffered transfer capability with interrupt

on buffer empty

22.05 kHz and 11.025 kHz sample rates mandatory

44.1 kHz sampling rate desirable

Optional stereo channels

No more than 10 percent of the CPU bandwidth required

to output 11.025 kHz or 22.05 kHz; no more than 15

percent for 44.1 kHz

8-bit (16-bit recommended) ADC with:

Linear PCM sampling

11.025 kHz mandatory (22.01 kHz or 44.1 kHz sampling

rate optional)

DMA or FIFO buffered transfer capability with interrupt

on buffer full

Microphone input

Internal synthesizer hardware with multivoice, multitimbral

capabilities, six simultaneous melody notes, and two

simultaneous percussive notes.  Internal mixing capabilities to

combine input from three (four recommended) sources and to

present the output as a stereo, line-level audio signal at the

back panel. The four sources are CD Red Book, synthesizer, DAC

(waveform), and an auxiliary input source (recommended but not

required).  Each input must have at least a 3-bit volume control

(eight steps) with a logarithmic taper (4-bit or greater volume

control is strongly recommended).  If all sources are sourced

with  10 dB (consumer line level: 1 milliwatt into 600 ohms = 0

dB) without attenuation, the mixer will not clip and will output

between 0 dB and 3 dB. Individual audio source and master

digital volume control registers and extra line-level audio

sources are highly recommended.

Guidelines for Synthesizer Implementation

The MicrosoftR WindowsT graphical environment with Multimedia

Extensions (referred to hereafter as Windows with Multimedia)

defines a MIDI file authoring standard and supports

device-independent playback of MIDI files authored to the

standard. To enable this, Multimedia PC synthesizers need to

follow the guidelines below.

Windows with Multimedia includes a user-configurable MIDI mapper

that can change MIDI channel and patch numbers in a MIDI data

stream, so synthesizers need not conform to the specific MIDI

channel assignments and patch numbers defined below. If the

synthesizer does not conform, however, a MIDI map file should be

provided to make configuring a Multimedia PC with a specific

synthesizer easier for the user.  Microsoft also recommends that

synthesizers created for the Multimedia PC (that is, using audio

boards) provide general MIDI mode (GMM). GMM is a standard patch

and channel map defined by the MIDI Manufacturers Association

and is fully compatible with the MIDI implementation for Windows

with Multimedia.

1. Base Multitimbral vs. Extended Multitimbral

Some Multimedia PC synthesizers can play back only a few timbres

(also called instruments or patches) simultaneously at any time.

Others can play back multiple timbres at one time. To cover the

wide range of capability, two synthesizer types are defined:

Base Multitimbral and Extended Multitimbral. Each type is

assigned a subset of the 16 MIDI channels.

Base Multitimbral specification: MIDI Channels 13 through 16.

Base Multitimbral synthesizers must be able to play three

melodic timbres (channels 13 through 15) and two percussive

timbres (channel 16) simultaneously. Base Multitimbral

synthesizers must also be able to play six melodic notes and two

percussive notes simultaneously. The melodic and percussive

notes may be allocated across the melodic and percussive

instruments, respectively, in any combination. For example, a

Base Multitimbral synthesizer could play three flute notes, two

harpsichord notes, one cello note, and two base drum  notes  at

one time. Base Multitimbral synthesizers may exceed these

minimum capacities, but MIDI files do not take advantage of

additional capacity.

Extended Multitimbral specification: MIDI Channels 1 through 10.

An Extended Multitimbral synthesizer must be able to exceed the

base specification by some undetermined amount. These

synthesizers should have capabilities as close as possible to

the following goals: Extended Multitimbral synthesizers should

be able to play nine melodic timbres (channels 1 through 9) and

eight percussive timbres (channel 10) simultaneously and 16

melodic notes and 16 percussive notes simultaneously. The

melodic and percussive notes may be allocated across the melodic

and percussive instruments in any combination. Extended

Multitimbral synthesizers may exceed these capabilities, but

MIDI files do not take advantage of additional capacity. (See

the  Authoring Standard  section below to learn how MIDI files

take advantage of these two synthesizer definitions.)

2. Note and Timbre Stealing

When a synthesizer encounters a note beyond its polyphony limit

(for example, a fifteenth melodic note for an Extended

Multitimbral synthesizer capable of playing only 14 melodic

notes at one time), it should  steal  the oldest melodic note

playing to play the new note. Melodic notes should not steal

percussive notes and percussive notes should not steal melodic

notes.

The melodic MIDI channels for each synthesizer type are in

priority order, with the lower channels a higher priority. When

a synthesizer encounters a melodic timbre beyond its

multitimbral limit (for example, a ninth melodic timbre for an

Extended Multitimbral synthesizer capable of playing only eight

melodic timbres at one time), it should drop the timbre on the

highest melodic MIDI channel channel 9, in this case.

3. Instrument Loading

MIDI sequencers under Windows with Multimedia tell the

synthesizer s driver what patches are to be used before it plays

back a MIDI file, using the midiOutCachePatches function. For

synthesizers that load instrument patches into PC or board RAM,

this permits the driver to preload the necessary instrument

patches. Drivers for synthesizers that load patches into board

RAM of limited capacity can buffer additional patches in PC RAM,

so they may be loaded at interrupt time when needed.

4. Standard Instruments

The table below lists the standard instruments synthesizers

should implement. When it is not feasible to implement all

instruments, at least one instrument from each group should be

implemented.

5. Authoring Standard

MIDI authors are required to provide two versions of every MIDI

composition. Both versions are stored in the same MIDI file.

Channels 1 through 10 hold the version written for Extended

Multitimbral synthesizers. Channels 13 through 16 hold the

version written for the Base Multitimbral synthesizer. MIDI

authors put the most important melodic timbres in lower melodic

MIDI channels, so they decide which timbres drop if the

synthesizer s multitimbral capacity is exceeded.

MIDI authors benefit from the additional effort of authoring for

two synthesizer types. Their files can be played back on a wide

variety of Multimedia PC synthesizers without being modified.

General MIDI Patch Name Source

Acoustic grand piano    0

Bright acoustic piano   1

Electric grand piano    2

Honky-tonk piano        3

Rhodes piano            4

Chorused piano          5

Harpsichord             6

Clavinet                7

Celesta                 8

Glockenspiel            9

Music box               10

Vibraphone              11

Marimba                 12

Xylophone               13

Tubular bells           14

Dulcimer                15

Hammond organ           16

Percussive organ        17

Rock organ              18

Church organ            19

Reed organ              20

Accordion               21

Harmonica               22

Tango accordion         23

Acoustic nylon guitar   24

Acoustic steel guitar   25

Electric jazz guitar    26

Electric clean guitar   27

Electric muted guitar   28

Overdriven guitar       29

Distortion guitar       30

Guitar harmonics        31

Acoustic bass           32

Electric bass fingered  33

Electric bass picked    34

Fretless bass           35

Slap bass 1             36

Slap bass 2             37

Synth bass 1            38

Synth bass 2            39

Violin                  40

Viola                   41

Cello                   42

Contrabass              43

Tremolo strings         44

Pizzicato strings       45

Orchestral harp         46

Timpani                 47

String ensemble 1       48

String ensemble 2       49

Synth strings 1         50

Synth strings 2         51

Choir aahs              52

Voice oohs              53

Synth voice             54

Orchestra hit           55

Trumpet                 56

Trombone                57

Tuba                    58

Muted trumpet           59

French horn             60

Brass section           61

Synth brass 1           62

Synth brass 2           63

Soprano sax             64

Alto sax                65

Tenor sax               66

Baritone sax            67

Oboe                    68

English horn            69

Bassoon                 70

Clarinet                71

Piccolo                 72

Flute                   73

Recorder                74

Pan flute               75

Bottle blow             76

Shakuhachi              77

Whistle                 78

Ocarina                 79

Synth lead 1 - Square wave lead         80

Synth lead 2 - Sawtooth wave lead       81

Synth lead 3 - Calliope lead            82

Synth lead 4 - Chiff lead               83

Synth lead 5 - Charang                  84

Synth lead 6 - Solo synth voice         85

Synth lead 7 - Bright saw wave lead     86

Synth lead 8 - Brass and lead           87

Synth pad 1 - Fantasia pad              88

Synth pad 2 - Warm pad                  89

Synth pad 3 - Poly synth pad            90

Synth pad 4 - Space voices pad          91

Synth pad 5 - Bowed glass pad           92

Synth pad 6 - Metal pad                 93

Synth pad 7 - Halo pad                  94

Synth pad 8 - Sweep pad                 95

Synth SFX 1 - Ice rain                  96

Synth SFX 2 - Soundtrack                97

Synth SFX 3 - Crystal                   98

Synth SFX 4 - Atmosphere                99

Synth SFX 5 - Brightness                100

Synth SFX 6 - Goblin                    101

Synth SFX 7 - Echo drops                102

Synth SFX 8 - Star theme                103

Sitar                                   104

Banjo                                   105

Shamisen                                106

Koto                                    107

Kalimba                                 108

Bagpipe                                 109

Fiddle                                  110

Shanai                                  111

Tinkle bell                             112

Agogo                                   113

Steel drums                             114

Woodblock                               115

Taiko drum                              116

Melodic tom                             117

Synth drum                              118

Reverse cymbal                          119

Guitar fret noise                       120

Breath noise                            121

Seashore                                122

Bird tweet                              123

Telephone ring                          124

Helicopter                              125

Applause                                126

Gunshot                                 127

Display Adapter

VGA-compatible display adapter and a color VGA-compatible monitor.

A basic Multimedia PC uses mode 12h (640-by-480, 16 colors). An

enhanced configuration referred to as VGA+ is recommended with

640-by-480, 256 colors.

The recommended performance goal for VGA+ adapters is to be able

to blt 1-bit-per-pixel, 4-bit-per-pixel, and 8-bit-per-pixel

device-independent bitmaps (DIBs) at 350K pixels per second

given 100 percent of the CPU and at 140K pixels per second given

40 percent of the CPU. This recommendation applies to run-length

encoded images and non-encoded images. The recommended

performance is needed to fully support high-performance

applications such as synchronized audiovisual presentations.

User Input

Standard 101-key IBMR-style keyboard with standard DIN connector

Two-button mouse with bus connector

I/O

Standard 9-pin or 25-pin asynchronous serial port, programmable

up to 9600 baud, and a switchable interrupt channel

Standard 25-pin bidirectional parallel port with interrupt

capability

One MIDI port with In, Out, and Thru; must have interrupt

support for input and FIFO transfer

IBM-style analog or digital joystick port

SYSTEM SOFTWARE

The Multimedia PC system software shall conform to the APIs,

functionality, and performance described in the Microsoft

Windows version 3.0 Software Development Kit (SDK)

Reference Volume I manual and Volume 2 manual (version 3.0) and

the Microsoft Multimedia Development Kit (MDK) Programmer s

Reference manual.