Function Requests Specification

There is a need for access to features from the MSCDEX redirector that transend DOS

capabilities. This proposal documents a means that the application can use to talk directly

to MSCDEX to request information or set parameters that only MSCDEX can provide. This

document outlines some of the features I think MSCDEX should support. Comments and

suggestions are welcome.

Access to these functions is provided through an INT 2Fh interface. AH contains 15h which

is what MSCDEX will use to tell its requests from those of other INT 2Fh handlers. AL will

contain the code of the function to be performed.

Function Request Command Codes:

Contents of AL   Function

00h              Get Number of CD-ROM Drive Letters

01h              Get CD-ROM Drive Device List

02h              Get Copyright File Name

03h              Get Abstract File Name

04h              Get Bibliographic Doc File Name

05h              Read VTOC

06h              Turn Debugging On

07h              Turn Debugging Off

08h              Absolute Disk Read

09h              Absolute Disk Write

0Ah              Reserved

0Bh             CD-ROM Drive Check

0Ch             MSCDEX Version

0Dh             Get CD-ROM Drive Letters

0Eh             Get/Set Volume Descriptor Preference

0Fh             Get Directory Entry

10h             Send Device Request

11h-0FFh         Reserved

Get Number of CD-ROM Drive Letters

AX   1500h

BX   Number of CD-ROM drive letters used

CX   Starting drive letter of CD-ROM drive letters (A=0, B=1, ...Z=25)

MSCDEX will return the number of CD-ROM drive letters in BX and the starting drive letter

in CX. The first CD-ROM device will be installed at the starting drive letter and subsequent

drives will be assigned the next greater drive letter. A single device driver may be assigned

to more than one drive letter, such as the case of a device driver that supports multiple

units. MSCDEX keeps track of which sub-unit a particular drive letter is assigned to.

NOTE: This function can be used to determine if MSCDEX is installed by setting BX to zero

before executing INT 2Fh. MSCDEX is not installed if BX is still zero on return.

Also, in a networking environment, one cannot assume that drive letters will always be

assigned contiguously beginning with the starting drive letter. Use function Get CD-ROM drive

letters instead.

Get CD-ROM Drive Device List

AX        1501h

ES:BX     Transfer address; pointer to buffer to copy drive letter device list

The buffer must be large enough to hold the device list. By calling function Get Number of

CD-ROM Drive Letters, one can find out the number of CD-ROM drive letters and the buffer size

will be a multiple of that. This will be an absolute maximum of 26. Each drive letter device

entry will consist of one byte for the sub-unit followed by 4 bytes for the address of the

device header assigned to that drive letter. This byte for the sub-unit takes care of the

problem of distinguishing which unit is assigned to which drive letter for device drivers

that handle sub-units.

For example: Suppose there are two installed CD-ROM device drivers, FOO, which supports 1

sub-unit, and BAR, which supports two sub-units, on a system with 2 floppy drives (A=0 and

B=1) and a hard disk (C=2). Then asking for the number of CD-ROM drive letters will report

that there are 3 drive letters used starting at drive letter D=3. ES:BX must point to a

buffer that is at least 3 * 5 = 15 bytes long. The buffer will be filled as follows:

ES:BX  = Buffer

Buffer   DB   0                        ; sub-unit of FOO on drive letter D:

DD   <far addr of FOO device header>

DB   0                        ; sub-unit of BAR on drive letter E:

DD   <far addr of BAR device header>

DB   1                        ; sub-unit of BAR on drive letter F:

DD   <far addr of BAR device header>

Get Copyright File Name

AX     1502h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

MSCDEX will copy the name of the copyright file in the VTOC for that drive letter into the

buffer space provided. The copyright filename is presently restricted in the High Sierra

proposal to 8.3 but we require 38 bytes here for the possibility at a later date of handling

31 character file names plus 6 bytes for a ';' and 5 digit version number and 1 byte for a

NULL at the end. Carry will be set if the drive letter is not a CD-ROM drive and

error_invalid_drive (15) will be returned in AX.

Get Abstract File Name

AX     1503h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

MSCDEX will copy the name of the abstract file in the VTOC for that drive letter into the

buffer space provided. The abstract filename is presently restricted in the High Sierra

proposal to 8.3 but we require 38 bytes here for the possibility at a later date of handling

31 character file names plus 6 bytes for a ';' and 5 digit version number and 1 byte for a

NULL at the end. Carry will be set if the drive letter is not a CD-ROM drive and

error_invalid_drive (15) will be returned in AX.

Get Bibliographic Documentation File Name

AX     1504h

ES:BX  Transfer address; pointer to a 38 byte buffer

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

NOTE: This function is provided in advance of the ISO standard. For discs complying with the

May 28th draft from the High Sierra Group, this function will return a null string as though

the field is blank on the disc.

MSCDEX will copy the name of the bibliographic documentation file in the VTOC for that drive

letter into the buffer space provided. The bibliographic documentation filename is presently

restricted in the High Sierra proposal to 8.3 but we require 38 bytes here for the

possibility at a later date of handling 31 character file names plus 6 bytes for a ';' and

5 digit version number and 1 byte for a NULL at the end. Carry will be set if the drive

letter is not a CD-ROM drive and error_invalid_drive (15) will be returned in AX.

Read VTOC

AX     1505h

ES:BX  Transfer address; pointer to a 2048 byte buffer

CX     CD-ROM Drive letter

DX     Sector index

This function is provided to scan the Volume Descriptors on a disc. A sector index of 0 will

read the first volume descriptor, 1 reads the second, etc. If there is no error, then AX will

return 1 if the volume descriptor read was the standard volume descriptor, 0FFh if it was

the volume descriptor terminator and there are no more volume descriptors to be read, and

0 for all other types.

If there is an error in processing the request, the Carry Flag will be set and AL will

contain the MS-DOS error code. These will be either error_invalid_drive (15) or

error_not_ready (21).

Turn Debugging On

AX     1506h

BX     Debugging function to enable

This is used for development and is reserved. It will be non-functional in the production

version of MSCDEX.

Turn Debugging Off

AX     1507h

BX     Debugging function to disable

This is used for development and is reserved. It will be non-functional in the production

version of MSCDEX.

Absolute Disk Read

AX     1508h

ES:BX  Disk Transfer Address; pointer to a buffer to copy data to

CX     CD-ROM Drive letter (A=0, B=1, ... Z=25)

DX     Number of sectors to read

SI:DI  Starting sector

This function corresponds to INT 25h. It will be converted directly into a READ_LONG device

driver request and sent to the correct device driver. There are no requirements for this call

to pop flags as there are with INT 25h. SI holds the high word and DI the low word for the

starting sector to begin reading from.

If there is an error in processing the request, the Carry Flag will be set and AL will

contain the MS-DOS error code. These will be either error_invalid_drive (15) or

error_not_ready (21).

Absolute Disk Write

AX     1509h

ES:BX  Disk Transfer Address; pointer to buffer to copy data from

CX     CD-ROM Drive letter

DX     Number of sectors to write

SI:DI  Starting sector

This function corresponds to INT 26h. It is not supported at this time and is reserved. It

is intended to be used by authoring systems.

CD-ROM Drive Check

AX     150Bh

BX     Signature word

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

This function returns whether or not a drive letter is a CD-ROM drive supported by MSCDEX.

If the extensions are installed, BX will be set to ADADh. If the drive letter is supported

by MSCDEX, then AX is set to a non- zero value. AX is set to zero if the drive is not

supported. One must be sure to check the signature word to know that MSCDEX is installed and

that AX has not been modified by another INT 2Fh handler.

MSCDEX Version

AX     150Ch

BX     MSCDEX Version

This function returns the version number of the CD-ROM Extensions installed on the system.

BH contains the major version number and BL contains the minor version. Values returned are

binary. For example, BX would contain 0x020a for version 2.10. This function does not work

on versions earlier than 2.00 so if BX is zero before and after this function is called, an

earlier version of MSCDEX is installed.

Get CD-ROM Drive Letters

AX     150Dh

ES:BX  Transfer address; pointer to buffer to copy drive letter device list

The buffer must be large enough to hold a list of drive letters. The buffer size will be a

multiple of the number of drives returned by the Get Number of CD-ROM Drive Letters function.

There are a maximum of 26 drive letters. Each drive letter entry is a single byte (0=A:, 1=B:

.. 25=Z:) that exactly corresponds each respective entry returned by the command Get CD-ROM

Drive Device List. This command is included to allow applications to locate CD-ROM drives

supported by MSCDEX. CD-ROM drive letters may sometimes be noncontiguous so this command is

necessary.

For example: Suppose there is an installed CD-ROM device driver FOO supporting 3 sub-units

on a system with 2 floppy drives (A=0 and B=1), a hard disk (C=2) and a network drive (E=4).

Note the network drive occupies one of the drive letters normally taken by a CD-ROM drive.

MSCDEX assigns that CD-ROM drive to the next available drive letter. Asking for the number

of CD-ROM drive letters reports there are 3 drive letters used starting at drive letter D=3.

ES:BX must point to a buffer that is at least 3 bytes long and will be filled as follows:

ES:BX   = Buffer

Buffer  DB   3                   ; drive letter for CD-ROM (D=3)

DB   5                   ; drive letter for CD-ROM (F=5)

DB   6                   ; drive letter for CD-ROM (G=6)

Get/Set Volume Descriptor Preference

AX     150Eh

BX     0 - Get Preference. 1 - Set Preference

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

DX     if BX = Get Preference

DX = 0

MSCDEX will return preference settings in DX

if BX = Set Preference

DH = volume descriptor preference

1 - PVD - Primary Volume  Descriptor

2 - SVD - Supplementary Volume Descriptor

DL = Supplementary Volume Descriptor Preference

if DH = PVD

DL = 0

if DH = SVD

1 - shift-Kanji (an unregistered ISO coded character set)

Normally, MSCDEX will scan for the PVD (Primary Volume Descriptor) when initializing a CD-

ROM. This behavior can be altered for each individual drive to scan for a SVD (Supplementary

Volume Descriptor) instead. A CD-ROM drive set to scan for an SVD will use the PVD if there

is no SVD present. There can be more than one SVD on a CD-ROM but at present, MSCDEX will

only recognize SVDs for shift-Kanji CD-ROMs. Carry will be set, AX will be set to

error_invalid_function (1) and DX will be set to 0 if the coded character set is not

recognized.

If BX contains Get_Preference, MSCDEX will report the present setting for that drive. If DX

is still zero on return, that version of MSCDEX does not support this function or reading

SVDs. Otherwise DX will contain the setting.

If the drive letter is not a CD-ROM drive, carry will be set and error_invalid_drive (15)

will be returned in AX. If BX is anything other than Get/Set_Preference, AX will be set to

error_invalid_function (1) and carry will be set.

Get Directory Entry

AX     150Fh

CX     CD-ROM Drive letter (A=0, B=1,...Z=25)

ES:BX  Pointer to buffer with null-terminated path name

SI:DI  Pointer to buffer to copy directory record information

AX     0 is returned if the disc is High Sierra, 1 is returned if the

disc is ISO-9660

The pathname expected is a null-terminated string e.g. char far *path = "\\a\\b\\c.txt";

(note: the "\\" characters map to a single '\' character in C so this would be '\a\b\c.txt'

if printed). The path must consist only of valid High Sierra or ISO-9660 filename characters

and must not contain any wildcards nor may it include entries for '.' or '..'.

The buffer to copy the directory record to can be a maximum of 255 bytes long including all

system use information. The directory record is a direct copy from the directory file and

it is up to the application to choose what fields to use.

Carry will be set and an error code returned if there were problems with the request. The

error codes will be error_invalid_drive (15) if the drive letter is incorrect,

error_not_ready (21) if the disc didn't initialize correctly, error_file_not_found (2) if

the file was not found and error_no_more_files (18) if the pattern fails to find a match or

if mscdex failed to allocate buffers.

The format of the directory record for High Sierra discs is:

/* High Sierra directory entry structure */

typedef struct hsg_dir_entry {

uchar      len_dr;        /* length of this directory entry  */

uchar      XAR_len;       /* length of XAR in LBN's          */

ulong      loc_extentI;   /* LBN of data Intel format        */

ulong      loc_extentM;   /* LBN of data Molorola format     */

ulong      data_lenI;     /* length of file Intel format     */

ulong      data_lenM;     /* length of file Motorola format  */

uchar      record_time[6];/* date and time                   */

uchar      file_flags_hsg;/* 8 flags                         */

uchar      reserved;      /* reserved field                  */

uchar      il_size;       /* interleave size                 */

uchar      il_skip;       /* interleave skip factor          */

ushort     VSSNI;         /* volume set sequence num Intel   */

ushort     VSSNM;         /* volume set sequence num Motorola*/

uchar      len_fi;        /* length of name                  */

uchar      file_id[...];  /* variable length name upto 32 chars       */

uchar      padding;       /* optional padding if file_id is odd length*/

uchar      sys_data[...]  /* variable length system data              */

} hsg_dir_entry;

The format of the directory record for ISO-9660 discs is:

/* ISO-9660 directory entry structure */

typedef struct iso_dir_entry {

uchar      len_dr;        /* length of this directory entry  */

uchar      XAR_len;       /* length of XAR in LBN's          */

ulong      loc_extentI;   /* LBN of data Intel format        */

ulong      loc_extentM;   /* LBN of data Molorola format     */

ulong      data_lenI;     /* length of file Intel format     */

ulong      data_lenM;     /* length of file Motorola format  */

uchar      record_time[7];/* date and time                   */

uchar      file_flags_iso;/* 8 flags                         */

uchar      il_size;       /* interleave size                 */

uchar      il_skip;       /* interleave skip factor          */

ushort     VSSNI;         /* volume set sequence num Intel   */

ushort     VSSNM;         /* volume set sequence num Motorola*/

uchar      len_fi;        /* length of name                  */

uchar      file_id[...];  /* variable length name upto 32 chars       */

uchar      padding;       /* optional padding if file_id is odd length*/

uchar      sys_data[...]  /* variable length system data              */

} iso_dir_entry;

The difference between the two forms is the file flag byte moved to account for an additional

byte of date and time used for a Greenwich mean time offset. See the May 28th draft of the

High Sierra proposal or ISO-9660 for a more complete explanation of the fields. Note that

the C structs above are not syntactically correct; C does not allow variable length arrays

as struct elements.

Send Device Driver Request

AX     1510h

CX     CD-ROM drive letter (A=0, B=1, ... Z=25)

ES:BX  Address of CD-ROM device driver request header

This function has been added to simplify communication with CD-ROM drivers and help prevent

contention between applications that wish to communicate with the device driver. It is highly

recommended that all applications communicate with device drivers through this function

request. Applications using this function will not have to locate the device driver. The

format of the request header is specified by the Microsoft MS-DOS CD-ROM Extensions Hardware-

Dependent Device Driver Specification.

Hardware-Dependent Device Driver Specification

Intent

This document (Document Number: 000080010-100-O00-1186) describes the CD-ROM hardware-

dependent device driver and its interface with MSCDEX.EXE, the MS- DOS CD-ROM Extensions

resident program. Differences between CD-ROM drives and hard- or floppy-disk drives account

for the differences in this device driver specification from the normal MS-DOS block and

character device driver specification. The chapters on device drivers in the MS-DOS

Programmer's Reference Manual (MS-PRM) provide more information.

The MS-DOS operating system reads CONFIG.SYS and installs the device. MSCDEX.EXE performs

an open system call on the device driver name in order to communicate with it and uses an

IOCTL call to ask the device driver for the address of its device header. From the device

header address, MSCDEX.EXE locates the device driver's interrupt and strategy routines. After

that, all requests the device driver receives come directly from MSCDEX.EXE, not MS- DOS.

To avoid reentrancy problems and allow MSCDEX to monitor all media changes, all other

applications that wish to communicate directly with CD- ROM device drivers should do so

through the Send Device Driver Request INT 2Fh function 10h. MSCDEX.EXE interfaces with MS-

DOS so that normal requests for I/O with files on a CD-ROM drive down to the MS-DOS INT 21h

service layer will work just as they would for a normal MS-DOS device.

Installation

The device driver will be installed in the same way as any other device with an entry in

CONFIG.SYS. The syntax is:

DEVICE=<filename> /D:<device_name> /N:<number of drives>

The following are examples:

DEVICE=HITACHI.SYS /D:MSCD001 /D:MSCD002

DEVICE=SONY.SYS    /D:MSCD003 /N:2

The arguments will be the character device names that will be used on the command line when

starting MSCDEX.EXE so that it can find and communicate with the device driver.

A device driver may support one or more physical drives or logical disks. This may be done

by having multiple device headers in the device driver file (in which case it will be

necessary to have more than one device_name on the command line - one for each device header;

see the HITACHI.SYS example above) or through the use of subunits. Each disk handled by a

device driver that supports multiple disks using subunits is addressed by the subunit field

of the request header when a request is made for that disk. A device driver that supports

more than one disk can share code and data instead of requiring separate device drivers for

each disk. A "jukebox" CD-ROM system would be an example of a CD-ROM device that might wish

to support more than one drive or a disk pack using a single device driver.

Device drivers that use multiple subunits should use the optional switch /n:<number of

drives> to say how many drives are present. If not present, the default number of drives is

1. If the driver can tell how many drives are installed without a command line switch, then

this argument is not necessary. Unless there are special considerations, it is better

practice to support multiple drives using subunits than to have multiple device headers in

the same device driver file.

Device Header

The device header is an extension to what is described in the MS-PRM.

DevHdr    DD   -1        ; Ptr to next driver in file or -1 if last driver

DW   ?         ; Device attributes

DW   ?         ; Device strategy entry point

DW   ?         ; Device interrupt entry point

DB   8 dup (?) ; Character device name field

DW   0         ; Reserved

DB   0         ; Drive letter

DB   ?         ; Number of units

The following are the device attributes for MSCDEX.EXE device drivers:

Bit 15         1       - Character device

Bit 14         1       - IOCTL supported

Bit 13         0       - Output 'till  busy

Bit 12         0       - Reserved

Bit 11         1       - OPEN/CLOSE/RM supported

Bit 10-4       0       - Reserved

Bit  3         0       - Dev is CLOCK

Bit  2         0       - Dev is NUL

Bit  1         0       - Dev is STO

Bit  0         0       - Dev is STI

MSCDEX.EXE device drivers will be character devices that understand IOCTL calls and handle

OPEN/CLOSE/RM calls.

The drive letter field is a read-only field for the device driver and is initialized to 0.

The field is for MSCDEX.EXE to use when it assigns the device driver to a drive letter (A

= 1, B = 2...Z = 26). It should never be modified by the device driver. For drivers that

support more than one unit, the drive letter will indicate the first unit, and each

successive unit is assigned the next higher drive letter. For example, if the device driver

has four units defined (0-3), it requires four drive letters. The position of the driver in

the list of all drivers determines which units correspond to which drive letters. If driver

ALPHA is the first driver in the device list, and it defines 4 units (0-3), they will be A,

B, C, and D. If BETA is the second driver and defines three units (0-2), they will be E, F,

and G, and so on. The theoretical limit to the number of drive letters is 63, but it should

be noted that the device installation code will not allow the installation of a device if

it would result in a drive letter > 'Z' (5Ah). All block device drivers present in the

standard resident BIOS will be placed ahead of installable device drivers in the list.

NOTE:

It is important that one set lastdrive=<letter> in CONFIG.SYS to accommodate the additional

drive letters that CD-ROM device drivers will require.

The number-of-units field is set by the device driver to the number of disks that are

supported. Normal character devices do not support more than one unit and MS-DOS does not

expect a character device to handle more than one unit or have a nonzero subunit value in

the request header. Since these device drivers are not called by MS-DOS directly, this is

not a problem. Nonetheless, the number of units returned by the device driver in the number-

of-units field during the INIT call must be 0, since MS-DOS makes the INIT call and does not

expect a nonzero value for a character device. MSCDEX.EXE will never see what is returned

anyway, and relies on the number- of-units field in the device header.

Sample device header:

HsgDrv    DD   -1             ; Pointer to next device

DW   0c800h         ; Device attributes

DW   STRATEGY       ; Pointer to device strategy routine

DW   DEVINT         ; Pointer to device interrupt routine

DB   'HSG-CD1 '     ; 8-byte character device name field

DW   0              ; Reserved (must be zero)

DB   0              ; Drive letter (must be zero)

DB   1              ; Number of units supported (one or more)

As with other MS-DOS device drivers, the code originates at offset 0, not 100H. The first

device header will be at offset 0 of the code segment. The pointer to the next driver is a

double word field (offset/segment) that is the address of the next device driver in the list,

or -1 if the device header is the only one or the last in the list. The strategy and

interrupt entry points are word fields and must be offsets into the same segment as the

device header. The device driver is expected to overwrite the name(s) in each of its one or

more device headers with the <device_name> command line arguments during its initialization.

MSCDEX.EXE will call the device driver in the following manner:

1.  MSCDEX.EXE makes a far call to the strategy entry.

2.  MSCDEX.EXE passes device driver information in a request header to the strategy

routine.

3.  MSCDEX.EXE makes a far call to the interrupt entry.

Request header

MSCDEX.EXE will call the device's strategy routine with the address of a request header in

ES:BX. The format of the request header is the same as what is described in the MS-PRM.

ReqHdr    DB   ?         ; Length in bytes of request header

DB   ?         ; Subunit code for minor devices

DB   ?         ; Command code field

DW   ?         ; Status

DB   8 dup (?) ; Reserved

Status

The status word also has the same format as described in the MS-PRM. It is 0 on entry and

is set by the device driver.

Bit 15         - Error bit

Bit 14-10      - Reserved

Bit  9         - Busy

Bit  8         - Done

Bit  7-0       - Error code (bit 15 on)

Bit 15, the error bit, is set by the device driver if an error is detected or if an invalid

request is made to the driver. The low 8 bits indicate the error code.

Bit 9, the busy bit, should be set by the device driver when the drive is in audio play mode.

Device drivers should fail all requests to the physical device that require head movement

when the device is playing and return the request with this bit and the error bit set and

an error code. Requests that would not interrupt audio play may return without error but will

also have this bit set when the drive is in audio play mode. Play mode can be terminated

prematurely with a reset or STOP AUDIO request and a new request can be made at that point.

Monitoring this bit in each successive request, an Audio Q-Channel Info IOCTL for example,

will tell when play mode is complete.

Bit 8, the done bit, is set by the device driver when the operation is finished.

Error codes are the following:

0  Write-protect violation

1  Unknown unit

2  Drive not ready

3  Unknown command

4  CRC error

5  Bad drive request structure length

6  Seek error

7  Unknown media

8  Sector not found

9  Printer out of paper

A  Write fault

B  Read fault

C  General failure

D  Reserved

E  Reserved

F  Invalid disk change

Command Code Field

The following values are valid command codes:

0  INIT

1   MEDIA CHECK (block devices)

2   BUILD BPB (block devices)

3  IOCTL INPUT

4   INPUT (read)

5   NONDESTRUCTIVE INPUT NO WAIT

6   INPUT STATUS

7  INPUT FLUSH

8   OUTPUT (write)

9   OUTPUT WITH VERIFY

10   OUTPUT STATUS

11  OUTPUT FLUSH

12  IOCTL OUTPUT

13  DEVICE OPEN

14  DEVICE CLOSE

15   REMOVABLE MEDIA (block devices)

16   OUTPUT UNTIL BUSY

128  READ LONG                (NEW)

129   Reserved

130  READ LONG PREFETCH       (NEW)

131  SEEK                     (NEW)

132  PLAY AUDIO               (NEW)

133  STOP AUDIO               (NEW)

134  WRITE LONG               (NEW)

135  WRITE LONG VERIFY        (NEW)

136  RESUME AUDIO             (NEW)

Unsupported or illegal commands will set the error bit and return the error code for Unknown

Command. This includes command codes 1, 2, 4, 5, 6, 8, 9, 10, 15, 16, and 129; and 11, 134

and 135 for systems that do not support writing.

If, in the time since the last request to that device driver unit, the media has changed,

the device driver will return the error code for invalid disk change and set the error bit.

MSCDEX.EXE will then decide whether to retry the request or abort it.

The minimal CD-ROM device driver will read cooked Mode 1 data sectors using HSG addressing

mode and return appropriate values for the IOCTL calls. Most other features enhance

performance or add useful capabilities.

INIT

Command code = 0

ES:BX = INIT

INIT      DB   13 dup (0); Request header

DB   0         ; Number of units (must be 0)

DD   ?         ; End address

DD   ?         ; Ptr to BPB array

DB   0         ; Block device number

This call is made only once, when the device is installed. INIT and a single IOCTL call for

the device header address are the only device driver calls that come directly from MS-DOS.

Because the INIT function is called from MS- DOS, the number of units returned is 0, as for

normal MS-DOS character devices. MSCDEX.EXE will get the number of units supported from the

device header.

The device must return the END ADDRESS, which is a DWORD pointer to the end of the portion

of the device driver to remain resident. Code and data following the pointer is used for

initialization and then discarded. If there are multiple device drivers in a single file,

the ending address returned by the last INIT call will be the one that MS-DOS uses, but it

is recommended that all the device drivers in the file return the same address. The code to

remain resident for all the devices in a single file should be grouped together low in memory

with the initialization code for all devices following it in memory.

The pointer to BPB array points to the character after the "=" on the line in CONFIG.SYS that

caused this device driver to be loaded. This data is read-only and allows the device driver

to scan the invocation line for parameters. This line is terminated by a carriage return or

a line feed.

During initialization, the device driver must set the device name field in the device header

to the argument provided on the invocation line in CONFIG.SYS. The device driver must also

check that the device_name command line argument is a legal 8-character filename and pad it

out to 8 characters with spaces (20H) when copying it to the device name field.

The block device number and number of units are both 0 for character devices.

READ (IOCTL Input)

Command code = 3

ES:BX = IOCTLI

IOCTLI    DB   13 dup (0); Request header

DB   0         ; Media descriptor byte from BPB

DD   ?         ; Transfer address

DW   ?         ; Number of bytes to transfer

DW   0         ; Starting sector number

DD   0         ; DWORD ptr to requested vol ID if error 0FH

The media descriptor byte, starting sector number, and volume ID fields are all 0.

The transfer address points to a control block that is used to communicate with the device

driver. The first byte of the control block determines the request that is being made. If

the command code is reserved or the function not supported, then the device driver will

return the error code for Unknown Command. If, for some reason, the device driver is not able

to process the request at that time, it will return the error code for Drive Not Ready.

Number of Bytes

Code       to Transfer                 Function

0             5            Return Address of Device Header

1             6            Location of Head

2             ?            Reserved

3             ?            Error Statistics

4             9            Audio Channel Info

5           130            Read Drive Bytes

6             5            Device Status

7             4            Return Sector Size

8             5            Return Volume Size

9             2            Media Changed

10             7            Audio Disk Info

11             7            Audio Track Info

12            11            Audio Q-Channel Info

13            13            Audio Sub-Channel Info

14            11            UPC Code

15            11            Audio Status Info

16-255        ?             Reserved

Return Address of Device Header

Raddr     DB   0         ; Control block code

DD   ?         ; Address of device header

The device driver will fill the 4-byte field with the address of its device header. This is

used by MSCDEX.EXE to locate the device driver's strategy and interrupt routines.

Location of Head

LocHead   DB   1         ; Control block code

DB   ?         ; Addressing mode

DD   ?         ; Location of drive head

The device driver will return a 4-byte address that indicates where the head is located. The

value will be interpreted based on the addressing mode. (See function READ LONG for more

information about addressing modes.)

NOTE: The drive could provide this information by monitoring the Q-channel on the disk.

Error Statistics

ErrStat   DB   3         ; Control block code

DB   N dup (?) ; Error statistics

The format of the Error Statistics is not yet defined.

Audio Channel Info

AudInfo   DB   4       ; Control block code

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 0

DB   ?       ; Volume control (0 - 0xff) for output channel 0

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 1

DB   ?       ; Volume control (0 - 0xff) for output channel 1

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 2

DB   ?       ; Volume control (0 - 0xff) for output channel 2

DB   ?       ; Input  channel (0, 1, 2, or 3) for output channel 3

DB   ?       ; Volume control (0 - 0xff) for output channel 3

This function returns the present settings of the audio channel control set with the Audio

Channel Control Ioctl Write function. The default settings for the audio channel control are

for each input channel to be assigned to its corresponding output channel (0 to 0, 1 to 1,

etc.) and for the volume control on each channel is set at 0xff.

Read Drive Bytes

DrvBytes  DB   5          ; Control block code

DB   ?          ; Number bytes read

DB   128 dup (?); Read buffer

Data returned from the CD-ROM drive itself can be read using this function. The number-bytes-

read field returns the length of the number of bytes read, which will not exceed 128 per

call. If more than this needs to be returned, the call will be repeated until the number

returned is 0.

The function and content of these bytes are entirely device and device driver dependent. This

function is provided to allow access to device- specific features that are not addressed

under any other portion of the device driver spec.

Device Status

DevStat   DB   6         ; Control block code

DD   ?         ; Device parameters

The device driver will return a 32-bit value. Bit 0 is the least significant bit. The bits

are interpreted as follows:

Bit 0     0    Door closed

1    Door open

Bit 1     0    Door locked

1    Door unlocked

Bit 2     0    Supports only cooked reading

1    Supports cooked and raw reading

Bit 3     0    Read only

1    Read/write

Bit 4     0    Data read only

1    Data read and plays audio/video tracks

Bit 5     0    No interleaving

1    Supports interleaving

Bit 6     0    Reserved

Bit 7     0    No prefetching

1    Supports prefetching requests

Bit 8     0    No audio channel manipulation

1    Supports audio channel manipulation

Bit 9     0    Supports HSG addressing mode

1    Supports HSG and Red Book addressing modes

Bit 10-31 0    Reserved (all 0)

Return Sector Size

SectSize  DB   7         ; Control block code

DB   ?         ; Read mode

DW   ?         ; Sector size

The device driver will return the sector size of the device given the read mode provided.

In the case of CD-ROM, the value returned for cooked is 2048, and the return value  for raw

is 2352.

Return Volume Size

VolSize   DB   8         ; Control block code

DD   ?         ; Volume size

The device driver will return the number of sectors on the device. The size returned is the

address of the lead-out track in the TOC converted to a binary value according to FRAME +

(SEC * 75) + (MIN * 60 * 75). A disc with a lead out track starting at 31:14.63 would return

a volume size of 140613. The address of the lead-out track is assumed to point to the first

sector following the last addressable sector recorded on the disc.

Media Changed

MedChng   DB   9         ; Control block code

DB   ?         ; Media byte

The normal media check function (command code 1) is not performed on character devices and

contains additional semantics that are not needed for CD-ROM device drivers. This is why

there is an IOCTL request for this function.

When the device driver receives a call to see if the media has changed on that subunit, it

will return one of the following values:

1         Media not changed

0         Don't know if changed

-1 (0FFh)  Media changed

If the driver can assure that the media has not been changed (through a door-lock or other

interlock mechanism), performance is enhanced because MSCDEX.EXE does not need to reread the

VTOC and invalidate in-memory buffers for each directory access. For drives that do not

report if the media has changed, CD-ROM device drivers can utilize the same solution that

has been applied to floppy disks. In some floppy-disk device drivers, if the MEDIA CHECK

occurs within 2 seconds of a floppy-disk access, the driver reports "Media not changed." It

is highly recommended though that drives be able to detect and report media changes.

If the drive can enforce a door lock mechanism so that the device driver is notified when

the door lock has been unlocked or the device driver is requested to do so by MSCDEX.EXE,

then to improve performance, the driver could return that the media has not changed without

bothering to communicate with the physical device.

If the media has not been changed, MSCDEX.EXE will proceed with the disk access. If the value

returned is "Don't know," or "Media changed," then MSCDEX.EXE will check to see if the disk

has changed. It will  continue if it has not, and reinitialize what it knows about the disk

if it has.

It is not necessary for the device driver to do anything for the volume ID when the media

has changed.

Audio Disk Info

DiskInfo  DB   10        ; Control block code

DB   ?         ; Lowest track number

DB   ?         ; Highest track number

DD   ?         ; Starting point of the lead-out track

This function returns TOC (Table of Contents) information from the Q-Channel in the lead-in

track indicating what the first and last track numbers are and the Red Book address for the

lead-out track (PMIN/PSEC/PFRAME when POINT = A2). The first and last track numbers are

binary values and not BCD. It is recommended that the information for Audio Disk Info and

Audio Track Info should be read by the drive when the disc is initialized and made accessible

to the driver so that when these functions are called, the drive or driver do not have to

interrupt audio play to read them from the TOC. If the TOC is not made available to the

driver and the driver must obtain the information itself from the lead-in track, the driver

should read and and attempt to cache the disk and track information during the Audio Disk

Info command and invalidate this information only if the media changes.

Audio Track Info

TnoInfo   DB   11        ; Control block code

DB   ?         ; Track number

DD   ?         ; Starting point of the track

DB   ?         ; Track control information

This function takes a binary track number, from within the range specified by the lowest and

highest track number given by the Audio Disk Info command, and returns the Red Book address

for the starting point of the track and the track control information for that track. The

track control information byte corresponds to the byte in the TOC in the lead-in track

containing the two 4-bit fields for CONTROL and ADR in the entry for that track. The CONTROL

information is in the most significant 4 bits and the ADR information is in the lower 4 bits.

The track control information is encoded as follows:

00x00000  - 2 audio channels without pre-emphasis

00x10000  - 2 audio channels with pre-emphasis

10x00000  - 4 audio channels without pre-emphasis

10x10000  - 4 audio channels with pre-emphasis

01x00000  - data track

01x10000  - reserved

11xx0000  - reserved

xx0x0000  - digital copy prohibited

xx1x0000  - digital copy permitted

Audio Q-Channel Info

QInfo     DB   12        ; Control block code

DB   ?         ; CONTROL and ADR byte

DB   ?         ; Track number (TNO)

DB   ?         ; (POINT) or Index (X)

; Running time within a track

DB   ?         ; (MIN)

DB   ?         ; (SEC)

DB   ?         ; (FRAME)

DB   ?         ; (ZERO)

; Running time on the disk

DB   ?         ; (AMIN) or (PMIN)

DB   ?         ; (ASEC) or (PSEC)

DB   ?         ; (AFRAME) or (PFRAME)

This function reads and returns the most up to date Q-channel address presently available.

It should not interrupt the present status of the drive as one of its intended purposes is

to monitor the location of the read head while playing audio tracks. This function should

return valid information even when no audio tracks are being played and the head is

stationary. The fields returned correspond to the data that is stored in the Q-channel as

described in the Red Book. The values in MIN-SEC-FRAME, AMIN-ASEC-AFRAME and PMIN-PSEC-PFRAME

are converted by the driver from BCD to binary so that minutes range from 0 to 59+, seconds

from 0 to 59, and frames from 0 to 74. The Control and ADR byte, TNO, and POINT/Index bytes

are always passed through as they appear on the disc and are not converted. If the drive

returns Q-channel information when ADR is not equal to 1, then when ADR is not equal to 1

all ten bytes of information are passed through unmodified to the caller.

Audio Sub-Channel Info

SubChanInfo DB   13        ; Control block code

DD   ?         ; Starting frame address

DD   ?         ; Transfer address

DD   ?         ; Number of sectors to read

This function takes a Red Book address for a particular frame (also known as a block or

frame) and copies 96 bytes of sub-channel information per frame for all the sectors that are

requested sequentially at the transfer address given. Each 96 bytes of information do not

include the two sync patterns (S0 and S1) that head the subcoding block but only the the 96

bytes of subcoding symbols each with one bit of information for the eight different channels

(P-W) that follow them. P is the MSB, W is the LSB of each byte.

The caller is responsible for making sure that 96 * Number_of_sectors_to_read bytes are

available at the transfer address for the device driver to store the results.

Data definition and integrity restrictions for data received with this command are

interpreted according to the CD-ROM standard (Red and Yellow Book).

UPC Code

UPCCode   DB   14        ; Control block code

DB   ?         ; CONTROL and ADR byte

DB   7 dup (?) ; UPC/EAN code

; (last 4 bits are zero; the low-order nibble of ; byte 7)

DB   ?         ; Zero

DB   ?         ; Aframe

This function returns the UPC/EAN (Universal Product Code - BAR coding) for the disc. This

information is stored as a mode-2 (ADR=2) Q-channel entry. The UPC code is 13 successive BCD

digits (4 bits each) followed by 12 bits of zero. The last byte is the continuation of FRAME

in mode-1 though in the lead-in track (TNO=0) this byte is zero. If the CONTROL/ADR byte is

zero or if the 13 digits of UPC code are all zero, then either no catalog number was encoded

on the disc or it was missed by the device driver. If the command is not supported, then the

driver will return an error code of Unknown Command. If the command is supported but the disc

does not have a UPC Code recorded, then the driver will return an error code of Sector not

Found.

Audio Status Info

AudStat DB   15        ; Control block code

DW   ?         ; Audio status bits

; Bit 0 is Audio Paused bit

; Bits 1-15 are reserved

DD   ?         ; Starting location of last Play or for next Resume

DD   ?         ; Ending location for last Play or for next Resume

The Audio Paused bit and Starting and Ending locations are those referred to in the RESUME

command.

WRITE (IOCTL OUTPUT)

Command code = 12

ES:BX = IOCTLO

IOCTLO    DB   13 dup (0); Request header

DB   0         ; Media descriptor byte from BPB

DD   ?         ; Transfer address

DW   ?         ; Number of bytes to transfer

DW   0         ; Starting sector number

DD   0         ; DWORD ptr to requested vol ID if error 0FH

The media descriptor byte, starting sector number, and volume ID fields are all 0.

The transfer address points to a control block that is used to communicate with the device

driver. The first byte of the control block determines the request that is being made. The

Length of Block is the number of bytes to transfer.

Length of Code   Block            Function

0      1              Eject Disk

1      2              Lock/Unlock Door

2      1              Reset Drive

3      9              Audio Channel Control

4      ?              Write Device Control String

5      1              Close Tray

6-255  ?              Reserved

Eject Disk

Eject          DB   0         ; Control block code

The device driver will unlock the drive and eject the CD-ROM disk from the drive unit. The

door will report as being open until the user has inserted a disk  into the drive unit and

closed the door. The status bit for door open can be monitored to determine when a disk has

been reinserted.

Lock/Unlock Door

LockDoor  DB   1         ; Control block code

DB   ?         ; Lock function

When this function is received, the device driver will ask the CD-ROM drive to unlock or lock

the door. If lock function is 0, the device driver will unlock the door. If lock function

is 1, it will lock the door.

Reset Drive

ResetDrv  DB   2         ; Control block code

This function directs the device driver to reset and reinitialize the drive.

Audio Channel Control

AudInfo   DB   3     ; Control block code

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 0 DB   ?     ;

Volume control (0 - 0xff) for output channel 0

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 1 DB   ?     ;

Volume control (0 - 0xff) for output channel 1

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 2 DB   ?     ;

Volume control (0 - 0xff) for output channel 2

DB   ?     ; Input  channel (0, 1, 2, or 3) for output channel 3         DB   ?

; Volume control (0 - 0xff) for output channel 3

This function is intended to provide playback control of audio information on the disk. It

allows input channels on the CD-ROM to be assigned to specific output speaker connections.

The purpose of this function is to allow two independent channels to be recordedin

different languages for exampleand to play back only one of them at a time or to be able

to manipulate an audio signal so that the source appears to moveto make a sound seem to

move from left to right for example.

Output channel 0 is the left channel, 1 is right, 2 is left prime, and 3 is right prime. The

Red Book specification allows for 4 audio channels. The two "prime" channels (2 and 3) extend

stereo to quadrophonic stereo.

An audio volume setting of 0 means off. Drives that don't support 4 output audio channels

may ignore output to channels 2 and 3. Assignment of input channels 2 and 3 to output

channels 0 and 1 may be treated as though the volume control for that channel is 0.

Drives that do not support variable audio control will treat a setting of 0 as off and 1-

0xff as on. Drives that support less than 256 volume settings will do their best to break

up the 256 settings among the settings they can support. E.g. if there are 16 settings

supported, then the first setting will cover 0x01-0x10, the second 0x11-0x20...the sixteenth

0xf1-0xff. Drives that can't play a single channel in both must play only that one channel

and try to suppress the other if possible. Drives that can't swap channels should play the

channel that was moved in its normal channel.

Write Device Control String

DrvBytes  DB   4         ; Control block code

DB   N dup (?) ; Write buffer

This function is provided to allow programs to talk directly to the CD-ROM drive. All

remaining bytes are sent uninterpreted to the drive unit.

The function and content of these bytes are entirely device and device driver dependent. This

function is provided to allow access to device- specific features that are not addressed

under any other portion of the device driver spec.

Close Tray

CloseTray DB   5         ; Control block code

This command is the logical complement to the Eject Disk command. This command will instructs

drives that can do so to close the door or tray.

READ LONG

Command code = 128

ES:BX = ReadL

ReadL     DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to read

DD   ?         ; Starting sector number

DB   ?         ; Data read mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

The request block is different from a normal character device READ to accommodate the larger

size and different characteristics of CD-ROM devices.

The media descriptor byte, which has no meaning for character devices, is now the addressing

mode field. The following values are recognized addressing modes:

0      HSG addressing mode

1      Red Book addressing mode

2-255  Reserved

The default addressing mode is the HSG addressing mode. Long (DWORD) address values are

treated as logical block numbers, as defined by the High Sierra proposal. When Red Book

addressing mode is on, all disk addresses are interpreted as Minute/Second/Frame addresses,

according to the Philips/Sony Red Book standard. Each of these fields is 1 byte. The frame

byte is the least significant byte of the address field, the "second" byte the next most

significant, the minute byte the next, and the most significant byte of the 4-byte field is

unused. These values are represented in binary rather than

in BCD format. For example, if we are referencing the sector addressed by minute 36, second

24, frame 12, the hex long value for this would be 0x0024180C. The relationship between High

Sierra sectors and Red Book frames is described by the equation:

Sector = Minute * 60 * 75 + Second * 75 + Frame - 150

The byte/sector count field becomes the number of sectors to read and the starting sector

number expands from one word to two, which  means we can address up to 4 giga-sectors (over

8 terabytes). The DWORD ptr for requested volume ID is eliminated and MSCDEX.EXE will keep

track of what volume is needed.

MSCDEX.EXE handles buffering requests, but performance may be improved if the device driver

reads ahead or uses a sector caching scheme, given the slow seek times of CD-ROM drives. The

operating system will use the prefetch function when it can to give hints to the driver.

The data read mode field will be one of the following:

0      Cooked mode

1      Raw mode

2-255  Reserved

Cooked mode is the default mode in which the hardware typically handles the EDC/ECC and the

device driver returns 2048 bytes of data per sector read. When raw mode is set, the driver

will return all 2352 bytes of user data, including any EDC/ECC present independent of the

actual sector mode (Mode 2 Form 1 vs. Mode 2 Form 2). User programs will have to consider

this and allow enough room for buffer space when reading in raw mode as each sector returned

will take up 2352 bytes of space. Drives that cannot return all 2352 bytes will return what

they can and leave blank what they cannot. For example, drives that can return all 2336 bytes

except the 16 byte header will leave a space in the first 16 bytes where the header would

go so that the sectors align on 2352 byte boundaries. Drivers should do what they can to

return as much of the user data per sector as possible.

The two interleave parameters are for drivers that support interleaved reading. If the driver

does not support interleaving, these fields are both ignored. If it does, interleave size

is the number of consecutive logical blocks or sectors that are stored sequentially, and the

interleave skip factor is the number of consecutive logical blocks or sectors that separate

portions of the interleaved file.

READ LONG PREFETCH

Command code = 130

ES:BX = ReadLPre

ReadLPre  DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   0         ; Transfer address

DW   ?         ; Number of sectors to read

DD   ?         ; Starting sector number

DB   ?         ; Read mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

This function is similar in form to READ LONG, but control returns immediately to the

requesting process. The device driver is not obligated to read in the requested sectors but

can instead consider the request for these sectors as hints from the operating system that

they are likely to be needed. It is recommended that at a minimum, the driver seek to the

location provided. The attribute in the device status for prefetching is used to distinguish

drivers that do more than just seek to the given location. The requests are low priority and

preemptible by other requests for service. A READ LONG PREFETCH with 0 number of sectors to

read should be treated as an advisory seek, and the driver can, if it is not busy, move the

head to the starting sector. Since prefetching requests are advisory, there will be no

functional difference between a device driver that supports prefetching from one that does

not, except in terms of performance. The transfer address is not applicable for this call

as the driver is not meant to transfer any data into the user address space.

SEEK

Command code = 131

ES:BX = SeekReq

SeekReq   DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   0         ; Transfer address

DW   0         ; Number of sectors to read

DD   ?         ; Starting sector number

Control returns immediately to the caller without blocking and waiting for the seek to be

completed. The number of sectors to be read and the transfer address are ignored. SEEK is

used to relocate the head in order to begin playing audio or video tracks, or in anticipation

of reading in a particular region on the disk. Further requests for disk activity will wait

until the given SEEK is completed. This seek is not advisory and the head must move to the

desired location.

PLAY AUDIO

Command code = 132

ES:BX = PlayReq

PlayReq   DB   13 dup (0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Starting sector number

DD   ?         ; Number of sectors to read

This function will cause the driver to play the selected audio tracks until the requested

sectors have been exhausted or until play is interrupted with a AUDIO STOP request. Control

returns immediately to the caller. Monitoring the busy bit in the status word will determine

if the drive is presently playing audio and also when the play request is completed.

STOP AUDIO

Command code = 133

ES:BX = StopPlayReq

StopPlayReq    DB   13 dup (0)     ; Request header

This function is included to interrupt the drive unit when it is currently in play mode. At

the next stopping point it reaches, the drive will discontinue playing and process the next

request. If the drive is not currently playing or does not support playing, this request is

ignored.

RESUME AUDIO

Command code = 136

ES:BX = ResumeReq

ResumeReq DB   13 dup (0)     ; Request header

This function is used to resume playing audio tracks when play has been interrupted with the

STOP AUDIO command. Its behavior should correspond to the following:

RESET, NEW DISC, PLAY/RESUME COMPLETED

playing = FALSE;

paused  = FALSE;

last_startloc = 0;

last_endloc   = 0;

PLAY_AUDIO(startloc, endloc) {

if (play(startloc, endloc) != SUCCESSFUL) {

return error;

playing = TRUE;

paused = FALSE;

last_startloc = startloc

last_endloc = endloc

return no error;

}

STOP_AUDIO() {

if (playing) {

last_startloc = present q-channel location

playing = FALSE;

paused = TRUE;

if (stop() == SUCCESSFUL)

return no error;

return error;

}

else {

playing = FALSE;

paused = FALSE;

last_startloc = 0;

last_endloc = 0;

return no error;

}

}

RESUME_AUDIO() {

if (paused) {

if (play(last_startloc, last_endloc) != SUCCESSFUL)

return error;

playing = TRUE;

paused = FALSE;

return no error;

}

else

return error;

Note that the playing flag corresponds to the state that should be reported by the busy bit

in the status word in the request header when the drive is in audio play mode. The paused

flag corresponds to the Audio Paused bit and last_startloc and last_endloc correspond to the

starting and ending location in the Audio Status Info IOCTL.

WRITE LONG

Command code = 134

ES:BX = WriteL

WriteL    DB   (dup 13 0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to write

DD   ?         ; Starting sector number

DB   ?         ; Write mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

The device will copy the data at the transfer address to the CD RAM device at the sector

indicated. The media must be writable for this function to work. Data is written sector by

sector, depending on the current write mode and the interleave parameters. The following

values are recognized as valid write modes:

0      Mode 0

1      Mode 1

2      Mode 2 Form 1

3      Mode 2 Form 2

4-255  Reserved

Writing in Mode 1 is the default and must be supported. If the device driver supports the

other modes, then they can be used. If Mode 0 is used, the transfer address is ignored and

all sectors are written with zeroes. If the current write mode is Mode 1 or Mode 2 Form 1,

each sector will consist of 2048 bytes of data located sequentially at the transfer address.

If the write mode is Mode 2 Form 2, the device driver will expect 2336 bytes of data per

sector at the transfer address.

WRITE LONG VERIFY

Command code = 136

ES:BX = WriteLV

WriteLV   DB   (dup 13 0); Request header

DB   ?         ; Addressing mode

DD   ?         ; Transfer address

DW   ?         ; Number of sectors to write

DD   ?         ; Starting sector number

DB   ?         ; Write mode

DB   ?         ; Interleave size

DB   ?         ; Interleave skip factor

This function is identical to WRITE LONG, with the addition that the device driver is

responsible for verifying the data written to the device.

INPUT FLUSH

Command code = 7

ES:BX = FlushI

FlushI         DB   13 dup (0)     ; Request header

Requests that the device driver free all input buffers and clear any pending requests.

OUTPUT FLUSH

Command code = 11

ES:BX = FlushO

FlushO         DB   (dup 13 0)     ; Request header

Requests that the device driver write all unwritten buffers to the disk.

DEVICE OPEN

DEVICE CLOSE

Command code = 13,14

ES:BX = DevOpen, DevClose

DevOpen   DB   13 dup (0)     ; Request header

Used by the device driver to monitor how many different callers are currently using the CD-

ROM device driver. All new device drivers should support these calls even if nothing is done

with the information.