Amiga 'Boot' Sequence

After getting a number of inquiries from fellow Amiga users about what's

wrong with their Amigas when their Caplock key starts flashing, or, why

the screen flashed odd colors, I decided to create this file.

The Amiga goes through a rather elaborate startup sequence which is a

series of diagnostic tests the Amiga goes through during each 'cold' or

'warm' boot. The following is a list of what happens during each and

every 'boot':

1. Clear all old data in RAM

2. Disable DMA and interrupts

3. Clear the screen

4. Check hardware (is the 680x0 working?)

5. Change screen color indicating pass/fail

6. Perform checksum test of all ROMs

7. Change screen color indicating pass/fail

8. Begin system startup

9. Check RAM at $C0000..move SYSBASE there

10. Test CHIP RAM

11. Change screen color indicating pass/fail

12. Check if software is coming in OK

13. Change screen color indicating pass/fail

14. Set up CHIP RAM to receive data

15. Link libraries

16. Check for any additional RAM and link it

17. Turn on DMA and interrupts

18. Start default task

19. Check processor and MMU type

20. Check for an Exception (processor error)

21. If found, do a reset

22. If not, normal 'boot'

As these tests are running, the Capslock key will normally 'flash' once,

and the screen will cycle colors. The chart below used in conjunction

with the above steps indicates both a 'normal' and an 'abnormal' boot.

NORMAL                                 Fault Condition

Sreen color:

Dark Grey                           Red (indicates error in ROM)

Light Grey                          Green (error in CHIP RAM)

White                               Blue (error in a Custom Chip)

Yellow (680x0 found an error)

Capslock Key:

Stays on for several secs            One blink (ROM checksum failed)

Two blinks (RAM test failed)

Three blinks (Whatchdog timer)

Four blinks (A short exists between

2 lines or one of the

special keys)

Please note that the capslock 'blinks' are short in duration (about one

second) and that your screen colors may falsely indicate a problem when

you do a 'warm' boot depending on what state the Amiga was in before the

warm boot. Its best to watch for these diagostic errors upon initial

startup. If your Amiga exhibits any of these errors frequently, you should

take it to an Authorised Service Center for service. The most common

error you will encounter is that you'll see a bright green screen at the

initial boot..this usually indicates that an addon RAM board is not

seated in its socket properly. Try reseating it before you take it all to

your dealer. You may save yourself some money;)

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X