;	Commodore 1541 / OC-118 Disk Drive Memory Map

;	V1.3	18 Jan 1995

;	Covered in this file:

;	   1541 / OC-118 Disk Drive RAM Map

;	   1541 / OC-118 Disk Drive I/O Area

;

The following text was grabbed from "The anatomy of the 1541" by

Lothar Englisch and Norbert Szczepanowski.

Additions from "1541 levyasema".

Layout of the 2Kb RAM area

==========================

;	Commodore 1541 Disk Drive RAM Map

;

; The number in brackets after description is the default value after

; power-up.

; Work queue

0000		Command code for buffer 0

0001		Command code for buffer 1

0002		Command code for buffer 2

0003		Command code for buffer 3

0004		Command code for buffer 4

0005		Command code for buffer 5 (unused)

; ***

0006 - 07	Track and sector for buffer 0

0008 - 09	Track and sector for buffer 1

000a - 0b	Track and sector for buffer 2

000c - 0d	Track and sector for buffer 3

000e - 0f	Track and sector for buffer 4

0010 - 11	Track and sector for buffer 5 (unused)

0012 - 13	Disk ID, drive 0

0014 - 15	Disk ID, drive 1

0016		Header block: ID

0018		Header block: Track

0019		Header block: Sector

001a		Header block: Parity

001b		Not used (Disk Controller work)

001c		Flag for disk change, drive 0 [01]

001d		Flag for disk change, drive 1 [01]

001e		Flag for write protect sense, drive 0 [01]

001f		Flag for write protect sense, drive 1

0020		Drive 0 status (disk and step motor)

;		   bit 7: Disk drive ready (1 = No, 0 = Yes)

;		   bit 6: Read/Write head moving (1 = Yes, 0= No)

;		   bit 5: Motor on (1 = Yes, 0 = No)

;		   bit 4: Switch off motor (1 = Yes, 0 = No)

0021		Drive 1 status (disk and step motor)

0022		Current track for drive 0

0023		Flag for 1541 (0), 1540 (not 0)

;		Not used (track for drive 1)

0024		Scratch pad of GCR conversion

;		Storage for BIN -> GCR conversions

002e		Pointer for temporary storage			WORD

0030		Buffer pointer for disk controller		WORD

;		Pointer: Active buffer

0032		Pointer: *** active track

0033		Pointer: *** active sector

0034		Pointer to last converted byte

0035		Not used (decode error, GCR)

0036		Byte counter for GCR/BIN conversion

0037		Not used (bit counter)

0038		Constant 7, ID mark for start of data block

0039		Constant 8, ID mark for start of block header

003a		Parity for data buffer

003b		Not directly used

003c		Not directly used

003d		Motor flag

;		Disk drive number, on 1541 always 00

003e		Active drive (FF, if not active)

003f		Buffer number for disk controller

;		Previous work place in queue (0 - 5)

0040		Byte counter for GCR conversion

0041		Number of next work in queue (0 - 5)

0042		Destination track (to move R/W head to)		DATA

0043		Number of sectors per track for formatting	DATA

0044		Temp. work area; Scratch pad			DATA

0045		Work code temp. storage

0046		Not used

0047		Data block ID char, default 07.

;	By changing this value, a data block can be written

;	with different ID. The value must be less than 10.

0048		Counter for head movement

;		Timer: R/W head acceleration

0049		Stack pointer temp. storage [39]

004a		Step counter for head transport

;	Values 0 - 127 move the head outside and values over

;	 128 move towards the centre.

004b		? (Temp. storage)				DATA

004c		Last read sector				DATA

004d		Next sector					DATA

004e - 4f	Pointer to buffer for GCR->BIN conversion	WORD

0050		Flag for GCR format (0=BIN, 1=GCR)		DATA

0051		Current track number for formatting [FF]	DATA

0052 - 55	Storage for 4 BIN bytes for GCR coding		DATA

0056 - 5d	Storage for 5 GCR bytes				DATA

005e		Number of steps for head motor accelerating/decl. [04]

005f		accelerating/decl. factor [04]

0062 - 63	Pointer to routine for head movement [FA05]	WORD

0064		Minimum number of steps [C8]

0065 - 66	UI vector [$EB22]				WORD

0067		Flag: NMI

0068		Flag for disk initialisation			DATA

;	Automatic disk initialisation if ID MISMATCH occurred (1 = no, 0 = yes)

0069		Step size for sector division [0A]

;	Sector number interleave for SEQ files, default 10 (0A)

006a		Number of read attempts [5]			DATA

006b - 6c	Pointer: Start of Jump table for U commands [FFEA]	WORD

006d - 6e	Pointer: Start of bitmap [0400]

006f - 70	Pointer to address for M & B commands		WORD

0071		Temp. work area

0072		Temp. work area [FF]

0073		Temp. work area

0074		Temp. work area

0075 - 76	Indirect pointer [0100]

0077		Listener address (Device number + $20) [28]

0078		Talker address (Device number + $40) [48]

0079		Flag: Active listener

007a		Flag: Active talker

007b		?

007c		Flag for ATN from serial bus receiving

;		ATN interrupt

007d		Flag for ATN on serial bus active

007e		Last handled program

007f		Drive number (on 1541 always 00)

0080		Current Track number

0081		Current Sector number

0082		Channel number (Logical index)

0083		Secondary address

0084		Original Secondary address [6F]

0085		Current Data byte [3F]

0086 - 8a	Temp Results

008b - 8e	Work storage for division

;		Result, Multiply and divide

008f - 93	?

0094 - 95	Current buffer pointer				WORD

;		Pointer: Directory buffer (0204)

0096		?

0099 - 9a	Pointer: Next byte in buffer 0 [0300]

009b - 9c	Pointer: Next byte in buffer 1 [0400]

009d - 9e	Pointer: Next byte in buffer 2 [0500]

009f - a0	Pointer: Next byte in buffer 3 [0600]

00a1 - a2	Pointer: Next byte in buffer 4 [0700]

00a3 - a4	Pointer: Next byte in command buffer [0200]

00a5 - a6	Pointer: Next byte in error message buffer [02D6]

00a7 - ad	Table: Non-active channel numbers for each buffer

00ae - b4	Table:

00b5 - ba	Table: Record # low , block # low

00bb - c0	Table: Record # high, block # high

00c1 - c6	Write pointer for REL file

;		Table: next record numbers

00c7 - cc	Table: Record length for REL file

00cd - d2	Table: Side sectors

00d3		?

00d4		Pointer in record for REL file

00d5		Side sector number

00d6		Pointer to data block in side sector

00d7		Pointer to record in REL file

00d8 - dc	Directory sectors

00dd - e1	Index: Directory

00e2 - e6	Default disk drive [00]

00e7 - eb	File type (wild cards, @, closed)

00ec - f1	File type (channel)

00f2 - f7	Channel status

00f8		Flag for EOI

00f9		Current work (Buffer number)

00fa - fe	Last used table

00ff		Flag: FF = drive 0 not ready (No disk), 00 = ready

0100		Flag: FF = drive 1 not ready (No disk), 00 = ready

0101		Format marker, drive 0

;		DOS version (from track 18 sector 0)

0102		Format marker, drive 1

0103		Not used

0104 - ff	Stack area

0146		?

01ba		Buffer for GCR code

0200 - 29 	Buffer for command string

022a		Command code

;022b - 2d	Array: Logical index (LINDX)

;		Array of status bytes for each channel (secondary address)

;		Possible values:

;			FF = inactive

;			81 = opened for writing

;			41 = read/write

;			01 = opened for reading

022b		Logical index, channel 0

022c		Logical index, channel 1

022d		Logical index, channel 2

022e - 43	Last read/written byte for each channel

0244 - 49	Pointer: Last char on channel.

;		Points to the last character read/written in buffer.

024a		Type of active file

024b		String length

024c		Temp. channel number (secondary address)

024d		Current work with drive number

024e		Work area to find the best sector

024f - 50	Buffer allocated

0251		Flag: BAM changed, drive 0

0252		Flag: BAM changed, drive 1

0253		Flag for directory entry found (FF = Not found, 00 = Found)	DATA

0254		Flag for directory output

0255		Flag: Waiting for command

0256		?

0257		Last used buffer

0258		Record length

0259		Track of side sector

025a		Sector of side sector

025b - 5f	Last work (buffers)

0260 - 65	Directory sector (buffers)

0266 - 6b	File's index in directory (buffers)

026c		Counter for LED flash

;		Error Message

026d		Error LED

026e		Drive for last program

026f		sector for last program

0270		Write LINDX

0271		Read LINDX

0272 - 73	Number of blocks (temp)

0274		Length of input line

0275		Char to interpret

0276		Index: End of filename in command buffer

0277		?

0278		Number of file names

0279		?

027a - 7f	Pointer table: Filenames

0280 - 84	Track of a file

0285 - 89	Sector of a file

028a		Flag: wild cards

028c		Number of drive(s ?) to look for

028d		Flag: Looking for drive

028e		drive with last write/open error, used as default drive

028f		Flag: Found in directory

0290		Directory sector

0291		Sector for first available file

0292		Index (in directory) for first available file

0293		For the last directory entry 0

0294		Current buffer-index

0295		Counter: Files

0296		Flag: Name matching wild cards

0297		Active operating mode (R or W)

0298		Flag: Work return

0299		Pointer: Re-read error

029a		Total tracks

029b - 9c	Pointer: BAM last update

029d - a0	BAM: image sector (drives 0 and 1)

02a1 - b0	BAM image

02b1 - d4	Buffer for directory output

02d5 - f8	Buffer for error message

02f9		Flag: Don't write BAM

;		This flag is reset to zero before and after each command

02fa - fb	Number of free blocks, low byte for drives 0 ja 1

02fc - fd	Number of free blocks, hi byte for drives 0 ja 1

02fe - ff	Stepper motor sequence

0300		Buffer 0

0400		Buffer 1

0500		Buffer 2

0600		Buffer 3

0621 - 22	Formatting counter

0700		Buffer 4

07ff		End of RAM

Layout of the I/O Ports (VIA 6522)

==================================

VIA 1: 6522, port for serial bus

-------------------------------

$1800	PB, port B

$1801	PA, port A

$1802	CB, data direction port B

$1803	CA, data direction port A

$1805	Timer

PB 7, CB2:	ATN IN

PB 6,5:	Device address preset switches

PB 4:	ATN acknowledge OUT

PB 3:	CLOCK OUT

PB 2:	CLOCK IN

PB 1:	DATA OUT

PB 0:	DATA IN

VIA 2: 6522, port for motor and read/write head control

------------------------------------------------------

$1C00	PB, control port B

$1C01	PA, port A (data to and from read/write head)

$1C02	CB, data direction port B

$1C03	CA, data direction port A

PB 7:	SYNC IN

PB 6,5:	Bit rate D1 and D0

PB 4:	WPS - Write Protect Sense (1 = On)

PB 3:	ACT - LED on drive

PB 2:	MTR - drive motor

PB 0,1:	step motor for head movement

CA 1:	Byte ready

CA 2:	SOE - Set Overflow Enable for 6502

CA 3:	read/write

Disk Controller Routines

========================

This table contains the addresses and the assigned buffers:

JOB	TRACK  SECTOR	 BUFFER

00	  06	  07	  0300-03FF

01	  08	  09	  0400-04FF

02	  0A	  0B	  0500-05FF

03	  0C	  0D	  0600-06FF

04	  0E	  0F	  0700-07FF

05	  10	  11	  -no ram-

If a disk controller routine is to be performed in the interrupt, one

writes the job code and the track and sector numbers of the block to be

processed (if necessary) in the appropriate memory locations. Now the

job code can be passed. This job code has a value greater than 127 (the

7th bit is set). In order to wait until the job is finished, only the

seventh bit need be tested. If this bit is cleared, the job is completed.

The disk controller can perform the following jobs:

JOB   NAME	DESCRIPTION

80    READ	Read sector

90    WRITE	Write sector

A0    VERIFY	Verify sector

B0    SEEK	Find sector

C0    BUMP	Bump, Find track 1

D0    JUMP	Execute program in buffer

E0    EXECUTE	Execute program, first switch drive on and find track

The verify job (A0) is performed automatically after writing a blcok.

If the job ends in an error then the memory location with job command

code is replaced with and error code. Here is a list of the error numbers:

CODE	MEANING				DOS ERROR MESSAGE

01	Everything OK			00, OK

02	Header block not found		20, READ ERROR

03	SYNC not found			21, READ ERROR

04	Data block not found		22, READ ERROR

05	Checksum error in data block	23, READ ERROR

07	Verify error			25, WRITE ERROR

08	Disk write protected		26, WRITE PROTECT ON

09	Checksum error in header block	27, READ ERROR

0B	Id mismatch			29, DISK ID MISMATCH

0F	Disk not inserted		74, DRIVE NOT READY

Error codes 06 and 0A do not occur on the 1541.