From: David Carter <davidc@pyr.gatech.edu>

[A condensed version of this article, along with the pin connections to use a

2000 keyboard on a 1000 and vice-versa, would probably be a good thing to add

to the monthly comp.sys.amiga.tech newuser posting.  Anyone else agree?  Who

do I tell?]

The usual disclaimers about voiding warranties and "It's not my fault if you

torch your Amy" apply here.  Also, Commodore may change their design at any

time, making this information inaccurate for new machines.

The Amiga 500 and 2000 keyboards are nearly identical!  If you don't believe

it, open up one of each and compare them side-by-side as I have.  They are

the same printed circuit board.  There =are= two different =styles= of

keyboards, one with a membrane array, and the other with individual

keyswitches.  But the circuits themselves are identical.  Both 2000s and 500s

have come with both types of keyboards.  Quick way to tell the difference:

you can see a metal rivet-like thing between the Del and Help keys on the

keyswitch style.  I can also tell by the feel (the membrane style is

"mushier").

I have taken a 500 keyboard and used it on a 2000 (with 'control-Amiga-Amiga'

reset working perfectly) and a 2000 keyboard on a 500 (my current setup, again

with reset working perfectly).

There are two small differences in the keyboards: The 500's has one extra

transistor, type 2N3904 at position labeled Q1.  The 2000's has empty holes

at this position.  If you want to use a 2000 keyboard on a 500 and want

reset to work, you must add this transistor.  Doing so sends the reset

signal to pin 3 on the DIN plug (yes, there is an extra conductor in the

coiled cord for this pin.  It is a NC inside the 2000 computer).  To use

a 500 keyboard on a 2000, you must remove the transistor.

The other difference is that the 2000 keyboard has empty holes where the

"power" and "drive" LEDs would go on the 500.  Unfortunately, there are no

wires for these in the coiled cord.  They are still a good thing to have,

so you should think about getting some extra LEDs and mounting them somewhere

in the A500's case.

Here are the pinouts for the connectors:

The 2000's Keyboard Plug is a male 5 pin DIN connector.  The diagram here is

looking at the solder side (back) of the female part -- what you'll have to

buy to plug the keyboard into.  On mine, the numbers are written on the

plastic on the back, next to each pin.

2

5     4       Female 5-pin DIN socket,

3       1      looking at rear (solder pins)

6

Pin  Signal  Description

---  ------  -----------

1   KBCLK   Keyboard Clock

2   KBDATA  Keyboard Data

3   NC      Not Connected (becomes Reset after adding transistor Q1)

4   GND     ground

5   +5v     +5 volt power

6   SHIELD  shield

The A500's connector is an 8 pin "molex type" connector (single row, 0.1"

spacing).  Looking at it from the front of the A500, pin 1 is the leftmost pin.

KBCLK  KBDATA  RESET  +5v  NC  GND  Power  Drive

1       2      3     4    5   6     7      8

NOTE: The A500 schematics in the "Introduction To The 500" and the "500/2000

Technical Reference" manuals INCORRECTLY show pin 1 as KBDATA and 2 as KBDCLK.

"Power" and "Drive" go high (+5v) to light the two LEDs.

So the cable you'll need to make (which I've been using in my A500 without

any problems for nearly a year) looks like:

molex plug   DIN socket to

into A500     2000 keyboard     Signal

1           1               KBCLK

2           2               KBDATA

3           3               RESET

4           5               +5v   Careful!!

5           -               NC on A500 motherboard

6           4               gnd   Careful!!

7           -               Optional: "Power" LED

8           -                         "Drive" LED

-           6               Shield ground from keyboard;

connect to RF shield of A500

The other sides of the Power and Drive LEDs go through 220 ohm resistors

to ground.  The 500's LEDs are actually dual (two LEDs in series in a single

plastic lens).  So if you use single LEDs, you should probably use slightly

higher resistors, like 680 ohm.

One last note: Since the A500 uses a hard reset line (another long story),

when you unplug or plug the 2000 keyboard to your A500, you will reset the

computer (unlike on the 2000 computer).

David Carter            davidc@pyr.gatech.edu