Article 13320 of comp.sys.cbm:

Path: oswego!ub!dsinc!spool.mu.edu!howland.reston.ans.net!zaphod.mps.ohio-state.edu!pacific.mps.ohio-state.edu!linac!att!cbnewsk!cbnewsj!bjh

From: bjh@cbnewsj.cb.att.com (bernard.j.hudson)

Newsgroups: comp.sys.cbm

Subject: X1541.EXE Cable & Docs.

Summary: Connect 1541 to PC

Keywords: 1541 C64 IBM PC

Message-ID: <1993Jan13.203920.13485@cbnewsj.cb.att.com>

Date: 13 Jan 93 20:39:20 GMT

References: <1993Jan12.020437.9573@oswego.Oswego.EDU>

Sender: bjh@linc.att.com (BJ Hudson)

Followup-To: comp.sys.cbm

Organization: AT&T

Lines: 99

Hi,

This program is great !  It has been needed for years.

Here is the pinout spec and some brief English docs.      ....BJ 1/93

bjh@linc.att.com

>Good work ! ciao, Leopoldo

--------------------------------------------------------

X1541.EXE

by  GHIELMET@eldi.epfl.ch (Leopoldo Ghielmetti)

The X1541 program requires an adapter cable to connect your PC parallel

printer port (LPT1/LPT2/LPT3) and a commodore disk drive (1541/1581).

*(The Female port pinouts are shown below. The normal cable is Male-Male.)

CBM IEEE serial port *Female*:

IBM Lpt port *Female*:                ___ ___

,-----,                          .-   -   -.

Strobe--+-1*  '---,                    .-           -.  _____

|      *14+--AutoFeed    Data--+--*5     1*--+--SrqIn  _____

(Data 0)| 2*-,    |                   (       6*------)---------Reset

|    |_*15|(Error)      Clock--+--*4     2*--+---Ground

| 3*      |  ____              ._    3*     _.

|      *16+--Init                .____|____.

| 4*      |                       Atn-|

|      *17+--SelectIn  -------------------------------------------

| 5*      |            | C64signal |  IBMsignal       Lptport +2 |

| 6*   *18+-+-Ground   |  2 Ground | 18..25 Ground               |

|         | |          |  3 Atn    |      1 Strobe      bit0     |

| 7*   *19+-|          |  4 Clock  |     14 AutoFeed    bit1     |

vvvvvvvvvvv |          |  5 Data   |     17 SelectIn    bit3     |

|      *25+-+          |  6 Reset  |     16 Init        bit2     |

|13*  ,---'            -------------------------------------------

'-----'

X1541.EXE  IBM <--> C64 file reader

type "x1541"   for the interface scheme (see above)

type "x1541 ?" for the options list

options are : "/b" force monochrome mode

"/c" force color mode

"/LPTn" force I/O from LPT port n (n in range 1..3)

<ALT>     Activate Menu or to dismiss menu and go to file list

Also used to dismiss status boxes (after COPY)

<ENTER>   Highlight memu or Select menu item

<DOWN>    Highlight menu

<ESC> or  Go back; Highlight previous menu

<LEFT>or

<UP>      ( Sub-menu stays screen, but is no longer highlighted)

<TAB>     Move between 1541 and IBM filelists

Also used move to "OK" in COLOR menu

Cursor keys    Move through menus

<PAGE UP> &    and Move through filelists

<PAGE DN>

<SPACE>   Select files in filelist for copying

1541-COMMAND Note:     command letter (e.g. N0:Test Format,tt)

Must be capital letter --^

**** Brief command sequence for file copy ********

1. OPEN 1541   <ENTER> to select and highlight Drive# sub-menu

<ENTER> to select drive number (8-11)

2. OPEN IBM

3. <TAB> to source filelist

4. <SPACE> selects file(s)

5. <ALT> for menu

6. COPY

7. <ALT> to dismiss status box after copy is complete

DE NADA

Well I forgot to mention it, to make a file with another extension (seq,

usr, rel (not yet supported)) you should give the correct extension to the

IBM file Eg. *.seq will be transferred as sequential file (seq)

In normal condition 1581 and 1571 have the same mechanism as 1541 but for

lack of documentation I did not implement the fast serial mode feature.

ciao, Leopoldo

/**************************************\

* Ghielmetti Leopoldo                  *

\**************************************/*****\

*   e-mail : ghielmet@eldi.epfl.ch   *

\************************************/

Posting and docs by bjh@linc.att.com

Downloaded From P-80 International Information Systems 304-744-2253