386ID	 -- Return The 386/486 Component Identifier

The purpose of this program is to display the component and revision

level of your 386 or later CPU.

As you may already be aware, the 386 and 486 have a built-in mechanism

which is the definitive marker of the chip's component and revision

level.	For example, an Intel 386 CPU identifier of 0308 corresponds

to the D1-step which is an external name applied to revision levels.

Different revision levels represent different internal workings,

usually when improvements are made.  In contrast, the 32-by-32

multiply instruction error in the 386 was not corrected by a new

revision level, only a more careful screening of existing chips.

Chips with this correction are identified by a double sigma on the top

of the chip.

According to the Pentium Processor User's Manual, the component and

revision level fields are split into three parts:  a stepping ID (bits

0-3), a model field (bits 4-7), and a family field (bits 8-11).

Although marked as reserved, bits 12-15 are used in other CPUs as a

family member field.  For example, the 386 DX D1-step has a family

member field value of 0h, a family field value of 3h, a model field

value of 0h, and a stepping ID of 08h, yielding a value in DX of

0308h.	Furthermore on 486 CPUs, a model type of 0 is used for 25 and

33 MHz parts (040xh), 1 for 50 MHz parts (041xh), and 2 for 486/487 SX

parts (042xh).

Alas, until the Pentium Processor came along, a CPU's identifier is

difficult to obtain.  Like a mole unaccustomed to the daylight, it

surfaces briefly in the DX register when the system is reset and then

disappears.  User code has but a slim chance to latch onto this

slippery value.

The Plan of Attack

------------------

The accompanying program attempts to obtain this value through one of

several techniques.

The first technique considered is to see if this CPU supports the

CPUID instruction, and if so, use that to get the component ID.

The second technique considered is to invoke the relatively unknown

BIOS call to return the information.  If this call (AX=0C910h, INT

15h) is supported by the BIOS, it returns the component ID in CX.

The third technique considered is to shutdown the system in the

usual way, and, upon regaining control, save the value in DX.  My

experience is that this approach is only infrequently successful.  The

problem is that by the time our program gains control to save DX, that

register might have been clobbered by the BIOS code.

The fourth technique considered provides a means by which our program

can gain control after a system reset, but BEFORE the BIOS does!  To

understand how this can happen, we must explain some of the inner

workings of the system along with some digressions.

The First Few Microseconds

--------------------------

After system reset, the initial value for CS:IP is F000:FFF0.  You

might expect this means that the first instruction fetch is from the

physical address 000FFFF0 = (CS shl 4) + IP.  However, at system reset

time (and only then) the CPU artificially forces address lines A20

through A31 high by initializing the base address in the invisible

descriptor cache for CS to FFFF0000 as opposed to the expected

000F0000 (CS shl 4).

Descriptor Caches

-----------------

A descriptor cache is an optimization technique introduced in the

80286 and continued into later processors.  It is different from both

a disk cache and memory cache, although the intent in each case is the

same:  to expend a small amount of effort in the beginning to achieve

a benefit in the future many times over.

In this case, the cache is a separate location on the CPU to store a

summary of frequently accessed data.  In fact, it is more than just an

optimization technique as it is relied upon by any program which

switches from Real Mode (RM) to Protected Mode (PM).

The cache is used by the CPU as follows:  whenever a program changes a

segment register in RM or virtual 8086 mode (VM), or a selector in PM,

the CPU calculates the actual linear address corresponding to the

value and saves it on the chip in the cache for that register, along

with other information about that register such as the access rights

word.  Each segment/selector register as well as other frequently

accessed data on the chip have a separate cache.  The terms segment

and selector registers refer to the same items such as CS, DS, ES, FS,

GS, and SS; the names simply reflect the different mode in which they

are used -- segment for RM and VM, selector for PM.  In RM or VM the

base address in the descriptor cache is set to the actual value times

16.  In PM, the CPU looks up the selector in the appropriate (global

or local) descriptor table and uses the base address in that table to

set the base address in the descriptor cache.  Because this table

lookup is somewhat more time-consuming in PM, descriptor caches

actually have much greater value in that mode.	These caches are

called invisible because although the CPU has full access to them,

they can't be set or read directly by an application program.

The first far jump or call resets the base address in the descriptor

cache for CS at which time address lines A20 through A31 are no longer

artifically held high.

The First Fetch

---------------

Back to fetching the first instruction.  Because the descriptor cache

for CS is FFFF0000, the first instruction fetch comes from the

physical address FFFFFFF0 (using an IP of FFF0), that is 16 bytes

below 4GB.  As we want this fetch to come from our system BIOS,

vendors must decode the BIOS ROM at this address; typically they

actually decode it at multiple locations including FFFF0000 (required)

as well as 000F0000 (F000:0).  That is, the same physical ROM can be

accessed from different physical addresses.

Compatibility with 8086/8088 Programs

-------------------------------------

Now for a bit of history on how the IBM AT and later systems maintain

a certain degree of compatibility with the 8086/8088 processors.  When

these early CPUs encountered a segment and offset which together

exceed the limit of its 20-bit address space (such as FFFF:0010 =

00100000), they automatically wrap at one megabyte back down to zero.

Thus FFFF:0010 is identical to 0:0, or more accurately, memory

references to the former location are satisfied from the latter

location.  When the IBM AT was designed to use a 80286 CPU, its

architects faced a problem.  The newer processor was oriented more

toward PM operation.  Consequently, it didn't automatically wrap

addresses at 1MB, as that would defeat the purpose of having a

continuous stream of RAM at 1MB and beyond.  Nonetheless, the AT's

designers decided to emulate this odd behavior because they understood

well the need for compatibility.  That is, there were (and still are)

many programs which rely upon the 1MB wrap as it is called.

Because the 8086/8088 processors have a 20-bit address space, they

need address bits A0 to A19 only.  The maximum address which can be

generated in RM (FFFF:FFFF) has 21 significant bits, that is, of the

bits beyond A19 only A20 is non-zero.  The AT's designers realized

that they could emulate the 1MB wrap by artificially forcing A20 to

zero.  Thus the mechanism used to emulate this behavior and obtain

compatibility is called the A20 gate.  Originally, it was controlled

by a single bit in the output port byte (a kitchen sink catchall) of

the 8042 PPI (Programmable Peripheral Interface) chip on the system

board.	The two values of this bit correspond to the two states of the

A20 gate.  The value zero means that A20 is forced to zero regardless

of what address the CPU generates; a value of one means that A20 is

untouched, that is, controlled by the CPU to have whatever value the

CPU calculates.  In this way, with A20 disabled (forced to zero)

although the CPU may generate an address of 00100000 when presented

with FFFF:0010, the actual address the bus sees is 00000000.

As a side note, the A20 gate has caused fits for several system

designers.  When memory cache controllers were introduced into PC

systems, some designers forgot about the A20 gate.  That is, they

designed their systems such that (from a logical perspective) the

sequence of control was from the CPU to the memory cache to the A20

gate circuitry to the bus.  A problem occurs in the following sequence

with (say) A20 enabled:  read from FFFF:0010 (thus putting the value

at physical address 1MB into the cache), disable the A20 gate, and

read again from FFFF:0010.  The program expects to get the second

value from physical address zero (because A20 is now disabled), but if

the memory cache controller doesn't know that the A20 gate has changed

it will provide the CPU with the (incorrect) value from its memory

cache for physical address 1MB.

Moreover, consider what happens with the 486 CPU which has an onboard

memory cache.  Without really having any choice, Intel decided to

immortalize this behavior in silicon by defining an input pin (A20M#)

on the 486 to allow external circuitry to control the state of the A20

address line which the cache sees so that it would work correctly.

One more digression about the A20 gate:  why is it so important to

emulate the 1MB wrap?  Isn't this a problem that could be solved

simply by more careful programming?  Yes, but it's too late.  Quite

likely, your hard disk is infested with numerous programs which under

certain circumstances will fail if the 1MB wrap weren't emulated.  The

major perpetrator of this feature is the EXEPACK program as well as

the corresponding option to the linker.  In particular, the code which

EXEPACK prefixes onto your packed .EXE files is dependent upon the 1MB

wrap.  In particular, if such a program is loaded with its code

segment at 0FF0h or lower (essentially anywhere within the first 64KB)

and the A20 gate is enabled (thus not emulating the 1MB wrap), the

unpacking code fails with the message "Packed file is corrupt!".

Moreover, MS-DOS 5.0 and later versions go to extra effort when

loading a program to patch a packed .EXE file to avoid the 1MB wrap

dependency.  This was down in MS-DOS 5.0 and not earlier versions

because that was the first version which could free up enough low DOS

memory (via DOS=HIGH) to allow a program to run below 64 KB and thus

exacerbate the packed .EXE file bug.

The First Fetch Continued

-------------------------

Now back to the A20 gate and fetching the first instruction.  When the

system first powers on, the A20 gate is enabled (and the 1MB wrap is

not emulated).	What happens if subsequently the system is reset after

the A20 gate is disabled?  The CPU generates an instruction fetch from

FFFFFFF0 but the A20 gate circuitry artifically pulls down address

line A20.  Thus the bus fetches data from address FFEFFFF0 (that is,

4GB - 1MB - 16).  Because this address typically corresponds not to

memory but thin air, the data fetched is usually FF FF FF ...  which

when interpreted by the CPU is an invalid opcode.  Hence, INT 06h (the

Invalid Opcode interrupt) is signalled BEFORE the first instruction in

the system BIOS is executed.  Thus, by placing our own INT 06h handler

into the RM interrupt descriptor table, we can gain control

immediately upon system reset and save the component ID before the

BIOS can use that register for some other purpose.  After saving away

the component ID, our INT 06h handler enables A20 and resets the

system again at which point the first instruction is fetched from

FFFFFFF0 as expected and the system BIOS begins execution.

Note that this technique can fail.  In particular, some systems do not

have what is called a fully-terminated bus.  That is, some systems

return arbitrary data from addresses with which there is no

corresponding memory (such as FFEFFFF0).  As the technique described

above is dependent upon reading FF FF from such addresses, it can

fail.  To detect this failure, we read two bytes from FFEFFFF0 and

ensure that they are FF FF.  If not, we don't attempt a system

shutdown with A20 disabled.

A combination of all these techniques is used in 386ID.COM.  That is,

first we try the CPOUD instruction and then the BIOS call.  Then we

install an INT 06h handler, and shutdown the system.  The value of DX

from either or both of the INT 06h handler or the system shutdown is

captured and displayed.  This maximizes our chance of obtaining the

elusive CPU identifier.  One or more of the following messages is

displayed at the end:

1.  The component ID is ____ from the CPUID instruction.

2.  The component ID is ____ from the BIOS.

3.  The component ID is ____ from the INT 06h handler (____:____).

4.  The component ID is ____ from the type 0A shutdown.

The address in the third message is that of the invalid opcode and

should always be F000:FFF0, thus re-affirming the validity of the

technique used.

For reference, here's a partial list of component and revision levels

for several 386 and 486 CPUs.  Note that, for 386 CPUs the family

member field is not consecutive:  0h for the DX, 2h for the SX, and 4h

for the SL.

Comp	  Type	 Step

---------------------

0303	 386 DX   B1

0305	 386 DX   D0

0308	 386 DX   D1/D2/E1

2304	 386 SX   A0

2305	 386 SX   D0

2308	 386 SX   D1

43??	 386 SL   ??

0400	 486 DX   A1

0401	 486 DX   Bx

0402	 486 DX   C0

0404	 486 DX   D0

0410	 486 DX   cAx

0411	 486 DX   cBx

0420	 486 SX   A0

0433	 486 DX2-66

If you find component IDs other than the ones above, please contact

the author.

Programming Notes

-----------------

This program does a lot, so don't be surprised if you don't understand

everything at first.  Enabling and disabling A20 is not a simple task,

complicated both by the several different ways system vendors have

defined as well as the difficulty of dealing with a mostly

undocumented interface (the keyboard controller, typically an 8042,

and the keyboard chip, typically a 6805).  For simplicity, 386ID

handles ISA/EISA and Micro Channel systems only.  There are many ways

of gating A20, not all of which can be represented here.  If you have

a problem running this program, try installing HIMEM.SYS (without

EMM386) and try again.

This program is driven by the need to shutdown the system, so we'll

describe its operation from that viewpoint.  The various shutdowns and

the trailing code are summarized as follows:

1.  Type 0Ah (8259 not re-initialized):  Preparation up to this point

includes ensuring we're on a 386 or later CPU, saving control

registers to be restored later, installing interrupt handlers,

splitting case depending upon the system type (ISA/EISA and Micro

Channel), degating A20 (unless the bus is not fully terminated),

disabling the interrupt mask register in the 8259, disabling the

watchdog timer (if present), saving the original value of the shutdown

byte, setting the shutdown return address, disabling NMI, and finally

shutting down the processor.

This shutdown has one purpose:	Save DX without executing 8259

initialization code in case that code would clobber DX.

2.  Type 05h (8259 re-initialized):  In preparation for this shutdown,

we enable A20 to avoid a second INT 06h interrupt.  If we didn't

signal INT 06h on the first try, there's no reason to expect this

shutdown will have any better luck.

3.  Second type 05h:  This shutdown is generated from within the

Invalid Opcode handler after saving DX and re-enabling A20.

4.  After the last shutdown, we disable A20 (presumably to its

original state).  In any case, we enable NMI, restore the original

value of the shutdown byte, as well as the original state of the 8259,

and the INT 06h handler, and then display messages based upon what

happened.

To avoid cluttering up the main assembler file, most equates, records

and structures have been relegated to include files of which there

are several.

One final note:  although you might expect to learn much more about

this program by running it through your favorite debugger,

unfortunately that's not practical because of the several system

shutdowns it requires.	A 386 hardware debugger such as Periscope

Model IV is invaluable when debugging programs such as this.

Bob Smith -- bobs@digex.net

Finally, a word of acknowledgement.  Thanks go to Jeff Bozbin formerly

of Phoenix Technologies for explaining the role A20 plays in this

sequence of events.

For completeness, here are the possible error messages:

��> The CPU is not an 80386 or later.

(Self-explanatory.)

��> Unable to run from Virtual 8086 Mode.

(Because we need to shutdown the system, we need to start from

Real Mode.)

��> Unable to disable the A20 line.

(There's a problem with the A20 gate that is hardware-related.)

��> This system does not have a fully-terminated bus,

so we did not attempt system shutdown.

(We tried reading data from 4GB - 1MB - 16 (FFEFFFF0) and either

the call failed or the data returned wasn't FF FF).