The program 87ERROR.COM is a simple 8087 error interrupt handler for

the IBM Personal Computer. To install the error handler just run 87ERROR.COM

from DOS (ver. 1.1, 2.XX). When an error interrupt is generated by the 8087

a letter in reverse video will appear in the upper right hand corner of the

monochrome display. The letter will indicate the general type of error which

caused the 8087 to generate an interrupt as below in the table:

D --------- Denormalized operand

Z --------- Divide by zero

O --------- Overflow

U --------- Underflow

P --------- Precision error

The interrupt pin on the 8087 is connected with the parity checking

logic on the mother board. Because of this fact the program needs to jump to

the ROM BIOS just in case the NMI (Non Maskable Interrupt) was generated by a

parity error and not by the 8087. The address of the jump at the end of the

interrupt handler "JMP FAR PTR ROMADR" may need to be changed depending on

what version of the IBM ROM BIOS you have.

The correct address can be found at 0H:8H to 0H:0BH for the jump to the

IBM ROM BIOS. The first two memory locations contain the offset and the next

two memory locations contain the segment address. Change the address on the

line "ROM	SEGMENT AT 0F000H" to the segment address just found. Then

change the address on the line "	ORG	0E2C3H" to the offset address

just found. You can now generate a new 87ERROR.COM file from the new

87ERROR.ASM source file.

Leland C. Scott

18140 Locherbie

Birmingham, Michigan 48009

(313) 642-9783