QUIcK power UP routines

Programs and documentation copyright 1983 by Herb Shear.

QUIKUP.COM is a short assembly language routine which meets the following

objectives:

1.  Permit utilization a RAM in excess of the 544KB limit imposed by

the system board switches.

2.  Permit fast power ups by allowing the system board switches to be

set to a low value.

3.  Be compatible with AUTOEXEC.BAT for no hands booting.

4.  No untested or potential parity error bytes.

To enjoy its features, employ the following steps:

1.  Copy QUIKUP.COM to your DOS SYSTEM (boot) disk.

2.  Make QUIKUP the first entry in the AUTOEXEC.BAT file.

3.  Set the system board switches for a total memory of 64KB.  You

may later have to increase this in order to run a certain class

of software that deserves no further mention.

4.  Power up.

QUIKUPQD.COM is basically identical, with a slight modification so that it

will work in cooperation with Quadram's QM.EXE, QDXT.EXE and QMXT.EXE RAM

Drive programs (which just happen to be on this very same SVCS Club Disk

No. 43).  [EDITOR'S NOTE:  Not so on this SFpcUG Disk #106, however.]  Note

that these three ramdrives do not check for a Quadram board (mighty nice of

them).  Further note that the `XT' only implies DOS 2.0's 9 sector tracks,

and should not be construed as a restriction to XT hardware.

In the second step above make QUIKUPQD the first AUTOEXEC.BAT file

entry followed by the desired ramdrive.  QUICKUPQD and Q* n/A will

work together to resolve the endless reboot dilemma.