TABLE OF CONTENTS

Description ..................2

Syntax .......................2

Labels .....................3

Opcodes ....................3

Operands ...................4

Pseudo ops .................5

Error Messages ................6

Source code ...................6

Disclaimer ....................7

Revisions......................8

Op code table .................9

1

ASSEMBLE.COM

DESCRIPTION

This is a two pass assembler written in Turbo Pascal. It

was written because I was a new owner of an IBM PCjr with little

software and was not aware of any public domain assemblers that

had reasonable performance. Additionally I had recently purchased

Turbo Pascal from Borland International and wanted a project to

help me learn Pascal.

This is a simple assembler for Intel 8088/8086 instruction

set. It closely follows the syntax of the instruction set

described in THE 8806 BOOK by Russel Rector and George Alexy. It

is also patterned after CHASM.BAS version 1.9 written in basic by

David Whitman of Whitman Software (there have been numerious

changes to avoid copywrite violations). It is not a macro

assembler and therefore recognizes only a few pseudo op codes.

Listed later in this documentation is a complete list of op codes

and pseudo ops recognized by ASSEMBLE.COM.

The input requirements to ASSEMBLE.COM are ordinary DOS

files which can be created with most text editors. The output

will be a listing sent to the screen, printer or a disk file and

a .COM file. This assembler was designed to create executable

programs and assembly code for use in BASIC or Turbo Pascal

programs. It will not generate code that can be linked to other

other programs. See your BASIC or TURBO instruction manuals for

including executable code in your programs. Since this assembler

is intended for small projects you will probably find bypassing

the Link and conversion from .EXE to .COM files is a convenience.

If you intend to write large software projects, I recommend you

get a macro assembler such as IBM's or CHASM version 4.0 (also

written in Turbo Pascal). Although intended for small projects,

since both the input file and output file(s) are on the disc the

only limitations to the size of your program is the disk space,

number of labels you use and your endurance. Labels and their

memory address are stored on the first pass in a data array. This

array is limited to 400 lables. My goal is to keep this program

under 35K bytes so it can be run on small systems. If you have

the memory this program can easily be recompiled with a larger

array defined. See the notice at the end for the source code.

SYNTAX

Each line of source code begins with a label or blank space

followed by the op code (8088 instruction or pseudo op) and then

the operand(s) (if required by the opcode). The source code may be

followed with a semicolon (;) and any comments for that line.

Optionally a line may be only comments if it starts with a semi

colon. A blank space must precede and follow the op code field.

For readability I recommend one or more spaces between the label

and the op code and between the op code and the operand. A comma

must be used to separate operands. The requirement for a comma

between operands differs from CHASM and should be noted when

converting programs.

2

Example

1stLabel mov  ax,90H    ;format example

LABLES

Although labels may be longer only the first 12 letters of

the label are stored for future reference therefore any labels

with the first twelve characters the same will cause a duplicate

label error. Also since the line parseing routine converts all

source code not in single quotes to upper case prior to decodeing

the line, you can not use upper and lower case to distinguish

labels. For example LongLabelxxx1 and longlabelxxx2 are both

stored as LONGLABLEXXX therefore would cause an duplicate label error.

If you are going to use numbers to distinguish labels I suggest

you use them at the beginning of a label such as 2LongLabelXXX.

OP CODES

All of the op codes specified in THE 8086 BOOK are supported

however in order to resolve some ambiguous op codes the syntax

was modified. The first ambiguous opcode is JMP which can be

either a 8 or 16 bit displacement jump. Eight bit displacement

jumps are resolved by specifying JMPS for short jump. Jumps

useing mem/reg (indirect) addressing for their destination must

specify Near or Far to indicate a jump within the current CS or

an intersegment jump. These jumps are coded as JMPN or JMPF. This

same logic is used for CALLN and CALLF when using the mem/reg

addressing mode. The other major area of ambiguity comes from

using op codes that do not specify a register as either the

destination or source. This assembler requires you to append the

op code with a B or W to distinguish between bytes and words ie.

MOVSB for move a string of bytes or MOVW [bx],8 to load 8 into

the word address pointed to by BX.

Normaly all data moves are assumed to be relative to the DS

(data segment) register. This default can be over ridden one

instruction at a time by using the SEG op code in the line prior

to the desired over ride.

Example

SEG   ES

MOV   AX,[BX]

This moves a word into the accumulator from the address in the

extra segment offset by the bx register. This is a little used

function since ASSEMBLE assumes all of the segment registers are

set to the same location as is required for the start of a .COM

program. Access of system resources should be done with BIO or

DOS calls when possible rather than going directly to a hardware

memory location outside your program.

The opcode table lists the available opcodes and pseudo ops

and the various addressing modes associated with each. Please

note that the mem/reg addressing mode includes several sub modes

such as base relative (using BX as an offset), stack relative

(using BP as an offset), and indexed (useing the SI or DI). See

the 8086 BOOK for an explanation of each mode.

3

OPERANDS

The operands describe to the assembler the destination and

source of the data to be operated on. The 8088 uses a number of

addressing modes to determine where that data is and should go.

You will discover by looking at the op code table, not all modes

can be used with an individual op code. Addressing modes are :

Accumulator - data is transferred to/from the accumulator.

Displacement - the displacement value is added to the present IP.

Immediate - data is assembled into the instruction.

Memory/Reg - data is transferred to/from address pointed to

by [mem] or [reg].

Register - data is transferred to/from the register.

Lables can be used in the Immediate, Displacement and Memory

addressing modes. This assembler follows the Intel convention of

treating a lable operand as a refference to the value in a

memory location unless it was defined by an EQU. If a value is

added to that lable it is then treated as a refference to the

location address. To use an offset to the lable to obtain a value

in a memory location the lable and offset must be contained in

brackets just as a numeric value would.

Operand        Meaning

Lable         refference value in the memory location

Offset Lable  refference the memory location

Lable+5       refference the memory location at lable+5

This is the same as Offset Lable+5.

$-Lable       refference to a memory location or a numeric

value i.e. when defineing a buffer length.

[Lable+5]     refference the value in memory location lable+5

[1234]        refference the value in memory location 1234

Addition, subtraction, multiplication and division are supported

by the parser. Lables are treated as numbers when used in a math

expression.

Accumulator: The accumulator(s) are AX or AL and AH where AX is a

16 bit accumulator, AL is the lower 8 bits of AX and AH is the

higher 8 bits.

DISPLACEMENT

A displacement value to be added to the instruction pointer

(IP) is included as immediate data in the opcode. The assembler

calculates the amount of displacement based on the location of

the opcode and then location of the address in the operand. The

address in the operand can be expressed as a number (binary,hex

or decimal) but is most commonly expressed as a label.

Example

LABEL MOV AX,[BX]

CMP AX,10H

JLE LABEL

With this example the assembler calculates a negative

displacement to jump back to LABEL when then value in AX is less

than or equal 10H.

4

IMMEDIATE

All immediate data is assembled into the instruction

code. This data can be represented in two ways. First immediate

data can be presented in binary, decimal or hexidecimal format in

a signed range of -32768 to 32767 (8000H to 7FFFH) or if the sign

bit is not used, 0 to 65535 (0000H to FFFFH). As in these

examples a 'H' is appended to the number to indicate hexidecimal.

Binary numbers are expressed as a series of up to 16 ones and

zeros followed by a B, i.e. 11010B represents 26. The other method

of representing immediate data is with labels. The value of the

label is the address at which the label was used or the value

assigned to the label in an EQU pseudo op.

Example

Lable equ 10

Here db  10

MOV BX,LABEL         ;Load BX with the value of Label

MOV BX,OFFSET(Here)  ;Load BX with the address of Here

MEMORY/REGISTER

This addressing mode is also called indirect addressing. The

operand is used to point to a memory location that contains the

data rather than the instruction containing the data as in the

immediate addressing mode. The operand can be a memory location

expressed as a label, decimal number or a hexidecimal number or

it can be a memory location pointed to by a register. The

following indirect modes are allowed:

MOV Reg,[BP]

MOV [BX],Reg

MOV [BX+SI],Reg  ;BX plus SI displacement equal location

MOV Reg,[BX+DI]  ;BX plus DI       "        "      "

MOV Reg,[BP+SI]  ;BP plus SI       "        "      "

MOV [BP+DI],Reg  ;BP plus SI       "        "      "

MOV [DI],Reg

MOV Reg,[SI]

MOV LABEL,Reg

MOV [1234],Reg

Any of the general purpose registers can be used in place of Reg

in these examples. Immediate data may also be substituted for a

source register however then the opcode most be appended with W

or B so the assembler knows if you are pointing to a word or byte

address. In addition to the above when an indirect address using

a register is chosen a displacement may also be used.

Example

MOV Reg,10H[BP]     ;Source address equal BP+10H

MOV Reg,-5[BP+SI]   ;Source address equal BP+SI-5

DEMO  EQU FFH

MOV DEMO[BX+DI],Reg ;Destination address equal BX+DI+255

REGISTER

In this addressing mode the data is contained in or is to be

stored in one of the 8088 registers. The  registers are AX

(AL+AH), BX (BL+BH), CX (CL+CH), DX (DL+DH), BP, DI, SI  and the

four segment registers CS, DS, ES, SS. All math operations use

the accumulator (AX, AL or AH) plus the MUL and DIV use the DX

register when 32 bit numbers are involved. The BX and BP

5

registers can be used as base pointers in the Data or Stack

segments respectively. The CX register can be used for a

automatic counter for some instructions. As demonstrated in

earlier examples the SI and DI registers can be used as indexing

registers. Any of the numerous assembly language books for the

IBM PC or PCjr will give you an explanation of each of the

processor registers and their uses.

PSEUDO OPCODES

Pseudo opcodes are assembler directives that control the

generation of the object code. The available pseudo ops are DB,

DS, DW, ENDP, EQU, ORG and PROC.

DB = define byte and has operands of one or more bytes and/or

a string ( DB 20H,'Demo' ). Strings are set off by single

quotes. Numbers are less than 256 and expressed in binary,

decimal or hexidecimal.

DS = define segment and initializes a string of memory

locations. The first operand defines the number of bytes to

be initialized. If included the second operand defines the

value the memory is to be initialized to. The default value

is zero. ( DS 20,FFH ;initialize 20 bytes to 255).

DW = define word and its operand(s) must be a number or a

label. With DW the low order byte is stored first in memory

as this is the format used by the 8088 for integer storage

(i.e. dw 1020H = db 20H,10H).

EQU= define the value of a label. All label definitions must

occur at the beginning of your program or errors may occur

in the assembly process. The most common error message

received from defining a label late in the program is 'PHASE

ERROR'. A phase error indicates the assembler generated a

address for a label on the second pass different from that

of the  first pass.

ORG= reset the location counter to new origin. Since all .COM

programs start at 100H the default setting for ASSEMBLE.COM

is 100H however you may have a need to start at 00H for a

driver routine or a machine language routine for BASIC.

PROC and ENDP are used together to define a program or procedure

as Near or Far. This information is used to determine the

type of return to be generated when a RET is encountered.

If no procedure is defined a Near procedure is assumed. The

syntax is:

PROC NEAR ;Proc must be followed by Near or Far

....

....

ENDP

If PROC is used an ENDP must be used.

ERROR MESSAGES

All error messages and diagnostics are printed immediately

before the line in which the error occurs. The total number of

error and diagnostic messages will be displayed at the end of the

source code print out immediately prior to the symbol table dump.

I have made an attempt to make error messages as user

friendly as possible. The most cryptic of the error messages is

6

the series you receive when there is a syntax error. This message

will be the opcode and ASSEMBLE.COM's interpretation of the type

of data included in the operands. For example the message

*** Syntax Error: MOV (16 bit immediate or 8 bit immediate), (none)

would appear immediate before a line of code containing the

instruction MOV 45H. By reviewing the type of data and the

allowable operands for each instruction you should be able to

locate the error. In the instruction above both a destination and

source operand are required and if immediate data is used it must

be the source operand.

'Phase Error' is most commonly caused by referencing an

equate before defineing it. I strongly recommend you only use the

EQU pseudo op at the beginning of your source code. This practice

should prevent this error and will make your source code easier

to read.

'Error: EQU without symbol' is received when you use the equate

pseudo-op without a label.

'Error: EQU with forward reference' is received if you

attempt to use a forward reference when equating a label.

'Error: ENDP without PROC'. You must specify where the

procedure begins.

'Error: Missing ENDP'. You must specify where the procedure

ends if PROC is used.

'Error: Procedures nested too deeply'. Only 10 levels of

nesting are allowed.

'Error: Duplicate label'. See section on labels.

'Error: Data too long' indicates use of a byte operand where

the data is out of the range of 0 to 255.

'Error: Too far for short jump' indicates a jump attempt

longer than -128 or +127 bytes.

'Error: Undefined Symbol' plus the operand  is displayed

when no match is found in the symbol table. Frequently caused by

bad syntax in the operand.

'Error: Illegal or undefined argument for OFFSET' is simular

to 'Undefined Symbol'

Two diagnostic messages may be given. The first follows a

syntax error and is 'Specify word or byte operand' if the

assembler could not determine which to use. The opcode must be

corrected by appending a B or W to it. The assembler is not smart

enough to give you this very often. The other message is just a

notice that you used a long jump where you could have used a

short jump and saved a byte of object code.

SOURCE CODE

Turbo Pascal source code for this assembler is avaliable for

those who wish to customize it for their own needs (or those who

would like to see what makes it tick). If you would like a copy

of the source code send a formatted disk and $10 to

George Fulford

RR 1 Box 163c

Shellsburg Ia 52332

Although I have no intentions of entering the software market at

this time I do plan to make corrections to this program as the

bugs are found. If you obtain a copy of the source code from me

7

it will be the most up to date version.

WARRENTY/GUARANTEE

There is NONE.

This assembler runs on my PCjr and since I used all standard

Turbo Pascal it should run on any PC DOS machine. If it does not

I probably won't be able to help you.

I have spent quite a bit of time debuging but I am sure there are

still a few bugs lurking in the code. I will attempt to stamp out

any reported.

UPDATES

Changes made by version 1.01

1) A more sophisticated number parsing routine was added. An

offset may now be added to a lable. Multiplication and

subtraction now occur in the correct order.

2) $ can now be used to indicate the present memory location.

3) Due to memory management problems with the PCjr lables are no

longer stored in a linked list that uses all of the avaliable

memory. Lables are stored in a predefined array of 400. This

should be sufficient for all small programs. The 'Stack Overflow

Error' is no longer applicable.

4) Lable storage has been expanded to twelve characters to

improve the readability of lables.

5) A larger input buffer has been allocated to reduce disk access

time and other minor changes in code to inprove performance.

6) Bugs in DIV, SHR, SHL, ORB instruction have been corrected.

DIVB and DIVW have been added to allow 8 or 16 bit divides useing

indirect addressing.

7) A bug in the line parsing routine that stopped the parsing at

a semicolon that was enclosed in quotes has been corrected.

8) An error in interpeting [BP] has been corrected.

8

OP CODE TABLE

addressing modes supported  (b/w = must specify byte or word)

A = acumulator reg(ax, ah, al)

b/w = must add B or W to opcode for this addressing mode

D = displacememt (8 or 16 bit as required by the instruction)

I = immediate (byte for 8 bit registers, word for 16 bit reg)

M/R = memory or register indirect addressing

N = none

R = register(bx, cx, dx, bp, si, di)

S = segment register (cs, ds, es, ss)

Op                   Operand

types

dest.  N | A |  A  | R | R | M/R |  R  | M/R | I | I | D | M/R

source N | I | M/R | I | N |  R  | M/R |  I  | I | N | N |  N

AAA      x |   |     |   |   |     |     |     |   |   |   |

AAD      x |   |     |   |   |     |     |     |   |   |   |

AAM      x |   |     |   |   |     |     |     |   |   |   |

AAS      x |   |     |   |   |     |     |     |   |   |   |

ADC        | x |     | x |   |  x  |  x  | b/w |   |   |   |

AND        | x |     | x |   |  x  |  x  | b/w |   |   |   |

CALL       |   |     |   |   |     |     |     | x |   | x |

CALLF      |   |     |   |   |     |     |     |   |   |   |  x

CALLN      |   |     |   |   |     |     |     |   |   |   |  x

CBW      x |   |     |   |   |     |     |     |   |   |   |

CLC      x |   |     |   |   |     |     |     |   |   |   |

CLD      x |   |     |   |   |     |     |     |   |   |   |

CLI      x |   |     |   |   |     |     |     |   |   |   |

CMC      x |   |     |   |   |     |     |     |   |   |   |

CMP        | x |     | x |   |  x  |  x  | b/w |   |   |   |

CMPS   b/w |   |     |   |   |     |     |     |   |   |   |

CWD      x |   |     |   |   |     |     |     |   |   |   |

DAA      x |   |     |   |   |     |     |     |   |   |   |

DAS      x |   |     |   |   |     |     |     |   |   |   |

DB         |   |     |   |   |     |     |     | x | x |   |

DEC        |   |     |   | x |     |     |     |   |   |   | b/w

DIV        |   |     |   | x |     |     |     |   |   |   | b/w

DS         |   |     |   |   |     |     |     | x | x |   |

DW         |   |     |   |   |     |     |     | x | x |   |

ENDP     x |   |     |   |   |     |     |     |   |   |   |

EQU        |   |     |   |   |     |     |     |   | x |   | memory

HLT      x |   |     |   |   |     |     |     |   |   |   |

IDIV       |   |  x  |   |   |     |     |     |   |   |   |

IMUL       |   |  x  |   |   |     |     |     |   |   |   |

IN         | x |note1|   |   |     |     |     |   |   |   |

9

Op                   Operand

types

dest.  N | A |  A  | R | R | M/R |  R  | M/R | I | I | D | M/R

source N | I | M/R | I | N |  R  | M/R |  I  | I | N | N |  N

INC        |   |     |   | x |     |     |     |   |   |   | b/w

INT      x |   |     |   |   |     |     |     |   | x |   |

INTO     x |   |     |   |   |     |     |     |   |   |   |

IRET     x |   |     |   |   |     |     |     |   |   |   |

JA         |   |     |   |   |     |     |     |   |   | x |

JAE        |   |     |   |   |     |     |     |   |   | x |

JB         |   |     |   |   |     |     |     |   |   | x |

JBE        |   |     |   |   |     |     |     |   |   | x |

JCXZ       |   |     |   |   |     |     |     |   |   | x |

JE         |   |     |   |   |     |     |     |   |   | x |

JG         |   |     |   |   |     |     |     |   |   | x |

JGE        |   |     |   |   |     |     |     |   |   | x |

JL         |   |     |   |   |     |     |     |   |   | x |

JLE        |   |     |   |   |     |     |     |   |   | x |

JMP        |   |     |   |   |     |     |     |   |   | x |

JMPF       |   |     |   |   |     |     |     |   |   | x |

JMPN       |   |     |   |   |     |     |     |   |   | x |

JMPS       |   |     |   |   |     |     |     |   |   | x |

JNE        |   |     |   |   |     |     |     |   |   | x |

JNO        |   |     |   |   |     |     |     |   |   | x |

JNP        |   |     |   |   |     |     |     |   |   | x |

JNS        |   |     |   |   |     |     |     |   |   | x |

JNZ        |   |     |   |   |     |     |     |   |   | x |

JO         |   |     |   |   |     |     |     |   |   | x |

JP         |   |     |   |   |     |     |     |   |   | x |

JPE        |   |     |   |   |     |     |     |   |   | x |

JPO        |   |     |   |   |     |     |     |   |   | x |

JS         |   |     |   |   |     |     |     |   |   | x |

JZ         |   |     |   |   |     |     |     |   |   | x |

LAHF     x |   |     |   |   |     |     |     |   |   |   |

LDS        |   |     |   |   |     |note2|     |   |   |   |

LEA        |   |     |   |   |     |note2|     |   |   |   |

LES        |   |     |   |   |     |note2|     |   |   |   |

LOCK     x |   |     |   |   |     |     |     |   |   |   |

LODS   b/w |   |     |   |   |     |     |     |   |   |   |

LOOP       |   |     |   |   |     |     |     |   |   | x |

LOOPE      |   |     |   |   |     |     |     |   |   | x |

LOOPNE     |   |     |   |   |     |     |     |   |   | x |

LOOPNZ     |   |     |   |   |     |     |     |   |   | x |

LOOPZ      |   |     |   |   |     |     |     |   |   | x |

10

Op                   Operand

types

dest.  N | A |  A  | R | R | M/R |  R  | M/R | I | I | D | M/R

source N | I | M/R | I | N |  R  | M/R |  I  | I | N | N |  N

MOV   note3|   |  x  |   |   |  x  |  x  | b/w |   |   |   |

MOVS   b/w |   |     |   |   |     |     |     |   |   |   |

MUL        |   |  x  |   |   |     |     |     |   |   |   |

NEG        |   |     |   | x |     |     |     |   |   |   |

NOP      x |   |     |   |   |     |     |     |   |   |   |

NOT        |   |     |   | x |     |     |     |   |   |   | b/w

OR         | x |     | x |   |  x  |  x  | b/w |   |   |   |

ORG        |   |     |   |   |     |     |     |   | x |   |

OUT        |   |note1|   |   |     |     |     |   |   |   |

POP        |   |     |   | x |     |     |     |   |   |   |x or seg

POPF     x |   |     |   |   |     |     |     |   |   |   |

PROC  note4|   |     |   |   |     |     |     |   |   |   |

PUSH       |   |     |   | x |     |     |     |   |   |   |x or seg

PUSHF    x |   |     |   |   |     |     |     |   |   |   |

RCL        |   |     |   | x |     |     |     |   |   |   | b/w

RCR        |   |     |   | x |     |     |     |   |   |   | b/w

REP      x |   |     |   |   |     |     |     |   |   |   |

REPE     x |   |     |   |   |     |     |     |   |   |   |

REPNE    x |   |     |   |   |     |     |     |   |   |   |

REPNZ    x |   |     |   |   |     |     |     |   |   |   |

REPZ     x |   |     |   |   |     |     |     |   |   |   |

RET      x |   |     |   |   |     |     |     |   |   | x |

ROL        |   |     |   | x |     |     |     |   |   |   | b/w

ROR        |   |     |   | x |     |     |     |   |   |   | b/w

SAHF     x |   |     |   |   |     |     |     |   |   |   |

SAR        |   |     |   | x |     |     |     |   |   |   | b/w

SBB        | x |     | x |   |  x  |  x  | b/w |   |   |   |

SCAS   b/w |   |     |   |   |     |     |     |   |   |   |

SEG        |   |     |   |   |     |     |     |   | x |   |

SHL        |   |     |   | x |     |     | b/w |   |   |   |

SHR        |   |     |   | x |     |     | b/w |   |   |   |

STC      x |   |     |   |   |     |     |     |   |   |   |

STD      x |   |     |   |   |     |     |     |   |   |   |

STI      x |   |     |   |   |     |     |     |   |   |   |

STOS   b/w |   |     |   |   |     |     |     |   |   |   |

SUB        | x |     | x |   |  x  |  x  | b/w |   |   |   |

TEST       | x |     | x |   |  x  |  x  | b/w |   |   |   |

WAIT     x |   |     |   |   |     |     |     |   |   |   |

XCHG       |   |note5|   |   |  x  |  x  |     |   |   |   |

XLAT     x |   |     |   |   |     |     |     |   |   |   |

XOR        | x |     | x |   |  x  |  x  | b/w |   |   |   |

11

note 1 IN/OUT supports  DX<-acum(8 or 16) and  port<-acum(8 or 16).

note 2 These instructions can use only memory reference

in the source operand.

note 3 MOV also supports mem<-acum  seg<-M/R  and  M/R<-seg(or CS).

note 4 Must specify near or far PROCedure.

note 5 The accumulator can be exchanged with any of the registers

using the form XCHG AX,BX or XCHG BX

12

#