SAMPLE DEVICE DRIVER (VDSK) INSTRUCTIONS

These instructions are for the Sample Device Driver shown

on pages 14-27 through 14-34 of the DOS 2.0 Manual. The source

code is located on the Assembler Language BBS: (703) 425-7229.

1.  Assemble and Link the source code.

2.  Use the DOS EXE2BIN to convert the .EXE file produced

by the DOS LINK program to create the filename you

desire.

example:   we assembled - a program called

VDSK.ASM   and used the VDSK.OBJ

as input to the LINK to produce

THE VDSK.EXE FILE.

NOW:   A>EXE2BIN VDSK.EXE VDSK.COM

(This will produce the COM file

we will use next.)

When DOS 2.0 boots, it will look for a file on the disk we are

booting from named 'CONFIG.SYS' if it finds it there it expects it

to contain the names of the device drivers we want to use while run-

ning DOS. So we can use EDLIN or another editor to produce a file

called 'CONFIG.SYS' that will consist of one record that will contain

our device driver. On page 9-7 in the DOS 2.0 Manual you will find

the format of the 'DEVICE' Command.

So, our file will consist of a record that looks like:

DEVICE=VDSK.COM

and that is all we need. Our CONFIG.SYS file has the name of

our device driver (VDSK.COM) and make sure our VDSK.COM is on the

Booting disk. If you want to have the VDSK.COM on another diskette

just insert the disk designator into the name, (device=b:vdsk.com)

Now we have to reboot DOS so that DOS can find CONFIG.SYS

and install our driver.  You probably won't see DOS do it so when

DOS quiets down do a 'chkdsk c:' and you should see it out there.

If you have problems, give me a call and we will see if

we can get you on the right track. Either leave me a message on

this bbs or call (703) 425-6308 (BOB BLACKWELL).

HAVE FUN.....

#u on the right track. Either leave me a messag