[Start of file: vga.doc]

0.    Preamble: about this document

==    =============================

0.01  Title, subtitle, author, version, copyright

====  ===========================================

TITLE:       VGA Programmers Master Reference Manual

SUBTITLE:    Concepts and pragmatics of computational modelling,

visual model representation and manipulation, using

a Video Graphics Array controller chip.

{This is a somewhat expansive subtitle, suitable for }

{a somewhat more comprehensive work than the present.}

{This is justified only when additional work is done.}

EDITION:     First edition,  Summer 1991,  Identity:  91.07.52

AUTHOR:      Andrew Scott

COPYRIGHT:   (c) 1991 by Andrew Scott, world rights reserved

0.02  List of contents

====  ================

0.           Preamble: about this document

0.01    Title, subtitle, author, version, copyright

0.02    List of contents

0.03    Notice of copyright

0.04    Suggested license fees

0.05    Shareware and how it works

0.06    Obtaining a print of this document

0.07    Your expert input to this document

0.08    Contacting the author

0.09    Registration and Receipt Form

0.s     Summary

1.           Preface: how this document will help you

1.01    Goals of this document

1.02    Suggested audience orientation

1.03    How to use this document

1.s     Summary

2.           Components of a computer system

2.01    A computer system structure

2.02    Hardware of a computer system

2.03    Hardware: the input unit

2.04    Hardware: the processing unit

2.05    Hardware: the memory unit

2.06    Hardware: the control unit

2.07    Hardware: the output unit

2.08    Hardware: the external environment

2.09    Hardware: control by program

2.s     Summary

3.           Modelling using a computer system

3.01    Computer system operation

3.02    The original computer

3.03    Computation as modelling

3.04    The essentials of computation

3.05    Problem solving through model building

3.06    Types of models

3.07    Objects and structural models

3.08    Attributes, values and value spaces

3.09    Some properties of value spaces

3.10    Simple values, collected values

3.11    Models with memory

3.12    The memory store operation

3.13    The memory load operation

3.14    Modelling constants, variables

3.15    The problem of addresses

3.16    Structures expressed by means of values

3.17    The program as a model

3.18    The simplest operations for computation

3.s     Summary

4.           Visual modelling

4.01    Complexity control and abstraction

4.02    Human interaction with a computer system

4.03    Display quality issues

4.04    Context representation

4.05    Building blocks of a visual modelling scheme

4.s     Summary

5.           Display device characteristics

5.01    Raster scan display systems

5.02    Raster scan image refresh

5.03    Signals for image generation

5.04    Timing issues

5.05    Image generation can be dangerous!

5.06    Persistance in human vision

5.07    Interlaced display

5.08    Screen border display

5.09    Cursor generation

5.10    Multi-synch monitors

5.s     Summary

6.           VGA controller: Computer interface

6.01    Memory interface

6.02    Input, output, port interface

6.03    Complete input, output port accessible register list

6.04    Register specifications: Sequencer

6.05    Register specifications: CRT Controller

6.06    Register specifications: Graphics Controller

6.07    Register specifications: Attribute Controller

6.08    Register specifications: DAC Support Logic

6.09    Register specifications: General Registers

6.10    Register specifications: Special Registers

6.s     Summary

7.           VGA controller: Architecture

7.01    Controller timing generator:    { Sequencer            }

7.02    Display timing generator:       { CRT Controller       }

7.03    Image information processor:    { Graphics Controller  }

7.04    Image presentation processor:   { Attribute Controller }

7.05    Colour presentation processor:  { DAC Support Logic    }

7.s     Summary

8.           VGA controller: Text

{Only available in second edition}

8.01    Text image generation

8.02    Text font representation

8.03    Text attribute representation

8.04    Text character set extensions

8.05    Text mode support for graphics

9.           VGA controller: Graphics

{Only available in second edition}

9.01    Graphic image generation

9.02    Graphic multi-plane memory

9.03    Graphic packed pixel memory

9.04    Graphic line drawing

9.05    Graphic object drawing

10.          VGA controller: Hardware Interface

10.01   Hardware connector layout

10.02   Monitor auto-detection operation

10.03   Monitor auto-configuration operation

References.

Glossary.

Index.

0.03  Notice of copyright

====  ===================

This file is copyright (c) 1991 by A.J.B. Scott.  Restricted permission to copy

is given provided that the following conditions are met in full, without any

qualification:

1.  No fee shall be charged for the material content of this file, excepting

only that a small fee for media and copying time may be charged for.

2.  The file shall be copied entirely, without modification or ammendment,

the copyright notice shall be present in the original form, and these

conditions shall appear intact.

3.  The copying of this material shall not be for commercial gain with

the intent or effect of depriving the copyright owner of legitimate

income deriving from licenses granted to users for this work.

4.  The material appearing here is assigned by the author to distribution as

shareware.  This means that copyright remains with the author, and that

this material may be used in entirety for a short period for review.  At

completion of review, the user may decide that the material is useful,

in which case a registration fee presented by the user to the author will

secure a license to use, for the registering user.  The user may decide

what to pay for the license to use, depending upon the value to the user

of the material.  Guidelines are given below regarding a suitable fee.

A suggested period for review is thirty (30) days elapsed time.  After

this time, either the material should be licensed, or if unsuitable for

the purpose intended by the user, it should be passed in entirety to

another interested person or else completely destroyed.  Erasure from

the carrier medium is for this purpose equivalent to destruction.  Any

printed version must also be destroyed.

5.  The user is responsible for deciding the suitability or otherwise of this

material for purposes decided by the user.  The user makes use of this

work at the users own risk.  The author explicitly forbids any user from

using any material contained in this document in connection with any and

all life critical systems.

0.04  Suggested license fees

====  ======================

A suggested scale of license fees is given below, based upon the degree

of use to which the user puts or expects to put the enclosed material:

1.  If you find that certain portions of the material are sufficiently

useful that you need to refer to them in order to PROGRESS SOME

PROJECT that you are involved in, then you really are depending on

the material, so an appropriate payment would be SIXTEEN (16) POUNDS

U.K. STERLING.  This is the LICENSE FEE applicable when you make ONE

PRINTED COPY of this document.

2.  For registered users as in (1) above, FURTHER PRINTED COPIES of

the document may be taken, and registered for use, by payment of

FOUR (4) POUNDS U.K. STERLING PER PRINTED COPY.  There is NO

discount available for additional printed copies.

3.  If as a user, you find the material sufficiently interesting or

relevant that you READ ALL THE MATERIAL, then you could consider

that the material was either entertaining or extremely relevant to

you and your interests.  In this case an appropriate payment would

be TWELVE (12) U.K. POUNDS STERLING.  This assumes that you have

NOT MADE A PRINTED COPY.

4.  If you find that although you cannot use the material at this time,

you would EXPECT TO USE it at some time IN the FUTURE, you are of

the opinion that the material is potentially useful to you, so a

registration fee of EIGHT (8) U.K. POUNDS STERLING is appropriate.

This assumes that you have NOT MADE A PRINTED COPY.

5.  If you simply want to maintain the RIGHT TO LEGALLY USE the material

WHEN IT BECOMES IMPORTANT to you, then IN ORDER TO RETAIN it ON YOUR

COMPUTER SYSTEM, a payment of FOUR (4) U.K. POUNDS STERLING is the

suggested license fee.  This applies when you wish to KEEP material

for BACKUP PURPOSES or for "what if" or "just in case" situations.

This assumes that you have NOT MADE A PRINTED COPY.

6.  For REGISTERED USERS who wish to INCREASE their LEVEL OF REGISTRATION

at a later date, A DISCOUNT IS AVAILABLE.  To take advantage of this,

include your registration number with your order, and deduct ONE pound

sterling FOR EACH FOUR pounds DIFFERENCE in your existing and upgraded

registration levels.  This does NOT APPLY TO additional PRINTED COPIES

of this work.  See (2) above.

7.  IF you reviewed the material but do not find it interesting, or if

YOU CANNOT MAKE any USE OF IT, THEN you are requested to pass all the

material to someone who may be interested, or ENTIRELY DESTROY your

COPY of it, INCLUDING any PRINTED VERSION you may have made for review

purposes.  THIS PROVIDES you with a LEGITIMATE way of stating that

the material DOES NOT MEET your NEEDS.  If you need to KEEP A COPY,

then you SHOULD consider REGISTERING as in (5) above.

0.05  Shareware and how it works

====  ==========================

It is important to emphasise that this material did not just suddenly appear

as if by magic.  It had to be created, but only after the expenditure of much

thought and effort.  As with any human endevour, people who create works or

perform work for other people do expect and do need to be rewarded for their

efforts.

The purpose of the shareware concept is to provide a vehicle whereby talented

people can produce high quality work, which may subsequently be distributed

by many different channels, but always at low cost.  The ultimate goal is that

clients who use shareware, can obtain useful relevant products at a very low

price, without any compromise on quality.

For a shareware author, the business of marketing and distribution is taken

care of, leaving the author with the time and energy to concentrate on product

development, helping to ensure production of a quality product.

So, if you find any shareware product useful, make sure that you take the time

to give your vote of confidence in the product and its author, by registering

the product enclosing the appropriate fee to the author.

If you do not do this, the author will not be in a position to produce further

products that you will find useful, and, ultimately, you will be saying that

you do not respect the product you are using, the author who produced it, and

that you would rather buy an expensive product in a shiny wrapper than support

the shareware concept.

The simple fact is, you either register your use of a product, or see the means

that brought you that product (shareware) wither and die.  The choice is yours.

On the other hand, you should definately not register a product which is no use

to you, and will never be any use to you.  Many authors like to hear why their

product does not provide what you actually need, and would like your comments.

Why not support them and provide your views?  You can only benefit long term.

Some people  have found that authors of shareware products can be very flexible

when new updates are issued.  Some users have suddenly found that their views

have helped give them exactly what they need in a new product.

So, it's over to you.  Enjoy using this material or let me know how you would

like to see it improved or reoriented.  Thank you and good luck.

0.06  Obtaining a print of this document

====  ==================================

To make best use of this document, for evaluation purposes, or, after you have

registered and paid the appropriate license fee (see above for details), for

reference use, it is beneficial to obtain a printed version of this document.

For those of you who wish to print this document, it is necessary to make note

that the ascii text comprising this document is completely devoid of control

characters, excepting only cr {carriage return} and lf {line feed}.  No other

non-printing characters are used.  This has been done on purpose, so that any

printing device which supports ascii character sets may successfully produce

a print.

The absence of ff {form feed} control characters means that there will not be

a division of this document into complete page sections.  This should not be a

problem, since the material is indexed by the contents list, and there are

differences of page lengths supported by different printing devices, which in

practise would change the page numbering.

For these reasons, the concept of page has been completely ignored throughout

this document.

The printable width of lines is constrained to be 80 characters or less.  In

this way, virtually any industry standard output device may be used to create

a readable image of this text, whether on video display unit, or print device.

Note however, that the actual size of a line may be just over 80 characters

when the carriage return, line feed pair are taken into account.  This might

affect the setup of certain types of printer drivers.

If you do make a printed version of this document, and then decide that you do

not wish to register your copy, you should either pass the printed copy with

the computer readable copy to another person who may have an interest, or you

should destroy both copies.  This removes any obligation from you to register

your use by making a payment to the author.

UNDER NO CIRCUMSTANCES SHOULD YOU PRODUCE MORE THAN ONE PRINTED COPY WITHOUT

PAYMENT OF A REGISTRATION FEE TO THE AUTHOR.  If you have registered your use

and need to register more than one printed copy, please see above for the

beneficial rates available for this situation.

0.07  Your expert input to this document

====  ==================================

If you are an expert or have recently become an expert in areas from which this

work would benefit, and you would like to provide input to the author so that

others may benefit from your knowledge, please contact the author.  Any extra

materials provided will be included at this authors sole discretion, but such

included material will always be given special acknowledgement.  Any anonymous

material will be excluded unconditionally from further consideration.

If you have found this document to be useful but feel that it could be better,

please write to the author and make constructive suggestions.  Criticism will

be welcomed but only if it is of a constructive nature.  Your input as a user

is needed to help ensure that this document meets your needs today, tomorrow,

and into the future.

If you do wish to provide updates to this material, please provide these on a

1.44M or 720K 3.5 inch floppy disk.  Use a simple ascii file format without

using any control codes except carriage return and line feed.  When marking

changes in the existing text, use the following conventions.

Any source material which is to be changed, should be surrounded with special

brackets, indicating that it is subject to change.  These brackets appear as

follows:

!-!...original text to modify...!=!...new replacement text...!+!

It may be most useful to use this convention with complete lines of text, then

less confusion will result when many small changes are made to detail:

!-!

original body of text

which needs

some changes made

!=!

new text as

a replacement

for the above

!+!

Source material which is to be unconditionally deleted is dealt with as in the

following manner:

!-!

body of text which needs

deleting without replacement

!-!

Source material which is to be inserted as entirely new text, is introduced in

the following way:

!+!

body of text which needs

interting as entirely

new material

!+!

Comments which need to be made, but which should not be included in the body of

the text, may be enclosed with appropriate brackets as follows:

!'!

Note ' is a single quote !

comments which describe

the reasons for performing

edits, deletion or insertion

of new material

Note " is a double quote !

!"!

Please note that the above markers cannot contain each other.  That is, it is

possible only to operate with a flat file, and nesting of markers is illegal.

Markers can however be conjoined together one set following immediately after

another set, without intervening text.

These conventions must be adopted since the author relies on automated library

processing software.

All such material will become the property and copyright of the present author.

Regretfully, no payment can be made for any material provided in this way, and

disks will not be returned.  The reasons for these conditions are simple.  The

author does not have unlimited time available to service this work, so any way

of making administration easier must be seriously considered.   Who knows, if

this material attracts sufficient registrations, more time might be available.

0.08  Contacting the author

====  =====================

The author is a practising software/hardware engineer who often travels abroad

in connection with an international consultancy business.  The contact address

to use (which is only a forewarding address) is as follows:

_____________________

|                     |

|   Andrew Scott      |

|   SHAREWARE 01      |

|   c/o 1 Bramcote    |

|   Aylesbury         |

|   Buckinghamshire   |

|   HP20 1QE          |

|   ENGLAND           |

|_____________________|

There is no possibility of telephone contact simply because the author spends

long periods of time abroad.  This situation is regretted but could change if

sufficient registrations warrant a different approach.

If you wish to communicate put your letter on a 720K or 1.44M  3.5 inch disc

with a filename of 'letter.doc'.  Do not forget to put your name and address

on the disc in a file called 'address.doc' then mail it to the above address.

All files should be in plain ascii, without control codes other than carriage

return and line feed.  No other file formats are acceptable.  An 'address.doc'

file should print your name and address exactly as it should be written on an

envelope to be posted to you.  Please note that discs will not be accepted if

they contain executable files of any type whatsoever.  This is to prevent any

difficulties with software licensing, and to help ensure freedom from viruses.

Please note that the author uses a standard procedure with any received discs.

This involves using a special bootable program to wipe all files from the disc

which do not have the extension '.doc'.  In particular, the disc is rendered

unbootable.  Only after this special virus proof program has been run is the

disc read using a conventional industry standard operating system, with a set

of appropriate adjustments made to the floppy disc drive control software for

further security protection.

Do please remember this if you need to send a disc.

If you need to send program code, then it must be sent in source form, and you

must be the author, or possess a license sufficient to transmit source to a

third party.  Please include details of this license in a separate file with

the name 'license.doc'.

In any case, program fragments are to be preferred, since they make a technical

point without introducing irrelevant detail.  Source should be in some genus of

80x86 assembly language.

No written communications can be accepted, since they involve so much handling

overhead.  All discs become the property of the present author on receipt.

The author regrets that failure to observe these requirements will prevent any

attempted communication from taking place.

The policy of the author is that any licensed files will be erased as soon as

it is practically possible to do so.  The author does not have any interest

whatsoever in 'data collection' of any type.  This is particularly true in the

case of material which may be the licensable property of others.

Although the author will attempt to respond where appropriate, no undertaking

can be given on this point, simply due to pressure of time.  This unfortunate

circumstance could change should the level of user support of this work prove

to be of an appropriate level.

0.09  Registration and Receipt Form

====  =============================

Registrations are made only by means of a cheque drawn on a U.K. bank in pounds

sterling as detailed above.  Please make the cheque payable to:

___________________

|                   |

|   Andrew Scott    |

|___________________|

All registrations MUST be accompanied by the Registration and Receipt Form that

appears below.  Use your text editor to extract this form then fill it in, or

alternatively, use the separate file 'register.doc' in the same way.  Having

filled in the form, print it out and enclose it with your cheque.  As before,

the motivation for this is the saving of administration time.  Cheques provided

without the form will be accepted, but of course, no receipt can then be given.

It may be necessary to allow some time to elapse before receiving your receipt.

Do not be alarmed by this.  If you do not require a receipt, please either tick

the box to state this, or alternatively, just send your cheque unaccompanied.

Please note that it is not the authors policy to keep names and addresses of

registrees in computer readable form, since this can cause difficulties with

existing U.K. legislation in terms of overhead cost in time and money.

Please find below the form which MUST accompany your registration cheque.  It

is followed by an example which has been filled in for a ficticious client at

a U.K. address.  After that, another example follows, for a foreign client.

==============================================================================

Registration and Receipt Form  [Start]

==============================================================================

When completed, please send a printed version of this form together

with the appropriate value cheque to the address following:

_____________________

|                     |

|   Andrew Scott      |

|   SHAREWARE 01      |

|   c/o 1 Bramcote    |

|   Aylesbury         |

|   Buckinghamshire   |

|   HP20 1QE          |

|   ENGLAND           |

|_____________________|

Client Name and Address (as for an envelope to be posted).

You MUST use your text editor; handwriting is unreliable.

Simply leave any unused lines free of text.  Please leave

the numeric fields as they stand, unaltered.  Use a clear

simple typeface when printing this form prior to mailing.

001:

----------------------------------------------------

002:

----------------------------------------------------

003:

----------------------------------------------------

004:

----------------------------------------------------

005:

----------------------------------------------------

006:

----------------------------------------------------

007:

----------------------------------------------------

008:

----------------------------------------------------

009:

----------------------------------------------------

010:

----------------------------------------------------

011:

----------------------------------------------------

012:

----------------------------------------------------

Receipt Required?  Please delete 'Yes' if no receipt is required.

120:      Yes

-------------

Please indicate here the actual license REGISTRATION FEE you are paying,

WITHOUT including any provision for U.K. local tax {V.A.T.}.

230:

-------------

If you have an ADDRESS in the U.K. (V.A.T. area) you MUST INCLUDE V.A.T.

at the CURRENT RATE.  It is 17.5% at this time, but may of course change

later.  Your RECEIPT will PROVIDE you with PROOF of V.A.T. SPENDING.

OVERSEAS client ADDRESSES do NOT pay V.A.T., so for them, this is zero.

Please indicate here the amount of V.A.T. you are paying.

231:

-------------

This amount MUST match the VALUE of your CHEQUE.

YOUR CHEQUE MUST BE DRAWN ON A U.K. BANK IN POUNDS STERLING.

If you have a U.K. address YOU MUST INCLUDE V.A.T.

If you have a FOREIGN address, you DO NOT pay V.A.T.

Please indicate here the value of your registration fee together with any

local U.K. tax {V.A.T.} you are paying.

234:

-------------

Registration Number issued in receipt.  Please leave this blank

unless you have already registered and are increasing the level

of your existing registration.  Do not forget a discount may be

available to you.  See {0.04} Suggested license fees.

340:

---------------------------------------------------------

==============================================================================

Registration and Receipt Form  [End]

==============================================================================

==============================================================================

EXAMPLE UK         Registration and Receipt Form  [Start]  EXAMPLE UK

==============================================================================

When completed, please send a printed version of this form together

with the appropriate value cheque to the address following:

_____________________

|                     |

|   Andrew Scott      |

|   SHAREWARE 01      |

|   c/o 1 Bramcote    |

|   Aylesbury         |

|   Buckinghamshire   |

|   HP20 1QE          |

|   ENGLAND           |

|_____________________|

Client Name and Address (as for an envelope to be posted).

You MUST use your text editor; handwriting is unreliable.

Simply leave any unused lines free of text.  Please leave

the numeric fields as they stand, unaltered.  Use a clear

simple typeface when printing this form prior to mailing.

001:       John Smith

----------------------------------------------------

002:       Basement Flat

----------------------------------------------------

003:       Number 3

----------------------------------------------------

004:       Holly Lane

----------------------------------------------------

005:       Beringsbeare

----------------------------------------------------

006:       Tullingdon

----------------------------------------------------

007:       Near Great Minsforth

----------------------------------------------------

008:       Shrafton

----------------------------------------------------

009:       Shropshire

----------------------------------------------------

010:       SH12 4PB

----------------------------------------------------

011:

----------------------------------------------------

012:

----------------------------------------------------

Receipt Required?  Please delete 'Yes' if no receipt is required.

120:      Yes

-------------

Please indicate here the actual license REGISTRATION FEE you are paying,

WITHOUT including any provision for U.K. local tax {V.A.T.}.

230:   16.00

-------------

If you have an ADDRESS in the U.K. (V.A.T. area) you MUST INCLUDE V.A.T.

at the CURRENT RATE.  It is 17.5% at this time, but may of course change

later.  Your RECEIPT will PROVIDE you with PROOF of V.A.T. SPENDING.

OVERSEAS client ADDRESSES do NOT pay V.A.T., so for them, this is zero.

Please indicate here the amount of V.A.T. you are paying.

231:   2.80

-------------

This amount MUST match the VALUE of your CHEQUE.

YOUR CHEQUE MUST BE DRAWN ON A U.K. BANK IN POUNDS STERLING.

If you have a U.K. address YOU MUST INCLUDE V.A.T.

If you have a FOREIGN address, you DO NOT pay V.A.T.

Please indicate here the value of your registration fee together with any

local U.K. tax {V.A.T.} you are paying.

234:  18.80

-------------

Registration Number issued in receipt.  Please leave this blank

unless you have already registered and are increasing the level

of your existing registration.  Do not forget a discount may be

available to you.  See {0.04} Suggested license fees.

340:

---------------------------------------------------------

==============================================================================

EXAMPLE UK         Registration and Receipt Form  [End]  EXAMPLE UK

==============================================================================

==============================================================================

EXAMPLE FOREIGN    Registration and Receipt Form  [Start]  EXAMPLE FOREIGN

==============================================================================

When completed, please send a printed version of this form together

with the appropriate value cheque to the address following:

_____________________

|                     |

|   Andrew Scott      |

|   SHAREWARE 01      |

|   c/o 1 Bramcote    |

|   Aylesbury         |

|   Buckinghamshire   |

|   HP20 1QE          |

|   ENGLAND           |

|_____________________|

Client Name and Address (as for an envelope to be posted).

You MUST use your text editor; handwriting is unreliable.

Simply leave any unused lines free of text.  Please leave

the numeric fields as they stand, unaltered.  Use a clear

simple typeface when printing this form prior to mailing.

001:       Johan Schmidt

----------------------------------------------------

002:       Heliger Strasse 13

----------------------------------------------------

003:       Markt Windsheim

----------------------------------------------------

004:       Langenburg

----------------------------------------------------

005:       Aschaffenburg

----------------------------------------------------

006:       Frankfurt a/d Main

----------------------------------------------------

007:       Germany

----------------------------------------------------

008:

----------------------------------------------------

009:

----------------------------------------------------

010:

----------------------------------------------------

011:

----------------------------------------------------

012:

----------------------------------------------------

Receipt Required?  Please delete 'Yes' if no receipt is required.

120:      Yes

-------------

Please indicate here the actual license REGISTRATION FEE you are paying,

WITHOUT including any provision for U.K. local tax {V.A.T.}.

230:   16.00

-------------

If you have an ADDRESS in the U.K. (V.A.T. area) you MUST INCLUDE V.A.T.

at the CURRENT RATE.  It is 17.5% at this time, but may of course change

later.  Your RECEIPT will PROVIDE you with PROOF of V.A.T. SPENDING.

OVERSEAS client ADDRESSES do NOT pay V.A.T., so for them, this is zero.

Please indicate here the amount of V.A.T. you are paying.

231:   0.0

-------------

This amount MUST match the VALUE of your CHEQUE.

YOUR CHEQUE MUST BE DRAWN ON A U.K. BANK IN POUNDS STERLING.

If you have a U.K. address YOU MUST INCLUDE V.A.T.

If you have a FOREIGN address, you DO NOT pay V.A.T.

Please indicate here the value of your registration fee together with any

local U.K. tax {V.A.T.} you are paying.

234:  16.00

-------------

Registration Number issued in receipt.  Please leave this blank

unless you have already registered and are increasing the level

of your existing registration.  Do not forget a discount may be

available to you.  See {0.04} Suggested license fees.

340:

---------------------------------------------------------

==============================================================================

EXAMPLE FOREIGN    Registration and Receipt Form  [End]  EXAMPLE FOREIGN

==============================================================================

0.s   Summary

===   =======

This document is a copyright work, for which a license to use is required.

Details are given for registering a user license, with appropriate payment.

Technical notes on the source character set employed in this document are

made for those needing to produce a printed version of this work.

Expert input from people who have seen this document is invited with a view to

increasing the usefulness, relevance and quality for existing and prospective

registered users.

Details are given on procedure for contact with the author, and for making

payment for a user license.

A disclaimer is presented concerned with fitness for use of this work.

1.    Preface: how this document will help you

==    ========================================

1.01  Goals of this document

====  ======================

The goal of this document is the description of means for programming the VGA

controller chips found in contemporary Intel 80x86 processor powered industry

standard architecture computer systems.

To program any device effectively requires intimate knowledge of environmental

issues which determine the context within which the device finds application,

the technical issues which determine the structure of the device, and finally,

full details of the programming interface available to the host machine.

We will first consider the background against which any computer system works.

This will broaden into a brief overview of the structures which comprise any

computer system, and which are necessary to support computation.  Computation

is then considered as a modelling activity.  We will consider means for making

models, and the representation of models in people oriented terms.

These considerations will naturally broaden into a discussion of video image

generation and the employment of images as an essential part of the process

of communication between people and computer systems.

We consider the reasons why images can be appropriate for the expression of

complex modelling relations, which then leads naturally to considerations for

the technical specification of image generating capabilities.

The architectural aspects of the host machine are then discussed, in particular

the implications of memory mapped interfaces, and the means for controlling the

operation of a very highly configurable video image generator chip.  This gives

reasons for the register input output structures commonly used when programming

the VGA controller chip.

Details of the conceptual and physical structure of the video image generator

are examined, with discussion of the motivation for the various programmable

features introduced into the hardware.  The characteristics of the hardware

are given, with details of the user oriented operations which motivate them,

and the programming strategies to be adopted in their use for achievement of

maximally high image manipulation performance.

The main part of this work is centred upon the highly complex register set that

provides the VGA controller with all its many programmable option settings.  It

is the interaction of these settings which, for the most part, makes the VGA so

difficult to understand and program.  We will describe the data content of each

register, in a systematic manner.  There will be discussion concerning the real

design motivation for providing the particular feature which is being described

which will in most cases considerably clarify complex issues.  Further, related

features are brought into focus, with either a useful comment as introduction,

or with a full discussion.  In all cases, cross references are given at the top

of each register description.  These will yield considerable insight, when used

to direct attention on a particular point of interest.

By following the cross references in a random manner the informed reader will

very quickly build up a complete picture of essential VGA operation.  This is

very important prior to starting any serious programming for this very highly

flexible but very complex device.  Excellent advice to follow before creating

any program for the VGA device is to obtain from the original manufacturer of

your particular VGA absolutely all the technical documentation available.  In

this way, you will hopefully have some chance of being adequately informed as

most VGA controller chips now available support extensive additional features

which will, for the most part, be completely non-standard.

1.02  Suggested audience orientation

====  ==============================

This document is primarily concerned with a particularly technical aspect of

programming the industry standard architecture machines employing Intel 80x86

processor chips.  As such, the reader would benefit considerably from a good

knowledge of Intel 80x86 assembly language programming, including architectural

appreciation of that line of processor chips.  In particular, the memory access

and input output model of the 8086 processor chip should be understood.

If the reader is not concerned with the intimate details of VGA programming,

this document should provide all the source material neeeded to form a view of

the capabilities and attendant design issues facing a VGA programmer.  This

might be of interest to a system designer who is addressing the question of

whether to develop a product using the full capability of the VGA video system.

Knowledge of hardware structures would be useful, but should not be essential

for obtaining spectacular performance from the hardware described.  However,

some degree of conceptual awareness of hardware structures will be developed in

this document, and without this knowledge, it would be very difficult to obtain

original practical results for a special application.  This level of knowledge

is fairly simple, so should not prove onerous to even the most wisened hardware

illiterate programmer.

Programming templates will be given in a generic 8086 assembly language,

without any pseudo-operation context.  The intention is to produce practical

conceptual knowledge, which is perpetually reuseable, rather than a set of

recipes for various commonly occuring problems.  The emphasis is firmly on

high performance, which means writing code with a solid understanding of what

is happening in the hardware.  Only in this way can the full power of the VGA

controller chip be realised in practical application.

It should be fairly clear by now that we will not be concerned with means by

which the VGA can be programmed with involvement of a video bios {basic input

output system}.  We will instead consider the questions involved in writing

our own application specific bios, using only the naked resources provided by

the VGA controller chip itself.  People interested in programming using the

various bios facilities will, nevertheless, find this document interesting in

the sense that having understood this material, the operations of the bios

will become much clearer.  This means more effective programming, even at the

more constrained level of bios operations.  In addition, the potential always

exists to perform mixed bios and customised high speed programming of the VGA

controller for special applications which do not warrant the development of a

full custom programmed solution.  In all these situations, the material given

here should help the programmer to obtain the desired results without too much

fuss or heartache.

Since the price of VGA controllers with massive amounts of memory has reached

a very affordable level at the time of writing {summer of 1991}, we will not

dwell on the various video standards which preceeded VGA, but will from time

to time make parenthetic notes about particular compatibility considerations

which motivate various aspects of the hardware design.  This will represent

the full extent of our interest in such compatibility issues.  In particular,

we will often assume that we are using a VGA controller with certain specific

characteristics.  That is, we assume a Trident 8900 VGA controller chip, with

a full one megabyte of video memory.  These assumptions will not generally

upset the validity of the concepts and programming techniques discussed, but

do serve to provide a complete specification of the environment within which

this document was conceived and written.

The most important qualification for gaining benefit from this document is an

interest in understanding the subject matter, and a relatively keen intellect.

The rest is the ability to author well conceived program code, and the strength

to accept that mistakes need to be made before outstanding results are seen.

Do remember that when dealing with inherantly complex issues, the only way to

succeed is to use the appropriate approach.  As Sir Winston Churchill once said

when speaking to the Oxford University Student Union:

"Never give up".   "Never, never, give up".   "Never, never, never, give up".

This was his address in its entirety, which received rapturous applause and

doubtless helped to change the lives of many as a result.

1.03  How to use this document

====  ========================

We spend some time developing a model of computation, which embraces the

hardware and software structures necessary for practical problem solving

using computer based modeling techniques.  This material can be ommited

by those who are interested in serious descriptions of the hardware form

of the VGA controller chip, and the software interface available providing

control using the host machine.

Students of computation may find the discussion of interst, since it draws a

parallel between the general basis for computation and the generic hardware

manipulation facilities provided within the VGA controller chip.  In this

way, the VGA controller chip can be seen as a specialised processor adapted

for the specific operations required for image transformation and transport.

Specifically, some VGA controller image manipulation primitives can be seen

to have the control element of the image computation provided explicitly by

the host processor.  This partitioning of the control leads to a flexibility

which would not otherwise exist, and can be interpreted as a natural extension

of the classical architecture of a general purpose digital computer system.

To employ the Intel nomenclature, the VGA controller chip can under practical

conditions be viewed as a very specialised (so therefore complex) co-processor

device.

1.s   Summary

===   =======

This document is concerned with high performance programming for the video

graphics array controller Trident 8900, equipped with 1 megabyte of memory.

The emphasis is on programming for high performance image manipulation and

presentation, principally using assembly language without use of a video bios.

A guide to the relevant chapters is given for those with various backgrounds,

motivations and interests.

2.    Orientation - the computer system

==    =================================

2.01  A computer system structure

====  ===========================

A computer system is a collection of many different component parts.  Some of

these are fixed and invariant in structure, whilst others are only partially

fixed in structure so may in practise have a variable structure.

We usually think of these two distinct properties as belonging to the hardware

components and software components, respectively, of a computer system.  These

two types of components make up an entire computer system, which consists only

of hardware components and software components.

The hardware components of a computer system represent the finite fixed means

available to enact a computation described in precise detail by the software

components of that computer system.

The software components of a computer system determine every detail of the

computation of that system, by means of a moment by moment selection of the

actual hardware capabilities brought into play, from the entire set of possible

capabilities available.  We talk about the entire collection of software

components associated with a computer system as the program for that computer

system.  Therefore a program is a complete collection of software components,

sufficient to fully direct the hardware which makes up a computer system.

2.02  Hardware of a computer system

====  =============================

The physical reality of the computer system hardware appears as a number of

fundamental units, interconnected with each other, but each of which is needed

in any general purpose digital computer system.  {We distinguish this class of

computer system in particular, since there are other classes of computer system

with which we will not concern ourselves.}  These units are as follows:

1.  Input unit

2.  Processing unit

3.  Memory unit

4.  Control unit

5.  Output unit

Each of these units is physically constructed from a mix of components which

could be classed as mechanical, electro-mechanical, electro-optical or

electronic.  Other technologies may be used, but it is essential to understand

that the technology implementing hardware capability is not in itself crucial;

rather, it is the information processing capabilities provided by the hardware

units themselves which is important.

In some cases, only one class of component is used in the construction of a

particular type of unit.  For example, an input unit might use mechanical,

electro-mechanical and electronic components in its realisation.  This would

contrast with a processing unit which might use only electronic components in

its construction.  The construction is not as important as the fixed set of

capabilities supported by each unit, expressed in terms of information transfer

and modification abilities of a particular unit, as noted above.

2.03  Hardware: the input unit

====  ========================

The input unit has the fixed capability to obtain information from somewhere

outside the computer system and deliver it into the computer system in a timely

manner.  This is the only purpose for which the input unit is used.

The somewhere outside is usually referred to as the external environment within

which the computer system operates.

2.04  Hardware: the processing unit

====  =============================

The processing unit has a set of fixed capabilities for processing information,

that is, for changing information in a controlled manner.  Some of the ways in

which information may be changed are given below:

1.  Reduction of information.

This corresponds to removal of some of the information content of

data, thereby classifying data into a set of predetermined groups.

When one knows the classification of a data item, it is not then

possible to say with certainty what exact data derived that class,

since information loss has occured.  Therefore information loss is

associated with a one way process which cannot then be reversed to

obtain the original data.

2.  Transformation of information.

This does not change the information content of data, but merely

changes the representation of that data, to make it suitable for

use within a particular computational environment.

It is in principle possible to reverse the transformation of data

such that the transformed data could give rise to the original

data, assuming sufficient computational resources are available to

effect the inverse transformation.  {In fact, there may be reasons

why the inverse transformation does not exist in practise, although

this does not of itself deny the principle.}

3.  Comparison of information.

This actually creates information based upon the representation of

existing information.  This newly created information is available

for further processing just as any other information would be.

By changing the representation of data, the result of comparing two

data items can change.  The data representation is what determines

the comparison result, not the information content of the data.

These are the only ways available for processing information, so all possible

computations are built up from these types of information manipulation.

2.05  Hardware: the memory unit

====  =========================

The memory unit has the fixed capability of accepting information at one moment

in time, and delivering that information at a later moment in time without any

change having occured to that information in the intervening time.  The amount

of time which passes between the presentation of information to the memory unit

and the later recovery of that unaltered information is not important for the

operation of the memory.  In that sense it is completely time independant.

2.06  Hardware: the control unit

====  ==========================

The control unit mediates between the instructions which comprise a program and

the operation of all the hardware units which comprise the computer hardware.

The control unit manages and directs all flows of information within the entire

computer system.  No computation can proceed without the involvement of the

control unit, which initiates input information flows, directs the output of

information, determines the processing of information and the presentation to

and recovery from memory of information.

2.07  Hardware: the output unit

====  =========================

The output unit is equipped with fixed capability to accept information from

within the computer system and present that information in timely manner to

somewhere outside the computer system.  This destination is referred to as the

external environment, which is also the source of input information, as noted

above.

2.08  Hardware: the external environment

====  ==================================

This means that a computer system takes its input from, and delivers output to,

its external environment.  In this view of things, the computer system itself

is completely separate from the external environment which contains information

to process and which in turn accepts processed information.  This observation

is important, since it is essential that the computer system be able to act in

a manner completely independant of the information environment with which it is

associated.  It is only when this is true that a program can be seen to be the

sole determinant of the activities of a computer system.

2.09  Hardware: control by program

====  ============================

All these activities are controlled by the program which controls the operation

of the control unit and thence the operation of the entire computer system.  So

the hardware component of the computer system represented by the control unit

is the fixed means by which the entire computer system fixed capabilities are

organised and controlled, but the moment by moment operation of this unit is in

turn controlled through the statements which comprise the program controlling

the computer system.  This program comprises the full collection of individual

software components which together with the hardware components make up the

complete computer system, thereby determining its entire behaviour.

2.s   Summary

===   =======

The structure of a computer system is described in terms of components which

appear as hardware and software.  The nature of the hardware components and

their function is examined.  In particular, the types of information processes

available to an information processing unit are discussed.  The program as the

ultimate arbiter of computer system operational control is seen as a collection

of all the software components of a computer system.

3.    Modelling using a computer system

==    =================================

3.01  Computer system operation

====  =========================

We have seen that a computer system comprises hardware units with fixed sets

of capabilities which interact under the direction of the control unit.  This

control unit is itself controlled by the software components which when taken

together comprise a program.

So now we know what a computer system comprises.  This does not directly help

to answer the question concerning what it is a computer system does.  Once we

know what a computer system does, we will understand the motivation for being

interested in a computer system.  This question is what we will now look at.

3.02  The original computer

====  =====================

We can look back in time to the days prior to the existence of electronic

computers, when the english language held that a computer was a person who by

dint of intelligence, knowledge and insight, performed calculations of interest

using just paper and pen.

This is the sort of activity we engage in when we ask how much we have spent

on the basket of shopping we have just bought.  But we can also ask questions

about the future, such as how much would it cost to buy all the food necessary

to entertain a group of friends each of whom has a different taste from each of

the others.

If we decide that the proposed evening would be too expensive, we can then ask

what would be the most economical way of entertaining half our friends, knowing

the range of meals which each would find acceptable.  This might involve making

a decision regarding who should attend, followed by a decision as to what each

guest should be given to eat.

Of course, this latter problem would quickly prove impractical to solve if we

had a large number of friends, each of who delighted in a wide range of special

dishes from around the world.  This introduces the notion of the tractability

of solving a particular problem, given the context of a particular method for

solving that problem.

3.03  Computation as modelling

====  ========================

All of these activities are completely characterised by the posing of some kind

of question and the discovery of an appropriate answer.  The connection between

the question and the answer is known as a model.  The model provides all the

information required in order to specify the context in which the question is

framed, and within which the answer must be interpreted.  Naturally then, if

the model changes, then so too will the associated questions and their answers.

The person whos job was that of computer was required to solve problems given

as questions, by providing answers given numerically.  This meant that usually

the problems posed were of a numerical nature, requiring for their solution

the application of numerical formulae.  These formulae and the knowledge needed

to manipulate them were provided by the computer as a skilled worker.

In some other situations, there was no directly applicable numerical model for

solving a problem.  This was so when deciding upon the invitations for half our

friends to an evening meal.  In this case, a structural model is required, and

this model must be developed by the expert computer, before being applied to

the solution of the given problem.

3.04  The essentials of computation

====  =============================

It is in principle, possible to look in detail at all the activities of such a

skilled computer, in order to discover the simplest steps taken during the

solution of any problem.  If this is done, it turns out that there are a very

small number of simplest such steps, which suffice for the solution of any

problem whatsoever, providing that the problem is indeed soluable.  {There are

some problems which appear to be insoluable by any currently known methods, but

this should not concern us since most of these types of problems are not too

relevant from a practical point of view.}

Realising this, we could then conjecture that we could build a machine in some

appropriate technology, capable of realising each one of these simplest steps.

The exact sequence of simplest steps actually used to solve any given problem,

could then be written out as a sequence of instructions, in a form suitable for

controlling the machine we have just defined.

If this is done, we can then recognise all the elements of a computer system.

3.05  Problem solving through model building

====  ======================================

It is now possible to see why a computer system is interesting and useful.  We

can solve any soluable problem for which it is possible to express a model by

applying a computer system to the expression of the problem within that model.

We usually talk of a program to solve a particular problem, or type of problem

which occurs frequently in practise.  The use of a program to control the

operation of a computer system comprises the execution of that program by the

computer system.  The activity of a computer system while executing a program

is said to be that of computation.  Computation solves a problem expressed in

the context of a given model, by supplying a solution to the problem expressed

in that same context.

The essential point is that we can represent some aspect of a reality by means

of an appropriate model, which can then be manipulated by a computer system.

In this way we can ask questions and obtain answers, without affecting the real

world reality.  The advantage of this is clear when one considers the actual

cost of answering some difficult types of question by means of an experiment.

For example, how many atomic bombs of a cetain size would it take to destroy

mankind?  Clearly, if we insist on obtaining an answer to this particular type

of question, we would benefit considerably by using a computer based model.

3.06  Types of models

====  ===============

Computation is effectively a modelling process, so we will consider the types

of modelling with which real world problems may be effectively expressed.  In

this manner we can conclude that there are really two types of models:

1.  Numerical models

2.  Structural models

Numerical models rely completely upon relations between numeric values given

by formulae which themselves represent the model.  The values describe certain

attributes which are applicable in the modelled situation.  Given sufficient

information, it is possible to use numerical methods to obtain values for some

unknown quantities represented by the model.  In this way, problems which are

concerned with discovering values for unknown quantites can be solved, given

that sufficient known quantities are available.

Structural models represent collections of attributes which cluster in groups

known as objects.  The structural composition of any given object may vary in

permitted ways which are themselves characteristic of the type of object.  That

is, the structure of an object may not be necessarily fixed.  An object belongs

to a class of objects, which represents a collection of object types each of

which possesses common characteristics. This enables objects to represent those

situations where the morphology is either unknown in advance or is determined

dynamically on a moment by moment basis.

In general, a structural model will employ embedded within it, many numerical

models, each for use in connection with particular situations or circumstances

arising in the model itself, or in connection with manipulation of the model.

For this reason, it is normally the case that structural models tend to be more

complex than numerical models, because they are usually larger than numerical

models.  However, special types of numerical models do occur which are very

complex indeed.  {Linear programming equations for optimising the output from

an oil refinary give just one example of such a case.}

3.07  Objects and structural models

====  =============================

Objects may theselves be composed of other objects, with variable structure, so

in principle, any structure can be modelled by means of a hierarchy of various

component objects of arbitrary complexity.  There is no restriction regarding

the use of network structures as objects, so strictly hierarchial arrangements

of objects may be used or ignored according to the modelling requirement.  This

provides complete modelling freedom.

The types of questions which we might ask in connection with structural models

are generally concerned with forming collections of objects so that some given

attribute is minimised or maximised over the group.  This type of question can

provide the answers to questions concerned with finding minimal cost routes for

travelling between a collection of cities, or the cheapest or perhaps most time

efficient synthesis route for a molecule, given certain precursor stock.

Notice that although we have identified that an object is just a collection of

attributes, we have not identified what we might mean by attribute.  Similarly

we have associated values with attributes, but have not considered what a value

might be, or what it might represent.

3.08  Attributes, values and value spaces

====  ===================================

An attribute is nothing more than the expression of one particular possibility

from a collection of such possibilities.  An attribute takes on a value, and

that value has the potential to change from moment to moment.  {Of course, for

a given model, an attribute may take a single constant value.}  In this way, an

attribute is associated with a set of potential values, which does not change

during the passage of time.  The values associated with an attribute determine

the type of the attribute, that is, the nature of the information imparted by,

or associated with the attribute.

An attribute will take on a single value at any particular moment.  We speak of

a single value chosen from a value space.  A value space is simply the complete

collection of all possible values of a related type.  Here related type implies

that all the values play an identical role in any model of which they may be a

part.

The association of an attribute with a value represents the minimum expression

of information possible.  Although a value represents minimal information, it

is usually the case that the value space containing that value possesses some

internal structure which permits more information to be associated with the

value than might at first appear possible.

Typically this will be so where other values within the same value space are

combined by means of information processing steps, such that new models result

which represent different interpretations of the original model.  The values

which are combined in this way may be attributes of objects, or purely numeric

quantities associated with numerical models.

In fact, for processing of values to take place at all, certain properties must

exist in the underlying value space.  {In general, very weak properties in the

value space are adequate to support computation.  However, in any practical

computation scheme, stronger properties are needed which have a significant

effect on the richness of available algorithms to solve any particular problem.

This richness manifests itself as a change from algorithmic intractability to

eminently practical algorithms, as the properties of the value space increase

in strength.}

3.09  Some properties of value spaces

====  ===============================

In any type of modelling scheme, certain underlying properties of the various

value spaces present are employed so frequently that they become indispensible

to any practical modelling construction.  These properties are identical to the

properties of some axiomatic number systems studied by mathematicians.  Some of

the more important properties for computational modelling are included below:

1.  An attribute takes a single value from a finite set of similar values

known as a value space for the particular attribute type.

2.  There is a way of assessing the similarity or difference of any values

from the same set of possible values.  This provides for the relations

equal_to, not_equal_to, between any attributes of similar type.

3.  There may be an ordering specified for the set of possible values.  In

this case, it is possible to assess the similarity or difference of any

values from the set, in terms of the relations less_than, greater_than,

equal_to, not_equal_to.  It is also possible to arrange any group of

values according to their position within the specified ordering.  It

may be that there exists more than one possible ordering over any given

value space.

4.  Given two values from the same set of possible values, there is a way

of describing their difference, which is a measure of how distant they

are from each other assuming a special ordering of the value space made

available for this particular purpose.  One difference can be produced

for every pair of values in the value space.

5.  A difference can potentially be either a signed difference or an unsigned

difference.  A signed difference indicates a direction, and a magnitude,

whereas an unsigned difference has a magnitude only.  Some value spaces

do not have a signed difference defined for them.

6.  The difference between two values from the value space is itself a value

either of the same value space, or of another value space made available

for the particular purpose of expressing differences.

7.  Every value in a value space is associated with an ordinal number, which

gives the position in the value space at which the value occurs, assuming

the value space to be ordered according to an ordering made available for

just this purpose.

8.  Every value space is finite, and there is a value space which provides

values capable of describing the size of all other value spaces.  There

is a natural ordering specified for this size descriptive value space.

9.  There is a value space which contains all other value spaces, over which

a natural ordering is defined.  This natural ordering defines the natural

ordering for every contained value space.

There are many other properties which may be required in the practical solution

of computational models, but these properties give the essential features of a

value space necessary to support useful modelling.  The essential point to see

is that values can be thought of as being associated with a continuum which

supports the notions of closeness and distance.  These notions are vital for

the construction of computational models as we will soon appreciate.

3.10  Simple values, collected values

====  ===============================

A computational model consists of a collection of attributes, each of which

associates with a single value at any given moment.  Attributes may collect

together as objects.  In turn, objects may be associated with other objects.

This provides the ability to model collections of real world objects each of

which has relations with other real world objects.  For the real world object,

there will typically be a set of characteristics which describe that object.

Each of the charateristics can usefully be modelled by an attribute of a type

appropriate to the values associated with that particular characteristic.  As

an example, consider a model of a real world gas pipeline.  The properties of

interest might be as follows:

1.    [m]    Physical dimensions.

2.    [s]    Maximum service pressure.

3.    [m]    Chemical compatibility.

4.    [m]    Maximum flow rate data.

5.    [m]    Pressure drop / flow rate characteristics.

6.    [s]    Coefficient of expansion of a pipe segment.

7.    [m]    Optimum economic pressure drop.

8.    [s]    Date of commissioning.

9.    [s]    Major inspection interval.

10.   [m]    Results of previous inspections.

11.   [m]    Welding inspection certificate reference numbers.

These values are either simple values or a collection of a number of values.

For an example of simple values see (2,6,8,9) above.  The remaining attributes

collect together as multi-attributed objects which each describe what is in

effect, a single value of complex internal structure.

We can see that for models of practical interest, there must be a means for

collecting individual values together to represent multivalued aspects of the

modelled reality.  In similar fashion, it must be possible to collect together

objects into groups, which may collectively appear to behave as an object with

complex internal structure.

We have not yet considered how the representation of collections of attributes

might be possible in a computational setting.  To do this, we must consider an

aspect of computational modelling which we have so far ignored.

Until now, we have considered that processes are available for the manipulation

of information.  However, apart from suggesting that information is ultimately

represented as simple values selected from an appropriate value space, we have

not considered exactly how that information might be held in a computer system.

3.11  Models with memory

====  ==================

All models consist of constant components which never change in value, together

with variable components which do change in value, without which no model could

be used to support computational problem solving.  The fact that some values

always remain constant leads to the question of how it might be possible to

model constancy.  In fact we can conveniently manage this by means of the piece

of hardware described above as a memory unit.  A memory unit can store a value

without that value changing, no matter how much time has elapsed before the

request to retrieve the value is given.  Furthermore, no matter how many times

the value is retrieved, it does not change.  For this reason, a memory unit is

vitally important in any computational modelling activity.

Similarly, by using the properties of a memory unit in an appropriate manner,

we can make the memory preserve the value of attributes which themselves make

association with different values from moment to moment during the modelling

activity.  To see how this can be made to work, we must look in detail at the

manner in which a memory unit operates.

3.12  The memory store operation

====  ==========================

In order to get a memory unit to store a particular value, it is necessary to

provide a special value known as an address, which will be used by the memory

to give a unique identity to the value actually being stored.  So to store a

value, we first choose an address from the value space of addresses, then give

the memory an instruction to associate the address with the value to be stored.

The operation engaged in by the memory, in order to form this association, is

termed a store operation.  The instruction which causes the operation to be

carried out is termed a store instruction.

The association between an address value and the stored value during a store

operation is maintained for all time, until changed.  But before we consider

how such an association might be changed, we will see how a value stored by a

memory may be retrieved for further use outside the memory.

3.13  The memory load operation

====  =========================

A memory unit responds to two types of instruction.  We have seen above how the

store instruction can change the way an association is remembered by the memory

unit.  There is another instruction which is termed a load instruction.  This

results in the memory performing a load operation.  For a load operation to be

operate successfully, the memory unit must be presented with an address value

which must have been given at the time a store operation was made.  Given that

a store did actually take place using that address, the memory unit retrieves

the value.

This may not sound very useful, since it would appear that, in order to access

any stored information from a memory unit, a unique value must be associated

with every number stored.  In this way, it would seem that we would then have

to somehow remember the same number of address values as the number of values

stored in the memory.  This is of course a totally impractical situation.  We

will see shortly how a memory unit may be usefully employed without having to

remember endless quantities of address values.

We have seen how a store instruction provides the address and value association

which can then later be used to retrieve the value given the address.  We have

not seen how we might change an existing address and value association, so that

a model may be made of a value which changes from moment to moment during the

modelling process.

3.14  Modelling constants, variables

====  ==============================

If we fail to ever make an association between an address and a value, using a

store instruction, any load operation conducted using that address will return

a value whose nature is unpredictable.  A value is returned, but it will be of

a random nature.  If we perform a store operation, then retrieve the value by

using the same address in a load operation, the original stored value will be

returned.  If however, we perform two consecutive store operations to the same

address, using different values on each occasion, then perform a load using the

same address, the value returned will be the latest value stored.

In this way, a memory can be made to keep a value for as long as required, then

change it by means of a new store operation using the appropriate address.  In

this way, a memory unit can be made to model constancy and change.  The store

operation determines whether a load operation at a given address will retrieve

a constant value, or a variable value.  If only one store operation is made to

an address, then the value stored will always have the same value, so will be

constant.  Otherwise, the values stored to an address may be different from one

another, so any associated load operations will retrieve the changed value on

each occasion.

3.15  The problem of addresses

====  ========================

We noticed above that a memory unit would not be very practical if for each of

the stored values, it would be required to keep a note somewhere of the address

by means of which it might be retrieved.  We will now see why this difficulty

does not occur in practise.

A model is not something which remains static.  It is used to solve problems,

using computational activity to achieve a series of transformations of models

until a solution can be found.  The development of new interpretations of the

already available models is necessary to provide the information which will

ultimately solve the problem being addressed.

Implicit in all this is a set of instructions which dictate exactly how this

process of model refinement will take place.  These instructions can contain

constant values of direct or indirect use to the model.  An indirect use for

a constant value would be that of a memory address.  In this way, the question

of remembering addresses diminishes, since they can be included in the program

so long as they are constant in value.

In fact, we will not generally need the same number of address values as values

stored in a memory unit for other reasons which will soon become clear.  The

nature of information and therefore also of models, is that of natural grouping

into connected entities which we term objects.  Therefore, for any particular

aspect of a model, various values which are associated together, may easily be

stored close by each other in memory.  For this to work, we must have a notion

of closeness of two values.  Since an address is a value similar to any other

value, we will have the idea of closeness defined between two addresses.

In this way, we can choose a special address value representative of the close

grouping of addresses belonging to an associated group of values in memory.

This special address will be only a small distance away from any address values

assigned to values in the group.  Therefore we can apply a shorthand method of

accessing any of these related values.  This involves adding a small value to

the special address value, to yield the actual address in memory for any needed

value group member.  The special address value is generally known as a base

address, whilst the small value added to the base address is termed an offset.

3.16  Structures expressed by means of values

====  =======================================

We have seen that a memory is a vital component of any computational modelling

scheme, and that the representation of collections of values within a memory is

entirely natural.  We will now think about the way in which object collections

may be arranged into a grouping.

Any collection of values in memory will have a characteristic base address.  By

recognising that a base address is really just a value, it can be stored in a

memory just like any other value.  It is this possibility which permits groups

of objects to be formed with ease.  To form a group of objects, take the base

address of each individual component object, then store all the base addresses

as a collection of values making up a new compound object.  Given the base

address of the compound object, it becomes possible to access any value in any

of the member objects by means of judicious application of the relevant offset

values.

An important point to consider is that the number of objects sharing a given

internal structure (and thus set of associated characteristic offset values) is

usually large in a model of any complexity.  In this way, memory access using a

large base address value with a small offset value for each group member gives

a very compact representation for the final address of each value.  Since the

base addresses and offsets are stored with the instructions representing the

program, the program size is reduced as a consequence.

3.17  The program as a model

====  ======================

It should be clear by now that a program consists of instructions, and these

instructions may be closely associated in some way with constant values which

may be used in various ways.  One use for these values is as part of the model

which is being presented for solution of a problem.  Another, frequent use for

these constants is as representation of the base addresses and offset values

for accessing memory in order to retrieve constants and variable values which

represent the model proper.

Although we have not yet considered what may actually be necessary in order to

construct a program of instructions for solving a problem, we can now determine

some important properties of any program, no matter what the representation.

It should be clear that, whatever else a program is, it can only ultimately be

represented as a set of objects, each with any required internal structure, all

these objects being ultimately represented by a collection of single values.

This means that for any purposes, we can treat a program as an entity which can

itself be modelled in the same way that we build models of the real world.  The

reason that this is so important is that it enables a program to be stored in

the same type of memory unit that we use to store model values.  This means a

potential to build programs which take other programs as their data, using them

as models which can then be manipulated into forms and interpretations more

appropriate for solving certain proglems.  As an example of this type of model

manipulation, where the model is really a program, consider the translator for

a programming language.

3.18  The simplest operations for computation

====  =======================================

We will now look in some detail at the types of simplest operations necessary

to support meaningful computation.  These operations will have the capability

to process information by manipulating the basic information representations

that we use for modelling.

Using these manipulations, it is possible to make new interpretations of any

particular model, or indeed any particular aspect of a given model.  The new

interpretations are themselves models, but with a different information content

which may be either more specialised or more generalised than the source model

information.  In any event, it will be easier to obtain the answers to certain

questions when using the new model as compared with the original model.  This

fact provides the motivation for the creation and derivation of new models from

existing, but somehow deficient models, assuming a particular purpose has been

specified.

It is necessary to provide a model with new values from outside the model.  The

need for this is satisfied by an instruction which performs an input operation,

using the input unit described above.

In similar manner, an output instruction is made available, so that the output

unit may be commanded to deliver a new value to the computational environment.

We have already considered that store and load instructions are needed in order

to provide the memory unit with a means to function.

The processing of data requires a number of different instructions.  It must be

possible to introduce a new constant value into a computation.  Any given value

lies next to two nearest neighbouring values in the value space of which it is

a member.  Each of these nearest values must be easily and directly computable,

so that the notion of closeness may be directly used in modelling.  This can be

particularly important for the programmed manipulation of addresses, and in any

situation where counting is needed.  It should be possible to determine the

distance between two values; given a value and a difference, the value giving

the difference should be obtainable.  These operations correspond with addition

and subtraction.

Finally, it should be possible to control the proress of the computation.  This

means being able to take decisions dependant upon the values present from time

to time in the model.  In this way, the immediate progress of a computation may

be conditional upon situations detected in earlier parts of the computation.

3.s   Summary

===   =======

We have examined the operation of a computer system in order to find out what

it is that a computer actually does.  We have seen that computation can most

usefully be thought of as a modelling activity.  The motivation for modelling

is always as a means by which solutions can be found for what would otherwise

be difficult or impossible problems.

We have seen that there are essentially two types of models, and that the first

type is usually implicitly present in the second.  We have examined attributes

and their values, and means for combining attributes together to form objects.

The properties of values and their associated value spaces were described.  The

importance of the underlying structure of a value space was emphasised since it

is fundamentally important for supporting computation.

Memory units are seen to be the essential modelling vehicle for permitting the

simple expression of both constants and variables.  The concept of an address

value was introduced, with the consequent implications for representation of

programs and the modelling of objects.

We have seen that a program is itself a real world reality which may therefore

be modelled.  The resulting interesting properties of programs were described.

Finally, we examined the types of instruction which might be important for the

exppression of a program.  This concludes our overview of computation.

4.    Visual modelling

==    ================

4.01  Complexity control and abstraction

====  ==================================

In any modelling activity, there will be layers of structures built one upon

the next.  This is an important feature of modelling, since it implies that

certain details associated with a given object will be hidden from view as a

consequence of their presence within component objects of that object.  This

has the pleasant effect of enabling the component object properties to be

assumed present by abstraction, at the level of the given object.  In this way,

a complex model may be built up by combining simple component objects, or could

equally easily be built from the top down by assuming abstract properties for

needed components.

This proliferation of complexity within useful models does present certain

problems.  For example, there may be many possible ways of interpreting any

particular properties of a model.  This means that the ways we look at the

models we create must be flexible enough to accomodate the essential whilst

ommitting all unimportant detail.

Historically, when dealing with purely numeric models involving formulae which

relate different variable values together, it has been very useful for the good

understanding of such a model, to present graphs showing how under some assumed

constraints, two or more variables change with respect to each other.

It is easy to see that by visual means, a complex situation can be described

satisfactorily in terms of a two dimensional graph.  Sometimes, where an extra

dimension of change is involved, for example time, a sequence of graphs may be

the ideal representation.  Electronic engineers have embodied these principles

in devices such as the oscilloscope and logic analyser.

4.02  Human interaction with a computer system

====  ========================================

There are some special types of modelling situation where a drawing may include

multiple layers, each of which correspond to a level of design abstraction.  As

an example of this type of modelling system, one can consider the hierarchical

design systems with which silicon integrated circuits are designed.  In these

types of systems, much detail must be represented in a physically small screen

viewing area.  In addition, it must be possible to quickly change the scale at

which a design is presented, so that fine detail is clearly visible and more

importantly, manipulable.  This is an example of a direct visual model used to

represent a device which does not (yet) have any existance in the real world.

Typically, the user creating a drawing needs to be able to observe the existing

state of the drawing, and then must be able to select a portion of the drawing

and manipulate it.  It is necessary to be able to create new detail and to move

existing detail in the drawing.  For these types of operations which involve a

very precise spatial input from the user, as observed on the displayed image,

the mouse input device has proven very useful.  As an alternative to the mouse

device, a graphics tablet is also a powerful pointing device, with high useful

resolution and good positional accuracy.  Both these types of pointing device

provide for great sensitivity in pointing to a drawing feature, and therefore

enable modifications to be affected with ease.  The graphics tablet has a great

advantage over the mouse when input from an existing drawing is required.  The

tablet has excellent positional accuracy and repeatability, so can be readily

used to digitise an existing drawing into a computer system.  A mouse cannot

operate in this way, as it is a relative motion device with no capability for

absolute positional input.

4.03  Display quality issues

====  ======================

Usually, where a large amount of relevant detail must be shown on a drawing, it

it useful to inhibit the display of some detail, only enabling it to appear on

request.  In this respect, different colours used to highlight the significance

of important details can be advantageous in reducing any ambiguity in a crowded

drawing.  The number of possible colours available for display depends upon the

image generator and the display unit quality.  The image generator may generate

analogue signals with considerable colour information available, but unless the

display unit can accept this full colour bandwidth, the display colour palette

will remain limited.  This may result in some colours appearing to be identical

with each other, even though they are supposed to be different.

The ultimate capability of a displayed image is expressed in terms of the pixel

count in each screen dimension.  This depends on the image generation hardware

and the display unit.  The highest displayed spatial frequencies are limited by

the bandwidth of the output signal from the image generator and by the response

to that signal by the display unit.  In addition, there may be a physical limit

on image spatial resolution due to a shadow mask associated with the display.

When a view is changed by expanding the level of detail, or perhaps by changing

the scale at which it is displayed, it is important that the new image appear

without undue delay.  This is important to ensure continuity of concentration

for the person manipulating the design.  There is usually a trade-off between

the speed of update of an image and the level of detail actually displayed.  If

there is a great deal of image information, in general, update speeds will not

be as great as for a lower resolution image display.

In order to provide a display space which can be readily programmed to provide

high quality images within a range of different modelling requirements, it is

very important that the aspect ratio of individual display pixels be either

exactly 1:1 or very close indeed to this ideal ratio.  If aspect ratios other

than 1:1 are employed, it can be very difficult to obtain good visual results

when modelling fine detail, especially where curves must be presented.  In the

case of a circle, an aspect ratio of 1:2 will produce an effect not unlike that

of a broad nib fountain pen.  That is, the near vertical portions of the circle

will appear razor thin, whilst the near horizontals appear thick and blocklike.

The screen image must be absolutely rock steady, since any instability over a

prolonged period of time would be absolutely intolerable for someone engaged in

serious work.  This means that high quality design must be a prerequisite for

both the image generation hardware and the image display hardware.  In addition

the production and test procedures employed by the manufacturer have a vital

role to play in the production of a displayed image which is a pleasure to use.

4.04  Context representation

====  ======================

It is usually the case that when a change is contemplated to a visual model, it

must be considered in terms of at least two distinct views.  Often, it will be

necessary to consider many more than this number of separate views.  To see why

this is so, consider the common situation where a connection must be made from

one point on an image to a second, distinct point.  There are immediately two

regions of interest within the image.  If the displayed image is an uppermost

viewing level, then it may be prudent to take an enlargement about each of the

two points to be connected.  This means there will be three separate views of

the same image.

Practical considerations may mean that considerably more views are needed in

the performance of a particular manipulation.  Normally however, these views

are not needed concurrently.  As the manipulation proceeds (perhaps connecting

a line between points at different levels in a design hierarchy) the context

changes, so a previously necessary view can be relinquished, whilst a new view

is originated.  In this way, the visual presentation of the design is extremely

dynamic.  This need for multiple views can have an important impact on the way

a visual interface is designed to operate.  It must be easy and time efficient

to create and change multiple views into a design database.  One consequence of

displaying multiple views is the need to be able to place pairs of views one

upon another.  That is, it must be possible to choose the portion of one view

which is for the moment covered by a second view.  The implication is that any

view must be freely moveable within the display area.  In addition, it must be

possible to change the overlap order of views so that the obscured portions of

each view are exactly those parts that are visually unimportant.

4.05  Building blocks of a visual modelling scheme

====  ============================================

The building blocks of a visual modelling scheme are separable into two parts,

the visual context component, and the visual object component.

Visual context is concerned with the spaces available within a display screen

for presentation of a visual image which is bound to one aspect of the model

concerned.  When the visual context is changed, the displayed images change as

a consequence of overlap or of hiding, but the information which is visible in

any given context remains unchanged.  Visual context is concerned with the way

the display area of a screen is arranged so as to provide the maximum amount

of immediately visible, relevant information.

Within the display area associated with a given visual context, visual objects

will be made available for display.  {If any area of a context is not visible,

this will be as a consequence of the overlap of a number of contexts, with the

result that some are subjected to partial or complete hiding.}  These objects

will themselves be of two principle forms.   Geometric shapes comprise the main

element of visual modelling schemes, with text providing essential additional

information.

Geometric elements are provided for all the main families of geometric shape

which are likely to occur in real world objects.  Such shape families include

the square, the rectangle, the circle, the ellipse and the triangle.  Lines

are available for purposes of connecting any two points.

Text may be provided in various sizes, to suit the hierarchical level at which

the text associates with a design.  For example, in a silicon chip design, the

top level functional blocks will often employ large text with which to express

their names.  Details of interface lines between functional blocks will employ

a much smaller text, to emphasise the difference in overall importance of these

component parts.

In practise, there can be problems of representation for curves in any context

where an essentially rectilinear coordinate system is used, as is the case for

raster display systems.  The difficulty arises where line segment gradients

occur at angles antagonistic to accurate representation in a rectilinear system

of coordinates.  This happens at shallow angles to each major axis, and at the

angles which lie about one quarter of a right angle from the major axes.  These

critical angles result in jagged line segments, which are most disconcerting to

the eye.  There are means available to reduce problems of this nature, by using

an anti-aliasing technique.  This technique applies a small modification to the

displayed image by modifying the luminosity of pixels on or near the line being

processed.  The result deceives the eye into believing that a smooth gradient

is actually being displayed.  Under these conditions, the visual interpretation

becomes very close to the ideal shallow angle line segment.  However, to make

appropriate image adjustments is an extremely computationally intensive task.

4.s   Summary

====  =======

An introduction to visual modelling has been presented, showing the motivation

and historical background which preceeded computer based imaging systems.

The visual consequences of modelling with hierarchy are examined with a view to

understanding the design issues which must be addressed in ensuring that the

display system is capable of practical use in solving real problems.  Devices

available for closing the visual feedback loop by providing graphical input are

examined in terms of their comparative properties.

Some of the more important technical issues affecting displayed image quality

are reviewed.  This gives insight into motivations at work during the technical

design of a real display system.

The notion of visual context is introduced, with a discussion of the necessary

supporting manipulations required to enable control of visual context during

practical interaction with a visual modelling system.

Finally, the building blocks needed in order to construct a visual modelling

scheme are examined.  A discussion of some of the difficulties relating to use

of a rectilinear coordinate system in the representation of curves includes

mention of a technique for atonement of the problem.

5.    Display device characteristics

==    ==============================

5.01  Raster scan display systems

====  ===========================

There are a number of possible technologies used for producing high quality

visual images from computer produced data.  We are concerned with only one of

these technologies, raster scan display technology.  We will not mention any

of the other available systems, since we are concerned with video graphics

array controller chip programming, which implies using a raster scan display.

A raster scan display system is very simple in concept.  The generated visual

image is considered to be composed of a large number of lines of visual data,

with the orientation of each line being essentially horizontal.  One line is

displayed after another, so that successive lines are stacked vertically one

upon another, with minimal vertical separation.  In this way, a rectangular

image space is built up.  In order for this to work, the computer system must

provide data suitable for describing the visual content of each neighbouring

line, one after another, until each line has been described and the image is

complete.

We can think of a line as being composed of many individual picture elements,

each of which represent in some way, the smallest amount of information that it

is possible for a display to present.  A picture element is usually referred to

as a pixel, which is essentially a contraction of picture element, following

the fashion for contracting pictures to pics (or pix) and picture to pic.

A pixel has a number of useful properties.  It is entirely separate from any

other pixel, which means that any characteristics it possesses can be changed

independantly of the characteristics of any other pixel.  In this manner, we

can determine the resolving power of a display by means of a statement of how

many pixels the display can support during operation.  This is termed the

resolution of the display.  However, visual images are generated outside the

display itself, so the resolution of the image generator must be taken into

account when deciding the achievable resolution for the entire visual system.

The important physical properties of a pixel are those of size, aspect ratio

and colour capability.  A small pixel will pack more tightly into a given line

length therefore giving greater resolution for the presented image.  The aspect

ratio should ideally be 1:1 as noted above.  This gives best results using a

rectilinear coordinate system.  The colour capability of a pixel is expressed

as hue and luminosity.  Hue describes the mixture of the primary colours used

to generate a displayed colour, whilst luminosity is a measure of the strength

of the colour.

5.02  Raster scan image refresh

====  =========================

A pixel can be given exactly the properties required by means of descriptive

data sent to a display unit from a computer system.  However, a display unit

does not itself possess any memory for the characteristics given to a pixel in

this way.  That is, the display system can display a pixel according to data

describing that pixel, but that data is not remembered anywhere in the display

unit.  The effect is that a single image can be produced, which then starts to

fade away, until it becomes quite invisible.

Therefore, when building an image using a raster scan display system, we have

only to present the display device with an endless stream of data, composed of

the information we need to completely describe the characteristics of each and

every pixel, on a moment by moment basis.  The need to supply data to a display

on a continuous basis, arises because the display can only present a displayed

image for a small time before the image begins to fade.  However, if we make

the display data continuous, the image produced will not have time to begin to

fade, so will remain strong.  The fade time of an image determines the delay

before the image must be refreshed, this time decided by the chemicals used in

making the phosphor in the cathode ray tube.  {In fact, there is a short term

memory in a display unit, which is the result of a phosphor taking some time to

forget the information represented by the radiation of coloured light at some

particular intensity.  This short time permits the serial presentation of data

into a display device to result in a complete image over the entire screen.}

{Note: not all raster display systems use a cathode ray tube with its attendant

phosphor fade.  This was for many years the dominant technology, and is still

the principle display tchnology used today.  Other raster display technologies

use exactly the the same signals as would a cathode ray tube system, but do not

suffer from phosphor fade.  We mention the above simply to give an insight into

the original design motivation behind repeated data presentation into a raster

scan display device.  There are other consequences of the original technology

which will become apparent in later discussions.  Most of the timing issues in

raster scan image generation relate to the original technology.}

5.03  Signals for image generation

====  ============================

The sequential generation of an image requires that individual lines of visual

data be sent to a display device, one after another, until an entire image has

been constructed.  The whole process is then repeated indefinately.  The data

which represents the individual characteristics of each pixel must therefore be

synchronised in some way with the operation of the display device.  This must

be done in order to ensure that the data for each pixel actually changes the

same physical pixel on the screen, each time it is sent to the display unit.

The display unit operates to an extent without any knowledge of the data being

sent to it from the image generator.  Once an image has started, the display

alters each pixel in a display line simply by converting the colour information

arriving from the image generator into light of specified hue and luminosity.

The physical process used by the display unit involves scanning three separate

electron beams at a known and absolutely constant velocity across the pixels

making up the line to be displayed.  The colour information from the image

generator is used to change the intensity of each of the three electron beams.

The strength of an electron beam striking a phosphor determines the intensity

of the light emitted by the phosphor.  By making each pixel from three dots of

different coloured phosphors, and arranging that each electron beam focuses on

phosphor of a known colour, it is possible to completely control the colour at

each pixel position of the display.

The image generator has a way of telling the display unit that a complete new

image is about to be transmitted, and also a way of informing the display unit

that a new line of visual information is about to be sent.  Apart from these

two synchronisation signals, the displayed image positional stability depends

completely on the ability of the two units to maintain very accurate matched

timing in their operations.  The display scans each pixel in a line at a known

rate, so the image generator must supply pixel colour information at exactly

this same rate.  In this way, so long as the timing stays the same during the

period required to display one line, the image will be positionally stable.

The signals describing pixel characteristics provided by the image generator

represent intensities which are used by the display unit to modulate light from

each of the three phosphors which make up one complete display pixel.  These

phosphors generate the colours red, green and blue.  Using the emmision colour

mixing model, it is possible to represent any desired hue and luminance by

changing the absolute intensity value of each of these component colours.  In

this way an entire colour image is formed on the display screen.

5.04  Timing issues

====  =============

For practical generation of display image information suitable for presentation

into a raster scan display device, there must be provision for synchronisation

signals.  We will now examine some of the important timing characteristics of

these synchronisation signals by looking at the display unit operation in some

detail.  We will, as before, assume a cathode ray tube based display unit.

The most important aspect of timing information in a raster scan display based

on a cathode ray tube, is concerned with the finite energy required to move the

electron beams across the surface of the display tube.  This energy is used to

create a varying magnetic field, which permeates the display tube and results

in the deflection of the electrons passing through it.  This is the essential

mechanism by means of which the pixels in a display line are scanned.  However,

there are some important consequences of this use of a magnetic field and they

affect the timing signals presented by the image generator to the display unit.

The magnetic field which sweeps the beams of electrons across the display tube

is a strong magnetic field.  This means it possesses a large amount of energy.

That energy reaches a maximum at the extreme positions of a scan line.  So at

the start of a scan line the magnetic energy is at a maximum in one direction,

whilst at the end of a scan line the energy is a maximum in the very opposite

direction.  The implication is that between the extreme right position on one

scan line and the extreme left position of the following scan line, a large

amount of energy must be safely transferred.  This cannot happen without some

time being made available specially for the purpose.

Each time the display line is advanced from the top of the display tube towards

the bottom, a similar argument applies.  A magnetic field possessing energy is

used to direct the electron beams at right angles to those used for the pixel

scan along each line.  But the energy argument is just as valid in the vertical

direction.  Therefore in order to finish the last display line in the image it

is necessary to provide a time delay so that the magnetic field can be set up

ready for the first line in the display.  As before, considerable energy must

be transferred, which accounts for this requirement.

During the flyback period, which can be either line flyback or frame flyback,

the electron beams would normally re-traverse the screen, which would cause

difficulties with image interference.  This would be the result of any double

exposure of pixels to the electron beams.  In order to prevent this possibility

all the electron beams are inhibited during a flyback period.  This is termed

line blanking and frame blanking respectively.

The circuitry which controls the horizontal and vertical scanning uses a method

of improving the absolute positional accuracy of the electron beam deflection

called overscanning.  An overscan is the result of applying too much deflection

to the electron beams to the extent that they no longer strike the surface of

the screen.  Instead, they strike an interior portion of the display tube, and

do not generate any image as a consequence.  A difficulty arises when the beams

start to strike the display screen resulting in image generation.  The result

is an image which appears ragged, since the boundary between the non-displaying

and displaying part of the screen may not be well defined.  In addition, there

is the need to establish unambiguous synchronisation between the display unit

and the image generator.  This cannot be readily achieved by using the edge of

the screen as a guide to determining where each scan line should commence.

To combat this problem, horizontal blanking is used to switch off the electron

beams at the start and end of each horizontal scan line.  In this way some part

of the display screen remains unused, but the result is a much more stable and

accurately positioned display.  The horizontal synchronisation signal is used

to specify the exact timing characteristics of this special blanking.  It can

usefully be thought of as an extension of the line blanking due to flyback.

There is an identical extension to the vertical synchronisation signal for the

same reasons.

5.05  Image generation can be dangerous!

====  ==================================

The timing of the horizontal and vertical synchronisation signals is determined

completely by the image generator, but although this is so, these signals must

be interpreted correctly by the display unit if the intended image is ever to

be produced.  This means that there is potential for an image generator to send

visual signal information to a display unit, which that display unit cannot use

to produce an image.

It is important to see why this may be so, since if such signals are sent to a

display unit, one possible consequence is that permanent damage to the display

may result.  Obviously this situation is extremely undesirable, and likely to

be very costly if it does occur.  This is one of the rare situations where the

incorrect use of software can result in destruction of hardware.  {We refer to

industry standard systems only in making this remark.  There are large classes

of systems termed embedded systems where the coupling between hardware and the

software which controls it is so close, that this potential for destruction is

ever present.  Such systems are only programmed by experts as a result.}

A display unit has the important characteristic that it will correctly accept

synchronisation and image generation signals upto a certain maximum frequency.

This means that in a given time, only a certain quantity of information may be

accepted by the display unit from the image generator unit.  If this maximum

frequency is exceeded, the circuitry within the display unit will not function

correctly and may be damaged as a result.  This can happen because the quantity

of energy used in controlling the electron beam scanning is very high, making

it essential that electronic devices capable of withstanding high stress are

used in the circuitry.  Moreover, to improve display device reliability, energy

conservation circuits are introduced into the display unit.  This measure means

that the display circuits dissipate less heat and therefore operate at a lower

temperature than would otherwise be the case.  However, in order to for these

circuits to operate correctly, certain important assumptions must be made about

the frequency of operation.  This is in essence why it is possible to damage a

display device by feeding it with image information at too high a frequency.

5.06  Persistance in human vision

====  ===========================

The frequency of the image information presented to a display unit determines

the maximum image resolution of the resulting display, assuming an acceptable

match exists between the image generator and the display unit.  That is, the

greater the number of pixels to be displayed, the more information is required

for each single image displayed.  We term each of these images a frame.  More

information in a given amount of time results in a higher frequency.  We would

like to keep the image information frequency as low as possible, since this

helps to keep the display system quality high and the cost low.  This can only

be done by making a compromise.  We will now examine how this can be achieved.

We have seen above how it becomes necessary to repeat each frame at such a rate

that the resulting display image becomes constant.  We have seen how this can

be determined by the image persistance of the phosphors representing a pixel.

There is another important aspect of image persistance which we have not yet

considered.  The human eye itself has properties of image persistance.  So it

is possible for a human eye to provide an image for some time after the image

has vanished.  This is the important property which gives projected cinema film

the realism which makes cinema films possible.  In projecting a cinema film, a

single image is flashed onto the screen for a fraction of a second, then the

light illuminating the image is removed, the image is then replaced by the next

in sequence.  This whole process repeats many times per second.  Persistence of

vision is the means by which the cinema screen seems to give a steady portrayal

of stationary scenes and smooth representation of movement.

This same persistance is at least partly responsible for our ability to see the

image presented by a display unit as a single continuous image.  We know that a

phosphor also exibits persistance.  An interesting question therefore arises.

How many frames per second must we display in order to minimise the information

sent to a display unit, whilst still maintaining an acceptable displayed image?

This question is absolutely vital in determining the highest frequency that a

display unit must operate at, given a particular image resolution.  This whole

issue determines the cost of the display unit and the quality of the displayed

image.  It therefore also determines the characteristics of the image generator

which must produce the information for creating the image.  So it is a really

pivotal question for a visual display system.

A cinema projector usually operates at 24 frames per second, and at this frame

frequency a high quality image is perceived.  At lower frame frequencies, there

may be image flicker, which after some time can become very unpleasant.  It is

important to eliminate flicker so that a display system image can if necessary,

be observed for prolonged periods of time without creating undue stress.

A display unit frame is not directly comparable with a cinema film frame.  This

is so because a display unit produces an image by using scanned electron beams,

whilst the cinema film frame is a complete image in its own right.  This means

that in order to give the same freedom from perceived flicker, a display unit

must produce more frames per second than does a cinema projector.  In practise

a display unit will usually attempt to operate at a frame frequency of at least

40 frames per second.  Some display units operate at over 90 frames per second,

which results in an extremely stable, high quality image.  Of course, to run at

the higher frame rate means much more image information must be generated, so

the complete cost of the display system will be very much higher than a system

which uses a low frame rate.

5.07  Interlaced display

====  ==================

It is important to keep the frame rate of a display system as high as possible

in order to produce a good quality image which remains flicker free.  At the

same time, we wish to keep the information frequency as low as possible so as

to minimise the cost of the display system.  We will now examine an interesting

compromise which attempts to obtain the best of both situations by using what

really amounts to a clever visual trick.  This trick just happens to be simple

to implement using raster scan display technology, hence is of practical merit

in some types of display systems.

The trick is based on the observation that apparent flicker can be reduced when

two almost identical images are displayed one after another, but in a special

physical relationship with each other on the display screen.  All single images

are composed of a certain number of raster scan lines.  It is possible to take

a single image and make two separate images from it, by taking alternate scan

lines from the full image.  This places all even numbered scan lines in one of

the derived images, with all the odd numbered scan lines making up the second

image.  These derived images are then each composed of approximately half the

number of scan lines appearing in the original image.  In this way each derived

image is represented by approximately half the visual information needed in the

original image frame.  {We say approximately half, since we did not specify the

number of scan lines in the original image, which may not be divisible into two

identical parts.}

We can then display these two derived image frames alternately, which means the

visual information frequency is roughly halved, whilst the frame rate remains

constant.  Alternatively, the visual informatin frequency can remain constant,

whilst the frame rate is doubled.  To do this, the display unit interlaces the

two images, which means that physically, on the screen, the original image is

reconstituted exactly as it was prior to being dismembered.  Obviously in order

for this to work, the display unit positional error must be very low.  However,

this is a requirement of any display system, so does not present tremendous

difficulty.  In this way, flicker is reduced, whilst a reasonable quality image

is produced by the display system.  As a result the apparent display resolution

can be increased by a factor of two, without too much increase to the cost.

For the very highest quality display presentation, it is better not to use this

technique, since the ability of the display unit to correctly reconstitute the

image will leave something to be desired.  However in some situations interlace

can be useful.

5.08  Screen border display

====  =====================

It may prove useful to display an image which is substantially smaller than the

resolution of the display unit being used.  This might be the case where some

software assumes such a smaller image area when creating a visual image.  So it

can be displayed on the higher resolution display system, some of the pixels on

the display must be effectively removed from the true image area.  In this way,

the actual image display will appear smaller than the true available image area

on the display screen.

A convenient way to manage this reduction in display image size, is to create a

border surrounding the true active display area.  In this way, those pixels not

required in the image are set to some predetermined colour, which is termed the

border colour.  We mention this possibility since later, we will see that this

capability is supported by a video graphics array controller chip.

5.09  Cursor generation

====  =================

An image display system is an output component in a feedback loop incorporating

the computer system and the person using that system.  Within a displayed image

there is one point with which the user is presently concerned.  This point will

be subject to any manipulation deemed necessary by the user.  As the resolution

of the display system increases, the quantity of visual data becomes large, so

the user requires a means of determining where the active point is situated.

The movement of an input device can be related to a cursor which specifies the

exact position of the hot spot within the image.  This cursor will be easy to

distinguish from the image proper, and for this purpose will normally blink on

and off at a rate designed to be easily seen amongst a mass of visual detail.

Since the generation of a blinking cursor by software is a very high overhead

operation, it is beneficial for the image generation hardware to provide some

facilities for cursor generation and positional control.

5.10  Multi-synch monitors

====  ====================

There is some advantage in designing a display unit (also termed a monitor, or

sometimes video monitor) so that it will accept image information at different

frequencies.  The advantage is that the full display area is used, rather than

producing a small image with an unused border as discussed above.  The display

accepts lower frequencies of image information and, as a direct consequence of

this, the apparent physical width of a pixel changes.  The result is a reduced

resolution display, but one which takes up the full display area.

For this concept to work, there are certain frequencies assumed to be in use by

the display system, so that signals from the image generator can be analysed to

determine their timing properties, and hence determine the correct operation of

the display unit.  Since the signals which give image timing are synchronising

signals, the name of such devices is multi-synch monitors.  At any moment, only

one of the possible display unit operating frequencies is matched by the timing

of the horizontal and vertical synchronising signals.  If this timing should be

changed, then the monitor adopts the new frequency after a short period.  For a

short time during the change over period, the image produced by the monitor may

be dimensionally and visually unstable.  This is quite normal with commercially

available multi-synch monitors.  However, this should not be surprising, as the

operation of the monitor is affected so fundamentally by the change from one

input frequency to another.

5.s   Summary

====  =======

The concepts associated with raster scan display systems are introduced with

the special emphasis on the use of cathode ray tubes for the display unit of

such systems.

The pixel is identified as the fundamental unit for image presentation and also

display image resolution.  Properties of pixels are explored.

The properties of a cathode ray tube based display unit are seen to embrace the

need for image refresh.  This is seen to be a consequence of the absence of any

image data memory in a display unit.

The need for synchronisation between an image generator and a display unit is

identified and related to the physical processes employed by the display unit

for image presentation.  The concept of multi-phosphor pixels is introduced,

with consequences for the data encoding of colour information.

The need for periods of inactivity between the display of neighbouring scan

lines is outlined in terms of flyback energy, with similar requirements for

the frame flyback period.  The use of special linearising circuitry for both

line scanning and frame scanning is seen to have important implications for

the horizontal and vertical synchronisation signals respectively.

The properties of display units which make them suceptible to damage by image

generators are carefully examined.

Consequences for display system design deriving from human visual persistance

are examined.  These are seen to be of seminal importance.

The interlace technique is seen as a trade off between ultimate image quality

and image information frequency, with implications for display system cost.

The use of a border area is examined as a means for using low resolution images

with a high resolution display unit.

The advantage of an image generator supported cursor is reviewed.

Multi-synch monitor are described in relation to their frequency agility with

implications for display of images at a lower resolution than that supported

naturally by the display device.

6.    Trident 8900 VGA controller: Computer interface

==    ===============================================

6.01  Memory interface

====  ================

The video graphics array controller must repeatedly transmit image information

to the display device.  Since the display device does not possess an image data

memory, that memory must be provided by the image generator.  Image generation

is intimately managed by the controller chip, so the image memory can only be

accessed through the controller chip.  In this way, we do not have to concern

ourselves with the complicated hardware details of memory access, as the memory

interface provided by the controller chip suffices.   This interface will now

be given in broad outline, but important details will be omitted for clarity.

The Intel 80x86 processor family uses a segmented memory architecture.  In the

original 8086 processor, for which most current software is targetted, the size

of a segment is limited to a maximum of length of 64 kilo-bytes.  This quantity

of memory is therefore immediately accessible without the need for changes to a

segment register value.  For this reason, the memory access interface provided

by the VGA controller requires that the processor generate addresses inside the

64 kilo-byte span of a segment.  The motivation is speed of image access.

The image generator typically controls much more memory than 64 kilo-bytes, so

that high resolution colour images can be generated.  A usual configuration at

the time of writing would be 1024 kilo-bytes of image memory.  Clearly, so that

the processor be able to read and write such a large image memory using only 64

kilo-bytes of addressing, some type of access extension mechanism is needed.

We can think of the 64K {shorthand: 64 kilo-bytes} addressing range recognised

by the VGA controller as being a window into the much larger image memory.  If

the controller is reappraised of the alignment of this window within the entire

image memory range, it is possible to read and write to the full image memory.

Of course, for this scheme to work, we would periodically have to send special

information to the controller chip, which adjusts the positioning of the memory

window, so gaining access to memory outside the 64K furnished by the currently

defined window.

In order to specify where within the full image memory span the current access

window will be situated, the concept of a page is introduced.  A page contains

two segments, each of which is 64K.  So a page represents a memory unit of 128K

bytes of image memory.  Further, two pages combine to form a bank with 256K of

image memory.  Finally, one of four possible banks can be selected, giving full

access to one megabyte of image memory.  It is important to note that the image

memory is only accessible via the VGA controller chip.  It is not possible to

directly access this memory from the processor, except insofar that the memory

interface provided by the VGA controller permits.  This is equivalent to saying

that any read or write access to the 64K window provided by the VGA controller

has an effect in video memory determined by a pre-established context which has

to be explicitly programmed for the purpose at hand.  The memory interface must

be managed by the programmer, so nothing can be taken for granted.

In fact, there are a number of complications with this scheme, which we should

expect in a subsystem as powerful as the VGA controller.  This means that later

we will encounter additional VGA controller capabilities which affect the image

memory addresses which are generated by the processor.

6.02  Input, output, port interface

====  =============================

The VGA controller is a collection of mutually supporting logic blocks, which

interact to produce the full functionality required of the controller.  Each of

the individual logic blocks can sometimes be conveniently programmed as though

it were completely isolated from all the remaining blocks, although that is not

always the case.  For reasons related to ease of chip design, each block has an

interface provided which gives input output port access to a set of internal

control registers.  These registers determine the detailed operation for each

logic block so can be used to give fine control of image generation as required

in a sophisticated image display system.

The access to large numbers of internal programmable registers by a processor

provides a problem for the hardware designer.  The idea is to provide as much

functionality as possible, without using too many hardware resources providing

access to the large number of programmable registers needed for control.

To ease this hardware design problem, which is effectively an attempt to reduce

the device complexity and cost from the hardware point of view, a trade-off is

made with respect to ease of programmability.  This trade-off is the reason why

VGA controller chips are a little strange to program.  {People who think every

means possible should be employed to make every device easier to program should

reflect that even with contemporary technology, adherance to this philosophy is

not economically possible, from the hardware design and development viewpoint

and also because of manufacturing and marketing considerations.  The truth of

this is especially visible where commodity market chips are concerned.}

We now see the typical input output port interface provided to give access to

the programmable registers within one logic block of the VGA controller chip.

Each interface uses just two input output port addresses.  This simplifies the

decoder logic overhead within the logic block.

If the logic block we are going to program is the Sequencer, the two ports we

will access lie at the following addresses:

1.  Sequencer Address Register  {  Port 3c4 hex for Read, Write }

2.  Sequencer access port       {  Port 3c5 hex for Read, Write }

The correct programming sequence for accessing an internal register depends on

each of the registers being associated with an index number.  Programmers can

think of the internal registers being arranged in an array for which an index

must be provided in order to obtain access to an individual register.  Once the

index number for the register of interest is known, that index number is output

to the Sequencer Address Register.  The sequencer will then set itself up so an

access to the Sequencer access port actually performs input or output to the

required internal sequencer programmable register.

A code fragment to perform a write to the Character Map Select Register within

the sequencer, and then to read that value back is given below.

; start example 1

; Set up Sequencer Address Register ...

; ... for access to Character Map Select Register

mov     dx,3c4h         ; Sequencer Address Register port

mov     al,03h          ; Character Map Select Register index

out     dx,al           ; select the Character Map Select Register

; Write the Character Map Select Register

mov     dx,3c5h         ; Sequencer access port

mov     al,write_data   ; collect the data to write

out     dx,al           ; write the Character Map Select Register

; Read the Character Map Select Register

mov     dx,3c5h         ; Sequencer access port

in      al,dx           ; read the Character Map Select Register

; end example 1

Whilst this example does not appear to be particularly onerous, we will see in

later examples that the number of registers which must be programmed in concert

may lead to long sequences of code which do nothing else but select and access

VGA controller registers, prior to actually performing image manipulation.  It

would be misleading to pretend that the writing of such sequences is anything

other than tedious;  however, if excellent results (or indeed any results!) are

to be had, we must face this prospect.

There is a fortunate aspect of the Intel 80x86 input output port architecture

which can help to ease the programming burden associated with VGA registers.

When a word output is performed to a particular port number, two bytes are in

fact transferred.   The first is sent to the port mentioned and the second is

sent to that port number plus one.  This has the happy consequence that it is

possible using word output, to effect the index selection with the first port,

followed by data output to the just selected register.  This can help to ease

the amount of code to be written, and is of course much faster to execute.

; start example 2

; Set up Sequencer Address Register ...

; ... for access to Character Map Select Register

mov     dx,3c4h         ; Sequencer Address Register port

mov     al,03h          ; Character Map Select Register index

; Write the Character Map Select Register

mov     ah,write_data   ; collect the data to write

out     dx,ax           ; write the Character Map Select Register

; Read the Character Map Select Register

mov     dx,3c5h         ; Sequencer access port

in      al,dx           ; read the Character Map Select Register

; end example 2

In this way, it is possible to omit one output port instruction and one address

load instruction, making a saving of 2 instructions per 6 original.  This is a

very useful saving.  Useful execution speedup is also obtained in consequence.

6.03  Complete input, output port accessible register list

====  ====================================================

The registers are listed with a document reference number which is numeric only

or preceeded by an X.  The numeric only register reference numbers describe the

standard VGA registers.  Those beginning with an X, describe extended registers

which may be specific to a particular implementation, in this case, the Trident

8900 video graphics controller chip.

The port address is some instances contains a ? indicating that an alternative

port address can be made available under program control.  This choice will be

between the hex digit 'b' for monochrome emulation or 'd' for colour emulation.

The port address shown as 3?5h would be either 3b5h or 3d5h depending upon the

value of the status bit <0> in the Miscellaneous Output Register.

In some instances, the access definition for a particular register will change

for read and write.  Sometimes, for example, a different port is used depending

on whether a value is being written to, or being read from, the register.  This

difference will be highlighted by the appearance of two descriptive lines, with

the suffixes a,b indicating a closely related definition.

Note that not all port addresses are used.  This is also true of the register

index numbers used to access the internal arrays of registers.  In most cases

the hardware defines some internal datapath for these unused register indexes.

They are used only for testing purposes, if at all.  They are not significant

with respect to programmable functionality, so should not be accessed.  Where

a break in contiguous index numbers occurs, this is emphasised by means of a

dummy line, which breaks the pattern of smoothly increasing index numbers.

Document Reference

===---------------

Index Port Address

===    ====--------------

Access Port Address

===    ====    ====---------------

Index Number

===    ====    ====    ===---------

Read/Write Access

===    ====    ====    ===    ===--------------

Register Description

===    ====    ====    ===    ===    ====================

1    3c4h    ----    ---    R/W    Sequencer Address Register

2    3c4h    3c5h    00h    R/W    Reset Register

3    3c4h    3c5h    01h    R/W    Clocking Mode Register

4    3c4h    3c5h    02h    R/W    Map Mask Register

5    3c4h    3c5h    03h    R/W    Character Map Select Register

6    3c4h    3c5h    04h    R/W    Sequencer Memory Mode Register

----    ----    ---    ---

X 1    3c4h    3c5h    0bh    R--    Hardware Version Register {new definition}

X 2    3c4h    3c5h    0bh    --W    Version Selector Register {old definition}

----    ----    ---    ---

X 3    3c4h    3c5h    0ch    R/W    Configuration Port Register 1

X 4    3c4h    3c5h    0dh    R/W    Mode Control Register 2

X 5    3c4h    3c5h    0eh    R/W    Mode Control Register 1

X 6    3c4h    3c5h    0fh    R/W    Power-up Mode Register 2

7a   3b4h    ----    ---    R/W    CRT Controller Address Register

b   3d4h    ----    ---    R/W    CRT Controller Address Register

8    3?4h    3?5h    00h    R/W    Horizontal Total Register

9    3?4h    3?5h    01h    R/W    Horizontal Display Enable End Register

10    3?4h    3?5h    02h    R/W    Start Horizontal Blanking Register

11    3?4h    3?5h    03h    R/W    End Horizontal Blanking Register

12    3?4h    3?5h    04h    R/W    Start Horizontal Retrace Pulse Register

13    3?4h    3?5h    05h    R/W    End Horizontal Retrace Register

14    3?4h    3?5h    06h    R/W    Vertical Total Register

15    3?4h    3?5h    07h    R/W    CRT Controller Overfow Register

16    3?4h    3?5h    08h    R/W    Preset Row Scan Register

17    3?4h    3?5h    09h    R/W    Maximum Scan Line Register

18    3?4h    3?5h    0ah    R/W    Cursor Start Register

19    3?4h    3?5h    0bh    R/W    Cursor End Register

20    3?4h    3?5h    0ch    R/W    Start Address High Register

21    3?4h    3?5h    0dh    R/W    Start Address Low Register

22    3?4h    3?5h    0eh    R/W    Cursor Location High Register

23    3?4h    3?5h    0fh    R/W    Cursor Location Low Register

24    3?4h    3?5h    10h    R/W    Vertical Retrace Start Register

25    3?4h    3?5h    11h    R/W    Vertical Retrace End Register

26    3?4h    3?5h    12h    R/W    Vertical Display Enable End Register

27    3?4h    3?5h    13h    R/W    Offset Register

28    3?4h    3?5h    14h    R/W    Underline Location Register

29    3?4h    3?5h    15h    R/W    Start Vertical Blanking Register

30    3?4h    3?5h    16h    R/W    End Vertical Blanking Register

31    3?4h    3?5h    17h    R/W    CRTC Mode Control Register

32    3?4h    3?5h    18h    R/W    Line Compare Register

----    ----    ---    ---

X 7    3?4h    3?5h    1eh    R/W    CRTC Module Testing Register

X 8    3?4h    3?5h    1fh    R/W    Software Programming Register

----    ----    ---    ---

X 9    3?4h    3?5h    22h    R--    CPU Latch Read Back Register

----    ----    ---    ---

X10    3?4h    3?5h    24h    R--    Attribute State Read Back Register

----    ----    ---    ---

X11    3?4h    3?5h    26h    R--    Attribute Index Read Back Register

33    3ceh    ----    ---    R/W    Graphics Address Register

34    3ceh    3cfh    00h    R/W    Set/Reset Register

35    3ceh    3cfh    01h    R/W    Enable Set/Reset Register

36    3ceh    3cfh    02h    R/W    Colour Compare Register

37    3ceh    3cfh    03h    R/W    Data Rotate Register

38    3ceh    3cfh    04h    R/W    Read Map Select Register

39    3ceh    3cfh    05h    R/W    Graphics Mode Register

40    3ceh    3cfh    06h    R/W    Miscellaneous Register

41    3ceh    3cfh    07h    R/W    Colour Don't Care Register

42    3ceh    3cfh    08h    R/W    Bit Mask Register

----    ----    ---    ---

X12    3ceh    3cfh    0eh    R/W    Source Address Register

X13    3ceh    3cfh    0fh    R/W    Source Address Enable Register

43    3c0h    ----    ---    R/W    Attribute Address Register

44a   3c0h    3c0h    00h    --W    Palette Register 00

b   3c0h    3c1h    00h    R--    Palette Register 00

45a   3c0h    3c0h    01h    --W    Palette Register 01

b   3c0h    3c1h    01h    R--    Palette Register 01

46a   3c0h    3c0h    02h    --W    Palette Register 02

b   3c0h    3c1h    02h    R--    Palette Register 02

47a   3c0h    3c0h    03h    --W    Palette Register 03

b   3c0h    3c1h    03h    R--    Palette Register 03

48a   3c0h    3c0h    04h    --W    Palette Register 04

b   3c0h    3c1h    04h    R--    Palette Register 04

49a   3c0h    3c0h    05h    --W    Palette Register 05

b   3c0h    3c1h    05h    R--    Palette Register 05

50a   3c0h    3c0h    06h    --W    Palette Register 06

b   3c0h    3c1h    06h    R--    Palette Register 06

51a   3c0h    3c0h    07h    --W    Palette Register 07

b   3c0h    3c1h    07h    R--    Palette Register 07

52a   3c0h    3c0h    08h    --W    Palette Register 08

b   3c0h    3c1h    08h    R--    Palette Register 08

53a   3c0h    3c0h    09h    --W    Palette Register 09

b   3c0h    3c1h    09h    R--    Palette Register 09

54a   3c0h    3c0h    0ah    --W    Palette Register 10

b   3c0h    3c1h    0ah    R--    Palette Register 10

55a   3c0h    3c0h    0bh    --W    Palette Register 11

b   3c0h    3c1h    0bh    R--    Palette Register 11

56a   3c0h    3c0h    0ch    --W    Palette Register 12

b   3c0h    3c1h    0ch    R--    Palette Register 12

57a   3c0h    3c0h    0dh    --W    Palette Register 13

b   3c0h    3c1h    0dh    R--    Palette Register 13

58a   3c0h    3c0h    0eh    --W    Palette Register 14

b   3c0h    3c1h    0eh    R--    Palette Register 14

59a   3c0h    3c0h    0fh    --W    Palette Register 15

b   3c0h    3c1h    0fh    R--    Palette Register 15

60a   3c0h    3c0h    10h    --W    Attribute Mode Control Register

b   3c0h    3c1h    10h    R--    Attribute Mode Control Register

61a   3c0h    3c0h    11h    --W    Overscan Colour Register

b   3c0h    3c1h    11h    R--    Overscan Colour Register

62a   3c0h    3c0h    12h    --W    Colour Plane Enable Register

b   3c0h    3c1h    12h    R--    Colour Plane Enable Register

63a   3c0h    3c0h    13h    --W    Horizontal PEL Panning Register

b   3c0h    3c1h    13h    R--    Horizontal PEL Panning Register

64a   3c0h    3c0h    14h    --W    Colour Select Register

b   3c0h    3c1h    14h    R--    Colour Select Register

65a   3c2h    ----    ---    W--    Miscellaneous Output Register

b   3cch    ----    ---    --R    Miscellaneous Output Register

66    3c2h    ----    ---    R--    Input Status Register Zero

67    3?ah    ----    ---    R--    Input Status Register One

68    3c3h    ----    ---    R/W    Video Subsystem Enable Register

69    46e8h   ----    ---    R/W    Display Adapter Enable Register

70    3c6h    ----    ---    R/W    DAC Pixel Mask Register

71    3c7h    ----    ---    R--    DAC Status Register

72    3c7h    ----    ---    W--    DAC Read Data Address Register

73    3c8h    ----    ---    R/W    DAC Write Data Address Register

74    3c9h    ----    ---    R/W    DAC Data Register

This concludes the list of programmable registers for the VGA controller chip.

6.04  Register specifications: Sequencer

====  ==================================

6.04.01  Sequencer Address Register      { 3c4h    ---    R/W }

=======  ======================================================

<3..0>   Sequencer Register Address

--------------------------

Port 3c5h accesses the register pointed to by the index number in this

bit field.  Index number zero is represented by <=0000>.

This register is cleared to zero by hardware RESET.

<7..4>   Reserved. Always <=0000>.

-------------------------

6.04.02  Reset Register                  { 3c5h    00h    R/W }

=======  ======================================================

Cross reference with {6.04.03} Clocking Mode Register.

This register provides capability to stop the sequencer operation.  This must

be done before changing the highly critical mode selections which govern all

the timing signal generation for the VGA controller.  Changes to the critical

Clocking Mode Register must be preceeded by a reset to the sequencer and then

followed by a sequencer restart.

<0>      Fast Reset Command

------------------

<=0>  Apply an immediate reset to the sequencer.  This can result in

a loss of data from the image memory.  If this loss cannot be

tolerated, the safe reset command should be used.  This image

memory loss occurs because the sequencer action maintains the

memory refresh necessary to preserve memory data during normal

operation.  If the sequencer suddenly stops, the memory chips

cannot receive their refresh cycles resulting in lost data.

At the hardware level, all sequencer outputs go tristate when

this option is selected.

<=1>  No immediate reset condition requested.  This condition is not

sufficient to guarantee that the sequencer will run, since it

must occur in conjunction with an absence of Safe Reset.

<1>      Safe Reset Command

------------------

<=0>  Apply a safe reset to the sequencer.  The sequencer maintains

correct image memory signal operation during the reset period.

This reset should be used whenever the image memory must be

preserved across the reset.  Note that this reset cannot last

longer than a few tens of microseconds without the possibility

of image memory corruption due to lack of refresh activity.

The sequencer outputs do not go tristate for this setting.

<=1>  No safe reset condition requested.  This condition is not

sufficient to guarantee that the sequencer will run, since

it must occur in conjunction with an absence of Fast Reset.

<7..2>   Reserved. Always <=000000>.

---------------------------

6.04.03  Clocking Mode Register          { 3c5h    01h    R/W }

=======  ======================================================

Cross reference with {6.04.02} Reset Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.03} Horizontal Display Enable End Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.06} Start Horizontal Retrace Pulse Register.

Cross reference with {6.05.07} End Horizontal Retrace Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.07.21} Horizontal PEL Panning Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.04} Mode Control Register 2.

<0>      Character Dot Clock Select

--------------------------

<=0>  Generate character clocks using 9 dot clocks.  This provides for

separation between each character in a line of text by insertion

of one extra pixel between characters in a horizontal direction.

This pixel is normally given the background colour, so that the

apparent width of each character is increased.  In this way, the

character fonts can use the full 8 pixels width, without having

to allow for separation between neighbouring characters.

However, a problem can arise with the automatic insertion of an

extra pixel, since some characters are used for giving graphics

effects even though text mode is in use.  A graphics character

is normally used for drawing boxes around text, such that lines

representing the box are unbroken.  This does not work when the

hardware inserts one extra pixel for every eight actually in the

character font.  To overcome this problem, a special control bit

can be set to ensure that any character in the ascii range from

hex 'c0' to hex 'df' is treated so that the 8th pixel is copied

into the 9th pixel, thereby creating continuous line graphics.

See {6.07.18} Attribute Mode Control Register.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

<=1>  Generate character clocks using 8 dot clocks.  This results in

all text mode generated characters being 8 dot clocks width. On

the display, 8 pixels are available to represent the character,

in each scan line belonging to the character.  When designing a

character font for use with 8 pixel character width, it should

be remembered that character separation will only occur if the

font uses 7 of the 8 available pixels.  The remaining pixel is

used to provide background colour to prevent characters merging.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

Packed pixel 256 colour mode halves the dot clock frequency through a

rearrangement of the shift register structure, which shifts 4 bits at

each clock using 2 dot clocks per 8 bit unit to represent 256 colours

for display.

See {6.06.07} Graphics Mode Register.

<1>      Reserved.  Always <=0>.

-----------------------

<2>      Shift Load Two Control

----------------------

<=0>  Reload the video shift registers on each character clock.  This

assumes that the video memory can respond fast enough to permit

safe operation at the frequency of the character clock.  With a

contemporary design, this is not a problem.  This is the normal

value for the bit.

The ability to load more than one byte per character clock can

assist with display image memory address extension schemes, by

making certain of the low order address bits redundant.

See {6.05.25} CRTC Mode Control Register.

The loading of more than one shift register per character clock

also has implications for shift register interconnection.  This

is discussed elsewhere.

See {6.06.07} Graphics Mode Register.

Note this setting can be over-ridden by the setting of the Shift

Load Four Control bit to <=1>.  See below.

<=1>  Reload the video shift registers on every second character clock

permitting use of slow image memory chips.  The reload accesses

two memory locations in parallel;  the length of the video shift

register is doubled.  This permits using a slower video memory

than would otherwise be possible.

The ability to load more than one byte per character clock can

assist with display image memory address extension schemes, by

making certain of the low order address bits redundant.

See {6.05.25} CRTC Mode Control Register.

The loading of more than one shift register per character clock

also has implications for shift register interconnection.  This

is discussed elsewhere.

See {6.06.07} Graphics Mode Register.

Note this setting can be over-ridden by the setting of the Shift

Load Four Control bit to <=1>.  See below.

<3>      Dot Clock Divide by Two Enable

------------------------------

<=0>  The normal dot clock frequency is available.  This means that a

single pixel will be displayed for each dot clock period.  This

is the normal setting for this bit.  It provides maximum image

resolution on the display unit.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

<=1>  The dot clock frequency is divided by two.  In this manner, the

quantity of image data available is reduced by two.  This means

that the data from what would have been originally one pixel is

now used to specify two pixels.  The result is a reduction in

the image resolution to one half of the original value.  At the

same time, any image will extend to twice the original size.

Commonly used for low resolution text displays at 40 characters

per line.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

<4>      Shift Load Four Control

-----------------------

<=0>  Reload the video shift registers on each character clock.  This

assumes that the video memory can respond fast enough to permit

safe operation at the frequency of the character clock.  With a

contemporary design, this is not a problem.  This is the normal

value for the bit.

The ability to load more than one byte per character clock can

assist with display image memory address extension schemes, by

making certain of the low order address bits redundant.

See {6.05.25} CRTC Mode Control Register.

The loading of more than one shift register per character clock

also has implications for shift register interconnection.  This

is discussed elsewhere.

See {6.06.07} Graphics Mode Register.

Note this setting can be over-ridden by the setting of the Shift

Load Two Control bit to <=1>.  See above.

<=1>  Reload the video shift registers on every fourth character clock

permitting use of slow image memory chips.  The reload accesses

four memory locations in parallel;  the width of the video shift

register is quadrupled.  This permits a much slower video memory

to be used than would otherwise be possible.  This feature does

not find any use in contemporary designs unless used for address

extension schemes.

The ability to load more than one byte per character clock can

assist with display image memory address extension schemes, by

making certain of the low order address bits redundant.

See {6.05.25} CRTC Mode Control Register.

The loading of more than one shift register per character clock

also has implications for shift register interconnection.  This

is discussed elsewhere.

See {6.06.07} Graphics Mode Register.

Note this setting is dominant over the setting of the Shift Load

Two Control bit.  See above.

<5>      Screen Inhibit

--------------

<=0>  The display screen operates normally.  The contents of the image

memory are displayed as usual, depending on the specified modes

of the VGA controller chip.

<=1>  The display screen is blanked.  The image memory is not accessed

for purposes of maintaining the display.  This means that there

is full memory bandwidth available for processor access to image

memory.  During this period, the display screen remains blanked.

However the display remains synchronised with the VGA controller

so that the display can be re-enabled at any time.

This facility is sometimes used to update a complex image such

that the update itself cannot be viewed as it occurs.  When the

update is complete, the screen is re-enabled for normal use.

Since image memory access is much faster than normal, an update

made in this way can be completed in significantly less time.

<7..6>   Reserved.  Always <=00>.

------------------------

Prior to any change to the Clocking Mode Register, the sequencer must

be temporarily shut down.  This avoids corrupted timing generation in

the VGA controller.

See {6.04.02} Reset Register.

The display generation mode is either text mode or graphic mode.

See {6.06.08} Miscellaneous Register.

6.04.04  Map Mask Register               { 3c5h    02h    R/W }

=======  ======================================================

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.06.02} Set/Reset Register.

<0>      Map Plane 0 Write Enable Control

--------------------------------

<=0>  Disable memory plane 0.  This prevents data being written to the

image plane 0 by any processor write operation to image memory.

Using this facility, plane 0 contents can be preserved across a

write operation which updates some of the other memory planes.

<=1>  Enable memory plane 0.  A processor write operation will be able

to modify data stored in the image memory plane 0.  Unless there

is at least one memory plane enabled, processor writes will not

affect image memory in any way.

Note when operating in Packed Pixel mode, all memory planes must

be enabled.  See {6.04.06} Sequencer Memory Mode Register.

<1>      Map Plane 1 Write Enable Control

--------------------------------

<=0>  Disable memory plane 1.  This prevents data being written to the

image plane 1 by any processor write operation to image memory.

Using this facility, plane 1 contents can be preserved across a

write operation which updates some of the other memory planes.

<=1>  Enable memory plane 1.  A processor write operation will be able

to modify data stored in the image memory plane 1.  Unless there

is at least one memory plane enabled, processor writes will not

affect image memory in any way.

Note when operating in Packed Pixel mode, all memory planes must

be enabled.  See {6.04.06} Sequencer Memory Mode Register.

<2>      Map Plane 2 Write Enable Control

--------------------------------

<=0>  Disable memory plane 2.  This prevents data being written to the

image plane 2 by any processor write operation to image memory.

Using this facility, plane 2 contents can be preserved across a

write operation which updates some of the other memory planes.

<=1>  Enable memory plane 2.  A processor write operation will be able

to modify data stored in the image memory plane 2.  Unless there

is at least one memory plane enabled, processor writes will not

affect image memory in any way.

Note when operating in Packed Pixel mode, all memory planes must

be enabled.  See {6.04.06} Sequencer Memory Mode Register.

<3>      Map Plane 3 Write Enable Control

--------------------------------

<=0>  Disable memory plane 3.  This prevents data being written to the

image plane 3 by any processor write operation to image memory.

Using this facility, plane 3 contents can be preserved across a

write operation which updates some of the other memory planes.

<=1>  Enable memory plane 3.  A processor write operation will be able

to modify data stored in the image memory plane 3.  Unless there

is at least one memory plane enabled, processor writes will not

affect image memory in any way.

Note when operating in Packed Pixel mode, all memory planes must

be enabled.  See {6.04.06} Sequencer Memory Mode Register.

<7..4>   Reserved.  Always <=0000>.

6.04.05  Character Map Select Register   { 3c5h    03h    R/W }

=======  ======================================================

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.10.04} Mode Control Register 2.

<0>      Character Font Memory Map 'B' Partial Offset 16K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 16K byte

<1>      Character Font Memory Map 'B' Partial Offset 32K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 32K byte

<2>      Character Font Memory Map 'A' Partial Offset 16K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 16K byte

<3>      Character Font Memory Map 'A' Partial Offset 32K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 32K byte

<4>      Character Font Memory Map 'B' Partial Offset 08K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 08K byte

<5>      Character Font Memory Map 'A' Partial Offset 08K byte

-----------------------------------------------------

<=0>  Character Font Memory Map partial offset set to 00K byte

<=1>  Character Font Memory Map partial offset set to 08K byte

<7..6>   Reserved.  Always <=00>.

------------------------

Text mode results in character generation through hardware capability

with the character fonts being brought into use.

See {6.06.08} Miscellaneous Register.

This register only becomes active when the Memory Size Specification

indicates that more than 64K of image memory is present.

See {6.04.06} Sequencer Memory Mode Register.

When the fonts selected by the A and B maps are identical, there is a

change made to the text mode attribute byte bit 3 interpretation.  In

this situation, bit 3 specifies the foreground intensity.

See {6.05.22} Underline Location Register.

When the fonts are distinct, text mode attribute byte bit 3 selects

which of the character fonts to use, either A, or B.  When the value

of bit 3 is zero, font A is used.  Otherwise, font B is selected.

See {6.05.22} Underline Location Register.

There is the possibility to extend the character set selection with

an extension to the addressing capability for a character font base

address.

See {6.10.04} Mode Control Register 2.

For convenience, the bit patterns associated with the different font

memory offsets are specified below.  They are identical for both A,B

maps, but are shown in different sorted orders below for ease of use.

Note that the hardware requires all character font data to reside in

image memory plane 2.  This cannot be circumvented by software unless

the hardware character generator is not used.  This situation results

in a much slower text display than that obtainable with the hardware,

but is essential for generation of text in graphics display modes.

<5|3|2>  Character Map Select A

----------------------

<=000>  Select Character Map Memory offset 00K

<=100>  Select Character Map Memory offset 08K

<=001>  Select Character Map Memory offset 16K

<=101>  Select Character Map Memory offset 24K

<=010>  Select Character Map Memory offset 32K

<=110>  Select Character Map Memory offset 40K

<=011>  Select Character Map Memory offset 48K

<=111>  Select Character Map Memory offset 56K

<4|1|0>  Character Map Select B

----------------------

<=000>  Select Character Map Memory offset 00K

<=001>  Select Character Map Memory offset 16K

<=010>  Select Character Map Memory offset 32K

<=011>  Select Character Map Memory offset 48K

<=100>  Select Character Map Memory offset 08K

<=101>  Select Character Map Memory offset 24K

<=110>  Select Character Map Memory offset 40K

<=111>  Select Character Map Memory offset 56K

6.04.06  Sequencer Memory Mode Register  { 3c5h    04h    R/W }

=======  ======================================================

Cross reference with {6.04.04} Map Mask Register.

Cross reference with {6.04.05} Character Map Select Register.

Cross reference with {6.06.02} Set/Reset Register.

Cross reference with {6.06.06} Read Map Select Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.05} Mode Control Register 1.

<0>      Reserved.  Always <=0>.

-----------------------

<1>      Memory Size Specification

-------------------------

<=0>  Specifies that the total image memory installed is 64K bytes.

This feature was used historically when image memory was very

much more expensive than contemporary pricing.  The option is

not used today.  When this setting is active, feature support

for multiple character sets is inhibited.  In consequence the

Character Map Select Register is disabled.

See {6.04.05} Character Map Select Register.

<=1>  Specifies that more than 64K bytes of image memory is available.

This used to be 256K bytes in earlier design implementations but

this value is likely to be too small now.  It is usual for image

memory sizing to be 1024K bytes at the time of writing.  This is

is the standard setting in use today.  With this setting active,

the Character Map Select Register features become operative.  In

addition, the display memory address counter higher order output

bits become active, permitting use of a larger addressing range.

See {6.04.05} Character Map Select Register.

<2>      Processor Image Memory Access Mode Select

-----------------------------------------

<=0>  Text Memory Access Mode.  This affects a data transfer from the

processor to the image memory such that it is suitable for text

mode operations.  The processor transfers 16 bits per access to

image memory in text mode.  The data consists of a character in

the current character set together with an attribute byte which

exterts control over the character presentation on the display.

For text mode display character codes occupy image memory plane

zero with attribute data occupying plane one.  The image memory

planes are paired (0,1) and (2,3).  The Text Memory Access Mode

directs even address accesses to planes (0,2), with odd address

accesses directed to planes (1,3).  In this way maximum ease of

access is provided for textual data update.  This is sometimes

termed odd/even mode.

To use this mode effectively it is usual to disable writes to a

pair of planes so permitting update to a two plane pair.

See {6.04.04} Map Mask Register.

Only processor writes to image memory are affected by this mode.

That is, image memory organisation continues to be multiplane in

all respects except as described above during write addressing.

Processor Image Memory Access Mapping Select must be set to zero

for this setting to be effective.  See below.

<=1>  Graphics Memory Access Mode.  In graphics mode the image memory

planes respond to addresses from the processor in parallel.  In

this way, all four memory planes can be read or written using a

single access.  The processor data transfer supports only 8 bit

transfers, so various mechanisms are employed to map the 8 bits

to each of the four bytes actually accessed.  In some cases, it

is not necessary to perform any mapping of this type at all, as

the processor is used only to generate an address which is then

used internally for image manipulation.

The manipulation is specified in advance and all that is needed

is a stream of processor generated addresses to put the process

into operation.  This type of operation is very fast to execute.

It is discussed during the description of write mode 1.

See {6.06.02} Set/Reset Register.

Processor Image Memory Access Mapping Select must be set to zero

for this setting to be effective.  See below.

See {6.06.07} Graphics Mode Register.

This is rather similar to the Memory Plane Pairing Control.

See {6.06.08} Miscellaneous Register.

<3>      Processor Image Memory Access Mapping Select

--------------------------------------------

<=0>  The processor accesses all four image memory planes using a

single access.  The access for write is modified for each of

the memory planes by means of the Map Mask Register.  This

permits multiple plane writes to take place, which is useful

when graphics mode multi-plane operation is in effect.  Each

memory plane contains all the information of a given colour.

Using multi-plane writes gives the capability to selectively

change more than one colour plane for each of eight pixels,

using one memory access.  This makes for high speed updates.

See the {6.04.04} Map Mask Register.

The processor write accesses can be further determined with

the Processor Image Memory Access Mode Select, when this is

the selected setting.  See above.

See {6.06.06} Read Map Select Register.

<=1>  The processor accesses a single image memory plane during a

single access.  This memory plane is identified by the two

least significant bits of the memory address.  This provides

a convenient means for access to image memory when packed

pixel operation is in use.  Packed pixel images do not use

multiple memory planes for colour definition.  Instead, each

memory plane is accessed in turn, and the byte specifies the

colour directly.  For this type of image, the processor need

is for single byte access to successive image planes as this

setting provides.  Of course only 16K bytes of each plane is

accessible without changing the memory mapping in this mode.

{Four planes with a 64K processor access window.}

See {6.10.05} Mode Control Register 1.

All effects of Processor Image Memory Access Mode Select, as

specified above, are made null when this setting is active.

See {6.06.06} Read Map Select Register.

<7..4>   Reserved.  Always <=0000>.

--------------------------

A Synopsis of Display Image Memory Access Control

=================================================

Processor Image Memory Access Mapping Select

=-------------------------------------------

Processor Image Memory Access Mode Select

=    =----------------------------------------

Processor Display Memory Addressing

=    =    ===================================------------------

0    0    Text access; odd/even mode; the low order address bit

selects the pair of planes to access.  Even addresses

select the pair (0,2) and odd addresses (1,3).

In all other respects the memory organisation remains

multiplane.

Data transfers are by word from the processor in this

memory mode, with a character code and attribute byte

sent together.  Memory planes 2,3 are write inhibited

to preserve the character font data in plane 2.

The Read Map Select field bit 0 is not used for this

memory mode.  The remaining bit 1 selects a plane in

the pair of planes determined by the access address,

for read mode 0.

See {6.06.06} Read Map Select Register.

0    1    Multiplane access; the planes are written by using a

write mode and read using a read mode.  The plane is

selected by means of special registers used by these

read and write modes.  This organisation supports 16

colours, with 8 pixels per byte across 4 planes at a

single image memory byte address.

Data transfers are single byte from the processor in

this memory mode.

The Read Map Select field denotes the plane number to

be read by a read mode 0 access, in this memory mode.

Planes 0 through 3 are therefore accessible.

See {6.06.06} Read Map Select Register.

1    0    The two low order address bits are the plane address;

the remaining address bits are shifted down to become

a byte address into multiplane memory.  This supports

256 colour packed pixel mode, where a byte represents

the colour data for a single pixel.

Data transfers are single byte from the processor in

this memory mode.

All two bits of the Read Map Select field are ignored

in this memory mode, for read mode 0 access.

See {6.06.06} Read Map Select Register.

1    1    The two low order address bits are the plane address;

the remaining address bits are shifted down to become

a byte address into multiplane memory.  This supports

256 colour packed pixel mode, where a byte represents

the colour data for a single pixel.

Data transfers are single byte from the processor in

this memory mode, for read mode 0 access.

All two bits of the Read Map Select field are ignored

in this memory mode.

See {6.06.06} Read Map Select Register.

---------------------------------------------------------------

6.04.07  ................................{ 3c5h    07h    R/W }

6.04.07  Horizontal Character Counter Reset Register...........

=======  ======================================================

This existance of this register is not formally recognised.

It has only been discovered through reverse engineering the

original VGA controller design.  For this reason, it cannot

be guaranteed to exist in all clone chips.  The origin of

this information is the Chips 82C456 chip documentation.

<7..0>   Horizontal Character Counter Reset

----------------------------------

When any write operation occurs to this register, with data of any

value, a special state is entered.  This state holds the horizontal

character counter in a permanent reset, making it hold the value 0.

A write to any sequencer register in the index range 0..6 will make

the special reset state terminate.  Then the horizontal character

counter is free to respond to character clocks as usual.

The character row counter which counts all active display scan lines

is clocked using a horizontal character counter derived signal.  If

the character counter reset state is entered during the vertical

retrace period, then releasing the reset state will permit display

operation to recommence in a synchronised manner.

The motivation for this functionality is that of synchronisation to

an external event.  This affords the possibility of a display being

synchronised to some event of interest, which might be the case for

a system which photographs a display screen for example.  Since the

functionality has never been officially documented, the use is made

unsafe from a portability point of view across many VGA chips.

6.05  Register specifications: CRT Controller

====  =======================================

6.05.01  CRT Controller Address Register          { 3?4h    ---    R/W }

=======  ===============================================================

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.09} CPU Latch Read Back Register.

<5..0>   CRT Controller Register Address

-------------------------------

Port 3?5h accesses the register pointed to by the index number in this

bit field.  Index number zero is represented by <=000000>.

The value of the hexadecimal digit '?' used in defining input output

port addresses is either hex 'b' or hex 'd' depending upon the setting

of the Miscellaneous Output Register Input Output Address Select bit.

See {6.09.01} Miscellaneous Output Register.

Note that bit <5> is normally set to zero, but can be used for access

to registers normally used for manufacturing test.  These are special

registers which are specific to the Trident 8900 chip.

See {6.10.09} CPU Latch Read Back Register.

<7..6>   Reserved.  Always <=00>.

------------------------

6.05.02  Horizontal Total Register                { 3?5h    00h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.03} Horizontal Display Enable End Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.06} Start Horizontal Retrace Pulse Register.

Cross reference with {6.05.07} End Horizontal Retrace Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Horizontal Total

----------------

This register specifies the total time required to enable a scan line

to complete, and the next scan line to start, such that the positions

are identical on successive scan lines.  This is equivalent to seeing

scan line generation as a repetitive operation, and allowing a fixed

amount of time for the production of a full scan line followed by the

full retrace interval.  This register provides the means to tell when

a new scan line period is about to start.  Character periods are the

unit of measurement.

Note that this is not the same as determining when a new scan line is

about to start.  See below for details.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

Because the hardware is pipelined, the number actually appearing in

this register is the total time in character periods, minus five.

When the number actually actually appearing in this register matches

the contents of the horizontal character counter, the preparations

needed for reset of the latter register are put into effect.  After

five more character clock periods, the reset takes place.  For this

reason, the horizontal character counter will continue to count past

the value determined in the Horizontal Total Register.  This implies

that further value matches are possible before the counter is reset.

In practice, the End Horizontal Blanking Register matches during the

five character clock over-run period.  See below for details.

Prior to the reset, border colour generation should have recommenced

with the completion of the display blanking period.

See {6.05.05} End Horizontal Blanking Register.

See {6.07.19} Overscan Colour Register.

The effect of the reset is that border colour generation is finished,

and the display unit begins to display actual image data.

Overview of relationship with other registers

=============================================

This register is used with the horizontal character counter, which is

the master timing reference.  The values of this counter begin at zero

and increase successively through the scan line.  When the counter is

equal to the Horizontal Total, preparations are made for the counter

to be reset back to zero.  After five further character clock periods

this reset actually takes place.  All the horizontal timing is derived

from values which match this counter as it increases from zero to the

maximum value.  A scan line is commenced just before the counter is

reset to zero, so there is a small offset between the display physical

scan line position and the value of the horizontal character counter.

By convention, when the horizontal character counter is reset to zero,

the scan line begins to generate the display using image data.  Prior

to that moment, border colour will be generated, without use of image

data.  The display image will commence part way through the scan line

so that there is an offset between actual counter values and the real

position of the electron beam(s) forming the (colour) display image.

To make this point clear, we give a value map which relates the value

in the horizontal character counter to the activity of the display.

The actual values given in hex are taken from the settings used by the

bios, when video mode 10h is in effect.  These values are illustrative

of the conceptual approach required when formulating register values.

Please note that these values may have to be encoded before actually

being used.  For example, some registers do not fully specify all the

eight bits of the horizontal character counter.  Sometimes, a setting

is split across two different registers.  The values given here are

the functional values which do not take these coding conventions into

account.  We mention this point to avoid possible confusion between

functional values and their coded representation in registers.

See {6.05.02} Horizontal Total Register. {This.}

See {6.05.03} Horizontal Display Enable End Register.

See {6.05.04} Start Horizontal Blanking Register.

See {6.05.05} End Horizontal Blanking Register.

See {6.05.06} Start Horizontal Retrace Pulse Register.

See {6.05.07} End Horizontal Retrace Register.

Hex     Register name     Corresponding Display Activity

===     =============     ==============================

00      [Zero]

------------>     Horizontal Character Counter start.................

------------>     Border Colour end...............................  |

------------>     Image display start..............              |  |

|              |  |

|              |  |

4f      {6.05.03} Horizontal Display Enable End Register  |              |  |

|              |  |

------------>     Image display end...............|              |  |

------------>     Border Colour start.................           |  |

|           |  |

|           |  |

50      {6.05.04} Start Horizontal Blanking Register         |           |  |

|           |  |

------------>     Border Colour end..................|           |  |                                            |

------------>     Display blanked start........................  |  |

|  |  |

|  |  |

54      {6.05.06} Start Horizontal Retrace Pulse Register             |  |  |

|  |  |

------------>     Scan line end.............................  |  |  |

------------>     Horizontal Retrace start...............  |  |  |  |

|  |  |  |  |

|  |  |  |  |

5f      {6.05.02} Horizontal Total Register - {A}               |  |  |  |  |

|  |  |  |  |

------------>     Horizontal character                  |  |  |  |  |

------------>     counter prepared for                  |  |  |  |  |

------------>     reset to zero                         |  |  |  |  |

|  |  |  |  |

|  |  |  |  |

61      {6.05.07} End Horizontal Retrace Register               |  |  |  |  |

|  |  |  |  |

------------>     Horizontal Retrace end................|  |  |  |  |

------------>     Scan line start..........................|  |  |  |

|  |  |

|  |  |

62      {6.05.05} End Horizontal Blanking Register                    |  |  |

|  |  |

------------>     Display blanked end.........................|  |  |

------------>     Border Colour start............................|  |

|

|

{6.05.02} Horizontal Total Register - {B}                           |

|

64      ------------>     Horizontal character                              |

------------>     counter is actually                               |

reset to zero.....................................|

6.05.03  Horizontal Display Enable End Register   { 3?5h    01h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Horizontal Display Enable

-------------------------

This register specifies the total time during which a single scan line

will actually display image data. The character period is the unit for

measurement.  This determines the length of a scan line in character

periods.  This measure is independant of the means by which the image

is generated.  It is therefore valid for text mode or graphics mode.

See {6.05.02} Horizontal Total Register.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

Because the hardware represents this period by means of the horizontal

character counter which starts the active display period with a value

of zero, at the end of the active display period the counter will have

a value equal to the display time in character periods, minus one.  It

is that period value minus one which is programmed into this register.

This register specifies the start of border colour generation.

See {6.05.04} Start Horizontal Blanking Register.

See {6.07.19} Overscan Colour Register.

6.05.04  Start Horizontal Blanking Register       { 3?5h    02h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.03} Horizontal Display Enable End Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Start Horizontal Blanking

-------------------------

This register specifies the value of the horizontal character counter

when display blanking begins.  This measure is independant of the way

in which the image is generated.  It is therefore valid for text mode

or graphics mode.

See {6.05.02} Horizontal Total Register.

Before this point, border colour generation will have been started.

See {6.05.03} Horizontal Display Enable End Register.

See {6.07.19} Overscan Colour Register.

This point determines the end of the border colour generation.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

6.05.05  End Horizontal Blanking Register         { 3?5h    03h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.07} End Horizontal Retrace Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<4..0>   End Horizontal Blanking

-----------------------

This register specifies the value of the horizontal character counter

when blanking finishes.  This measure is independant of the means by

which the image is generated.  It is therefore valid for text mode or

graphics mode.   The full register width is six bits, with the higher

order bit stored elsewhere.

See {6.05.07} End Horizontal Retrace Register.

A discussion of vertical timing generation is available.

See {6.05.02} Horizontal Total Register.

This determines the start of the border colour generation.

See {6.07.19} Overscan Colour Register.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

<6..5>   Display Enable Skew

-------------------

<=00>  0 character clock delay

<=01>  1 character clock delay

<=10>  2 character clock delay

<=11>  3 character clock delay

This setting permits compensation for internal delays in the data path

of the VGA controller, due to pipelining.

<7>      Light Pen Register Enable

-------------------------

This bit is Reserved for the Trident 8900 chip.

This bit is used for test with the UM587 chip.

The following is valid for the Chips 82C456 chip.

<=0>  Enable Lightpen Registers

-------------------------

The CRTC Registers at indexes 10h, 11h, function as light pen

location registers.  In this mode, the register assignments

are as follows:

10h:  Lightpen High Register.

-----------------------

Read only register representing the character position at

which the lightpen became active.  This portion specifies

the 8 high order bits.

Effective only in MDA/Hercules modes, or when enabled as

described above.  This facility is not used much today,

essentially because of unfavourable results which do not

compare well with other pointing devices.

See {6.05.18} Vertical Retrace Start Register.

See {6.10.04} Mode Control Register 2.

11h:  Lightpen Low Register.

----------------------

Read only register representing the character position at

which the lightpen became active.  This portion specifies

the 8 low order bits.

Effective only in MDA/Hercules modes, or when enabled as

described above.  This facility is not used much today,

essentially because of unfavourable results which do not

compare well with other pointing devices.

See {6.05.19} Vertical Retrace End Register.

See {6.10.04} Mode Control Register 2.

<=1>  Normal Operation

----------------

The CRTC Registers at indexes 10h, 11h, function as normal.

See {6.05.18} Vertical Retrace Start Register.

See {6.05.19} Vertical Retrace End Register.

6.05.06  Start Horizontal Retrace Pulse Register  { 3?5h    04h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.07} End Horizontal Retrace Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Start Horizontal Retrace Pulse

------------------------------

This register specifies the value of the horizontal character counter

when the horizontal retrace pulse starts.  This measure is independant

of the means by which the image is generated.  It is therefore valid

for text mode or graphics mode.

This point determines where the scan line will end.  The display will

have been blanked from a time before the horizontal retrace pulse was

started, until some time after the horizontal retrace pulse finished.

See {6.05.02} Horizontal Total Register.

See {6.05.04} Start Horizontal Blanking Register.

See {6.05.05} End Horizontal Blanking Register.

See {6.05.07} End Horizontal Retrace Register.

This register permits control of display screen horizontal centering.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

6.05.07  End Horizontal Retrace Register          { 3?5h    05h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.06} Start Horizontal Retrace Pulse Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<4..0>   End Horizontal Retrace

----------------------

This register specifies the value of the horizontal character counter

when the horizontal retrace pulse ends.  This measure is independant

of the means by which the image is generated.  It is therefore valid

for text mode or graphics mode.

This point determines when the scan line will start.  The display is

blanked from a time before the horizontal retrace pulse was started,

until some time after the horizontal retrace pulse was finished.

See {6.05.02} Horizontal Total Register.

See {6.05.04} Start Horizontal Blanking Register.

See {6.05.05} End Horizontal Blanking Register.

See {6.05.06} Start Horizontal Retrace Pulse Register.

Note that the width of a character in text mode can be changed from

8 pixels to 9 pixels which would change the apparent scan line time.

See {6.04.03} Clocking Mode Register.

<6..5>   Horizontal Retrace Skew

-----------------------

<=00>  0 character clock delay

<=01>  1 character clock delay

<=10>  2 character clock delay

<=11>  3 character clock delay

This setting permits compensation for internal delays in the data path

of the VGA controller, due to pipelining.

<7>      End Horizontal Blanking Bit 5

-----------------------------

This bit provides the most significant bit for this 6 bit register.

See {6.05.05} End Horizontal Blanking Register.

6.05.08  Vertical Total Register                  { 3?5h    06h    R/W }

=======  ===============================================================

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.20} Vertical Display Enable End Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

Cross reference with {6.10.12} Clear Vertical Display Enable Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Vertical Total

--------------

This register specifies the lower eight bits of the ten bits effective

width register, used to specify the total number of display image scan

lines.  The two remaining high order bits are located elsewhere.

See {6.05.09} CRT Controller Overflow Register.

Because the hardware is pipelined, the number appearing in the ten bit

wide register is the total scan line count minus two.

When the number actually actually appearing in this extended register

matches the contents of the character row counter, the preparations

needed for reset of the latter register are put into effect.  After a

further two scan line periods, the reset takes place.  For this reason

the character row counter will continue to count past the value given

by the extended Vertical Total Register.  This is pipelining at work.

This register is used in conjunction with the character row counter.

The character row counter is a principle timing reference, which gives

control over all operations which are determined by the number of the

current scan line being generated.  The character row counter changes

at the end of each scan line which generates an active image display,

to reflect the number of each scan line created.  This count operation

can be changed on option, to count pairs of scan lines.  This option

has the important property of changing the timing resolution of every

aspect of vertical timing generation.

See {6.05.25} CRTC Mode Control Register.

Overview of relationship with other registers

=============================================

This register is used in conjunction with the character row counter,

which is a principle timing reference.  This counter starts at zero

and steadily increases as the display develops each new scan line.

By convention, the value zero corresponds with the first scan line

which actually develops displayable data for the frame.

When the content of the Vertical Total register matches the character

row counter, preparations are put into effect to reset the latter, to

begin counting scan lines for a new frame.  Two scan lines later, the

reset actually takes place.

All the vertical timing is derived from values which match the counter

as it increases from zero to the maximum value.  The value zero is by

convention associated with the very first display image scan line in a

frame.  This is not the same as the first scan line in a frame.  Prior

to the display of this first image scan line the vertical retrace will

have returned the notional position of the electron beams to the upper

left of the display screen.

Since this traversal across the screen would damage any image display

if the electron beams were switched on, the vertical blanking period

starts some time before, and extends to some time after, the vertical

retrace period.  In this manner the electron beams are inhibited, so

preserving the display with complete integrity.

The character row counter controls the vertical blanking and retrace,

before being reset to begin counting active image scan lines.  This is

the reason why the physical display unit scan line does not correspond

directly with the scan line in the character row counter.  There is a

small offset between the two, such that when the character row counter

is reset to zero, the display unit has already displayed some overscan

lines immediately following the vertical retrace period.

The active image display extends from scan line zero upto the maximum

scan line specified by the Vertical Display Enable End Register.  The

overscan colour is generated from this point onwards, until the onset

of the vertical blanking period.  The vertical retrace period activity

then takes place during vertical blanking.  When the vertical blanking

period finishes, overscan colour is again produced, but this occurs at

the top of the display, since retrace has occured.

To make these points clear we give a value map which relates the value

in the character row counter to the activity of the display.

The actual values given in hex are taken from the settings used by the

bios, when video mode 10h is in effect.  These values are illustrative

of the conceptual approach required when formulating register values.

Please note that these values may have to be encoded before actually

being used.  For example some registers do not fully specify all the

ten bits of the character row counter.  Sometimes, a setting appears

split across two different registers.  The values here are functional

values which do not take these coding conventions into account.  This

point is mentioned to avoid possible confusion between the functional

values and their coded representation in registers.

See {6.05.08} Vertical Total Register. {This.}

See {6.05.18} Vertical Retrace Start Register.

See {6.05.19} Vertical Retrace End Register.

See {6.05.20} Vertical Display Enable End Register.

See {6.05.23} Start Vertical Blanking Register.

See {6.05.24} End Vertical Blanking Register.

See {6.05.26} Line Compare Register.

Hex     Register name     Corresponding Display Activity

===     =============     ==============================

00      [Zero]

------------>     Character Row Counter start........................

------------>     Overscan Colour end.............................  |

------------>     First image scan line start......              |  |

|              |  |

|              |  |

??      {6.05.26} Line Compare Register                   |              |  |

|              |  |

------------>     Start address of display        |              |  |

------------>     is reset to zero.  Makes        |              |  |

------------>     a new window in which it        |              |  |

------------>     is possible to completely       |              |  |

------------>     inhibit display panning.        |              |  |

|              |  |

|              |  |

15d      {6.05.20} Vertical Display Enable End Register    |              |  |

|              |  |

------------>     Last image scan line end........|              |  |

------------>     Overscan Colour start...............           |  |

|           |  |

|           |  |

163      {6.05.23} Start Vertical Blanking Register           |           |  |

|           |  |

------------>     Overscan Colour end................|           |  |                                            |

------------>     Display blanked start.....................     |  |

|     |  |

|     |  |

183      {6.05.18} Vertical Retrace Start Register                  |     |  |

|     |  |

------------>     Vertical Retrace start.................  |     |  |

|  |     |  |

|  |     |  |

185      {6.05.19} Vertical Retrace End Register                 |  |     |  |

|  |     |  |

------------>     Vertical Retrace end..................|  |     |  |

|     |  |

|     |  |

1ba      {6.05.24} End Vertical Blanking Register                   |     |  |

|     |  |

------------>     Display blanked end......................|     |  |

------------>     Overscan Colour start..........................|  |

|

|

1bf      {6.05.08} Vertical Total Register - {A}                             |

|

------------>     Character row counter                             |

------------>     prepared for reset to                             |

------------>     zero                                              |

|

|

{6.05.08} Vertical Total Register - {B}                             |

|

1c1      ------------>     Character row counter is                          |

------------>     actually reset to zero............................|

6.05.09  CRT Controller Overflow Register         { 3?5h    07h    R/W }

=======  ===============================================================

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.20} Vertical Display Enable End Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register provides for the high order bits needed to complete some

wide internal registers of greater than eight bits total width.  This

is necessary since the processor input output port transfer capability

supports only eight bit transfers.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<0>      Vertical Total Bit 8

--------------------

See {6.05.08} Vertical Total Register.

<1>      Vertical Display Enable End Bit 8

---------------------------------

See {6.05.20} Vertical Display Enable End Register.

<2>      Vertical Retrace Start Bit 8

----------------------------

See {6.05.18} Vertical Retrace Start Register.

<3>      Start Vertical Blank Bit 8

--------------------------

See {6.05.23} Start Vertical Blanking Register.

The highest bit of the ten bit Start Vertical Blank Register is found

in the Maximum Scan Line Register.

See {6.05.11} Maximum Scan Line Register.

<4>      Line Compare Bit 8

------------------

See {6.05.26} Line Compare Register.

The highest bit of the ten bit Line Compare Register is located in the

Maximum Scan Line Register.

See {6.05.11} Maximum Scan Line Register.

<5>      Vertical Total Bit 9

--------------------

See {6.05.08} Vertical Total Register.

<6>      Vertical Display Enable End Bit 9

---------------------------------

See {6.05.20} Vertical Display Enable End Register.

<7>      Vertical Retrace Start Bit 9

----------------------------

See {6.05.18} Vertical Retrace Start Register.

6.05.10  Preset Row Scan Register                 { 3?5h    08h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.21} Offset Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.07.21} Horizontal PEL Panning Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

<4..0>   Preset Row Scan

---------------

This binary value determines the notional scan line within a character

font, immediately after a vertical retrace has arrived at the display.

When the display is operating in text mode, this value is used to give

the correct position on the display for the first line of text.  Later

lines of text have their position on the display controlled relative

to the preceeding lines, using the controls provided for this purpose.

See {6.05.11} Maximum Scan Line Register.

This capability provides for the smooth vertical scrolling of lines of

text.  During the generation of the first line of text on the display,

some of the uppermost scan lines of a character font can be inhibited

by use of this facility.  When successively more of the first line of

text is ommitted in this manner, the result will give the visual idea

that the whole screen of text is slowly being scrolled upwards.

To make this situation more concrete, we will look at how a character

display is generated by the VGA controller when text mode is in use.

A character font contains 256 characters, each of which is 8 pixels in

width, with 32 scan lines depth.  This means that 32 bytes, each of 8

bits, will represent one character completely.  However most character

fonts do not require the full 32 pixel by 8 pixel format.  In order to

display all the 32 rows of 8 pixels, it is necessary to arrange that a

conversion of each character code in a line of text takes place using

the appropriate 8 consecutive pixels during each scan line.  This scan

line indexing into the 32 byte array of pixel data representing a font

can be performed by means of a special scan line counter reserved just

for character generation purposes.

If this is done, it is then necessary to decide how to initialise the

scan line counter after a vertical retrace occurs.  During character

generation, we must know the maximum value of scan line which is part

of the character font in current use.  This permits any depth of font

to be used even though the storage format provided by the hardware is

standardised at 32 rows by 8 columns.  The maximum scan line used for

text font generation is specified by the Maximum Scan Line Register.

See {6.05.11} Maximum Scan Line Register.

<6..5>   Byte Panning Control

--------------------

The value of this bit field determines byte panning in text mode.  The

effect is to shift the displayed image left by the specified number of

eight or nine pixel groups.  The selection of eight or nine dot clocks

is performed by the Clocking Mode Register, which directly determines

the maximum number of pixels controlled for panning purposes.  The pan

takes effect by shifting the screen image successively to the left of

the screen, with increasing binary panning control values.

See {6.04.03} Clocking Mode Register.

This panning capability is fully complementary with the pixel panning

control available using the Horizontal PEL Panning Register.

See {6.07.21} Horizontal PEL Panning Register.

Note that in graphics modes employing 256 colour packed pixel format,

the Byte Panning Control and the Horizontal PEL Panning Register must

not be used.  Instead, use the image start address to obtain a single

pixel pan effect.  Where this technique is used the screen width must

be increased by use of the Offset Register, so that panning does not

cause the data from a scan line to become part of the following scan

line.  This is the same as saying that panning can only modify a view

to the extent that the view remains within a displayable image stored

within the video memory.  See below for discussion.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

See {6.05.21} Offset Register.

See {6.10.07} CRTC Module Testing Register.

Essential Display Panning Concepts

==================================

In order to understand panning, it is necessary to understand the big

difference in display image generation when in text mode as opposed to

graphics mode.  This difference is absolutely fundamental, and arises

because of the distinctions between the ways in which the information

which comprises the displayed image is stored in image memory.

In text mode, the image memory holds character codes and attributes,

which, when combined together with font definitions for a character

set, enable the VGA controller to generate characters to the display.

Each character is represented by either 8 or 9 pixels in each of the

scan lines in which it appears, depending upon the specified dot clock

selection made by the Clocking Mode Register.

See {6.04.03} Clocking Mode Register.

If the pixels appearing at the extreme left of the display are removed

from the display, then the result gives the impression of the complete

screen image having been panned left.  This function is controlled by

the Horizontal PEL Panning Register, which takes values dependant upon

the maximum pan required, either 8 or 9 pixels:

1.  8 pixel pan, binary value specifies left pan movement, with

number zero representing a pan of left zero positions.  The

pan magnitude is taken modulo 8 as in:

left_pan = (number_field + 0) mod 8

The pan range is therefore 0 .. 7 pixels.

2.  9 pixel pan, binary value specifies a left pan movement of the

value plus one, with number 8 representing a left pan movement

of zero positions.  The pan magnitude is taken modulo 9 with

an offset of one as in:

left_pan = (number_field + 1) mod 9

The pan range is therefore 0 .. 8 pixels.

Once the limit of pixel panning is reached, it is necessary to change

the data used to generate the display.  This is where byte panning is

used.  To achieve a smooth panning effect, the pixel panning register

is gradually increased to the maximum, then changed to zero, whilst

simultaneously increasing byte panning.  This enables panning to take

place smoothly across large portions of the displayed image.

In graphics mode there are two separate image generation methods used.

Multi-plane graphics images are produced by reading the image memory

planes using a single address, which yields four parallel bytes, one

each from each memory plane.  These bytes together represent eight of

the pixels for display.  By interpreting the same numbered bit in all

these four bytes at the same time, it is possible to produce a coding

for 16 separate colours.  In this way, the four bytes are shifted out

to the display in parallel, so defining the colour of each successive

pixel in the display image as one of a possible 16 distinct colours.

For this type of image, the Horizontal PEL Panning Register operates

exactly as described above, except that there is no 9 dot clock used.

{This is only for use in text modes.}

In packed pixel mode, graphics images are produced in a different way.

There are actually three different types of packed pixel mode.  These

are:

1.  Colour 2 Packed Pixel Mode.    {Uses 1 bit per pixel.}

2.  Colour 4 Packed Pixel Mode.    {Uses 2 bits per pixel.}

3.  Colour 256 Packed Pixel Mode.  {Uses 8 bits per pixel.}

For the Colour 2 Packed Pixel Mode, the image consists of individual

pixels represented directly by the bits in a byte, thus the high order

bit represents the leftmost pixel colour, foreground or background.

This type of image is easily panned, exactly as described above for

the 8 dot clock selection.

The Colour 256 Packed Pixel Mode represents a single pixel colour in

one complete byte of image memory.  In this case the bit shifting of

the Horizontal PEL Panning Register is superfluous together with the

Byte Panning Control.  Instead, the starting address of the display

should be used to give pixel panning in both the horizontal and the

vertical directions.

Colour 4 Packed Pixel Mode uses the image memory planes in a modified

multi-plane mode, which uses two of the four planes to specify one of

four possible colour selections for each pixel.  Neighbouring memory

planes are used, either planes (0,1) or (2,3).  In this situation the

full Horizontal PEL Panning Register capability is used.  Only the 8

dot clock selection is applicable.

A facility is available which affords the possibility of creating two

separate windows within a single screen display.  This has an impact

on some aspects of pixel panning.

The two windows are distinguished in that the first always specifies

a non-zero starting address into the image memory, and is subject to

panning control.  The second window always begins at a display image

memory address of zero, and may either be panned in the same manner

as the first window, or alternatively may be made immune to panning,

as determined by option setting.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

See {6.07.18} Attribute Mode Control Register.

The scan line where the first window ends and where the second lower

window starts is determined by the Line Compare Register.

See {6.05.26} Line Compare Register.

The control governing the second window which determines whether it

responds to panning specified for the first upper window is applied

by the Attribute Mode Control Register.

See {6.07.18} Attribute Mode Control Register.

A discussion of pixel panning is available.

See {6.07.21} Horizontal PEL Panning Register.

<7>      Reserved.  Always <=0>.

-----------------------

6.05.11  Maximum Scan Line Register               { 3?5h    09h    R/W }

=======  ===============================================================

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

A single bit <7> only is affected.  {Double Scan Line Display.}

See {6.10.07} CRTC Module Testing Register.

<4..0>   Maximum Scan Lines

------------------

The binary value of this field determines the highest scan line to be

used in generating characters from any currently selected font.  This

binary value matches an internal scan line counter which commences at

zero, so to permit generation of characters with 14 scan lines, the

value 13 would be programmed into this field.  This internal counter

keeps track of the scan line for character generation, so is separate

from the character row counter that counts scan lines in the display.

The internal counter is reset to zero when it has a value equal to the

Maximum Scan Lines field.  It therefore continually cycles through all

values from zero to the maximum.

The effect of this field is to decide when it is necessary to commence

reading the data in image memory associated with the following line of

text.  Until that time, each line of text is repeatedly read and sent

to the character generator logic, once for each scan line in the font

being used.  In this manner, all the scan lines are built up and sent

to the display to represent the image of one complete line of text.

<5>      Start Vertical Blank Bit 9

--------------------------

This bit represents bit 9 of the 10 bit wide representation for the

Start Vertical Blank value.

See {6.05.09} CRT Controller Overflow Register.

See {6.05.23} Start Vertical Blanking Register.

<6>      Line Compare Bit 9

------------------

This bit represents bit 9 of the 10 bit wide representation for the

Line Compare Register value.

See {6.05.09} CRT Controller Overflow Register.

See {6.05.26} Line Compare Register.

<7>      Double Scan Line Display

------------------------

<=0>  Each scan line is displayed once only.  This represents the

normal operational value.

<=1>  Each scan line is displayed twice so increasing the depth of the

display by a factor of two despite using the same amount of data

needed to specify only half the number of scan lines. This is a

compatibility feature used for low resolution display emulation.

The pixel aspect ratio doubles when using this setting.

This capability advances the display memory address counter to

the next scan line in image memory only every second scan line,

so data is repeated exactly twice during successive scan lines.

This means the display memory address counter is actually reset

after the first scan line of every pair, so that the same data

can be output during the second scan line of the pair.

Warning: do not confuse this capability with that provided by

the Horizontal Retrace Divisor.

See {6.05.25} CRTC Mode Control Register.

6.05.12  Cursor Start Register                    { 3?5h    0ah    R/W }

=======  ===============================================================

Cross reference with {6.05.13} Cursor End Register.

Cross reference with {6.05.16} Cursor Location High Register.

Cross reference with {6.05.17} Cursor Location Low Register.

<4..0>   Cursor Row Scan Start

---------------------

This binary field represents the first scan line used to present the

cursor when in text mode.  The very first scan line is numbered zero,

corresponding with the first scan line in the character font used to

produce the text.  It is therefore not a scan line relative to the

start of the frame being displayed.

The displayed cursor will extend from this value, through to the value

specified in the Cursor Row Scan End field.

The hardware generated cursor is only active in text mode.

See {6.05.13} Cursor End Register.

See {6.05.16} Cursor Location High Register.

See {6.05.17} Cursor Location Low Register.

<5>      Cursor Enable

-------------

<=0>  Text mode hardware cursor is displayed.  Cursor on.

<=1>  Text mode hardware cursor is inhibited.  Cursor off.

<7..6>   Reserved.  Always <=00>.

------------------------

6.05.13  Cursor End Register                      { 3?5h    0bh    R/W }

=======  ===============================================================

Cross reference with {6.05.12} Cursor Start Register.

Cross reference with {6.05.16} Cursor Location High Register.

Cross reference with {6.05.17} Cursor Location Low Register.

<4..0>   Cursor Row Scan End

-------------------

This binary field represents the first scan line used to present the

cursor when in text mode.  The very first scan line is numbered zero,

corresponding with the first scan line in the character font used to

produce the text.  It is therefore not a scan line relative to the

start of the frame being displayed.

The displayed cursor will extend from the value specified in the field

Cursor Row Scan End, through to this value, inclusive.

The hardware generated cursor is only active in text mode.

See {6.05.12} Cursor Start Register.

See {6.05.16} Cursor Location High Register.

See {6.05.17} Cursor Location Low Register.

<6..5>   Cursor Skew Control

-------------------

This binary field specifies the amount of right skew applied to the

cursor, when it is visible.  In this way, it is possible to displace

the cursor to the right of the character where it would normally be

produced.  This feature can sometimes be of benefit when very small

characters are produced during high resolution screen operation.

It should be used with care, however, since visual disorientation

can result from over-use of the higher displacement values.

The main motivation for this control is to compensate for internal

pipeline delays in the VGA controller.

See {6.05.16} Cursor Location High Register.

See {6.05.17} Cursor Location Low Register.

<7>      Reserved.  Always <=0>.

-----------------------

6.05.14  Start Address High Register              { 3?5h    0ch    R/W }

=======  ===============================================================

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.21} Offset Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.05} Mode Control Register 1.

Cross reference with {6.10.07} CRTC Module Testing Register.

<7..0>   Start Address High

------------------

This binary value represents 8 of the possible 16 to 17 display start

address bits.  The display generation uses this address to determine

where to commence reading image memory.  This address corresponds to

the upper left of the display screen.

In the following display modes, addresses wrap as indicated:

Colour Graphics Adapter    ==>   16K or 14 bits wide addressing.

Monochrome Display Adapter ==>   32K or 15 bits wide addressing.

Hercules mode              ==>   64K or 16 bits wide addressing.

See {6.10.04} Mode Control Register 2.

There may be some address alignment implications, depending upon the

way in which memory addresses are used during the display generation

process.

See {6.05.22} Underline Location Register.

See {6.05.25} CRTC Mode Control Register.

The lower bits <7..0> are found in the Start Address Low Register.

See {6.05.15} Start Address Low Register.

The upper bit <16> is a switchable option.  Assume a maximum image

size of 1024K arranged in four planes of 256K; it is possible with

17 bits of start address to specify upto 128K of offset into image

memory.  This is adequate to support two separate display pages.

See {6.10.07} CRTC Module Testing Register.

There is also a start address bit <19> available for configurations

with more than 512K of image memory.  This bit defines which memory

bank is to be used for display purposes, so is quite distinct from

that accessible to the processor.  The display hardware is not able

to address images larger than 512K unless 1024x768 256 colour mode

is used.  For display images smaller than 512K, bit <19> is useful.

See {6.10.05} Mode Control Register 1.

During display operation, there is potential to reset the display

image current address to zero, coincidently with the commencement

of a new display scan line.  This facility is useful for providing

two separate window areas on the display screen.  The first window

possesses a start address which is greater than zero, and displays

for the required number of scan lines.  Then, at a scan line given

by the Line Compare Register, the current display address is reset

to zero, thereby displaying the second window.

See {6.05.26} Line Compare Register.

This second window has the useful property that it can be protected

from panning.  This means that during panning operations affecting

the upper window, the lower window will either pan in the same way,

or remain completely unchanged depending upon an option bit setting.

See {6.07.18} Attribute Mode Control Register.

6.05.15  Start Address Low Register               { 3?5h    0dh    R/W }

=======  ===============================================================

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.21} Offset Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.05} Mode Control Register 1.

Cross reference with {6.10.07} CRTC Module Testing Register.

<7..0>   Start Address Low

-----------------

This binary value represents 8 of the possible 16 to 17 display start

address bits.  The display generation uses this address to determine

where to commence reading image memory.  This address corresponds to

the upper left of the display screen.

In the following display modes, addresses wrap as indicated:

Colour Graphics Adapter    ==>   16K or 14 bits wide addressing.

Monochrome Display Adapter ==>   32K or 15 bits wide addressing.

Hercules mode              ==>   64K or 16 bits wide addressing.

See {6.10.04} Mode Control Register 2.

There may be some address alignment implications, depending upon the

way in which memory addresses are used during the display generation

process.

See {6.05.22} Underline Location Register.

See {6.05.25} CRTC Mode Control Register.

The upper bits <15..8> are found in the Start Address High Register.

See {6.05.14} Start Address High Register.

The upper bit <16> is a switchable option.  Assume a maximum image

size of 1024K arranged in four planes of 256K; it is possible with

17 bits of start address to specify upto 128K of offset into image

memory.  This is adequate to support two separate display pages.

See {6.10.07} CRTC Module Testing Register.

There is also a start address bit <17> available for configurations

with more than 512K of image memory.  This bit defines which memory

bank is to be used for display purposes, so is quite distinct from

that accessible to the processor.  The display hardware is not able

to address images larger than 512K unless 1024x768 256 colour mode

is used.  For display images smaller than 512K, bit <17> is useful.

See {6.10.05} Mode Control Register 1.

During display operation, there is potential to reset the display

image current address to zero, coincidently with the commencement

of a new display scan line.  This facility is useful for providing

two separate window areas on the display screen.  The first window

possesses a start address which is greater than zero, and displays

for the required number of scan lines.  Then, at a scan line given

by the Line Compare Register, the current display address is reset

to zero, thereby displaying the second window.

See {6.05.26} Line Compare Register.

This second window has the useful property that it can be protected

from panning.  This means that during panning operations affecting

the upper window, the lower window will either pan in the same way,

or remain completely unchanged depending upon an option bit setting.

See {6.07.18} Attribute Mode Control Register.

6.05.16  Cursor Location High Register            { 3?5h    0eh    R/W }

=======  ===============================================================

Cross reference with {6.05.12} Cursor Start Register.

Cross reference with {6.05.13} Cursor End Register.

Cross reference with {6.05.17} Cursor Location Low Register.

Cross reference with {6.10.04} Mode Control Register 2.

<7..0>   Cursor Location High

--------------------

This binary value represents an image memory address.  As the display

is created, image data is read from image memory.  When this address

is accessed in order to create the display, the cursor will be placed

to coincide with the corresponding character position on the screen,

subject only to any applied cursor right skew.

A hardware cursor is only generated in text mode.

See {6.05.12} Cursor Start Register.

See {6.05.13} Cursor End Register.

See {6.05.17} Cursor Location Low Register.

In the following display modes, addresses wrap as indicated:

Colour Graphics Adapter    ==>   16K or 14 bits wide addressing.

Monochrome Display Adapter ==>   32K or 15 bits wide addressing.

Hercules mode              ==>   64K or 16 bits wide addressing.

See {6.10.04} Mode Control Register 2.

6.05.17  Cursor Location Low Register             { 3?5h    0fh    R/W }

=======  ===============================================================

Cross reference with {6.05.12} Cursor Start Register.

Cross reference with {6.05.13} Cursor End Register.

Cross reference with {6.05.16} Cursor Location High Register.

Cross reference with {6.10.04} Mode Control Register 2.

<7..0>   Cursor Location Low

-------------------

This binary value represents an image memory address.  As the display

is created, image data is read from image memory.  When this address

is accessed in order to create the display, the cursor will be placed

to coincide with the corresponding character position on the screen,

subject only to any applied cursor right skew.

A hardware cursor is only generated in text mode.

See {6.05.12} Cursor Start Register.

See {6.05.13} Cursor End Register.

See {6.05.16} Cursor Location High Register.

In the following display modes, addresses wrap as indicated:

Colour Graphics Adapter    ==>   16K or 14 bits wide addressing.

Monochrome Display Adapter ==>   32K or 15 bits wide addressing.

Hercules mode              ==>   64K or 16 bits wide addressing.

See {6.10.04} Mode Control Register 2.

6.05.18  Vertical Retrace Start Register          { 3?5h    10h    R/W }

=======  ===============================================================

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Vertical Retrace Start

----------------------

This binary value represents the 8 lower bits of the 10 bit wide

Vertical Retrace Start Register.  This register is compared with

the character row counter which therefore matches when a certain

scan line is displayed.  The upper bits are stored elsewhere.

See {6.05.09} CRT Controller Overflow Register.

When the match occurs, the vertical retrace period begins.  This

generates the vertical retrace signal to the display unit, which

then commences the return of the electron beam scanning position

to the upper left of the display screen.  The vertical retrace

period continues until the character row counter is of identical

value with that in the Vertical Retrace End Register.

See {6.05.19} Vertical Retrace End Register.

At the time the vertical retrace commences, the vertical blanking

period will already have begun.  This is so that the retrace will

not disturb the image present on the screen.  Vertical blanking

continues until after the vertical retrace period has ended.

See {6.05.23} Start Vertical Blanking Register.

See {6.05.24} End Vertical Blanking Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

A detailed discussion of vertical timing issues is available with

a timing diagram.

See {6.05.08} Vertical Total Register.

The following is not true for the Trident 8900 chip.

The following is valid for the Chips 82C456 chip.

The Vertical Retrace Start Register can be made invisible at this

index address, when the Lightpen High Register is placed at this

same register index.

For details see the Light Pen Register Enable description.

See {6.05.05} End Horizontal Blanking Register.

6.05.19  Vertical Retrace End Register            { 3?5h    11h    R/W }

=======  ===============================================================

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.09.03} Input Status Register Zero.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<3..0>   Vertical Retrace End

--------------------

This binary value represents the 4 lower bits of the 10 bit wide

character row counter.  The character row counter determines the

current scan line being generated for the display.  When a match

occurs in these lower four bits, the vertical retrace period end

is signalled.  The comparison between these low bits only starts

when the vertical retrace period has commenced, consequently the

possibility of an ambiguous match cannot occur.  This coding has

the property of limiting the maximum period to 15 scan lines.

See {6.05.18} Vertical Retrace Start Register.

At the time the vertical retrace commences, the vertical blanking

period will already have begun.  This is so that the retrace will

not disturb the image present on the screen.  Vertical blanking

continues until after the vertical retrace period has ended.

See {6.05.23} Start Vertical Blanking Register.

See {6.05.24} End Vertical Blanking Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

A detailed discussion of vertical timing issues is available with

a timing diagram.

See {6.05.08} Vertical Total Register.

<4>      Clear Vertical Interrupt

------------------------

This bit is cleared by hardware RESET.

<=0>  Clear the vertical interrupt state.

<=1>  Normal vertical interrupt state operation.

This interrupt status is obtainable from a status bit provided for

just this purpose.  It can be useful when interrupts are inhibited

yet it is still necessary to obtain vertical timing information.

See {6.09.03} Input Status Register Zero.

<5>      Enable Vertical Interrupt

-------------------------

This bit is cleared by hardware RESET.

<=0>  Enable the vertical interrupt {assigned by hardware to IRQ2}.

This will only occur if the programmable interrupt controller

8259 was initialised appropriately, since all interrupts must

first be routed through this chip before being serviced by an

interrupt program in the processor.

<=1>  Disable the vertical interrupt.

This interrupt status is obtainable from a status bit provided for

just this purpose.  It can be useful when interrupts are inhibited

yet it is still necessary to obtain vertical timing information.

See {6.09.03} Input Status Register Zero.

<6>      Refresh Bandwidth Select

------------------------

<=0>  Generate 3 image memory refresh cycles per horizontal scan

line.  These refresh cycles prevent data stored in dynamic

RAM {random access memory} chips from being lost.

<=1>  Generate 5 image memory refresh cycles per horizontal scan

line.  If more refresh cycles are needed to keep the image

memory intact, less time is available for processor access.

This field is sometimes termed CPU Bandwidth Select, for reasons

which will become apparent below.

Small image memories require lower numbers of refresh periods than

do larger memories with more data to refresh.

The greater the refresh activity, the less opportunity there is to

access the memory for processor reads and writes.  With high image

resolution displays the issue of processor access to image memory

can become critical in determining the display update speed.

The processor can make a request to access image memory, which then

cannot be granted immediately as all display activity has priority.

A relatively long access delay can then ensue until the access is

finally granted.  Although the delay is measured in microseconds,

this still represents a significant delay for fast processors which

could have executed possibly many instructions during this period.

A memory can be either read, written or refreshed at any one time.

The sequencer determines the moment by moment activity for all the

image display RAM chips, thereby ensuring that the maximum use is

made of image memory capability at all times.

Bandwidth is a hardware term which is concerned with describing a

rate at which information is transferred, or at which an activity

is, or can be, performed.  Typically, bandwidth is measured using

a number which represents the count of things taking place over a

fixed time period, usually one second.  Because this measure is

identical with the measure of frequency, the unit of bandwidth is

the Hertz, or Hz.  Display memory bandwidth therefore describes

the number of units of data which can be transferred into or out

of the display memory, in one second.  It provides a broad measure

of display unit capability.  However, like all broad measures, it

should not be taken too seriously.

<7>      CRTC Register Protect

---------------------

This bit is cleared by hardware RESET.

<=0>  Enable writes to CRTC Registers at indexes 0 .. 7.

<=1>  Disable writes to CRTC Registers at indexes 0 .. 7.

The ability to protect certain important register groups can assist

with emulation, by preventing programs from changing critical data.

Typical use allows direct register writes from software, by keeping

registers which cannot be changed intact, whilst permitting others

to be modified.  This is an emulation issue only, so is of little

importance for new software architecture implementations.

See {6.05.02} Horizontal Total Register.                 {0}

See {6.05.03} Horizontal Display Enable End Register.    {1}

See {6.05.04} Start Horizontal Blanking Register.        {2}

See {6.05.05} End Horizontal Blanking Register.          {3}

See {6.05.06} Start Horizontal Retrace Pulse Register.   {4}

See {6.05.07} End Horizontal Retrace Register.           {5}

See {6.05.08} Vertical Total Register.                   {6}

See {6.05.09} CRT Controller Overflow Register.          {7}

The following is not true for the Trident 8900 chip.

The following is valid for the Chips 82C456 chip.

The Vertical Retrace End Register can be made invisible at this

index address, when the Lightpen Low Register is placed at this

same register index.

For details see the Light Pen Register Enable description.

See {6.05.05} End Horizontal Blanking Register.

6.05.20  Vertical Display Enable End Register     { 3?5h    12h    R/W }

=======  ===============================================================

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.07.19} Overscan Colour Register.

Cross reference with {6.10.12} Clear Vertical Display Enable Register.

<7..0>   Vertical Display Enable End

---------------------------

This binary value represents the eight lower order bits of a ten bit

width Vertical Display Enable End value.  This ten bit value is used

in conjunction with the character row counter.  When a match between

these two values occurs, the active portion of the display finishes.

This happens when the current display scan line runs into the first

non-display scan line.   At that time, the display unit produces the

colour specified by the Overscan Colour Register.  Typically several

scan lines, termed overscan lines, are produced both at a frame end

and at a frame start.  The two high order bits are stored elsewhere.

See {6.05.09} CRT Controller Overflow Register.

See {6.07.19} Overscan Colour Register.

The active part of the display image is produced from the time when

the character row counter has the value zero, representing the first

scan line of the image proper, through to the value in this register.

The only time this is not true is when the vertical display enable

state is cleared asynchronously by special command.

See {6.10.12} Clear Vertical Display Enable Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

A discussion of vertical timing and image generation is available,

with a timing diagram.

See {6.05.08} Vertical Total Register.

6.05.21  Offset Register                          { 3?5h    13h    R/W }

=======  ===============================================================

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

<7..0>   Display Image Offset

--------------------

The value of this field defines the image width for the complete image

retained in the image memory.  This width is typically the same width

as that of the image display, or possibly a greater width.

This value employs a memory addressing unit of variable size according

to the units of address employed by the display memory address counter

determined by option settings:

Display Memory Address Unit 2

See {6.05.25} CRTC Mode Control Register.

Display Memory Address Unit 4

See {6.05.22} Underline Location Register.

Display Memory Address Unit 4

=----------------------------

Display Memory Address Unit 2

=    =----------------------------

Unit of display addressing

=    =    ==========================--------------------

0    0    Word mode addressing          { 2 byte units }

0    1    Byte mode addressing          { 1 byte units }

1    0    Double word mode addressing   { 4 byte units }

1    1    Double word mode addressing   { 4 byte units }

--------------------------------------------------------

Whenever a new display scan line commences the current display address

is adjusted to point to the data representing the new scan line.  This

might involve skipping some of the data lying within the previous scan

line.  This capability is provided so that an image much too large for

display in a single screen can be maintained and manipulated easily in

image memory.

The screen can be made to display different portions of a much larger

image by means of panning, in both the horizontal and vertical planes.

In fact, for panning to work effectively, there must be a larger image

present in image memory than that which can actually be displayed.

See {6.05.10} Preset Row Scan Register.

There is a circumstance where the value of the current display address

register is not dependant upon the Display Image Offset value.  There

is a facility which can be used to provide two windows within a single

display screen.  The lower window can be made immune from panning.  In

this situation, the display address is reset to zero.

See {6.05.10} Preset Row Scan Register.

Note that the display memory address counter is initialised with

the value of the Start Address Register, which automatically has

a value expressed in the same addressing units.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

6.05.22  Underline Location Register              { 3?5h    14h    R/W }

=======  ===============================================================

Cross reference with {6.04.05} Character Map Select Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.21} Offset Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.10.04} Mode Control Register 2.

<4..0>   Underline Location

------------------

This field specifies which character font relative scan line will be

used to display an underline invoked by a character attribute in the

Monochrome Graphics Adapter, Colour Graphics Adapter or the Hercules

modes.  The first scan line in a font is numbered zero.  The feature

operates only in text mode.

See {6.06.08} Miscellaneous Register.

The underline attribute appears as follows in these modes.

See {6.10.04} Mode Control Register 2.

Character Attribute Byte, Monochrome Emulation Mode

===================================================

For monochrome emulation mode, set the Display Generation Mode.

See {6.07.18} Attribute Mode Control Register.

<2..0>  Foreground

----------

<=000>  Black

<=001>  Underline

<=111>  White

<3>     Intensity

---------

<=0>  Normal

<=1>  High Intensity {alternatively character set select}

See {6.04.05} Character Map Select Register.

<6..4>  Background

----------

<=000>  Black

<=111>  White

<7>     Blink

-----

<=0>  Normal

<=1>  Blink {alternatively high intensity select}

See {6.07.18} Attribute Mode Control Register.

Character Attribute Byte, Colour Emulation Mode

===============================================

For colour emulation mode, set the Display Generation Mode.

See {6.07.18} Attribute Mode Control Register.

<0>     Foreground Blue

---------------

<=0>  No blue colour

<=1>  Output blue colour

<1>     Foreground Green

----------------

<=0>  No green colour

<=1>  Output green colour

<2>     Foreground Red

--------------

<=0>  No red colour

<=1>  Output red colour

<3>     Intensity

---------

<=0>  Normal

<=1>  High Intensity {alternatively character set select}

See {6.04.05} Character Map Select Register.

<4>     Background Blue

---------------

<=0>  No blue colour

<=1>  Output blue colour

<5>     Background Green

----------------

<=0>  No green colour

<=1>  Output green colour

<6>     Background Red

--------------

<=0>  No red colour

<=1>  Output red colour

<7>     Blink

-----

<=0>  Normal

<=1>  Blink {alternatively high intensity select}

See {6.07.18} Attribute Mode Control Register.

<5>      Display Address Dwell Count 4

-----------------------------

The display memory address counter increments the memory address at a

rate dependant upon the character clock frequency.  It is possible to

increment the display memory address every character clock, or permit

a number of character clock periods to elapse before the increment is

put into effect.  When the display address does not change with every

change of character clock, the display address experiences dwell.  It

is possible to change the dwell through option settings:

Display Address Dwell Count 4  {This.}

Display Address Dwell Count 2

See {6.05.25} CRTC Mode Control Register.

Display Address Dwell Count 4  {This.}

=----------------------------

Display Address Dwell Count 2

=    =----------------------------

Unit of display address dwell

=    =    =============================----------------------

0    0    Dwell = 1    { 1 character clocking per increment }

0    1    Dwell = 2    { 2 character clocking per increment }

1    0    Dwell = 4    { 4 character clocking per increment }

1    1    Dwell = 2    { 2 character clocking per increment }

-------------------------------------------------------------

<=0>  Provide a single display address increment after each character

clock period.  This provides a new image memory address for all

new character clock periods.

If the image memory is arranged in multiplane format, this will

permit all four planes to be read at the beginning of every new

character period.  This is the usual way to access image memory

in 16 colour multiplane format.  It does not waste image space.

The alternative facilities provided elsewhere are dominant over

this option setting.  See the table above.

See {6.05.25} CRTC Mode Control Register.

<=1>  Specifies that four character clock periods must elapse before

the display address increment takes place.  Therefore a single

image memory address is issued with four consecutive character

clocks.

This is useful where the first character clock loaded four data

planes using the new address and it is simply required that any

new address generated be generated four character clocks later.

In this case the intention is that the first clock of the group

will load data whilst the remaining three clocks remain unused.

This would be the case where the unit of addressing of the image

memory has been changed, so permitting a much larger image to be

displayed, without needing a display memory address counter with

an increased number of bits.

See Display Memory Address Unit 4, immediately below.

Alternatively, all image planes could be loaded at an identical

image memory address on each new character clock, providing for

repeated presentation of image data.  The image produced depends

upon the image format, either multiplane or packed pixel.  Each

situation would be examined individually for appropriate merit.

The alternative facilities provided elsewhere are dominant over

this option setting.  See the table above.

See {6.05.25} CRTC Mode Control Register.

<6>      Display Memory Address Unit 4

-----------------------------

The display memory address counter employs a memory addressing

unit of variable size according to option settings:

Display Memory Address Unit 4  {This.}

Display Memory Address Unit 2

See {6.05.25} CRTC Mode Control Register.

Display Memory Address Unit 4  {This.}

=----------------------------

Display Memory Address Unit 2

=    =----------------------------

Unit of display addressing

=    =    ==========================--------------------

0    0    Word mode addressing          { 2 byte units }

0    1    Byte mode addressing          { 1 byte units }

1    0    Double word mode addressing   { 4 byte units }

1    1    Double word mode addressing   { 4 byte units }

--------------------------------------------------------

<=0>  Provide no bit shift between the display memory address counter

and the image memory address lines.  This means that an address

is generated into the display memory with granularity specified

by Display Memory Address Unit 2.  See the table above.

See {6.05.25} CRTC Mode Control Register.

With this option selected the bit width of the display memory

address counter determines the maximum number of bytes in the

display image.  No further bytes can contribute to the image,

since the image address space is fixed by this finite counter

width.  This can represent a problem with large images, using

packed pixel display format with 256 colours, since each byte

represents one pixel.  With 18 bits of address counter, image

size is restricted to 256K/512K which may not be adequate.

When the display generation employs multiplane mode, the 256K

address capability is sufficient to give full access to every

pixel that can be stored in the image memory.  This is because

with 1024K of image memory, arranged in four parallel planes,

256K of address capacity covers the entire address space.

Note that the display memory address counter is initialised with

the value of the Start Address Register, which automatically has

a value expressed in the same addressing units.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

<=1>  Left shift the display memory address counter by 2 bits before

using the resulting address to access display memory.  In this

way, each address is multiplied by four, so becoming a double

word address.  This provides an effective increase of two bits

in the addressing capability of the memory address counter.

This option increases the maximum display image size to four

times the original limit, which enables larger packed pixel

images to be displayed at high resolution in 256 colours.

Since the display memory address counter is 18 bits width, the

adoption of two additional bits of addressing span provide for

20 bits of total addressing capability.  This gives sufficient

capacity to address a full 1024K display memory.

A side effect of this greater memory addressing span is that it

becomes impossible to address image data in a unit smaller than

a group consisting of 4 bytes.  As a direct consequence of this

the display hardware must always use load a single 4 byte group

at each new display memory address counter value.  Only if this

is done can the full image memory capacity be realised. In fact

this method of addressing is ideally suited to packed pixel 256

colour mode.

Note that the display memory address counter is initialised with

the value of the Start Address Register, which automatically has

a value expressed in the same addressing units.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

The alternative facilities provided elsewhere are inhibited with

this option setting.  See the table above.

See {6.05.25} CRTC Mode Control Register.

<7>      Reserved.  Always <=0>.

-----------------------

6.05.23  Start Vertical Blanking Register         { 3?5h    15h    R/W }

=======  ===============================================================

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   Start Vertical Blanking

-----------------------

This binary field specifies the lower order 8 bits of the 10 bit width

value representing the scan line at which vertical blanking commences.

The two remaining bits are stored elsewhere.

See {6.05.11} Maximum Scan Line Register.

See {6.05.09} CRT Controller Overflow Register.

The 10 bit value is compared with the character row counter until the

values match.  The character row counter counts all active scan lines

in the frame, commencing at zero, which is the first scan line to use

image data in the display.

When the match occurs the vertical blanking period begins.  During the

blanking period the display electron beams which illuminate the screen

are temporarily inhibited, for the entire period of vertical retrace.

See {6.05.18} Vertical Retrace Start Register.

See {6.05.19} Vertical Retrace End Register.

The vertical retrace sends the notional position of the electron beams

to the top left of the screen.  As this would damage the display image

if the electron beams were operational, blanking prevents any problems

of image quality by inhibiting the electron beams during retrace.

The length of the blanking period is determined using a terminal value

to indicate that blanking should finish.

See {6.05.24} End Vertical Blanking Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

A discussion of vertical display timing is available together with a

timing diagram.

See {6.05.08} Vertical Total Register.

6.05.24  End Vertical Blanking Register           { 3?5h    16h    R/W }

=======  ===============================================================

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

<7..0>   End Vertical Blanking

---------------------

This specifies a compare value for use with the character row counter

giving the display scan line at which the vertical blanking period is

deemed to be complete.  During the vertical blanking period, vertical

retrace will have fully taken place.

See {6.05.18} Vertical Retrace Start Register.

See {6.05.19} Vertical Retrace End Register.

The character row counter is actually of 10 bits width but the compare

does not take place until after the vertical blanking period has begun

so there is no ambiguity in operation.  Therefore the 8 value bits are

sufficient under these circumstances to specify upto 255 scan lines of

vertical blanking period.  This is such a large number it would never

be used in practise.

See {6.05.23} Start Vertical Blanking Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

A discussion of vertical display timing is available together with a

timing diagram.

See {6.05.08} Vertical Total Register.

6.05.25  CRTC Mode Control Register               { 3?5h    17h    R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.20} Vertical Display Enable End Register.

Cross reference with {6.05.21} Offset Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

A single bit <2> only is affected.  {Horizontal Retrace Divisor.}

See {6.10.07} CRTC Module Testing Register.

<0>      Display Address 13 Remap Row 0

------------------------------

<=0>  Replace the display memory address counter bit 13 with the

character row counter bit 0.  This means that for even row

scan line generation, image data addresses are not offset.

Odd row scan lines incur an address offset with equivalent

value of 8K bytes.  In this way there is an 8K offset with

each alternate scan line display.  This facility provides

an emulation capability for Colour Graphics Adapter image

memory layout.  It is of little interest for construction

of new software which aims for high execution speeds.

See also Display Address 14 Remap Row 1, below.

<=1>  No modification to display memory address counter bit 13

takes place.  This is the normal setting for this option.

<1>      Display Address 14 Remap Row 1

------------------------------

<=0>  Replace the display memory address counter bit 14 with the

character row counter bit 1.  In this way, the pair number

of the scan line being displayed changes the image memory

address being used to generate the display.  On each pair

change, an equivalent memory offset of 16K is introduced.

This option is usually employed in conjunction with that

of Display Address 13 Remap Row 0, in order to provide a

four bank image memory layout.  This is usually used for

Hercules adapter emulation.  It is of little interest in

terms of new software aiming for high speed operation.

<=1>  No modification to display memory address counter bit 14

takes place.  This is the normal setting for this option.

<2>      Horizontal Retrace Divisor

--------------------------

<=0>  This option selects horizontal retrace as the source of the

decision to increment the character row counter which keeps

track of the scan line being generated for display.  Normal

operation of the generation process takes place using this

option setting.  The character row counter increments after

every scan line of the active image is completed.

<=1>  This option counts two horizontal retrace events before the

increment of the character row counter takes place.  After

two scan lines have completed, the character row counter is

incremented by only one scan line.  In this way, the timing

period for all vertical timing is increased to double the

usual period.  At the same time, the maximum number of scan

lines resolvable in the 10 bits width character row counter

increases from 1024 to 2048.  This can be used to create a

display with much higher vertical resolution.

See {6.05.08} Vertical Total Register.

See {6.05.18} Vertical Retrace Start Register.

See {6.05.19} Vertical Retrace End Register.

See {6.05.20} Vertical Display Enable End Register.

See {6.05.23} Start Vertical Blanking Register.

See {6.05.24} End Vertical Blanking Register.

See {6.05.26} Line Compare Register.

While this mode is operational, the display memory address

counter operation is unaffected and so behaves as normal.

This means that the image data is always read from display

memory with continually increasing addresses.  Every scan

line is built using image data unique to that scan line.

Warning: do not confuse this capability with that provided

by the Double Scan Line Display.

See {6.05.11} Maximum Scan Line Register.

<3>      Display Address Dwell Count 2

-----------------------------

The display memory address counter increments the memory address at a

rate dependant upon the character clock frequency.  It is possible to

increment the display memory address every character clock, or permit

a number of character clock periods to elapse before the increment is

put into effect.  When the display address does not change with every

change of character clock, the display address experiences dwell.  It

is possible to change the dwell through option settings:

Display Address Dwell Count 4

See {6.05.22} Underline Location Register.

Display Address Dwell Count 2  {This.}

Display Address Dwell Count 4

=----------------------------

Display Address Dwell Count 2  {This.}

=    =----------------------------

Unit of display address dwell

=    =    =============================----------------------

0    0    Dwell = 1    { 1 character clocking per increment }

0    1    Dwell = 2    { 2 character clocking per increment }

1    0    Dwell = 4    { 4 character clocking per increment }

1    1    Dwell = 2    { 2 character clocking per increment }

-------------------------------------------------------------

<=0>  Provide a single display address increment after each character

clock period.  This provides a new image memory address for all

new character clock periods.

If the image memory is arranged in multiplane format, this will

permit all four planes to be read at the beginning of every new

character period.  This is the usual way to access image memory

in 16 colour multiplane format.  It does not waste image space.

The alternative facilities provided elsewhere are available with

this option setting.  See table above.

See {6.05.22} Underline Location Register.

<=1>  Specifies that two character clock periods must elapse before a

new display memory address is produced by increment.  Thus just

one new display memory address is produced each two consecutive

character clocks.

This is useful where the first character clock loaded two data

planes using the new address and it is simply required that any

new address generated be generated two character clocks later.

In this case the intention is that the first clock of the group

will load data while the remaining clock period remains unused.

This would be the case where the unit of addressing of the image

memory has been changed, so permitting a much larger image to be

displayed, without needing a display memory address counter with

an increased number of bits.

See Display Memory Address Unit 2, below.

The alternative facilities provided elsewhere are dominated by

this option setting.  See table above.

See {6.05.22} Underline Location Register.

<4>      Reserved.  Always <=0>.

-----------------------

<5>      Display Address Unit 2 Remap

----------------------------

<=0>  When option Display Memory Address Unit 2 {see below} is active,

this selection specifies that the display memory address counter

bit 0 will take on the value of memory address counter bit 13.

Address counter bit 13 measures addresses in terms of 8K units.

One 8K unit is sometimes termed a bank.

The effect is to double the address span which can be achieved,

by using bit 0 to specify which memory bank to select.  In word

address mode, bit 0 is never used for address generation since

byte addresses are always incremented by 2.  Therefore bit 0 is

always left unchanged.

When display image memory is organised as multiple planes, each

of the four planes shares an identical, parallel address space.

In this situation, each word address accesses all the planes in

parallel, by using what amounts to an even byte address.  When

the increment occurs the odd byte address situated between the

two even addresses is skipped, so remaining inaccessible.

This has the effect of wasting half the address space.  To avoid

such waste, the display memory address counter bit 0 is used to

differentiate between even and odd numbered byte addresses.  The

display unit will only access even byte addresses initially, but

after reading half an 8K bank, will proceed to read that bank by

accessing only odd byte addresses.  In this way, the bank space

is effectively doubled.

The option of selecting bit 13 is decided by the maximum amount

of image memory present.  Were only 64K bytes available, there

would be 16K of multiplane memory address space present with 4

bytes at each memory address.  In this situation 14 effective

memory address counter bits <13..0> would provide 16K units of

addressing.  With this option selected, that enables the whole

of the image memory to be used for image generation.

This option simply changes the significance and manner of use of

the display memory address counter bits.  It should be used only

for emulation purposes.  Note that use of this option cannot be

entirely avoided when the Display Memory Address Unit 2 option

is active.

<=1>  When option Display Memory Address Unit 2 {see below} is active,

this selection specifies that the display memory address counter

bit 0 will take on the value of memory address counter bit 15.

Address counter bit 15 measures addresses in terms of 32K units.

One 32K unit is sometimes termed a bank.

The effect is to double the address span which can be achieved,

by using bit 0 to specify which memory bank to select.  In word

address mode, bit 0 is never used for address generation since

byte addresses are always incremented by 2.  Therefore bit 0 is

always left unchanged.

When display image memory is organised as multiple planes, each

of the four planes shares an identical, parallel address space.

In this situation, each word address accesses all the planes in

parallel, by using what amounts to an even byte address.  When

the increment occurs the odd byte address situated between the

two even addresses is skipped, so remaining inaccessible.

This has the effect of wasting half the address space.  To avoid

such waste, the display memory address counter bit 0 is used to

differentiate between even and odd numbered byte addresses.  The

display unit will only access even byte addresses initially, but

after reading half a 32K bank, will proceed to read that bank by

accessing only odd byte addresses.  In this way, the bank space

is effectively doubled.

The option of selecting bit 15 is decided by the maximum amount

of image memory present.  Where 256K bytes is available, there

would be 64K of multiplane memory address space present with 4

bytes at each memory address.  In this situation 16 effective

memory address counter bits <15..0> would provide 64K units of

addressing.  With this option selected, that enables the whole

of the image memory to be used for image generation.

This option simply changes the significance and manner of use of

the display memory address counter bits.  It should be used only

for emulation purposes.  Note that use of this option cannot be

entirely avoided when the Display Memory Address Unit 2 option

is active.

<6>      Display Memory Address Unit 2

-----------------------------

The display memory address counter employs a memory addressing

unit of variable size according to option settings:

Display Memory Address Unit 2  {This.}

Display Memory Address Unit 4

See {6.05.22} Underline Location Register.

Display Memory Address Unit 4

=----------------------------

Display Memory Address Unit 2  {This.}

=    =----------------------------

Unit of display addressing

=    =    ==========================--------------------

0    0    Word mode addressing          { 2 byte units }

0    1    Byte mode addressing          { 1 byte units }

1    0    Double word mode addressing   { 4 byte units }

1    1    Double word mode addressing   { 4 byte units }

--------------------------------------------------------

<=0>  Left shift the display memory address counter by 1 bit, before

using the resulting address to access display memory.  In this

way, each address is multiplied by two, thus becoming a double

byte address.  This provides an apparent effective increase of

one bit in the memory address counter addressing capability.

However, it also fails to use bit 0 of the resulting effective

memory address, which would decrease the size of the resulting

address space.  There is automatic capability to use the extra

bit 0 to increase the size of the address space.  Note that it

has the effect of returning the address space size to the same

size it was before Display Memory Address Unit 2 became active.

Thus this option simply changes the interpretation attributed

to the display memory address counter, and does not increase

the address range generated.  However, the organisation of the

address space is changed.  Use this facility with due caution,

probably only for emulation purposes.

See Display Address Unit 2 Remap, immediately above.

This option is often termed word address mode, since two bytes

make up one single word.

Note that the display memory address counter is initialised with

the value of the Start Address Register, which automatically has

a value expressed in the same addressing units.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

The alternative facilities provided elsewhere are dominant over

this option setting.  This is shown clearly in the table above.

See {6.05.22} Underline Location Register.

<=1>  Provide no bit shift between the display memory address counter

and the image memory address lines.  This means that an address

is generated into the display memory with a granularity of one

single byte.  This option is often termed byte address mode.

With this option selected the bit width of the display memory

address counter determines the maximum number of bytes in the

display image.  No further bytes can contribute to the image,

since the image address space is fixed by this finite counter

width.  This can represent a problem with large images, using

packed pixel display format with 256 colours, since each byte

represents one pixel.  With 18 bits of address counter, image

size is restricted to 256K.

Note that the display memory address counter is initialised with

the value of the Start Address Register, which automatically has

a value expressed in the same addressing units.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

The alternative facilities provided elsewhere are dominant over

this option setting.  This is shown clearly in the table above.

See {6.05.22} Underline Location Register.

<7>      Horizontal/Vertical Retrace Reset

---------------------------------

<=0>  Reset the horizontal and vertical retrace signals to the

display unit.

<=1>  Generate horizontal and vertical retrace signals to the

display unit.  This is the usual setting for the option.

A note on addresses produced by the display memory address counter

==================================================================

Addresses are generated by the display memory address counter, but

these addresses are then mapped to the actual memory address lines

before being used.  The table shown below gives the detail of this

mapping.

Physical memory address used for display generation

====-----------------------------------------------

Display Memory Address Counter Bit

====    -------------------------------------------------

|           |           |

|           |           |

Byte Mode Addressing    |           |           |

====    ====---------------------           |           |

|           |

|           |

Word Mode Addressing        |           |

====    ====    ====-------------------------           |

|

|

Double Word Mode Addressing     |

====    ====    ====    ====-----------------------------

Comments

====    ====    ====    ====    ------------------------------------

PA00    MA00     [1]     [2]    [1] is defined by the option

Display Address Unit 2 Remap

PA01    MA01    MA00     [3]    see above <5>.  This is only

valid in word address mode.

PA02    MA02    MA01    MA00

PA03    MA03    MA02    MA01

[2] is defined by the display

PA04    MA04    MA03    MA02    mode in use.

PA05    MA05    MA04    MA03    See Shift Load Two Control

{6.04.03} Clocking Mode Register

PA06    MA06    MA05    MA04    See Shift Load Four Control

{6.04.03} Clocking Mode Register

PA07    MA07    MA06    MA05

PA08    MA08    MA07    MA06

[3] is defined by the display

PA09    MA09    MA08    MA07    mode in use.

PA10    MA10    MA09    MA08    See Shift Load Two Control

{6.04.03} Clocking Mode Register

PA11    MA11    MA10    MA09    See Shift Load Four Control

{6.04.03} Clocking Mode Register

PA12    MA12    MA11    MA10

PA13    MA13    MA12    MA11

PA14    MA14    MA13    MA12

PA15    MA15    MA14    MA13

PA16    MA16    MA15    MA14

PA17    MA17    MA16    MA15

PA18    ----    MA17    MA16

PA19    ----    ----    MA17

--------------------------------------------------------------------

6.05.26  Line Compare Register                    { 3?5h    18h    R/W }

=======  ===============================================================

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.07.21} Horizontal PEL Panning Register.

<7..0>   Line Compare

------------

This binary field represents the lower eight bits of the full 10 bit

wide value of the Line Compare Register.  The 10 bit value specifies

the scan line registered by the character row counter when a change

should be made to a second display window in the two window screen.

The higher order bits are held elsewhere.

See {6.05.09} CRT Controller Overflow Register.

See {6.05.11} Maximum Scan Line Register.

The two windows are distinguished in that the first always specifies

a non-zero starting address into the image memory, and is subject to

panning control.  The second window always begins at a display image

memory address of zero, and may either be panned in the same manner

as the first window, or alternatively may be made immune to panning,

as determined by option setting.

When the match occurs with the character row counter, the display

memory address counter is reset to zero, ready for display of the

second, lower window.  Simultaneously, any panning controls which

were in operation for the upper window are optionally cancelled,

until the next frame display commences.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

See {6.07.18} Attribute Mode Control Register.

A discussion of pixel panning is available.

See {6.05.10} Preset Row Scan Register.

See {6.07.21} Horizontal PEL Panning Register.

A discussion of vertical display timing generation is available with

a timing diagram.

See {6.05.08} Vertical Total Register.

The units in which vertical timing is measured can be changed by a

factor of two upon option setting.

See {6.05.25} CRTC Mode Control Register.

6.06  Register specifications: Graphics Controller

====  ============================================

6.06.01  Graphics Address Register    { 3ceh    ---    R/W }

=======  ===================================================

<3..0>   Graphics Register Address

-------------------------

Port 3cfh accesses the register pointed to by the index number in this

bit field.  Index number zero is represented by <=0000>.

<7..4>   Reserved. Always <=0000>.

-------------------------

6.06.02  Set/Reset Register           { 3cfh    00h    R/W }

=======  ===================================================

Cross reference with {6.04.04} Map Mask Register.

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.06.03} Enable Set/Reset Register.

Cross reference with {6.06.05} Data Rotate Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.10} Bit Mask Register.

Cross reference with {6.10.09} CPU Latch Read Back Register.

<0>      Set/Reset Data Image Plane 0

----------------------------

During multiplane image memory write access operation by the processor

this bit helps to determine the value written to the byte lying within

display memory image plane 0.

<=0>  The data value 0 is selected for image plane zero, so long as

the plane is suitably enabled for this operation.

See {6.06.03} Enable Set/Reset Register.

<=1>  The data value 1 is selected for image plane zero, so long as

the plane is suitably enabled for this operation.

See {6.06.03} Enable Set/Reset Register.

The precise effect of this bit is determined by the write mode used to

perform the update operation.  There are four different write modes as

follows:

Write Mode 0

============

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

Processor data for write arrive as an 8 bit wide value.  These bits

are rotated.  The rotated bits are copied in parallel to data paths

8 bits wide associated with each of the four image memory planes.

Where Set/Reset Enable Image Plane 0 is set <=1>, all eight data path

bits associated with image plane 0 are set to just one value given by

the single bit Set/Reset Data Image Plane 0.  All other memory planes

are processed in an identical manner.  Four output data paths result,

each of eight bits width, making a total of 32 bits.

These result data paths are then operated upon by a logic unit with

capability to perform combining operations with corresponding eight

bit contents of four read latches, one each per image memory plane.

The read latches must be well defined for this processing to be of

any value.  The combining operations are NO_OP, AND, OR, XOR.  The

NO_OP passes the data through unchanged, ignoring read latch data,

but the read latches are updated with the data as a consequence.

Thereafter, each plane in parallel has the result of the previously

described processing combined with the contents of the read latches

(which must previously have been defined using a read operation).

This combining operation selects for each image plane data path, and

for each of eight bits in one data path, the value of the processing

path bit or the value of the read latch bit, in identical position.

The selection is based on all eight bits of a bit mask register used

specially for just this purpose.  Any given bit position in the mask

register will select either the process data path or the read latch

output path, in identical manner, for the corresponding bit position

in each of the four image planes.

The output from this operation is another 32 bit wide data path, made

up of four eight bit wide paths, one for each of four image planes.

This final output data is written in a single write operation to each

corresponding image plane in parallel, subject to plane write inhibit

state.  Each plane can be write inhibited in complete independance of

any other plane.  A write inhibit state prevents any write operations

from modifying data within the corresponding image plane.

Write mode 3 processes for relative colour.  By contrast, write mode

0 should be seen as processing in terms of absolute colour.

The use of the Set/Reset Enable and Set/Reset Data input is to specify

a four bit colour value across a single bit position within all planes

of the image memory, in parallel.  In this way, the processor supplied

data is either used unchanged, is modified partially, or is completely

ignored when deciding how to process for update.  The resulting output

is used by the logic unit together with the read latch contents.  This

provides extremely powerful colour processing capability when properly

used.  This write mode is probably the most generally used write mode.

See {6.06.07} Graphics Mode Register.

Schematic Overview of Data Path for Write Mode 0

================================================

================================================

Processor Data

===========---

<7654.3210>

|

[All Planes]

|

|

|

|             Multiplane

|             Rotation

|             Select

|             ==========-

|             <7654.3210>

|                  |

.|             [All planes]

|                   |

|                   |

| ..................|

| |

| |

| |

Data Rotation

===========--

<7654.3210>

|

[All Planes]

|

|

|

| Copy all data into four identical data paths

|.........................................................

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

.....|             .....|             .....|             .....|

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|  Force Value     |  Force Value     |  Force Value     |  Force Value

|  Enable bit      |  Enable bit      |  Enable bit      |  Enable bit

|  ===========     |  ===========     |  ===========     |  ===========

|       |          |       |          |       |          |       |

|   [Plane 0]      |   [Plane 1]      |   [Plane 2]      |   [Plane 3]

|       |          |       |          |       |          |       |

|       |          |       |          |       |          |       |

| ......|          | ......|          | ......|          | ......|

| |                | |                | |                | |

| |                | |                | |                | |

| |                | |                | |                | |

| |  Force Value   | |  Force Value   | |  Force Value   | |  Force Value

| |  Data bit      | |  Data bit      | |  Data bit      | |  Data bit

| |  ===========   | |  ===========   | |  ===========   | |  ===========

| |       |        | |       |        | |       |        | |       |

| |   [Plane 0]    | |   [Plane 1]    | |   [Plane 2]    | |   [Plane 3]

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| |...    |        | |...    |        | |...    |        | |...    |

|    |    |        |    |    |        |    |    |        |    |    |

|    |    |        |    |    |        |    |    |        |    |    |

|... | ...|        |... | ...|        |... | ...|        |... | ...|

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

Force Value        Force Value        Force Value        Force Value

of each bit        of each bit        of each bit        of each bit

to the same        to the same        to the same        to the same

equal value        equal value        equal value        equal value

===========        ===========        ===========        ===========

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

...|               ...|               ...|               ...|

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|   Read Latch 0   |   Read Latch 1   |   Read Latch 2   |   Read Latch 3

|   ===========-   |   ===========-   |   ===========-   |   ===========-

|   <7654.3210>    |   <7654.3210>    |   <7654.3210>    |   <7654.3210>

|        |         |        |         |        |         |        |

|    [Plane 0]     |    [Plane 1]     |    [Plane 2]     |    [Plane 3]

|        |         |        |         |        |         |        |

|        |         |        |         |        |         |        |

|.. .....|         |.. .....|         |.. .....|         |.. .....|

| |                | |                | |                | |

| |                | |                | |                | |

| |                | |                | |                | |

| |  Multiplane    | |  Multiplane    | |  Multiplane    | |  Multiplane

| |  Function      | |  Function      | |  Function      | |  Function

| |  Select        | |  Select        | |  Select        | |  Select

| |  ===========   | |  ===========   | |  ===========   | |  ===========

| |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>

| |       |        | |       |        | |       |        | |       |

| |  [All planes]  | |  [All planes]  | |  [All planes]  | |  [All planes]

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| | ......|        | | ......|        | | ......|        | | ......|

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

Logic Block        Logic Block        Logic Block        Logic Block

...NO_OP...        ...NO_OP...        ...NO_OP...        ...NO_OP...

...AND.....        ...AND.....        ...AND.....        ...AND.....

...OR......        ...OR......        ...OR......        ...OR......

...XOR.....        ...XOR.....        ...XOR.....        ...XOR.....

===========        ===========        ===========        ===========

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

.......|           .......|           .......|           .......|

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|   Read Latch 0   |   Read Latch 1   |   Read Latch 2   |   Read Latch 3

|   ===========-   |   ===========-   |   ===========-   |   ===========-

|   <7654.3210>    |   <7654.3210>    |   <7654.3210>    |   <7654.3210>

|        |         |        |         |        |         |        |

|    [Plane 0]     |    [Plane 1]     |    [Plane 2]     |    [Plane 3]

|        |         |        |         |        |         |        |

|        |         |        |         |        |         |        |

| .......|         | .......|         | .......|         | .......|

| |                | |                | |                | |

| |                | |                | |                | |

| |                | |                | |                | |

| |  Multiplane    | |  Multiplane    | |  Multiplane    | |  Multiplane

| |  Select Bit    | |  Select Bit    | |  Select Bit    | |  Select Bit

| |  Definition    | |  Definition    | |  Definition    | |  Definition

| |  ===========   | |  ===========   | |  ===========   | |  ===========

| |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>

| |       |        | |       |        | |       |        | |       |

| |  [All Planes]  | |  [All Planes]  | |  [All Planes]  | |  [All Planes]

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| | ......|        | | ......|        | | ......|        | | ......|

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

Every output       Every output       Every output       Every output

bit selected       bit selected       bit selected       bit selected

using choice       using choice       using choice       using choice

for that bit       for that bit       for that bit       for that bit

===========-       ===========-       ===========-       ===========-

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|  Plane 0         |  Plane 1         |  Plane 2         |  Plane 3

|  Inhibit         |  Inhibit         |  Inhibit         |  Inhibit

|   Write          |   Write          |   Write          |   Write

|  =======         |  =======         |  =======         |  =======

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

-----------        -----------        -----------        -----------

Write Logic        Write Logic        Write Logic        Write Logic

for Plane 0        for Plane 1        for Plane 2        for Plane 3

Memory             Memory             Memory             Memory

-----------        -----------        -----------        -----------

Key to schematic for Write Mode 0

=================================

--------------------------

Multiplane Rotation Select

--------------------------

See {6.06.05} Data Rotate Register.

----------------------

Force Value Enable bit

----------------------

See {6.06.03} Enable Set/Reset Register.

--------------------

Force Value Data bit

--------------------

See {6.06.02} Set/Reset Register.  {This.}

--------------------------

Multiplane Function Select

--------------------------

See {6.06.05} Data Rotate Register.

--------------------------------

Multiplane Select Bit Definition

--------------------------------

See {6.06.10} Bit Mask Register.

-----------------------

Plane 'N' Inhibit Write

-----------------------

See {6.04.04} Map Mask Register.

Write Mode 1

============

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

There is no processor data involved in using write mode 1, however the

processor must generate a read memory or write memory operation to the

image memory space. This provides the control input required to effect

the data transfers which are provided by this mode.

Write mode 1 provides for a very fast multiplane image data movement

between one region of image memory to another region within the same

image memory.

This copy capability is very flexible.  It can be used to perform an

image copy, or an initialisation operation using constant image data

to specify the value for the complete image.  In both cases, 32 bits,

that is four bytes, one each per image plane are transferred, unless

writes are inhibited to a particular plane.  Any individual plane is

subject to write inhibit control, independantly of all other planes.

To perform a true copy, a read address is generated to the multiplane

image.  This will have the effect of loading four read latches with

data from each of the four image planes.

Then a write operation is generated onto the multiplane image.  This

gives the effect of writing each of four image planes using the read

latch data associated with each plane, in parallel.  In this way, a

single write is able to transfer 32 bits of data simultaneously.

This accounts for the large image transfer bandwidth available from

this write mode.

In order to initialise an image, it is necessary to load the four read

latches once and then to repeatedly write them into the required image

addresses.  This provides for very fast image initialisation.

There is no use made of the Set/Reset Enable and Set/Reset Data during

operations in write mode 1.

Write mode 1 is mentioned in connection with Graphics Memory Access

Mode and Processor Image Memory Access Mode Select.

See {6.04.06} Sequencer Memory Mode Register.

Schematic Overview of Data Path for Write Mode 1

================================================

================================================

Read Latch 0       Read Latch 1       Read Latch 2       Read Latch 3

===========-       ===========-       ===========-       ===========-

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|  Plane 0         |  Plane 1         |  Plane 2         |  Plane 3

|  Inhibit         |  Inhibit         |  Inhibit         |  Inhibit

|   Write          |   Write          |   Write          |   Write

|  =======         |  =======         |  =======         |  =======

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

-----------        -----------        -----------        -----------

Write Logic        Write Logic        Write Logic        Write Logic

for Plane 0        for Plane 1        for Plane 2        for Plane 3

Memory             Memory             Memory             Memory

-----------        -----------        -----------        -----------

Key to schematic for Write Mode 1

=================================

-----------------------

Plane 'N' Inhibit Write

-----------------------

See {6.04.04} Map Mask Register.

Write Mode 2

============

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

Processor data for write arrive as a 4 bit wide value.  Each of the

bits are associated with a corresponding image plane, so bit 0 will

provide data for use in conjunction with image plane 0, and so on.

Each bit is copied into a data path of eight bits with each of these

eight bits taking on the same value as the original bit.  The result

is four data paths of eight bits, one data path for each of the four

image planes.  Within a data path all eight bits are the same value,

determined by the corresponding bit in the four bit processor data.

These result data paths are then operated upon by a logic unit with

capability to perform combining operations with corresponding eight

bit contents of four read latches, one each per image memory plane.

The read latches must be well defined for this processing to be of

any value.  The combining operations are NO_OP, AND, OR, XOR.  The

NO_OP passes the data through unchanged, ignoring read latch data.

Thereafter, each plane in parallel has the result of the previously

described processing combined with the contents of the read latches

(which must previously have been defined using a read operation).

This combining operation selects for each image plane data path, and

for each of eight bits in one data path, the value of the processing

path bit or the value of the read latch bit, in identical position.

The selection is based on all eight bits of a bit mask register used

specially for just this purpose.  Any given bit position in the mask

register will select either the process data path or the read latch

output path, in identical manner, for the corresponding bit position

in each of the four image planes.

The output from this operation is another 32 bit wide data path, made

up of four eight bit wide paths, one for each of four image planes.

This final output data is written in a single write operation to each

corresponding image plane in parallel, subject to plane write inhibit

state.  Each plane can be write inhibited in complete independance of

any other plane.  A write inhibit state prevents any write operations

from modifying data within the corresponding image plane.

There is no use made of the Set/Reset Enable and Set/Reset Data during

operations in write mode 2.

See {6.06.07} Graphics Mode Register.

Schematic Overview of Data Path for Write Mode 2

================================================

================================================

Processor Data

===========---

<----.3210>

||||

||||......................................................

|||                                                      |

|||....................................                  |

||                                    |                  |

||..................                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

Repeat bit         Repeat bit         Repeat bit         Repeat bit

value over         value over         value over         value over

all 8 bits         all 8 bits         all 8 bits         all 8 bits

==========-        ==========-        ==========-        ==========-

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

....|              ....|              ....|              ....|

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|   Read Latch 0   |   Read Latch 1   |   Read Latch 2   |   Read Latch 3

|   ===========-   |   ===========-   |   ===========-   |   ===========-

|   <7654.3210>    |   <7654.3210>    |   <7654.3210>    |   <7654.3210>

|        |         |        |         |        |         |        |

|    [Plane 0]     |    [Plane 1]     |    [Plane 2]     |    [Plane 3]

|        |         |        |         |        |         |        |

|        |         |        |         |        |         |        |

|.. .....|         |.. .....|         |.. .....|         |.. .....|

| |                | |                | |                | |

| |                | |                | |                | |

| |                | |                | |                | |

| |  Multiplane    | |  Multiplane    | |  Multiplane    | |  Multiplane

| |  Function      | |  Function      | |  Function      | |  Function

| |  Select        | |  Select        | |  Select        | |  Select

| |  ===========   | |  ===========   | |  ===========   | |  ===========

| |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>

| |       |        | |       |        | |       |        | |       |

| |  [All planes]  | |  [All planes]  | |  [All planes]  | |  [All planes]

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| | ......|        | | ......|        | | ......|        | | ......|

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

Logic Block        Logic Block        Logic Block        Logic Block

...NO_OP...        ...NO_OP...        ...NO_OP...        ...NO_OP...

...AND.....        ...AND.....        ...AND.....        ...AND.....

...OR......        ...OR......        ...OR......        ...OR......

...XOR.....        ...XOR.....        ...XOR.....        ...XOR.....

===========        ===========        ===========        ===========

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

.......|           .......|           .......|           .......|

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|   Read Latch 0   |   Read Latch 1   |   Read Latch 2   |   Read Latch 3

|   ===========-   |   ===========-   |   ===========-   |   ===========-

|   <7654.3210>    |   <7654.3210>    |   <7654.3210>    |   <7654.3210>

|        |         |        |         |        |         |        |

|    [Plane 0]     |    [Plane 1]     |    [Plane 2]     |    [Plane 3]

|        |         |        |         |        |         |        |

|        |         |        |         |        |         |        |

| .......|         | .......|         | .......|         | .......|

| |                | |                | |                | |

| |                | |                | |                | |

| |                | |                | |                | |

| |  Multiplane    | |  Multiplane    | |  Multiplane    | |  Multiplane

| |  Select Bit    | |  Select Bit    | |  Select Bit    | |  Select Bit

| |  Definition    | |  Definition    | |  Definition    | |  Definition

| |  ===========   | |  ===========   | |  ===========   | |  ===========

| |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>   | |  <7654.3210>

| |       |        | |       |        | |       |        | |       |

| |  [All Planes]  | |  [All Planes]  | |  [All Planes]  | |  [All Planes]

| |       |        | |       |        | |       |        | |       |

| |       |        | |       |        | |       |        | |       |

| | ......|        | | ......|        | | ......|        | | ......|

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

| | |              | | |              | | |              | | |

Every output       Every output       Every output       Every output

bit selected       bit selected       bit selected       bit selected

using choice       using choice       using choice       using choice

for that bit       for that bit       for that bit       for that bit

===========-       ===========-       ===========-       ===========-

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|  Plane 0         |  Plane 1         |  Plane 2         |  Plane 3

|  Inhibit         |  Inhibit         |  Inhibit         |  Inhibit

|   Write          |   Write          |   Write          |   Write

|  =======         |  =======         |  =======         |  =======

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

-----------        -----------        -----------        -----------

Write Logic        Write Logic        Write Logic        Write Logic

for Plane 0        for Plane 1        for Plane 2        for Plane 3

Memory             Memory             Memory             Memory

-----------        -----------        -----------        -----------

Key to schematic for Write Mode 2

=================================

--------------------------

Multiplane Function Select

--------------------------

See {6.06.05} Data Rotate Register.

--------------------------------

Multiplane Select Bit Definition

--------------------------------

See {6.06.10} Bit Mask Register.

-----------------------

Plane 'N' Inhibit Write

-----------------------

See {6.04.04} Map Mask Register.

Write Mode 3

============

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

Processor write data is not used directly to determine image data in

this mode.  Instead the eight bit data written from the processor is

interpreted as a bit map with each bit corresponding to one pixel in

multiplane image memory.  The value of the bit determines the result

of a computation which ultimately provides the new pixel value to be

associated with the bit.

This computation permits a pixel to take on the value for background

colour or the value for foreground colour depending upon the written

bit value from the processor associated with the pixel.  This allows

for modifying selected portions of an image by means of relative use

of colour.  That is, the image can be modified such that the changes

are visible no matter what the previous underlying image colours. In

this way, important data can be written without fear of occlusion.

Write mode 3 processes for relative colour.  By contrast, write mode

0 should be seen as processing in terms of absolute colour.

Processor data for write arrive as an 8 bit wide value.  These 8 bits

are rotated.  The rotated bits are logically ANDed with eight bits of

a register specially provided for purposes of specifying which pixels

are to be modified, at a given image memory address.  The result then

provides pixel modify permission for 8 pixels as an 8 bit wide value.

This modify permission value is used later.

The four eight bit read latch outputs associated with the four image

memory planes are formed into four data paths, one per memory plane.

These data paths are used as input to a logic unit, described later.

Each of four bits in a specially provided register are used to define

the value of all eight bits of a data path associated with one memory

plane.  In this way, four bits define four data paths, and every data

path consists of eight bits, all of which share the same value. There

is a data path associated with each of the four memory image planes.

These result data paths are then operated upon by a logic unit with

capability to perform combining operations with corresponding eight

bit contents of four read latches, one each per image memory plane.

The read latches must be well defined for this processing to be of

any value.  The combining operations are NO_OP, AND, OR, XOR.  The

NO_OP passes the data through unchanged, ignoring read latch data.

Thereafter, each plane in parallel has the result of the previously

described processing combined with the contents of the read latches

according to the pixel modify permission bits derived above.  These

determine whether a pixel is preserved, or changed.  {We assume the

read latches were loaded from the same pixel group currently being

modifed.  Since the read latch data need not be defined this way it

is prudent to point out the assumption.}

This combining operation selects for each image plane data path, and

for each of eight bits in one data path, the value of the processing

path bit or the value of the read latch bit, in identical position.

The selection is based on all eight bits of a permission value used

specially for just this purpose.  This permission applies to all the

memory image planes, but affects only the corresponding bit position

in each plane.  Therefore each permission bit applies to four bits,

each of identical bit number in each image memory plane.  These four

bits will all take their value from the same data path chosen by the

common permission bit.  Note that a data path is associated with one

memory plane only.  This means that despite sharing a permission bit

the resulting bits in each plane will be of distinct value.  This is

a direct result of maintaining each data path in strict association

with one single memory plane at all times.

The output from this operation is another 32 bit wide data path, made

up of four eight bit wide paths, one for each of four image planes.

This final output data is written in a single write operation to each

corresponding image plane in parallel, subject to plane write inhibit

state.  Each plane can be write inhibited in complete independance of

any other plane.  A write inhibit state prevents any write operations

from modifying data within the corresponding image plane.

Write mode 3 processes for relative colour.  By contrast, write mode

0 should be seen as processing in terms of absolute colour.

There is no use made of the Set/Reset Enable during operations in

write mode 3.  However, despite this, the Set/Reset Data register

provides the value for Data Source in the write mode 3 schematic.

This value has the dimensions of a colour value but it is used in

a potentially complex manner.  See schematic below.

See {6.06.07} Graphics Mode Register.

See Set/Reset Enable Image Plane 0, below.

Schematic Overview of Data Path for Write Mode 3

================================================

================================================

Processor Data

===========---

<7654.3210>

|

[Permission]

|

|

|

|             Permission

|             Rotation

|             Select

|             ==========-

|             <7654.3210>

|                  |

|             [Permission]

|                  |

|                  |

| .................|

| |

| |

| |

Data Rotation                            Applicable

===========--                            Permission

<7654.3210>                              Selector

|                                   ===========

[Permission]                             <7654.3210>

|                                        |

|                                   [Permission]

|                                        |

|                                        |

|...................    .................|

|    |

|    |

|    |

Permission

Resolution

...AND....

==========

<7654.3210>

|

[Permission]

|

|

|

|...............................................

|

|

Multiplane                                                                |          |

Data Source                                                               |            |

===========                                                               |            |

<----.3210>                                                               |            |

||||                                                                |

||||                                                                |

||||......................................................          |

|||                                                      |          |

|||                                                      |          |

|||....................................                  |          |

||                                    |                  |          |

||                                    |                  |          |

||..................                  |                  |          |

|                  |                  |                  |          |

|                  |                  |                  |          |

|                  |                  |                  |          |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]      |

|                  |                  |                  |          |

|                  |                  |                  |          |

|                  |                  |                  |          |

Repeat bit         Repeat bit         Repeat bit         Repeat bit      |

value over         value over         value over         value over      |

all 8 bits         all 8 bits         all 8 bits         all 8 bits      |

==========-        ==========         ==========         ==========      |

<7654.3210>        <7654.3210         <7654.3210         <7654.3210      |

|                  |                  |                  |          |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]      |

|                  |                  |                  |          |

|                  |                  |                  |          |

.......|           .......|           .......|           .......|          |

|                  |                  |                  |                 |

|                  |                  |                  |                 |

|                  |                  |                  |                 |

|                  |                  |                  |                 |

|   Read Latch 0   |   Read Latch 1   |   Read Latch 2   |   Read Latch 3  |

|   ===========-   |   ===========-   |   ===========-   |   ===========-  |

|   <7654.3210>    |   <7654.3210>    |   <7654.3210>    |   <7654.3210>   |

|        |         |        |         |        |         |        |        |

|    [Plane 0]     |    [Plane 1]     |    [Plane 2]     |    [Plane 3]    |

|        |         |        |         |        |         |        |        |

|        |         |        |         |        |         |        |        |

| .......|         | .......|         | .......|         | .......|        |

| |                | |                | |                | |               |

| |                | |                | |                | |               |

| |                | |                | |                | |               |

| | Multiplane     | | Multiplane     | | Multiplane     | | Multiplane    |

| | Function       | | Function       | | Function       | | Function      |

| | Select         | | Select         | | Select         | | Select        |

| | ===========    | | ===========    | | ===========    | | ===========   |

| | <7654.3210>    | | <7654.3210>    | | <7654.3210>    | | <7654.3210>   |

| |      |         | |      |         | |      |         | |      |        |

| | [All planes]   | | [All planes]   | | [All planes]   | | [All planes]  |

| |      |         | |      |         | |      |         | |      |        |

| |      |         | |      |         | |      |         | |      |        |

| |      |         | |      |         | |      |         | |      |        |

| |....  |         | |....  |         | |....  |         | |....  |        |

|      | |         |      | |         |      | |         |      | |        |

|      | |         |      | |         |      | |         |      | |        |

|      | |         |      | |         |      | |         |      | |        |

|..... | |         |..... | |         |..... | |         |..... | |        |

| | |              | | |              | | |              | | |        |

| | |              | | |              | | |              | | |        |

| | |              | | |              | | |              | | |        |

Logic Block        Logic Block        Logic Block        Logic Block     |

...NO_OP...        ...NO_OP...        ...NO_OP...        ...NO_OP...     |

...AND.....        ...AND.....        ...AND.....        ...AND.....     |

...OR......        ...OR......        ...OR......        ...OR......     |

...XOR.....        ...XOR.....        ...XOR.....        ...XOR.....     |

===========        ===========        ===========        ===========     |

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>     |

|                  |                  |                  |          |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]      |

|                  |                  |                  |           |

|                  |                  |                  |           |

.....|             .....|             .....|             .....|           |

|                  |                  |                  |                |

|                  |                  |                  |                |

|                  |                  |                  |                |

|  Read Latch 0    |  Read Latch 1    |  Read Latch 2    |  Read Latch 3  |

|  ===========-    |  ===========-    |  ===========-    |  ===========-  |

|  <7654.3210>     |  <7654.3210>     |  <7654.3210>     |  <7654.3210>   |

|       |          |       |          |       |          |       |        |

|   [Plane 0]      |   [Plane 1]      |   [Plane 2]      |   [Plane 3]    |

|       |          |       |          |       |          |       |        |

|       |          |       |          |       |          |       |        |

| ......|          | ......|          | ......|          | ......|        |

| |                | |                | |                | |              |

| |                | |                | |                | |              |

| |                | |                | |                | |              |

| |      ......... | | .............. | | .............. | | .............P

| |      |         | |      |         | |      |         | |      |

| |      |         | |      |         | |      |         | |      |

| |      P         | |      P         | |      P         | |      P

Every output       Every output       Every output       Every output

bit selected       bit selected       bit selected       bit selected

using choice       using choice       using choice       using choice

for that bit       for that bit       for that bit       for that bit

===========-       ===========-       ===========-       ===========-

<7654.3210>        <7654.3210>        <7654.3210>        <7654.3210>

|                  |                  |                  |

[Plane 0]          [Plane 1]          [Plane 2]          [Plane 3]

|                  |                  |                  |

|                  |                  |                  |

|                  |                  |                  |

|  Plane 0         |  Plane 1         |  Plane 2         |  Plane 3

|  Inhibit         |  Inhibit         |  Inhibit         |  Inhibit

|   Write          |   Write          |   Write          |   Write

|  =======         |  =======         |  =======         |  =======

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

|     |            |     |            |     |            |     |

-----------        -----------        -----------        -----------

Write Logic        Write Logic        Write Logic        Write Logic

for Plane 0        for Plane 1        for Plane 2        for Plane 3

Memory             Memory             Memory             Memory

-----------        -----------        -----------        -----------

Key to schematic for Write Mode 3

=================================

--------------------------

Permission Rotation Select

--------------------------

See {6.06.05} Data Rotate Register.

------------------------------

Applicable Permission Selector

------------------------------

See {6.06.10} Bit Mask Register.

----------------------

Multiplane Data Source

----------------------

See {6.06.02} Set/Reset Register.

--------------------------

Multiplane Function Select

--------------------------

See {6.06.05} Data Rotate Register.

-----------------------

Plane 'N' Inhibit Write

-----------------------

See {6.04.04} Map Mask Register.

<1>      Set/Reset Data Image Plane 1

----------------------------

As for Set/Reset Image Plane 0, immediately above.

See Set/Reset Enable Image Plane 1, below.

<2>      Set/Reset Data Image Plane 2

----------------------------

As for Set/Reset Image Plane 0, immediately above.

See Set/Reset Enable Image Plane 2, below.

<3>      Set/Reset Data Image Plane 3

----------------------------

As for Set/Reset Image Plane 0, immediately above.

See Set/Reset Enable Image Plane 3, below.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.06.03  Enable Set/Reset Register    { 3cfh    01h    R/W }

=======  ===================================================

Cross reference with {6.06.02} Set/Reset Register.

<0>      Set/Reset Enable Image Plane 0

------------------------------

During multiplane image memory write access operation by the processor

this bit helps to determine the value written to the byte lying within

display memory image plane 0.

<=0>  Image plane zero is disabled from accepting any set/reset data,

so long as the plane is able to support this operation.

See {6.06.02} Set/Reset Register.

<=1>  Image plane zero is enabled to accept set/reset data, so long as

the plane is able to support this operation.

See {6.06.02} Set/Reset Register.

The precise effect of this bit is determined by the write mode used to

perform the update operation.  There are four different write modes as

follows:

Write Mode 0

============

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

Where Set/Reset Enable Image Plane 0 is set <=1>, all eight data path

bits associated with image plane 0 are set to just one value given by

the single bit Set/Reset Data Image Plane 0.  All other memory planes

are processed in an identical manner.  Four output data paths result,

each of eight bits width, making a total of 32 bits.  These data path

outputs are then subjected to further processing, before any data are

actually stored in image memory.

The use of the Set/Reset Enable and Set/Reset Data input is to specify

a four bit colour value across a single bit position within all planes

of the image memory, in parallel.  In this way, the processor supplied

data is either used unchanged, is modified partially, or is completely

ignored when deciding how to process for update.  The resulting output

is used by the logic unit together with the read latch contents.  This

provides extremely powerful colour processing capability when properly

used.  This write mode is probably the most generally used write mode.

A schematic for write mode 0 is available.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

Write Mode 1

============

There is no use made of this bit value for write mode 1 operations.

A schematic for write mode 1 is available.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

Write Mode 2

============

There is no use made of this bit value for write mode 2 operations.

A schematic for write mode 2 is available.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

Write Mode 3

============

There is no use made of this bit value for write mode 3 operations.

A schematic for write mode 3 is available.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

<1>      Set/Reset Enable Image Plane 1

------------------------------

As for Set/Reset Enable Image Plane 0, immediately above.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

<2>      Set/Reset Enable Image Plane 2

------------------------------

As for Set/Reset Enable Image Plane 0, immediately above.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

<3>      Set/Reset Enable Image Plane 3

------------------------------

As for Set/Reset Enable Image Plane 0, immediately above.

See {6.06.02} Set/Reset Register.

See {6.06.07} Graphics Mode Register.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.06.04  Colour Compare Register      { 3cfh    02h    R/W }

=======  ===================================================

Cross reference with {6.06.06} Read Map Select Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.09} Colour Don't Care Register.

<0>      Colour Compare Plane 0

----------------------

During multiplane image memory read access operation by the processor,

this bit can determine the value of the byte representing the result

returned to the processor.

<=0>  Image plane zero is compared with the data value 0, so long as

the plane is enabled to support this operation.

See {6.06.09} Colour Don't Care Register.

<=0>  Image plane zero is compared with the data value 1, so long as

the plane is enabled to support this operation.

See {6.06.09} Colour Don't Care Register.

The effect of this bit is determined by the read mode used to perform

the image memory read access operation.  Two different read modes are

provided as follows:

Read Mode 0

===========

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

When read mode 0 is active, a read operation by the processor returns

a single byte, which is associated with one of the four image planes.  The plan

The particular image plane is selected by means of a special register

provided specifically for this purpose.

See {6.06.06} Read Map Select Register.

See {6.06.07} Graphics Mode Register.

In read mode 0, the Colour Compare Register plays no active role in

processor read operation so the value of the Colour Compare Plane 0

bit is irrelevant.

Read Mode 1

===========

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

It is often required to determine where the border pixels of an area

are situated, perhaps in order to provide a colour fill function for

a graphics software package.  Usually in such a situation, either of

the colours associated with the foreground or background are a known

constant.  The business of discovering the border pixels becomes one

of tracing a region consisting of pixels with a known colour.

There is a way of doing this, using read mode 0 to recover the four

bytes of colour plane data at a given image memory address.  But it

would then be necessary to change those four bytes into eight pixels

each of 4 bits.  Whilst this is computationally simple, it is a very

time consuming operation to perform, especially with large images.

This is where read mode 1 can provide assistance.  Read mode 1 uses

a colour match specification to inspect an eight pixel image memory

address in parallel, producing a colour match bit for each pixel in

the result byte.  A matching colour produces a set bit result.

The match value for any plane can be made irrelevant by using a mask

value for the plane indicating a colour match don't care.  Any don't

care planes are considered to match under all circumstances.  So for

some colour ranges it is possible to obtain sensible match behaviour

even though the match result is expressed in a single bit.

See {6.06.07} Graphics Mode Register.

See {6.06.09} Colour Don't Care Register.

<1>      Colour Compare Plane 1

----------------------

As for Colour Compare Plane 0, immediately above.

See {6.06.06} Read Map Select Register.

See {6.06.09} Colour Don't Care Register.

<2>      Colour Compare Plane 2

----------------------

As for Colour Compare Plane 0, immediately above.

See {6.06.06} Read Map Select Register.

See {6.06.09} Colour Don't Care Register.

<3>      Colour Compare Plane 3

----------------------

As for Colour Compare Plane 0, immediately above.

See {6.06.06} Read Map Select Register.

See {6.06.09} Colour Don't Care Register.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.06.05  Data Rotate Register         { 3cfh    03h    R/W }

=======  ===================================================

Cross reference with {6.06.02} Set/Reset Register.

<2..0>   Data Rotate

-----------

This binary value specifies the data rotate count for the following

write modes {0,3}.  The rotation is to the right.

There is a description of these modes available with a schematic of

the data flow.

See {6.06.02} Set/Reset Register.

<4..3>   Logic Unit Function Code

------------------------

This binary field specifies the logic unit function code.  There are

two data path inputs to the logic unit, each of which contains image

memory plane data for each of the four memory planes.  One plane has

eight data bits associated with it, making 32 bits total for each of

two input data paths.  So the logic unit combines 64 input bits with

a corresponding output of 32 result bits.

The logic function always takes one input data path to be the memory

read latches, with the other input depending on the particular write

mode selected.

The logic unit functions are effective in the following write modes

{0,2,3} and are assigned as follows:

<=00>  NO_OP.  No operation, the data passes straight through.  No use

is made of the read latch data input to the logic unit,

but THE READ LATCHES ARE UPDATED WITH THE DATA.

<=01>  AND.    Logical AND, bit position for bit position between the

input data path and the read latch data path.

<=10>  OR.     Logical OR, bit position for bit position between the

input data path and the read latch data path.

<=11>  XOR.    Logical XOR, bit position for bit position between the

input data path and the read latch data path.

See {6.06.02} Set/Reset Register.

<7..5>   Reserved.  Always <=000>.

-------------------------

6.06.06  Read Map Select Register     { 3cfh    04h    R/W }

=======  ===================================================

Cross reference with {6.06.04} Colour Compare Register.

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.10.09} CPU Latch Read Back Register.

<1..0>   Read Map Select

---------------

The binary value of this field represents the multiplane image memory

plane from which the processor will read a byte of pixel data when in

read mode 0.  The actual effect depends upon the display image memory

organisation as follows:

Processor Image Memory Access Mapping Select

=-------------------------------------------

Processor Image Memory Access Mode Select

=    =----------------------------------------

Processor Display Memory Addressing

=    =    ===================================------------------

0    0    Text access; odd/even mode; the low order address bit

selects the pair of planes to access.  Even addresses

select the pair (0,2) and odd addresses (1,3).

In all other respects the memory organisation remains

multiplane.

Data transfers are by word from the processor in this

memory mode, with a character code and attribute byte

sent together.  Memory planes 2,3 are write inhibited

to preserve the character font data in plane 2.

The Read Map Select field bit 0 is not used for this

memory mode.  The remaining bit 1 selects a plane in

the pair of planes determined by the access address,

for read mode 0.

0    1    Multiplane access; the planes are written by using a

write mode and read using a read mode.  The plane is

selected by means of special registers used by these

read and write modes.  This organisation supports 16

colours, with 8 pixels per byte across 4 planes at a

single image memory byte address.

Data transfers are single byte from the processor in

this memory mode.

The Read Map Select field denotes the plane number to

be read by a read mode 0 access, in this memory mode.

Planes 0 through 3 are therefore accessible.

1    0    The two low order address bits are the plane address;

the remaining address bits are shifted down to become

a byte address into multiplane memory.  This supports

256 colour packed pixel mode, where a byte represents

the colour data for a single pixel.

Data transfers are single byte from the processor in

this memory mode.

All two bits of the Read Map Select field are ignored

in this memory mode, for read mode 0 access.

1    1    The two low order address bits are the plane address;

the remaining address bits are shifted down to become

a byte address into multiplane memory.  This supports

256 colour packed pixel mode, where a byte represents

the colour data for a single pixel.

Data transfers are single byte from the processor in

this memory mode, for read mode 0 access.

All two bits of the Read Map Select field are ignored

in this memory mode.

---------------------------------------------------------------

See {6.04.06} Sequencer Memory Mode Register.

See {6.06.07} Graphics Mode Register.

This field decides which cpu data latch to access during a cpu latch

read back operation.  Each cpu data latch is associated with an image

memory plane.

See {6.10.09} CPU Latch Read Back Register.

This field does not have any influence over processor read operations

in read mode 1.  These are colour compare read operations.

See {6.06.04} Colour Compare Register.

See {6.06.07} Graphics Mode Register.

<7..2>   Reserved.  Always <=000000>.

----------------------------

6.06.07  Graphics Mode Register       { 3cfh    05h    R/W }

=======  ===================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.04.04} Map Mask Register.

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.06.02} Set/Reset Register.

Cross reference with {6.06.03} Enable Set/Reset Register.

Cross reference with {6.06.04} Colour Compare Register.

Cross reference with {6.06.05} Data Rotate Register.

Cross reference with {6.06.06} Read Map Select Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.06.09} Colour Don't Care Register.

Cross reference with {6.06.10} Bit Mask Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.07.20} Colour Plane Enable Register.

Cross reference with {6.07.21} Horizontal PEL Panning Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.09} CPU Latch Read Back Register.

<1..0>   Processor Write Mode

--------------------

The display may be used in either text mode or graphics mode.

See {6.06.08} Miscellaneous Register.

There is capability provided to read back the data written to

the display image memory data latches by the processor.

See {6.10.09} CPU Latch Read Back Register.

<=00>  Write Mode 0

------------

This write mode is used to process absolute colour values, when

image memory is in multiplane format.  There are many possible

types of image processing operations which can be achieved with

this write mode, which permits processor modification of any of

one through eight pixels per single access.  The data might be

stored in any of one to four image memory planes some or all of

which may be modified simultaneously as the result of one write

operation by the processor.  This write mode is the most often

used, because of the great flexibility it provides.  It is most

suitable for writing text whilst in graphics mode, which is one

of the most demanding operations the processor can perform with

a multiplane image.  In this write mode the read latch contents

have a very important role in either preserving existing memory

background image content, or in establishing a new background.

However, because there are so many processing operations which

are carried out in parallel, sometimes considerable thought is

required to produce the best results from this mode.  Otherwise

it is easy to program simply by ignoring most of the available

processing power.  If this approach is adopted, it is possible

to implement improved graphics algorithms as they are developed

provided always that simple functional interfaces are used with

the graphics software components.

This is an absolute colour processing update operation, whereas

write mode 3 provides for relative colour update operation.

There is a discussion of write mode 0 available, together with

a schematic of the data flow invlved.

See {6.06.02} Set/Reset Register.

A discussion of graphics shift register configuration with data

flow schematics is available.

See {6.04.03} Clocking Mode Register.

See table below.

See schematic diagrams below.

The registers used in this mode are as follows.

See {6.04.04} Map Mask Register.

See {6.06.02} Set/Reset Register.

See {6.06.03} Enable Set/Reset Register.

See {6.06.05} Data Rotate Register.

See {6.06.10} Bit Mask Register.

<=01>  Write Mode 1

------------

Write mode 1 provides the facilities for fast multiplane memory

image movement and initialisation.  This write mode ignores all

data written by the processor.  Instead it uses the associated

addresses generated by the processor, together with the type of

memory access (read, write) to control the internal processing

carried out by the VGA controller.  In this manner, great speed

of processing is achieved, since all internal data paths are 32

bits wide.

The processing ability possessed by this mode is minimal which

means that in practise, it is very easy to set up for use.  In

this way, it is a practical mode for use in high speed program

operation, with important application where large images must

be updated using predefined image components.  Such components

are generally held in a library, which is located within a part

of the display image memory which is not displayed directly.

The fast copy ability of this mode can replicate such standard

image parts wherever they are needed, with low overhead.

There is a discussion of write mode 1 available, together with

a schematic of the data flow invlved.

See {6.06.02} Set/Reset Register.

The registers used in this mode are as follows.

See {6.04.04} Map Mask Register.

<=10>  Write Mode 2

------------

This mode can write from one upto eight pixels with a processor

specified colour, when in multiplane image memory format.  This

enables complex colour change image updates to take place where

the processor must determine colour according to possibly quite

sophisticated criteria.  The read latch contents are important

in this mode for specifying the image for regions which are not

directly being modified.  The read latches may either be given

a single value, which is then subsequently used for many write

operations, or may be initialised immediately prior to a write

in order to preserve existing image data.

This mode can be used in line drawing where the colour of the

line must be changed frequently.  It can also be used to fill

an area with colour.  As logic operations can be performed in

this mode, it can find application in image animation.

There is a discussion of write mode 2 available, together with

a schematic of the data flow invlved.

See {6.06.02} Set/Reset Register.

A discussion of graphics shift register configuration with data

flow schematics is available.

See {6.04.03} Clocking Mode Register.

See table below.

See schematic diagrams below.

The registers used in this mode are as follows.

See {6.04.04} Map Mask Register.

See {6.06.05} Data Rotate Register.

See {6.06.10} Bit Mask Register.

<=11>  Write Mode 3

------------

Write mode 3 provides the capability for processing an image

according to the current colour data existing in that image.

The colour value for each pixel is processed by the logic unit

and the processor supplied data is then used to determine what

data source to use for the new pixel colour value.  The effect

is to selectively preserve or update the current pixel colour.

This type of processing is useful for update operations which

must guarantee that all new image data written must be visible

no matter what the colour distribution in the existing image.

This is a relative colour processing update operation, whereas

write mode 0 provides for absolute colour update operation.

There is a discussion of write mode 3 available, together with

a schematic of the data flow invlved.

See {6.06.02} Set/Reset Register.

A discussion of graphics shift register configuration with data

flow schematics is available.

See {6.04.03} Clocking Mode Register.

See table below.

See schematic diagrams below.

The registers used in this mode are as follows.

See {6.04.04} Map Mask Register.

See {6.06.02} Set/Reset Register.

See {6.06.05} Data Rotate Register.

See {6.06.10} Bit Mask Register.

<2>      Reserved.  Always <=0>.

-----------------------

<3>      Processor Read Mode

-------------------

<=0>  Read Mode 0

-----------

This read mode returns a byte of pixel data from one of the four

image memory planes in multiplane image format.  Each plane has

responsibility for storing one bit of colour data for each pixel

so a byte gives the setting of one colour bit over eight pixels.

Sometimes it is useful to read data from just one of the planes,

so that functionality is provided by read mode 0.  Any read will

load the read latches from each of the four image memory planes

when read mode 0 is operational.  This happens despite the fact

that only one plane will deliver data to the processor.  Latches

are selected for processor read using a special register.

See {6.06.06} Read Map Select Register.

When the logic unit is used during a write mode, it is possible

to arrange for the read latches to be updated as a result of a

NO_OP function code.

See {6.06.02} Set/Reset Register.

There is another discussion of read mode 0 available.

See {6.06.04} Colour Compare Register.

<=1>  Read Mode 1

-----------

This read mode provides for deciding on the colour value for a

group of eight neighbouring pixels.  The four colour bits that

make up each pixel are compared with a predetermined colour.

The result of the comparison process over eight pixels is given

as an eight bit byte, indicating whether or not the colour was

present for each pixel.

There are provisions for indicating that certain image planes

are to be ignored for comparison purposes, and this read mode

gives a considerable speed advantage for some operations such

as colour fill.  Without read mode 1, colour boundary search

operations would be extremely slow to execute.

See {6.06.09} Colour Don't Care Register.

Note that the image memory organisation must be multiplane if

read mode 1 is to function.  A processor read using read mode

1 will load all four read latches identically with the colour

compare result.  This provides a way of initialising the read

latches within a single processor instruction.

If the colour don't care register is set to zero, the colour

compare is guaranteed to succeed, independantly of the pixel

data.  This will generate a full byte result where every bit

is set to one.  In addition the all bits set pattern appears

in all four read latches.  This can be very useful when used

with a processor instruction which performs an image address

read access, then performs a corresponding write access.  An

example of this could be:

;

; Demonstrate multiple access to image memory when using

; read mode 1 in order to initialise the read latches to

; a value of all ones.

;

and      [bx],al

;

; End example

;

In this example, the logical operation uses the image memory

addressed through register 'bx' for both read and write.  In

reading the image memory, the read latches are filled with 1

bits, since the colour don't care register is set to zeros.

The processor then combines this all bits set value with the

value of register 'al', producing no change from that value.

This value is then sent to the VGA controller using whatever

write mode is in force which then stores the image memory at

the same address previously accessed for read.

It is also possible to arrange loading the read latches with 0

bits, by using a colour don't care register value of all ones.

It is also necessary to set the colour compare register to one

particular colour value which is never used.  Then all compare

operations will fail resulting in each read latch being set to

all zero bits.

Note that there are other ways of initialising the read latch

registers.  It can be useful to write into image memory those

initial values which will commonly be used.  These values can

then be read prior to any image manipulation.  Until the read

latches are modified by another processor read operation, the

value they possess remains unchanged.  This property provides

for writing fast executing software, since the problem with a

separate initialisation on each write is the overhead of data

bus speed (which is relatively low).

When the logic unit is used during a write mode, it is possible

to arrange for the read latches to be updated as a result of a

NO_OP function code.

See {6.06.02} Set/Reset Register.

There is another discussion of read mode 1 available.

See {6.06.04} Colour Compare Register.

<4>      Graphics Memory Access Mode

---------------------------

<=0>  Graphics Memory Access Mode.  In graphics mode the image memory

planes respond to addresses from the processor in parallel.  In

this way, all four memory planes can be read or written using a

single access.  The processor data transfer supports only 8 bit

transfers, so various mechanisms are employed to map the 8 bits

to each of the four bytes actually accessed.  In some cases, it

is not necessary to perform any mapping of this type at all, as

the processor is used only to generate an address which is then

used internally for image manipulation.

The manipulation is specified in advance and all that is needed

is a stream of processor generated addresses to put the process

into operation.  This type of operation is very fast to execute.

It is discussed during the description of write mode 1.

See {6.06.02} Set/Reset Register.

Processor Image Memory Access Mapping Select must be set to zero

for this setting to be effective.  See below.

See {6.06.07} Graphics Mode Register.

<=1>  Text Memory Access Mode.  This affects a data transfer from the

processor to the image memory such that it is suitable for text

mode operations.  The processor transfers 16 bits per access to

image memory in text mode.  The data consists of a character in

the current character set together with an attribute byte which

exterts control over the character presentation on the display.

For text mode display character codes occupy image memory plane

zero with attribute data occupying plane one.  The image memory

planes are paired (0,1) and (2,3).  The Text Memory Access Mode

directs even address accesses to planes (0,2), with odd address

accesses directed to planes (1,3).  In this way maximum ease of

access is provided for textual data update.  This is sometimes

termed odd/even mode.

To use this mode effectively it is usual to disable writes to a

pair of planes so permitting update to a two plane pair.

See {6.04.04} Map Mask Register.

Only processor writes to image memory are affected by this mode.

That is, image memory organisation continues to be multiplane in

all respects except as described above during write addressing.

This option setting should be reflected in the setting applied within

the sequencer, Processor Image Memory Access Mode Select.  Note these

bits are of opposite sense.

See {6.04.06} Sequencer Memory Mode Register.

<5>      Graphics Shift Register Mode

----------------------------

<=0>  Normal shift register operation.  This is only available when

the Graphics 256 Colour Control is set <=0>.  See table below

for full details.

A number of shift registers can be loaded together at one time

which affords powerful possibilities.  The shift registers can

change their interconnection when this occurs so as to support

a number of different types of image generation.  See below.

See {6.04.03} Clocking Mode Register.

<=1>  The shift registers support 4 colour packed pixel mode.  This

is only available when the Graphics 256 Colour Control is set

<=0>.  See table below for full details.

A number of shift registers can be loaded together at one time

which affords powerful possibilities.  The shift registers can

change their interconnection when this occurs so as to support

a number of different types of image generation.  See below.

See {6.04.03} Clocking Mode Register.

<6>      Graphics 256 Colour Control

---------------------------

<=0>  The Graphics Shift Register Mode, above, controls the operation

of the shift registers.  See table below for full details.

A number of shift registers can be loaded together at one time

which affords powerful possibilities.  The shift registers can

change their interconnection when this occurs so as to support

a number of different types of image generation.  See below.

See {6.04.03} Clocking Mode Register.

<=1>  The shift registers support the 256 colour packed pixel mode so

the Graphics Shift Register Mode, above, performs no function.

This option connects the shift registers so that 4 bits are sent

together to a special register, then another 4 bits are received

by that register, making 8 bits in total.  These 8 bits give the

full colour code needed to represent 256 colours.  Since these 8

bits are assembled from two 4 bit fields, it takes twice as long

to create each 8 bit colour value as it would take to issue each

4 bit colour value.  For this reason, the effective frequency of

the dot clock is halved in this mode.  To produce a display with

equivalent high resolution to a 16 colour multiplane display the

dot clock frequency must be doubled.

See {6.04.03} Clocking Mode Register.

See table below for full details.

The internal pixel palette is bypassed in this mode, due to two

bits being transmitted in sequence from each of the four planes

output lines.  This multiplexing prohibits the possibility that

the internal colour palette could exert any colour control over

the display.  However each of the shift register outputs should

be enabled, so that the two bits can be safely transmitted into

the shift register assembly latches.

See {6.07.20} Colour Plane Enable Register.

See table below.

See schematic diagrams below.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

A number of shift registers can be loaded together at one time

which affords powerful possibilities.  The shift registers can

change their interconnection when this occurs so as to support

a number of different types of image generation.  See below.

See {6.04.03} Clocking Mode Register.

<7>      Reserved.  Always <=0>.

-----------------------

Shift Register Mode Control Summary

===================================

Graphics 256 Colour Control

=--------------------------

Graphics Shift Register Mode

=    =---------------------------

Colour Output Bit Plane Number

=    =    =-----------------------------

Shift Register Operation Specification

=    =    =    ======================================----------------

=    =    =    shift direction    ------------------>    [output bit]

Bit 0         Bit 2         Bit 4         Bit 6

=    =    =    ====-         ====-         ====-         ====-

Bit 1         Bit 3         Bit 5         Bit 7

=    =    =           ====-         ====-         ====-         ====-

=    =    =    ====   ====   ====   ====   ====   ====   ====   ====

-------------------- multiplane 16 colour mode ---------------------

0    0    0    p0b0   p0b1   p0b2   p0b3   p0b4   p0b5   p0b6   p0b7

1    p1b0   p1b1   p1b2   p1b3   p1b4   p1b5   p1b6   p1b7

2    p2b0   p2b1   p2b2   p2b3   p2b4   p2b5   p2b6   p2b7

3    p3b0   p3b1   p3b2   p3b3   p3b4   p3b5   p3b6   p3b7

-------------------- packed pixel 4 colour mode --------------------

0    1    0    p1b0   p1b2   p1b4   p1b6   p0b0   p0b2   p0b4   p0b6

1    p1b1   p1b3   p1b5   p1b7   p0b1   p0b3   p0b5   p0b7

2    p3b0   p3b2   p3b4   p3b6   p2b0   p2b2   p2b4   p2b6

3    p3b1   p3b3   p3b5   p3b7   p2b1   p2b3   p2b5   p2b7

-------------------- packed pixel 256 colour mode ------------------

1    0    0    p3b0   p3b4   p2b0   p2b4   p1b0   p1b4   p0b0   p0b4

1    p3b1   p3b5   p2b1   p2b5   p1b1   p1b5   p0b1   p0b5

2    p3b2   p3b6   p2b2   p2b6   p1b2   p1b6   p0b2   p0b6

3    p3b3   p3b7   p2b3   p2b7   p1b3   p1b7   p0b3   p0b7

-------------------- packed pixel 256 colour mode ------------------

1    1    0    p3b0   p3b4   p2b0   p2b4   p1b0   p1b4   p0b0   p0b4

1    p3b1   p3b5   p2b1   p2b5   p1b1   p1b5   p0b1   p0b5

2    p3b2   p3b6   p2b2   p2b6   p1b2   p1b6   p0b2   p0b6

3    p3b3   p3b7   p2b3   p2b7   p1b3   p1b7   p0b3   p0b7

------------------------------------------------------------------

When the shift registers are not loaded every character clock, they

are chained together such that the output from the preceeding shift

register becomes the input to that of the following shift register.

See {6.04.03} Clocking Mode Register.

This organisation changes depending upon the shift register mode, so

that monochrome bit map, 4 colour packed pixel and 256 colour packed

pixel displays may be supported.  This organisation change takes the

individual shift register bits and changes their interconnections so

that the different functionality required can be supported.

There are some important consequences of changes to the organisation

of the shift register bits.  From the diagrams below it will be seen

that in the rearranged modes, various output bits are not used.  The

deselection of these bits is under control of a special register for

just this purpose.  A deselected bit becomes a constant zero output,

which is necessary to ensure stable indexing into the colour tables.

See {6.07.20} Colour Plane Enable Register.

The various organisations are detailed below, in diagramatic form.

Individual Shift Register Connection, Multiplane 16 colour mode

===============================================================

[Display Address]

|

|

|

|  [Load Clock]      --  derived from character clock

|  |

|  |

|  |

|  |  [Shift Clock]  --  dot clock

|  |  |

|  |  |

|  |  |

|  |  sc..........................................

|  |  |                                          |       _______

|  |  |                   Memory Plane 3         |...>...Shift 3

|  lc...................>.| [plane 3, bit 7]...........>.| [7]

|  |  |                   | [plane 3, bit 6].........>...| [6]

|  |  |                   | [plane 3, bit 5]...........>.| [5]

da......................>.| [plane 3, bit 4].........>...| [4]

|  |  |                   | [plane 3, bit 3]...........>.| [3]

|  |  |                   | [plane 3, bit 2].........>...| [2]

|  |  |                   | [plane 3, bit 1]...........>.| [1]

|  |  |                   | [plane 3, bit 0].........>...| [0]

|  |  |                                                     |

|  |  |                                                     |

|  |  sc....................................                |

|  |  |                                    |       _______  |

|  |  |             Memory Plane 2         |...>...Shift 2  |

|  lc.............>.| [plane 2, bit 7]...........>.| [7]    |

|  |  |             | [plane 2, bit 6].........>...| [6]    |

|  |  |             | [plane 2, bit 5]...........>.| [5]    |

da................>.| [plane 2, bit 4].........>...| [4]    |

|  |  |             | [plane 2, bit 3]...........>.| [3]    |

|  |  |             | [plane 2, bit 2].........>...| [2]    |

|  |  |             | [plane 2, bit 1]...........>.| [1]    |

|  |  |             | [plane 2, bit 0].........>...| [0]    |

|  |  |                                               |     |

|  |  |                                               |     |

|  |  sc..............................                |     |

|  |  |                              |       _______  |     |

|  |  |       Memory Plane 1         |...>...Shift 1  |     |

|  lc.......>.| [plane 1, bit 7]...........>.| [7]    |     |

|  |  |       | [plane 1, bit 6].........>...| [6]    |     |

|  |  |       | [plane 1, bit 5]...........>.| [5]    |     |

da..........>.| [plane 1, bit 4].........>...| [4]    |     |

|  |  |       | [plane 1, bit 3]...........>.| [3]    |     |

|  |  |       | [plane 1, bit 2].........>...| [2]    |     |

|  |  |       | [plane 1, bit 1]...........>.| [1]    |     |

|  |  |       | [plane 1, bit 0].........>...| [0]    |     |

|  |  |                                         |     |     |

|  |  |                                         |     |     |

|  |  sc........................                |     |     |

|  |                           |       _______  |     |     |

|  |    Memory Plane 0         |...>...Shift 0  |     |     |

|  lc.>.| [plane 0, bit 7]...........>.| [7]    |     |     |

|       | [plane 0, bit 6].........>...| [6]    |     |     |

|       | [plane 0, bit 5]...........>.| [5]    |     |     |

da....>.| [plane 0, bit 4].........>...| [4]    |     |     |

| [plane 0, bit 3]...........>.| [3]    |     |     |

| [plane 0, bit 2].........>...| [2]    |     |     |

| [plane 0, bit 1]...........>.| [1]    |     |     |

| [plane 0, bit 0].........>...| [0]    |     |     |

|     |     |     |

|     |     |     |

|     |     |     |

|   Plane   |   Plane

|   1 Out   |   3 Out

|           |

Plane       Plane

0 Out       2 Out

===========================

16 colour output

delivered at full

dot clock frequency

===========================

Note each of the output bits are associated with a particular image

plane and each of these shift register output bits can individually

be enabled or disabled.  This control is exerted by means of a zero

bit substitution for the data bit that would normally be output.  A

specially appointed control register is provided for this purpose.

See {6.07.20} Colour Plane Enable Register.

-------------------------------------------------------------------

Individual Shift Register Connection, Packed Pixel 2 colour mode

================================================================

[Display Address]

|

|

|

|  [Load Clock]      --  derived from character clock

|  |

|  |

|  |

|  |  [Shift Clock]  --  dot clock

|  |  |

|  |  |

|  |  |

|  |  sc..........................................

|  |  |                                          |       ________

|  |  |                   Memory Plane 3         |...>...Shift 3

|  lc...................>.| [plane 3, bit 7]...........>.| [7]

|  |  |                   | [plane 3, bit 6].........>...| [6]

|  |  |                   | [plane 3, bit 5]...........>.| [5]

da......................>.| [plane 3, bit 4].........>...| [4]

|  |  |                   | [plane 3, bit 3]...........>.| [3]

|  |  |                   | [plane 3, bit 2].........>...| [2]

|  |  |                   | [plane 3, bit 1]...........>.| [1]

|  |  |                   | [plane 3, bit 0].........>...| [0]

|  |  |                                                     |

|  |  |                                                     |

|  |  |                                               ......|

|  |  sc....................................          |     |

|  |  |                                    |       ___|____ |

|  |  |             Memory Plane 2         |...>...Shift 2  |

|  lc.............>.| [plane 2, bit 7]...........>.| [7]    |

|  |  |             | [plane 2, bit 6].........>...| [6]    |

|  |  |             | [plane 2, bit 5]...........>.| [5]    |

da................>.| [plane 2, bit 4].........>...| [4]    |

|  |  |             | [plane 2, bit 3]...........>.| [3]    |

|  |  |             | [plane 2, bit 2].........>...| [2]    |

|  |  |             | [plane 2, bit 1]...........>.| [1]    |

|  |  |             | [plane 2, bit 0].........>...| [0]    |

|  |  |                                               |     |

|  |  |                                               |     |

|  |  |                                         ......|     |

|  |  sc..............................          |     |     |

|  |  |                              |       ___|____ |     |

|  |  |       Memory Plane 1         |...>...Shift 1  |     |

|  lc.......>.| [plane 1, bit 7]...........>.| [7]    |     |

|  |  |       | [plane 1, bit 6].........>...| [6]    |     |

|  |  |       | [plane 1, bit 5]...........>.| [5]    |     |

da..........>.| [plane 1, bit 4].........>...| [4]    |     |

|  |  |       | [plane 1, bit 3]...........>.| [3]    |     |

|  |  |       | [plane 1, bit 2].........>...| [2]    |     |

|  |  |       | [plane 1, bit 1]...........>.| [1]    |     |

|  |  |       | [plane 1, bit 0].........>...| [0]    |     |

|  |  |                                         |     |     |

|  |  |                                         |     |     |

|  |  |                                   ......|     |     |

|  |  sc........................          |     |     |     |

|  |                           |       ___|____ |     |     |

|  |    Memory Plane 0         |...>...Shift 0  |     |     |

|  lc.>.| [plane 0, bit 7]...........>.| [7]    |     |     |

|       | [plane 0, bit 6].........>...| [6]    |     |     |

|       | [plane 0, bit 5]...........>.| [5]    |     |     |

da....>.| [plane 0, bit 4].........>...| [4]    |     |     |

| [plane 0, bit 3]...........>.| [3]    |     |     |

| [plane 0, bit 2].........>...| [2]    |     |     |

| [plane 0, bit 1]...........>.| [1]    |     |     |

| [plane 0, bit 0].........>...| [0]    |     |     |

|     |     |     |

|     |     |     |

|     |     |     |

|   Plane   |   Plane

|   1 Out   |   3 Out

|   [Not ]  |   [Not ]

|   [Used]  |   [Used]

|           |

|         Plane

|         2 Out

|         [Not ]

|         [Used]

|

Plane

0 Out

==========================

Monochrome Output

delivered at full

dot clock frequency

==========================

Note each of the output bits are associated with a particular image

plane and each of these shift register output bits can individually

be enabled or disabled.  This control is exerted by means of a zero

bit substitution for the data bit that would normally be output.  A

specially appointed control register is provided for this purpose.

See {6.07.20} Colour Plane Enable Register.

-------------------------------------------------------------------

Individual Shift Register Connection, Packed Pixel 4 colour mode

================================================================

[Display Address]

|

|

|

|  [Load Clock]      --  derived from character clock

|  |

|  |

|  |

|  |  [Shift Clock]  --  dot clock

|  |  |

|  |  |

|  |  |

|  |  sc..........................................

|  |  |                                          |       ________

|  |  |                   Memory Plane 3         |...>...Shift 3

|  lc...................>.| [plane 3, bit 7]...........>.|    [7]

|  |  |                   | [plane 3, bit 6].........>...| [6] |

|  |  |                   | [plane 3, bit 5]...........>.|  | [5]

da......................>.| [plane 3, bit 4].........>...| [4] |

|  |  |                   | [plane 3, bit 3]...........>.|  | [3]

|  |  |                   | [plane 3, bit 2].........>...| [2] |

|  |  |                   | [plane 3, bit 1]...........>.|  | [1]

|  |  |                   | [plane 3, bit 0].........>...| [0] |

|  |  |                                                     |  |

|  |  |                                                     |  |

|  |  |                                               ......|  |

|  |  |                                               |        |

|  |  |                                               |  ......|

|  |  |                                               |  |

|  |  sc....................................          |  |

|  |  |                                    |       ___|__|_

|  |  |             Memory Plane 2         |...>...Shift 2

|  lc.............>.| [plane 2, bit 7]...........>.|  | [7]

|  |  |             | [plane 2, bit 6].........>...| [6] |

|  |  |             | [plane 2, bit 5]...........>.|  | [5]

da................>.| [plane 2, bit 4].........>...| [4] |

|  |  |             | [plane 2, bit 3]...........>.|  | [3]

|  |  |             | [plane 2, bit 2].........>...| [2] |

|  |  |             | [plane 2, bit 1]...........>.|  | [1]

|  |  |             | [plane 2, bit 0].........>...| [0] |

|  |  |                                               |  |

|  |  |                                               |  |

|  |  |                                               |  |

|  |  |                                         ......e  |

|  |  |                                         |     |  |

|  |  |                                         |     |  |

|  |  |                                         |     e  |

|  |  |                                         |  ......o...

|  |  |                                         |  |  e     |

|  |  |                                         |  |  |     |

|  |  sc..............................          |  |  |     |

|  |  |                              |       ___|__|_ |     |

|  |  |       Memory Plane 1         |...>...Shift 1  |     |

|  lc.......>.| [plane 1, bit 7]...........>.|  | [7] |     |

|  |  |       | [plane 1, bit 6].........>...| [6] |  |     |

|  |  |       | [plane 1, bit 5]...........>.|  | [5] |     |

da..........>.| [plane 1, bit 4].........>...| [4] |  |     |

|  |  |       | [plane 1, bit 3]...........>.|  | [3] |     |

|  |  |       | [plane 1, bit 2].........>...| [2] |  |     |

|  |  |       | [plane 1, bit 1]...........>.|  | [1] |     |

|  |  |       | [plane 1, bit 0].........>...| [0] |  |     |

|  |  |                                         |  |  |     |

|  |  |                                         |  |  |     |

|  |  |                                   ......|  |  |     |

|  |  |                                   |        |  |     |

|  |  |                                   |  ......|  |     |

|  |  |                                   |  |        |     |

|  |  sc........................          |  |        |     |

|  |                           |       ___|__|_       |     |

|  |    Memory Plane 0         |...>...Shift 0        |     |

|  lc.>.| [plane 0, bit 7]...........>.|  | [7]       |     |

|       | [plane 0, bit 6].........>...| [6] |        |     |

|       | [plane 0, bit 5]...........>.|  | [5]       |     |

da....>.| [plane 0, bit 4].........>...| [4] |        |     |

| [plane 0, bit 3]...........>.|  | [3]       |     |

| [plane 0, bit 2].........>...| [2] |        |     |

| [plane 0, bit 1]...........>.|  | [1]       |     |

| [plane 0, bit 0].........>...| [0] |        |     |

|  |        |     |

|  |        |     |

..........................|  |        |     |

|                            |        |     |

|                ............|        |   Plane

|                |                    |   3 Out

|                |                    |   2 Out

|                |                    |   ODD

|                |                    |   [Not ]

|                |                    |   [Used]

|                |                    |

|                |                   Plane

|                |                   3 Out

|                |                   2 Out

|                |                   EVEN

|                |                   [Not ]

|                |                   [Used]

|                |

Plane            Plane

1 Out            1 Out

0 Out            0 Out

EVEN             ODD

==========================

4 colour output

delivered at full

dot clock frequency

==========================

Note each of the output bits are associated with a particular image

plane and each of these shift register output bits can individually

be enabled or disabled.  This control is exerted by means of a zero

bit substitution for the data bit that would normally be output.  A

specially appointed control register is provided for this purpose.

See {6.07.20} Colour Plane Enable Register.

-------------------------------------------------------------------

Individual Shift Register Connection, Packed Pixel 256 colour mode

==================================================================

[Display Address]

|

|

|

|  [Load Clock]      --  derived from character clock

|  |

|  |

|  |

|  |  [Shift Clock]  --  dot clock

|  |  |

|  |  |

|  |  |

|  |  sc..........................

|  |  |                          |       ______________

|  |  |   Memory Plane 3         |...>...Shift 3

|  lc...>.| [plane 3, bit 0]...........>.|..........[0]

|  |  |   | [plane 3, bit 1].........>...|.......[1] |

|  |  |   | [plane 3, bit 2]...........>.|....[2] |  |

da......>.| [plane 3, bit 3].........>...|.[3] |  |  |

|  |  |   | [plane 3, bit 4]...........>.|..|..|..|.[4]

|  |  |   | [plane 3, bit 5].........>...|..|..|.[5] |

|  |  |   | [plane 3, bit 6]...........>.|..|.[6] |  |

|  |  |   | [plane 3, bit 7].........>...|.[7] |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  sc..........................          |  |  |  |

|  |  |                          |       ___|__|__|__|_

|  |  |   Memory Plane 2         |...>...Shift 2  |  |

|  lc...>.| [plane 2, bit 0]...........>.|..|..|..|.[0]

|  |  |   | [plane 2, bit 1].........>...|..|..|.[1] |

|  |  |   | [plane 2, bit 2]...........>.|..|.[2] |  |

da......>.| [plane 2, bit 3].........>...|.[3] |  |  |

|  |  |   | [plane 2, bit 4]...........>.|..|..|..|.[4]

|  |  |   | [plane 2, bit 5].........>...|..|..|.[5] |

|  |  |   | [plane 2, bit 6]...........>.|..|.[6] |  |

|  |  |   | [plane 2, bit 7].........>...|.[7] |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  sc..........................          |  |  |  |

|  |  |                          |       ___|__|__|__|_

|  |  |   Memory Plane 1         |...>...Shift 1  |  |

|  lc...>.| [plane 1, bit 0]...........>.|..|..|..|.[0]

|  |  |   | [plane 1, bit 1].........>...|..|..|.[1] |

|  |  |   | [plane 1, bit 2]...........>.|..|.[2] |  |

da......>.| [plane 1, bit 3].........>...|.[3] |  |  |

|  |  |   | [plane 1, bit 4]...........>.|..|..|..|.[4]

|  |  |   | [plane 1, bit 5].........>...|..|..|.[5] |

|  |  |   | [plane 1, bit 6]...........>.|..|.[6] |  |

|  |  |   | [plane 1, bit 7].........>...|.[7] |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  |                                     |  |  |  |

|  |  sc..........................          |  |  |  |

|  |  |                          |       ___|__|__|__|_

|  |  |   Memory Plane 0         |...>...Shift 0  |  |

|  lc...>.| [plane 0, bit 0]...........>.|..|..|..|.[0]

|     |   | [plane 0, bit 1].........>...|..|..|.[1] |

|     |   | [plane 0, bit 2]...........>.|..|.[2] |  |

da......>.| [plane 0, bit 3].........>...|.[3] |  |  |

|   | [plane 0, bit 4]...........>.|..|..|..|.[4]

|   | [plane 0, bit 5].........>...|..|..|.[5] |

|   | [plane 0, bit 6]...........>.|..|.[6] |  |

|   | [plane 0, bit 7].........>...|.[7] |  |  |

|                                     |  |  |  |

|                                     |  |  |  |

|                                     |  |  |  |

|                                     |  |  |  |

|                                ___________________

|

|                                 these are the 4

|                                 plane output bits

|                                 which have little

|                                 significance for

|                                 256 colour mode

|                                 (see note below)

|                                ___________________

|                                     |  |  |  |

|                                     |  |  |  |

|                                     |  |  |  |

|                                     |  |  |  |

____|______                               |  |  |  |

Divide by 2                               |  |  |  |

| ODD.....|..........................     |  |  |  |

| EVEN....|.....                    |     |  |  |  |

|                    |     |  |  |  |

|                    |     |  |  |  |

|     ...............|.....a  |  |  |

|     |  ............|.....|..b  |  |

|     |  |  .........|.....|..|..c  |

|     |  |  |  ......|.....|..|..|..d

|     |  |  |  |     |     |  |  |  |

|     |  |  |  |     |     |  |  |  |

|     |  |  |  |     |     |  |  |  |

|     |  |  |  |     |     |  |  |  |

|    _|__|__|__|_    |    _|__|__|__|_

|   |Latch 4 bits    |   |Latch 4 bits

|.>.|making 8 bit    |.>.|making 8 bit

|256 colours         |256 colours

<7><6><5><4>         <3><2><1><0>

|  |  |  |           |  |  |  |

|  |  |  |           |  |  |  |

|  |  |  |           |  |  |  |

|  |  |  |           |  |  |  |

=================================

256 colour output

delivered at only half

the dot clock frequency

=================================

Important note: the concept of output bits corresponding with each

display memory image plane is redundant in 256 colour packed pixel

mode.  Two distinct colour bits flow from each of four output bits

during the two dot clock period necessary to build an 8 bit colour

value.  For this reason, the internal colour palette cannot effect

any colour control for 256 colour packed pixel modes. So that each

of the four shift register output bits can deliver two colour bits

making a total of 8 bits per pixel the colour plane outputs should

all be enabled during 256 colour modes.

See {6.07.20} Colour Plane Enable Register.

-------------------------------------------------------------------

6.06.08  Miscellaneous Register       { 3cfh    06h    R/W }

=======  ===================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.04.05} Character Map Select Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.05} Mode Control Register 1.

<0>      Display Generation Mode

-----------------------

<=0>  Text Mode

---------

Hardware character generation is invoked for display generation.

There is no capability to generate graphics in the usual manner

but a software technique can be used, giving a limited graphics

capability.  The technique depends upon assigning characters to

a display area, then re-writing the character fonts assigned to

those characters.  In this manner a limited graphics capability

can be supported within a relatively small area of the display.

The text mode display stores character codes in memory plane 0,

with their associated attribute bytes in plane 1.  The hardware

expects character font data to be stored at specified addresses

in plane 2.  Memory plane 3 is unused in this display mode.

The main characteristics of this mode are high speed operation,

with very low associated host processor overhead.  A number of

separate pages of text can be maintained, and switched between

at high speed.  This is the ideal mode to employ where no true

graphics capability is required, other than that provided with

the specially reserved graphics drawing characters.  Two fonts

can be used within one display screen, each font consisting of

256 characters in total.  User defined fonts can be used, when

occasion demands.

See {6.04.03} Clocking Mode Register.

See {6.04.05} Character Map Select Register.

See {6.05.22} Underline Location Register.

<=1>  Graphics Mode

-------------

The display will be generated with reference to an image memory,

organised as a collection of pixel data.  There is no capability

for the support of hardware text generation, but software can be

written to support multiple character fonts and any size of font

necessary for the application.

Display image memory organisation is dependant upon the range of

colours available for definition of a pixel.  The principle mode

options include:

1.    Multiplane 16 Colour

--------------------

The display memory is arranged as four parallel planes

each of which holds data for eight pixels by splitting

four colour bits across four planes.  Each plane gives

access to a single byte when presented with an address

so with parallel address spaces, each access returns 4

bytes.  The 32 bits specify 4 bits of colour data, for

each of eight pixels.

See {6.06.07} Graphics Mode Register.

2.    Packed pixel Monochrome

-----------------------

The display memory is arranged as a linear array which

is addressed in a manner corresponding directly to the

screen pixel display.  One bit per pixel specifies any

colour required, from a large available palette.  This

choice is normally restricted by making black the fore

ground colour.  The display image is easily managed as

it is of small size.  A large number of distinct pages

can be maintained and quickly switched for one another

which is very useful for hierarchical design systems.

See {6.06.07} Graphics Mode Register.

3.    Packed Pixel 4 Colour

---------------------

This display mode requires double the amount of memory

compared with monochrome mode, but provides 4 colours,

which can be much more flexible in some applications.

Each pixel is represented by two neighbouring bits, so

four pixels are collected together in one byte.  There

are special hardware capabilities available, to decode

this special colour representation.

Since images are still very compact large displays can

be manipulated without great memory overhead; a number

of distinct pages can be held in memory and swapped at

high speed.

See {6.06.07} Graphics Mode Register.

4.    Packed Pixel 256 Colour

-----------------------

This display mode ensures the highest image resolution

and complexity.  As a result the display memory needed

is larger than with any other mode.  This can mean the

quantity of data accessed when changing an image is an

important determinant of processing speed.  A pixel is

represented by a single byte of data.  In this way, it

is the simplest mode, from the point of view of memory

organisation.

See {6.06.07} Graphics Mode Register.

<1>      Memory Plane Pairing Control

----------------------------

<=0>  No plane pairing.  Address bit 0 is not replaced.

<=1>  Memory planes are paired.  The processor address bit 0 gives the

plane pair to access.  Even addresses access planes (0,2) whilst

odd addresses access planes (1,3).  Address bit 0 is replaced by

a higher order address bit.

This is similar to Processor Image Memory Access Mode Select.

See {6.04.06} Sequencer Memory Mode Register.

<3..2>   Memory Map Mode

---------------

This field specifies the mapping of the display image memory into the

processor memory address space.  This changes the range of processor

addresses which are recognised as being display memory references.

<=00>  Host processor address range is {a0000h .. bffffh}.

<=01>  Host processor address range is {a0000h .. affffh}.

High resolution graphics controls require this processor map

for correct operation {Trident 8900 only}.

See {6.10.05} Mode Control Register 1.

<=10>  Host processor address range is {b0000h .. b7fffh}.

<=11>  Host processor address range is {b8000h .. bffffh}.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.06.09  Colour Don't Care Register   { 3cfh    07h    R/W }

=======  ===================================================

Cross reference with {6.06.04} Colour Compare Register.

Cross reference with {6.06.06} Read Map Select Register.

Cross reference with {6.06.07} Graphics Mode Register.

<0>      Colour Don't Care Plane 0

-------------------------

During multiplane image memory read access operation by the processor,

this bit can determine the value of the byte representing the result

returned to the processor.

<=0>  Image plane zero data is not actively used for colour comparison

rather it is considered to have generated an unconditional match

with the colour data value specified, assuming read mode 1.

See {6.06.04} Colour Compare Register.

<=1>  Image plane zero is actively compared with the colour data value

specified.  The result depends directly on data from this plane,

assuming read mode 1.

See {6.06.04} Colour Compare Register.

The effect of this bit is determined by the read mode used to perform

the image memory read access operation.  Two different read modes are

provided as follows:

Read Mode 0

===========

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

When read mode 0 is active, a read operation by the processor returns

a single byte, which is associated with one of the four image planes.  The plan

The particular image plane is selected by means of a special register

provided specifically for this purpose.

See {6.06.06} Read Map Select Register.

See {6.06.07} Graphics Mode Register.

In read mode 0, the Colour Compare Register plays no active role in

processor read operation so the value of the Colour Compare Plane 0

bit and the Colour Don't Care Plane 0 bit is irrelevant.

Read Mode 1

===========

At any single address, each image plane stores one eight bit byte so

providing for 32 bits total storage spread across four planes.  Each

bit position on each of the four planes provides a four bit value so

that 16 colours can be represented.  In this way, eight neighbouring

pixels are completely described by the 32 bits at a single address.

This convention for image memory organisation is termed multiplane.

It is often required to determine where the border pixels of an area

are situated, perhaps in order to provide a colour fill function for

a graphics software package.  Usually in such a situation, either of

the colours associated with the foreground or background are a known

constant.  The business of discovering the border pixels becomes one

of tracing a region consisting of pixels with a known colour.

There is a way of doing this, using read mode 0 to recover the four

bytes of colour plane data at a given image memory address.  But it

would then be necessary to change those four bytes into eight pixels

each of 4 bits.  Whilst this is computationally simple, it is a very

time consuming operation to perform, especially with large images.

This is where read mode 1 can provide assistance.  Read mode 1 uses

a colour match specification to inspect an eight pixel image memory

address in parallel, producing a colour match bit for each pixel in

the result byte.  A matching colour produces a set bit result.

The match value for any plane can be made irrelevant by using a mask

value for the plane indicating a colour match don't care.  Any don't

care planes are considered to match under all circumstances.  So for

some colour ranges it is possible to obtain sensible match behaviour

even though the match result is expressed in a single bit.

See {6.06.04} Colour Compare Register.

See {6.06.07} Graphics Mode Register.

<1>      Colour Don't Care Plane 1

-------------------------

As for Colour Don't Care Plane 0, immediately above.

See {6.06.04} Colour Compare Register.

See {6.06.07} Graphics Mode Register.

<2>      Colour Don't Care Plane 2

-------------------------

As for Colour Don't Care Plane 0, immediately above.

See {6.06.04} Colour Compare Register.

See {6.06.07} Graphics Mode Register.

<3>      Colour Don't Care Plane 3

-------------------------

As for Colour Don't Care Plane 0, immediately above.

See {6.06.04} Colour Compare Register.

See {6.06.07} Graphics Mode Register.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.06.10  Bit Mask Register            { 3cfh    08h    R/W }

=======  ===================================================

Cross reference with {6.06.02} Set/Reset Register.

<0>   Update Value Source Bit 0

-------------------------

<=0>  This bit position in all write enabled planes will be sourced from

the current read latch contents associated with those planes.  The

interpretation of this bit can be simplified when the read latches

have been loaded with the original image data.  In that case, this

option signifies that this pixel number will be write inhibited in

all four image memory planes.

This bit is used in write modes (0, 2, 3).

See {6.06.02} Set/Reset Register.

<=1>  This bit position in all write enabled planes will be sourced from

the logic unit output data path associated with those planes.  The

interpretation of this bit can be simplified when the read latches

have been loaded with the original image data.  In that case, this

option signifies that this pixel number will be enabled for write,

across all the four image memory planes.

This bit is used in write modes (0, 2, 3).

See {6.06.02} Set/Reset Register.

<1>   Update Value Source Bit 1

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<2>   Update Value Source Bit 2

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<3>   Update Value Source Bit 3

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<4>   Update Value Source Bit 4

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<5>   Update Value Source Bit 5

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<6>   Update Value Source Bit 6

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

<7>   Update Value Source Bit 7

-------------------------

This bit is used in write modes (0, 2, 3).

See Update Value Source Bit 0, immediately above.

See {6.06.02} Set/Reset Register.

6.07  Register specifications: Attribute Controller

====  =============================================

6.07.01  Attribute Address Register       { 3c0h/I/W  3c0h/R  ---  R/W }

=======  ===================================================================

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.09.04} Input Status Register One.

Cross reference with {6.10.10} Attribute State Read Back Register.

Cross reference with {6.10.11} Attribute Index Read Back Register.

<4..0>   Attribute Address

-----------------

The attribute controller address register defines the index number of

the register in the internal register array.  All reads and writes to

the attribute controller are made to this selected register.

Port 3c0h is used for writes to both the Attribute Address Register,

and to the selected register array index.  Port 3c0h is also used to

read the Attribute Address Register.  The selected internal register

index can always be read using port 3c1h.

In this scheme, port 3c0h is used for writing the index value for the

internal register access control, and also for writing the value into

the selected register.  Obviously there must be a means for resolving

this ambiguity of write access.

There is an internal state which chooses whether to write the index

value, or the register being indexed.  This state can be reset with

a read to port 3?ah {where ? represents either hex 'b' or hex 'd'},

so that the next write is to the Attribute Address Register rather

than to an indexed internal register.

See {6.09.01} Miscellaneous Output Register.

See {6.09.04} Input Status Register One.

Having written the Attribute Address (that is, the register index),

successive writes are to the indexed register, then to the value of

the index, and so on, alternately.  This is very useful when a word

is written to port 3c0h, since the first byte controls the index of

the register, which is then written into by the second byte.

The internal register index value can be obtained, as can the state

which determines index access or register access for write purposes.

See {6.10.10} Attribute State Read Back Register.

See {6.10.11} Attribute Index Read Back Register.

Note that no official documentation describes the existance of this

decode of port 3c1h for such write purposes.  However, the hardware

does respond correctly in this manner.  This is a standard feature.

<5>      Palette Control

---------------

<=0>  Disable the generation of a video signal, permitting processor

access to the attribute controller colour registers.  Note the

display will become unstable, should this option be set whilst

the active image is being generated.  For this reason vertical

retrace should be active for the duration of this setting.

<=1>  Enable video signal generation and prevent processor access to

the attribute controller colour registers.

<7..6>   Reserved.  Always <=00>.

------------------------

6.07.02  Palette Register 00              { 3c0h/I/W  3c1h/R  00h  R/W }

=======  ===================================================================

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.07.03} Palette Register 01.

Cross reference with {6.07.04} Palette Register 02.

Cross reference with {6.07.05} Palette Register 03.

Cross reference with {6.07.06} Palette Register 04.

Cross reference with {6.07.07} Palette Register 05.

Cross reference with {6.07.08} Palette Register 06.

Cross reference with {6.07.09} Palette Register 07.

Cross reference with {6.07.10} Palette Register 08.

Cross reference with {6.07.11} Palette Register 09.

Cross reference with {6.07.12} Palette Register 10.

Cross reference with {6.07.13} Palette Register 11.

Cross reference with {6.07.14} Palette Register 12.

Cross reference with {6.07.15} Palette Register 13.

Cross reference with {6.07.16} Palette Register 14.

Cross reference with {6.07.17} Palette Register 15.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.07.20} Colour Plane Enable Register.

Cross reference with {6.07.22} Colour Select Register.

Cross reference with {6.08.01} DAC Pixel Mask Register.

Cross reference with {6.08.05} DAC Data Register.

<5..0>   Colour Value

------------

This binary field expresses the value of an index into the DAC colour

table.  {DAC is an acronym for Digital to Analogue Converter.}  A DAC

converts eight bit digital colour information to analogue red, green,

blue information from which all required colours can be mixed.  There

is a great deal of context surrounding colour generation, which needs

to be understood before the palette operation can become clear.

The manner in which pixel colour is converted into display colour is

actually non-trivial, offering powerful control facilities which are

suitable for high speed animation.  We now describe all the features

for colour control of an image working forewards from the pixel data

and backwards from the displayed colours.

The size of a pixel in bits determines the range of colours which can

be generated within any displayed image.  However, the actual colours

displayed can be composed from a very wide range of possible colours.

To see how this can be, we look at the DACs which generate colour out

to the display unit.  Each DAC accepts 8 bits of digital code thereby

producing 8 bits of analogue signal into one of the three colour guns

in the display tube.  So all combinations of three 8 bit colours will

give a colour spectrum of 24 bits resolution. This gives a full range

of 16_777_216 colours in total.

The three DAC units accept 8 bit digital colour code from a specially

designed memory unit, which stores for each of 256 addresses, 24 bits

of colour data.  Each 24 bit block contains three groups of eight bit

colour data, associated respectively with the red green and blue DAC.

The 256 addresses can be fully determined with an 8 bit digital code.

The 256 colour memory unit is termed an external colour palette.  It

is this palette which governs the true colours actually sent into the

display unit.  Since the external colour palette converts 8 bit codes

into displayed colours, it is clear that no display can use more than

256 distinct colours.

See {6.08.05} DAC Data Register.

There is another colour palette memory unit, which provides for just

16 separate addresses.  This colour palette is the internal palette,

which is located within the VGA controller.  It is distinguished from

the external colour palette by being termed simply the colour palette.

It stores 6 bits of colour data for each of the 16 addresses.  Only 4

bits of digital code are needed to completely represent 16 addresses.

Display image memory is split into four separately accessible planes,

each of which is associated with a shift register, which presents bit

data to an output bit corresponding with the same plane.  In this way

data from image memory is converted into a serial data stream so that

parallel bits in the stream specify a colour to display.

See {6.06.07} Graphics Mode Register.

These parallel bits are used to index the colour palette, which gives

the 4 bits of address needed to completely range over every entry the

colour palette contains.  As a result of this, the colour palette can

deliver a 6 bit code output.  This 6 bit code is combined with 2 bits

from a special register to make the 8 bit code needed by the external

colour palette to generate real colour signals into the display unit.

See {6.07.22} Colour Select Register.

Note that the number of displayed colours is now constrained to 4 bit

colour specified by pixel representation, rather than the 8 bits used

as input to the external colour palette.

That is the overview.  In practise, things can be more involved.

In multiplane 16 colour mode, the situation is altered slightly by

virtue of there being a choice of formation of the 8 bit code sent

into the external colour palette.  The shift register outputs sent

to the internal colour palette can be individually inhibited.  The

effect of this is to reduce the number of displayed colours whilst

increasing the quantity of image information available.  Therefore

each of the four image memory planes, can be individually selected

for display.  This display will be monochrome, but 4 images can be

defined, maintained and rapidly switched between.

See {6.07.20} Colour Plane Enable Register.

We will assume for the moment that all 4 colour bits are operational

so the display will be in 16 colours. There are two separate methods

for determining how the pixel data becomes display colour.

The first method mixes just 2 additional bits with the 6 bits output

from the colour palette.  This provides a three way control for each

displayed colour.  The two additional bits provide instant selection

between 4 distinct colour maps, so by modifying these 2 bits, colour

modification of the entire image will instantly take place.  Also it

is possible to change the contents of the colour palette so that the

colours generated change.  External colour palette data changes will

also alter the displayed image colour at any time but the larger the

palette, the longer it takes to modify.

See {6.07.18} Attribute Mode Control Register.

See {6.07.22} Colour Select Register.

The second code formation method mixes 4 additional bits with the

lowest 4 bits from the colour palette output.  This provides more

capability to instantly change the colour mapping between upto 16

separate colour assignments, each of a full 16 colours.

See {6.07.18} Attribute Mode Control Register.

See {6.07.22} Colour Select Register.

When the display is generated in 256 colour packed pixel mode the

shift register outputs become completely divorced from notions of

image plane association.  Instead, the colour palette is ignored,

with the shift register outputs driving the 8 bit external colour

palette inputs directly.  In this case, only by changing the data

held in the external colour palette can the display colour change

be effected.  However, there is potential to reduce the number of

displayed colours, which results in greater ability to change the

actual colour set displayed.  This happens by means of inhibition

of some of the 8 bits input to the external colour palette.  When

only 7 bits are actually in use, changing the bit 7 for bit 8 can

radically alter the colour display.  If more bits are kept out of

immediate use, the greater the potential for rapid changes in the

display colour set.

See {6.08.01} DAC Pixel Mask Register.

<7..6>   Reserved.  Always <=00>.

------------------------

Schematic Data Flow From Shift Register Outputs to Display Colours

==================================================================

Scheme A:  Colour Select Register provides 2 additional bits.

=============================================================

See {6.07.22} Colour Select Register.

Colour Select

===========--

<----.32-->

||

||

||

||            Multiplane

||            Shift Register

||            Colour Outputs

||            ===========---

||            <----.3210>

||                  ||||

||                  ||||

||                  ||||

||                  ||||

||                  ||||          [Internal]

||                  ||||          Colour

||                  ||||          Palette

||                  ||||          ===========

||                  ||||     <0>  | 00

||                  ||||........>.| 01

||                  |||.......>...| 02

||                  ||..........>.|  .

||                  |.........>...|  .

||                           <3>  |  .

||                                |  .

||                                | 13

||                                | 14

||                                | 15

||                                ===========

||                                <--54.3210>

||                                   || ||||

||                                   || ||||

||                                   || ||||

||...................................|| ||||

|...................................||| ||||

|||| ||||

|||| ||||

|||| ||||

|||| ||||

DAC Pixel Mask                           |||| ||||

===========---                           |||| ||||

<7654.3210>                              |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                             -------------

|||| ||||.........................>...| mask bit0 |

|||| |||........................>.....| mask bit1 |

|||| ||...........................>...| mask bit2 |

|||| |..........................>.....| mask bit3 |

||||..............................>...| mask bit4 |

|||.............................>.....| mask bit5 |

||................................>...| mask bit6 |

|...............................>.....| mask bit7 |

-------------

|||| ||||

|||| ||||

|||| ||||

|||| ||||

[256 Colours]

|||| ||||

|||| ||||

|||| ||||

|||| ||||

.........................................|||| ||||

|.........................................||| ||||

||.........................................|| ||||

|||.........................................| ||||

|||| .........................................||||

|||| |.........................................|||

|||| ||.........................................||

|||| |||.........................................|

|||| ||||

|||| ||||

|||| ||||

|||| ||||

|||| ||||           External Colour Palette

|||| ||||           RED        GREEN      BLUE

|||| ||||      <0>  =================================

|||| ||||.........>.| 000      | 000      | 000

|||| |||........>...| 001      | 001      | 001

|||| ||...........>.| 002      | 002      | 002

|||| |..........>...|  .       |  .       |  .

||||..............>.|  .       |  .       |  .

|||.............>...|  .       |  .       |  .

||................>.|  .       |  .       |  .

|...............>...| 253      | 253      | 253

<7>  | 254      | 254      | 254

| 255      | 255      | 255

=================================

<7654.3210><7654.3210><7654.3210>

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  ==========

|||| ||||  |||| ||||  BLUE DAC |

|||| ||||  |||| ||||  inputs   |..>......

|||| ||||  |||| ||||  ==========        |

|||| ||||  ===========                  |

|||| ||||  GREEN DAC |                  |

|||| ||||  inputs    |..>.........>.... |

|||| ||||  ===========                | |

=========                             | |

RED DAC |                             | |

inputs  |..>............>.........>.. | |

=========                           | | |

| | |

| | |

| | |

| | |

========

Analogue

Output

========

See {6.06.07} Graphics Mode Register.

See {6.07.22} Colour Select Register.

See {6.08.01} DAC Pixel Mask Register.

-------------------------------------------------------------------

Schematic Data Flow From Shift Register Outputs to Display Colours

==================================================================

Scheme B:  Colour Select Register provides 4 additional bits.

=============================================================

See {6.07.22} Colour Select Register.

Colour Select

===========--

<----.3210>

||||

||||

||||

||||          Multiplane

||||          Shift Register

||||          Colour Outputs

||||          ===========---

||||          <----.3210>

||||                ||||

||||                ||||

||||                ||||

||||                ||||

||||                ||||          [Internal]

||||                ||||          Colour

||||                ||||          Palette

||||                ||||          ===========

||||                ||||     <0>  | 00

||||                ||||........>.| 01

||||                |||.......>...| 02

||||                ||..........>.|  .

||||                |.........>...|  .

||||                         <3>  |  .

||||                              |  .

||||                              | 13

||||                              | 14

||||                              | 15

||||                              ===========

||||                              <----.3210>

||||                                    ||||

||||                                    ||||

||||................................... ||||

|||...................................| ||||

||...................................|| ||||

|...................................||| ||||

|||| ||||

|||| ||||

|||| ||||

|||| ||||

DAC Pixel Mask                           |||| ||||

===========---                           |||| ||||

<7654.3210>                              |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                             -------------

|||| ||||.........................>...| mask bit0 |

|||| |||........................>.....| mask bit1 |

|||| ||...........................>...| mask bit2 |

|||| |..........................>.....| mask bit3 |

||||..............................>...| mask bit4 |

|||.............................>.....| mask bit5 |

||................................>...| mask bit6 |

|...............................>.....| mask bit7 |

-------------

|||| ||||

|||| ||||

|||| ||||

|||| ||||

[256 Colours]

|||| ||||

|||| ||||

|||| ||||

|||| ||||

.........................................|||| ||||

|.........................................||| ||||

||.........................................|| ||||

|||.........................................| ||||

|||| .........................................||||

|||| |.........................................|||

|||| ||.........................................||

|||| |||.........................................|

|||| ||||

|||| ||||

|||| ||||

|||| ||||

|||| ||||           External Colour Palette

|||| ||||           RED        GREEN      BLUE

|||| ||||      <0>  =================================

|||| ||||.........>.| 000      | 000      | 000

|||| |||........>...| 001      | 001      | 001

|||| ||...........>.| 002      | 002      | 002

|||| |..........>...|  .       |  .       |  .

||||..............>.|  .       |  .       |  .

|||.............>...|  .       |  .       |  .

||................>.|  .       |  .       |  .

|...............>...| 253      | 253      | 253

<7>  | 254      | 254      | 254

| 255      | 255      | 255

=================================

<7654.3210><7654.3210><7654.3210>

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  ==========

|||| ||||  |||| ||||  BLUE DAC |

|||| ||||  |||| ||||  inputs   |..>......

|||| ||||  |||| ||||  ==========        |

|||| ||||  ===========                  |

|||| ||||  GREEN DAC |                  |

|||| ||||  inputs    |..>.........>.... |

|||| ||||  ===========                | |

=========                             | |

RED DAC |                             | |

inputs  |..>............>.........>.. | |

=========                           | | |

| | |

| | |

| | |

| | |

========

Analogue

Output

========

See {6.06.07} Graphics Mode Register.

See {6.07.22} Colour Select Register.

See {6.08.01} DAC Pixel Mask Register.

-------------------------------------------------------------------

Schematic Data Flow From Shift Register Outputs to Display Colours

==================================================================

Scheme C:  Packed Pixel 256 Colour Format

=========================================

Packed Pixel

Shift Register

Assembly Latch

Colour Outputs

===========

<7654.3210>

|||| ||||

|||| ||||

|||| ||||

|||| ||||

DAC Pixel Mask                           |||| ||||

===========---                           |||| ||||

<7654.3210>                              |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                               |||| ||||

|||| ||||                             -------------

|||| ||||.........................>...| mask bit0 |

|||| |||........................>.....| mask bit1 |

|||| ||...........................>...| mask bit2 |

|||| |..........................>.....| mask bit3 |

||||..............................>...| mask bit4 |

|||.............................>.....| mask bit5 |

||................................>...| mask bit6 |

|...............................>.....| mask bit7 |

-------------

|||| ||||

|||| ||||

|||| ||||

|||| ||||

[256 Colours]

|||| ||||

|||| ||||

|||| ||||

|||| ||||

.........................................|||| ||||

|.........................................||| ||||

||.........................................|| ||||

|||.........................................| ||||

|||| .........................................||||

|||| |.........................................|||

|||| ||.........................................||

|||| |||.........................................|

|||| ||||

|||| ||||

|||| ||||

|||| ||||

|||| ||||           External Colour Palette

|||| ||||           RED        GREEN      BLUE

|||| ||||      <0>  =================================

|||| ||||.........>.| 000      | 000      | 000

|||| |||........>...| 001      | 001      | 001

|||| ||...........>.| 002      | 002      | 002

|||| |..........>...|  .       |  .       |  .

||||..............>.|  .       |  .       |  .

|||.............>...|  .       |  .       |  .

||................>.|  .       |  .       |  .

|...............>...| 253      | 253      | 253

<7>  | 254      | 254      | 254

| 255      | 255      | 255

=================================

<7654.3210><7654.3210><7654.3210>

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  |||| ||||

|||| ||||  |||| ||||  ==========

|||| ||||  |||| ||||  BLUE DAC |

|||| ||||  |||| ||||  inputs   |..>......

|||| ||||  |||| ||||  ==========        |

|||| ||||  ===========                  |

|||| ||||  GREEN DAC |                  |

|||| ||||  inputs    |..>.........>.... |

|||| ||||  ===========                | |

=========                             | |

RED DAC |                             | |

inputs  |..>............>.........>.. | |

=========                           | | |

| | |

| | |

| | |

| | |

========

Analogue

Output

========

See {6.06.07} Graphics Mode Register.

See {6.08.01} DAC Pixel Mask Register.

-------------------------------------------------------------------

6.07.03  Palette Register 01              { 3c0h/I/W  3c1h/R  01h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.04  Palette Register 02              { 3c0h/I/W  3c1h/R  02h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.05  Palette Register 03              { 3c0h/I/W  3c1h/R  03h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.06  Palette Register 04              { 3c0h/I/W  3c1h/R  04h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.07  Palette Register 05              { 3c0h/I/W  3c1h/R  05h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.08  Palette Register 06              { 3c0h/I/W  3c1h/R  06h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.09  Palette Register 07              { 3c0h/I/W  3c1h/R  07h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.10  Palette Register 08              { 3c0h/I/W  3c1h/R  08h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.11  Palette Register 09              { 3c0h/I/W  3c1h/R  09h  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.12  Palette Register 10              { 3c0h/I/W  3c1h/R  0ah  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.13  Palette Register 11              { 3c0h/I/W  3c1h/R  0bh  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.14  Palette Register 12              { 3c0h/I/W  3c1h/R  0ch  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.15  Palette Register 13              { 3c0h/I/W  3c1h/R  0dh  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.16  Palette Register 14              { 3c0h/I/W  3c1h/R  0eh  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.17  Palette Register 15              { 3c0h/I/W  3c1h/R  0fh  R/W }

=======  ===================================================================

Cross reference with {6.07.02} Palette Register 00.

See {6.07.02} Palette Register 00.

6.07.18  Attribute Mode Control Register  { 3c0h/I/W  3c1h/R  10h  R/W }

=======  ==================================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.04.05} Character Map Select Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.07.02} Palette Register 00.

Cross reference with {6.07.21} Horizontal PEL Panning Register.

Cross reference with {6.07.22} Colour Select Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.04} Mode Control Register 2.

<0>      Display Generation Mode

-----------------------

See {6.06.08} Miscellaneous Register.

<=0>  Text Mode

---------

Hardware character generation is invoked for display generation.

There is no capability to generate graphics in the usual manner

but a software technique can be used, giving a limited graphics

capability.  The technique depends upon assigning characters to

a display area, then re-writing the character fonts assigned to

those characters.  In this manner a limited graphics capability

can be supported within a relatively small area of the display.

The text mode display stores character codes in memory plane 0,

with their associated attribute bytes in plane 1.  The hardware

expects character font data to be stored at specified addresses

in plane 2.  Memory plane 3 is unused in this display mode.

The main characteristics of this mode are high speed operation,

with very low associated host processor overhead.  A number of

separate pages of text can be maintained, and switched between

at high speed.  This is the ideal mode to employ where no true

graphics capability is required, other than that provided with

the specially reserved graphics drawing characters.  Two fonts

can be used within one display screen, each font consisting of

256 characters in total.  User defined fonts can be used, when

occasion demands.

See {6.04.03} Clocking Mode Register.

See {6.04.05} Character Map Select Register.

See {6.05.22} Underline Location Register.

<=1>  Graphics Mode

-------------

The display will be generated with reference to an image memory,

organised as a collection of pixel data.  There is no capability

for the support of hardware text generation, but software can be

written to support multiple character fonts and any size of font

necessary for the application.

Display image memory organisation is dependant upon the range of

colours available for definition of a pixel.  The principle mode

options include:

1.    Multiplane 16 Colour

--------------------

The display memory is arranged as four parallel planes

each of which holds data for eight pixels by splitting

four colour bits across four planes.  Each plane gives

access to a single byte when presented with an address

so with parallel address spaces, each access returns 4

bytes.  The 32 bits specify 4 bits of colour data, for

each of eight pixels.

See {6.06.07} Graphics Mode Register.

2.    Packed pixel Monochrome

-----------------------

The display memory is arranged as a linear array which

is addressed in a manner corresponding directly to the

screen pixel display.  One bit per pixel specifies any

colour required, from a large available palette.  This

choice is normally restricted by making black the fore

ground colour.  The display image is easily managed as

it is of small size.  A large number of distinct pages

can be maintained and quickly switched for one another

which is very useful for hierarchical design systems.

See {6.06.07} Graphics Mode Register.

3.    Packed Pixel 4 Colour

---------------------

This display mode requires double the amount of memory

compared with monochrome mode, but provides 4 colours,

which can be much more flexible in some applications.

Each pixel is represented by two neighbouring bits, so

four pixels are collected together in one byte.  There

are special hardware capabilities available, to decode

this special colour representation.

Since images are still very compact large displays can

be manipulated without great memory overhead; a number

of distinct pages can be held in memory and swapped at

high speed.

See {6.06.07} Graphics Mode Register.

4.    Packed Pixel 256 Colour

-----------------------

This display mode ensures the highest image resolution

and complexity.  As a result the display memory needed

is larger than with any other mode.  This can mean the

quantity of data accessed when changing an image is an

important determinant of processing speed.  A pixel is

represented by a single byte of data.  In this way, it

is the simplest mode, from the point of view of memory

organisation.

See {6.06.07} Graphics Mode Register.

<1>      Display Generation Mode

-----------------------

<=0>  Select colour display attribute definitions for supporting

colour emulation mode.

<=1>  Select monochrome display attribute definitions for supporting

monochrome emulation mode.

This selection governs the treatment of attribute byte decoding, in

text mode.  Differences exist in expressive capability of attributes

depending upon whether monochrome or colour emulation is active.

To see these differences, consult the specification of the attribute

byte under both emulation schemes.

See {6.05.22} Underline Location Register.

<2>      Line Graphics Character Code Control

------------------------------------

<=0>  When using 9 dot character clocks, this option specifies that

the ninth dot will always be set to the background colour for

character generation in text mode.  This setting implies that

there are no special continuous line graphics characters used

in the character font.

See {6.04.03} Clocking Mode Register.

<=1>  When using 9 dot character clocks, and a graphics character is

to be generated in text mode with a code value between hex 'c0'

and hex 'df' inclusive the ninth dot will be displayed exactly

as was the eighth dot.  This provides for character fonts with

the full eight pixels width, without any problems of merging.

Where continuous line graphics are used, however, merging must

occur selectively for all the graphics character codes.  These

graphic codes are not always present in all character sets, so

there must be a means to disable them selectively.

See {6.04.03} Clocking Mode Register.

<3>      Text Mode Attribute Select

--------------------------

<=0>  The text mode attribute byte associated with each character for

display defines bit <7> in one of two different ways: with this

option selection, bit <7> specifies background intensity.  When

the intensity bit is set a high background colour luminosity is

displayed, drawing attention to the foreground text display.

See {6.05.22} Underline Location Register.

<=1>  The text mode attribute byte associated with each character for

display defines bit <7> in one of two different ways: with this

option selection, bit <7> specifies background blink.  When the

blink bit is set, the background switches on and off, driven by

the vertical retrace signal.  In this manner attention is drawn

to the text display in the foreground.

See {6.05.22} Underline Location Register.

<4>      Reserved.  Always <=0>.

-----------------------

<5>      Split Screen Panning Control

----------------------------

<=0>  Scroll both screens horizontally {this is panning} in the same

manner, as defined by the various control registers.  This has

utility only when a split screen display is operational.

<=1>  Scroll only the upper screen {that is upper window} of a split

screen display in the horizontal direction, leaving the screen

in the lower position intact.  This only has significance when

a split screen display creates an upper and lower window.

See {6.05.10} Preset Row Scan Register.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

See {6.05.26} Line Compare Register.

See {6.07.21} Horizontal PEL Panning Register.

<6>      Shift Register 256 Colour Assembly Latch

----------------------------------------

<=0>  The 4 bit shift register output bits are sent for translation

into the internal 16 address colour palette, producing 6 bits

of colour output.  These bits are increased to 8 bits through

various options, and the resulting 8 bit code is presented to

the external colour palette, which drives the display unit.

See External Colour Palette Input Select, immediately below.

This setting should be used for 2 colour, 4 colour, 16 colour

displays, in either graphics mode or text mode.

See {6.06.07} Graphics Mode Register.

See {6.06.08} Miscellaneous Register.

<=1>  The 4 bit shift register output bits do not go to the internal

colour palette for translation.  Instead, using two successive

dot clock periods 4 bits of shift register output are combined

together to produce one 8 bit pixel colour value.  Every other

dot clock produces a new 256 colour pixel which is sent direct

to the external colour palette and thence the display unit.

This setting should only ever be used to produce packed pixel

displays supporting 256 colours.  There are implications for

the dot clock frequency when this mode is employed.

See {6.06.07} Graphics Mode Register.

There are controls available to select dot clock frequency and

dot clock division ratios.

See {6.09.01} Miscellaneous Output Register.

See {6.10.04} Mode Control Register 2.

<7>      External Colour Palette Input Select

------------------------------------

<=0>  For all display modes other than 256 colour packed pixel mode,

this option selects the source of bits <5..4> which are output

to the external colour palette for driving the display unit.

With this option, these bits are sourced from the internal

colour palette output bits <5..4>.  In this case there are

two bits provided by a register to make up these 6 bits to

the 8 bits required to drive the external colour palette.

See {6.07.22} Colour Select Register.

<=1>  For all display modes other than 256 colour packed pixel mode,

this option selects the source of bits <5..4> which are output

to the external colour palette for driving the display unit.

For this option setting, these bits are sourced from a special

register provided for the purpose.  This register normally has

responsibility for providing two additional bits to make the 8

bit group needed for input to the external colour palette.  In

this case, the register provides the original two bits with an

extra two bits, making four in total.  Under this circumstance

the internal colour palette output is restricted to four bits,

instead of the usual six bits.  These four bits are the lowest

significance bits.

See {6.07.22} Colour Select Register.

Note that the Shift Register 256 Colour Assembly Latch must be <=0>

if this option is to have any effect.  See immediately above.

See Shift Register 256 Colour Assembly Latch, immediately above.

See {6.06.07} Graphics Mode Register.

6.07.19  Overscan Colour Register         { 3c0h/I/W  3c1h/R  11h  R/W }

=======  ===================================================================

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.03} Horizontal Display Enable End Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.20} Vertical Display Enable End Register.

<7..0>   Overscan Colour

---------------

At times when the active image is not being produced, and yet there

is no blanking in progress the external colour palette must produce

some displayable colour.  The selection of this colour is performed

by the binary field in this register.

A discussion of horizontal image scan line generation is available.

This is appropriate to border colour generation on the boundaries

of each active image scan line.

See {6.05.02} Horizontal Total Register.

See {6.05.03} Horizontal Display Enable End Register.

See {6.05.04} Start Horizontal Blanking Register.

See {6.05.05} End Horizontal Blanking Register.

See {6.05.20} Vertical Display Enable End Register.

A discussion of vertical timing generation is available.  This is

useful to appreciate border colour generation during non-active

image scan line generation.

See {6.05.08} Vertical Total Register.

6.07.20  Colour Plane Enable Register     { 3c0h/I/W  3c1h/R  12h  R/W }

=======  ===============================================================

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.07.02} Palette Register 00.

Cross reference with {6.09.04} Input Status Register One.

<0>      Colour Plane Enable 0

---------------------

<=0>  Disable all output from a shift register(s) associated with this

image memory plane.  The output is set to be a constant zero bit

to provide a stable input to the colour tables.

<=1>  Enable shift register(s) output associated with this image plane

so the data shifted out will provide input to the colour tables.

There is discussion of shift register operation together with diagrams

showing shift register data paths and output data paths.

See {6.06.07} Graphics Mode Register.

<1>      Colour Plane Enable 1

---------------------

<=0>  Disable all output from a shift register(s) associated with this

image memory plane.  The output is set to be a constant zero bit

to provide a stable input to the colour tables.

<=1>  Enable shift register(s) output associated with this image plane

so the data shifted out will provide input to the colour tables.

There is discussion of shift register operation together with diagrams

showing shift register data paths and output data paths.

See {6.06.07} Graphics Mode Register.

<2>      Colour Plane Enable 2

---------------------

<=0>  Disable all output from a shift register(s) associated with this

image memory plane.  The output is set to be a constant zero bit

to provide a stable input to the colour tables.

<=1>  Enable shift register(s) output associated with this image plane

so the data shifted out will provide input to the colour tables.

There is discussion of shift register operation together with diagrams

showing shift register data paths and output data paths.

See {6.06.07} Graphics Mode Register.

<3>      Colour Plane Enable 3

---------------------

<=0>  Disable all output from a shift register(s) associated with this

image memory plane.  The output is set to be a constant zero bit

to provide a stable input to the colour palette.

<=1>  Enable shift register(s) output associated with this image plane

so the data shifted out will provide input to the colour tables.

There is discussion of shift register operation together with diagrams

showing shift register data paths and output data paths.

See {6.06.07} Graphics Mode Register.

<5..4>   Display Status Select

---------------------

This binary field selects two of the eight colour select output bits

to be read back using the register reserved for this use.  Which are

selected are detailed by the table below.

See {6.09.04} Input Status Register One.

Display Status Select

=====----------------

Input Status Register One, bit 5

=====          ==------------------------------

Input Status Register One, bit 4

=====          ==          ==------------------------------

<=00>          c2          c0

<=01>          c5          c4

<=10>          c3          c1

<=11>          c7          c6

------------------------------------------------------------

<7..6>   Reserved.  Always <=00>.

------------------------

6.07.21  Horizontal PEL Panning Register  { 3c0h/I/W  3c1h/R  13h  R/W }

=======  ===============================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.26} Line Compare Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

<3..0>   Horizontal PEL Panning

----------------------

PEL panning {Picture ELement panning} operates in both graphics mode

and text mode displays, so it is always operational.  Panning shifts

the entire display image to the left of the screen loosing pixels on

the left and introducing new pixels to the right.  It is implemented

in hardware by missing out pixels at the beginning of a scan line so

that the scan line active display starts at a point after the actual

start of image data for the scan line.  This means of course, that a

scan line to be panned in this way must be wider than the screen, so

that there is always some data which cannot be displayed.  It is the

hidden data which is brought into view by the panning process.

There is a discussion of panning elsewhere, which is concerned with

byte panning controls which can be used with pixel panning controls

to achieve continuous, smooth horizontal panning.

See {6.05.10} Preset Row Scan Register.

The value provided in order to control pixel panning varies depending

upon the way the display is generated.  In particular, there may be 8

or 9 dot clock periods per character clock period.  This reflects the

fact that in text mode displays, there is a choice for interpretation

of a character font.  When the full 8 pixel width of a font expresses

a true 8 pixel width character shape, then an extra pixel must be put

into the display after each character position, so that characters do

not merge with each other.  This is the motivation for providing nine

dot clock periods per character clock.  When a character font ignores

the 8th pixel, setting it always to the background colour there is no

need to provide for character separation by hardware.  In this case 8

dot clock periods per character clock period are all that is required.

See {6.04.03} Clocking Mode Register.

The type of graphics display generation scheme also affects the value

used to control pixel panning operation.  This arises because panning

is implemented in hardware by means of special shift registers, which

use the structure of a serial data stream to shift the image.  In the

most commonly used display mode, 16 colour multiplane mode each pixel

is represented by one position in the panning shift registers.  Where

a pixel occupies more than one panning shift register position, shift

values must change accordingly.  This affects the behaviour of images

built using 9 dot clocks, and those using packed pixel modes with the

pixel represented by more than one bit.

Control Value for Horizontal Pixel Panning

===---------------------------------------

Actual panned distance, in pixels, on screen

===    ===============================================================

Text Mode, 8 dot clock character period

===    ===------------------------------------

Text Mode, 9 dot clock character period

===    ===    ===------------------------------------

Multiplane 16 colour mode

===    ===    ===    ===----------------------

Packed Pixel 2 colour mode

===    ===    ===    ===    ===-----------------------

Packed Pixel 4 colour mode

===    ===    ===    ===    ===    ===-----------------------

Packed Pixel 256 colour mode

===    ===    ===    ===    ===    ===    ===-------------------------

0      0      1 !    0      0      0      0 !

1      1      2 !    1      1      1      * !

2      2      3 !    2      2      2      1 !

3      3      4 !    3      3      3      * !

4      4      5 !    4      4      4      2 !

5      5      6 !    5      5      5      * !

6      6      7 !    6      6      6      3 !

7      7      8 !    7      7      7      * !

8      0      0 !    0      0      0      * !

Notes:

======

!  specifies that this entry is different from the majority of entries

and that special attention should be given to the entry as a result

of this.

*  specifies that the entry is 'not applicable'.  This arises because

of the way packed pixel 256 colour mode uses two dot clock periods

to construct an 8 bit colour code.  Each dot clock makes the shift

registers deliver one packet of 4 bits of colour code.  After each

two consecutive dot clocks, two four bit packets are combined with

the required 8 bit colour code as a result.  Therefore a two place

shift is required per pixel in 256 colour packed pixel mode.  This

'not applicable' entry should be avoided since it will not produce

the expected results.

See {6.06.07} Graphics Mode Register.

Formulae concerned with 8 dot clock character periods and 9 dot clock

character periods are as follows:

left_pan = (number_field + 0) mod 8

left_pan = (number_field + 1) mod 9

See {6.04.03} Clocking Mode Register.

----------------------------------------------------------------------

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.07.22  Colour Select Register           { 3c0h/I/W  3c1h/R  14h  R/W }

=======  ===============================================================

Cross reference with {6.07.02} Palette Register 00.

Cross reference with {6.07.18} Attribute Mode Control Register.

The Colour Select Register provides additional colour definition bits

which are used to increase the either 4 or 6 bits output, produced by

the internal colour palette, to the 8 colour bits as required for the

external colour palette input.

The number of bits produced by the internal colour palette is selected

by means of a control bit, which also governs the number of bits taken

from the Colour Select Register.  This is arranged so that there are 8

bits of resulting colour code, under all circumstances.  This control

is provided by the bit External Colour Palette Input Select.

See {6.07.18} Attribute Mode Control Register.

However, when the packed pixel 256 colour mode is in use, there is no

need to provide any additional colour data as all 8 bits required for

input to the external colour palette come directly from display image

memory.  Then there is no use for the Colour Select Register.  Option

control is governed by the Shift Register 256 Colour Assembly Latch.

See {6.07.18} Attribute Mode Control Register.

A detailed discussion with schematics is available for the data flow

between the memory image plane shift register outputs and the display

unit analogue colour inputs.

See {6.07.02} Palette Register 00.

<1..0>   External colour palette bits <5..4>

-----------------------------------

In all display generation modes except packed pixel 256 colour mode,

these bits are always available for optional use, controlled by the

Attribute Mode Control Register.  When selected for use, these bits

replace the internal colour palette output bits <5..4> becoming the

bits <5..4> input to the external colour palette.

See {6.07.18} Attribute Mode Control Register.

<3..2>   External colour palette bits <7..6>

-----------------------------------

In all display generation modes except packed pixel 256 colour mode,

these bits are always available for unconditional use.  They become

bits <7..6> input to the external colour palette.

See {6.07.02} Palette Register 00.

<7..4>   Reserved.  Always <=0000>.

--------------------------

6.08  Register specifications: DAC Support Logic

====  ==========================================

6.08.01  DAC Pixel Mask Register          { 3c6h   R/W }

=======  ===============================================

Cross reference with {6.07.02} Palette Register 00.

Cross reference with {6.08.02} DAC Status Register.

<0>      DAC Pixel Mask 0 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

The shift registers produce an output stream of width dependant upon

the display generation mode in use.  No matter what this data output

width actually is, ultimately it will be expanded into the 8 bits of

colour data required as input by the external colour palette.  Where

it is required that a reduction in addressing into this 256 location

memory be put into effect, it is possible to substitute zero bits in

place of specified colour input bits.  The substitution is permitted

or disabled by means of this DAC Pixel Mask Register, with a bit for

bit control correspondance.

<=0>  Send zero into the external colour palette at this input bit

position.

<=1>  Permit colour data at this input bit position to flow freely

into the external colour palette.

A detailed discussion of colour generation is available, with data

flow schematics.

See {6.07.02} Palette Register 00.

By selectively enabling and disabling individual colour select bits,

an image can be manipulated for different colour sets very rapidly.

Also, multiple images can be associated with different colour data,

and combined in different ways using this masking control technique.

Note that if the external colour palette is correctly initialised it

is possible to associate multiple images with identical colour sets,

thereby making multiple image animation sequences possible.  This is

possible only by using identical colour definitions, repeated across

many distinct addresses.

For operational purposes it should be noted that accesses to the

DAC Pixel Mask Register are not relevant for DAC Status Register

purposes.

See {6.08.02} DAC Status Register.

<1>      DAC Pixel Mask 1 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<2>      DAC Pixel Mask 2 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<3>      DAC Pixel Mask 3 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<4>      DAC Pixel Mask 4 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<5>      DAC Pixel Mask 5 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<6>      DAC Pixel Mask 6 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

<7>      DAC Pixel Mask 7 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------------------

As for DAC Pixel Mask 0 above.

6.08.02  DAC Status Register              { 3c7h   R-- }

=======  ===============================================

Cross reference with {6.08.01} DAC Pixel Mask Register.

Cross reference with {6.08.03} DAC Read Data Address Register.

Cross reference with {6.08.04} DAC Write Data Address Register.

Cross reference with {6.08.05} DAC Data Register.

<1..0>   DAC Status

----------

<=00>  The last access to the DAC Data Register was a processor write

access.

<=01>  This value significance is not assigned.

<=10>  This value significance is not assigned.

<=11>  The last access to the DAC Data Register was a processor read

access.

This binary field indicates the type of transaction last performed by

the processor on the DAC Data Register.  This information can be used

in a situation when an interrupt routine periodically writes into the

external colour palette, to modify the colour data held there.  After

such a write sequence has been completed in the interrupt routine, it

is important that any read or write activity which was in progress at

the time of the interrupt is restored transparently.  In practise, an

interrupt routine might well need to read and write colour data; this

situation is taken care of equally well by the DAC Status Register if

it is correctly used.

See {6.08.05} DAC Data Register.

An interrupt routine executes without synchronisation to main program

code.  This program could read from or write to the DAC Data Register

at any time, so when the interrupt occurs, the transaction state with

the DAC Data Register is effectively unknown.  Despite this, any such

trnsaction must be permitted to continue undisturbed, after interrupt

program execution is complete.

It is this problem which the DAC Status Register is designed to give

the solution to.

When the interrupt program gains control, there could be either of a

read transaction or a write transaction in progress from the program

which was executing immediately prior to the interrupt.  Whilst this

program would typically be the main program it could also be another

interrupt program, assigned to a different, lower interrupt level.

The first job the newly entered interrupt program must perform is to

secure the status of that program which has just had control wrested

from it.  This involves determining which type of transaction was in

progress at the time of interrupt.  By using the DAC Status Register

an indication is given as to the type of transaction, which was last

in progress.  This transaction might well be complete or it could be

still in progress.  This point is unknown but does not matter if the

object is only to preserve the status of the DAC Data Register.

Any read or write sequence involving the DAC Data Register begins by

means of a write from the processor into either of the two registers

DAC Read Data Address Register, or DAC Write Data Address Register.

This write to a particular address register is used to associate the

DAC Data Register with the appropriate transaction type, either read

or write.  It is also used to specify which of the address registers

should be associated with the autoincrement capability that provides

for long transfers of one type without need to update the addressing

register for that transaction type.

The read address is initialised prior to any read sequence, with the

write address set prior to any write sequence.  During that sequence

the relevant address register is updated automatically, so that long

data transfers in either direction can occur without the overhead of

resetting the address register.  This makes for fast program running

times and eases programming overheads.

However, after any other transaction type is used, the autoincrement

feature is switched off.  This means that writing a value to another

address register or performing a transaction of the opposite type is

sufficient to switch off autoincrement to the address register being

presently used.  This has the effect of freezing an address register

which in turn provides for recovering the present contents and later

reinstating them, for example, after an interrupt has completed.

It should now be clear that for an interrupt program to preserve any

existing read or write sequence on the DAC Data Register it is quite

enough to determine the last transaction type, and then to read from

the associated address register the current read or write address in

use by the previously executing program.  After the interrupt, there

is no difficulty in restoring the DAC Data Register program state by

using the preserved value from interrupt entry time.

See {6.08.03} DAC Read Data Address Register.

See {6.08.04} DAC Write Data Address Register.

For operational purposes it should be noted that accesses to the

DAC Pixel Mask Register are not relevant for DAC Status Register

purposes.

See {6.08.01} DAC Pixel Mask Register.

<7..2>   Reserved.  Always <=000000>.

----------------------------

6.08.03  DAC Read Data Address Register   { 3c7h   W-- }

=======  ===============================================

Cross reference with {6.08.02} DAC Status Register.

Cross reference with {6.08.04} DAC Write Data Address Register.

Cross reference with {6.08.05} DAC Data Register.

<7..0>   DAC Read Data Address

---------------------

This binary field determines the initial value for the address index

into the external colour palette, at which processor reads will take

place.  The external colour palette contains 256 distinct addresses,

each of which holds three colour values of up to 8 bits each for the

colours read, green and blue.  Typically either 6 bits or 8 bits for

each colour intensity are provided, depending upon the hardware.

Setting a new DAC Read Data Address value has the effect of freezing

the current value of the DAC Write Data Address, and also associates

the address autoincrement operation with the read transaction on the

DAC Data Register.

See {6.08.04} DAC Write Data Address Register.

See {6.08.05} DAC Data Register.

At this point, the processor can perform three read transactions on

the DAC Data Register, after which an automatic increment will make

the address point to the next group of three intensity values.  The

possibility of an interrupt occuring during this 3 consecutive read

sequence must be taken into account as the hardware state cannot be

interrogated to discover which of the three colours was the last to

be transferred.  In practise, most programs adopt a policy eshewing

use of interrupts during the three colour data transfer.  This will

effectively prevent such a transfer from being interrupted, and the

problems of an unknown hardware transfer state simply disappear.

The current read address can be discovered at any time, by virtue of

a read on the DAC Write Data Address Register.  This is necessary to

recover the hardware state at time of entry to an interrupt routine.

See {6.08.04} DAC Write Data Address Register.

The address register in use {either read or write} can also be found

by using a special status register.

See {6.08.02} DAC Status Register.

6.08.04  DAC Write Data Address Register  { 3c8h   R/W }

=======  ===============================================

Cross reference with {6.08.02} DAC Status Register.

Cross reference with {6.08.03} DAC Read Data Address Register.

Cross reference with {6.08.05} DAC Data Register.

<7..0>   DAC Write Data Address

----------------------

This binary field determines the initial value for the address index

into the external colour palette at which processor writes will take

place.  The external colour palette contains 256 distinct addresses,

each of which holds three colour values of up to 8 bits each for the

colours read, green and blue.  Typically either 6 bits or 8 bits for

each colour intensity are provided, depending upon the hardware.

Updating the DAC Write Data Address value has the effect of freezing

the current value of the DAC Read Data Address which also associates

the address autoincrement feature with any write transaction, to the

DAC Data Register.

See {6.08.03} DAC Read Data Address Register.

See {6.08.05} DAC Data Register.

At this point the processor can perform three write transactions on

the DAC Data Register, after which an automatic increment will make

the address point to the next group of three intensity values.  The

possibility of an interrupt occuring during these three consecutive

writes must be carefully considered as the hardware state cannot be

interrogated to discover which of the three colours was the last to

be transferred.  In practise, most programs adopt a policy eshewing

use of interrupts during the three colour data transfer.  This will

effectively prevent such a transfer from being interrupted, and the

problems of an unknown hardware transfer state simply disappear.

The current write address can be discovered at any time by virtue of

a read on the DAC Write Data Address Register.  This is necessary to

recover the hardware state at time of entry to an interrupt routine.

See {6.08.04} DAC Write Data Address Register.

The address register in use {either read or write} can also be found

by using a special status register.

See {6.08.02} DAC Status Register.

6.08.05  DAC Data Register                { 3c9h   R/W }

=======  ===============================================

Cross reference with {6.07.02} Palette Register 00.

Cross reference with {6.08.02} DAC Status Register.

Cross reference with {6.08.03} DAC Read Data Address Register.

Cross reference with {6.08.04} DAC Write Data Address Register.

<7..0>   DAC Data

--------

All processor read and write operations which transfer colour values

to and from the external colour palette are implemented using access

to this register.  The actual read or write address accessed is that

specified in the address register for either read or write functions.

See {6.08.03} DAC Read Data Address Register.

See {6.08.04} DAC Write Data Address Register.

The external colour palette accepts an 8 bit value which ranges over

256 separate addresses.  Each address stores three colour values for

red, green and blue intensities, respectively.  Since each colour is

of either 6 or 8 bits width to completely access all the colour data

at any given address, 3 byte sized transfers are used, in the strict

order red, green, blue.  The processor must issue 3 consecutive read

or write transactions to complete the access to the data at a single

address.  This can mean in practise that during the 3 byte transfer,

interrupts remain inhibited so that no indeterminate length accesses

can occur, so avoiding the hardware entering an unknown state.

To prevent the hardware entering an unknown state, all transactions

must first determine the current transaction type and address, then

initialise the relevent read or write address register fro the type

of intended transaction, before proceeding.

The current access address can be discovered at any time by means of

a read on the DAC Write Data Address Register.  This is necessary to

recover the hardware state at time of entry to an interrupt routine.

See {6.08.04} DAC Write Data Address Register.

The address register in use {either read or write} can also be found

by using a special status register.  This is sometimes necessary for

purposes of determining the hardware state at the time of activation

of an interrupt routine.

See {6.08.02} DAC Status Register.

A discussion of the role of the external colour palette is available

elsewhere.

See {6.07.02} Palette Register 00.

6.09  Register specifications: General Registers

====  ==========================================

6.09.01  Miscellaneous Output Register  { 3c2h  W-- }

=======  ============================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.05.01} CRT Controller Address Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.07.01} Attribute Address Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.09.02} Miscellaneous Output Register.

Cross reference with {6.09.04} Input Status Register One.

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register is subject to write protection through option setting.

The whole register is affected.

See {6.10.07} CRTC Module Testing Register.

This register can be read back using a different input output port

address provided specially for this purpose.

See {6.09.02} Miscellaneous Output Register.

<0>      Input Output Address Select

---------------------------

<=0>  Select the CRT Controller input/output port address to be

hex '3b?', where '?' is some hex digit.

See {6.05.01} CRT Controller Address Register.

Also select the Input Status Register One port address to

hex '3ba'.

See {6.09.04} Input Status Register One.

<=1>  Select the CRT Controller input/output port address to be

hex '3d?', where '?' is some hex digit.

See {6.05.01} CRT Controller Address Register.

Also select the Input Status Register One port address to

hex '3da'.

See {6.09.04} Input Status Register One.

<1>      Display Memory Access Enable

----------------------------

This control can be used to prevent image memory from responding

to processor generated addresses, in circumstances where another

device is (temporarily) enabled for the same address span.

<=0>  Disable processor access to display image memory.

<=1>  Enable processor access to display image memory.

There are controls available to select the address range employed

for processor access to display image memory.

See {6.06.08} Miscellaneous Register.

<3..2>   Dot Clock Frequency Select

--------------------------

Dot clock selection controls horizontal pixel resolution and also

character clock generation.  The clock frequency control modifies

the dot clock period in conjunction with a programmable dot clock

division ratio select.  This resulting dot clock is then used for

character clock definition.  The character clock specifies a scan

line period, which in turn determines the frame period for a full

image display to be produced.  This frame period then decides the

display refresh rate.  The dot clock is a master timing reference.

<=00>  Clock select 25.175 MHz clock for

640 total Horizontal Pixels.

<=01>  Clock select 28.322 MHz clock for

720 total Horizontal Pixels.

<=10>  Clock select external user clock

up to a maximum of 65.000 MHz.

<=11>  Clock select external user clock

up to a maximum of 65.000 MHz.

See {6.04.03} Clocking Mode Register

See {6.05.02} Horizontal Total Register.

See {6.05.08} Vertical Total Register.

See {6.06.07} Graphics Mode Register.

See {6.10.04} Mode Control Register 2.

<4>      Reserved.  Always <=0>.

-----------------------

<5>      Address Extension Bit

---------------------

<=0>  Select the low 64K page.

<=1>  Select the high 64K page.

This bit is the 64K page bit for the Text Memory Access Mode.  For

more details consult Processor Image Memory Access Mode Select.

See {6.04.06} Sequencer Memory Mode Register.

This option controls processor access to display image memory, and

offers a larger number of possible display pages in text mode.  It

is only used in conjunction with text mode displays.

See {6.06.08} Miscellaneous Register.

<6>      Horizontal Synchronisation Signal Polarity

------------------------------------------

This polarity with Vertical Synchronisation Signal Polarity, below,

encodes the vertical scan line count for the display unit.  This is

an aid to the display unit, in helping to select the vertical frame

rate.  See table below.

<=0>  Positive horizontal retrace signal select.

<=1>  Negative horizontal retrace signal select.

<7>      Vertical Synchronisation Signal Polarity

----------------------------------------

This polarity with Horizontal Synchronisation Signal Polarity below

encodes the vertical scan line count for the display unit.  This is

an aid to the display unit, in helping to select the vertical frame

rate.  See table below.

<=0>  Positive vertical retrace signal select.

<=1>  Negative vertical retrace signal select.

Encoding of Vertical Scan Line Count for Display Unit

=====================================================

Vertical Synchronisation Signal Polarity

=---------------------------------------

Horizontal Synchronisation Signal Polarity

=    =-----------------------------------------

Vertical Display Size

=    =    ===----------------------------------------

0    0    reserved, not used at present

0    1    400 scan lines

1    0    350 scan lines

1    1    480 scan lines

-----------------------------------------------------

6.09.02  Miscellaneous Output Register  { 3cch  --R }

=======  ============================================

Cross reference with {6.09.01} Miscellaneous Output Register.

<7..0>   Miscellaneous Output

--------------------

This is a read only register, provided for use with the write

only Miscellaneous Output Register.

The value in this register can be written using the different

input output port address provided for this purpose.

See {6.09.01} Miscellaneous Output Register.

6.09.03  Input Status Register Zero     { 3c2h  R-- }

=======  ============================================

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.09.04} Input Status Register One.

Cross reference with {10.02}   Monitor auto-detection operation.

<3..0>   Reserved.  Always <=0000>.

--------------------------

<4>      Monitor Detect Status Bit

-------------------------

<=0>  There is a high video signal current flowing at this time.  This

is a real-time scan line status, which comes directly from video

DAC output measurement.

<=1>  There is a low video signal current flowing at this time.  This

is a real-time scan line status which comes directly from video

DAC output measurement.

This read only bit provides an indication of the video signal current

flow into a display unit during active video signal generation.  This

provides information to determine which of a monochrome display unit,

or a colour display unit is connected to the image generator.  Use of

this bit is in practise non-trivial, as is described elsewhere.  This

is a facility which must only be used when the original documentation

from the VGA subsystem manufacturer is available.

See {10.02} Monitor auto-detection operation.

<6..5>   Reserved.  Always <=00>.

------------------------

<7>      CRT Controller Interrupt State

------------------------------

<=0>  There is no current vertical retrace interrupt active, which is

because there really is no vertical retrace at present, or this

status is itself permanently inactive through option selection.

See {6.05.19} Vertical Retrace End Register.

<=1>  A vertical retrace interrupt is pending.  This interrupt can be

received by the processor only where 8259 interrupt hardware is

correctly initialised.  This status can always be accessed by a

continuous polling operation by software, providing always that

this status is itself enabled by option selection.

See {6.05.19} Vertical Retrace End Register.

This status bit can be used to synchronise software operation with the

display unit frame production.  In this manner, certain critical types

of operation can be made to occur during frame flyback time.  This has

the effect of preventing any visual disturbance to the display screen,

which is psychologically far better for the user of the display unit.

See {6.09.04} Input Status Register One.

See {10.02}   Monitor auto-detection operation.

6.09.04  Input Status Register One      { 3?ah  R-- }

=======  ============================================

Cross reference with {6.07.01} Attribute Address Register.

Cross reference with {6.07.20} Colour Plane Enable Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.09.03} Input Status Register Zero.

Cross reference with {10.02}   Monitor auto-detection operation.

The input/output port address of this register can be modified under

program control by unsing the Input Output Address Select option bit

in the Miscellaneous Output Register.  The selectable port addresses

are either hex '3ba' or alternatively, hex '3da'.

See {6.09.01} Miscellaneous Output Register.

A read of this register resets the write state associated with the

attribute controller write port 3c0h.  This ensures that the first

write to port 3c0h will select the Attribute Address Register thus

permitting specification of the index for the internal register to

be accessed next.

See {6.07.01} Attribute Address Register.

<0>      Inverted Display Enable Status

------------------------------

<=0>  This represents the active image scan line display time, where

the display is not blanked.  This status is set dynamically in

real-time, so can be used by software to synchronise operation

with display activity.

<=1>  The display is in a blanking period.  This might be either of

horizontal or vertical blanking.  It is possible to decide on

the basis of the vertical interrupt state, or by using timing

information to make the deduction.

See Vertical Retrace State, below.

See {6.09.03} Input Status Register Zero.

<1>      Reserved.  Always <=0>.

-----------------------

<2>      Reserved.  Always <=0>.  {Trident 8900 <=1>.}

-----------------------

<3>      Vertical Retrace State

----------------------

<=0>  The display is in an active state, which might in practise mean

that information is being displayed, or a horizontal retrace is

in progress.  However, there is no vertical retrace active.

<=1>  The display is in a vertical retrace period.  The exact time at

which the display entered this state can be determined by means

of software continuously inspecting this state, until the start

of vertical retrace is detected.  If the software maintains all

operations in approximate synchronism thereafter, a synchronise

becomes possible in a comparatively short time after each frame

is completed.

<5..4>   Video Sampling State

--------------------

<=00>  Sampled digital video data as selected by option.

<=01>  Sampled digital video data as selected by option.

<=10>  Sampled digital video data as selected by option.

<=11>  Sampled digital video data as selected by option.

These two bits have a significance determined by an option setting.

They are provided for diagnotic use, and can be used to sample the

bits sent out from the attribute controller to the external colour

palette.  There are 8 bits sent in total so there must be means to

select those to be sampled.

See {6.07.20} Colour Plane Enable Register.

<7..6>   Reserved.  Always <=00>.

------------------------

6.10  Register specifications: Special Registers

====  ==========================================

Trident 8900 only

====  ==========================================

standard register officially undocumented

====  ==========================================

6.10.01  Hardware Version Register {new definition}  { 3c5h    0bh    R-- }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.05} Mode Control Register 1.

Note that reading this register has a special side effect for Trident

8900 VGA controller chips.

The definitions for the two mode control registers are transformed to

their new definitions.

See {6.10.04} Mode Control Register 2.

See {6.10.05} Mode Control Register 1.

<3..0>   Chip version identification   {Trident}

---------------------------

<=0000>  Unassigned.

<=0001>  Unassigned.

<=0010>  Trident 8800CS

<=0011>  Trident 8900B

<=0100>  Trident 8900C

<=0101>  Unassigned.

<=0110>  Unassigned.

<=0111>  Unassigned.

<=1000>  Unassigned.

<=1001>  Unassigned.

<=1010>  Unassigned.

<=1011>  Unassigned.

<=1100>  Unassigned.

<=1101>  Unassigned.

<=1110>  Unassigned.

<=1111>  Unassigned.

<7..4>   Reserved.

---------

6.10.02  Version Selector Register {old definition}  { 3c5h    0bh    --W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.04} Mode Control Register 2.

Cross reference with {6.10.05} Mode Control Register 1.

Note that writing this register has a special side effect for Trident

8900 VGA controller chips.

The definitions for the two mode control registers are transformed to

their old definitions.  The data written is irrelevant, as it will be

ignored.  It is the write operation which indicates the definition is

to be changed.

See {6.10.04} Mode Control Register 2.

See {6.10.05} Mode Control Register 1.

6.10.03  Configuration Port Register 1               { 3c5h    0ch    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.05} Mode Control Register 1. {Old definition.}

Cross reference with {6.10.05} Mode Control Register 1. {New definition.}

This register is read/write only for Trident 8900.

This register can be write protected by option setting.

See {6.10.05} Mode Control Register 1.  {New definition.}

There are two separate definitions for this register, as

determined by the NMI state option setting.

See {6.10.05} Mode Control Register 1.  {Old definition.}

-----------------------------------------------------------------------------

Definition {only when NMI disabled}

-----------------------------------------------------------------------------

<0>      Address Decode Source

---------------------

<=0>  Employ latched system address for A16 decode.

<=1>  Employ unlatched A19..A17 system address for A16 decode.

<1>      ROM Chip Count Specify

----------------------

<=0>  Two BIOS ROM chips present.

<=1>  One BIOS ROM chip present.

<2>      Reserved.

---------

<3>      ROM Address Span Specify

------------------------

<=0>  Decode 24K ROM BIOS address space.

<=1>  Decode 32K ROM BIOS address space.

<4>      VGA Control Port Specify

------------------------

<=0>  Select hex '46e8' as VGA Control Port.

This is a standard plug-in board control port address.

<=1>  Select hex '3c3' as VGA Control Port.

This is a standard motherboard control port address.

<6..5>   DRAM Configuration

------------------

<=00>  64K  by 4 bits {either 8 or 16 chips}.

<=01>  256K by 4 bits {2 chips}.

<=10>  256K by 4 bits {4 chips}.

<=11>  256K by 4 bits {8 chips}.

<7>      DRAM Access Width

-----------------

<=0>  8 bit memory access.

<=1>  16 bit memory access.

-----------------------------------------------------------------------------

-----------------------------------------------------------------------------

Definition {only when NMI enabled}

-----------------------------------------------------------------------------

<0>      Software Status

---------------

Read and write to this bit by software to store a binary status flag.

<1>      NMI State {Non Maskable Interrupt State}

----------------------------------------

This state has influence over the interpretation of this register.

This interpretation only exists when the NMI state is set <=1>.

Therefore the NMI state can only be turned off here.

<=0>  NMI Disable.

Remove this interpretation of the register.

<=1>  NMI Enable.

Consistent with this register interpretation.

<3..2>   Testing Bits

------------

<=00>  Manufacturing test.

<=01>  Manufacturing test.

<=10>  Manufacturing test.

<=11>  Normal operation.

<7..4>   NMI Vector Enable

-----------------

<=0000>  No NMI vector specified.

<=0001>  NMI vector is specifed as hex '3x8'.

<=0010>  NMI vector is specifed as hex '3d9'.

<=0011>  Illegal value.

<=0100>  NMI vector is specifed as hex '3dd'.

<=0101>  Illegal value.

<=0110>  Illegal value.

<=0111>  Illegal value.

<=1000>  NMI vector is specifed as hex '3bf'.

<=1001>  Illegal value.

<=1010>  Illegal value.

<=1011>  Illegal value.

<=1100>  Illegal value.

<=1101>  Illegal value.

<=1110>  Illegal value.

<=1111>  Illegal value.

-----------------------------------------------------------------------------

6.10.04  Mode Control Register 2                     { 3c5h    0dh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.04.03} Clocking Mode Register.

Cross reference with {6.04.05} Character Map Select Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.16} Cursor Location High Register.

Cross reference with {6.05.17} Cursor Location Low Register.

Cross reference with {6.05.22} Underline Location Register.

Cross reference with {6.06.07} Graphics Mode Register.

Cross reference with {6.07.18} Attribute Mode Control Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.01} Hardware Version Register {new definition}.

Cross reference with {6.10.02} Version Selector Register {old definition}.

Cross reference with {6.10.07} CRTC Module Testing Register.

This register has two different interpretations depending upon the

last transaction to either of the following registers:

See {6.10.01} Hardware Version Register {new definition}.

See {6.10.02} Version Selector Register {old definition}.

-----------------------------------------------------------------------------

New Definition

-----------------------------------------------------------------------------

See {6.10.01} Hardware Version Register {new definition}.

<0>      Dot Clock Extended Select

-------------------------

This bit extends the clock frequency select bits already available

elsewhere from 2 bits to 3 bits.  This bit is the high order bit.

See {6.09.01} Miscellaneous Output Register.

<=0>  Select normal clocks.

<=1>  Select extended clocks.

<2..1>   Dot Clock Division Ratio Select

-------------------------------

The dot clock frequency can be selectively divided to provide other

useful frequencies without need for additional oscillators.

<=00>  No division.

<=01>  Division ratio is 2.0

<=10>  Division ratio is 4.0

<=11>  Division ratio is 1.5

See {6.04.03} Clocking Mode Register.

<3>      Software Status

---------------

Read and write to this bit by software to store a binary status flag.

<5..4>   Software Status

---------------

Read and write to these bits by software to store a pair of binary

status flags.

<7..6>   General Purpose Outputs

-----------------------

No documented function.  There are two VGA controller chip outputs

which can be used to control aspects of a subsystem as the designer

wishes.  These bits control the logic levels on these two hardware

lines.  Their use is therefore totally design specific.  These are

Trident 8900 outputs only.

-----------------------------------------------------------------------------

-----------------------------------------------------------------------------

Old Definition

-----------------------------------------------------------------------------

See {6.10.02} Version Selector Register {old definition}.

<2..0>   Video Generation Mode Select

----------------------------

<=000>  VGA mode {with analogue monitor} select.

<=001>  Not used.

<=010>  EGA mode {with analogue monitor} select.

<=011>  EGA mode {with TTL monitor} select.

<=100>  CGA, MDA, Hercules modes {with analogue monitor} select.

<=101>  CGA, MDA, Hercules modes {with TTL monitor} select.

<=110>  Not used.

<=111>  Not used.

These options can alter the significance of an option bit in

another register.  {Multipurpose Select bit.}

See {6.10.07} CRTC Module Testing Register.

<3>      Processor Bandwidth during Blanking

-----------------------------------

<=0>  Normal access.

<=1>  Provide full bandwidth during blanking.

<4>      Memory Plane Architecture Select

--------------------------------

<=0>  Planes 0 .. 3 active.

<=1>  Planes 0 .. 7 active.

<5>      DRAM Clock Control

<=0>  Enable  DRAM clock.

<=1>  Inhibit DRAM clock.

<7..6>   Character Set Select High Extend

--------------------------------

These bits are used to select the character set base address in the

display image memory plane 2.  hey provide two high order bits such

that 4 times the number of character fonts can be selected over the

basic VGA architecture.

See {6.04.05} Character Map Select Register.

-----------------------------------------------------------------------------

6.10.05  Mode Control Register 1                     { 3c5h    0eh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.04.06} Sequencer Memory Mode Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.06.08} Miscellaneous Register.

Cross reference with {6.10.01} Hardware Version Register {new definition}.

Cross reference with {6.10.02} Version Selector Register {old definition}.

Cross reference with {6.10.03} Configuration Port Register 1.

Cross reference with {6.10.06} Power-up Mode Register 2.

Cross reference with {6.10.13} Source Address Register.

Cross reference with {6.10.14} Source Address Enable Register.

This register has two different interpretations depending upon the

last transaction to either of the following registers:

See {6.10.01} Hardware Version Register {new definition}.

See {6.10.02} Version Selector Register {old definition}.

The interpretations and layout of the Source Address Register match

exactly those for this register, provided the facility is enabled.

See {6.10.13} Source Address Register.

See {6.10.14} Source Address Enable Register.

-----------------------------------------------------------------------------

New Definition

-----------------------------------------------------------------------------

See {6.10.01} Hardware Version Register {new definition}.

This new definition is suitable for high resolution graphics use.

To use the features correctly, the processor display image memory

must be set to hex 'a0000' for 64K bytes length.

See {6.06.08} Miscellaneous Register.

<0>      Segment Selector

----------------

<=0>  Select lower 64K of 128K page.

<=1>  Select upper 64K of 128K page.

<1>      Page Selector

-------------

Caution!  These bits are WRITTEN with opposite polarity to that which

is actually required.

READ SENSE !

<=0>  Selected bank is LOWER 128K of 256K bank.

<=1>  Selected bank is upper 128K of 256K bank.

WRITE SENSE !

<=0>  Select UPPER 128K of 256K bank.

<=1>  Select lower 128K of 256K bank.

<3..2>   Bank Selector

-------------

<=00>  Select 256K bank 0.  {Power on default, lowest memory bank}

<=01>  Select 256K bank 1.

<=10>  Select 256K bank 2.

<=11>  Select 256K bank 3.  {Highest memory bank}

<6..4>   Software Status

---------------

Read and write to these bits by software to store a triple of binary

status flags.

These bits can be made subject to write protection by the option

Write Protection State, immediately below.

<7>      Write Protection State

----------------------

This state controls write protection for Software Status, immediately

above, in addition to some other registers.

See {6.10.03} Configuration Port Register 1.

See {6.10.06} Power-up Mode Register 2.

<=0>  Write Protect.  {Default}

<=1>  Write Permit.

-----------------------------------------------------------------------------

-----------------------------------------------------------------------------

Old Definition

-----------------------------------------------------------------------------

See {6.10.02} Version Selector Register {old definition}.

<0>      Display Access Bank Selector

----------------------------

This bit controls addresses for display generation only,

not for display image memory processor access.

<=0>  Select lower 512K bank.  {Default}

<=1>  Select upper 512K bank.

<2..1>   Processor Access Bank Selector

------------------------------

This bit controls addresses for processor access only,

not for display generation.

<=00>  Select 256K bank 0.  {Power on default, lowest memory bank}

<=01>  Select 256K bank 1.

<=10>  Select 256K bank 2.

<=11>  Select 256K bank 3.  {Highest memory bank}

<3>      DRAM Access Width

-----------------

<=0>  8 bit memory access.

<=1>  16 bit memory access.

<4>      Reserved for Dot Clock Extended Select

--------------------------------------

Not implemented at present {Trident 8900}.

<5>      NMI State {Non Maskable Interrupt State}

----------------------------------------

This state has influence over the interpretation of other registers.

See {6.10.03} Configuration Port Register 1.

<=0>  NMI Enable.

<=1>  NMI Disable.

<6>      Interrupt Request Polarity Select

---------------------------------

<=0>  Positive logic polarity.

<=1>  Negative logic polarity.

<7>      Clock Pins Function Select

--------------------------

There are two clock pins available for the Trident 8900, which can be

used either to select an external oscillator, or to accept oscillator

outputs.  They are denoted CLK2, CLK3.

When used to select an external oscillator these pins output from the

VGA controller to the external environment.

When accepting external oscillator generated clock signals, the pins

act as input pins.

<=0>  Select INPUT operation for CLK2, CLK3 pins.

<=1>  Select OUTPUT operation for CLK2, CLK3 pins.

-----------------------------------------------------------------------------

6.10.06  Power-up Mode Register 2                    { 3c5h    0fh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.05} Mode Control Register 1. {New Definition}

<3..0>   DIP Switch Image

----------------

These bits are a copy of the configuration DIP switch settings.

Note the inverted polarity!

<=0>  DIP switch is ON.

<=1>  DIP switch is OFF.

<4>      Bus Protocol Select

-------------------

<=0>  Micro channel bus protocol is selected.

<=1>  ISA PC/AT bus protocol is selected.

<5>      Input Output Address Map Select

-------------------------------

<=0>  Select alternate input output address map at hex '2xx'.

<=1>  Select ISA input output address map at hex '3xx'.

<6>      ROM BIOS Address Decode Enable

------------------------------

<=0>  ROM address decode is disabled.

<=1>  ROM address decode is enabled.

<7>      ROM BIOS Data Path Width

------------------------

<=0>  8 bit wide BIOS ROM.

<=1>  16 bit BIOS ROM.

6.10.07  CRTC Module Testing Register                { 3?5h    1eh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.05.01} CRT Controller Address Register.

Cross reference with {6.05.02} Horizontal Total Register.

Cross reference with {6.05.03} Horizontal Display Enable End Register.

Cross reference with {6.05.04} Start Horizontal Blanking Register.

Cross reference with {6.05.05} End Horizontal Blanking Register.

Cross reference with {6.05.06} Start Horizontal Retrace Pulse Register.

Cross reference with {6.05.07} End Horizontal Retrace Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.09} CRT Controller Overflow Register.

Cross reference with {6.05.10} Preset Row Scan Register.

Cross reference with {6.05.11} Maximum Scan Line Register.

Cross reference with {6.05.14} Start Address High Register.

Cross reference with {6.05.15} Start Address Low Register.

Cross reference with {6.05.18} Vertical Retrace Start Register.

Cross reference with {6.05.19} Vertical Retrace End Register.

Cross reference with {6.05.23} Start Vertical Blanking Register.

Cross reference with {6.05.24} End Vertical Blanking Register.

Cross reference with {6.05.25} CRTC Mode Control Register.

Cross reference with {6.09.01} Miscellaneous Output Register.

Cross reference with {6.10.04} Mode Control Register 2. {Old Definition}

<0>      Horizontal Clock Select

-----------------------

<=0>  Normal operation.  {Power on default}

<=1>  Manufacturing test.

<1>      Character Clock Select

----------------------

<=0>  Normal operation.  {Power on default}

<=1>  Manufacturing test.

<2>      Interlace Enable

----------------

<=0>  Non-interlace display generation mode.  {Power on default}

<=1>  Interlace display generation mode.

<3>      Multipurpose Select

-------------------

--------------------------------------------------------------------

Vertical Retrace Clock Edge Select

==================================

This operates only in CGA, MDA modes.

See {6.10.04} Mode Control Register 2. {Old Definition}

<=0>  Vertical retrace on rising clock edge.  {Power on default}

<=1>  Vertical retrace on falling clock edge.

--------------------------------------------------------------------

Font Load Address Select

========================

This operates only in EGA, VGA modes.

See {6.10.04} Mode Control Register 2. {Old Definition}

<=0>  Load from low address.

<=1>  Load from high address.

--------------------------------------------------------------------

<4>      CRT Controller Register Protection State

----------------------------------------

<=0>  No protection.  {Power on default}

<=1>  Protection applied.

The following register indexes are protected:

Register Index

===-----------

Bit Numbers

===    ===--------

Register Description and Bit Description

===    ===    ========================================----------------

00h    0-7    {6.05.02} Horizontal Total Register.

01h    0-7    {6.05.03} Horizontal Display Enable End Register.

02h    0-7    {6.05.04} Start Horizontal Blanking Register.

03h    0-7    {6.05.05} End Horizontal Blanking Register.

04h    0-7    {6.05.06} Start Horizontal Retrace Pulse Register.

05h    0-7    {6.05.07} End Horizontal Retrace Register.

06h    0-7    {6.05.08} Vertical Total Register.

07h    0-7    {6.05.09} CRT Controller Overflow Register.

09h     7     {6.05.11} Maximum Scan Line Register.

bit --> Double Scan Line Display.

10h    0-7    {6.05.18} Vertical Retrace Start Register.

11h    0-7    {6.05.19} Vertical Retrace End Register.

15h    0-7    {6.05.23} Start Vertical Blanking Register.

16h    0-7    {6.05.24} End Vertical Blanking Register.

17h     2     {6.05.25} CRTC Mode Control Register.

bit --> Horizontal Retrace Divisor.

----------------------------------------------------------------------

<5>      Start Address Extension Bit 16

------------------------------

This feature must be enabled.  See Start Address Extension Enable,

almost immediately below.

See {6.05.14} Start Address High Register.

See {6.05.15} Start Address Low Register.

<6>      Miscellaneous Output Register Protect State

-------------------------------------------

<=0>  No protection.  {Power on default}

<=1>  Protection applied.

See {6.09.01} Miscellaneous Output Register.

<7>      Start Address Extension Enable

------------------------------

This feature is not an ISA feature, so must be specially enabled by

means of this option setting, before use.

<=0>  Disable start address extension bit 16.

<=1>  Enable start address extension bit 16.

6.10.08  Software Programming Register               { 3?5h    1fh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.05.01} CRT Controller Address Register.

This register provides means for storage and retrieval of software

status associated, typically, with BIOS operation.

<1..0>   Reserved for BIOS use.

----------------------

<=00>  Memory size is 256K bytes.

<=01>  Memory size is 512K bytes.

<=10>  Memory size is 768K bytes.

<=11>  Memory size is 1024K bytes.

<3..2>   Software Status Bits

--------------------

Used for software defined purposes.

<7..4>   Reserved for BIOS use.  Do NOT disturb.

---------------------------------------

6.10.09  CPU Latch Read Back Register                { 3?5h    22h    R-- }

=======  ==================================================================

standard register officially undocumented

=======  ==================================================================

Cross reference with {6.06.02} Set/Reset Register.

Cross reference with {6.06.06} Read Map Select Register.

Cross reference with {6.06.07} Graphics Mode Register.

This is a standard VGA controller register which was never

officially documented.

<7..0>   CPU Latch Read Back

-------------------

This register provides access to the four data latches used in the

data path between the processor and the display image memory.  The

latch number directly associates with the image memory plane which

receives data from the latch during memory writes by the processor

so this plane is specified by the Read Map Select Register.

See {6.06.02} Set/Reset Register.

See {6.06.06} Read Map Select Register.

See {6.06.07} Graphics Mode Register.

6.10.10  Attribute State Read Back Register          { 3?5h    24h    R-- }

=======  ==================================================================

standard register officially undocumented

=======  ==================================================================

Cross reference with {6.07.01} Attribute Address Register.

Cross reference with {6.10.11} Attribute Index Read Back Register.

This is a standard VGA controller register which was never

officially documented.

-------------------------------------------------------------------------------

standard register officially undocumented

-------------------------------------------------------------------------------

<4..0>   Attribute Controller Index

--------------------------

This provides the index value which selects an attribute controller

register so that the processor can obtain access.  This value is

defined by writing to the attribute controller address register and

the appropriate internal associated state must be defined.

See {6.07.01} Attribute Address Register.

<5>      Colour Palette Address Source {Internal palette}

-----------------------------

<=0>  Colour palette is accessed only by the processor, no video can

be generated.

<=1>  Colour palette generates video, processor access is denied.

<6>      Reserved.

---------

<7>      Attribute State Read Back

-------------------------

<=0>  This specifies that the Attribute controller is set to point to

the attribute controller address register.  This permits a new

attribute controller register index value to be specified.

<=1>  This specifies that the Attribute controller is set to point to

one particular internal attribute controller register.  This is

specified by the index value in the address register, and it is

this indexed register which will be accessed by the processor.

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Trident 8900 only

-------------------------------------------------------------------------------

See {6.10.11} Attribute Index Read Back Register.

<6..0>   Reserved.  Undefined.

---------------------

<7>      Attribute State Read Back

-------------------------

<=0>  This specifies that the Attribute controller is set to point to

the attribute controller address register.  This permits a new

attribute controller register index value to be specified.

<=1>  This specifies that the Attribute controller is set to point to

one particular internal attribute controller register.  This is

specified by the index value in the address register, and it is

this indexed register which will be accessed by the processor.

-------------------------------------------------------------------------------

6.10.11  Attribute Index Read Back Register          { 3?5h    26h    R-- }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.07.01} Attribute Address Register.

Cross reference with {6.10.10} Attribute State Read Back Register.

<4..0>   Attribute Index

---------------

This binary field indicates the index value which determines which of

the internal registers will be accessed by the processor assuming the

correct required internal state is present.

See {6.07.01} Attribute Address Register.

An alternative ISA register may be available on non-Trident hardware,

which may provide a slightly more standard way to obtain this data.

See {6.10.10} Attribute State Read Back Register.

<7..5>   Reserved.

---------

6.10.12  Clear Vertical Display Enable Register      { 3?5h    3nh    W-- }

=======  ==================================================================

standard register officially undocumented

=======  ==================================================================

Note: the index address 'n' indicates any value, that is, a don't care value.

Cross reference with {6.05.01} CRT Controller Address Register.

Cross reference with {6.05.08} Vertical Total Register.

Cross reference with {6.05.20} Vertical Display Enable End Register.

This is a standard VGA controller register which was never

officially documented.

<0>      Clear Vertical Display Enable

-----------------------------

<=0>  Do not clear the vertical display enable state.

See {6.05.08} Vertical Total Register.

See {6.05.20} Vertical Display Enable End Register.

<=1>  Clear the vertical display enable state.  This has the effect

of immediately blanking the screen and causes the generation

of display data to terminate.  The sequencer then changes the

image memory timing so permitting greater bandwidth access by

the processor.  The display will be subject to disturbance if

appropriate synchronisation is not provided.  In any case the

use of this facility will produce some flicker.

The character row counter achievement of the vertical total in

the usual manner will set the vertical display enable state so

permitting normal production of the following display frame.

See {6.05.08} Vertical Total Register.

See {6.05.20} Vertical Display Enable End Register.

This might be used to temporarily increase the processor speed

of image update at the expense of display disturbance.

<7..1>   Unused.  Any value can be present.

----------------------------------

6.10.13  Source Address Register                     { 3cfh    0eh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.05} Mode Control Register 1.

Cross reference with {6.10.14} Source Address Enable Register.

There is a difficulty of low speed when manipulating large graphics

images across memory bank boundaries.  This arises when an image is

located in a number of display memory banks, only one of which will

be accessible to the processor at any given moment.

In this situation, it is often required to copy a part of the image

from one bank into another.  This entails performing a read, then a

change of processor mapping into the display memory following which

a corresponding processor write can take place.

Needless to say, this whole operation is cumbersome in the extreme.

For this reason, it is possible to specify a source and destination

bank selection invoked by processor read write access respectively.

In this way any processor read is directed to the source address in

the source bank while a write access is directed to the destination

bank in similar manner.

This Source Address Register specifies the details of the source

address, while the Mode Control Register 1 specifies all details

for the destination address.

See {6.10.05} Mode Control Register 1.

This facility must be enabled appropriately before use, since it is

not a standard VGA controller capability.

See {6.10.14} Source Address Enable Register.

6.10.14  Source Address Enable Register              { 3cfh    0fh    R/W }

=======  ==================================================================

Trident 8900 only

=======  ==================================================================

Cross reference with {6.10.05} Mode Control Register 1.

Cross reference with {6.10.13} Source Address Register.

This register controls the apparent existance of a very useful feature

which can greatly speed up graphics programming image manipulations if

correctly used.

See {6.10.05} Mode Control Register 1.

See {6.10.13} Source Address Register.

<0>      Source Address Enable

---------------------

<=0>  Disable the Source Address Register.

<=1>  Enable the  Source Address Register.

<7..1>   Reserved.  Always <=0000000>.

-----------------------------

6.s   Summary

====  =======

Details have been given of some special registers which provide for the most

part, proprietary features concerned with VGA architecture extension.  These

proprietary details relate in this case to the Trident 8900 chip.

There are however, certain features described here which are standard in that

the ISA contains these features, yet they remain officially undocumented.  It

has only been possible to decide on their existance, by means of a program of

reverse engineering.  This analyses a semiconductor chip in order to obtain a

complete logic schematic from the circuit physical structure.

Using these undocumented yet standard features may cause difficulty because

companies which design their compatible products on the basis of documented

interfaces may not have performed the reverse engineering necessary to make

a fully hardware compatible product.

The simple fact is with a device as complex as the VGA controller, it is very

important to obtain original technical material before starting any serious

programming work.  Most importantly, before purchase of a VGA controller, is

the possibility for review of such technical material as may be available in

order to assess it for quality.

7.    VGA controller: architecture

====  ============================

7.01  Controller timing generator:  { Sequencer }

====  ===========================================

The video graphics array controller is a complex collection of distinct logic

blocks each of which has various connections to other such blocks.  Since the

notion of time used by each logic block must agree with that used by associated

blocks a special set of timing, that is, clock pulses is distributed throughout

the device.  Each clock pulse keeps all the logic which uses it in common step,

so that all the events which must be maintained in lock-step remain so at all

times.  This is the purpose of a clock pulse.  It regulates the changing flow

of events within a logic circuit.

There will be more than one clock pulse needed by such a complex device as the

VGA controller chip, so it becomes important for each clock pulse to maintain

a particular relationship with all other clock pulses.  This is so as to permit

the reliable and timely flow of information between circuitry which determines

events using distinct time scales.  These time scales are naturally represented

by the different periods associated with each of the separate clock pulses.  As

an example of just two distinct time scales we will be interested in using, we

can think of the horizontal and vertical synchronisation pulses that the image

generator regularly sends to a display unit.  In this way, we can see that the

synchronisation signals are really just special clock pulses sent by the image

generator to the display unit in order to keep the display unit in lock-step,

therefore ensuring the correct image is displayed at all times.

There are essentially two distinct image generation modes available.  Text mode

creates a text image on the display, possibly using a variety of character sets

whilst graphics mode dispenses with characters, and presents purely graphical

images.  These two modes are handled quite differently within the controller.

Note that it is still possible to present text for display while using graphics

modes, but in this case, software will have to prepare the character outlines

ready for display.  The distinction between graphics mode presentation of text

and text mode presentation of text is simply that in the latter, hardware takes

care of all the character formation chores, which results in much faster output

of text than would be possible using software in graphics mode.  Of course, it

is only possible to create text in text mode, whereas text and graphics can be

created in graphics mode.

The sequencer generates a number of timing signals, that is, clock pulses.  We

list the more interesting ones below:-

1.  Dot_clock

2.  Character_clock

The dot clock is the fundamental unit of timing from which all other timing is

derived for image generation.  This clock defines the amount of time available

for displaying one pixel.  It therefore determines the image information output

frequency from the image generator to the display unit.  Each time this clock

marks a new pixel period, the image generator must transmit all information for

defining the next pixel to the display.  {There is one exception to this which

distinguishes 256 colour packed pixel mode from other graphics display modes.}

Facilities are provided to change the oscillator associated with the dot clock

which has the effect of selecting a different dot clock frequency.  Also it is

possible to invoke a frequency division on the selected oscillator frequency,

in order to produce new frequencies without using more oscillator units.

The character clock is derived from the dot clock, and one period of this clock

represents the width of a single character output to the display unit.  This is

only relevant when the image generator is operating in text mode. The character

clock can either represent characters of width 8 pixels width, or alternatively

of 9 pixels width.

The character clock represents the unit of timing for display unit purposes and

this is true in either of text display mode or graphics display mode.

7.02  Display timing generator:  { CRT Controller }

====  =============================================

The Cathode Ray Tube Controller accepts the Character_clock from the Sequencer

and generates the synchronisation signals required to keep the display unit in

lock-step with the image generator.  All the timing for horizontal and vertical

synchronisation is defined in terms of character widths.  For this reason, the

CRT Controller is also responsible for generating the hardware cursor which can

only appear in character aligned positions within the display image.  Some of

the signals developed by the CRT controller are:

1.  Horizontal Synchronisation

2.  Horizontal Blanking

3.  Vertical Synchronisation

4.  Vertical Blanking

5.  Display Enable

6.  Cursor Enable

7.  Underline Enable

8.  Increment Horizontal Character Counter

9.  Reset Horizontal Character Counter

10.  Increment Character Row Counter

11.  Reset Character Row Counter

12.  Increment Character Font Scan Line Counter

13.  Reset Character Font Scan Line Counter

14.  Increment Display Memory Address Counter

15.  Reset Display Memory Address Counter

The effect of these signals is explored fully, in the main body of the text.

These control the intimate details of all information required from display

image memory in text or graphics mode display generation.  A display address

is generated by the CRT Controller which then provides the display image data

to the graphics controller, and thence to the attribute controller.

7.03  Image information processor:  { Graphics Controller }

====  =====================================================

The Graphics Controller acts as an interface between the processor and display

image memory, and also between the CRT Controller and display image memory.  A

simultaneous access attempt by both the processor and the CRT Controller gives

the access to the CRT Controller in all cases.

The Graphics Controller provides the processor with extensive image processing

ability for various graphics display modes which are available.  Facilities to

efficiently process multiplane image data are provided, with very low overhead.

The Graphics Controller provides pixel panning capability for generated image

data, which does not depend upon text mode or graphics mode operation.  Shift

registers provide conversion of multiplane display image memory format to the

serial multiplane parallel format required by the attribute controller.  They

have special functionality built in so that a wide range of these conversions

can be easily handled.  Shift register interconnections and architecture will

change depending upon the graphics mode in use.

7.04  Image presentation processor:  { Attribute Controller }

====  =======================================================

The Attribute Controller accepts image data from the Graphics Controller and

processes it before sending it out to the digital to analogue converters for

final display.  This involves performing colour selection code translation,

so that less image memory can be used for storage of images which use small

sets of colours.  The translation uses an internal colour palette register

which accepts a 4 bit colour code and produces either a 4 or 6 bit output.

Bit shift pixel panning is implemented, to be used in conjuction with those

complementary facilities provided by the Graphics Controller.  Selection of

display colour during blanking periods is also performed.

In text mode display generation, the attribute byte which is associated with

every text character is processed into the video data, to provide a blinking

cursor, underlining of text, and colour selection functions.

The output from the Attribute Controller is a stream of 8 bit wide image data

which specifies a pixel colour, suitable for input to the DAC Support Logic.

7.05  Colour presentation processor:  { DAC Support Logic }

====  =====================================================

The Digital to Analogue Converter support logic accepts 8 bit colour data, and

translates it into either 18 bit or 24 bit colour data, to drive three primary

colour electron guns in the display unit.  The translation is performed with a

large look-up table memory unit, termed the external colour palette.  This can

control the displayed colour set with a great degree of finess.  There is also

the possibility provided to edit some of the 8 bits of input data substituting

zero bits for data bits on option.  This provides for animation effects or for

multiple image storage with very fast recall.  This can be very useful for the

support of hierarchical design systems.

7.s   Summary

====  =======

A very brief sketch has been given for each of the major logic blocks involved

in producing a display image from data supplied by a processor.  The functions

described are explored in considerable detail in the main body of the text.

8.    VGA controller: Text

====  ====================

8.01  Text image generation

8.02  Text font representation

8.03  Text attribute representation

8.04  Text character set extensions

8.05  Text mode support for graphics

Chapter in preparation.  Should be available for second edition.

----------------------------------------------------------------

9.    VGA controller: Graphics

====  ========================

9.01  Graphic image generation

9.02  Graphic multi-plane memory

9.03  Graphic packed pixel memory

9.04  Graphic line drawing

9.05  Graphic object drawing

Chapter in preparation.  Should be available for second edition.

----------------------------------------------------------------

10.    VGA controller: Hardware Interface

===    ==================================

10.01  Hardware connector layout

=====  =========================

This information is provided for completeness only.  It is intended only as a

guide.  Further information should be sought if either serious design work or

diagnostic work needs to be performed.

The VGA subsystem pulls the Identity signal lines to +5V through a resistor.

The display then selectively connects these lines to ground.  The VGA then

detects the signal level and interprets the code in the appropriate manner.

Details are available on the coding scheme employed.

See {10.03} Monitor auto-configuration operation.

Video Connector Pinouts

=======================

Pin Number  { 9 pin connector }

==-----------------------------

Pin Number  { standard 15 pin connector }

==    ==---------------------------------------

Signal Description

==    ==    ==================---------------------------------------

1     1    Red intensity             {analogue 75 ohms}

2     2    Green intensity           {analogue 75 ohms}

3     3    Blue intensity            {analogue 75 ohms}

-     4    Identity bit 2            {logic pullup to +5V by VGA}

-     5    Self test / must be grounded for use with 9 pin connector

6     6    Red intensity ground      {analogue 75 ohms}

7     7    Green intensity ground    {analogue 75 ohms}

8     8    Blue intensity ground     {analogue 75 ohms}

-     9    -  no connection

9    10    Synch ground

-    11    Identity bit 0            {logic pullup to +5V by VGA}

-    12    Identity bit 1            {logic pullup to +5V by VGA}

4    13    Horizontal synch

5    14    Vertical synch

-    15    -  no connection

---------------------------------------------------------------------

10.02  Monitor auto-detection operation

=====  ================================

A VGA subsystem can be arranged to detect whether a monochrome display unit or

alternatively, a colour display unit is in use.  This is made possible through

sensing the current flow in the lines carrying the analogue red green and blue

pixel intensity signals from the image generator to the display unit.

A colour display unit will make use of each one of these three signals, which

can be detected by suitable hardware measurement of current flow.  However, a

monochrome display unit will only use the green signal.  This difference will

cause the video subsystem to select an operational mode suitable for the unit

in use.

A simplified schematic is given below showing the arrangement provided for the

measurement of output current.  Note that the software must read the status at

the active video image generation time within a scan line.  This is decided by

means of the display enable status bit.  This bit should be monitored so as to

achieve synchonism with the program.  Then, the measurement can be confidently

made during the active display period of the horizontal scan.  This involves a

synchronisation with the vertical retrace so that the complete activity of the

display can be accurately predicted in advance of the measurement.

See {6.09.04} Input Status Register One.

The software interface for this hardware detection operation is provided by a

single status bit.

See {6.09.03} Input Status Register Zero.

Schematic of Video Output Current Measurement

=============================================

---------

| DAC   |                   Red into

| RED   |...........r....>  Display

---------           |       Unit

|

|

|

---------           |

| DAC   |           |       Green into

| GREEN |........g.......>  Display

---------        |  |       Unit

|  |

|  |

|  |

---------        |  |

| DAC   |        |  |       Blue into

| BLUE  |.....b..........>  Display

---------     |  |  |       Unit

|  |  |

|  |  |

|  |  |

|  |  |

|  |  |      |-----*

---------     |  |  r....>.| +      *

| VOLT  |     |  |         | compare   *............

| REFER-|..v.............>.| -      *              |

| ENCE  |  |  |  |         |-----*                 |

---------  |  |  |                                 |

|  |  |                                 |

|  |  |                                 |

|  |  |                                 |

|  |  |         |-----*                 |

|  |  g.......>.| +      *              |

|  |            | compare   *.........  |

v.............>.| -      *           |  |

|  |            |-----*              |  |

|  |                                 |  |

|  |                                 |  |

|  |                                 |  |

|  |                                 |  |

|  |            |-----*              |  |

|  b..........>.| +      *           |  |

|               | compare   *......  |  |

v.............>.| -      *        |  |  |

|-----*           |  |  |

|  |  |

|  |  |

|  |  |

|  |  |

=============

NOR GATE

=============

|

|

|

|

|

=================

STATUS BIT OUTPUT

=================

---------------------------------------------------------

10.03  Monitor auto-configuration operation

=====  ====================================

There is another means for determining the type of display unit attached to a

VGA subsystem.  This depends on a simple code provided by the display unit to

the VGA subsystem, according to a predefined convention.

Identity bit 2

=-------------

Identity bit 1

=    =-------------

Identity bit 0

=    =    =-------------

Significance

=    =    =    ============--------------------------------------

0    0    0    --  unassigned

0    0    1    --  unassigned

0    1    0    Interlaced colour display unit is connected

0    1    1    --  unassigned

1    0    0    --  unassigned

1    0    1    Monochrome display unit is connected

1    1    0    Non-interlaced colour display unit is connected

1    1    1    No display unit connected to VGA subsystem

-----------------------------------------------------------------

Note that strapping an identity signal line to ground has the effect of sending

a binary zero to the VGA subsystem sensing logic.

See {10.01} Hardware connector layout.

10.s   Summary

====   =======

A brief description of some of the hardware connections between a VGA subsystem

and a corresponding display unit has been given.

References  {in alphabetic order by company name}

===============================================================================

001    82C456 Enhanced Flat Panel/CRT VGA Controller Data Sheet

Chips and Technologies Inc.

002    HT216 VGA Controller

Headland Technology Inc.

003    iAPX 86/88, 186/188 User's Manual, Programmer's Reference

Intel Corporation

004    TVGA8900C Technical Reference Manual

Trident Microsystems

005    Computer IC's 1990-1991 Data Book

United Microelectronics Corporation

006    WD90C20/WD90C22 VGA Flat Panel Display Controllers

Western Digital Corporation

007    Many other documents were consulted, being too numerous to mention.

These generally clarified only a single point, which in many cases

could only be understood in combination with many other documents.

It is felt that mentioning all of these could only add to the very

great confusion surrounding this whole area of subject matter.  In

some cases documents were found to be self contradictory.  Some of

the documents were found to be factually correct, but no framework

was provided for practical use and understanding of the material.

Very often, there was no cross reference between different aspects

of the VGA operation.  This presents what is felt by the author to

be a difficult situation.  It is hoped that this work will address

the worst of these problems, and provide a useful framework within

which real knowledge can be gained.

At the same time, the reader should appreciate that there are many

different proprietary products in the marketplace with each giving

proprietary extensions to the basic architecture.  It is vital for

all serious work, that the user obtain all official technical data

direct from the manufacturer concerned.  That is, unless only BIOS

access is to be gained to the VGA facilities provided.  However it

would be prudent even in this case to obtain technical data, since

every BIOS is different, by definition.  The best way to avoid any

implementation problems is to remain as well informed as possible.

Glossary

===============================================================================

Please note that this glossary is vestigial in the extreme with the motivation

for including it at all being completeness.  However, it is unlikely that this

paucity will impact significantly on the useability of this text, since jargon

is relatively scarcely used.  In those places where jargon is introduced there

is normally an explanation provided, except for those terms which are so basic

that a reader of this document must be expected to have knowledge of them.

80x86

-------------

A family of related processor chips, produced by Intel Corp.  This

processor family is important because it is used as the basis for

all industry standard architecture computer systems.

Analogue

-------------

A coding and interpretation scheme which does not rely on discrete

steps.  Instead, continuous variation of some characteristic such

as voltage or current is used as the analogue of a represented

value.  Hence analogue, as opposed to digital, representation.  A

significant practical advantage of this representation is the lack

of need for more than two wire connections, which can save expense

for suitable systems.  A display monitor might accept analogue or

digital signals, but will be able to support many more colours if

an analogue video signal is used.  Contrast this with digital.

BIOS

-------------

Basic Input Output System.  This is a program which provides a set

of services (usually standardised) to other programs.  VGA systems

usually have their own BIOS which permits the VGA to support those

interfaces usually provided by an industry compatible architecture

computer system to the software running on it.

Blanking

-------------

The action of display unit operation inhibition, during the time

required to reinitialise the display ready for production of the

next part of the image.  This is needed because an image is made

up from a number of scan lines which are displayed left to right

on the screen.  To begin again at the left side the display must

be inhibited, to prevent the retrace from disturbing the image.

In practise, there are horizontal and vertical blanking periods,

which perform a similar function in orthogonal directions.

Bus

-------------

A means for transmitting and receiving information at high speed

within the confines of a single computer system.  Bus types that

are used with VGA controllers fall into two categories.  Either

the ISA PC/AT bus is used, or sometimes, Micro channel bus.  The

latter is proprietary to IBM and is not so widely used by other

manufacturers.

CGA

-------------

Colour Graphics Array.  A rather old, but still widely used video

graphics standard which is principally of interest for emulation.

Chip

-------------

A highly integrated electronic device which is implemented as a

silicon integrated circuit.  This makes possible the practical

realisation of very complex circuits, such as processor chips

and VGA controller chips.

Clock

-------------

Almost all electronic systems operate by means of a regular signal

which regulates in a uniform fashion, each information flow within

the component circuits.  Since this signal has a constant form, it

has come to be known as a clock signal, or sometimes clock pulse.

The practical significance of a clock is that it controls the data

rate within a system.  For this reason selecting a clock frequency is

is vitally important for determining the quantity of data which is

processed by, or transmitted from, a VGA controller.

It is possible to derive a clock of lower frequency, directly from

a master clock of higher frequency.  In the VGA controller this is

a very common occurence.

CRT

-------------

Cathode Ray Tube.  An important display technology which has been

vitally important in establishing currently used video generation

concepts and signal conventions.  See tube.

DAC

-------------

Digital to Analogue Converter.  This is an electronic device which

is able to convert a digital code into a high precision analogue

signal suitable for describing an image in a form acceptable by a

display unit.  These devices operate at very high speed.

Digital

-------------

A coding and interpretation scheme which relies upon discrete step

representation of values typically using a binary coding alphabet.

Each digit is represented by a signal value over two wires.  This

means that the scheme is very robust, but there is an attendant

overhead cost in terms of the number of connections which must be

made in order to convey digital information.  Contrast this with

analogue.

DIP Switch

-------------

Dual In-line Plastic Switch.  This is a miniture electronic switch

component which is used to provide selectable option settings with

hardware configuration use.  Typically a bank of 4, 6, or 8 of the

switches are employed, with each governing one particular feature.

Such features are generally set once and then forgotten since they

control hardware compatibility which does not often need changing.

Display Unit

-------------

A device which accepts information and produces a visual image as

a translation of that information.  See monitor.

Dot

-------------

A term used to describe a monochrome pixel.  This term arises in

connection with the dot clock which is a principle timing source

for VGA controller and also therefore, display unit operation.

DRAM

-------------

Dynamic Random Access Memory.  A type of memory technology which

stores data using a charged capacitor.  As charge will leak from

a capacitor, every so often all the stored data must be read out

and replaced afresh, thereby restoring the charge strength.  The

process which performs the charge restitution is termed refresh.

The refresh must be completed by accessing every DRAM address in

a short period of time termed the refresh period.  This time for

refresh is measured in a small number of milli-seconds.  See RAM.

EGA

-------------

Extended Graphics Array.  This is a video graphics standard which

is becoming less important in numerical terms of installed system

count, as time goes on.  Emulation may be important for EGA.

Emulation

-------------

The capability of a particular electronic device to mimic exactly

another, different electronic device so that the operation of the

former is indestinguishable from that of the latter.  This can be

important for maintaining compatibility between software produced

over a long period of time, and a wide range of computer systems.

Hercules

-------------

A Hercules Computer Technology trademark.  This company produces

video graphics image generators and Hercules refers particularly

to a widely used product for which emulation can be helpful.

Hardware

-------------

A description of electronic components, usually assembled into an

electronic system, or subsystem.  This represents an unchangeable

structure which may provide facilities for selection of different

interconnection schemes between constituent components.  Any type

of ability to vary the apparent internal structure, and therefore

the externally visible behaviour of hardware is normally referred

to as programmability.  Such hardware may be programmed.  Control

of such hardware is exercised by a program which is distinguished

from hardware by the term software.

IBM

-------------

Inernational Business Machines, Corp.  This company was the first

to introduce MDA, CGA, EGA, VGA graphics controllers, with their

associated display units.  Many companies follow IBM in building

products which are deemed IBM Compatible.  VGA controller chips

are a good example of this, although compatibility does have many

limits in practise.

Image Generator

-------------

An electronic device which accepts control information and image

information from a processor and translates this into a form of

information acceptable as input to a display unit.

ISA

-------------

Industry Standard Architecture.  This refers to a particular type

of computer system usually built from widely available components

which ideally fit together easily to produce a standard device of

known capability.  In practise, there may be difficulties because

of subtle incompatibility between various components.  Usually it

is possible to provide a certain set of capabilities adequate for

most practical purposes, in this manner.

MDA

-------------

Monochrome Display Adaptor. This is a video graphics standard that

has been important in times past, but which is now of interest for

emulation purposes.

Micro channel

-------------

A type of bus, which is proprietary to IBM.

Monitor

-------------

An alternative term for display unit.

MultiSync

-------------

A Nippon Electric Company (NEC) trademark.  Describes a family of

display units which are able to accept a number of distinct video

signals with different characteristics.  Importantly, the rate at

which image information is transferred to the display unit can be

changed so providing for a much higher definition image.  This is

achieved without any changes to the display unit, which can adapt

to the video signals by analysing them.  This automatically gives

rise to any changes necessary in the display unit operation.

NMI

-------------

Non Maskable Interrupt.  This is a higher priority interrupt than

any other interrupt available to the processor.  Specifically, it

is of higher priority than all maskable interrupts.  This fact is

often used to provide emulation facilities in software, triggered

by certain hardware conditions.  For example, write protection of

special registers useful in emulation, is often achieved by using

NMI interrupts in conjunction with special handler programs.

PC/AT

-------------

Personal Computer, Model AT.  This type of IBM compatible computer

is one of the most ubiquitous in the world.  Because of the force

of sheer numbers, many software and hardware products have been

built for use with this type of machine.  VGA controller chips are

normally configured for use with the bus provided by this machine.

Pixel

-------------

Picture ELement.  A display screen is made up from large numbers

of elementary displayable elements.  Every element is controlled

quite independantly of all other elements.  Since the assemblage

of all the available elements constitutes a picture the smallest

picture component has become known as a pixel.  See dot.

Processor

-------------

The computational device which acts as the seat of all activity in

a computer system.  Sometimes known as the CPU.  The processor is

the main determinant of capability for a computer system.  It is

implemented as a highly integrated electronic device.

RAM

-------------

Random Access Memory.  This type of memory technology is able to

store and retrieve information at high speed and is able to give

access to any address in the same uniform access time.  There is

no constraint such as sequential access to addresses, hence the

term random.  Any address can be accessed at any time with equal

facility.  Used for storing video image information in a digital

form suitable for manipulation by an image generator.  See DRAM.

Refresh

-------------

A process used to maintain the information stored in a DRAM.  For

more details see the entries for RAM, and for DRAM.

ROM

-------------

Read Only Memory.  This type of memory technology is manufactured

complete with all the information it is to contain.  This data is

immune to change so permanently required information such as font

data describing the shape of the characters in a character set is

usefully stored in this type of device.  A BIOS is usually stored

in a ROM.  Contrast with RAM, DRAM.

Software

-------------

The information associated with a particular function obtained by

programming hardware, so that some required external behaviour is

made available.  See hardware.

Standard

-------------

A term applied to many computer systems and their components.  It

is rather doubtful that this term actually means anything at all.

It has been the subject of so much abuse by so many, for so long,

that it has become almost meaningless.  See ISA.  In practise, it

is possible to find some particular products or interfaces which

are ubiquitous to the point of becoming a 'de facto' standard.

In these cases the term may have some use, providing it indicates

demonstrable compatibility with some readily obtained reference.

Synchronisation

-------------

The state in which one electronic system is operated as though it

was actually receiving clocking information from another separate

system.  This apparent synchronism between the respective clocks,

gives rise to the term synchronisation when describing the state.

It is vitally important that a display unit operate with absolute

synchronisation to the image generator from which it receives the

input video signals.

TTL

-------------

Transistor Transistor Logic, a logic family which gives rise to a

characteristic interface for digital signals known as TTL.  These

signals have certain well specified characteristics, so can be of

use in transmission of digital information between systems.  Some

display units accept digital data using TTL compatible signals.

Tube

-------------

The common term for a display tube or CRT.  See CRT.

VGA

-------------

Video Graphics Array.  This is the most important video graphics

standard at the time of writing.  Inexpensive to produce this is

becoming numerically the most common display generation scheme.

It is capable of simple extension, but supports a common set of

facilities.  It is particularly interesting for support given to

emulation of other graphics standards.  It is more than adequate

for almost all applications except the most demanding.

VGA Controller

-------------

This is a highly integrated electronic device which contains all

the logic functions required to implement a VGA controller which

is the image generator for producing VGA video information ready

for display.

Video

-------------

An electronic signal carrying picture information into a display

unit.  This information is in a form suitable for translation by

a display unit into an image readily comprehensible to the eye.

Index

===============================================================================

There are extensive cross references provided for all the VGA hardware register

descriptions, and these cross references do not rely on any concepts of page or

line numbers.   This is done with a purpose.  Many different page sizes can be

produced by current printing equipment, so it is suggested that the user employ

a readily available indexing program if a conventional index is really needed.

This will of course, depend entirely upon the page format selected by the user

in printing the document.  Guidelines for obtaining a print are available.

See {0.06} Obtaining a print of this document.

The contents list gives a useful overview of the chapter and section contents,

which, when used with the extensive cross references provides for an entirely

practical navigation aid around the text.  For this reason it is not envisaged

that any users will find the need to produce a conventional index.

There are two indexes included below.  The first is concerned with a complete

list of all the functional register groups available to the VGA programmer.

The second expands on the first, by including each of the bit fields present

in each register.  The format in both cases is the same as that for the entry

proper, in the main body of the text.  Details of input output port addresses

are therefore included, with register index values if needed, and read, write

or index functionality as appropriate.

Complete Register Listing By Functional Grouping

------------------------------------------------

Sequencer Registers

------------------------------------------------------------------------------

6.04.01  Sequencer Address Register      { 3c4h    ---    R/W }

6.04.02  Reset Register                  { 3c5h    00h    R/W }

6.04.03  Clocking Mode Register          { 3c5h    01h    R/W }

6.04.04  Map Mask Register               { 3c5h    02h    R/W }

6.04.05  Character Map Select Register   { 3c5h    03h    R/W }

6.04.06  Sequencer Memory Mode Register  { 3c5h    04h    R/W }

6.04.07  ................................{ 3c5h    07h    R/W }

6.04.07  Horizontal Character Counter Reset Register...........

------------------------------------------------------------------------------

CRT Controller Registers

------------------------------------------------------------------------------

6.05.01  CRT Controller Address Register          { 3?4h    ---    R/W }

6.05.02  Horizontal Total Register                { 3?5h    00h    R/W }

6.05.03  Horizontal Display Enable End Register   { 3?5h    01h    R/W }

6.05.04  Start Horizontal Blanking Register       { 3?5h    02h    R/W }

6.05.05  End Horizontal Blanking Register         { 3?5h    03h    R/W }

6.05.06  Start Horizontal Retrace Pulse Register  { 3?5h    04h    R/W }

6.05.07  End Horizontal Retrace Register          { 3?5h    05h    R/W }

6.05.08  Vertical Total Register                  { 3?5h    06h    R/W }

6.05.09  CRT Controller Overflow Register         { 3?5h    07h    R/W }

6.05.10  Preset Row Scan Register                 { 3?5h    08h    R/W }

6.05.11  Maximum Scan Line Register               { 3?5h    09h    R/W }

6.05.12  Cursor Start Register                    { 3?5h    0ah    R/W }

6.05.13  Cursor End Register                      { 3?5h    0bh    R/W }

6.05.14  Start Address High Register              { 3?5h    0ch    R/W }

6.05.15  Start Address Low Register               { 3?5h    0dh    R/W }

6.05.16  Cursor Location High Register            { 3?5h    0eh    R/W }

6.05.17  Cursor Location Low Register             { 3?5h    0fh    R/W }

6.05.18  Vertical Retrace Start Register          { 3?5h    10h    R/W }

6.05.19  Vertical Retrace End Register            { 3?5h    11h    R/W }

6.05.20  Vertical Display Enable End Register     { 3?5h    12h    R/W }

6.05.21  Offset Register                          { 3?5h    13h    R/W }

6.05.22  Underline Location Register              { 3?5h    14h    R/W }

6.05.23  Start Vertical Blanking Register         { 3?5h    15h    R/W }

6.05.24  End Vertical Blanking Register           { 3?5h    16h    R/W }

6.05.25  CRTC Mode Control Register               { 3?5h    17h    R/W }

6.05.26  Line Compare Register                    { 3?5h    18h    R/W }

------------------------------------------------------------------------------

Graphics Controller Registers

------------------------------------------------------------------------------

6.06.01  Graphics Address Register    { 3ceh    ---    R/W }

6.06.02  Set/Reset Register           { 3cfh    00h    R/W }

6.06.03  Enable Set/Reset Register    { 3cfh    01h    R/W }

6.06.04  Colour Compare Register      { 3cfh    02h    R/W }

6.06.05  Data Rotate Register         { 3cfh    03h    R/W }

6.06.06  Read Map Select Register     { 3cfh    04h    R/W }

6.06.07  Graphics Mode Register       { 3cfh    05h    R/W }

6.06.08  Miscellaneous Register       { 3cfh    06h    R/W }

6.06.09  Colour Don't Care Register   { 3cfh    07h    R/W }

6.06.10  Bit Mask Register            { 3cfh    08h    R/W }

------------------------------------------------------------------------------

Attribute Controller Registers

------------------------------------------------------------------------------

6.07.01  Attribute Address Register       { 3c0h/I/W  3c0h/R  ---  R/W }

6.07.02  Palette Register 00              { 3c0h/I/W  3c1h/R  00h  R/W }

6.07.03  Palette Register 01              { 3c0h/I/W  3c1h/R  01h  R/W }

6.07.04  Palette Register 02              { 3c0h/I/W  3c1h/R  02h  R/W }

6.07.05  Palette Register 03              { 3c0h/I/W  3c1h/R  03h  R/W }

6.07.06  Palette Register 04              { 3c0h/I/W  3c1h/R  04h  R/W }

6.07.07  Palette Register 05              { 3c0h/I/W  3c1h/R  05h  R/W }

6.07.08  Palette Register 06              { 3c0h/I/W  3c1h/R  06h  R/W }

6.07.09  Palette Register 07              { 3c0h/I/W  3c1h/R  07h  R/W }

6.07.10  Palette Register 08              { 3c0h/I/W  3c1h/R  08h  R/W }

6.07.11  Palette Register 09              { 3c0h/I/W  3c1h/R  09h  R/W }

6.07.12  Palette Register 10              { 3c0h/I/W  3c1h/R  0ah  R/W }

6.07.13  Palette Register 11              { 3c0h/I/W  3c1h/R  0bh  R/W }

6.07.14  Palette Register 12              { 3c0h/I/W  3c1h/R  0ch  R/W }

6.07.15  Palette Register 13              { 3c0h/I/W  3c1h/R  0dh  R/W }

6.07.16  Palette Register 14              { 3c0h/I/W  3c1h/R  0eh  R/W }

6.07.17  Palette Register 15              { 3c0h/I/W  3c1h/R  0fh  R/W }

6.07.18  Attribute Mode Control Register  { 3c0h/I/W  3c1h/R  10h  R/W }

6.07.19  Overscan Colour Register         { 3c0h/I/W  3c1h/R  11h  R/W }

6.07.20  Colour Plane Enable Register     { 3c0h/I/W  3c1h/R  12h  R/W }

6.07.21  Horizontal PEL Panning Register  { 3c0h/I/W  3c1h/R  13h  R/W }

6.07.22  Colour Select Register           { 3c0h/I/W  3c1h/R  14h  R/W }

------------------------------------------------------------------------------

DAC Registers

------------------------------------------------------------------------------

6.08.01  DAC Pixel Mask Register          { 3c6h   R/W }

6.08.02  DAC Status Register              { 3c7h   R-- }

6.08.03  DAC Read Data Address Register   { 3c7h   W-- }

6.08.04  DAC Write Data Address Register  { 3c8h   R/W }

6.08.05  DAC Data Register                { 3c9h   R/W }

------------------------------------------------------------------------------

General Registers

------------------------------------------------------------------------------

6.09.01  Miscellaneous Output Register  { 3c2h  W-- }

6.09.02  Miscellaneous Output Register  { 3cch  --R }

6.09.03  Input Status Register Zero     { 3c2h  R-- }

6.09.04  Input Status Register One      { 3?ah  R-- }

------------------------------------------------------------------------------

Special Registers

------------------------------------------------------------------------------

6.10.01  Hardware Version Register {new definition}  { 3c5h    0bh    R-- }

6.10.02  Version Selector Register {old definition}  { 3c5h    0bh    --W }

6.10.03  Configuration Port Register 1               { 3c5h    0ch    R/W }

6.10.04  Mode Control Register 2                     { 3c5h    0dh    R/W }

6.10.05  Mode Control Register 1                     { 3c5h    0eh    R/W }

6.10.06  Power-up Mode Register 2                    { 3c5h    0fh    R/W }

6.10.07  CRTC Module Testing Register                { 3?5h    1eh    R/W }

6.10.08  Software Programming Register               { 3?5h    1fh    R/W }

6.10.09  CPU Latch Read Back Register                { 3?5h    22h    R-- }

6.10.10  Attribute State Read Back Register          { 3?5h    24h    R-- }

6.10.11  Attribute Index Read Back Register          { 3?5h    26h    R-- }

6.10.12  Clear Vertical Display Enable Register      { 3?5h    3nh    W-- }

6.10.13  Source Address Register                     { 3cfh    0eh    R/W }

6.10.14  Source Address Enable Register              { 3cfh    0fh    R/W }

------------------------------------------------------------------------------

Complete Register Listing Including Bit Fields

----------------------------------------------

Sequencer Registers

------------------------------------------------------------------------------

6.04.01  Sequencer Address Register      { 3c4h    ---    R/W }

------------------------------------------------------

<3..0>   Sequencer Register Address

<7..4>   Reserved. Always <=0000>.

------------------------------------------------------

6.04.02  Reset Register                  { 3c5h    00h    R/W }

------------------------------------------------------

<0>      Fast Reset Command

<1>      Safe Reset Command

<7..2>   Reserved. Always <=000000>.

------------------------------------------------------

6.04.03  Clocking Mode Register          { 3c5h    01h    R/W }

------------------------------------------------------

<0>      Character Dot Clock Select

<1>      Reserved.  Always <=0>.

<2>      Shift Load Two Control

<3>      Dot Clock Divide by Two Enable

<4>      Shift Load Four Control

<5>      Screen Inhibit

<7..6>   Reserved.  Always <=00>.

------------------------------------------------------

6.04.04  Map Mask Register               { 3c5h    02h    R/W }

------------------------------------------------------

<0>      Map Plane 0 Write Enable Control

<1>      Map Plane 1 Write Enable Control

<2>      Map Plane 2 Write Enable Control

<3>      Map Plane 3 Write Enable Control

<7..4>   Reserved.  Always <=0000>.

------------------------------------------------------

6.04.05  Character Map Select Register   { 3c5h    03h    R/W }

------------------------------------------------------

<0>      Character Font Memory Map 'B' Partial Offset 16K byte

<1>      Character Font Memory Map 'B' Partial Offset 32K byte

<2>      Character Font Memory Map 'A' Partial Offset 16K byte

<3>      Character Font Memory Map 'A' Partial Offset 32K byte

<4>      Character Font Memory Map 'B' Partial Offset 08K byte

<5>      Character Font Memory Map 'A' Partial Offset 08K byte

<7..6>   Reserved.  Always <=00>.

------------------------------------------------------

6.04.06  Sequencer Memory Mode Register  { 3c5h    04h    R/W }

------------------------------------------------------

<0>      Reserved.  Always <=0>.

<1>      Memory Size Specification

<2>      Processor Image Memory Access Mode Select

<3>      Processor Image Memory Access Mapping Select

<7..4>   Reserved.  Always <=0000>.

------------------------------------------------------

6.04.07  Horizontal Character Counter Reset Register...........

6.04.07  ................................{ 3c5h    07h    R/W }

------------------------------------------------------

<7..0>   Horizontal Character Counter Reset

------------------------------------------------------

------------------------------------------------------------------------------

CRT Controller Registers

------------------------------------------------------------------------------

6.05.01  CRT Controller Address Register          { 3?4h    ---    R/W }

---------------------------------------------------------------

<5..0>   CRT Controller Register Address

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.05.02  Horizontal Total Register                { 3?5h    00h    R/W }

---------------------------------------------------------------

<7..0>   Horizontal Total

---------------------------------------------------------------

6.05.03  Horizontal Display Enable End Register   { 3?5h    01h    R/W }

---------------------------------------------------------------

<7..0>   Horizontal Display Enable

---------------------------------------------------------------

6.05.04  Start Horizontal Blanking Register       { 3?5h    02h    R/W }

---------------------------------------------------------------

<7..0>   Start Horizontal Blanking

---------------------------------------------------------------

6.05.05  End Horizontal Blanking Register         { 3?5h    03h    R/W }

---------------------------------------------------------------

<4..0>   End Horizontal Blanking

<6..5>   Display Enable Skew

<7>      Light Pen Register Enable

---------------------------------------------------------------

6.05.06  Start Horizontal Retrace Pulse Register  { 3?5h    04h    R/W }

---------------------------------------------------------------

<7..0>   Start Horizontal Retrace Pulse

---------------------------------------------------------------

6.05.07  End Horizontal Retrace Register          { 3?5h    05h    R/W }

---------------------------------------------------------------

<4..0>   End Horizontal Retrace

<6..5>   Horizontal Retrace Skew

<7>      End Horizontal Blanking Bit 5

---------------------------------------------------------------

6.05.08  Vertical Total Register                  { 3?5h    06h    R/W }

---------------------------------------------------------------

<7..0>   Vertical Total

---------------------------------------------------------------

6.05.09  CRT Controller Overflow Register         { 3?5h    07h    R/W }

---------------------------------------------------------------

<0>      Vertical Total Bit 8

<1>      Vertical Display Enable End Bit 8

<2>      Vertical Retrace Start Bit 8

<3>      Start Vertical Blank Bit 8

<4>      Line Compare Bit 8

<5>      Vertical Total Bit 9

<6>      Vertical Display Enable End Bit 9

<7>      Vertical Retrace Start Bit 9

---------------------------------------------------------------

6.05.10  Preset Row Scan Register                 { 3?5h    08h    R/W }

---------------------------------------------------------------

<4..0>   Preset Row Scan

<6..5>   Byte Panning Control

<7>      Reserved.  Always <=0>.

---------------------------------------------------------------

6.05.11  Maximum Scan Line Register               { 3?5h    09h    R/W }

---------------------------------------------------------------

<4..0>   Maximum Scan Lines

<5>      Start Vertical Blank Bit 9

<6>      Line Compare Bit 9

<7>      Double Scan Line Display

---------------------------------------------------------------

6.05.12  Cursor Start Register                    { 3?5h    0ah    R/W }

---------------------------------------------------------------

<4..0>   Cursor Row Scan Start

<5>      Cursor Enable

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.05.13  Cursor End Register                      { 3?5h    0bh    R/W }

---------------------------------------------------------------

<4..0>   Cursor Row Scan End

<6..5>   Cursor Skew Control

<7>      Reserved.  Always <=0>.

---------------------------------------------------------------

6.05.14  Start Address High Register              { 3?5h    0ch    R/W }

---------------------------------------------------------------

<7..0>   Start Address High

---------------------------------------------------------------

6.05.15  Start Address Low Register               { 3?5h    0dh    R/W }

---------------------------------------------------------------

<7..0>   Start Address Low

---------------------------------------------------------------

6.05.16  Cursor Location High Register            { 3?5h    0eh    R/W }

---------------------------------------------------------------

<7..0>   Cursor Location High

---------------------------------------------------------------

6.05.17  Cursor Location Low Register             { 3?5h    0fh    R/W }

---------------------------------------------------------------

<7..0>   Cursor Location Low

---------------------------------------------------------------

6.05.18  Vertical Retrace Start Register          { 3?5h    10h    R/W }

---------------------------------------------------------------

<7..0>   Vertical Retrace Start

---------------------------------------------------------------

6.05.19  Vertical Retrace End Register            { 3?5h    11h    R/W }

---------------------------------------------------------------

<3..0>   Vertical Retrace End

<4>      Clear Vertical Interrupt

<5>      Enable Vertical Interrupt

<6>      Refresh Bandwidth Select

<7>      CRTC Register Protect

---------------------------------------------------------------

6.05.20  Vertical Display Enable End Register     { 3?5h    12h    R/W }

---------------------------------------------------------------

<7..0>   Vertical Display Enable End

---------------------------------------------------------------

6.05.21  Offset Register                          { 3?5h    13h    R/W }

---------------------------------------------------------------

<7..0>   Display Image Offset

---------------------------------------------------------------

6.05.22  Underline Location Register              { 3?5h    14h    R/W }

---------------------------------------------------------------

<4..0>   Underline Location

<5>      Display Address Dwell Count 4

<6>      Display Memory Address Unit 4

<7>      Reserved.  Always <=0>.

---------------------------------------------------------------

6.05.23  Start Vertical Blanking Register         { 3?5h    15h    R/W }

---------------------------------------------------------------

<7..0>   Start Vertical Blanking

---------------------------------------------------------------

6.05.24  End Vertical Blanking Register           { 3?5h    16h    R/W }

---------------------------------------------------------------

<7..0>   End Vertical Blanking

---------------------------------------------------------------

6.05.25  CRTC Mode Control Register               { 3?5h    17h    R/W }

---------------------------------------------------------------

<0>      Display Address 13 Remap Row 0

<1>      Display Address 14 Remap Row 1

<2>      Horizontal Retrace Divisor

<3>      Display Address Dwell Count 2

<4>      Reserved.  Always <=0>.

<5>      Display Address Unit 2 Remap

<6>      Display Memory Address Unit 2

<7>      Horizontal/Vertical Retrace Reset

---------------------------------------------------------------

6.05.26  Line Compare Register                    { 3?5h    18h    R/W }

---------------------------------------------------------------

<7..0>   Line Compare

---------------------------------------------------------------

------------------------------------------------------------------------------

Graphics Controller Registers

------------------------------------------------------------------------------

6.06.01  Graphics Address Register    { 3ceh    ---    R/W }

---------------------------------------------------

<3..0>   Graphics Register Address

<7..4>   Reserved. Always <=0000>.

---------------------------------------------------

6.06.02  Set/Reset Register           { 3cfh    00h    R/W }

---------------------------------------------------

<0>      Set/Reset Data Image Plane 0

<1>      Set/Reset Data Image Plane 1

<2>      Set/Reset Data Image Plane 2

<3>      Set/Reset Data Image Plane 3

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------

6.06.03  Enable Set/Reset Register    { 3cfh    01h    R/W }

---------------------------------------------------

<0>      Set/Reset Enable Image Plane 0

<1>      Set/Reset Enable Image Plane 1

<2>      Set/Reset Enable Image Plane 2

<3>      Set/Reset Enable Image Plane 3

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------

6.06.04  Colour Compare Register      { 3cfh    02h    R/W }

---------------------------------------------------

<0>      Colour Compare Plane 0

<1>      Colour Compare Plane 1

<2>      Colour Compare Plane 2

<3>      Colour Compare Plane 3

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------

6.06.05  Data Rotate Register         { 3cfh    03h    R/W }

---------------------------------------------------

<2..0>   Data Rotate

<4..3>   Logic Unit Function Code

<7..5>   Reserved.  Always <=000>.

---------------------------------------------------

6.06.06  Read Map Select Register     { 3cfh    04h    R/W }

---------------------------------------------------

<1..0>   Read Map Select

<7..2>   Reserved.  Always <=000000>.

---------------------------------------------------

6.06.07  Graphics Mode Register       { 3cfh    05h    R/W }

---------------------------------------------------

<1..0>   Processor Write Mode

<2>      Reserved.  Always <=0>.

<3>      Processor Read Mode

<4>      Graphics Memory Access Mode

<5>      Graphics Shift Register Mode

<6>      Graphics 256 Colour Control

<7>      Reserved.  Always <=0>.

---------------------------------------------------

6.06.08  Miscellaneous Register       { 3cfh    06h    R/W }

---------------------------------------------------

<0>      Display Generation Mode

<1>      Memory Plane Pairing Control

<3..2>   Memory Map Mode

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------

6.06.09  Colour Don't Care Register   { 3cfh    07h    R/W }

---------------------------------------------------

<0>      Colour Don't Care Plane 0

<1>      Colour Don't Care Plane 1

<2>      Colour Don't Care Plane 2

<3>      Colour Don't Care Plane 3

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------

6.06.10  Bit Mask Register            { 3cfh    08h    R/W }

---------------------------------------------------

<0>   Update Value Source Bit 0

<1>   Update Value Source Bit 1

<2>   Update Value Source Bit 2

<3>   Update Value Source Bit 3

<4>   Update Value Source Bit 4

<5>   Update Value Source Bit 5

<6>   Update Value Source Bit 6

<7>   Update Value Source Bit 7

---------------------------------------------------

------------------------------------------------------------------------------

Attribute Controller Registers

------------------------------------------------------------------------------

6.07.01  Attribute Address Register       { 3c0h/I/W  3c0h/R  ---  R/W }

---------------------------------------------------------------

<4..0>   Attribute Address

<5>      Palette Control

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.02  Palette Register 00              { 3c0h/I/W  3c1h/R  00h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.03  Palette Register 01              { 3c0h/I/W  3c1h/R  01h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.04  Palette Register 02              { 3c0h/I/W  3c1h/R  02h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.05  Palette Register 03              { 3c0h/I/W  3c1h/R  03h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.06  Palette Register 04              { 3c0h/I/W  3c1h/R  04h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.07  Palette Register 05              { 3c0h/I/W  3c1h/R  05h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.08  Palette Register 06              { 3c0h/I/W  3c1h/R  06h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.09  Palette Register 07              { 3c0h/I/W  3c1h/R  07h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.10  Palette Register 08              { 3c0h/I/W  3c1h/R  08h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.11  Palette Register 09              { 3c0h/I/W  3c1h/R  09h  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.12  Palette Register 10              { 3c0h/I/W  3c1h/R  0ah  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.13  Palette Register 11              { 3c0h/I/W  3c1h/R  0bh  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.14  Palette Register 12              { 3c0h/I/W  3c1h/R  0ch  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.15  Palette Register 13              { 3c0h/I/W  3c1h/R  0dh  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.16  Palette Register 14              { 3c0h/I/W  3c1h/R  0eh  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.17  Palette Register 15              { 3c0h/I/W  3c1h/R  0fh  R/W }

---------------------------------------------------------------

<5..0>   Colour Value

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.18  Attribute Mode Control Register  { 3c0h/I/W  3c1h/R  10h  R/W }

---------------------------------------------------------------

<0>      Display Generation Mode

<1>      Display Generation Mode

<2>      Line Graphics Character Code Control

<3>      Text Mode Attribute Select

<4>      Reserved.  Always <=0>.

<5>      Split Screen Panning Control

<6>      Shift Register 256 Colour Assembly Latch

<7>      External Colour Palette Input Select

---------------------------------------------------------------

6.07.19  Overscan Colour Register         { 3c0h/I/W  3c1h/R  11h  R/W }

---------------------------------------------------------------

<7..0>   Overscan Colour

---------------------------------------------------------------

6.07.20  Colour Plane Enable Register     { 3c0h/I/W  3c1h/R  12h  R/W }

---------------------------------------------------------------

<0>      Colour Plane Enable 0

<1>      Colour Plane Enable 1

<2>      Colour Plane Enable 2

<3>      Colour Plane Enable 3

<5..4>   Display Status Select

<7..6>   Reserved.  Always <=00>.

---------------------------------------------------------------

6.07.21  Horizontal PEL Panning Register  { 3c0h/I/W  3c1h/R  13h  R/W }

---------------------------------------------------------------

<3..0>   Horizontal PEL Panning

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------------------

6.07.22  Colour Select Register           { 3c0h/I/W  3c1h/R  14h  R/W }

---------------------------------------------------------------

<1..0>   External colour palette bits <5..4>

<3..2>   External colour palette bits <7..6>

<7..4>   Reserved.  Always <=0000>.

---------------------------------------------------------------

------------------------------------------------------------------------------

DAC Registers

------------------------------------------------------------------------------

6.08.01  DAC Pixel Mask Register          { 3c6h   R/W }

-----------------------------------------------

<0>      DAC Pixel Mask 0 {Digital to Analogue Converter Pixel Mask}

<1>      DAC Pixel Mask 1 {Digital to Analogue Converter Pixel Mask}

<2>      DAC Pixel Mask 2 {Digital to Analogue Converter Pixel Mask}

<3>      DAC Pixel Mask 3 {Digital to Analogue Converter Pixel Mask}

<4>      DAC Pixel Mask 4 {Digital to Analogue Converter Pixel Mask}

<5>      DAC Pixel Mask 5 {Digital to Analogue Converter Pixel Mask}

<6>      DAC Pixel Mask 6 {Digital to Analogue Converter Pixel Mask}

<7>      DAC Pixel Mask 7 {Digital to Analogue Converter Pixel Mask}

-----------------------------------------------

6.08.02  DAC Status Register              { 3c7h   R-- }

-----------------------------------------------

<1..0>   DAC Status

<7..2>   Reserved.  Always <=000000>.

-----------------------------------------------

6.08.03  DAC Read Data Address Register   { 3c7h   W-- }

-----------------------------------------------

<7..0>   DAC Read Data Address

-----------------------------------------------

6.08.04  DAC Write Data Address Register  { 3c8h   R/W }

-----------------------------------------------

<7..0>   DAC Write Data Address

-----------------------------------------------

6.08.05  DAC Data Register                { 3c9h   R/W }

-----------------------------------------------

<7..0>   DAC Data

-----------------------------------------------

------------------------------------------------------------------------------

General Registers

------------------------------------------------------------------------------

6.09.01  Miscellaneous Output Register  { 3c2h  W-- }

--------------------------------------------

<0>      Input Output Address Select

<1>      Display Memory Access Enable

<3..2>   Dot Clock Frequency Select

<4>      Reserved.  Always <=0>.

<5>      Address Extension Bit

<6>      Horizontal Synchronisation Signal Polarity

<7>      Vertical Synchronisation Signal Polarity

--------------------------------------------

6.09.02  Miscellaneous Output Register  { 3cch  --R }

--------------------------------------------

<7..0>   Miscellaneous Output

--------------------------------------------

6.09.03  Input Status Register Zero     { 3c2h  R-- }

--------------------------------------------

<3..0>   Reserved.  Always <=0000>.

<4>      Monitor Detect Status Bit

<6..5>   Reserved.  Always <=00>.

<7>      CRT Controller Interrupt State

--------------------------------------------

6.09.04  Input Status Register One      { 3?ah  R-- }

--------------------------------------------

<0>      Inverted Display Enable Status

<1>      Reserved.  Always <=0>.

<2>      Reserved.  Always <=0>.  {Trident 8900 <=1>.}

<3>      Vertical Retrace State

<5..4>   Video Sampling State

<7..6>   Reserved.  Always <=00>.

--------------------------------------------

------------------------------------------------------------------------------

Special Registers

------------------------------------------------------------------------------

6.10.01  Hardware Version Register {new definition}  { 3c5h    0bh    R-- }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<3..0>   Chip version identification   {Trident}

<7..4>   Reserved.

------------------------------------------------------------------

6.10.02  Version Selector Register {old definition}  { 3c5h    0bh    --W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

see main text body for details

------------------------------------------------------------------

6.10.03  Configuration Port Register 1               { 3c5h    0ch    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

Definition {only when NMI disabled}

------------------------------------------------------------------

<0>      Address Decode Source

<1>      ROM Chip Count Specify

<2>      Reserved.

<3>      ROM Address Span Specify

<4>      VGA Control Port Specify

<6..5>   DRAM Configuration

<7>      DRAM Access Width

------------------------------------------------------------------

Definition {only when NMI enabled}

------------------------------------------------------------------

<0>      Software Status

<1>      NMI State {Non Maskable Interrupt State}

<3..2>   Testing Bits

<7..4>   NMI Vector Enable

------------------------------------------------------------------

6.10.04  Mode Control Register 2                     { 3c5h    0dh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

New Definition

------------------------------------------------------------------

<0>      Dot Clock Extended Select

<2..1>   Dot Clock Division Ratio Select

<3>      Software Status

<5..4>   Software Status

<7..6>   General Purpose Outputs

------------------------------------------------------------------

Old Definition

------------------------------------------------------------------

<2..0>   Video Generation Mode Select

<3>      Processor Bandwidth during Blanking

<4>      Memory Plane Architecture Select

<5>      DRAM Clock Control

<7..6>   Character Set Select High Extend

------------------------------------------------------------------

6.10.05  Mode Control Register 1                     { 3c5h    0eh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

New Definition

------------------------------------------------------------------

<0>      Segment Selector

<1>      Page Selector

<3..2>   Bank Selector

<6..4>   Software Status

<7>      Write Protection State

------------------------------------------------------------------

Old Definition

------------------------------------------------------------------

<0>      Display Access Bank Selector

<2..1>   Processor Access Bank Selector

<3>      DRAM Access Width

<4>      Reserved for Dot Clock Extended Select

<5>      NMI State {Non Maskable Interrupt State}

<6>      Interrupt Request Polarity Select

<7>      Clock Pins Function Select

------------------------------------------------------------------

6.10.06  Power-up Mode Register 2                    { 3c5h    0fh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<3..0>   DIP Switch Image

<4>      Bus Protocol Select

<5>      Input Output Address Map Select

<6>      ROM BIOS Address Decode Enable

<7>      ROM BIOS Data Path Width

------------------------------------------------------------------

6.10.07  CRTC Module Testing Register                { 3?5h    1eh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<0>      Horizontal Clock Select

<1>      Character Clock Select

<2>      Interlace Enable

<3>      Multipurpose Select

<4>      CRT Controller Register Protection State

<5>      Start Address Extension Bit 16

<6>      Miscellaneous Output Register Protect State

<7>      Start Address Extension Enable

------------------------------------------------------------------

6.10.08  Software Programming Register               { 3?5h    1fh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<1..0>   Reserved for BIOS use.

<3..2>   Software Status Bits

<7..4>   Reserved for BIOS use.  Do NOT disturb.

------------------------------------------------------------------

6.10.09  CPU Latch Read Back Register                { 3?5h    22h    R-- }

---------------------------------------------------------------------------

standard register officially undocumented

------------------------------------------------------------------

<7..0>   CPU Latch Read Back

------------------------------------------------------------------

6.10.10  Attribute State Read Back Register          { 3?5h    24h    R-- }

---------------------------------------------------------------------------

standard register officially undocumented

------------------------------------------------------------------

<4..0>   Attribute Controller Index

<5>      Colour Palette Address Source {Internal palette}

<6>      Reserved.

<7>      Attribute State Read Back

------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<6..0>   Reserved.  Undefined.

<7>      Attribute State Read Back

------------------------------------------------------------------

6.10.11  Attribute Index Read Back Register          { 3?5h    26h    R-- }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<4..0>   Attribute Index

<7..5>   Reserved.

------------------------------------------------------------------

6.10.12  Clear Vertical Display Enable Register      { 3?5h    3nh    W-- }

---------------------------------------------------------------------------

standard register officially undocumented

------------------------------------------------------------------

<0>      Clear Vertical Display Enable

<7..1>   Unused.  Any value can be present.

------------------------------------------------------------------

6.10.13  Source Address Register                     { 3cfh    0eh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

see main text body for details

------------------------------------------------------------------

6.10.14  Source Address Enable Register              { 3cfh    0fh    R/W }

---------------------------------------------------------------------------

Trident 8900 only

------------------------------------------------------------------

<0>      Source Address Enable

<7..1>   Reserved.  Always <=0000000>.

------------------------------------------------------------------

------------------------------------------------------------------------------

Thank you for using this reference work.  If you have any useful comments which

others would benefit from, please do take a little of your valuable time to let

me know your thoughts.  Only by so doing will this work improve over time.  The

author really wants to hear!

[End of file: vga.doc]