KBG_11B.EXE

A Fuzzy Logic

Knowledge Base Generator

for the

MC68HC11 Inference Engine.

This tutorial demonstrates how to use the program KBG_11B.EXE. The program is a preprocessor which generates a Knowledge Base for the MC68HC11 Inference Engine, FUZZY11B.ASM. Please note that it cannot be used with FUZZY11A.ASM. The "A" version of Fuzzy11 requires a Macro Assembler and generates it's own Knowledge Base with the help of the Macro Assembler. The Inference Engine, FUZZY11B.ASM, requires the Fuzzy Knowledge Base to be in the form of "FCBs". This is a data structure generated by KBG_11B.EXE. The assembler used with FUZZY11B.ASM does not have to have macro capability. KBG_11B allows you to:

1) Create a Knowledge Base using a natural language format.

2) Load an existing Knowledge Base.

3) Save the current Knowledge Base.

4) Edit a Knowledge Base.

5) Show the formatted Knowledge Base to:

a) Console Screen.

b) Print file.

6) Generate the MC68HC11 Assembly Code File.

The current version of KBG_11B is compiled to accept:

1) 8 Inputs per Fuzzy application.

2) 4 Outputs per Fuzzy application.

3) 8 Membership Functions per Input.

4) 8 Membership Functions per Output.

5) 1024 Rule clauses (Inference/Consequence).

6) Any number of Inferences per Rule.

7) Any number of Consequences per Rule.

8) 15 Characters per Name (Input, Output, Member).

9) 4 Points per Input Member.

10) 1 Point per Output Member.

KBG_11B is written in a very generic "C" language style so that it can be ported to other machines or languages, specifically Small-C or BASIC. No structures or data types other than CHAR and INT are used. Currently UNIX and IBM-PC versions with source code are available on the Motorola Freeware BBS. A MAC version is planned but not scheduled. You may reach the Freeware BBS at (512) 891-FREE. Connect at 2400, 1200, or 300 baud with 8 bits, No Parity, and 1 Stop Bit. The Freeware BBS also has the code for the FUZZY11B Inference Engine.

This tutorial uses the IBM-PC version to demonstrate how to create, modify, and save a new Knowledge Base for the FUZZY11B Inference Engine. KBG_11B uses some ANSI screen control characters. Thus, you must load ANSI.SYS before using KBG_11B. The UNIX version also uses ANSI control characters so you must use a terminal which understands ANSI.

This example shows the coding of the classic inverted pendulum balancing problem. The problem consists of a variable amount of mass atop a variable length rod. The base of the rod is moved to balance the rod and mass. This example show how to code the problem as presented in chapter 8 of Bart Kosko's book, NEURAL NETWORKS AND FUZZY SYSTEMS (Prentice Hall).

The figure to the right shows a schematic of the problem situation. For our example, we consider only motion on the X-Y axis, not the Z-Y axis. This simplifies the problem to only two inputs. Input 1 is the angle that the pendulum makes to the vertical. Input 2 is the rate of change of the angle in degrees per second.

Once the inputs have been identified, we must create Membership Functions for each input. For Input 1, there are seven Membership Functions. For a negative deflection of the angle, we have identified three members. A large deflection, -36 to -90 degrees, is labeled Negative_Large (NL). A lesser defection, between -18 and -54 degrees, is labeled Negative_Medium (NM). A deflection of 0 to -36 degrees is labeled Negative_Small (NS). There are three positive deflections which are symmetrical. A deflection of Zero (ZE) occurs from -18 to +18 degrees. Note here that as is the case with fuzzy logic a particular deflection can fall into more than one group. For example, a deflection of -5 degrees is Zero to a great extent and Negative_Small to a lesser extent.

Input 2 is almost identical, but refers to rate of change. Thus, a value of +30 means a change of +30 degrees per second. See the diagram below for the details of Input Membership Functions.

There is only one output for this system. This output controls the motor rotation to move the balance point along the X-axis, to the right or left. The Membership Functions for the output are shown below. There are three levels of drive current for the motor in each direction and zero current for the stopped condition. The seven motor current levels are:

NL	Negative_Large

NM	Negative_Medium

NS	Negative_Small

ZE	Zero

PS	Positive_Small

PM	Positive_Medium

PL	Positive_Large

The Inference Engine defines Outputs as singletons, thus, there is only one discrete point for each output.

Now let's begin the coding exercise. The first step is to call KBG_11B.EXE with the filename of our Knowledge Base. The following DOS command executes KBG_11B.EXE using a default name of "balance" for all data files. Note that no files are loaded, saved, or created as a result of this DOS command. The name "balance" is saved for future use with load, save, or create commands.

C:> kBG_11B balance

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >3

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >1

The current Knowledge Base is aware of the following INPUTS:

Input #1 ----> ~

Input #2 ----> ~

Input #3 ----> ~

Input #4 ----> ~

Input #5 ----> ~

Input #6 ----> ~

Input #7 ----> ~

Input #8 ----> ~

We see here that there are no inputs defined yet for this Knowledge Base. Undefined inputs are represented by the Tilde symbol. Using the KBG_11B prompt, we select (A)dd to begin the process of supplying input names and data. Answering the following prompts, we assign the name, "Angle", to input #1 and tell the Knowledge Base that it is measured in Degrees, ranging from -90 degrees to +90 degrees. Internally, KBG_11B represents all Input and Output ranges as an integer with a value of 0 to 255. However, all internal storage and scaling details are transparent to the user. You may enter the range in any approriate natural units and all user interaction will be in those units.

Do you wish to (A)dd INPUTS, (D)elete INPUTS, or (Q)uit? >A

What is the NAME of this input? Input #1 > >Angle

What UNITS(MPH, DegC, PPM, etc..) are used to describe it? >Degrees

What is the MINIMUM value that this Input may assume? >-90

What is the MAXIMUM value that this Input may assume? >90

KBG_11B is now aware of the new input and displays it's knowledge of all inputs, including the new one. We repeat the process, describing the Delta_Angle as input #2.

The current Knowledge Base is aware of the following INPUTS:

Input #1 ----> Angle

Input #2 ----> ~

Input #3 ----> ~

Input #4 ----> ~

Input #5 ----> ~

Input #6 ----> ~

Input #7 ----> ~

Input #8 ----> ~

Do you wish to (A)dd INPUTS, (D)elete INPUTS, or (Q)uit? >a

What is the NAME of this input? Input #2 > >Delta_Angle

What UNITS(MPH, DegC, PPM, etc..) are used to describe it? >Deg_Second

What is the MINIMUM value that this Input may assume? >-90

What is the MAXIMUM value that this Input may assume? >90

The current Knowledge Base is aware of the following INPUTS:

Input #1 ----> Angle

Input #2 ----> Delta_Angle

Input #3 ----> ~

Input #4 ----> ~

Input #5 ----> ~

Input #6 ----> ~

Input #7 ----> ~

Input #8 ----> ~

Both inputs are now defined correctly; It's time to enter the output definition. First, we exit the input definition process and select the menu entry (#2) to define outputs.  Output definition is similar to input definition, as shown below.

Do you wish to (A)dd INPUTS, (D)elete INPUTS, or (Q)uit? >Q

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >2

The current Knowledge Base is aware of the following OUTPUTS:

Output #1 ----> ~

Output #2 ----> ~

Output #3 ----> ~

Output #4 ----> ~

Do you wish to (A)dd OUTPUTS, (D)elete OUTPUTS, or (Q)uit? >A

What is the NAME of this output? Output #1 > >Motor_Current

What UNITS(MPH, DegC, PPM, etc..) are used to describe it? >Mill_Amp

What is the MINIMUM value that this Output may assume? >-18

What is the MAXIMUM value that this Output may assume? >18

The current Knowledge Base is aware of the following OUTPUTS:

Output #1 ----> Motor_Current

Output #2 ----> ~

Output #3 ----> ~

Output #4 ----> ~

Do you wish to (A)dd OUTPUTS, (D)elete OUTPUTS, or (Q)uit? >Q

Now both inputs and outputs are defined. In this example we defined inputs first, then outputs. We could have defined outputs first as the order is not relevant.

It is time to define Membership Functions for each of the inputs and the output. The order is irrelevant; you may define output Membership Functions first, then input Membership Functions. In this case, we define Membership Functions in the same order as the menu selections, inputs first.

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >3

The current Knowledge Base is aware of the following INPUTS:

Input #1 ----> Angle

Input #2 ----> Delta_Angle

Input #3 ----> ~

Input #4 ----> ~

Input #5 ----> ~

Input #6 ----> ~

Input #7 ----> ~

Input #8 ----> ~

To which INPUT (number) would you like to add FUNCTION members? >1

We start with Input #1 (Angle); it would be equally correct to begin with Input #2 (Delta_Angle). The Membership Functions may be defined in any order. In this case, we start with the left hand side (-90 degrees) of the Membership Function graph and work toward the right side (+90 degrees).

As with the input and output definition, KBG_11B shows, at each step, the current state of Membership definition. As we begin, there are no Membership Functions defined. Each new definition is added to the list, then the list is re-displayed as verification.

Input #1

INPUT NAME   	Minimum Value	Maximum Value	Units

Angle		-90			90		Degrees

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	               ~	      0	 0	 0	 0

2	               ~	      0	 0	 0	 0

3	               ~	      0	 0	 0	 0

4	               ~	      0	 0	 0	 0

5	               ~	      0	 0	 0	 0

6	               ~	      0	 0	 0  	 0

7	               ~	      0	 0	 0	 0

8	               ~	      0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

What is the NAME of this function? Function #1 > >Negative_Large

Input Membership is defined by four points of inflection.

Points P1, P2, P3, P4 must monotonically increase.

Enter P1: >-90

Enter P2: >-90

Enter P3: >-54

Enter P4: >-36

Input #1

INPUT NAME   	Minimum Value	Maximum Value	Units

Angle		-90			90		Degrees

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	  Negative_Large		-90	-90	-54	-36

2	               ~	      0	 0	 0	 0

3	               ~	      0	 0	 0	 0

4	               ~	      0	 0	 0	 0

5	               ~	      0	 0	 0	 0

6	               ~	      0	 0	 0	 0

7	               ~	      0	 0	 0	 0

8	               ~	      0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

What is the NAME of this function? Function #2 > >Negative_Medium

Input Membership is defined by four points of inflection.

Points P1, P2, P3, P4 must monotonically increase.

Enter P1: >-54

Enter P2: >-36

Enter P3: >-36

Enter P4: >-18

Input #1

INPUT NAME   	Minimum Value	Maximum Value	Units

Angle	     -90	           90	     Degrees

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	  Negative_Large		-90	-90	-54	-36

2	 Negative_Medium		-54	-36	-36	-18

3	               ~	      0	 0	 0	 0

4	               ~	      0	 0	 0	 0

5	               ~	      0	 0	 0	 0

6	               ~	      0	 0	 0	 0

7	               ~	      0	 0	 0	 0

8	               ~	      0	 0	 0	 0

We continue, in like manner, adding inputs until all seven Membership Functions have been defined. When that occurs, we see the following data:

Input #1

INPUT NAME   	Minimum Value	Maximum Value	Units

Angle	     -90	            90	      Degrees

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	  Negative_Large		-90	-90	-54	-36

2	 Negative_Medium		-54	-36	-36	-18

3	  Negative_Small		-36	-18	-18	0

4	            Zero		-18	0	 0	18

5	  Positive_Small		0	18	18	36

6	 Positive_Medium		18	36	36	54

7	  Positive_Large		36	54	90	90

8	               ~		0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >Q

Now Input #1 is finished. We move on to Input #2, Delta_Angle, and perform exactly the same process.

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >3

The current Knowledge Base is aware of the following INPUTS:

Input #1 ----> Angle

Input #2 ----> Delta_Angle

Input #3 ----> ~

Input #4 ----> ~

Input #5 ----> ~

Input #6 ----> ~

Input #7 ----> ~

Input #8 ----> ~

To which INPUT (number) would you like to add FUNCTION members? >2

Input #2

INPUT NAME   	Minimum Value	Maximum Value	Units

Delta_Angle	     -90	           90       Deg_Second

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	               ~	       0	 0	 0	 0

2	               ~	       0	 0	 0	 0

3	               ~	       0	 0	 0	 0

4	               ~	       0	 0	 0	 0

5	               ~	       0	 0	 0	 0

6	               ~	       0	 0	 0	 0

7	               ~	       0	 0	 0	 0

8	               ~	       0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

What is the NAME of this function? Function #1 > >Negative_Large

Input Membership is defined by four points of inflection.

Points P1, P2, P3, P4 must monotonically increase.

Enter P1: >-90

Enter P2: >-90

Enter P3: >-72

Enter P4: >-48

Input #2

INPUT NAME   	Minimum Value	Maximum Value	Units

Delta_Angle	     -90	            90	      Deg_Second

Function #	MEMBERSHIP FUNCTION	 P1	P2  	P3	P4

1	  Negative_Large	     -90  -90 -72  -48

2	               ~	       0	 0	 0	 0

3	               ~	       0	 0	 0	 0

4	               ~	       0	 0	 0	 0

5	               ~	       0	 0	 0	 0

6	               ~	       0	 0	 0	 0

7	               ~	       0	 0	 0	 0

8	               ~	       0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

As in the example for Input #1, we continue adding Membership Functions until Input #2 looks like this:

Input #2

INPUT NAME   	Minimum Value	Maximum Value	Units

Delta_Angle	     -90	            90	      Deg_Second

Function #	MEMBERSHIP FUNCTION	P1	P2	P3	P4

1	  Negative_Large	     -90  -90  -72   -48

2	 Negative_Medium	     -72  -48  -48   -24

3	  Negative_Small	     -48  -24  -24	 0

4	            Zero	     -24	 0	 0	24

5	  Positive_Small	       0	24	24	48

6	 Positive_Medium	      24	48	48	72

7	  Positive_Large	      48	72	90	90

8	               ~	       0	 0	 0	 0

(A)dd Membership FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >Q

Now we do the same for the single output.

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >6

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >4

The current Knowledge Base is aware of the following OUTPUTS:

Output #1 ----> Motor_Current

Output #2 ----> ~

Output #3 ----> ~

Output #4 ----> ~

To which OUTPUT (number) would you like to add FUNCTION members? >1

Output #1

OUTPUT NAME    	Minimum Value	Maximum Value	Units

Motor_Current	           -18              18        Mill_Amp

Function #	MEMBERSHIP FUNCTION	P1

1	               ~	       0

2	               ~	       0

3	               ~	       0

4	               ~	       0

5	               ~	       0

6	               ~	       0

7	               ~	       0

8	               ~	       0

Do you wish to (A)dd FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

As we add Membership Functions to this output, notice how outputs differ from the inputs we did previously. Inputs require four data points to describe a trapezoid function. Outputs are singletons; they have only one output value.

What is the NAME of this function? Function #1 > >Negative_Large

Output Membership is defined by one singlton (P1).

Enter P1: >-18

Output #1

OUTPUT NAME    	Minimum Value	Maximum Value	Units

Motor_Current	           -18	            18	      Mill_Amp

Function #	MEMBERSHIP FUNCTION	P1

1	  Negative_Large	     -18

2	               ~	       0

3	               ~	       0

4	               ~	       0

5	               ~	       0

6	               ~	       0

7	               ~	       0

8	               ~	       0

Do you wish to (A)dd FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >A

Once again, we continue until all Membership Functions are complete. The display will appear as below.

Output #1

OUTPUT NAME    	Minimum Value	Maximum Value	Units

Motor_Current	           -18	            18	      Mill_Amp

Function #	MEMBERSHIP FUNCTION	P1

1	  Negative_Large	     -18

2	 Negative_Medium	     -12

3	 Negative_Small	      -6

4	            Zero	       0

5	  Positive_Small	       6

6	 Positive_Medium	      12

7	  Positive_Large	      18

8	               ~	       0

Do you wish to (A)dd FUNCTIONS, (D)elete FUNCTIONS, or (Q)uit? >Q

Now that all input and output data has been entered, we can get to the heart of the matter, the Rules. The Rules and their treatment by the Inference Engine are the prime function of Fuzzy Logic. As the Rules are written, they are parsed, checked for syntax, and entered into the Knowledge Base. The Knowledge Base's Input and Output tables are also searched to find names which appear in a Rule. Thus, Inputs and Outputs must be fully defined before defining a Rule which uses that Input or Output. If not, KBG_11B generates an error message complaining that a name cannot be found. In case of error, no Rule is generated.

Rules have the form:

IF Input1 IS Member1 THEN output1 IS Member2

This may sometimes generate rather stilted and erroneous sounding English as the verb "IS" happens to be singular and your Input or Output name (noun) may be plural. For example, a Rule is  correct if it states:

IF Wheels IS Stopped THEN Gears IS Neutral

Please resist the temptation to use the verb "ARE" in Rules. Even though it is correct in English, "ARE" is not a valid verb in this syntax and the parser will complain bitterly. Rules may also use the connective, "AND", in either the "IF" clause or the "THEN" clause. For example:

IF Speed IS Slow AND Gear IS First

THEN Shift IS No AND Fuel IS Small

The keywords "IF", "IS", "AND", and "THEN" are converted to upper case characters, regardless of which case you type. However, Input, Output, and Membership Function names are not changed. You may mix upper and lower case characters when forming their names. Special characters are also allowed if they are printing characters. The tilde is not permitted to be the FIRST character in a name.

Now, let's write some Rules to balance the Pendulum. We select #5 from the menu, then select (A)dd.

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >5

(A)dd RULES, (D)elete RULES, or (Q)uit ? >A

RULES have the following form:

IF   in IS label1   THEN   out IS label2   (CR)

in is an  INPUT name & label1 is a Membership FUNCTION of  in.

out is an OUTPUT name & label2 is a Membership FUNCTION of out.

'IF' and 'THEN' clauses may be joined with the keyword 'AND'.

Enter rules now. (CR only to exit.)

>if Angle is Negative_Large and Delta_Angle is Zero then Motor_Current is Positive_Large

Enter rules now. (CR only to exit.)

>if Angle is Negative_Medium and Delta_Angle is Zero then Motor_Current is Positive_Medium

Enter rules now. (CR only to exit.)

>if Angle is Negative_Small and Delta_Angle is Zero then Motor_Current is Positive_Small

Enter rules now. (CR only to exit.)

>if Angle is Negative_Small and Delta_Angle is Positive_Small then Motor_Current is Positive_Small

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Negative_Large then Motor_Current is Positive_Large

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Negative_Medium then Motor_Current is Positive_Medium

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Zero then Motor_Current is Zero

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Positive_Small then Motor_Current is Negative_Small

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Positive_Medium then Motor_Current is Negative_Medium

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Positive_Large then Motor_Current is Negative_Large

Enter rules now. (CR only to exit.)

>if Angle is Positive_Small and Delta_Angle is Negative_Small then Motor_Current is Negative_Small

Enter rules now. (CR only to exit.)

>if Angle is Positive_Small and Delta_Angle is Zero then Motor_Current is Negative_Small

Enter rules now. (CR only to exit.)

>IF Angle is Positive_Medium AND Delta_Angle is Zero THEN Motor_Current is Negative_Medium

Enter rules now. (CR only to exit.)

>if Angle iS Positive_Large And Delta_Angle is Zero THEn Motor_Current is Negative_Large

Enter rules now. (CR only to exit.)

>

(A)dd RULES, (D)elete RULES, or (Q)uit ? >q

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >6

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >4

Output to (S)creen or to (F)ile? >s

Input #1

INPUT NAME   	Minimum Value	Maximum Value	Units

Angle	     -90	           90	     Degrees

MEMBERSHIP FUNCTION	 P1	 P2 	P3	 P4

Negative_Large	-90	-90	-54	-36

Negative_Medium	-54	-36	-36	-18

Negative_Small	-36	-18	-18	  0

Zero	-18	  0	  0	 18

Positive_Small	  0	 18	 18	 36

Positive_Medium	 18	 36	 36	 54

Positive_Large	 36	 54	 90	 90

~	  0	  0	  0	 0

Input #2

INPUT NAME   	Minimum Value	Maximum Value	Units

Delta_Angle	     -90             90       Deg_Second

MEMBERSHIP FUNCTION		P1	P2	P3	P4

Negative_Large		-90	-90	-72	-48

Negative_Medium		-72	-48	-48	-24

Negative_Small		-48	-24	-24	0

Zero		-24	0	0	24

Positive_Small		0	24	24	48

Positive_Medium		24	48	48	72

Positive_Large		48	72	90	90

~		0	0	0	0

Output #1

OUTPUT NAME  	Minimum Value	Maximum Value	Units

Motor_Current	     -18     	      18	     Mill_Amp

MEMBERSHIP FUNCTION	      P1

Negative_Large	     -18

Negative_Medium	     -12

Negative_Small	      -6

Zero	       0

Positive_Small	       6

Positive_Medium	      12

Positive_Large	      18

~	       0

Rule #1

IF      Angle IS Negative_Large

AND Delta_Angle IS Zero

THEN    Motor_Current IS Positive_Large

Rule #2

IF      Angle IS Negative_Medium

AND Delta_Angle IS Zero

THEN    Motor_Current IS Positive_Medium

Rule #3

IF      Angle IS Negative_Small

AND Delta_Angle IS Zero

THEN    Motor_Current IS Positive_Small

Rule #4

IF      Angle IS Negative_Small

AND Delta_Angle IS Positive_Small

THEN    Motor_Current IS Positive_Small

Rule #5

IF      Angle IS Zero

AND Delta_Angle IS Negative_Large

THEN    Motor_Current IS Positive_Large

Rule #6

IF      Angle IS Zero

AND Delta_Angle IS Negative_Medium

THEN    Motor_Current IS Positive_Medium

Rule #7

IF      Angle IS Zero

AND Delta_Angle IS Zero

THEN    Motor_Current IS Zero

Rule #8

IF      Angle IS Zero

AND Delta_Angle IS Positive_Small

THEN    Motor_Current IS Negative_Small

Rule #9

IF      Angle IS Zero

AND Delta_Angle IS Positive_Medium

THEN    Motor_Current IS Negative_Medium

Rule #10

IF      Angle IS Zero

AND Delta_Angle IS Positive_Large

THEN    Motor_Current IS Negative_Large

Rule #11

IF      Angle IS Positive_Small

AND Delta_Angle IS Negative_Small

THEN    Motor_Current IS Negative_Small

Rule #12

IF      Angle IS Positive_Small

AND Delta_Angle IS Zero

THEN    Motor_Current IS Negative_Small

Rule #13

IF      Angle IS Positive_Medium

AND Delta_Angle IS Zero

THEN    Motor_Current IS Negative_Medium

Rule #14

IF      Angle IS Positive_Large

AND Delta_Angle IS Zero

THEN    Motor_Current IS Negative_Large

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >2

Save Knowledge Base to File <balance.knb> (CR to accept) >

Saving Knowledge Base to File <balance.knb>

Whoops! We forgot to enter one Rule. Now we go back to Data Menu and enter the Rule. The order that Rules are entered are irrelevant as the Inference Engine evaluates all Rules as if they occurred at the same time.

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >3

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >5

(A)dd RULES, (D)elete RULES, or (Q)uit ? >A

RULES have the following form:

IF   in IS label1   THEN   out IS label2   (CR)

in is an  INPUT name & label1 is a Membership FUNCTION of  in.

out is an OUTPUT name & label2 is a Membership FUNCTION of out.

'IF' and 'THEN' clauses may be joined with the keyword 'AND'.

Enter rules now. (CR only to exit.)

>if Angle is Zero and Delta_Angle is Negative_Small then Motor_Current is Positive_Small

Enter rules now. (CR only to exit.)

>

(A)dd RULES, (D)elete RULES, or (Q)uit ? >Q

KBG_11B Data Entry Menu.

1) Specify INPUT signal names.

2) Specify OUTPUT signal names.

3) Specify INPUT Membership Functions.

4) Specify OUTPUT Membership Functions.

5) Specify RULES.

6) Exit to Main Menu.

Enter your selection: >6

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >6

The example is complete. Now we exit KBG_11B.

The current Knowledge Base has not been saved. Save it? >y

Whoops! Entry of the last Rule changed the Knowledge Base and we forgot to save it. KBG_11B keeps track of changes of the Knowledge Base for us. We answer (y)es and proceed to exit KBG_11B.

Save Knowledge Base to File <balance.knb> (CR to accept) >

Saving Knowledge Base to File <balance.knb>

Thank you for using KBG_11B.

At this point, there exists a file called balance.knb. This file contains all the Knowledge Base data that we have just entered. This file remains the "source code" for our Knowledge Base. However, this is not the data required by the Inference Engine. The data must be scaled and encoded into constant bytes for the Inference Engine. Motorola assemblers require constant byte data to be coded as "FCB dd", where dd is one byte of data. This is the last and most important function that KBG_11B must accomplish. When all edits to the Knowledge Base, balance.knb, are complete, we execute KBG_11B one final time.

C:> kBG_11B balance

MC68HC11 Fuzzy Logic Preprocessor V1.1

Copyright Motorola Inc. 1991,1992.

Author: John Dumas

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >1

Load Knowledge Base from balance.knb (CR to accept) >

Loading Knowledge Base from File <balance.knb>

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >5

Now we select menu item #5 to create the FCB data base for the Inference Engine.

Create Output File <balance.asm> (CR to accept) >

Creating Assembly File <balance.asm>

KBG_11B Main Menu.

1) Load a (previously created) Knowledge Base.

2) Save the current Knowledge Base.

3) Enter new data to current Knowledge Base.

4) Show the current Knowledge Base.

5) Create .ASM file.

6) Exit to OS.

Enter your selection: >6

Thank you for using KBG_11B.

At last, the task has been accomplished! KBG_11B has created a file called balance.asm. This file contains the data needed by the Inference Engine. Let's look at the file. Notice that there is data with the value of zero generated for all Inputs, Outputs, or Membership Functions that you did not define.

C:> type balance.asm

*	KBG_11B Preprocessor Version V1.1

*	Author: John Dumas

*	Copyright Motorola 1991,1992

*	File Name = balance.asm

INPUT_MFS	EQU	*	Input Membership Functions.

IN0MF	EQU	*	           Angle

FCB	$00,$00,$33,$0A	  Negative_Large

FCB	$33,$0A,$4C,$0A	 Negative_Medium

FCB	$4C,$0A,$66,$0A	  Negative_Small

FCB	$66,$0A,$7F,$0A	            Zero

FCB	$7F,$0A,$99,$0A	  Positive_Small

FCB	$99,$0A,$B2,$0A	 Positive_Medium

FCB	$B2,$0A,$FF,$00	  Positive_Large

FCB	$7F,$00,$7F,$00	               ~

IN1MF	EQU	*	     Delta_Angle

FCB	$00,$00,$19,$07	  Negative_Large

FCB	$19,$07,$3B,$07	 Negative_Medium

FCB	$3B,$07,$5D,$07	  Negative_Small

FCB	$5D,$07,$7F,$07	            Zero

FCB	$7F,$07,$A1,$07	  Positive_Small

FCB	$A1,$07,$C3,$07	 Positive_Medium

FCB	$C3,$07,$FF,$00	  Positive_Large

FCB	$7F,$00,$7F,$00	               ~

IN2MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

IN3MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

IN4MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

IN5MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

IN6MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

IN7MF	EQU	*	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

FCB	$0,$0,$0,$0	               ~

SGLTN_POS	EQU	*	Output Membership Functions.

OUT0MF	EQU	*	   Motor_Current

FCB	$00		  Negative_Large

FCB	$2A		 Negative_Medium

FCB	$55		  Negative_Small

FCB	$7F		            Zero

FCB	$AA		  Positive_Small

FCB	$D4		 Positive_Medium

FCB	$FF		  Positive_Large

FCB	$7F		               ~

OUT1MF	EQU	*	               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

OUT2MF	EQU	*	               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

OUT3MF	EQU	*	               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

FCB	$0		               ~

RULE_START EQU	*

FCB	$0

FCB	$19

FCB	$86

FCB	$8

FCB	$19

FCB	$85

FCB	$10

FCB	$19

FCB	$84

FCB	$10

FCB	$21

FCB	$84

FCB	$18

FCB	$1

FCB	$86

FCB	$18

FCB	$9

FCB	$85

FCB	$18

FCB	$19

FCB	$83

FCB	$18

FCB	$21

FCB	$82

FCB	$18

FCB	$29

FCB	$81

FCB	$18

FCB	$31

FCB	$80

FCB	$20

FCB	$11

FCB	$82

FCB	$20

FCB	$19

FCB	$82

FCB	$28

FCB	$19

FCB	$81

FCB	$30

FCB	$19

FCB	$80

FCB	$18

FCB	$11

FCB	$84

END_OF_RULE FCB $FF

This concludes the use of KBG_11B to create the Knowledge base. We now have a file, balance.asm, which contains the knowledge needed by the Inference Engine. The file "balance.asm" must now be merged with the Source code for the Inference Engine "fuzzy11b.asm" to create a fuzzy logic processor. For this example, we assume that the code will be loaded into a M68HC11EVBU board for debugging. This board has user RAM beginning at $0100. We use this area to load the Knowledge Base. EEPROM exists beginning at $B600. We use this area to load the Inference Engine.

In order to create a complete application, we need a header file to tell the assembler where the code segments are to be located. This file is called "header1.asm". Let's look it:

C:> type header1.asm

*	Header file for the M68HC11EVBU.	Jan 20 1992

KNOWBASE		EQU		$0100	Start of User RAM.

ENGINE		EQU		$B600	Start of User EEPROM.

ORG		KNOWBASE

********

*	Any other code for your application goes here.

*	The Knowledge Base is added below this code.

********

The final source code for the full application will reside in a file called "fuzz_app.asm". We begin creation of this file with the following DOS commands:

C:> copy header1.asm fuzz_app.asm

C:> type balance.asm >>fuzz_app.asm

The next thing to add is the engine. It is contained in a file called "fuzzy11b.asm". Let's look at it:

C:> type fuzzy11b.asm

*	Fuzzy Logic Inferrence Engine

*

** This is the second variation of this fuzzy engine. The first used

*  a macro and conditional assembly for entry of rules and MFs. It

*  also combined fuzzification with rule evaluation. This version

*  is more efficient because fuzzy ins are only calculated once.

*  Also a c program was written to simplify rule and MF entry rather

*  than a macro and conditional assembly which were not very

*  portable from one assembler to another. This version uses fixed

*  data structures for 4 ins and 2 outs but rule format allows easy

*  expansion to 8 ins & 4 outs and could be expanded further.

*

*** Data structures and variables

*

CURRENT_INS RMB     4       ;Storage for 4 8-bit inputs

FUZ_INS     RMB     32      ;Storage for fuzzy inputs

FUZ_OUTS    RMB     16      ;Storage for fuzzy outputs

COG_OUTS    RMB     2       ;Defuzzified outputs

SUM_OF_FUZ  RMB     2       ;11-bit sum of fuzzy outs

SUM_OF_PROD RMB     3       ;19-bit sum of products

COGDEX      RMB     1       ;Current out # for COG loop

LP_COUNT    RMB     1       ;Index for fuz loop

SUMDEX      RMB     1       ;Index for sum loop

*

* Input membership functions are defined by four 8-bit values per

* input label. Up to 8 labels per input so data structure is

* 4*8*4=128 bytes.

* Membership functions are trapezoids with the base greater than or

* equal to the top. Values are entered into this program as four

* constants representing 2 points and 2 slopes. A separate c

* program is used to translate natural language into FCBs.

*

******************REPLACE THIS ENTIRE COMMENT BLOCK WITH OUTPUT*********** ******************FILE FROM THE KNOWLEDGE BASE GENERATOR****************** *

* For now all MFs are filled with $FF. Use c tool to generate required FCBs *

*INPUT_MFS   EQU     *                    ;Input membership functions *

*IN0MF       EQU     *                    ;(2) TEMPERATURE

*            FCB     $FF,$FF,$FF,$FF      ;    (0) COLD

*            FCB     $FF,$FF,$FF,$FF      ;    (1) COOL

*            FCB     $FF,$FF,$FF,$FF      ;    (2) NORM

*            FCB     $FF,$FF,$FF,$FF      ;    (3) WARM

*            FCB     $FF,$FF,$FF,$FF      ;    (4) HOT

*            FCB     $FF,$FF,$FF,$FF      ;    (5) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (6) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (7) Unused

*

*IN1MF       EQU     *                    ;(1) EXAMPLE1

*            FCB     $FF,$FF,$FF,$FF      ;    (0) TEST1

*            FCB     $FF,$FF,$FF,$FF      ;    (1) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (2) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (3) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (4) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (5) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (6) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (7) Unused

*IN2MF       EQU     *                    ;(0) ROTATION

*            FCB     $FF,$FF,$FF,$FF      ;    (0) STOPPED

*            FCB     $FF,$FF,$FF,$FF      ;    (1) NOT_STOPPED

*            FCB     $FF,$FF,$FF,$FF      ;    (2) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (3) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (4) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (5) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (6) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (7) Unused

*

*IN3MF       EQU     *                    ;(3) DAYS_SINCE_RAIN

*            FCB     $FF,$FF,$FF,$FF      ;    (0) NONE

*            FCB     $FF,$FF,$FF,$FF      ;    (1) SHORT

*            FCB     $FF,$FF,$FF,$FF      ;    (2) MEDIUM

*            FCB     $FF,$FF,$FF,$FF      ;    (3) LONG

*            FCB     $FF,$FF,$FF,$FF      ;    (4) VERY_LONG

*            FCB     $FF,$FF,$FF,$FF      ;    (5) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (6) Unused

*            FCB     $FF,$FF,$FF,$FF      ;    (7) Unused

*

*****

*SGLTN_POS   EQU     *      ;Output singleton positions

*

*OUT0MF      EQU     *      ;(0) WATERING_TIME

*            FCB     $FF    ;    (0) CUT_OFF

*            FCB     $FF    ;    (1) DECREASE_GREATLY

*            FCB     $FF    ;    (2) DECREASE

*            FCB     $FF    ;    (3) NORMAL

*            FCB     $FF    ;    (4) INCREASE

*            FCB     $FF    ;    (5) INCREASE_GREATLY

*            FCB     $FF    ;    Unused

*            FCB     $FF    ;    Must fill 8 per output

*OUT1MF      FCB     $FF,$FF,$FF,$FF,$FF,$FF,$FF,$FF ;Unused

*

*

* Each If part is of the form 000X XAAA where XX is an input (0-3)

* and AAA is a label (0-7) of that input. If parts are connected by

* ANDs. A rule may have any number of if parts (usually 2-4). Then

* parts are of the form 1000 YCCC where the MSB set indicates a

* then part, Y is the output number (0-1), and CCC is the output

* label (singleton 0-7). A rule may have any number of then parts

* (usually 1 or 2).

* A $FF indicates the end of a series of rules (number not limited).

*

*	FCB	8*INx + LABa	If input XX is label AAA

*	FCB	$80+8*OUTy+LABc	Then output Y is label CCC

*

*RULE_START  EQU     *                   Start of first rule

* Example rule: If TEMPERATURE is VERY_HOT and DAYS_SINCE_RAIN is LONG * Then WATERING_TIME is INCREASE_GREATLY

*RULE_1      FCB     (8*2)+4,(8*3)+3,$80+(8*0)+5

*END_OF_RULE FCB     $FF

*********END OF REPLACED TEXT

ORG	ENGINE

***** Fuzzy Inferrence Engine Starts Here

*

FUZZIFY     LDY     #FUZ_INS      ;Point at fuzzy input table

LDX     #CURRENT_INS  ;Point at current input values

LDD     2,X           ;Put input values on stack

PSHB

PSHA

LDD     0,X

PSHB

PSHA

LDX     #INPUT_MFS    ;Point at input MF specs

NXTIN_LP    PULA                  ;Get next current input value

LDAB    #7            ;For 8 loop passes

STAB    LP_COUNT      ;Initialize loop counter

GRAD_LOOP   EQU     *             ;Get grade for 1 label of 1 input

*******************************************************************

* GET_GRADE - Routine to project a discrete input value onto      *

*   the associated input membership function (fuzzification).     *

*   Enter with Input value in A register and X pointing at the    *

*   points and slopes defining the membership function.           *

*   Finishes with grade in B, X points at next MF spec (X+4)      *

*   and A is unchanged.                                           *

*******************************************************************

GET_GRADE   PSHA                  ;Save input value of A

CLRB                  ;In case grade = 0

SUBA    2,X           ;Input value ─ pt2 -> A

BLS     NOT_SEG2      ;If input < pt2

LDAB    3,X           ;Slope 2

BEQ     HAV_GRAD      ;Skip if vert slope

MUL                   ;(In ─ pt1) * slp2 -> A:B

TSTA                  ;Check for > $FF

BEQ     NO_FIX        ;If upper 8 = 0

CLRB                  ;Limit grade to 0

BRA     HAV_GRAD      ;In limit region of seg 2

NO_FIX      SUBB    #$FF          ;B ─ $FF

NEGB                  ;$FF ─ B

BRA     HAV_GRAD      ;($FF ─((In ─ pt2) * slp2))

NOT_SEG2    ADDA    2,X           ;Restore input value

SUBA    0,X           ;Input value ─ pt1 -> A

BLO     HAV_GRAD      ;In < pt1 so grade = 0

LDAB    1,X           ;Slope 1

BEQ     VERT_SLP      ;Skip if vert slope

MUL                   ;(In ─ pt1) * slp1 -> A:B

TSTA                  ;Check for > $FF

BEQ     HAV_GRAD      ;Result OK in B

VERT_SLP    LDAB    #$FF          ;Limit region or vert slope

HAV_GRAD    INX                   ;Point at next MF spec

INX

INX

INX

PULA                  ;Restore A register

STAB    0,Y           ;Save @ fuzzy input

INY                   ;Point at next

DEC     LP_COUNT      ;

BPL     GRAD_LOOP     ;For 8 labels of 1 input

CMPY    #FUZ_INS+32   ;Done with all fuzzy ins?

BNE     NXTIN_LP      ;If not, process next input

* Done with fuzzification, evaluate rules next

LDX     #FUZ_OUTS     ;Point at first fuzzy output

LDAA    #16           ;16 fuzzy outputs

CLR_OUTS    CLR     0,X           ;Clear a fuzzy output

INX                   ;Point at next

DECA                  ;Loop index

BNE     CLR_OUTS      ;Continue till all fuzzy outs 0

LDY     #RULE_START   ;Point to start of 1st rule

RULE_TOP    LDAA    #$FF          ;Begin processing a rule string

* A will hold grade of smallest (min) if part

IF_LOOP     LDAB    0,Y           ;Get rule byte 000X XAAA; If X is A             BMI     THEN_LOOP     ;If MSB=1, exit to then loop

INY                   ;Point at next rule byte

LDX     #FUZ_INS      ;Point at fuzzy inputs

ABX                   ;Point to specific fuzzy input

CMPA    0,X           ;Is this fuzzy input lower?

BHS     IF_LOOP       ;If not don't replace lowest

LDAA    0,X           ;Replace lowest if

BNE     IF_LOOP       ;Unless zero, do next rule byte

* A zero value fuzzy input makes rule completely not true

* so skip rest of if parts and all then parts to start of next rule

FIND_THEN   LDAB    0,Y           ;Get next rule byte

BMI     FIND_IF       ;MSB set means its a then part

INY                   ;Point at next rule byte

BRA     FIND_THEN     ;Loop till pointing at a then part

FIND_IF     INY                   ;Adv rule pointer to if part

LDAB    0,Y           ;Get next rule byte

BPL     RULE_TOP      ;MSB clear means its an if part

CMPB    #$FF          ;$FF is no more rules marker

BNE     FIND_IF       ;Continue looking for if or $FF

BRA     DEFUZ         ;When all rules done, go defuzzify

THEN_LOOP   LDX     #FUZ_OUTS     ;Point at fuzzy outputs

ANDB    #$0F          ;Save 8 times out # + label #

ABX                   ;X points at fuzzy output

* Grade of membership for rule is in A accumulator

CMPA    0,X           ;Compare to fuzzy output

BLO     NOT_HIER      ;Branch if not higher

STAA    0,X           ;Grade is higher so update

NOT_HIER    INY                   ;Adv rule pointer to next byte

LDAB    0,Y           ;Get rule byte

BPL     RULE_TOP      ;If MSB=0 its a new rule

CHK_END     CMPB    #$FF          ;Check for end of rules flag

BNE     THEN_LOOP     ;If not $FF, must be a then part

* All rules evaluated. Now defuzzify fuzzy outputs

DEFUZ       LDY     #SGLTN_POS    ;Point at 1st output singleton

LDX     #FUZ_OUTS     ;Point at 1st fuzzy output

CLR     COGDEX        ;Loop index will run from 0->1

COG_LOOP    LDAB    #8            ;8 fuzzy outs per COG output

STAB    SUMDEX        ;Inner loop runs 8->0

LDD     #$0000        ;Used for quicker clears

STD     SUM_OF_FUZ    ;Sum of fuzzy outputs

STD     SUM_OF_PROD+1 ;Low 16-bits of sum of products

STAA    SUM_OF_PROD   ;Upper 8-bits

SUM_LOOP    LDAB    0,X           ;Get a fuzzy output

CLRA                  ;Clear upper 8-bits

ADDD    SUM_OF_FUZ    ;Add to sum of fuzzy outputs

STD     SUM_OF_FUZ    ;Update RAM variable

LDAA    0,X           ;Get fuzzy output again

LDAB    0,Y           ;Get Output singleton position

MUL                   ;Position times weight

ADDD    SUM_OF_PROD+1 ;Low 16-bits of sum of products

STD     SUM_OF_PROD+1 ;Update low 16-bits

LDAA    SUM_OF_PROD   ;Upper 8-bits

ADCA    #0            ;Add carry from 16-bit add

STAA    SUM_OF_PROD   ;Upper 8-bits of 24-bit sum

INY                   ;Point at next singleton pos.

INX                   ;Point at next fuzzy output

DEC     SUMDEX        ;Inner loop index

BNE     SUM_LOOP      ;For all labels this output

PSHX                  ;Save index for now

CLRA                  ;In case divide by zero

LDX     SUM_OF_FUZ    ;Demominator for divide

BEQ     SAV_OUT       ;Branch if denominator is 0

TST     SUM_OF_PROD   ;See if more than 16-bit

BNE     NUM_BIG       ;If not zero, # is > 16-bits

LDD     SUM_OF_PROD+1 ;Numerator for divide

IDIV                  ;Result in low 8-bits of X

XGDX                  ;Result now in B

TBA                   ;Move result to A

BRA     SAV_OUT       ;Go save output

NUM_BIG     LDD     SUM_OF_PROD   ;Numerator upper 16 of 24-bit

TST     SUM_OF_PROD+2 ;Check for rounding error

BPL     NO_ROUND      ;If MSB clear, don't round

ADDD    #1            ;Round numerator up 1

NO_ROUND    FDIV                  ;D/X -> X, use upper 8 of 16

XGDX                  ;Result now in A

SAV_OUT     LDX     #COG_OUTS     ;Point to 1st defuz output

LDAB    COGDEX        ;Curent output number

ABX                   ;Point to correct output

STAA    0,X           ;Update defuzzified output

PULX                  ;Recover index

INCB                  ;Increment loop index

STAB    COGDEX        ;Update

CMPB    #2            ;Done with two outs?

BNE     COG_LOOP      ;If not, continue loop

*****

* Inference engine has completed one pass of all rules.

*****

Note the line containing the "ORG ENGINE" statement. We edited this line before beginning the application. Now we add this file to the application with the following DOS command:

C:> type fuzzy11b.asm >>fuzz_app.lst

Now the Knowledge Base, Inference Engine, and other user code are in a single file. The following DOS command assembles the application. This command line assumes the use of the Freeware assembler, AS11.EXE. When executed, AS11.EXE assembles the entire file generating a listing, fuzz_app.lst, and an S-record file, fuzz_app.s19.

C:> as11 -l fuzz_app.asm >fuzz_app.lst

The file, fuzz_app.s19 contains the object code for the application. It is encoded into Motorola S-record format. The data represented by the contents of fuzz_app.s19 may now programmed into PROM or downloaded into a development system to be debugged. Bugs in the non-fuzzy part of the application are fixed in the conventional way. Bugs in the fuzzy part of the applications are fixed by modifying the Knowledge Base with kBG_11B. In either case, the process consists of edit, assemble, and debug. The process is repeated until no bugs remain.

You must NOT make changes to the Inference Engine unless you are VERY sure that you know what you are doing. The program, kBG_11B, makes a number of assumptions about how the Inference Engine works and the data format that it expects. If you change the Engine, the Knowledge Base may no longer work with your version of the Engine.