NAME

readchar -- read a character from serial port

ready_rcv -- check for character availability

ready_xmt -- check if port can accept a char to send

setport -- set port configuration

writechar -- send a char to the serial port

SYNOPSIS

r = readchar(port);

x = ready_rcv(port);

x = ready_xmt(port);

(void)setport(port, r);

(void)writechar(port, r);

unsigned char r;   character to send or receive

int port;          port number

int x;             TRUE if condition exists, else FALSE (0)

DESCRIPTION

These are very low level (direct port access) subroutines

to enable I/O through COM1 through COM4.  Only setport() uses

BIOS int 14H.  These functions are not intended to be the

ultimate in async functions, but a starting point for an

integrated set of async functions, and a means of getting at

the serial port directly for speed and to work around the

problems that some "clone" BIOS routines have in returning

proper status values.  File "smdefs.h" contains #defines

to handle these ports mnemonically.

The value to use for the setport() function is an 8-bit

value, bitwise organized as follows:

bits 1,0:     10 = 7-bit data, 11 = 8-bit data

bit  2:        0 = 1 stop bit, 1 = 2 stop bits

bits 4,3:     x0 = No Parity, 01 = odd, 11 = even

bits 7,6,5:   baud rate:

000 = 110 baud (use 2 stop bits)

001 = 150 baud

010 = 300 baud

011 = 600 baud

100 = 1200 baud

101 = 2400 baud

110 = 4800 baud

111 = 9600 baud

ready_rcv() returns a TRUE condition if a character is

waiting to be read, but no error conditions are returned.

ready_xmt() returns a TRUE condition if the transmit

holding register is empty; i.e., the transmitter can accept

a character to send.  In both these functions, the status

returned is a result of reading the port status and masking

all but the relevant bit.  These routines will NOT return

errors such as overrun or parity.

async functions,      page 2

readchar() will return the character waiting in the receiver

register of the port.  It is returned as an unsigned char

to facilitate 8-bit binary transfer protocols.

writechar() will place the specified unsigned char into the

transmitter buffer register where it will be sent.

If the program "loop" is short and fast enough, these functions

will be ample to allow communications at up to 9600 baud.

A program has already been implemented to communicate with a

prom programmer at this rate, so it can be done.  However,

when the program does not poll the receiver fast enough it

is likely that characters may be lost and the maximum usable

baud rate will be lower.  It is highly unlikely that baud rates

less than 2400 baud will be necessary if the program is properly

structured.

EXAMPLE

/* this routine receives characters from COM1 at one

** speed and resends them to COM2 at another speed */

unsigned char i;

setport(SER1, 0xe3);  /* set COM1 to 9600 baud, No parity,

** 8-bits, 1 stop bit */

setport(SER2, 0xc3);  /* set COM2 the same, except 4800 baud */

while(TRUE) {

if(!ready_rcv(SER1)) continue;  /* wait for a char */

i = readchar(SER1);     /* fetch it */

while(!ready_xmt(SER2));/* wait for transmitter to be ready */

writechar(SER2, i);  /* then send it */

}

This function is found in SMDLx.LIB for the Datalight Compiler.