Accessing Extended Memory

from Turbo C and Turbo Pascal

"Extended memory" refers to the memory at addresses above

the 1 Megabyte that DOS can handle.  It is only available on the

AT and compatibles and the XT-286.  The PC and the XT do not have

extended memory.

In order to access extended memory, you must first determine

that there is, in fact, extended memory present, and the amount

of extended memory present.  This is done through BIOS interrupt

15H, function 88H.  On return, the carry flag will be set if an

error occurred (for instance, if the program is running on a PC,

where function 88H isn't supported).  If the carry flag is

cleared on return, then register AX contains the number of

kilobytes of extended memory present in the system.

One caution on detecting extended memory: technically, as I

understand it, your program should also check that the switches

on the extended memory board have been set so that the board will

back-fill main memory out to 640K, and only use the remainder for

extended memory.  I don't know what happens if you haven't set

the switches this way, and I haven't been able to get the test

for the switch settings to work.  Anybody who wants to poke into

VDISK.LST and sort it out is welcome to.

Once you've determined the amount of extended memory

present, you still have to figure out if any other program (such

as a RAMDISK) is using extended memory.  There is no system-level

memory manager for extended memory, so it's up to each program to

talk to any other programs and sort out their own allocations.

Ordinarily, all you'll have to worry about is device drivers like

VDISK.  In fact, the only program I know how to detect is VDISK.

When VDISK is installed, it takes over interrupt 19H, which

is invoked during a boot to load MS-DOS from your boot disk.  The

only reason it does this is so that later installations of VDISK

can tell that they're not the first.  The only processing that

VDISK does on an int 19H call is to restore the original

interrupt vector and jump to that routine.

The benefit of redirecting interrupt 19H is that the jump

vector for interrupt 19H can be used to find where VDISK is

installed.  The segment where VDISK is located is found in the

second word of the interrupt 19H vector.  The address of that

word is 0000:0066.  Take the word located at that address as the

segment of VDISK.

To actually detect the presence of VDISK, your program has

to check offset 12H within the segment where VDISK is located.

Beginning at that location VDISK will contain the character

string "VDISK  V3.2".  (Note -- two spaces between VDISK and

V3.2).  If that string isn't present, VDISK isn't installed and

all of extended memory is available to your program.

If VDISK is installed, the address of the first available

location in extended memory is stored at offsets 2CH, 2DH, and

2EH in the segment where VDISK is located.  Just grab those three

bytes and save them somewhere.  If VDISK isn't installed, the

first available location in extended memory is 100000H.

Once you know where available extended memory begins, you

can figure out exactly how much space is available for your

program to use.  Just subtract 100000H from the address of the

first availabe location in extended memory and you'll have the

amount of extended memory already in use.  Subtract this number

from the total size of extended memory (gotten with int 15H,

function 88H), and you'll have the amount of space available.

Now that you know how much extended memory space is

available and where it begins, you're ready to use it.

Unfortunately, all you can do is copy data into it or out of it.

You can't directly access it.  To copy data to or from extended

memory you use interrupt 15H, function 87H.  You call function

87H by setting register AH to 87H, register CX to the number of

words to be moved (maximum of 32768 words), and setting ES:SI to

point to a Global Descriptor Table.

Now, Global Descriptor Tables are extremely important in

programming the 80286 in protected mode.  Here, they're just a

hassle, but it's something we have to go through in order to get

function 87H to work.  First, we have to set up a DESCRIPTOR.  A

DESCRIPTOR describes a segment of memory.  It's laid out like

this:

Offset    # of Bytes     Description

00          2            Maximum size of segment

02          2            Low word of address of segment

04          1            High byte of address

05          1            Access rights byte

06          2            Reserved (must be 00)

The maximum size of the segment will always be less than or

equal to 65536, since that's the largest number of bytes we can

move at a time.  In the demonstration program it's set each time

to the number of bytes to actually be moved, but just leaving it

at 65536 would probably work as well.

Addresses in protected mode are 24 bits long.  They're

treated as a high byte and a low word.  So, for example, the

first byte of extended memory is at address 100000H, which splits

up into a high byte of 10H and a low byte of 0000H.  That's easy.

To convert a DOS-style address (segment:offset), just use 16

times the segment plus the offset.  Strip off the low 8 bytes as

the low word, and the remaining 4 bytes as the high word.  For

example,  5EB4:002C becomes 5EB6C, having a high byte of 05H and

a low word of EB6CH.

For our purposes, the access rights byte will always be set

to 93H, and the reserved area will always be set to 0.

Now, to produce a GDT, we need six of these DESCRIPTORs.  Do

this by rote -- don't expect it to make a lot of sense.  They

should be laid out like this:

Dummy Descriptor

GDT Descriptor

Source Descriptor

Target Descriptor

Code Segment Descriptor

Stack Segment Descriptor

All those descriptors should be initialized to 0, except for

the Source Descriptor and the Target Descriptor.  The Source

Descriptor should be filled in with the appropriate data for the

block to be copied, and the Target Descriptor should be filled in

for the location to be copied to.

Once that's all done, just call interrupt 15H, function 87H.

When it returns, if the Carry flag is cleared, the copy was

successful.

Now, isn't that simple?

Oh, one note concerning Turbo C and interrupt 15H: if you

try to access function 88H through int86() it will always return

with the carry flag set.  That's why the demonstration code uses

geninterrupt() instead of int86().

Concerning the code: the Pascal code will compile and run

under Turbo Pascal version 3.  The C code will compile and run

under Turbo C version 1.0.  The C code is broken up into separate

files as follows:

xmemasm.c      contains the function 88H call.  Must be

compiled under TCC, because it uses inline

assembler code

xmemcopy.c     contains the routine to copy blocks using

interrupt 15H, function 88H

xmemc.c        contains the rest of the support routines

xmemtest.c     contains the main() program and the calls to

the initialization routines

xmemtest.prj   .PRJ file for xmemtest.

To compile the C code, compile xmemasm.c with TCC.  Then use

TC and the project file xmemtest.prj to build xmemtest.exe.

The demonstration programs both do the same thing: they

check the system paramaters and give you some information on the

available extended memory.  Then they initialize a 16384 byte

buffer with recognizable data, copy the buffer into extended

memory, clear the buffer, copy the data back from extended memory

into the buffer, then check the data against the original data.