============================================================

FAQ Version:  updated 95/06/21

"Official" FAQ Maintainer:  Viet-Tam Luu

(vtluu@uwaterloo.ca)

Preface

=======

Hello world!

A few points/thoughts I would like to mention:

- I'd like feedback specifically about whether or

not this FAQ should be split into two parts,

seeing as how it's around 75K in size.  Well?

- The abbreviation "a.b.s.m" has been changed to

"a.b.s.midi" to avoid confusion with the group

alt.binaries.sounds.mods (and possibly others).

- A reminder about the a.b.s.midi FAQ is auto-posted

on alt.binaries.sounds.midi at 23:59 on Sundays,

Tuesdays, and Fridays.

- Terry Law (law@legend.pl.my) has put up his own

HTML'ized version of the FAQ.  Nice job!

- All sections marked with a pound sign (#) are

being written.

- All sections marked with an exclamation mark (!)

have not been completed.

- Remember, this is the *alt.binaries.sounds.midi*

FAQ; in other words, if you're reading this then

this is *your* FAQ...  CONTRIBUTE!  See Appendix B

for further information on how to do this.

Index

=====

Introduction

1.  MIDI

1.1  What is MIDI all about?

1.2  What are "MIDI's"?

! 1.2.1  What is GS and how is it different

from GM?

1.3  What are differences between sound files,

.MOD-type files, and MIDI files?

1.4  Why can't I convert a .WAV file to a MIDI

file?

1.5  What is a .KAR file?

2.  alt.binaries.sounds.midi

2.1  What is alt.binaries.sounds.midi all about?

2.2  How do I decode MIDI files from a.b.s.midi?

2.2.1  Can I get viruses from MIDI files?

2.3  How do I send MIDI files to a.b.s.midi?

2.4  Some guidelines for posting MIDI files

3.  MIDI playback devices

!# 3.1  What's the difference between FM synthesis

and wavetable synthesis?

3.2  MIDI devices

3.2.1  Gravis UltraSound / Ultrasound Max

3.2.2  Turtle Beach Multisound Classic

3.2.3  Turtle Beach Multisound Monterey

3.2.4  Sound Blaster 2.0

3.2.5  Sound Blaster 16

3.2.6  Sound Blaster AWE32

.

.  (Add your favorite synth here)

.

4.  MIDI software

! 4.1  MIDI sequencers

! 4.1.1  Cakewalk / Cakewalk Pro

! 4.1.2  Midisoft Recording Session / Studio

! 4.1.3  WinJammer

.

.  (Add your favorite sequencer here)

.

4.2  MIDISCAN - MIDI OCR Software

4.3  MIDI-file players

4.3.1  Cubic Player

5.  MIDI on the Internet

5.1  What are some FTP sites where I can get MIDI

and MIDI-related files?

5.2  Where and how can I get the official MIDI

spec's?

5.3  Other MIDI-related addresses (WWW, etc.)

Appendix A - Off-line sources of information

Appendix B - Contributing to the a.b.s.midi FAQ

Appendix C - List of contributors

Introduction

============

This is the FAQ (Frequently-Asked Questions) document

for the USENET newsgroup alt.binaries.sounds.midi

(a.b.s.midi).  Its objective is to answer questions

that would be asked repeatedly (and answered) by users

of a.b.s.midi (both new and not-so-new), and thereby

free up the newsgroup to new discussion and MIDI files.

This FAQ document *should* be posted on a weekly basis

on a.b.s.midi.  In addition to this, the most recent

revision may be found at any times at these addresses:

FTP:     ftp.netcom.com /pub/jo/jonin/soundcards/

faqs/absm-faq.zip

Gopher:  (N/A)

WWW:     http://csclub.uwaterloo.ca/~vtluu/

absm_faq.html  (plain-text version)

http://tnt.microimages.com/~dwilliss/

absmfaq.html  (HTML version)

http://www.intac.com/~cully/law/

midifaq.html  (HTML version)

Contributions to this document are welcome (and

needed!).  No credentials whatsoever are needed, though

it might be nice if you know what you are talking about

:-) .  For more information on how to contribute to

this FAQ document, see Appendix B.

Any opinions expressed in this document are solely

those of their respective authors, and do not reflect

in any way the views of the FAQ maintainer (unless if

he is the author), or anybody else affiliated in any

way with a.b.s.midi.

1.  MIDI

========

1.1  What is MIDI all about?

============================

Viet-Tam Luu (vtluu@uwaterloo.ca)

MIDI stands for Musical Instrument Digital Interface.

It is a standard, first published in 1983 by the

International MIDI Association (IMA), that allows

different musical instruments (commonly keyboards, but

also guitars, violins, etc.) and devices (synthesizer

modules, computers, sound cards, etc.) to be connected

together.

The last "I" in MIDI stands for "interface" and neatly

describes what MIDI *is* exactly.  It is a common

interface, largely device-independent, that allows

different devices made by different manufacturers to

communicate with each other.  Nothing more, nothing

less.

MIDI data consists of signals, in the form of a series

of codes or "events" that tell an instrument or synthe-

sizer, "start playing this note at this volume," "stop

playing this note," "play this channel using this

instrument sound," and so on.  Of course it is actually

more complex than this, but essentially the important

notion to grasp is that MIDI data does not describe the

sound of the instruments used in a song (in most cases,

anyways), but rather how those instruments are used

(i.e. played) to form the entire song.

For musicians, MIDI offers many possibilities,

discussion of which would be better left to someone

more qualified.  One example of these "possibilities"

is MIDI recording, which allows a single musician to

compose songs that would otherwise require several

people to play the instruments.  This makes the art of

composing more accessible, so anybody with a decent

MIDI synthesizer or MIDI-equipped computer system has

the tools to put out some fairly impressive work.

For us non-musicians, MIDI is another way to experience

and enjoy music.  Whereas the 1500 or so MIDI files in

my MIDI file collection take up about forty megabytes

of storage, a single digitally-recorded (CD quality)

song may take well over twice this space.

There are two solutions to this storage problem:  use a

medium that can store this massive amount of data in a

compact, manageable form (i.e., DAT's and CD's), or

make the songs in such a way that they take up much

less space (i.e., MIDI files).

Of course, CDDA (CD Digital Audio) and MIDI are two

quite different media, and IMHO should not be compared.

In other words, those who complain that a song in MIDI

format doesn't sound as good as the CD version should

go out and buy the CD.  (Which leads to another

important point:  MIDI files are easily, and in most

cases freely, distributed, while CD's tend to be

comparatively more pricey, and basically impractical

(not to mention illegal) to duplicate.)

MIDI music should be viewed in its own light, not as

cheap approximations "real" music.  Innovations such as

computers, the Internet, etc. have made it possible for

"ordinary" people to express their views, their

feelings and ideas on a worldwide scale; MIDI has done

the same thing for amateur musicians.  As for MIDI

tunes that cover an existing work, it's fine to

sequence a MIDI file to sound as much as possible like

the original tune, but no matter how good the MIDI

artist or the synthesizer, nobody can get a MIDI file

to sing, nor should we expect it to do so.

1.2  What are "MIDI's"?

=======================

Viet-Tam Luu (vtluu@uwaterloo.ca)

"MIDI's" and "a MIDI" are terms that have been coined

up most likely by computer users, referring to what are

formally called "MIDI files".

MIDI files are data files that contain a sequence

(hence "a sequence," a term referring loosely to MIDI

files, and "to sequence", meaning to create a MIDI

file) of MIDI events.  MIDI events are codes that tell

a MIDI device (such as a sound card with MIDI support,

a stand-alone synthesizer module, etc.), for example,

to play a certain note, use a certain instrument sound

to play it, play at a certain tempo, etc.  All these

events, put together, make up a single song or piece;

some MIDI files contain a medley of several songs,

although these are infrequent.

1.3  What are differences between sound files,

.MOD-type files, and MIDI files?

==============================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

Sound files, such as WAV, VOC and AU files, consist of

what is called "waveform data."  Sound is propagated

in sound waves, which are (in simple terms) waves of

variation in air pressure caused by physical phenomena

such as vibrating vocal cords, the vibrations of the

reed as air flows through a flute, aircraft breaking

the sound barrier, or anything else that produces

sound.

Waveform data means a digital representation of those

sound waves.  CD's and DAT's (Digital Audio Tapes)

store digital waveform data.  (Normal audio cassettes

and vinyl records store analog waveform data.)

"Digital" means that the analog sound data are

converted into numbers in the recording process.  In

playback, the reverse occurs:  numbers are translated

back into analog signals used to activate speakers and

thus re-create the original recorded sound.  Thus,

WAV-type files and the like truly "describe" a sound.

It is important to remember that (the greate majority

of) MIDI files do not actually contain any sound data.

MIDI data does not, generally, describe a sound; it

indicates how to play specific sounds (at certain

pitches, with certain volumes, for certain amounts of

time, etc.), in such a way that we can appreciate (or

not appreciate :-) which we call "music."

MOD-type files could be described as a hybrid of MIDI

files and sound files.  By this we mean that they

have characteristics of both MIDI files and sound

files.  MOD-type files incorporate sound data, called

"samples," and control codes that cause these samples

to be played back as music.

The actual structure of these control codes is quite

different from that of MIDI events in MIDI files, but

in both cases they serve to "describe" the song.

Programs exist that will convert MOD-type files to MIDI

files, and vice-versa, with varying degrees of success.

1.4  Why can't I convert a .WAV file to a MIDI file?

====================================================

1.4.1  The short, simple answer

===============================

Viet-Tam Luu (vtluu@uwaterloo.ca)

No program exists that will analyze a .WAV-type file

and "compose" a corresponding MIDI file, for the simple

reason that with current technology,

IT CAN'T BE DONE.

1.4.2  A few more details...

============================

Most "proposed" schemes involve operations such as Fast

Fourier Transforms (FFT's) and wavelet transforms, etc.

These take a .WAV-type file, generally called "waveform

data," and, in the case of the Fourier transform,

breaks it down into its component sinusoidal wave

frequencies and amplitudes. The problem is that none of

these algorithms yield the amount of information nor

precision needed for the job.

The problem is that when, say, the sounds of a hundred

instruments in an orchestra become mixed together and

are recorded as a single sound (.WAV), much of the

information pertaining to each of those specific

instruments is *irreversibly* lost.  Since you can't

get something from nothing, it is impossible to simply

mechanically convert a .WAV back into its component

sounds and then into a MIDI file.

Humans can transcribe .WAV to MIDI because we are

intelligent (or at least we think we are).  This means

that our brains, with all the previously-learned infor-

mation stored in them, can take this scant information

and "fill in the blanks."  For computers to do this is

a similar but much more difficult-to-implement process.

"Difficult" simply because of the sheer complexity of

the algorithms that would be required, and because of

the barely imaginable computing power (that isn't yet

available needed to perform all these operations at a

reasonable rate.

(For a more detailed analysis of the problem, see

section 1.4.3.)

There *is* of course a fairly easy way to *literally*

convert a .WAV file to a MIDI file, with wavetable

devices that can take instrument sounds saved on disk

or such, for example the Gravis UltraSound and the

Sound Blaster AWE32.  One simply converts the .WAV file

into a very large MIDI instrument sound, and then makes

a MIDI file playing back that instrument at the

appropriate frequency.  But what would be the point,

when one might as well keep the .WAV file in the first

place?

1.4.3  The definitive answer

============================

Randy Lynn Tusch (drt@rec1.roc.servtech.com)

My background is in real-time digital audio & video.  I

have many designs which address many related issues

currently in use around the world.  My colleagues and I

have written pattern-matching algorithms for the last

several years and we have established several hard and

fast guidelines to governing this process.  (I'm sorry

if I sound like I'm lecturing...  I just want everyone

to understand the scope of this undertaking.  I hope it

will prove to be helpful.)  The following should help

everyone to see what is involved in any digital

recognition process.

1.  Waveform data CAN be processed to detect OBVIOUS

tempo and rhythm information by looking at the

consistant spikes (quick bursts) of sound where there

are quick attack and decay characteristics.  (such as a

drum beat.)  Where a pattern is distinguishable, a

tempo MIGHT be discernible.  (Any irregularities in the

flow could throw off the timing parameters the computer

may be establishing.)

2.  Any single instrument during a solo could possibly

be detected as a trackable pattern for recognizing

individual notes and chords provided there are no

irregularities about the instrument (such as a variance

in attack or differing harmonics produced during

variations in volume of the individual notes played,

etc.).  The pattern can be filtered out of a simple mix

provided it does not get "lost" in the mix and can be

used to identify individual notes played by THAT

INSTRUMENT (as opposed to all the other instruments in

the mix [The whole point of this process!]) which is

the basis for generating a MIDI file.

NOW...  If this sounds complicated enough, we are about

to get into the REAL issue of the WAV to MIDI

conversion process.  Let's look into what reality

brings into the picture...

1.  Most music is an amalgam of several instruments

(instrument sounds) mixed together to produce the final

effect.

2.  Instrument sounds are rarely ever "dry" in the mix.

(In other words, there is almost inevitably a series of

special effects and signal processing techniques

applied to each instrument or instrument group such as

reverb, flanging, distortion, delay, etc.)

3.  Several types of sounds have sharp attack and decay

characteristics which can be interperated as

"percussive" in nature.

4.  The overall subtleties that create the mood and

"human factor" (non-mechanical feeling) of the

production are the key factors used to set the

velocity, sustain, and decay (note-on, sustain

controller, and note off) and any related information

in the resulting MIDI file.

Now let's look into the ENORMOUS task the recognition

process would have to undergo...

1.  Since the instrument sounds are most often mixed

with other sounds, there is no "automatic" way to tell

the difference between, say, rich strings, a French

horn, and a guitar mixed together.  True, we can

detect the attack characteristic of the French horn as

differing from that of the strings, but at the precise

moment the attack is detected, how many other sounds

are playing?  Since waveform data does not identify

that the sound we are currently hearing consists of,

say, 3 or 10 or 50 different instruments, the

recognition process records THE AMALGAM or MIX of ALL

INSTRUMENT SOUNDS present at that moment AS THE

INSTRUMENT THAT GENERATED THE ATTACK.  After which, the

computer will look for a close match to the amalgam

recorded previously when another attack is detected.

Since that is unlikely to occur, our French horn (with

several other sounds) amalgam (or complex instrument)

will likely not be perceived by the computer again

during the duration of the song.  Instead, the attack

will inevitabley create a recording of a new "complex

instrument" to be identified as yet one more instrument

in the MIDI file.  The overall effect is one or two

notes in the MIDI file for each of hundreds of "complex

instruments" over the duration of the song.  (Hardly

what we're looking for.)

THE SOLUTION?  We, as humans, can identify individual

instruments in a mix of many as a result of a recorded

memory of all the different instrument sounds we are

exposed to over our lifetimes.  When we hear a new

sound, we can usually distinguish it from the other

known instruments by "extrapolation" of the unknown

from the known.  Therefore, we can expand our memory

(database) of instruments dynamically and (hopefully)

perpetually.

1.  For the computer to accomplish the same task, it

has to be given the same database (memory) of

instrument sounds we have acquired (ideally, all

instrument sounds known to mankind).  Additionally, we

have to give it the correct artificial intelligence

programming to allow its database to grow as it

discerns new sounds.

2.  As we may (or may not) be aware, there exists an

exhaustive list of special effects (which grows as

quickly as the list of known instrument sounds) to

further complicate things.  Even given a database of

known instrument sounds, the computer hears a "dry"

guitar (one with no effects- such as distortion- at

all) as being a different instrument from the same

guitar with distortion and reverb effects applied.

Meaning, we now have to expand the computer's database

to include the several billion known effect styles.

Additionally, we have to program it with the ability to

distinguish between new instrument sound and new

effect. (I love that one!)

3.  Given that we are successful up to this point, we

can reward ourselves with the knowledge that we now

have the basis for isolating the sharp attack

characteristic of a drum as being different from the

same type of attack a piano (also a percussive

instrument) will generate.  This does mean, however,

that we must be expecting our percussive instruments to

be the traditional snare, kick (bass) drum, hand clap,

and any other instrument normally used to establish the

overall rhythm or tempo.  If a non traditional approach

is used, we now have more programming to implement.

Fortunately, if it is as simple as a unique sound being

used to establish the meter (such as the finger snap

used in "I'm Gonna Get You Baby" by Bizarre Inc. to

maintain the constant rhythm.)  we can simply identify

to the computer that it is to track that sound and use

it to generate the tempo settings for the MIDI file.

But what if the song is a capella (i.e. a song made up

of purely vocal melody and harmony having no instrument

sounds at all)?  We, then, have no instrument sounds to

lock onto for the purpose of establishing tempo.  Here,

again, we have to do our programming to give the

computer the ability to use the human equivalent of the

intuitive "feel" of the song's tempo.

4.  To be fair, we can use more than strictly attack

and pattern characteristics to detect an instrument.

We could likely use the frequency content and

characteristics of an instrument sound to further

increase the odds of successful recognition.  (This is,

in fact, one of the many additional factors used in any

recognition process to narrow in on and isolate

individual producers of similar patterns, such as two

different people saying the same word.)

5.  Of the most difficult issues to address, subtleties

in the recording are probably the most difficult

qualities to translate into MIDI data. Capturing

controller data proposes the greatest challange to even

a human musician.  Of these, sustain and, possibly,

pitch-bend would be the easiest to handle.  Velocity

information (volume of each note as it is played) is a

highly subjective concept to the listener.  The reason

stems from the differences in each person's hearing

awareness and sensitivity, coupled with the

psychological biasing of the individual's social,

educational, and artisic makeup as well as any

preferences the listener may have.  What a computer

perceives may match the programmer's critera while

being totally foreign to other musicians.  I will leave

the philosophy of this portion of our discussion for

you to ponder at your own convenience.

To conclude this adventure, I must add the following

points and summary:

** We have only discussed four MAJOR aspects of the

recognition process affecting the possibility of WAV to

MIDI conversion.  There are MANY more factors to

consider.  It should be noted that this endeavor makes

even the most extravagant voice recognition project

look like "baby food" by comparison.  (I would like to

acknowledge the success of all those who have produced

successful voice recognition systems...  They have

beaten many odds and proved that the human mind has the

potential to discover ways to overcome nearly any

obstacle if the "user's" own determination and

creativity flows abundantly.)

In summary:

1. The database necessary to store a decent set of

samples of each of the several billion known instrument

sounds and samples of the billions of known special

effects would take up several MILLION TERABYTES of

storage.  (Not to mention the allocation of storage

space necessary to allow the database to grow!)

2.  The number of bytes needing to be evaluated,

compared, and filtered through in any one second of

recorded time (not necessarily real time) again reaches

a value 176400 times greater than the total size of the

database mentioned in (1) above.  (176400 represents

the number of bytes per second in a stereo CD.  You

have 44100 discrete samples of sound per second times 2

bytes per 16-bit sample times 2 channels for stereo

equaling 176400 bytes per second.)  If the database was

already several MILLION terra-bytes in size, this

process would have to work with several BILLION

TERABYTES for every recorded second of material.

3.  Even with supercomputer technology where it is now,

it would take 1000000 of the fastest supercomputers

known to man (like a few Cray Y-MP's and DEC Alpha

500's w/4 processors side by side) SEVERAL WEEKS to

process ONE SECOND of recorded material.

Think of it this way:  If you don't mind spending more

than the US national debt on computer equipment and

waiting a few years for the job to complete, you can

have a system that MIGHT accurately convert the

digital waveform data of a 5 minute song into a small,

compact MIDI file.

Otherwise, you can blow a couple of thousand dollars

hiring a professional band of studio musicians and

engineers who can probably give you what you want in

about one day.

1.5  What is a .KAR file?

=========================

Han de Bruijn (rcpshdb@dutrun2.tudelft.nl)

A .KAR file or (MIDI-) karaoke file is just a special

case of a MIDI file.  Hence you can play any karaoke

file with your favorite midi player, without having to

change anything.  (Some players require a name ending

with .MID though; in that case, just rename the file.)

You can see the details if you convert a .KAR file to

readable text with help of the "mf2t" programs by Piet

van Oostrum.  (These are downloadable from

'ftp.cs.ruu.nl:/pub/MIDI/PROGRAMS/MSDOS/mf2t.zip').

.KAR files make extensive use of the Meta Text command,

which is defined for standard MIDi files.

A karaoke player is a MIDi player which displays the

"Meta Text"s while music is playing.  A decent one is

"MPLAY".  It is free software; the author can be

contacted at henryso@panix.com.

2.  alt.binaries.sounds.midi

============================

2.1  What is alt.binaries.sounds.midi all about?

================================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

Alt.binaries.sounds.midi, or a.b.s.midi is a Usenet

newsgroup that exists for several purposes, not all of

which everyone agrees upon.  A.b.s.m has no official

mandate, but here are *a few* of the things that are

commonly done on it:

- Distribution of new MIDI files, including both

sequences of existing music and original

compositions.

- Requests for MIDI files, including specific

requests and general requests (by certain artists,

etc.).

- Discussion of MIDI-related issues, including MIDI

software, sound cards and synthesizers, and,

believe it or not, MIDI files!  :-)

A.b.s.m is a non-moderated newsgroup, so it is up its

users to decide what is and what is not appropriate for

posting, and to keep the newsgroup free of useless,

irrelevant clutter.

Another newsgroup dedicated to MIDI is alt.music.midi.

The biggest difference between a.b.s.midi and a.m.m is

that no binaries (files) are posted on the latter; in

other words, alt.music.midi is reserved for discussion

only.  Also, whereas a.b.s.midi tends more towards MIDI

with computers, a.m.m is more about MIDI in general,

with or without computers.

2.2  How do I decode MIDI files from a.b.s.midi?

================================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

MIDI files are posted to a.b.s.midi through one of two

different methods:  UU-encoding, and MIME.

UU-encoding ("UU" means "Unix-to-Unix") is simply a way

of taking binary data (typically broken down into

8-bit "bytes" each having a value from 0 to 255) and

transforming it into ASCII data.  This ASCII data

consists of printable characters:  letters, numbers,

and some symbols.  In UU-encoding, it takes 8 ASCII

characters to represent 6 bytes of binary data.  This

is why files take up more space when they are UU-

encoded.

To decode UU-encoded files, a UU-decoder utility is

required.  Some newsreader programs have built-in

support of UU-decoding.  Otherwise, the article

containing the UU-encoded file should be saved to

disk and then processed with a UU-decoder.

It is easy to recognize UU-encoded data.  It always

begins with a line resembling:

begin 755 myfile.zip

UU-encoded data resembles lines of jumbled characters;

these lines are always 61 characters long (except the

line at the end of the data), and always begin with the

character 'M':

M0TJJ)E*J`KH0$`U%V4QI)#3)K.>F=[UI9 ... (shortened)

Some UU-encoded files are split up into several posts.

For example, the three sections of a three-part UU-

encoded post of a file called MYFILE.ZIP might have

subject lines like the following:

Subject: FILE: "My Title" - MYFILE.ZIP (0/3)

Subject: FILE: "My Title" - MYFILE.ZIP (1/3)

Subject: FILE: "My Title" - MYFILE.ZIP (2/3)

Subject: FILE: "My Title" - MYFILE.ZIP (3/3)

Part "0/n" usually contains brief description of the

UU-encoded file to follow.  Part "x/n" means the xth

part of n total parts.

"Multi-part" posting is done, by those posting the

files, because some newsservers and newsreaders can't

handle UU-encoded data longer than about a thousand

lines, or 64 kB.  Some UU-decoders support multiple-

file UU-encoded data; others do not.  In the latter

case, one must manually cut and paste the parts

together.

MIME is a facility that allows, among other things, a

binary file to be attached to E-mail and Usenet

articles.  When MIME support is installed, a newsreader

program encountering a MIME text will run certain

scripts and/or programs to process the MIME article.

In this case, a newsreader would automatically execute

the appropriate programs to decode an encoded file in

a MIME article.

Even if MIME support is not installed, or should the

MIME support fail, it may still be possible to decode

the binary file attached.  This is because, in many

cases, the standard used by MIME to attach binary

files in a MIME text is UU-encoding.  Thus, the MIME

text could still be saved to disk and decoded with a

UU-decoder.

2.2.1  Can I get viruses from MIDI files?

=========================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

No.

Every so often there is a panicked person who posts an

article to a.b.s.midi saying that such-and-such a MIDI

file posted here contains a virus, much to the chagrin

and/or amusement of others.

Quite simply, MIDI files contain only *data*, as

opposed to *code*, and therefore it is *impossible*

to get a virus from a MIDI file.  (Unless one tries,

for whatever twisted, perverted reason, to execute a

MIDI file. :-)

2.3  How do I send MIDI files to a.b.s.midi?

============================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

MIDI files posted on alt.binaries.sounds.midi are

always encoded in such a way that allows binary files

to be sent as ASCII text.  This is done usually in one

of two ways:  UU-encoding and MIME attachments.

UU (Unix-to-Unix)-encoding is done using a UU-encoder;

different UU-encoders exist for DOS, OS/2, Windows,

etc. and can be found at your favorite FTP sites.  Once

a MIDI file has been UU-encoded into an ASCII file,

that ASCII file is then read, cut-and-pasted, etc. into

the article posted to a.b.s.midi.

MIME is a document format that allows attachments to

accompany an E-mail message or Usenet posting.  Certain

scripts are associated with each type of attachment.

Often, MIME binary-file attachments use UU-encode to

encode binary files.  The main advantage of MIME is

that it's easy to use; with a MIME-supporting news-

reader, one can just specify what (MIDI) files to

attach and this is taken care of immediately.  However,

not all newsreader programs support MIME so many users

don't like it.

2.4  Some guidelines for posting MIDI files

===========================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

Though there is no officially agreed-upon way to post

MIDI files on a.b.s.midi, the following guidelines are

often and generally observed by those who post there:

"Subject" line of the article:  Posted files usually

have a subject line that begins with "FILE:", followed

by the title of the actual piece (if any or if known),

the file name of the MIDI posted, and the expression

"(X/N)" where N is the total number of articles this

posting takes and X is the sequence of this article

out of those N articles.  If there is a comment or

introduction in a separate article accompanying the

file data, this is denoted as article "(0/N)".

For example, take the subject lines of several articles

constituting a posting of a file called MYFILE.ZIP:

Subject: FILE: "My Title" - MYFILE.ZIP (0/3)

Subject: FILE: "My Title" - MYFILE.ZIP (1/3)

Subject: FILE: "My Title" - MYFILE.ZIP (2/3)

Subject: FILE: "My Title" - MYFILE.ZIP (3/3)

Generally it's a good idea to split an article bigger

than 64 kilobytes into several postings, because some

newsreaders can't handle such "big" files.

Compression:  Compression works well with most MIDI

files, because of the way those files tend to use

repeating patterns, etc.  Most MIDI files will shrink

by 60-80% when compressed.  Compressing posted MIDI

files is optional, though it is quite useful when

posting several MIDI songs in the same article.  The

compression utilities most often used, by far, are

PK-Zip and Info-zip which produce compatible files.

Title:  It's a good idea to include the full, known

title of the MIDI file being posted.  It annoys many

users to have to download a file with no title, only

to discover they already have it, or didn't want it at

all to begin with.

3.  MIDI playback devices

=========================

3.2  MIDI devices

=================

3.2.1  Gravis Ultrasound / Ultrasound Max

=========================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

-----------------------------------------------------

| Gravis UltraSound (GUS)                             |

| (Advanced Gravis Computer Technologies Ltd.)        |

|-----------------------------------------------------|

| Chipset:  Gravis GF1                                |

| Output channels:  32 @19kHz/ch down to 14 @44kHz/ch |

| Output rate/resolution:  44.1 kHz, 16-bit           |

| Digital input:  Stereo, 44.1 kHz 8-bit              |

| MIDI synth.:  wavetable, patches on disk (6 MB),    |

|               32-note polyphony, GM                 |

| Custom sample upload:  Yes.                         |

| On-board memory:  256 kB DRAM, expandable to 1 MB   |

| Connections:  MIDI adapter, high-speed joystick,    |

|               mic. in, line in, line out, amp. out, |

|               CD-ROM audio in                       |

| Expansion:  Proprietary daughtercard support        |

|-----------------------------------------------------|

| Driver support:  MS-DOS/Windows, OS/2, Linux        |

| Compatibility:  Sound Blaster and MT-32 emulation   |

| Suggested retail price (USD):  $150                 |

-----------------------------------------------------

-----------------------------------------------------

| Gravis UltraSound Max (GUS Max)                     |

| (Advanced Gravis Computer Technologies Ltd.)        |

|-----------------------------------------------------|

| Chipset:  Gravis GF1, Crystal CODEC                 |

| Output channels:  32 @19kHz/ch down to 14 @44kHz/ch |

|                   2 @48kHz/ch                       |

| Output rate/resolution:  48 kHz max., 16-bit        |

| Digital input:  Stereo, 48 kHz 16-bit, compression  |

|                 supported in hardware               |

| MIDI synth.:  wavetable, patches on disk (6 MB),    |

|               32-note polyphony, GM                 |

| Custom sample upload:  Yes.                         |

| On-board memory:  512 kB DRAM, expandable to 1 MB   |

| Connections:  MIDI adapter, high-speed joystick,    |

|               mic. in, line in, line out, amp. out, |

|               CD-ROM audio in, CD-ROM controller    |

|               (Mitsumi, Panasonic, Sony)            |

| Expansion:  Proprietary daughtercard support        |

|-----------------------------------------------------|

| Driver support:  MS-DOS/Windows, OS/2, Linux        |

| Compatibility:  Sound Blaster and MT-32 emulation   |

| Suggested retail price (USD):  $200                 |

-----------------------------------------------------

Viet-Tam Luu (vtluu@uwaterloo.ca)

When the GUS was first introduced into the market in

1992, it brought in a term that was virtually unknown

in the PC sound card scene:  wavetable synthesis.  The

quality of MIDI playback that it offered, at the time

topped virtually every other card.

Whether it is the "best" MIDI card today (short of

high-end professional cards such as those by Roland or

Turtle Beach) now is a matter of much debate.  Still,

it is the card of choice for many PC users for several

reasons:  its low price, compared to "rival" cards such

as the Sound Blaster AWE-32 or Wave Blaster;  its

digital output capabilities, making it ideal for games

and playing MOD-type files; software support (which has

improved greatly, because of the GUS's popularity

despite its biggest "drawback", the lack of hardware-

level compatibility with the Sound Blaster) compared to

other cheap wavetable cards.

The GUS's major weakness is in digital sampling.

Firstly, it is only 8-bit.  Secondly, GUS's have a

cut-off filter that filters out most input above a

certain frequency which is quite low, as low as 6 kHz

in the case of early GUS's.  The addition of the 16-bit

recording daughtercard does solve the latter problem.

This problem has been addressed in the GUS Max, which

records with 16-bit resolution, and has a much better

frequency response.  The GUS Max also features an

"digital sound processor," actually the Crystal CODEC

which adds hardware audio compression support.  The GUS

Max also features more RAM included, and an on-board

CD-ROM controller.

3.2.2  Turtle Beach Multisound Classic

======================================

Hussam Eassa (eassa@earth.execpc.com)

-----------------------------------------------------

| Multisound Classic                                  |

| (Turtle Beach Systems)                              |

|-----------------------------------------------------|

| Chipset:  Proteus 1/XR, Motorola 56001 DSP          |

| Output channels:  2 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz (max.), 16-bit    |

|                          24 bit internal data path  |

| Digital input:  Stereo, 44.1kHz 16-bit              |

| MIDI synth.:  Wavetable synth., patches in ROM      |

|               copied to onboard RAM, 32-note poly.  |

| On-board memory:  4 MB ROM + 4 MB RAM, no expansion |

| Custom Sample Upload:  No.                          |

| Connections:  MIDI adapter, joystick, line in, aux  |

|               in, line out                          |

| Expansion:  None                                    |

| System req.:  1 port, 1 IRQ and a 32 KB user-       |

|               defined BIOS Address segment.         |

|-----------------------------------------------------|

| Driver support:  Windows 3.x (Win '95 rumored) only |

|                  NO support for any other OS / env. |

| Compatibility:  None.  Windows drivers only.        |

| Street price (typical) (USD): $350 (see note below) |

-----------------------------------------------------

Note:  Although the TB Multisound Classic is officially

discontinued, It is still available in limited quantity

directly from TB and large mail order outlets.

The TB Multisound Classic was (and still is in many

ways) the premier MIDI/digital sampling PC sound card.

The Proteus MIDI engine is well regarded by many MIDI

enthusiasts and it's digital record/playback

performance is second to none even today. It features

two banks of 384 MIDI patch sets that are mappable to

the 126 GM patch set. All samples are 16-bit

uncompressed.

The digital audio is unmatched by anything from other

vendors.  It features ruler flat response (+/- .5dB)

from DC to 19kHz.  The distortion (<.02%) and noise

levels (>86dB) are very low.

The Multisound Classic did not have a wide mass appeal

due to the high initial price ($1000) and the lack of

any type of DOS support.  This eventually resulted in

TB superceding it with the Multisound Monterey.

3.2.3  Turtle Beach Multisound Monterey

=======================================

Hussam Eassa (eassa@earth.execpc.com)

-----------------------------------------------------

| Multisound Monterey                                 |

| (Turtle Beach Systems)                              |

|-----------------------------------------------------|

| Chipset:  ICS WaveFront 2115, Motorolla 56001 DSP   |

| Output channels:  2 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz (max.), 16-bit    |

|                          24 bit Internal data path  |

| Digital input:  Stereo, 44.1kHz 16-bit              |

| MIDI synth.:  WaveTable synth, Patches in ROM       |

|               32 voice polyphony.                   |

| On-board memory:  4 MB ROM, optional 4MB RAM        |

| Custom sample upload:  Yes.                         |

| Connections:  MIDI adapter, joystick, line in, aux  |

|               in, line out.                         |

| Expansion:  None.                                   |

| System req.:  1 port, 1 IRQ and a 32 KB user-       |

|               defined BIOS Address segment.         |

|-----------------------------------------------------|

| Driver support:  Windows 3.x.                       |

| Compatibility:  MPU-401.  Windows drivers.          |

| Street price (typical) (USD):  $320                 |

-----------------------------------------------------

The TB Monterey is the heir to the Multisound Classic.

It inherits exactly the same digital audio processing

circuits and hence the same legendary sampling

performance.  (See Multisound Classic)

The MIDI section remains somewhat controversial.  The

Proteus 1/XR of the MS Classic is replaced with the ICS

WaveFront synthesizer.  This new engine is capable of

adding user-defined amounts of chorus and reverb and is

quite sophisticated in that regard.  It also allows the

upload of samples to onboard expansion RAM although it

has been observed by many that the upload speed can be

frustrating.  This is due to the slow serial link that

is used (the Sound Blaster compatible Wave Blaster

interface to the RAM).  Some Multisound Classic

diehards still maintain that the quality of the patches

of the ICS does not come up to the level of the Proteus

1/XR patches of the Classic.  In spite of this, it is

still highly regarded with a strong following.  Wider

appeal probably will not be forthcoming due to the lack

of Sound Blaster compatibility (hardware or software)

and the lack of a CD-ROM interface.

3.2.4  Sound Blaster 2.0

========================

Viet-Tam Luu (vtluu@uwaterloo.ca)

-----------------------------------------------------

| Sound Blaster 2.0 (SB)                              |

| (Creative Labs, Inc.)                               |

|-----------------------------------------------------|

| Chipset:  Creative, Yamaha OPL-2                    |

| Output channels:  1 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz (max.), 8-bit     |

| Digital input:  Mono, 15 kHz (max.), 8-bit          |

| MIDI synth.:  FM, 11-note polyphony, GM instruments |

| On-board memory:  None                              |

| Connections:  MIDI adapter, joystick, mic. in, line |

|               in, amp. out                          |

| Expansion:  None                                    |

|-----------------------------------------------------|

| Driver support:  MS-DOS/Windows, OS/2, Linux        |

| Compatibility:  Sound Blaster 2.0                   |

| Street price (typical) (USD):  $50                  |

-----------------------------------------------------

-----------------------------------------------------

| Sound Blaster Pro (SBPro)                           |

| (Creative Labs, Inc.)                               |

|-----------------------------------------------------|

| Chipset:  Creative, Yamaha OPL-3                    |

| Output channels:  2 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz (max.), 8-bit     |

| Digital input:  Stereo, 44.1 kHz (max.), 8-bit      |

| MIDI synth.:  FM, 20-note polyphony, GM instruments |

| On-board memory:  None                              |

| Connections:  MIDI adapter, joystick, mic. in, line |

|               in, amp. out, PC-speaker in, CD-ROM   |

|               interface (Creative)                  |

| Expansion:  None                                    |

|-----------------------------------------------------|

| Driver support:  MS-DOS/Windows, OS/2, Linux        |

| Compatibility:  Sound Blaster 2.0                   |

| Street price (typical) (USD):  $75                  |

-----------------------------------------------------

This was a great card...  when it first came out, about

ten years ago.  It offered what the de facto standard

at the time, the AdLib card, didn't:  digital input and

output.  It impressed many people (except Amiga users).

The problem is that it's still around and being sold.

No-one, but no-one, will argue that this is a good

card; still, it's better than the PC speaker, which is

why people still buy it.  And it's cheap, dirt cheap.

And the zillion clones of this card are even cheaper.

The Sound Blaster Pro is basically a stereo version

of the Sound Blaster, plus a CD-ROM interface.

3.2.5  Sound Blaster 16

=======================

Viet-Tam Luu (vtluu@uwaterloo.ca)

-----------------------------------------------------

| Sound Blaster 16 (SB16)                             |

| (Creative Labs, Inc.)                               |

|-----------------------------------------------------|

| Chipset:  Creative, Yamaha OPL-3                    |

| Output channels:  2 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz (max.), 16-bit    |

| Digital input:  Stereo, 44.1 kHz (max.), 16-bit     |

| MIDI synth.:  FM, 20-note polyphony, GM instruments |

| On-board memory:  None                              |

| Connections:  MIDI adapter, joystick, mic. in, line |

|               in, amp. out, PC-speaker in, CD-ROM   |

|               controller (Creative), CD-audio in    |

| Expansion:  Daughtercard support                    |

|-----------------------------------------------------|

| Driver support:  MS-DOS/Windows, OS/2, Linux        |

| Compatibility:  Sound Blaster 16, MPU-401           |

| Street price (typical) (USD):  $100                 |

-----------------------------------------------------

This card is basically a step up from the Sound Blaster

Pro (a stereo version of the Sound Blaster 2.0), with

support 16-bit playback and recording, a CD-ROM

controller, and support for a daughtercard.  It is not,

by itself, regarded as a serious card for playing MIDI

files, as all it has is a stereo version of the OPL-2

FM synthesizer chip found on the original Sound Blaster

card.  Software exists that allow wavetable MIDI

playback with the digital sound hardware, but this is

no replacement for true hardware wavetable support.

To add wavetable support to this otherwise lame card,

several different daughterboards are on the market,

varying widely in prices and capabilities, including

Creative Lab's WaveBlaster, the Gravis Ultrasound ACE,

and two different Roland Sound Canvas cards.  However,

considering the total price of the SB16 + daughtercard,

it may be more economical to consider one of the many

cheap cards out there (by Logitech, Orchid, Gravis,

etc.) in the same price range that offer built-in

wavetable synthesis.

3.2.6  Sound Blaster AWE-32

===========================

Hussam Eassa (eassa@earth.execpc.com)

-----------------------------------------------------

| Sound Blaster AWE-32                                |

| (Creative Labs, Inc.)                               |

|-----------------------------------------------------|

| Chipset:  EMU8000, Yamaha OPL3                      |

| Output channels:  2 @44.1kHz                        |

| Output rate/resolution:  44.1 kHz, 16-bit           |

| Digital input:  Stereo, 44.1kHz 16-bit              |

| MIDI synth.:  WaveTable synth, Patches in ROM       |

|               and RAM, 32 voice polyphony.          |

| On-board memory:  1 MB ROM, 512KB RAM               |

| Memory Expansion: 28 MB RAM                         |

| Custom sample upload:  Yes.                         |

| Connections:  MIDI adapter, joystick, line in, line |

|               out, microphone in.                   |

| CD-ROM Support:  Sony CDU-31A/33A, Mitsumi LU005 &  |

|                  FX001 Series and Creative CR-523 & |

|                  563                                |

| Expansion:  Wave Blaster connector.                 |

| System Requirements:  2 ports, 1 IRQ and 2 DMA      |

|-----------------------------------------------------|

| Driver support:  Windows 3.x, Win95.                |

| Compatibility:  Sound Blaster-16, Adlib, MPU-401    |

|                 (software driver) (see note).       |

| Street price (typical) (USD):  $300                 |

-----------------------------------------------------

Note:  The AWE-32 is software MPU-401 compatible in DOS

as shipped.  It can be upgraded to hardware DOS

compatibility by addition of any of the available Wave

Blaster compatible daughterboards.

The AWE-32 is Creative Lab's entry into the mid-tier

MIDI market. It appears to have been designed to

provide MIDI performance somewhere between the low end

FM cards and the High end cards such as the Roland

SCC1, RAP-10 and the high-end Turtle Beach Multisound

cards.  Exactly where it actually falls in that range

has fueled many sound card "wars," especially with

Gravis fans.

This card is really a jack of all trades and is quite

capable.  It features very good backward compatibility

and upgradability.  The EMU8000 provides the capability

to add chorus and reverb in DOS and Windows.  But as

with jacks of all trades, it cannot be viewed as a

master of any specific area.

The MIDI performance is quite good but the use of

either more RAM and custom patch banks or a daughter

board is really necessary for exceptional performance.

Either of these options adds to the cost significantly.

The digital audio section is very good but falls short

of that of the Turtle Beach products.  All in all, this

is a good card for good quality, general purpose use

with full Sound Blaster compatibility and good

upgradeability.

4.  MIDI software

=================

4.2  MIDISCAN - MIDI OCR Software

=================================

Steve Adams (steven@ivory.jpl.nasa.gov)

I picked up MIDISCAN at a computer trade show last

Sunday for $99.  The fellow had his scanner and

computer set up to give demos.  After letting me pick a

piece on my own, we digitized a Clementi sonata.

Sounded pretty good.  He must have had a Pentium.  I

have a Compaq Deskpro 386/25 here at work and an IBM

486SLC2/66 at home.  Neither of my computers

"converted" a file as fast as his did.  In fact my home

computer locked up several times on a Telemann fugue.

Tried it at work also, and to my surprise it locked up

also.  Memory is not a problem, my home machine has 16

meg and my work machine has 8. Must be something with

that particular piece.

Have tried a variety of music the past several days.

Mostly classical.  Did a Telemann fantasia (3/4 page

size), a Telemann fugue, a one page Beethoven piece and

the Brahms waltz.  The Telemann fantasia converted

pretty well.  I sequenced this piece before, and

MIDISCAN did a decent job.  As mentioned, couldn't

convert the fugue.  The Beethoven was the simplest, and

was converted the most acurately.

How it works:  One needs to scan the sheet music first.

I only have access to a Sharp flatbed scanner hooked up

to a Mac quadra here at work.  Used Photoshop.

MIDISCAN only reads TIFF files.  Photoshop can output

this format, just be sure to save *without*

compression.  MIDISCAN cannot handle LZW compression.

Also if using a Mac, save in PC TIFF format, *not* Mac

TIFF format.  Then dumped the TIFF files to floppy.

(Here a procedure tip:  Each 8x10 scanned sheet of TIFF

format is approx 1 MB in size.  LZW will reduce that to

about 150 kB.  What I did was save on the Mac with LZW.

Put a whole bunch on floppy, read on the PC and used my

PC version of Photoshop to resave without LZW.)  Open

MIDISCAN (a Windoze program) and tell it which TIFF

files to convert.  If a piece of music is more than one

page, (or several TIFF files, no problem, just list

them all, in order to be converted).  MIDISCAN then

takes over.  On the 486SLC2/66 it takes about 3-5

minutes to convert each TIFF file.  On the 386, about

10-15 minutes.  On the fellow's Pentium (I assume) it

only took about a minute.  MIDISCAN then displays in

split screen fashion both the original TIFF scan and

its interpretation.  They are displayed "side-by-side"

so that one can see each measure for easy comparison.

One then has the option to use the built-in editor to

"clean-up" the intrepreted file, (and *some* needs to

be done.)  The editor is fairly easy and intuitive to

use.  One does not need the manual to run this program.

Finally, another drop-down has the option to save as a

MIDI file.  (Again, the two files included are the raw

intrepreted version, and a slightly cleaned up version.

The next step would be to read in the MIDI file into a

sequencer for final clean-up.)

My impressions on how MIDISCAN did:  not bad.  All text

is ignored.  The manual recommends that one "delete"

the text from the TIFF file before the conversion step.

They include a simple TIFF editor to do this.  It

consists of identifying the upper-left and lower-right

corner to delete.  I ran my test with all text as part

of the files.  No problem.  The program did indeed

ignore text, except in a couple of cases where finger

numbers were intrepreted as a note.  Most key

signatures were intrepreted correctly.  It had a

problem with the time signature, but that was easy to

fix.  Seems to have a problem some times distinguishing

between a quarter note and a half note.  Again, pretty

easy to fix, and occured only about 2-5% of the time.

In this Brahms piece, slurs were intrepreted as ties.

Again, easy to edit out.  All dynamics are ignored, so

these will have to be put in by hand in your sequencer.

Accidentals are picked up, but I question its accuracy

of intrepretation.  One thing I noticed that it did not

pick up is the stacatto notes.  It pickes up the note,

but not the stacatto indication.

I tested version 1.1c, which is the latest available.

Got a coupon for upgrade to 2.0 The only difference is

that 2.0 includes TWAIN support and another minor one

that I cannot remember now.  The upgrade will cost me

another $50.  (Not sure I'll get it.)

Finally, I guess the program did okay.  As far as a

user getting music into MIDI, it falls someplace

between someone who knows how to play the piano real

well, and can use his/her sequencer to record while

they are playing and someone (like me) who has to

step-enter the notes one by one.  I'll probably

continue to use the program for the first cut at

getting the music into the computer, but not enough to

buy a scanner for myself at home.  All in all, quite a

programming feat.  Remember, this really hasn't been

done on a commercial scale before.  MIDISCAN is the

first to pull it off.  They deserve credit for that,

even if the program is not 100% accurate (technology

development has to start somewhere.)

4.3  MIDI-file players

======================

4.3.1  Cubic Player 0.96

========================

Terry Law (law@legend.pl.my)

Cubic Player (CP) is a music player which plays a

variety of sound formats on Sound Blaster compatible

cards and the UltraSound.  Just in case you are not the

lucky owner of such a device, we have implemented a

quiet play option, in order to let you experience the

player.

You have to have the GUS patches for MIDI support.  If

you haven't got a GUS you can find them on many BBS's,

FTP sites (see section 5.1) they usually come as "GUS

drivers" and are about 5-6 disks big.  GUS users can

skip the following part.  (Hey, don't mess around with

DEFAULT.CFG, this could cause errors; if you make

changes to the DEFAULT.CFG that refuse to work with CP,

first make a copy of the original one and call it

CPMIDI.CFG.)  After installing them into any directory,

add the following statement to CP.CFG:  (if there is no

CP.CFG yet, create it in the CP directory)

-mpPATH   where PATH is the directory of your GUS

patches.

-mcnn     use nn channels for MIDI output

Let's say if you put all the patches in C:\MIDI\PATCHES

and you wish to play MIDI files in maximum 32 channels,

you would type:

"dos4gw.exe cp.exe -mpc:\midi\patches -mc32"

Dos4gw is a DOS extender for Cubic Player; through it,

Cubic will run in (386+) 32-bit protected mode.

For what I've tested, you can just buy 1, 2 or 3

additional speakers, connect them as shown below and

press F5 twice and F4 in CP, and you'll get a real COOL

surround sound!

Stereo

+ L -           - R +

+-+ +-+         +-+ +-+

+-+ +-+         +-+ +-+

I   I           I   I

I   I           I   I

+----+   +-----+-----+   +-----+

I    I         I         I     I

I    I         I         I     I

I    I         -         I     I

I    I     M-Speaker     I     I

I    I         +         I     I

I    I         I         I     I

I    +   - ----+---- -   +     I

I  L-Speaker        R-Speaker  I

I                              I

I                              I

+---+  +-------------+  +------+

I  I             I  I

+  -             -  +

S1-Speaker      S2-Speaker

I've tested the latest version of Cubic (v.96); though

Cubic can't play 100% of all MIDI files, the quality is

great when compared to FM-synthesis, especially for

Sound Blaster card MIDI users!

Here's some tips for Cubic users:

- You can just zip all your MIDI files in a .ZIP file;

Cubic will scan through it!  This is a way to save

disk space; I'm using my MIDI files in 3.5 megabytes

instead of 15 megabytes!

- I've tested Cubic in OS/2 -- it works OK!

5.  MIDI on the Internet

========================

5.1  What are some FTP sites where I can get MIDI

and MIDI-related files?

=================================================

This section is based largely upon information from the

a.b.s.midi "mini-FAQ" maintained by Piet van Oostrum

(piet@cs.ruu.nl).  Other sources include various posts

on the topic that have appeared on a.b.s.midi.

Site/address:  ftp.cs.ruu.nl

Directory:  /pub/MIDI/SONGS, /pub/MIDI/...

Notes:  Large collection of MIDI files.  MIDI software,

specifications, information, etc.

Site/address:  ftp.eng.ufl.edu

Notes:  MIDI files.

Site/address:  wuarchive.wustl.edu

Directory:  /systems/ibmpc/ultrasound/sound/midi/files

Notes:  Large collection of MIDI files, some specific

to the Gravis UltraSound but most GM.

Site/address:  ftp.ucsd.edu [128.54.16.7]

Directory:  /midi

Notes:  Appears to be out of date.

Site/address:  atari.archive.umich.edu [141.211.120.11]

Directory:  /atari/Music

Notes:  "Atari stuff."

Site/address:  scam.berkeley.edu [128.32.138.1]

Directory:  /pub/misc/netjam/submissions

Notes:  Netjam MIDI files may be found here.

Site/address: alf.uib.no [129.177.30.3]

Directory:  /pub/sds

Notes:  MIDI Sample Dump Standard programs and samples.

Site/address:  sweaty.palm.cri.nz [161.66.1.11]

Directory:  /sds

Notes:  MIDI Sample Dump Standard programs and samples.

EMAX I/II stuff (samples, FAQ, mailing list

archive, software).

Site/address:  wagner.musicnet.ua.edu [130.160.156.75]

Directory:  /pub/music/patches/jv80

Notes:  Patches for Roland JV-80/880/1000.

Site/address:  oak.oakland.edu [141.210.10.117]

Directory:  /pub/eps/...

Notes:  EPS/EPS-16+/ASR stuff.  Also accessible through

Gopher (gopher.acs.oakland.edu, port 70) and

WWW (http://www.acs.oakland.edu).

Site/address:  ftp.uwp.edu

Directory:  /pub/music/lists/kurzweil

Notes:  Kurzweil K2000 stuff.

Site/address:  bach.nevada.edu

Directories:  /pub/k2000, /pub/midi

Notes:  Kurzweil K2000 stuff.  Christian music MIDI

files.

Site/address:  qiclab.scn.rain.com

Directory:  /pub/music

Notes:  Various conversion programs.

Site/address:  lotus.uwaterloo.ca

Directory:  /pub/sgroup

Notes:  Information, utilities, archives, samples.

Site/address:  ftp.informatik.uni-freiburg.de

[132.230.150.1]

Directory:  /pub/sounds/tx16w

Notes:  TX-16W samples, TX-16W newsletter.  Accessible

through WWW (http://ls7-www.informatik.uni-

dortmund.de/html/tx16w.html)

Site/address:  cattell.psych.upenn.edu [12128.91.2.173]

Directory:  pub/Music.Research

Notes:  Archives of the Music-Research Digest.

Site/address:  impaqt1.mem.drexel.edu [129.25.10.1]

Directory:  /pub/files/ibm/midi

Notes:  "Has a couple of MS-DOS files."

Site/address:  ftp.funet.fi [128.214.6.100]

Directory:  pub/msdos/sound/cakewalk

Notes:  Cakewalk (.WRK) files.

Site/address:  ftp.cs.colorado.edu

Directories:  /users/mccreary/archive/cakewalk

/users/mccreary/archive/vfx

Notes:  Cakewalk stuff, archives of Cakewalk mailing

list.  Accessible through WWW (http://www.cs.

olorado.edu/~mccreary/archive/cakewalk).

VFX archive.

Site/address:  mort.isvr.soton.ac.uk

Directory:  /pub/pc/cakewalk

Notes:  Cakewalk stuff.  Accessible through WWW

http://www.isvr.soton.ac.uk/People/ccb/

Cakewalk).

Site/address:  ftp.cica.indiana.edu

Directory:  pub/pc/win3/sounds

Notes:  MIDI programs for MS Windows.

Site/address:  ftp-os2.cdrom.com [192.153.46.2]

Directories:  /pub/os2/2_x/mmedia, /pub/os2/2_1/mmedia

Notes:  OS/2 MIDI-related files.

Site/address:  hobbes.nmsu.edu

Directories:  /multimedia/mid, /os2/mmedia

Notes:  MIDI files.  OS/2 MIDI-related software.

Site/address:  mac.archive.umich.edu [141.211.165.41]

Directory:  /mac/sound/midi

Notes:  Macintosh MIDI archive.

Site/address:  ftp.neuroinformatik.ruhr-uni-bochum.de

Directory:  /pub/outgoing/heja/sy-list

Notes:  Yamaha SY stuff.  "IMPORTANT NOTE:  go directly

to the above mentioned directory!"  Accessible

through WWW (http://www.neuroinformatik.ruhr-

uni-bochum.de/ini/PEOPLE/heja/sy-list.html).

"Mostly original material."

Site/address:  ftp.ecn.nl

Directory:  /pub/RP-1

Notes:  DIGITECH RP-1 MIDI guitar effect pedals stuff.

Site/address:  cco.caltech.edu [131.215.6.10]

grind.isca.uiowa.edu [128.255.21.233]

Directory:  pub/apple2/music/synthlab/

Notes:  Apple 2 MIDI Synth archives.

Site/address:  ftp.ircam.fr

Directory:  /pub/music/...

Notes:  Information and files relating to music, DSP

(Digital Signal Processing), MIDI, etc.

MIDI standard definition.

Accessible through WWW (http://www.ircam.fr).

Site/address:  mitpress.mit.edu

Directory:  /pub/Computer-Music-Journal

Site/address:  ccrma-ftp.stanford.edu

Directory:  /pub/Publications/cmj

Notes:  Computer Music Journal Internet archive.  Also

accessible through WWW (http://mitpress.mit.

edu/pub/Computer-Music-Journal/CMJ.html).

Site/address:  ftp.mcc.ac.uk

Directory:  /pub/cubase

Notes:  Cubase archive.

Site/address:  ftp.waldorf-gmbh.de

Notes:  Information, patches, other MIDI stuff.

Site/address:  ftp.und.ac.za

Directory:  /pub/pc/midi

Notes:  MIDI files, programs for Sound Canvas and MT-32.

Site/address:  loose.apana.org.au

Directory:  pub/dw8000

Notes:  Patches for the Korg DW/EX8000.

Site/address:  ftp.netcom.com

Directory:  /pub/jo/jonin/midi/x3

Notes:  Korg X3 patches, programs, information, etc.

Site/address:  musie.phlab.missouri.edu

Directory:  /pub/korg

Notes:  Information on Korg products.  Accessible by

WWW (http://musie.phlab.missouri.edu/pub/korg/)

Site/address:  ftp.rc.tudelft.nl

Directory:  /pub/midi

Notes:  MIDI site for original MIDI compositions only.

5.2  Where and how can I get the official MIDI spec's?

======================================================

The most recent specifications for the MIDI standard

may be obtained, for a modest fee, from the

International MIDI Association, at the following

(snail-) mail address:

International MIDI Association

23634 Emelita Street

Woodland Hills, California 91367

USA

5.3  Other MIDI-related addresses (WWW, etc.)

=============================================

5.3.1  World-Wide Web MIDI Stuff

================================

URL:  http://tnt.microimages.com/~dwilliss/midi.html

Notes:  MIDI page; site of the latest HTML version of

the alt.binaries.sounds.midi FAQ (this

document)

Contact:  David C. Williss (dwilliss@microimages.com)

URL:  http://stud1.tuwien.ac.at/~e8925292/bestmid.htm

Notes:  The "Very Best of GUS MIDI" Collection

Contact:  Gerd Reichinger (e8925292@student.tuwien.ac.

at)

URL:  http://fandora.resnet.cornell.edu/~terry/midi.htm

Notes:  "MIDI Collection"

Contact: Terry Law (law@legend.pl.my)

URL:  http://199.227.124.2/best/best.htm

Notes:  "Best of the Best MIDI Files Page"

Contact:  Jeff L. (jledger@gate.net)

URL:  http://harmony-central.mit.edu/MIDI/

Notes:  WWW document devoted to MIDI

Contact:  Scott Lehman (slehman@MIT.EDU)

URL:  http://www.cs.umn.edu/users/fischer/Midi/

macmidi.html

Notes:  WWW page on using MIDI files with Macintosh

computers.

Appendix A - Off-line sources of information

============================================

Bibliography

============

Many books have been published on MIDI and MIDI-

related issues.  An extensive MIDI bibliography is

maintained by Piet van Oostrum (piet@cs.ruu.nl).

Unfortunately, due to its size it cannot be

included here.

The latest version of the bibliography can be obtained

by FTP from ftp.cs.ruu.nl [131.211.80.17] in the

MIDI/DOC/bibliography or by E-mail from

mail-server@cs.ruu.nl (send a message with HELP in the

body).

Official MIDI specifications

============================

See section 5.2.

Appendix B - Contributing to the a.b.s.midi FAQ

===============================================

Viet-Tam Luu (vtluu@uwaterloo.ca)

Please E-mail (*don't* post, my news server only keeps

about 20 a.b.s.midi posts at a time) your contributions

for any of the outlined sections, or any section you

may wish to add, to me.  You may wish to check with me

first, just to make sure someone else isn't already

working on whatever it is you want to do.

Please don't send me the entire FAQ if you have added

or revised a section, just that section.

Unless you are writing about a specific MIDI device/

synth, don't play favorites, i.e. don't "bash" other

sound cards.  Be objective.

The above also goes for people, as for sound cards.

Discriminatory statements/remarks, overt or implied,

of any form *will not* be tolerated.  Enough said.

As the FAQ maintainer I reserve the right to make

editorial changes/corrections to submissions, or to

refuse them outright.  I will not, however, add edited

submissions to the FAQ without the permission of their

authors.

If you have any questions about the a.b.s.midi FAQ,

don't hesitate to E-mail me.  Questions about MIDI-

related topics should be sent to a.b.s.midi, not me; I

don't claim to know anything much about MIDI.

Appendix C - List of contributors

=================================

The alt.binaries.sounds.midi FAQ has been made possible

through the contributions of the following people (if

your name isn't here, but should be, send E-mail to

vtluu@uwaterloo.ca):

Steve Adams (steven@ivory.jpl.nasa.gov)

Han de Bruijn (rcpshdb@dutrun2.tudelft.nl)

Hussam Eassa (eassa@earth.execpc.com)

Terry Law (law@legend.pl.my)

Viet-Tam Luu (vtluu@uwaterloo.ca)

Piet van Oostrum (piet@cs.ruu.nl)

Randy Lynn Tusch (drt@rec1.roc.servtech.com)

David C. Williss (dwilliss@microimages.com)