# TABLE 1:Summary of Status and Data Bytes#          Page 1

-----------------------------------------------------|-------------------------

STATUS BYTE                 |      DATA BYTES

-----------------------------------------------------|-----------|-------------

First Byte Value     |            Function         |  Second   |    Third

-----------------------|                             |           |

Binary  Hex  Decimal  |                             |           |

-----------------------------------------------------|-----------|-------------

10110011  B3   179     | Channel 4      Control      | See table | See table

10110100  B4   180     | Channel 5    mode change    |  three    |   three

10110101  B5   181     | Channel 6           "       |      "    |    "

10110110  B6   182     | Channel 7           "       |      "    |    "

10110111  B7   183     | Channel 8           "       |      "    |    "

10111000  B8   184     | Channel 9           "       |      "    |    "

10111001  B9   185     | Channel 10          "       |      "    |    "

10111010  BA   186     | Channel 11          "       |      "    |    "

10111011  BB   187     | Channel 12          "       |      "    |    "

10111100  BC   188     | Channel 13          "       |      "    |    "

10111101  BD   189     | Channel 14          "       |      "    |    "

10111110  BE   190     | Channel 15          "       |      "    |    "

10111111  BF   191     | Channel 16          "       |      "    |    "

11000000  C0   192     | Channel 1    Program change | Program # |   None

11000001  C1   193     | Channel 2           "       |  (0-127)  |    "

11000010  C2   194     | Channel 3           "       |      "    |    "

11000011  C3   195     | Channel 4           "       |      "    |    "

11000100  C4   196     | Channel 5           "       |      "    |    "

11000101  C5   197     | Channel 6           "       |      "    |    "

11000110  C6   198     | Channel 7           "       |      "    |    "

11000111  C7   199     | Channel 8           "       |      "    |    "

11001000  C8   200     | Channel 9           "       |      "    |    "

11001001  C9   201     | Channel 10          "       |      "    |    "

11001010  CA   202     | Channel 11          "       |      "    |    "

11001011  CB   203     | Channel 12          "       |      "    |    "

11001100  CC   204     | Channel 13          "       |      "    |    "

11001101  CD   205     | Channel 14          "       |      "    |    "

11001110  CE   206     | Channel 15          "       |      "    |    "

11001111  CF   207     | Channel 16          "       |      "    |    "

11010000  D0   208     | Channel 1         Channel   | Aftertouch|    "

11010001  D1   209     | Channel 2       aftertouch  |  pressure |    "

11010010  D2   210     | Channel 3           "       |  (0-127)  |    "

11010011  D3   211     | Channel 4           "       |      "    |    "

11010100  D4   212     | Channel 5           "       |      "    |    "

11010101  D5   213     | Channel 6           "       |      "    |    "

11010110  D6   214     | Channel 7           "       |      "    |    "

11010111  D7   215     | Channel 8           "       |      "    |    "

11011000  D8   216     | Channel 9           "       |      "    |    "

11011001  D9   217     | Channel 10          "       |      "    |    "

11011010  DA   218     | Channel 11          "       |      "    |    "

11011011  DB   219     | Channel 12          "       |      "    |    "

11011100  DC   220     | Channel 13          "       |      "    |    "

11011101  DD   221     | Channel 14          "       |      "    |    "

11011110  DE   222     | Channel 15          "       |      "    |    "

11011111  DF   223     | Channel 16          "       |      "    |    "

# TABLE 1:Summary of Status and Data Bytes#          Page 2

-----------------------------------------------------|-------------------------

STATUS BYTE                 |      DATA BYTES

-----------------------------------------------------|-----------|-------------

First Byte Value     |            Function         |  Second   |    Third

-----------------------|                             |           |

Binary  Hex  Decimal  |                             |           |

-----------------------------------------------------|-----------|-------------

11100000  E0   224     | Channel 1      Pitch wheel  |Pitch wheel|Pitch wheel

11100001  E1   225     | Channel 2         range     |    LSB    |    MSB

11100010  E2   226     | Channel 3           "       |  (0-127)  |  (0-127)

11100011  E3   227     | Channel 4           "       |      "    |    "

11100100  E4   228     | Channel 5           "       |      "    |    "

11100101  E5   229     | Channel 6           "       |      "    |    "

11100110  E6   230     | Channel 7           "       |      "    |    "

11100111  E7   231     | Channel 8           "       |      "    |    "

11101000  E8   232     | Channel 9           "       |      "    |    "

11101001  E9   233     | Channel 10          "       |      "    |    "

11101010  EA   234     | Channel 11          "       |      "    |    "

11101011  EB   235     | Channel 12          "       |      "    |    "

11101100  EC   236     | Channel 13          "       |      "    |    "

11101101  ED   237     | Channel 14          "       |      "    |    "

11101110  EE   238     | Channel 15          "       |      "    |    "

11101111  EF   239     | Channel 16          "       |      "    |    "

11110000  F0   240     | System exclusive            |      *    |    *

11110001  F1   241     | Sys.common undefined        |      ?    |    ?

11110010  F2   242     |Sys.com.song position pointer|     LSB   |   MSB

11110011  F3   243     |Sys.com.song select (song #) |   (0-127) |   none

11110100  F4   244     | Sys.common undefined        |      ?    |    ?

11110101  F5   245     | Sys.common undefined        |      ?    |    ?

11110110  F6   246     | Sys common tune request     |    None   |   None

11110111  F7   247     |Sys.com.end of sys.excl.(EOX)|      "    |    "

11111000  F8   248     | Sys.real time timing clock  |      "    |    "

11111001  F9   249     | Sys.real time undefined     |      "    |    "

11111010  FA   250     | Sys.real time start         |      "    |    "

11111011  FB   251     | Sys.real time continue      |      "    |    "

11111100  FC   252     | Sys.real time stop          |      "    |    "

11111101  FD   253     | Sys.real time undefined     |      "    |    "

11111110  FE   254     | Sys.real time active sensing|      "    |    "

11111111  FF   255     | Sys.ral time sys.reset      |      "    |    "

-------------------------------------------------------------------------------

* Note: System Exclusive(data dump) 2nd byte = Vendor ID followed by more data

bytes and ending with EOX.

#TABLE 2: Summary of MIDI Note numbers for Different Octaves#

-----------------------------------------------------------------------------

Octave #     |                        Note Numbers                        |

| C  | C# | D  | D# | E  | F  | F# | G  | G# | A  | A# | B   |

-----------------------------------------------------------------------------

0    | 0  | 1  | 2  | 3  | 4  | 5  | 6  | 7  | 8  | 9  | 10 | 11

1    | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23

2    | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35

3    | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47

4    | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59

5    | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71

6    | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83

7    | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95

8    | 96 | 97 | 98 | 99 | 100| 101| 102| 103| 104| 105| 106| 107

9    | 108| 109| 110| 111| 112| 113| 114| 115| 116| 117| 118| 119

10   | 120| 121| 122| 123| 124| 125| 126| 127|  * |  * |  * |  *

Note: "Middle C" equals MIDI note #60; "A440" equals MIDI note #57

#TABLE 3: Status Bytes 176-191; Control and Mode Changes (per channel)#

-----------------------------------------------------|-------------------------

STATUS BYTE                 |      THIRD BYTE

-----------------------------------------------------|-----------|-------------

First Byte Value     |                             |           |

-----------------------|                             |           |

Binary  Hex  Decimal  |       Function              | Value     |    Use

-----------------------------------------------------|-----------|-------------

00000000  00   0       | Continuous Controller #0    | 0-127     |     MSB

00000001  01   1       | Modulation wheel            | 0-127     |     MSB

00000010  02   2       | Breath control              | 0-127     |     MSB

00000011  03   3       | Continuous controller #3    | 0-127     |     MSB

00000100  04   4       | Foot controller             | 0-127     |     MSB

00000101  05   5       | Portamento time             | 0-127     |     MSB

00000110  06   6       | Data entry                  | 0-127     |     MSB

00000111  07   7       | Main volume                 | 0-127     |     MSB

00001000  08   8       | Continuous controller #8    | 0-127     |     MSB

00001001  09   9       | Continuous controller #9    | 0-127     |     MSB

00001010  0A   10      | Continuous controller #10   | 0-127     |     MSB

00001011  0B   11      | Continuous controller #11   | 0-127     |     MSB

00001100  0C   12      | Continuous controller #12   | 0-127     |     MSB

00001101  0D   13      | Continuous controller #13   | 0-127     |     MSB

00001110  0E   14      | Continuous controller #14   | 0-127     |     MSB

00001111  0F   15      | Continuous controller #15   | 0-127     |     MSB

00010000  10   16      | Continuous controller #16   | 0-127     |     MSB

00010001  11   17      | Continuous controller #17   | 0-127     |     MSB

00010010  12   18      | Continuous controller #18   | 0-127     |     MSB

00010011  13   19      | Continuous controller #19   | 0-127     |     MSB

00010100  14   20      | Continuous controller #20   | 0-127     |     MSB

00010101  15   21      | Continuous controller #21   | 0-127     |     MSB

00010110  16   22      | Continuous controller #22   | 0-127     |     MSB

00010111  17   23      | Continuous controller #23   | 0-127     |     MSB

00011000  18   24      | Continuous controller #24   | 0-127     |     MSB

00011001  19   25      | Continuous controller #25   | 0-127     |     MSB

00011010  1A   26      | Continuous controller #26   | 0-127     |     MSB

00011011  1B   27      | Continuous controller #27   | 0-127     |     MSB

00011100  1C   28      | Continuous controller #28   | 0-127     |     MSB

00011101  1D   29      | Continuous controller #29   | 0-127     |     MSB

00011110  1E   30      | Continuous controller #30   | 0-127     |     MSB

00011111  1F   31      | Continuous controller #31   | 0-127     |     MSB

00100000  20   32      | Continuous controller #0    | 0-127     |     LSB

00100001  21   33      | Modulation wheel            | 0-127     |     LSB

00100010  22   34      | Breath control              | 0-127     |     LSB

00100011  23   35      | Continuous controller #3    | 0-127     |     LSB

00100100  24   36      | Foot controller             | 0-127     |     LSB

00100101  25   37      | Portamento time             | 0-127     |     LSB

00100110  26   38      | Data entry                  | 0-127     |     LSB

00100111  27   39      | Main volume                 | 0-127     |     LSB

00101000  28   40      | Continuous controller #8    | 0-127     |     LSB

00101001  29   41      | Continuous controller #9    | 0-127     |     LSB

00101010  2A   42      | Continuous controller #10   | 0-127     |     LSB

00101011  2B   43      | Continuous controller #11   | 0-127     |     LSB

00101100  2C   44      | Continuous controller #12   | 0-127     |     LSB

00101101  2D   45      | Continuous controller #13   | 0-127     |     LSB

00101110  2E   46      | Continuous controller #14   | 0-127     |     LSB

00101111  2F   47      | Continuous controller #15   | 0-127     |     LSB

00110000  30   48      | Continuous controller #16   | 0-127     |     LSB

00110001  31   49      | Continuous controller #17   | 0-127     |     LSB

00110010  32   50      | Continuous controller #18   | 0-127     |     LSB

00110011  33   51      | Continuous controller #19   | 0-127     |     LSB

00110100  34   52      | Continuous controller #20   | 0-127     |     LSB

00110101  35   53      | Continuous controller #21   | 0-127     |     LSB

00110110  36   54      | Continuous controller #22   | 0-127     |     LSB

00110111  37   55      | Continuous controller #23   | 0-127     |     LSB

00111000  38   56      | Continuous controller #24   | 0-127     |     LSB

00111001  39   57      | Continuous controller #25   | 0-127     |     LSB

00111010  3A   58      | Continuous controller #26   | 0-127     |     LSB

00111011  3B   59      | Continuous controller #27   | 0-127     |     LSB

00111100  3C   60      | Continuous controller #28   | 0-127     |     LSB

00111101  3D   61      | Continuous controller #29   | 0-127     |     LSB

00111110  3E   62      | Continuous controller #30   | 0-127     |     LSB

00111111  3F   63      | Continuous controller #31   | 0-127     |     LSB

01000000  40   64      | Damper pedal on/off(Sustain)| 0=off     |    127=on

01000001  41   65      | Portamento on/off           | 0=off     |    127=off

01000010  42   66      | Sustenuto on/off            | 0=off     |    127=off

01000011  43   67      | Soft pedal on/off           | 0=off     |    127=off

01000100  44   68      | Undefined on/off            | 0=off     |    127=off

01000101  45   69      | Undefined on/off            | 0=off     |    127=off

01000110  46   70      | Undefined on/off            | 0=off     |    127=off

01000111  47   71      | Undefined on/off            | 0=off     |    127=off

01001000  48   72      | Undefined on/off            | 0=off     |    127=off

01001001  49   73      | Undefined on/off            | 0=off     |    127=off

01001010  4A   74      | Undefined on/off            | 0=off     |    127=off

01001011  4B   75      | Undefined on/off            | 0=off     |    127=off

01001100  4C   76      | Undefined on/off            | 0=off     |    127=off

01001101  4D   77      | Undefined on/off            | 0=off     |    127=off

01001110  4E   78      | Undefined on/off            | 0=off     |    127=off

01001111  4F   79      | Undefined on/off            | 0=off     |    127=off

01010000  50   80      | Undefined on/off            | 0=off     |    127=off

01010001  51   81      | Undefined on/off            | 0=off     |    127=off

01010010  52   82      | Undefined on/off            | 0=off     |    127=off

01010011  53   83      | Undefined on/off            | 0=off     |    127=off

01010100  54   84      | Undefined on/off            | 0=off     |    127=off

01010101  55   85      | Undefined on/off            | 0=off     |    127=off

01010110  56   86      | Undefined on/off            | 0=off     |    127=off

01010111  57   87      | Undefined on/off            | 0=off     |    127=off

01011000  58   88      | Undefined on/off            | 0=off     |    127=off

01011001  59   89      | Undefined on/off            | 0=off     |    127=off

01011010  5A   90      | Undefined on/off            | 0=off     |    127=off

01011011  5B   91      | Undefined on/off            | 0=off     |    127=off

01011100  5C   92      | Undefined on/off            | 0=off     |    127=off

01011101  5D   93      | Undefined on/off            | 0=off     |    127=off

01011110  5E   94      | Undefined on/off            | 0=off     |    127=off

01011111  5F   95      | Undefined on/off            | 0=off     |    127=off

-----------------------------------------------------------------------------

01100000  60   96      | Data entry +1               |          127

01100001  61   97      | Data entry -1               |          127

01100010  62   98      | Undefined                   |           ?

01100011  63   99      | Undefined                   |           ?

01100100  64   100     | Undefined                   |           ?

01100101  65   101     | Undefined                   |           ?

01100110  66   102     | Undefined                   |           ?

01100111  67   103     | Undefined                   |           ?

01101000  68   104     | Undefined                   |           ?

01101001  69   105     | Undefined                   |           ?

01101010  6A   106     | Undefined                   |           ?

01101011  6B   107     | Undefined                   |           ?

01101100  6C   108     | Undefined                   |           ?

01101101  6D   109     | Undefined                   |           ?

01101110  6E   110     | Undefined                   |           ?

01101111  6F   111     | Undefined                   |           ?

01110000  70   112     | Undefined                   |           ?

01110001  71   113     | Undefined                   |           ?

01110010  72   114     | Undefined                   |           ?

01110011  73   115     | Undefined                   |           ?

01110100  74   116     | Undefined                   |           ?

01110101  75   117     | Undefined                   |           ?

01110110  76   118     | Undefined                   |           ?

01110111  77   119     | Undefined                   |           ?

01110111  78   120     | Undefined                   |           ?

01111001  79   121     | Undefined                   |           ?

01111010  7A   122     | Local control on/off        |   0=on       127=off

01111011  7B   123     | All notes off               |           0

01111100  7C   124     | Omni mode off(all notes off)|           0

01111101  7D   125     | Omni mode on(all notes off) |           0

01111110  7E   126     | Poly mode on/off(all notes off)         *

01111111  7F   127     | Poly mode on(monomode;all notes off)    0

-------------------------------------------------------------------------------

*Note: This equals the number of channels, or zero if the number of channels

equals the number of voices in the receiver.