GETTING IN SYNC

In MIDI,  synchronization refers to various techniques of making several

time-dependent devices work together as one.  For example,  driving 3 or

4 synths with your favorite keyboard, or "synching-up"  a synthesizer to

keep in tempo with tracks on tape deck.  While all methods incorporate a

master/slave  scheme,  mechanisms such as FSK,  Chase Lock and MIDI Sync

are "tempo relative",  where  one  device  serves  as  conductor  to  an

orchestra of peripherals.  Other techniques such as SMPTE incorporate an

arbitrary  format  or  "fixed  time"  signal that has no relationship to

tempo.  Both timing methods refer  to  "striping",  as  the  process  of

individually  recording  a track of sync signals before other sequences,

songs or sounds are taped.

MIDI  Sync  is  a  "tempo  relative"   technique,   generally  used   by

"sequencer-like"  devices,  such  as  a  computer or rhythm composers to

drive multiple MIDI instruments.  At the heart beat of MIDI sync a  MIDI

clock  message,  referred to as MIDI clocks.  These MIDI Clocks are sent

at a rate of 24 clocks per  quarter  note,  creating  a  relative  tempo

usually expressed as Song Position Pointer (SPP), used to indicate where

a song or sequence begins and/or ends.

Frequency Shift Keying (FSK) is a simple "tempo relative" procedure that

uses  two  distinct  tones  to  form a signal.  While One tone fills the

tape,  the other marks the occurrence of a clock signal.  As the tape is

read,  the  sync  device generates a clock signal every time it sees the

transition from fill signal to  mark  signal.   Due  to  the  nature  of

digital  encoding,  FSK  can  be  written  for  various degrees of clock

resolution.  For example,  a program might use a clock resolution of  24

ticks  per beat to accommodate MIDI sync,  or adjust to various hardware

such as the MPU401's rate of 192  ticks  per  beat.   Unfortunately  the

on/off  nature  of  FSK  disallows  SPP,  means  sync-up can only at the

begging of a song or sequence.  Furthermore the frequency sensitive  FSK

offers   little   reliability   when   up  against  poor  tape  quality,

inconsistent speeds or volume distortion.

Chase Lock Sync (CLS)  can be best  describe  as  MIDI  Sync  for  tape.

Unlike  FSK,  CLS does not rely discrete tones but instead utilizes wave

forms.  Allowing for MIDI Clocks and SSP,  position information  can  be

read  by the sync device,  whereupon the sequencer responds by "chasing"

the current sequence to the location indicated.   This  usually  creates

split  second delay,  requiring that CLS be individually recorded before

other tracks.  Though CLS is limited to a clock resolution of  24  ticks

per  beat,  this  draw back is greatly over shadowed by CLS'  ability to

sequence from any location on the tape.   Furthermore,  the  ability  to

jump  to any position allows for recovery from disturbances such as drop

out.

The Society of Motion Picture and Television Engineers offers the "fixed

time"  method SMPTE.  While also using an analog signal,  SMPTE  differs

from  CLS  and  MIDI  Sync as synchronization is built upon "frames"  of

information instead  of  a  tempo  relationship.   The  "frame  address"

describes a group of bits defining a single location.  The rate at which

frames  are written to tape determine which of the SMPTE Format,  listed

below is used.

24 frame used by motion pictures.

25 frame used in Europe for video work because the television scan rate

is 25 frames per second.

30 frame non-drop used world wide for audio sync.

30 frame drop used in the US for video work as US color television scans

at  29.97 frames  per second. The term "drop" indicates that selected

frames  are periodically  dropped  to  yield  a  29.97  frames/second

scan rate.

In the US,  both 30 frame drop and non-drop are generally used, drop for

video/audio sync, and non-drop for audio sync.  Both implement an 80 bit

frame, written at a rate of 2400 bits per second,  each frame containing

a  time  stamp  address  and  frame number.  A time stamp of 01:37:22:19

would identify the frame address as frame 19, at 1 hour, 37 minutes,  22

seconds.

Being  an  established  standard  SMPTE allows a tape written by a SMPTE

writer can be read by any SMPTE reader that supports the  given  format,

whereas  FSK  and  CLS are subject to manufacturer's proprietary format;

making tape interchange impractical.  While offering the same advantages

of random positions starts and error recovery as CLS,  MIDI's acceptance

of  SMPTE  as MIDI Time Code messages allows SMPTE frame addresses to be

sent over MIDI cables in real time,  making SMPTE a powerful  triggering

mechanism.  Being a fixed format, SMPTE has the added advantage of tempo

changes at any time without the hassle of rewriting the sync track.

With  all  of  these  advantages the only disadvantage is not with SMPTE

itself, but the lack of SMPTE applications for the IBM PC.  This however

is  has  changed as developers  such as LTA Productions and  Twelve Tone

Systems  have been  hard  at  work upgrading  applications for the newly

released Yamaha C-1 Music Computer and Music Quest MQX-32 MIDI Interface

Allow me to Interrupt

Imagine 8 mail boxes in a row each with their own unique address,  and a

mail clerk that goes around raising the flags whenever information needs

to be exchanged.   This  analogy  roughly  describes  how  the  PC  uses

hardware  interrupts  (IRQ)  to  pass  information  to  and from various

interface cards.  Though most PC  users  are  unaware  that  their  MIDI

applications  use  IRQ2 in this way,  the ever expanding selection of PC

interfaces  has  produced  situations  where  more  than  one  card   is

contending for same IRQ.

One  can  quickly  tell  if  there  is an interrupt address problem when

software hangs during sequencing,  often locking up the entire system so

even  CTRL-ALT-DEL  won't  work.   You may be spared such frustration if

during installation or execution your software informs there is  no  MPU

card is present, even if you know one has been installed.

While  most  PC  MIDI  vendors  at  least  offer  fixes,  users may find

applications and interfaces that offer alternate  interrupts  easier  to

affect.   In either case,  if you are about upgrading your computer,  or

about to purchase new MIDI hardware or software,  call the vendor  first

and find out if they know how to handle the interrupt problem.

The  following table summarizes how interrupts are generally used in the

PC/XT/AT type machines (Industry Standard Architecture) :

IBM PC/XT (8086/88) IRQ Use     IBM AT (80286/386) IRQ Use

0 Timer                             0 Timer

1 Keyboard                      1 Keyboard

2 Reserved                      2 Orred summary of IRQ8-IRQ15

3 Secondary Async (COM2)        3 Async port 2 (COM2)

4 Primary async (COM1)          4 Async port 1 (COM1)

5 Hard disk controller          5 Parallel port 2 (LPT2)

6 Floppy disk controller        6 Floppy disk controller

7 Printer (not used by most)    7 Parallel port 1 (LPT1)

9 acts as IRQ2

If your PC/XT or compatible is having IRQ2 problems :

1.IRQ2 may be already in use by another interface such as a multi-

function card or an VGA or EGA video adapter;

2.installation of a hard disk renders IRQ5 unavailable;

3.if COM2 port is assigned, then IRQ3 cannot be used;

4.though IRQ7 is assigned for printer, the number of printer

adapters using them are very, very rare, making IRQ7 a good

alternative to IRQ2.

If you have IRQ problems and own an 80286 (IBM AT) or 80386 micro :

1.IRQ2 and/or IRQ9 may be occupied by another interface such as a

multi-function card or an VGA or EGA video adapter;

2.you cannot use IRQ3 if COM2 port has been installed;

3.if a second parallel printer port is enabled, IRQ5 is unavailable;

4.again, only a handful of printer adapters use IRQ5 or IRQ7. In

fact, the MS-DOS print spooler doesn't even support them, making

IRQ5 or IRQ7 good alternatives.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X