Hello, AWE32 lovers!

Here is a little application (for Windows 3.1x) to experiment with.

This was my second try to make this to work for everyone, but I can't promise

this works in Windows '95 because I don't have one ... it is too big

and too hungry for my 8M PC.

What's new in this second release (almost nothing):

-a few bugs corrected, sure not all of them (others still to come?)

-a more "conventional" way to access AWEMAN.DLL (I hope)

-added posibility to type the Hex digits with Shift+letter and NumPad even

if the "capture PC keyboard" checkbox is checked.

-modified right-click menu on sound font file-box

-added Note-velocity slider as another keyboard parameter

-MIDI channels used is now specified only in AWEPIANO.INI

-some other minor interface changes.

-MIDIROUTER driver usage info (posibility to record your moves)

-some shy programming info

(Now the old stuff..)

What AWEPIANO is trying to do:

- an Effects manager with chorus and reverb editing (you can make

new variations!)

- a piano-like keyboard on your own PC keyboard for testing and

playing with custom effects and other parameters (NRPNs, etc);

- a little MIDI player if you don't want to test those parameters

with your fingers :)

- a new AWE CP with SBK info, Digital Equalizer for EMU8000

(in real time-no need to restart Windows!) and Save/Load multiple

Mixer settings

- a MIDI driver that implements custom effects sending,bank loading&clearing,

changing synth emulation, and even mixer settings

(all these through System Exclusive messages, obviously);

Sort of a disclaimer

--------------------

Ok, this was the advertising part :), now back to reality:

-it is an overcrowded, somehow paranoic application

-it is free of charge , free of help :) but not free of bugs.

-it is made by only one guy; he is not a programmer but only a medicine

student so don't expect too much. I only want to share my work..

Anyway, if it works for you and you like it, please send me your opinions

and suggestions, or your ideeas about reverb paramters (see the adress  on

the end of this document)

First of all, this is my first windows application (as a Borland Delphi

project). I made it for my own use so I don't need a help file. Sorry for

not supporting a MIDI keyboard, I don't have such a device (if you send me

one, I'll do it :)

Special thanks to:

-----------------

-Vince Vu (I couldn't do anything without your docs!)

-Mathias Hjelt for his tough technical review

-Lysator guys for their AWE32 HomePage

Application NOTES (sorry, no help file)

=================

1.Keys Layout

2.Effects editing

3.Some interface specific tips

4.AWEPIANO.INI settings

5.Using the MIDIROUTER driver

6.Using my MIDI SysEx driver (optional)

7.Programming info

8.Known problems

1.Piano Keys layout

------------------

2 3   5 6 7   9 0   = \

Upper keyboard:  Q W E R T Y U I O P [ ]  (with Q being the C note)

S D   G H J   L ;

Lower keyboard:  Z X C V B N M , . /      (with Z being the C note)

2.Effects editing: beware !!!

-----------------------------

The effects editing process seems to be quite dangerous for your ears!

Be prepared to use the "panic" button (like in Cakewalk) to rapidly reset

the FX engine to a default state. After you change the values, you must

press the "Test" button to send your parameters.  I must set some limits on

those parameters, and I expect your support in that.

3.Some interface specific tips:

-----------------------------

-SpinEdit controls (edit controls with 2 arrows on the right-side)

can be changed with more speed if you right-click and drag up or down on

those arrows;

-a slider clicked outside the thumb will change only one unit (like the

AWE mixer sliders);

-on some sliders, double-click will force a predefined value

4. AWEPIANO.INI settings:

-------------------------

-you can disable the hints with ShowHints=0

-you can rename (or assign a name to) the reverb parameters by editing

the [Reverb parameters] section; attention: the names are those on the

left side of the equal sign, you don't need to add anything after it,

you only must replace those names.

-SBKEditor can be any Editor if you don't like creative defaults

-use UpperChan and LowerChan to choose the MIDI channels used by

the two virtual keyboards.

etc.

5.Using the MIDIROUTER driver

-----------------------------

This nice driver written by Zoltan Janosy can be used by my application by

choosing it as MIDI out while your sequencer (e.g. Cakewalk) is set to use

the same driver as MIDI IN. The only drawback I've found is that I cannot

send MIDI messages on two MIDI channels (lower and upper keyboards use the

same midi channel given by Cakewalk track). Anyway, I think is very helpful

to send NRPNs with my application slider and to record them in realtime.

6.Using my MIDI SysEx driver (optional)

---------------------------------------

From Windows Control Panel you chose to add an "Unlisted or updated driver"

and then choose the directory where you unzipped AWEPIANO.ZIP; then you

must restart Windows (you'll be asked to).

In Cakewalk you will look at MIDI Devices, to find out what Port my driver

will use. Then in the SysEx view you can load the .syx files saved by

AWEPIANO or made by your own, but don't forget to assign the Port for the

banks and to set them to "Auto" loading!

Sorry, in my first package I forgot to explain the System Exclusive messages

supported by my driver (AWESYSEX.DLL), and the driver had a serious memory

bug.

System exclusive messages supported:

1)  F0 41 57 45 33 32 43 <chorus values> F7

2)  F0 41 57 45 33 32 52 <reverb values> F7

<chorus values> and <reverb values> are bytes that correspond to

values edited by you in AWEPIANO Effects Manager. These SysEx messages

can be automatically generated by my program ('Save SysEx' button)

(technically speaking: the actual parameters you edit there are here

encoded to 7 bits in order to be accepted in the MIDI stream; the

driver will decode them, of course);

3)  F0 41 57 45 33 32 45 <synth_emulation> F7

where <synth_emulation> can be 00,01,02,03, corresponding to

(what else?) GM,GS,MT32,User synth;

4)  F0 41 57 45 33 32 55 <UserBankNumber> <.SBK filename (with path)> F7

the .SBK file specified will be loaded in <UserBankNumber> (1..127);

this kind of message also can be automatically generated in AWEPIANO

with the 'Save SysEx' Button on the AWE32 page;

The message

F0 41 57 45 33 32 55 <UserBankNumber> F7

will release (clear) the specified bank.

5)  F0 41 57 45 33 32 4D <mixer_values> F7

This kind of message can also be generated by AWEPIANO.EXE on the AWE32

page, with the right-click menu on the mixer panel.<mixer-values> are

a chain of 8 bytes (the plain values from my mixer)

Please note that messages 3) and 4) must be loaded with the Auto loading

option in SysEx View (Cakewalk). In other words, you cannot send them in

real-time, the midi playing will be interrupted and badly distorted!

The other messages can be used without this caution, but most probably

you'll hear some annoying clicks/pops when they are sent to AWE32.

I also noticed that if you send mixer sysex immediatelly after efx sysex

your efx does not sound right. I dont know why, but is clear that you must

always send mixer sysex first.

7.Programming info

------------------

-Vince Vu: The Un-official AWE32 programming guide

-Vince Vu: OMEGA.CPP - the source for his early MOD player.

These two docs are the BEST real AWE32 programming resources I've seen so

far on the net.

-AWEDIP - only as a theoretical background for accessing the AWEMAN.DLL. In

real life, the only normal way to use it is to replace AWEMANAGER messages

from the DIP with those "undocumented" messages used by Creative in their

AWECP.EXE (and by AWEVBank, of course)

Here is a little translation table I've used :)

(Pascal notation)

AWE_OPEN                $385

AWE_CLOSE               $386

AWE_GET_EFX             $388

AWE_SELECT_EFX          $389

AWE_GET_SYN_BANK        $38A

AWE_SELECT_SYN_BANK     $38B

AWE_LOAD_USER_BANK      $38C

AWE_GET_USER_BANK       $38D

AWE_CLEAR_USER_BANK     $38E

AWE_LOAD_USER_INSTR     $38F

AWE_GET_USER_INSTR      $390

AWE_CLEARINSTR          $391

For some other calls I've used the AWEDIP original messages.

For efx editing, mixer and digital equalizer I've used direct port access

(thanks to Vince Vu docs, and my own digging in some AWE32 apps).

8.Known problems

----------------

-unknown reverb parameters;

-chorus parameters doesn't seem to be too predictable.. (like the labels say)

-unknown initial values for NRPNs (these could be read from the file, but..)

-the Reset button from the "keyboard" tab doesn't work always. Sometime is

better to open and close the AWE MIDI driver instead,if you can (by selecting

it from MIDI Out)

-the MIDI player is very crude, I just ignore it so is not fully tested

-no support for new soundfont 2 specs

-ugly interface?

(Cheeky note:)

I could easily implement AWEVBank functionality - the ideea was great,

but I encountered two big problems:

1) AWE_LOAD_USER_INSTR call doesn't work for me how they said; it behaves

very strange, loading the hole bank not only the instrument..

2) Delphi seems to have a limit about the number of handles in an

application so I can't add the needed code in the same AWEPiano.

Comments & suggestions welcome (in fact, needed!)

E-mail to:

mihgeo@roearn.ici.ro

Geo Pertea

P.S.

I want to apologize for:

-my application bugs & problems

-the lack of a help file

-my english