WEDGE - Resident Software Wedge version 2.0

Release 09/25/93

WEDGE is a TSR which intercepts characters at the serial port,

and redirects them to the keyboard buffer. Application software

doesn't know whether the input came from the keyboard or a device

connected to the serial port, and it doesn't care.

Therefor, you can use WEDGE to have serial barcode scanners send

data to applications without specifically programming your app

for a scanner. (See the LEGAL STUFF at the bottom of this

document.)

WEDGE is also good for answering your phone, keying from the

"RING" signal executed by Hayes compatible modems, and assisting

in other serial-driven automation systems.

Because WEDGE excites every clock tick (about 18 times a second),

you might (and might not) notice a slight degradation in your

overall system speed.

SYNTAX

WEDGE C=comport B=baudrate P=parity W=WordLength S=StopBits

ComPort is either 1 or 2

BaudRate is 110,150,300,600,1200,2400,4800, or 9600

Parity is N, E, or O

WordLength is 7 or 8

StopBits is 1 or 2

Defaults are C=1 B=1200 P=N W=8 S=1

You can execute WEDGE by just entering WEDGE at the command

prompt, if you want the defaults. The defaults are used for

any parameters left out of the command line.

WEDGE takes up approximately 31K of RAM, and hooks vectors 08,

09, 0C, 13, 21, and 28. It uses an interrupt-driven 4K

communications buffer which is serviced independently of the

clock-driven keyboard-stuffing routine.

MODIFICATIONS

WEDGE is a modification of our standard TSR platform. The same

platform can do a number of jobs and background tasks, keying on

any hardware interrupt, including the keyboard.

If you have a specific modification you want us to make to WEDGE

for your application, call me or leave EMail on Compuserve. My ID

is [76207,1600] and my phone number in Memphis is (901) 682-0732.

TROUBLE REPORTS

Please report problems with WEDGE by Compuserve E-Mail to Rob

Cosgrove [76207,1600], the Internet to address:

76207.1600@compuserve.com, by FAX to (901) 681-0445, or as a last

resort by phone at (901) 682-0732. I'll need to know the type of

computer, the amount of memory, a list of drivers loaded in

CONFIG.SYS, a list of other TSR's loaded, the number of FILES and

BUFFERS in CONFIG.SYS, the operating system version, the network

type and version (if any), the type and speed of the input

device, and anything else that may help me diagnose your trouble.

If WEDGE is at fault, I'll modify it and ship you a new copy.

THE PRICE

WEDGE costs $49 per installation, or $98 per site. Try it before

you buy it to make sure it works with your hardware and other

software. If you want to use it, send your payment to:

Rob Cosgrove

654 South White Station Road

Memphis, TN  38117

Phone (901) 682-0732

You'll receive my thanks, a copy of WEDGE without the log-in

message and its delay, and my direct support if needed.

THE LEGAL STUFF

Although WEDGE has so far been a well-behaved TSR, there's always

the possibility it could confict with other TSRs and drivers. By

installing and running WEDGE, you relieve me of all

responsibility for any trouble it might cause. I guarantee that

at the time of its last upload from my computer, it is virus-

free.

I hate to have to do this, but in light of all of the recent

litigation resulting from barcode scanners misreading codes, and

clients dragging programmers into court to recover damages, I

have to. By installing and running WEDGE, you agree to the

following:

If WEDGE is used to redirect the input from a barcode scanner or

other type of scanner you agree to hold me harmless for any

losses resulting from the misreading of any data, or the loss of

any data or data stream. You also agree to defend me in the event

that your client names me as a defendent in any litigation

resulting from a loss caused by the misreading of data or the

possible failure of WEDGE to properly redirect data.

WEDGE does NOT interpret, decode, or translate barcode data.

I make no claims, expressed or implied, about the suitability of

WEDGE for any particular purpose. WEDGE is ONLY what its name

implies - a software wedge which redirects data from the serial

port to the keyboard buffer.