GENERAL DESCRIPTION

TController is a Delphi 2.0 Component that represents the important visual aspects of an industrial process controller.  The component displays the process variable (pv) and the set point (sp) and allows the user to increase or decrease the sp as desired.

Properties allow the application developer to change the color of the numerical display, the sp increment/decrement, the min/max sp, the number of digits appearing after the decimal point in the numerical display and the controller caption (“controller” in the picture above). The controller can be sized from a minimum of 90 x 90 pixels to 400 x 400 pixels.

Both the pv and the sp are properties so they can be set at design time.  A more likely scenario, however would be that the pv would be determined at run time by measuring the output of a process transmitter, e.g. thermocouple or flow transmitter, converting that data to engineering units and setting the pv property to the converted data.  Typically the sp value would result from the user incrementing or decrementing the displayed value with the up or down buttons.  This property would then be used to output a control signal, for example to a process heater, flow control valve, etc.  A PID (proportional/integral/differential) control loop could be implemented in software by comparing the sp and pv and outputting a control signal derived from the difference between the two.  Or a hardware PID controller could be used with the controller property sp used as the PID set point input.

The user can increase or decrease the sp by pressing the up or down button, respectively, either one click at a time or by holding either down.  In the latter case, the sp changes will be small at first, then gradually accelerating, just as in most common hardware controllers.  So a large change in the sp can be reached much more quickly.

An event tied to changes in the sp is made available to the application developer.  Typically this event would be used to output a control signal proportional to the new sp.  This event is triggered 1/2 second after the user stops changing the sp.  This eliminates triggering an excessive number of events as the user repeatedly clicks on either or both of the sp buttons, or as he/she holds one of the buttons down.

A “mode” button (red button in the center) is tied to an event so the application designer can implement most anything he/she wishes.  For example, the mode button click could be used to pop up a form allowing the user to change things at run time.

INSTALLATION

Install the following three files:

controlr.pas

controlr.res

CONTROLR.dcr

to your ..\Delphi 2.0\Lib directory.  From the Delphi menu, select Component|Install.  Click on Add and Browse for the controlr.pas file you installed.  Then click OK.   Eventually, the component will appear on the Samples page of your component pallet.

KEY PROPERTIES

Caption -- The controller caption appears on the front face of the controller.  In an application with multiple controllers, this is a good way to differentiate between them all and tell what each is actually controlling, and maybe give the units of the quantity.

Digits -- This property determines how many digits appear after the decimal point; the range is from 0 to 3.  You might set this according to the maximum sp and desired control resolution.

LEDColor -- Sets the color of the sp and pv digital displays.

MaxSp/MinSp -- Sets the range over which the user can change the sp when pressing the up or down buttons.

PV -- The displayed process variable.  As explained above, this would likely be set from some measured quantity.

SP -- The displayed set point.  This can be set with code, for example for startup conditions.  More likely, you would let the user change this with the up/down buttons.

SPIncrement -- Determines how much each click of the up or down button changes the sp.

KEY EVENTS

OnChange -- this event occurs 1/2 second after the user releases either the up or down buttons.  Use an event handler here to take the new value of sp and output a control signal based on sp.

OnModeClick  -- This event occurs whenever the user presses the “mode” button.  That’s the one between the up and down buttons.  You can use this for allowing the user to change parameters, or whatever.