GD64: Brief manual and impressions

(How to Send and Receive Files to/from GD64)

Written by your Favorite Knight.

[01-February-1997]

DOCTOR V64!

Brief instructions & a review by someone WHO ACTUALLY HAS ONE!

Parts included:

1. AC/DC adapter (220V HK type plug..)

2. V64

3. cart slot interface (white) to connect to N64 expansion port

This piece is very tight & almost impossible to remove

in the beginning.

4. V64 cart slot cover/lid (white)

5. Lockout Chip passthru.. or whatever you want to call it. (black)

so the cart lockout chip can be read & allowing you to play the

games.

OTHER ITEMS RECOMMENDED

1. TPC (pc to DV64 upload/download tool)

2. DB25 Parallel MALE<->MALE cable (same cable used for SWC)

3. Any cart!! Required for the lockout chip.. just like the SUPER MAGICOM.

This is not exactly the best format, but anyway:

BOX size = approximately the same as N64.

Input A/V cables from N64 to V64. V64 has its own port for A/V, which

you connect to a TV/MONITOR.

NTSC/PAL selection switch in the back for the Video Signal.

==========================================================================

OPERATING with the PC!!!!

READING CARTS TO PC:

1. connect the power supply to the V64

2. connect the plug into the electrical outlet (be sure to use proper voltage

& plug for your country)

3. connect a DB25 cable (same as the SWC<->PC) to the DV64.. and connect the

other end to the PC.

4. remove the Lid..

5. Insert a cart in the cart slot.

6. press FR & FF buttons to transfer cart image to the DV64

7. go to a DOS prompt & run TPC.EXE

8. type /GAMENAME

(where game name is whatever you want to call the file)

9. if you did anything right, WAIT! WAIT! WAIT! & WAIT!!!

the "." counter is approx 3 lines long.. for 64Mbit carts

10. if everything went right, you got the image. :)

(You do not need to turn on the N64 for this, or even have it attached

to the Copier!!)

SENDING FROM PC to DV64:

1. on the DV64 cart slot, insert the white cart slot interface.

2. put your N64 on the DV64 so that the N64 expansion port connects to the

DV64 cart slot interface.

3. insert the lockout chip passthru module in the N64 cart slot and then

insert a cartridge in the slot.

4. turn on the DV64 and run TPC on the PC

5. at the prompt, enter the GAMENAME and wait! wait! & WAIT!

then the file will finally transfer and the DV64 will the you the

game is ready to go!

6. turn on your N64 & the game comes on..

OVERALL OPINION.

This is the first generation machine. No one else has anything ready

so cant really complain. The entire operation is based on buttons. The

on-screen text is not processed thru N64. For programming purpose, you

dont have to shell out over $30,000.00 and waste time with nintendo

for a developers license. Much better than the SONY YAROZE Bullshit

which goes for TOO MUCH!!! Nice for hobbyists. Only drawback,

NO DEVELOPMENT TOOLS.. so make your own Assemblers, Debuggers,

Disassemblers and hack out your own registers. If there is interest,

contact me and we can setup something.

Yes, the grammers and all the other things are not so clear but enough

for most people to understand. Until next time, cya.

-Your favorite Knight :)