Modifying your Doctor and how to backup

How to modify the latest doctors which cannot load games:

Simple. Unscrew the V64. On the PCB you will notice some jumpers.

They should look like this:

BEFORE --> | |

|

Now just change it that it looks like an "U"

AFTER ---> |_|

That's it.

How to upgrade your bios:

1. Either burn the bios onto CD (quite uneconomic) or send it directly

via TPC into ram

2. Press the buttons "CH" & "Play" at the same time, a screen will appear

showing the current bios-version.

3. Now you should confirm the Bios-Upgrade by pressing the "menu"-Button.

How to backup a game:

1. You dont actually need the N64 to backup the games !

2. Plug the cart into the upper slot of the V64 and press

"FR" & "FF" at the same time.

3.The Screen will show how many megabits the game features and the name of

the cart. The game will be saved to ram automatically.

4. Afterwards you may download the game from ram via cable to the PC, or

plug the V64 on your N64 and start playing.

It doesnt work ? Useful hints:

-It is said that the printer port of the PC must be set in the PC-bios as

"normal", and not "EPP", "ECP" or "SPP".

-The cable from PC to the V64 should be no longer than 2m

-It is also said that some printer ports cannot communicate with the

doctor (i guess the older ones not being Uart1660).

Regards to Sigma7 for providing this Information

Actraiser.