Example PlayStation Disc Layout

Blocks          Description

------          ------------

000000-000015   Data track boot blocks        (see NOTE 1)

000016-024520   Data track program area       (see NOTE 2)

024521-024670   Postgap for data track        (see NOTE 3)

024671-024820   Pregap for first audio track  (see NOTE 4)

024821-048326   Audio track #1

048327-048476   Pregap for second audio track (see NOTE 5)

048477-072485   Audio track #2

.

.

.

191281-191430   Pregap for ninth audio track

191431-214349   Audio track #9

214350-??????   Leadout track                 (see NOTE 6)

NOTE 1: The first sixteen blocks of a PSX disc are the "boot" blocks.

They contain some territory information, but I don't know if this is

the actual data that is used by the "territory lockout" code.

NOTE 2: The rest of the data track is where the actual program (game)

is stored. The track is encoded in CDROM-XA format using both Form-1

and Form-2 sectors. The file system is in ISO9660 format.

NOTE 3: When any data track is followed by an audio track, it must

end with a postgap of at least 150 sectors. On all of the PSX discs

that I have analyzed, this postgap is comprised of 150 sectors of

zeroed data. The purpose of the postgap is to provide a "buffer zone"

between the real data in the track and the subsequent audio track.

This buffer is used because some CD readers can't read the last few

blocks of a track if they are followed by audio sectors.

NOTE 4: When an audio track follows a data track, it must start with

a pregap of at least 150 sectors. On the PSX discs that I have analyzed,

I have seen pregaps of both 150 (2 seconds) and 300 sectors (4 seconds).

This pregap serves the same purpose as the data postgap... to physically

separate the data and audio sectors to avoid reading errors (data and

audio sectors are usually read with separate commands and each command

can only read a specific sector type).

NOTE 5: The pre-gap for all audio tracks after track #1 seems to be

either 0 or 150 sectors for all PSX discs. PSXCOPY.EXE will copy

these gaps, but it doesn't bother to encode them as gaps in the

subcode.

NOTE 6: The LEADOUT track is written by the recorder automatically

(except for the Philips) and is not under user control. It is not

possible to read this area of the disc with a normal CDROM reader

or recorder.