3DGV Version 0.2 Beta

(c) Copyright 1991 John Swenson

Parts from the PoVRay raytracer (c) Copyright 1991 The PoV Team

3DGV is a 3D GIF viewer using the Sega 3D glasses. It reads either two

GIF files or a single MIG (Multi Image Gif) file. The two images are both

loaded into video memory, and the screen is toggled quickly back and

forth between them, while the lenses on the glasses are alternately darkened.

Thus one eye sees one image and the other sees the other image giving a

3D effect with appropriate images.

3DGV [<left GIF> <right GIF>] | [MIG GIF] -D<disp_type> -S<vidmem>

The first image in the MIG is the left, the second is the right

Your video board must have twice the usual amount of memory

necessary for a particular video mode, ie. 1 meg for 640x480

Sega 3D glasses must be attached to the parallel port

-d is followed by 2 characters, first is type second is resolution,

it must be present, currently there is no auto detect.

-s is followd by "5" or "10", -s5 means 512k and -s10 means 1024k

video ram. This MUST be specified to use SVGA modes.

-p specifies the parallel port -p1 or -p2. You can also specify

a specific port address: -px3c7 uses port 3c7 hex.

-i "inverts" the two images, right goes to the left eye and vice-versa.

video modes and types:

First char                   second char

1   Basic VGA                1 320 x 200

3   Tseng 3000               2 320 x 400

4   Tseng 4000               3 360 x 480

7   Video7                   4 640 x 400

9   Paradise                 5 640 x 480

C   Chips & Tech             6 800 x 600

F   Trident

to specify a 640 x 480 mode on a Tseng 4000 with 1meg use:

-d45 -s10

If your SVGA board is not listed, the best you can do is either

320 x 200 or 320 x 400 with BASIC_VGA. There is an auto resize feature

which comes into play if the image is bigger than the mode, thus you can

always use 320 x 400, no matter how big the original is.

It looks like ATI boards cannot use their SVGA modes with this program. It

appears to be a limitation of the hardware. Unless I find out otherwise,

the best you can do with an ATI is 320 x 400, BASIC VGA.

The 640 x 400 mode for a Tseng 4000 is currently set for STBs mode, which

is different than everybody elses 640 x 400. (I have an STB board!)

The 800 x 600 mode is only implemented on Tseng boards for now.

Unless VESA changes things, there will not be any VESA support either.

The Sega glasses are hooked up to the parallel port pins 4 and 5 if you

have two control wires, and to pin 4 only, if you just have one control

wire.

While the image is being displayed you can press the 'i' key to invert

the images, this does the same thing as -i, only you can do it on the

fly.

Future versions will hopefully add more SVGA boards, and add auto type and

memory detect. For now you MUST specify the parameters.

This is still an early version, let me know of any bugs, problems or

enhancements you would like.

John Swenson, CIS 75300,2136