SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0

USER'S GUIDE TO SYSTEM68

Version 1.0

Introduction

SYSTEM68 has been designed for developing and testing programs written in

assembly language for the Motorola MC68000 processor.  The  system  includes a

Vi-type editor  for creating  source code,  a cross-assembler  consistent with

most standard MC68000 assemblers and  a  simulator  which  provides a 16K-byte

address space for programs and data. The system provides menu driven access to

the  various  system  components as well as selections for performing some DOS

functions. The menu also allows access to a DOS shell which provides easy exit

from  and re-entry to SYSTEM68.  The editor includes a built-in syntax  filter

which will automatically flag MC68000 assembly language syntax errors.

This manual is divided into five sections as follows:

Section 1 - General Information

Section 2 - The Main Menu

Section 3 - The INVITE Editor

Section 4 - The ASM68 Cross-Assembler

Section 5 - The S68 Simulator

Section 6 - Warranty Information, License Agreement,

and Registration Form

In addition to the information provided herein,  there is some limited on-line

help within the SYSTEM68 itself.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page  2

Section 1

General Information

SYSTEM68 consists of the following collection of programs and files:

GO68.EXE    - main menu access program

INVITE.EXE  - the INVITE editor

ASM68.EXE   - the MC68000 cross-assembler

SIM68.EXE   - the MC68000 simulator

INVITE.HLP  - the editor help screens

SYS68.DOC   - this SYSTEM68 documentation file

HANOI.ASM   - a sample program (Towers of Hanoi)

All executable programs (.EXE files) and data files belonging to SYSTEM68

should reside in the same  directory or  subdirectory of the default drive.  A

copy of COMMAND.COM should reside in the  root directory of the default drive.

To create and/or access your own files in other drives and/or directories, in-

clude  the drive specifier,  if different from the default,  and complete path

name to the file when a filename is required.

It is assumed throughout this guide that the reader is generally familiar

with the use of the Vi editor, has some experience programming at the assembly

language level and has access to the MC68000 PROGRAMMERS'S REFERENCE MANUAL or

equivalent text.

Revisions with enhancements are  planned for the future.  Anticipated en-

hancements include a Wordstar-like editor  to  replace the current editor,  an

assembler option to generate S-records, a larger simulator memory by employing

virtual memory techniques, macro assembly capablilities, additional diagnostic

and debugging tools,  inclusion of MC68020 and MC68030 instructions,  enhanced

programmable I/O capabilities and whatever else the authors may dream up along

the way.

Comments and inquiries concerning SYSTEM68 are invited and may be addres-

sed to either of the authors, whose mailing addresses appear below:

Stephen J. Sheel                           John M. Samaras

1013 Greenbriar Avenue                     Route 9 Box 85R

Keene, TX  76059                           Valdosta, GA  31601

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page  3

SYSTEM68 was written and compiled under  Turbo Pascal 4.0.  SYSTEM68 will

run on any IBM PC, XT, AT, PS/2  or  true compatible  running PC/MS DOS 2.0 or

higher. A minimum RAM of 256K free bytes is required.   The system is designed

to provide optimum display on both monochrome and color monitors.  No graphics

adapter is needed.  A hard disk is strongly recommended, though not required.

IBM is a registered trademark of the International Business Machines Corp.

MS-DOS is a registered trademark of MicroSoft, Inc.

Turbo Pascal is a registered trademark of Borland International.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Section 1 (continued)             Version 1.0                          Page  4

Section 2

The Main Menu

The main menu provides quick access to the the editor, the assembler, the

simulator �and �several utilities, �including a DOS shell.  �The main menu �is

invoked by entering GO68 at the system prompt. After a startup message is dis-

played, the screen will appear as follows:

****  Options Menu  ****

E : Editor

A : Assembler

S : Simulator

V : View a file

C : Change directory

L : List directory

D : Dos Shell

Q : Quit

Choose:

The �first three choices in the list  above are explained in Section 3 through

Section 5 of this document. The remaining options are discussed below.

The �V option allows you to view any file.  �You are prompted for a �file

name �then �given �a choice of viewing it on the screen or sending it �to �the

printer.  Screen viewing is paged so that you may read at your leisure.

The �C �option �allows you to change to any subdirectory on �the �default

drive.  �Please remember that access to the editor, assembler or simulator can

be accessed only from directory in which those files reside. So if �you change

directory for whatever reason, be sure to change back to the one in which you

have SYSTEM68.

The L option allows you to list the contents of any directory. �You �will

be prompted for a directory mask. The default is *.*, but you may also provide

a �mask �of your own choosing. �You may include a path as well the �wild �card

characters * and ?.  �Just be sure to follow DOS conventions; �otherwise, �you

may receive strange error messages.

The �D option provides a DOS shell for executing any valid DOS �commands.

Please �note �that �COMMAND.COM must be in the root directory of �the �default

drive and, while in the DOS shell, you should not change the default drive.

The Q option is used to quit SYSTEM68 and return to DOS.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page  5

Section 3

The INVITE Editor

This �editor has been modeled after the popular Vi editor which �operates

under �Unix. �INVITE uses many of the Vi commands in much the same way. �There

are, �however,  some differences between the two editors, �the most notable of

which is inclusion of the File and Block management  commands.  �Another �dif-

ference is that a count (number of characters, �lines, words, etc.),  �if any,

must precede the command with which it is to be used. �Thus, �5dw is legal but

d5w is not. �When no count is given, 1 is assumed. �If the count is too large,

INVITE �will go or do as much as it can in most instances  without �squawking.

If a count is not meaningful with a given command, �the count is ignored �when

the command is executed.

The editor operates in two modes. Open, or edit, mode, which is entered by

using commands such as i, a, s, o, I, etc., �allows the user to make additions

and substitutions to the text. �Visual mode allows the user to move the cursor

from one line to another.  Visual mode encompasses all  of the cursor movement

commands (j, �k, �l, 25G, !, etc.),  the Block commands  (BB, BE, �BM, �etc.),

the File commands (^FS, �^FL),  �as well as the search commands (/, n, N) �and

redraw (^R).  The commands  delete character (x) and delete line (dd) work in

visual �mode, �even though they are technically editing commands.  �The filter

switch (^S - see next paragraph) works in both open and visual modes.

In �addition to the usual editing functions  INVITE also checks for �syn-

tax errors  (Motorola processor MC68000 �assembly language)  �in text lines as

they are entered. �Initially the filter is on by default. �In order to provide

maximum usefulness for INVITE, �however,  �the user may turn the filter on and

off �at �will.  �Either the F3 �key or the command ^S will toggle �the �filter

between �on �and off.  �F3 �or ^S may used at anytime in both open and �visual

modes.

The cursor-movement commands may �be �preceded �by �the �number of lines,

characters �or words to move. �If the number is too large (you can't go beyond

the first and last lines of text, �for example),  �a warning beep is given and

the �cursor �is �moved to the extreme in the direction specified.  �If �the �G

command is preceded  by a number,  the number is interpreted as a line number.

Thus, �25G moves the cursor to line 25 �(or to the last line if there are less

than 25 lines).

Most of the INVITE commands can be ESCaped from.  In particular, multiple

character commands, including those preceded by a number,  can be stopped by

pressing the ESC key before finishing the command. �File operations, �such �as

saving, loading or deleting, can be terminated by entering nothing in response

to the request for a file name.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page  6

Section 4

The ASM68 Cross-Assembler

The ASM68 cross-assembler provides a complete set of assembler directives

including EQU, ORG, DC, �DS, and END; can assemble all instructions in the MC-

68000 ��instruction set; �allows both comment and blank lines; �is capable �of

evaluating algebraic expressions; produces a standard format �listing �of �the

assembled �code and generates a readable object code file. �In the event �that

errors are discovered during assembly, a �list of those errors is written to a

file called ERROR.LST as well as to the screen.

Each �line in the source code file may contain only one assembly language

instruction.  The source line format is

[label]  mnemonic [operand(s)] [comment]

where the label �and comment fields are optional.  �The operand(s) �field will

contain no, one or two operands as required by the particular mnemonic appear-

ing in the line.  A label, if there is one, must begin in column 1 of the line

of source code. The mnemonic must start in column 2 or higher when no label is

present.  �At least one space must separate neighboring fields.  A �line which

contains only comment must start with an asterisk (*) in column 1.

Size �(i.e., �byte, �word, �longword) �specification is �accomplished �by

appending ".B", �".W", �or ".L", �respectively, �to the mnemonic, where appro-

priate.  ��The ".W" �is assumed when no size is given, �except of course �when

other �defaults �over-ride, �as in the case of the LEA instruction, �which �is

always a longword operation. �In such a case, �LEA.B and LEA.W are treated �as

errors.

Numeric values may be specified in either decimal or hexadecimal. Decimal

numbers must be in the range -32768 �to 32767 and contain no commas or decimal

points. �Hexadecimal numbers should be preceded by the dollar sign symbol ($).

The range for hexadecimal numbers is $00000000 �to $FFFFFFFF. �Leading �zeroes

are not required.  Character strings must be sandwiched between single quotes.

Characters are converted to their ASCII hexadecimal values during assembly.

Algebraic �expressions consist of some combination of numbers �and/labels

joined  by the symbols for addition (+), �subtraction (-), �multiplication (*)

and division (/). �Expressions �are �evaluated �from left to right with multi-

plication and division done first,  followed by addition and subtraction. This

precedence may be circumvented through the use of parentheses.

All of the MC68000 �addressing modes are recognized by ASM68. �The assem-

bler format for these modes is identical to that appearing in the MC68000 PRO-

GRAMMER'S REFERENCE MANUAL. �In addition the symbols SP, �USP, �CCR and SR are

understood �as �active �stack pointer, �user stack �pointer, ��condition �code

register and status register, respectively.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Section 4 (continued)             Version 1.0                          Page  7

When the assembler is invoked, �you will be prompted for the name of �the

file to assemble. The file name should follow the DOS conventions. Assume that

ASM68 performs an error-free assembly the file HANOI.ASM. The object code will

be �placed �in �a �file called HANOI.OBJ and the �listing �in �a �file �called

HANOI.LST. �Errors detected during assembly are written to the file ERROR.LST.

No .OBJ and .LST files are created.  �You should be aware that the object code

file is readable, and hence can be altered through editing.  �If you know what

you �are �doing, �feel free to make alterations to the object code �using �the

editor. A word of caution, however: BE CAREFUL!

NOTE: �if you use an editor other than that which comes with SYSTEM68 �to

create a source �code �file, �it should be a ordinary text editor.  �Word pro-

cessors such as WordStar or WordPerfect are specifically �not �recommended for

use with ASM68 �because they contain special purpose characters which �can not

be interpreted by the assembler.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page  8

Section 5

The SIM68 Simulator

SIM68 �allows the user �to �load �and �execute assembled Motorola MC68000

machine language programs. �The assembled code must be �a readable object code

file in the same format as produced by the �ASM68 cross-assembler described in

Section 4. �The SIM68 �command set gives the programmer total control over the

execution process.  �Register and memory contents may �be displayed or changed

and instructions �may be executed one at a time (trace mode on) or continously

(trace mode off). �Breakpoints may be set to stop execution at any given point

in the program. A value given in response to a request for an address or regi-

ster or memory contents may be in upper or lower case so long �as the value is

otherwise a valid hexadecimal number.  Exception processing provides a display

of �register �contents, �the nature of the error causing �the exception and �a

trace of the last ten instructions executed.

The complete command set is shown below:

A - set/display Address register contents, prompt follows.

B - set a Breakpoint, prompt follows.

C - Clear a breakpoint, prompt follows.

D - set/display Data register contents, prompt follows.

F - read an object File from disk.

H - display this Help menu.

I - list Instructions in unassembled form, prompt follows.

L - List memory contents, prompt follows.

M - change Memory contents, prompt follows.

P - set Program counter, prompt follows.

Q - Quit simulator.

R - display current Register contents.

S - Stop execution now.

T - toggle Tracemode switch.

U - dUmp memory contents, prompt follows.

W - Write memory contents to disk.

X - eXecute program beginning at current PC.

Commands �may be entered in either upper �or �lower �case letters. �Any of the

above commands may be used when the "Cmd" prompt is displayed. The "S" command

is used while a program is executing. �The "S" �command acts as a break key to

stop, for example, a program caught in an infinite loop.

Input �and �output �in the conventional MC68000-based machine is done �by

memory mapping.  �Memory mapped I/O is particularly simple to perform and gen-

erally involves moving �a �value to a predetermined address to get the process

started. Since SIM68 �only simulates the  68K environment, another approach to

programmed �I/O �has been taken.  �In the original MC68000 ��instruction �set,

Motorola left two op code groups unassigned.  �To facilitate I/O in SIM68, two

of those op codes have been implemented. �Output �is accomplished through �the

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Section 5 (continued)             Version 1.0                          Page  9

"instruction" �$F000 �while input is done using the "instruction" $A000.  �For

assembly purposes the user should encode these instructions using a DC.W.

The output emulator $F000 takes the character to be printed from the low-

order byte of D0.  �The value contained there is assumed �to be ASCII encoded.

For example, �to print the letter "A" the low-order byte of D0 �should contain

$41 ��at the point at which the $F000 �"instruction" �is fetched and executed.

Output is to the screen at the current cursor position. �All screen control is

the �responsiblity �of the programmer. �The values to use for �line �feed �and

carriage return are usually $0A and $0D, �respectively.  �A �sample routine to

print the message "Welcome to the World of SIM68" is shown below.

LEA       MESSAGE,A0     * init. to start of message

OUTLOOP   CMPI.B    #$00,(A0)      * $00 is end-of-text marker

BEQ       DONE

MOVE.B    (A0)+,D0       * D0 <= char to be printed

DC.W      $F000          * output emulator

BRA       OUTLOOP

DONE       .          .

.          .

MESSAGE   DC.B      'Welcome to the World of SIM68',$0A,$0D,$00

The input emulator $A000 �places a character in the low-order byte of D0.

Thus, ��any previous contents should be moved elsewhere if they will be needed

later. Input is a single character from the keyboard device. To input a string

the programmer should code �a loop structure, �as shown below, �which is �ter-

minated when the RETURN key is pressed.

MOVEM.L   D0,-(A7)       * save contents of D0

LEA       BUFFER,A0      * init. to start of buffer area

INLOOP    DC.W      $A000          * input emulator

MOVE.B    D0,(A0)+       * character goes to buffer

CMPI.B    #$0D,D0        * string ends with CR

BNE       INLOOP

MOVE.B    #$00,(A0)      * move in end-of-text marker

MOVEM.L   (A7)+,D0       * restore D0

.          .

.          .

BUFFER    DS.B      255            * input chars are put here

SIM68 �provides the user with a very clean method of program termination.

At the protected location $FFFF FFFE resides �an instruction to return control

to SIM68.  Normal termination of a user program is accomplished by JMP  $FFFE.

Short �form addressing is used here but long form ($FFFFFFFE) ��works just �as

well.  �The message "Execution halted at 0000�0000" �will be displayed on �the

screen. �If the program starts at address $0000 0000, �as it should, �the user

can rerun his program without having to reset the program counter.

SYSTEM68              MC68000 Assembler/Simulator System          June 7, 1988

Version 1.0                          Page 10

Section 6

WARRANTY STATEMENT

The SYSTEM68  MC68000 Assembler/Simulator,  Copyright (C) 1988 by John M.

Samaras  and  Stephen J. Sheel,  is provided  AS IS  without any warranty, ex-

pressed or implied.  SYSTEM68 is licensed, not sold.  The owners shall have no

liability for damages of any kind arising from the use or modification of this

software.

SYSTEM68 MC68000 Assembler/Simulator System

LICENSE AGREEMENT

SYSTEM68: Copyright (C) 1988, John M. Samaras and Stephen J. Sheel. All

rights reserved.

You are free to use, copy and distribute  SYSTEM68  for noncommercial use

provided that

NO FEE IS CHARGED FOR USE, COPYING OR DISTRIBUTION,

and that

IT IS NOT MODIFIED IN ANY WAY.

Commercial and/or educational use are prohibited  without written consent from

the owners. Commercial and educational site licenses are available on request.

Write to either of the addresses below for additional information.

If you find SYSTEM68 useful  for  development and testing of MC68000 pro-

grams, a registration fee of $25 would be appreciated. With each contribution of

$45 or more you will be registered to receive a diskette with the next version

of SYSTEM68 when available.  A registration form is provided on the next page.

Please be sure to indicate the version number of your copy on the form.

SYSTEM68

Motorola MC68000 Assembler/Simulator System

* * * *  R E G I S T R A T I O N    F O R M  * * * *

Name ____________________________________________________________________

Address _________________________________________________________________

City ______________________________   State _________   Zip _____________

Please provide the information requested below:

Version number of your copy: ______________

How received?    ___ bulletin board    ___ individual    ___ computer store

___ educational institution    ___ other: _________________

I am enclosing the following fee:

___ $25.  Please register my current copy and let me know about future

updates.

___ $45.  Please register my current copy and send me the next update

as soon as it becomes available.

Send completed form along with your contribution to either address below:

Stephen J. Sheel                           John M. Samaras

1013 Greenbriar Avenue                     Route 9 Box 85R

Keene, TX  76059                           Valdosta, GA  31601

Phone (817)556-0510                        (912)559-4982

Note: Please do not send cash through the mail.  Money order, certified check,

or personal check is acceptable.