User's guide to single-precision floating-point library FLOAT.LIB.

Assembly Language Version.

Copyright Bob Kline 1988

General notes:

One of the frustrations which assembly-language pro-

grammers occasionally encounter is the lack of in-

structions for dealing with floating-point operations.

Even in architectures where a math coprocessor is

designed to handle floating-point work efficiently,

the problem is often still not resolved, as it fre-

quently turns out that most machines are not equipped

with the extra math chip.  A good example is the IBM

PC family and clones, which can use a math coprocessor

to do floating-point work in hardware.  Only a very

small percentage of these computers are equipped with

such a coprocessor.

This library consists of procedures for performing

basic single-precision floating-point operations in

assembly language programs written for the 808x family

of computers not equipped with a math coprocessor.

The procedures have been built for use with the 'small'

memory model, that is, for programs which use 16-bit

pointers and near procedure calls.  It should not be

an inordinantly difficult task to rework the code for

other models, as (with the exception of the procedures

which take in or produce strings: ATOF, FTOA, and FTOE),

all parameters and results are passed in hardware

registers, and the new simplified .MODEL directives (a

feature of MASM 5.0) have been used.  Although at this

point only the most fundamental operations have been

provided, it is hoped that a subsequent student or

team of students will build on this foundation, adding

some of the geometric and exponential functions.

All floating-point values handled by the library are

4-byte single-precision reals in the format established

by the IEEE standard.  The details of this format are as

follows: the sign is stored in the most significant bit

(bit 31) of the 32-bit representation (set to 1 for a

negative number, reset to 0 for a positive value); the

exponent is stored as an eight-bit value in bits 23

through 30, and is biased by adding 127 to its actual

value -- that is, a (binary) exponent of 0 is

represented as 127, -1 as 126, 1 as 128, and so on; the

fractional portion of the mantissa is represented in

bits 0 through 22; except for a value of 0.0, which is

represented with all 32 bits of the float reset to

zeros, the integer bit of the mantissa is understood to

be 1, and does not therefore need to be explicitly

stored as part of the number.  In terms of decimal

representation, this format can handle a range from an

exponent of -38 through +38, with a precision of at

least 7 (possibly 8) digits.

Names for the procedures have been chosen which would

not conflict with those used by the math coprocessor.

So, for example, the routine to add to floating-point

numbers is called F_ADD rather than FADD.

The following descriptions give details about how each

of the procedures works, including which registers are

used for incoming and outgoing values, assumptions made

by the procedures, and behavior with error conditions.

The list is headed by a description of _errno, a global-

ly available variable used as a flag by several of the

procedures to indicate that an error has occurred.

A companion file is provided, FLOAT.INC, which can be

included in the source code for modules using this li-

brary, giving the assembler access to the table of ex-

ternal names which it will need for finding the library

routines.

*---------------------------------------------------------------*

EXTRN _errno:WORD

Used by several of the math routines to signal that an

error has taken place.  Two possible values are EDOM

(33), which indicates that an invalid parameter has been

passed, such as a divisor of zero; and ERANGE (34),

which indicates that the result does not fall within the

range of values which can be represented by the type

specified for the result.  The calling routine should

reset _errno to zero before the math operation is invoked

to ensure that a subsequent check is not looking at the

results of an error from an earlier operation.

*---------------------------------------------------------------*

ATOF

Converts the string pointed to by SI to a floating-

point value.  Leading spaces are skipped.  The string

contains an optional sign, followed by a series of

decimal digits, possibly with a decimal point at any

point in the series of digits, followed optionally

by 'e' or 'E' and a (possibly signed) integer

indicating the exponent.  The calling function in

responsible for making sure that the input string

actually contains a valid floating-point value.  On

return DX:AX contain the 4-byte real result; in

addition, the values in registers BP, SI, DI, BX,

and CX are destroyed.

*---------------------------------------------------------------*

ITOF

Converts the 2-byte integer value passed in the AX

register to a 4-byte real and stores the result in

DX:AX.  In addition, the value in CX is changed.

*---------------------------------------------------------------*

F_ADD

Adds the 4-byte real passed in CX:BX to the 4-byte real

passed in DX:AX and stores the result in DX:AX.  If the

result will not fit in a 4-byte real, _errno is set to

ERANGE, and the resulting value in DX:AX is

unpredictable.  In addition to DX:AX, registers BX, CX,

SI, DI, and BP and changed.

*---------------------------------------------------------------*

F_SUB

Subtracts the 4-byte real in CX:BX from the 4-byte real

passed in DX:AX, storing the result in DX:AX.  If the

result will not fit in a 4-byte real, _errno is set to

ERANGE, and the value in DX:AX is unpredictable.  In

addition to the DX:AX registers used for the return

value, the CX, BX, DI, SI, and BP registers will be

altered.

*---------------------------------------------------------------*

FABSVAL

Takes the 4-byte real value contained in DX:AX and makes

it positive; no other registers are affected.

*---------------------------------------------------------------*

FBINTODEC

Takes a 4-byte real value and breaks down the compo-

nents into their decimal equivalents.  The real value

passed to the procedure in the DX:AX registers.  On

return the DX:AX registers contain the mantissa, with

a decimal portion understood to be at the end, the CX

register contains the sign in its low bit, and the BX

register contains the signed, unbiased decimal expo-

nent.  In addition, the values in the SI and DI re-

gisters are destroyed.

*---------------------------------------------------------------*

FCMP

Compares 4-byte real value in DX:AX with 4-byte real

value in CX:BX to determine which is the larger.  On

return AX contains a positive integer if the value in

DX:AX is greater than that in CX:BX, a negative integer

if DX:AX is less than CX:BX, and zero if the two values

are equal.  In addition to the return value in AX,

registers DX, CX, BX, SI, DI, and BP are changed.

*---------------------------------------------------------------*

FDECTOBIN

Converts the decimal components of a floating-point

number to a 4-byte real in IEEE format.  On entry, DX:AX

contain the mantissa with the decimal point understood

to be at the end, CX contains the sign in its low bit,

and BX contains the signed, unbiased decimal exponent.

On return, the DX:AX registers contain the binary

floating-point result.	In addition, the values in the

CX, BX, SI, and DI registers are changed.

*---------------------------------------------------------------*

FDIVIDE

Divides the 4-byte real passed in DX:AX by the 4-byte

real passed in CX:BX, storing the result in DX:AX.  An

attempt to divide by 0.0 will cause _errno to be set to

EDOM.  If the result is too small or too large to be

represented _errno will be set to ERANGE, and the value

returned in DX:AX will be unpredictable.  In addition to

the DX:AX registers used for the return value, the CX,

BX, DI, SI, and BP registers will be changed.

*---------------------------------------------------------------*

FMULT

Multiplies the two 4-byte reals passed in DX:AX and

CX:BX, and returns the result in DX:AX.  If the result

will not fit in a 4-byte real, _errno is set to ERANGE,

and the value returned in DX:AX will be unpredictable.

In addition the the DX:AX registers used for the return

value, the CX, BX, DI, SI, and BP registers will be

altered.

*---------------------------------------------------------------*

FTOA

Converts the 4-byte real passed in the DX:AX registers

to an ASCII string, stored at the location pointed to by

DI.  An integer between 0 and 8, indicating the number

of digits to be given after the decimal point.  The

calling procedure is responsible for ensuring that there

is enough space for the new string.  If the left side of

the decimal point would have more than 18 digits, the

procedure uses scientific notation with the precision

found in the mantissa rather than that specified in the

CX register.  If CX is higher than 8, a precision of 8

is used; if a negative precision is passed, a precision

of 0 is used.  If the precision is 0, no decimal point

is given in the string.  The function will never produce

a string which could not be stored in an array of 30

bytes, and for most situations the programmer will be in

a position to identify a much smaller size as adequate.

If the precision specified results in a loss of signifi-

cant digits in the portion following the decimal point,

rounding is performed rather than simple truncation of

digits.  All non-segment registers (AX, BX, CX, DX, DI,

SI, and BP) are altered.

*---------------------------------------------------------------*

FTOI

Returns in the AX register the signed 16-bit integer

value produced by conversion of the 4-byte real passed

in the DX:AX registers.  Values (including negative

values) are always rounded toward zero.  If the result

cannot be represented by a 2-byte signed integer _errno

is set to ERANGE and the return value is unpredictable.

In addition to the AX register used for the return

value, the BX, CX, and DX registers are altered.

*---------------------------------------------------------------*

FTOE

Converts the 4-byte real passed in the DX:AX registers

to an ASCII string using scientific notation, and stores

the string at the address indicated by DI.  An integer

between 0 and 7, indicating the number of digits to be

given to the right of the decimal point, should be

passed in the CX register.  If CX contains less than 0,

a precision of 0 is used; if CX is greater than 7, a

precision of 7 will be used.  Therefore an array of 15

bytes will be large enough for the largest string which

could be constructed by the function.  The calling

function is responsible for ensuring that there is ade-

quate space for the new string.  If precision is 0, no

decimal point is given in the string.  The AX, BX, CX,

DX, DI, and SI registers are altered by the procedure.

*---------------------------------------------------------------*

FTOL

Returns in DX:AX the signed 32-bit long integer value

produced by conversion of the 4-byte real passed in

DX:AX.  Values (including negative values) are always

rounded toward zero.  If the result is too large or too

small to be represented by a 4-byte signed long integer,

_errno is set to ERANGE and the return value in DX:AX is

unpredictable.  In addition to the DX and AX registers

used for the return value, the values in the CX and BX

registers are altered.

*---------------------------------------------------------------*

LTOF

Converts the 4-byte long integer value passed in the

DX:AX registers to a 4-byte real and stores the result

in the same registers.  In addition to DX:AX used to

return the result, the CX register's value is altered.