����������������������������������ͻ

�      Complete documentation      �

�                                  �

�   to CDRom Pascal Unit v1.0 by   �

�                                  �

�           XTREME 1994            �

�                                  �

�         XTM:0001-63-001          �

����������������������������������ͼ

CD_Init :

=========

Internal procedure to initialize CD-ROM available and CD-ROM drive letter

IOCTL_CONVERSE :

================

Call with :

CD_IOCTL_Device_Status

----------------------

Returns :

r[1] : bit 0 - 7

0 - 0=door closed 1=door open

1 - 0=door locked 1=door unlocked

2 - 0=supports only cooked reading

1=supports cooked and raw reading

3 - 0=read only 1=read/write

4 - 0=data read only 1=data read and plays audio/video tracks

5 - 0=no interleaving 1=supports interleaving

6 - reserved

7 - 0=no prefetching 1=supports prefetching requests

r[2] : bit 0 - 7

0 - 0=no audio channel manipulation

1=supports audio channel manipulation

1 - 0=supports HSG addressing mode

1=supports HSG and red book addressing modes

2 - reserved

3 -    "

4 -    "

5 -    "

6 -    "

7 -    "

r[3] : bit 0 - 7

0 - reserved

1 -    "

2 -    "

3 -    "

4 -    "

5 -    "

6 -    "

7 -    "

r[4] : bit 0 - 7

0 - reserved

1 -    "

2 -    "

3 -    "

4 -    "

5 -    "

6 -    "

7 -    "

CD_IOCTLI_Media_Changed

-----------------------

Returns :

r[1]   : Media byte : 0   - Don't know if changed

1   - Media not changed

255 - Media changed

CD_IOCTLI_Audio_Disk_Info

-------------------------

Returns :

r[1]   : lowest tracknumber

r[2]   : highest tracknumber

r[3-6] : starting point of the lead-out track

CD_IOCTLI_Audio_Track_Info

--------------------------

Returns :

r[1]   : Track number

r[2-5] : Starting point of the track

r[6]   : Track control information

CD_IOCTLI_Audio_QChannel_Info

-----------------------------

Returns :

r[1]   : CONTROL and ADR byte

r[2]   : Tracknumber

r[3]   : (POINT) or Index (X)

Running time within a track

r[4]   : Min

r[5]   : Sec

r[6]   : Frame

r[7]   : Zero

Running time on the disk

r[8]   : (AMin) or (PMin)

r[9]   : (ASec) or (PSec)

r[10]  : (AFrame) or (PFrame)

CD_IOCTLI_UPC_Code

------------------

Returns :

r[1]   : CONTROL and ADR byte

r[2-8] : UPC/EAN Code (Last 4 bits are zero; the low-order nibble of r[8])

r[9]   : Zero

r[10]  : AFrame

CD_IOCTLI_Audio_Status_Info

---------------------------

Returns :

r[1-2] : bit 0 - 15

0  - Audio Paused Bit

1  - Reserved

...

15 - Reserved

r[3-6] : Startting location of last play or for next resume

r[7-10]: Ending location for last play or for next resume

CD_IOCTLO_Eject_Disk

--------------------

Unlock the drive and eject the CD-ROM disk from the drive unit

CD_IOCTLO_Lock_Unlock_Door

--------------------------

Call with :

r[1]   : 0= unlock door 1=lock door

CD_IOCTLO_Reset_Drive

---------------------

This function directs the device driver to reset and reinitialize the drive

CD_IOCTLO_Close_Tray

--------------------

This command will instuct drives that can do so to close the door or tray

CD_IOCTLO_Play_Audio :

======================

Call with :

SectorStart, NumberToRead

CD_IOCTLO_Stop_Audio :

======================

Causes the CD-ROM to stop playing

CD_IOCTLO_Resume_Audio :

========================

Causes the CD-ROM to resume playing at the same location it stopped

BCD2LongInt :

=============

Call with :

BCD number

Returns :

LongInt number

LongInt2BCD :

=============

Call with :

LongInt number

Returns :

BCD number

Red2HSG :

=========

Call with :

Red book addressingmode pointer

Returns :

HSG addressingmode pointer

CD_Volume_Size :

================

Returns :

Number of frames (1) on current CD

CD_Tray_Open :

==============

Returns :

True  if the CD-ROM Tray is open

False if the CD-ROM Tray is closed

CD_Open_Tray :

==============

Causes the CD-ROM Tray to open

CD_Close_Tray :

===============

Causes the CD-ROM Tray to close

CD_Lock_Tray :

==============

Locks the CD-ROM tray so that the user can't open it with the open-button

CD_Unlock_Tray :

================

Unlocks the CD_ROM tray so that the user can open it with the open-button

CD_Playing :

============

Returns :

True  if an audio track is playing

False if no audio track is playing

CD_Play :

=========

Causes the CD-ROM to start playing an audio track even if one is playing

Call with :

Track number (in the range of CD_First_Track to CD_Last_Track)

Length to play (one tracklength is estimated by CD_Track_Length)

CD_Stop :

=========

Causes the CD-ROM to stop playing any audio track

CD_Resume :

===========

Causes the CD-ROM to resume playing whereever the CD_Stop command was used

CD_Media_Changed :

==================

Returns :

True  if the media has changed

False if the media has not been changed

This function is not yet reliable, so do NOT use it to check wheter the user

has changed the CD or not

CD_Track_Start :

================

Call with :

Track number (in the range of CD_First_Track to CD_Last_Track)

Returns :

The start address of the track selected

CD_Track_Stop :

===============

Call with :

Track number (in the range of CD_First_Track to CD_Last_Track)

Returns :

The stop address of the track selected

CD_Track_Length :

=================

Call with :

Track number (in the range of CD_First_Track to CD_Last_Track)

Returns :

Length of track in frames (1)

This function can be used for passing information to CD_Play if you only

want to play one track

CD_Track_Time :

===============

Call with :

Track number (in the range of CD_First_Track to CD_Last_Track)

Returns :

Length of track in seconds (2)

CD_First_Track :

================

Returns :

The first audio track on the current CD

Number of tracks can be evaluated by using CD_Last_Track - CD_First_Track + 1

CD_Last_Track :

===============

Returns :

The last audio track on the current CD

Number of tracks can be evaluated by using CD_Last_Track - CD_First_Track + 1

CD_Current_Track :

==================

Returns :

The current tracknumber

Before using this you should check wether the CD-ROM is playing or not by using

the CD_Playing function

CD_Current_Track_Time :

=======================

Returns :

Position in seconds of the current track that is playing

You can check if a track is being played by using CD_Playing

CD_Current_Disk_Time :

======================

Returns :

Position in seconds of the whole CD when playing

You can check if the CD-ROM is playing by using CD_Playing

CD_Available :

==============

This is a boolean variable that is true if a CD-ROM-player is available and

false if not

CD_Error :

==========

After each call to any of the CD-ROM routines, an error code is returned in

the global word CD_Error.  Its contents are as follows :

bit 15 - error bit 0=ok 1=error

bit 14 - reserved

bit 13 - reserved

bit 12 - reserved

bit 11 - reserved

bit 10 - reserved

bit 9  - busy bit 0=not busy 1=busy

bit 8  - done bit 0=not done 1=done

bit 7  - \

bit 6  -  \

bit 5  -   \

bit 4  -    > error code (see below) if error bit=1 (see above)

bit 3  -   /

bit 2  -  /

bit 1  - /

Errorcodes :

0  - write-protect violation

1  - unknown unit

2  - drive not ready

3  - unknown command

4  - CRC error

5  - bad drive request structure length

6  - seek error

7  - unknown media

8  - sector not found

9  - printer out of paper

10 - write fault

11 - read fault

12 - general failure

13 - reserved

14 - reserved

15 - invalid disk change

CD_Drive :

==========

This is a byte variable that contains the letter of the CD-ROM-player if

available (0=A, 1=B etc).  You should check wheter CD_Available is true

before you use this

(1) : A frame is a 1/75 of a second.  To calculate number of seconds, use

x*75, where x is number of frames.

(2) : A timestring can be evaluated by taking (sec div 60)+':'+(sec mod 60)

Here you must make (sec div 60) and (sec mod 60) into a string first

with the STR command.