###############################################################################

# S                      DOT MATRIX PRINTER PRIMER                          S #

###############################################################################

Sooner or later, everyone with a computer, discovers the need for a

printer.  After all, word processing wouldn't be much of a replacement for the

typewriter if you couldn't obtain adequate hardcopy.  Programmers learn very

quickly that lengthy programs MUST be printed to aid in debugging.  BASIC's

editor simply doesn't allow the flexibility needed to debug a program.

So at some point you ran out and splurged.  You probably bought a dot

matrix printer.  No matter what type was purchased you, no doubt, discovered a

voluminous manual that represented many more hours of late-night frustration.

After all, who can forget all the spent midnight oil as we toiled over the DOS

manual that came with our computers?  Then there was the BASIC language

manual. And that two-inch thick Wordperfect manual.  That Lotus manual

certainly caused a lot of anxiety and the dBase III manual was the last straw.

By the time you finished with all those manuals you probably swore off reading

forever.

Well, if you didn't throw the printer manual away in disgust you probably

hid it from yourself. Those of you who braved the first few pages, might have

been turned off by all the discussion about serial and parallel interfaces,

dip switches, and escape, CHR$, ASCII, Hexadecimal and character codes.  If

you fall in this category and have a dot matrix printer then this section is

just for you.

You should print this section and then sit in front of your computer and

type the examples exactly as they are presented.  Hopefully, by the time you

are finished you will be much more intimately involved with your printer and

can begin to take advantage of some of its capabilities.  Wouldn't you like to

be able to print using some of those exotic features described in the sales

brochure or impressed upon you by the sales clerk?  Then spend the time it

takes to get through this section and you should be much the wiser.  But, be

forewarned, you'll have to read that printer manual eventually!  Perhaps it'll

seem easier once you've worked your way through this.

There is such a large variety of printers available for microcomputers

that we can not discuss use of each type and brand in detail.  Instead, this

discussion will provide a general approach so that the principals will apply

no matter what brand of printer you own.  There are dot matrix as opposed to

letter quality (daisy wheel) and laser printers.  Some printers use serial

interfaces as opposed to parallel. Some printers can even be used as a

typewriter as well as connect to a computer.  This discussion will apply

principally to dot matrix, parallel printers and we'll use the BASIC language

that came with your computer to access their capabilities.

It is interesting that most printer manufacturers include a discussion of

their printer's capabilities by using the BASIC language to "turn on" the

special features, such as italics, bold face, compressed print, etc.  Yet some

computer manufacturers don't even provide BASIC with the machines.

Another final thought you must always be aware of. Different printers

contain different features.  Your printer may or may not be able to perform

exactly as we're describing with the techniques presented.  For example, most

current dot matrix printers have built-in graphics capability. So, you would

be able to dump a high-resolution graphics screen without any difficulty.  But

there are still a few dot matrix printers that claim graphics capabilities but

will not allow dumping a high-resolution screen. These types of printers don't

really produce graphics as we know them. Instead, they produce special

graphics characters that, when combined in various ways, provide the illusion

of graphics capability.

Also, the different codes we present here to cause the printer to print in

a special way, might not work on your printer.  If your printer has the

capability we're discussing, and doesn't work with the codes we use, then

another code will be needed to get your printer to perform as described. To

learn these codes you'll have to refer to your printer manual.

----------------------------   GETTING STARTED  ----------------------------

First, let's learn a little about limited control you have over your

printer by using DOS.  There's a command available to you at the DOS prompt

called MODE.  Actually, this is not a command, but the name of a utility

program in your DOS directory.  You can verify that it is there.  At the C>

prompts type DIR C:\DOS /W/P and press <ENTER>.  This command will display the

names of all files in your DOS directory and pause when the screen is full.

Locate the file called MODE.COM or MODE.EXE.

NOTE: After starting your computer (leave Windows if necessary), to get to

this subdirectory type CD\DOS and press <ENTER>. Your prompt should change to

something like C:\DOS\>. Type DIR/W/P and press <ENTER>.  Now look for MODE on

this directory listing.  If you don't find it there's no telling where it is.

You'll have to search until you find it - perhaps it's on your original system

diskettes.

In the context of this tutorial there are four ways you can use the MODE

command.  At the DOS prompt type,

TYPE C:\AUTOEXEC.BAT > PRN

and press <ENTER>.  The contents of your AUTOEXEC.BAT file will be sent to the

printer.  If you don't have an AUTOEXEC.BAT file then try another file, such

as CONFIG.SYS or some other short ASCII (all text) file.  Anyway, this simply

produces a sample printout that you can use for comparison purposes.

Next, at the DOS prompt, type,

MODE LPT1:132,6

and press <ENTER>.  Then send the sample text file to the printer by typing,

TYPE C:\AUTOEXEC.BAT > PRN

Substitute the name you used earlier in place of AUTOEXEC.BAT. The above MODE

command should have converted your printer so that it can print 132 characters

on a line.  Notice how small the characters are. Also, the number 6 used after

the last comma tells the printer to print 6 lines of text per inch. The LPT1

parameter tells DOS the name of the printer to adjust.

Now try this. Type,

MODE LPT1:132,8

and press <ENTER>.  Then use the TYPE command again to send the sample file to

the printer.  This time you should see the lines packed closer together.

That's because this variation of the MODE command contains an 8 as the last

parameter.  The text is printed using 8 lines per inch.

You can return the printer to normal (default) mode by typing,

MODE LPT1:80,6

and pressing <ENTER>.  Use the TYPE command again to see the effect.  Finally,

you can also set the printer to 80 characters per line and 8 lines per inch by

typing,

MODE LPT1:80,8

As before, you can verify this by using the TYPE command.

Actually, there is one more version of the MODE command that affects

printer operation.  You can add a comma, followed by the letter P, at the end

of the command.  For example,

MODE LPT1:80,8,P

causes the printer (called LPT1) to be set for 80 characters per line and 8

lines per inch.  The ,P option tells the computer to keep looking for the

printer in the event this device was not recognized the first time printing

was attempted.  This function is helpful if you have accidentally bumped into

the ONLINE button of most printers (taking it off line).  Ordinarily, if the

printer is off line when the computer sends text to it, a DEVICE TIMEOUT

message appears on the screen and the print attempt quits.  With the P option

appended to the MODE command the computer will keep on looking for the printer

until you press the ONLINE button.  You can interrupt this action anytime by

pressing the CTRL and BREAK keys simultaneously.  But to keep the P option

from working during the next print attempt you'll need to disable it by typing

one of the MODE commands shown above, without the P.

That's about all the printer control you have with DOS, except for control

of serial printers that might be attached to your system.  Review the DOS MODE

command for other variations used with serial printers.

-----------------   USING BASIC TO ADJUST PRINTER SETTINGS ------------------

In this section we'll look at ways to control printing action using the

BASIC language that, hopefully, came with your computer.  Turn your printer

off and on again so that it is using default settings.  At the computer type

CD\ and press <ENTER>.  Your computer will be at the ROOT directory.

Load BASIC by typing BASIC, BASICA, or GWBASIC, depending on which you

have, and press <ENTER>.  If you're using DOS 5.0 or higher you should have a

version called QBASIC located in the DOS directory. The OK prompt should

appear on the left edge of the screen for all versions of BASIS except QBASIC.

If you get a message "FILE NOT FOUND" then you'll need to change to the

directory containing your version of BASIC and try again. More than likely

your BASIC is in the DOS directory. Change to that directory by typing CD\DOS

and press (enter). Then type the name for BASIC that you have. Use DIR/P if

you're unsure of your version name.

NOTE: When you start QBASIC there will be no OK prompt. Press the ESC key to

get rid of the initial opening screen then simply start typing commands. Just

ignore the references to the OK prompt in this text and make sure your cursor

is at the left side of the screen. Also, when the text below tells you to type

RUN don't do it. Instead press ALT R. When the RUN menu drops down select

START using the arrow keys and press ENTER. This will start your program.

There's a BASIC statement you can type at the OK prompt that provides you

with a limited amount of printer adjustment.  The statement is WIDTH and, in

general form looks like this:

WIDTH device name, size

where device name, in this context, is LPT1:.  Actually, there are other

device names you can use but we're only interested in your standard parallel

printer configuration. LPT1:, LPT2:, LPT3:, COM1:, COM2:, and SCRN: are valid

device names.

You use this command to set the line width of your printer.  This

determines how many characters will be printed on a single line before the

printer executes a line-feed and carriage return.  But, just for fun, try

this. Type,

WIDTH "SCRN:",40

and press <ENTER>.  You should see the characters on the screen double in size

so that only 40 characters will fit on a line. Neat huh?  Enough fun. Let's

get back to the task at hand - controlling the printer.

You can use the WIDTH statement as an immediate command, at the OK prompt,

or within your BASIC programs to control the printer line width at will.

First, restore the screen to normal by typing WIDTH "SCRN:",80 and pressing

<ENTER>.  Next, type a short program that sends some text to the printer.  At

the OK prompt type:

10 LPRINT "Now is the time for all good men to come to the aid of their

women.  In the land of the blind the one eyed man is the king.  Of all the

fishes in the sea the mermaid is the one for me."

and press <ENTER> and the end of the line.  Since this is a BASIC program the

computer will not carry out the instruction until you type RUN and press

<ENTER>.  Go ahead, try it.  You should be able to count 80 characters on each

line of the printout.

Now, at the OK prompt type,

5 WIDTH "LPT1:",40

and RUN the program again.  Notice the difference?  There should be no more

than 40 characters on each line.

Now change the width by typing,

5 WIDTH "LPT1:",100

and RUN the program again.  This time you see 80 characters on the first line

but the second line contains 20 characters.  The third line contains 80 and

the fourth line contains 20.  What happened here?

If you are using a printer that uses standard 8 1/2 x 11 in. paper then we

say you have an 80 column printer because it will print 80 characters on a

line and then a line feed - carriage return is executed automatically.  This

version of the WIDTH statement is telling the printer to print 100 characters

followed by a carriage return and line feed.  But, the printer can only print

80 characters on a line!  The printer forces a CRLF (carriage return, line

feed) after the 80th character and then the computer forces another CRLF after

the 100th character.

Using a size parameter greater than 80 in the WIDTH statement on an

80-column printer is not much use to you.  But consider a wide-carriage

printer that can print up to 132 characters or more on each line.  How many of

you have a wide-carriage printer but have been using 8 1/2 in. paper because

you couldn't get the printer to print on the entire width?  Well now you can.

If you have a wide carriage printer try this.  Type,

5 WIDTH "LPT1:",132

and press <ENTER>.  Run the sample program.  Did you see that?  If you count

the characters you should see 132 of them on a line.  Some wide carriage

printers will allow up to 164, or even 256 characters to be printed on each

line.  You could use the WIDTH statement to set the printer to whatever width

your printer allows, up to 255 characters.  The size parameter used in the

WIDTH statement will allow numbers from 0 to 255.  That's 256 characters

totally.

Now for some more fun.  Return the computer to DOS by typing SYSTEM and

pressing <ENTER>.  QBASIC users press ALT F and select exit. At the DOS prompt

type,

MODE LPT1:132,8

and press <ENTER>.

Load BASIC again and, at the OK prompt type,

5 WIDTH "LPT1:",132

and press <ENTER>.  Now quickly retype the short BASIC program and RUN it.

You should see 132 tiny little characters on a line.  Here's a way to get more

than 80 characters per line on an 80-column printer.  Make the characters

smaller using the DOS MODE command.  Then, using BASIC's WIDTH statement,

change the line width and then do your thing. Now how many characters do you

think you can print per line on a wide carriage printer?  Experiment and find

out.

Some of the effects you observed here are more easily accomplished with

modern printers. For example, many brands of printers offer programmability

via switches on the instrument panel.  You can place the printer in various

print modes by simply pressing a few keys.  There are many other brands that

offer similar features and varying degrees of programmability, either through

switch settings inside the printer, or switches on the control panel.  These

features may or may not conflict with the examples demonstrated here.  Some

printer features may also be enhanced by use of these examples.  No matter,

there's no substitute for reading that printer manual and exploiting your

printer's capabilities.

If you're learning to program using BASIC then you'll discover the need to

control the printer's output via program control.  You'll want to place the

printer in one mode or another according to the desired output.  For example,

you might want to print a line of text which contains some italics.  To do

this you'll need to print the normal text, place the printer in italics mode,

print the next word in italics, change the printer back to normal print mode,

and continue printing the sentence.  If your printer has these capabilities

then you'll be pleased to know that the BASIC language will allow you to

perform these functions with ease.  But, you'll need to learn the technique.

In "Printer Control (Part II)" we'll discuss use of CHR$ (a BASIC string

function) and methods for performing print functions described in the last

paragraph.  With a little background you'll discover just how easy it is to

perform all kinds of word processing tasks with your dot matrix printer.

You'll also learn that many of the techniques we'll discuss will apply, in a

limited way, to letter quality printers and lasers.

END

***