###############################################################################

# T                        PRINTER PRIMER (Part II)                         T #

###############################################################################

In Part I we discussed how to control certain features of your parallel

dot-matrix printer by using DOS commands and we provided an introduction to

use of certain BASIC language commands to obtain even more control.  This time

we'll examine use of BASIC more extensively to gain access to many special

features your printer may offer.

First things first, however.  Let's equip ourselves with a few tools.  In

your DOS or BASIC manual you should be able to locate an ASCII chart.  If you

don't know where it is then check the index for the proper page number.  Look

under ASCII and you'll probably see a page reference to ASCII characters.

Usually the ASCII chart is located in an appendix.

On inspection you'll see that there are a total of 256 characters

available inside your computer.  Associated with each of the characters is a

number referred to as an ASCII number.  Commit this table to memory.  Just

fooling silly!  Anyway, this chart represents all the characters your computer

is able to print on the video screen or send to the printer.  That is, if your

computer is IBM compatible.

As long as we're talking ASCII you may as well learn how to type those

special characters so they will be displayed on the screen.  You see, there

are many more characters listed in the ASCII chart than you have on the

keyboard so, unless you know the technique, you won't be able to use them.

Incidentally, the following short discussion applies to creating the special

characters whether you're using a word processing program, such as DeskMate,

WordStar, etc., or the BASIC language.

Start BASIC or Qbasic.  Next, locate a character in the ASCII chart that

you would like to display on the screen.  Note the ASCII number associated

with the character.  Now, to enter the character, hold the ALT key down.

While holding this key reach over and type the ASCII number into the numeric

keypad on the right side of your keyboard.  This technique will NOT work with

the numbers across the top of your keyboard.

Assuming you want to enter the character corresponding to ASCII code

number 234 (Omega).  Then hold the ALT key down and type 234 on the numeric

keypad.  Then release the keys.  The character should appear on the screen

like this:

[  �  ]

Did you get the same?  You should see the Omega symbol between the brackets.

If not you probably didn't hold the ALT key down while typing all three

numbers.  Try again with other characters this time.  You can print several

successive characters by using this technique repetitively.

Suppose you want to use some of the special graphic characters to draw a

border around a word.  You would use the same technique as described above

adding a touch of creativity.  Type the following numbers while holding the

ALT key down:

<ALT> 201 205 205 205 205 205 187

The characters should look like this:

������ͻ

Now press <ENTER> and use the space bar to move the cursor to the left edge of

the box you've started and enter ASCII number 186.  Don't forget to hold the

ALT key down first.  Space over to the right side and do the same again.  Your

box should look like this:

������ͻ

�      �

and press <ENTER> after typing the last character.  Beginning to get the idea?

Can you finish the box?  Go for it!  You are encouraged to try this technique

with your favorite wordprocessor also.

The only problem with placing special characters on the screen using this

technique arises when you want to print them on your printer.  If you have a

true IBM compatible printer then there will be no problem.  Some printers do

come with IBM compatibility listed as a feature but you must configure the

printer to, somehow, recognize the entire IBM character set.  If you have

printed this tutorial on your printer and the special characters we've

included above appear as characters other than the ones you see on the screen

then you have a compatibility problem.  Your printer may not be compatible

with the entire IBM character set.  On the bright side, maybe it is

compatible, has a compatibility mode, and you just haven't converted it yet.

What to do?

You must refer to your printer manual to determine if it is IBM compatible

or not.  There's simply no way we can provide you with a list of all the

different brands and models.  Let's take for example a very popular printer

produced by Star Micronics, the NX-10.  This printer is advertised as being

IBM compatible.  And, the price is right.  But, when you purchase the printer

it is NOT configured to produce the entire IBM ASCII character set.  If you

plug the printer into the computer and start using it everything will seem to

work at first.  But, when you attempt to print a document that contains ASCII

characters with numbers above 127 then some strange characters may appear in

place of the correct ones.  If, as we mentioned above, you print this document

and those little boxes we've created don't look the same then your printer is

not producing the full IBM character set.

If you read far enough into the NX-10 manual you'll discover that there

are two sets of dip switches inside the printer.  These switches are easily

accessible from outside the unit and serve to allow you to configure the

printer in various ways.  One of the switches allows you to configure the

printer so that it can produce the entire IBM character set, and the procedure

is simple.  You simply pop out the ribbon, raise a little black panel to

expose the switches, and flip number 6 toward the front of the printer.

That's all there is to it.  But those darned manuals tend to scare us

mercilessly sometimes and we seem to avoid any part of the electronic aspects

of our equipment for as long as possible.

Other brands of printers might require you to use a screwdriver to remove

the cover to gain access to the switches.  The process is a little more

bothersome but you're only working with half a printer if it can only produce

half the IBM character set.  So check your manual and dig in.  You're not

going to hurt anything.  Just make sure all equipment is off before performing

the task.

A little out of context, but equally important and worth mentioning here,

is the fact that many of you have been using your printers and living with a

problem.  That problem is double line spacing.  And, try as you might, you

simply couldn't get anyone to solve it for you.  Sound familiar?

The double-spacing problem arises from the fact that some computers will

allow DOS (your computer) to send a line-feed (Tandy 1000's are notorious for

this).  But, many printer manufacturers produce printers that provide a line

feed too.  The result, when the printer prints, is a line feed from the

computer and one from the printer - double line spacings.  All you need to do

in most cases is check the printer manual for the correct settings of some dip

switches located somewhere within the unit.  One of the switches on the NX-10

allows the user to turn off the printer's line-feed whenever the computer is

sending one too.  And let's become aware of what the other dip switch settings

can do for you.  For example, you might be able to adjust form size, turn off

the paper-out signal, select optional print fonts, etc.  It pays to become

intimately familiar with your printer.

Now let's get back to the task at hand and learn how to send those ASCII

characters to the printer with the use of BASIC. There are two BASIC functions

you can use to send any printable character to the printer, CHR$ and STRING$.

These functions are used in conjunction with the LPRINT command.  The LPRINT

command sends the text to the printer while the CHR$ and STRING$ functions

tell the computer which characters to send.  Let's see how.

NOTE: Recall from Part I of this three part series that, if you are using

Qbasic, you type the commands as we describe below and then press ALT R to

bring up the RUN menu. You then use the arrow keys on your keyboard to select

START. Then press (enter) and the computer will carry out the instructions you

have typed.

At BASIC's OK prompt type this statement:  LPRINT "HELLO" and press

<ENTER>.  There's nothing too exciting about this.  You've probably done this

many times before.  The word HELLO is sent to the printer by the LPRINT

command.  The LPRINT command will allow you to send any alphabetic character,

along with numbers and special characters, such as $, #, etc.  Unfortunately,

you cannot use the LPRINT command alone to send all those other characters

listed in the ASCII chart to the printer.  There is a way.  By using the CHR$

and STRING$ functions and a technique called CONCATENATION.

Let's assume you want to create a small box similar to the one we used in

our example above.  But, you want to send it to the printer.  Well, you will

still use the LPRINT statement but the CHR$ function can be used to tell the

computer which character to send.

To use the CHR$ function you simply place the ASCII number, for each

character you want to send to the printer, inside parentheses following CHR$.

For example, if you want to send the upper-left corner of the box, ASCII code

number 201, the command would look like the following.

LPRINT CHR$(201)

Of course you press <ENTER> at the end.  Type this command once and check that

the expected character got printed.  If not then your printer is not producing

the IBM character set.

Now let's draw the rest of the horizontal portion of the box.  Here you

want to send five characters (ASCII code 205), side-by-side. The command,

LPRINT CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205)

will do the trick.  The symbol (+) used above is the symbol for concatenation

and tells the computer to "Combine" the preceding character with the one that

follows.  Since all characters are separated by a + symbol they will all be

combined into one continuous line.  The major problem with this is that the

line you create with this command will be one line down from the left-corner

character you printed previously.  Not good at all.  To correct the situation

you merely add a semicolon to the end of the first LPRINT statement or use a +

symbol to separate all occurrences of CHR$.  Also, since you want to create

the upper-right hand corner with the same single command, the following

command will perform the entire operation.

LPRINT CHR$(201);CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205);CHR$(187)

or,

LPRINT CHR$(201)+CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205)+CHR$(205)+CHR$(187)

works equally well.

Boy, that's a lot of typing just to create one line of graphics symbols

isn't it?  There's a shorter approach available using the STRING$ function.

The STRING$ function has a general form:

STRING$(qty,ASCII #)

where qty tells the computer how many characters to print and ASCII # tells

the computer which character to print.  The comma must separate the two

parameters inside the parentheses.

The STRING$ function serves to produce a large number of the same

character but the box we wish to draw uses two corners that have different

ASCII numbers.  Use the CHR$ and STRING$ functions together with the + symbol

to combine characters.  The top of the box can then be drawn with a command

similar to this:

LPRINT CHR$(201)+STRING$(5,205)+CHR$(187)

Much shorter isn't it.  And a little more elegant.  And, you can use the above

mentioned technique to obtain some very high quality appearing forms.  Can you

complete the above box and include the word HELLO in the middle.  Just in case

you don't feel comfortable with the above technique yet, here's a short

program to do it.

10 LPRINT CHR$(201)+STRING$(10,205)+CHR$(187)

20 LPRINT CHR$(186);TAB(4)"HELLO";TAB(12)CHR$(186)

30 LPRINT CHR$(200)+STRING$(10,205)+CHR$(188)

There now.  Isn't that a nice way to decorate your printouts?

You should now move on to Part III (section 4 in this text).

END

***