###############################################################################

#                                PRINTER PRIMER                               #

# U                                (Part III)                               U #

###############################################################################

In Printer Primer Part II we discussed how you can produce the IBM

character set of special graphics characters using certain keyboard

procedures. We also discussed use of the BASIC language to send these special

symbols to your printer.  If you're following this tutorial then you should be

producing some elegant graphics of your own by now.

In this tutorial we'll discuss how you can gain access to most of your

printer's special capabilities through use of BASIC and/or a special Printer

Control program that we've included on this diskette for your personal use.

As before, this entire discussion pertains to computer systems equipped with a

Dot-Matrix printer.

When you purchased your dot-matrix printer you probably made a buying

decision based on the amount of built-in features for the price.  After all,

you wanted to get the most bang for the buck, as the saying goes. And no one

can blame you for wanting to stretch a dollar and get a good deal in the

process.

A typical dot-matrix printer offered by many vendors today will contain a

wide range of features: NLQ (Near Letter Quality) print, Pica or Elite fonts,

the ability to produce italics, compressed or expanded print, and graphics to

name a few.  Another important feature, for IBM or compatible computers, is

that the printer is IBM compatible and able to produce the entire IBM

character set. Unfortunately, not all printers are IBM compatible.

Some printers are programmable and contain a convenient panel with

switches you can use to access special print features.  Other printers may be

programmable but rely on special software to access the features you may want

to use.  Your favorite word processor, for example, might allow you to access

a part or all of your printer's special print features.  Practically all

printers are programmable using the BASIC language.  We'll be using the BASIC

language in this tutorial to access your printer's built-in features.

Unfortunately, as you've no doubt already discovered,  gaining access to

all your printer's capabilities can be a nightmare!  The printer manufacturers

seem always to assume that you are perfectly comfortable with binary,

hexadecimal, and base-10 number systems and know how to make conversions

between the three.  Additionally, it's assumed that you are familiar with

terms such as ASCII, escape sequences, control codes, dip switches, BASIC,

etc.  We all know what ASS-U-ME means don't we?  And isn't that how we felt

the first time we attempted to decipher our printer's operation manual?

Just to give you an example of how complex the printer manuals can be

consider this.  To place your printer in subscript mode a STAR NX-10 manual

tells you this:

PURPOSE           Selects Subscripts

CODE              <ESC>    "S"    1

(decimal ASCII)    27       83    1

REMARKS           This command lowers the following characters and

prints as subscripts until subscript is cancelled.  All

conditions described for superscripts also apply to

subscripts.  NOTE:  The character "1" (decimal code 49,

hexadecimal code 31) can be used instead of ASCII 1.

Did you get all that?  Makes a lot of sense doesn't it?  Suuuurrrre it

does - to someone with about 2 years experience in programming computers.  How

many secretaries out there are having computers, printers, and other

peripherals plopped in front of them by their bosses and are being left to

fend for themselves and become more productive?  I'll bet a year's pay that

not many are using printers to their fullest capacity, simply because they

can't comprehend the manual.  Well, let's see if we can ease the strain

somewhat with the following discussion about ESCAPE SEQUENCES, ASCII, and

other things.

To use the examples listed within this discussion you'll need to start

BASIC.  Turn your computer on (boot the system).  At the C> prompt type BASIC

and press <ENTER>.  NOTE: If your computer uses BASICA or GWBASIC then you'll

have to type the name of whichever you're using.  After typing the command you

should see BASIC's OK prompt appear on the screen.  If you get a BAD COMMAND

OR FILENAME error message then BASIC isn't on the disk or is in a

subdirectory.  Try this.  Type,

CD\DOS

and press <ENTER>.  Try typing BASIC (or BASICA, etc) again.  If the OK prompt

appears then you're in luck.  If you get the error message again then you'll

simply have to search through your system until you find it.  Heaven forbid if

the manufacturer didn't even give you a copy of the BASIC language with your

computer.  IBM includes BASIC with the DOS package you must purchase with your

computer.  Some other manufacturers may not, in which case, you'll have to

purchase it separately.

When the OK prompt appears on the screen you are ready to type any of the

special commands that tell your printer you want to activate certain print

features.

All dot-matrix printers will accept commands you send to it using BASIC's

LPRINT command.  If you want to send some text to the printer then you'll use

the LPRINT statement and include the desired message inside quotation symbols.

For example, to send the message "Hello John," at the BASIC OK prompt you

would type LPRINT "Hello John" and press <ENTER>.  The computer will

immediately send the text to the printer.  You would use a similar method to

send commands to the printer that turn on its special features. That is,

you would use the LPRINT statement followed by some special codes.  The

following describes how this is done.

Now, pay attention.  The next step is important.  To send special codes to

the printer you'll need to use the LPRINT statement.  On the same line

following LPRINT you'll need to use BASIC's CHR$ function at least once.

Depending on the print feature you're trying to activate you may need to

follow with another CHR$ function or a certain character enclosed within

quotation marks.  Each of the commands within the line will be separated by a

semi-colon.

Whenever you use the CHR$ function in an LPRINT statement (or anywhere

else for that matter) you must include a number within parentheses immediately

following.  The number inside parentheses is called an ARGUMENT.  The numbers

can range between zero and 255.  Each ASCII number corresponds to characters

available within your computer and printer.  So to access special printer

features you'll need to use the LPRINT command followed by the CHR$ function

and include an ASCII number within parentheses.  The ASCII number you use as

an argument with the CHR$ function will depend upon the feature you want to

access and is usually listed in a large table in the printer manual.

Let's see how to access a special print feature using only one occurrence

of CHR$.  For example, to access condensed print (which most IBM compatible

printers will have) you would type the following command,

LPRINT CHR$(15)

and press <ENTER>.  The argument used with the CHR$ function is an ordinary

base-10 ASCII value.  If your printer manual only gave you the hexadecimal

value (0F) then you would type the command like this:

LPRINT CHR$(&H0F)

where the &H that precedes the 0F simply tells the computer that the number is

hexadecimal instead of base-10.  The rest of this discussion will use ordinary

base-10 numbers as arguments with the CHR$ function.

To cancel the condensed print mode you would type the command,

LPRINT CHR$(18)

and press <ENTER>.  You can test this command right now by sending a message

to the printer after condensed print has been selected.  Actually you can

perform both actions on the same command line by typing something like this:

LPRINT CHR$(15);"Now is the time for all good men."

The CHR$(15) statement turns condensed mode on and the message inside

quotation marks is immediately sent to the printer.  You could go one step

further by setting the printer in condensed mode, print something, and cancel

condensed mode all in one line.  Here's how.

LPRINT CHR$(15);"Now is the time for all good men";chr$(18)

Note that only one LPRINT statement is needed because each part of the command

is separated by semi-colons.  You could type each command on a separate line

but then separate LPRINT statements would be needed.  The following is

equivalent to the above command.

LPRINT CHR$(15)

LPRINT "Now is the time for all good men"

LPRINT CHR$(18)

Of course, you would press <ENTER> after each command.

If you wanted to send a single line-feed to the printer a LPRINT command

with only one occurrence of CHR$ would be needed but the number used as an

argument would be different from the above examples.  LPRINT CHR$(10) would

accomplish this task.  You can even ring the printer bell (if it has one) with

LPRINT CHR$(7).  There are other features your printer may have that can be

accessed easily by using LPRINT commands with single occurrences of CHR$. The

only differences between them will be the number used as an argument with

CHR$. Check your printer manual.  Be aware that these commands might differ

between different brands of printers.  While CHR$(15) will turn condensed

print on with an EPSON printer it might cause an IBM printer to go up in

smoke!  Just kidding of course, but CHR$(15) might not perform the same

function on another brand of printer.  You must be made aware of the possible

differences so you can understand why your printer may not perform exactly as

someone else's.  And, these differences are the major cause of

incompatibilities between different printers and computers.

If you have a printer with a cornucopia of features then some of them can

only be activated by using multiple occurrences of CHR$ within the LPRINT

statement.  It is important to know that whenever multiple CHR$ functions are

used to select a print feature, the argument of the first CHR$ function must

be the ESCAPE code.  Your printer manual will usually tell you this, although

not necessarily in plain English.  Just realize that the ESCAPE code is the

number 27 and is used as the argument for the first occurrence of CHR$ in the

LPRINT statement.  For example, to place the printer in superscript mode

(those are tiny characters usually located at the upper right corner of a

regular sized character) you need a four part command, as shown below.

LPRINT CHR$(27);CHR$(83);CHR$(49)

Notice the four parts consist of a LPRINT statement followed by three CHR$

functions.  Each of the CHR$ functions are separated by semi-colons.

The command used to place the computer in superscript mode (last

paragraph) can be written differently.  The first CHR$(27) code is the ESCAPE

code and must be entered exactly as shown but the form used with the last two

CHR$'s is optional.  You could use the actual characters represented by the

arguments instead of the ASCII numbers.  If you find the ASCII number 83 in

the ASCII table listed in your printer manual you'll discover that this is the

code number for S.  Additionally, the number 49 is the ASCII number for the

number 1.  Knowing this you could retype the command as follows, with no

difference in the effect.

LPRINT CHR$(27);"S";CHR$(49)

or

LPRINT CHR$(27);"S";"1"

or

LPRINT CHR$(27);CHR$(83);"1"

The printer doesn't care which method you use as long as it receives the

correct codes.  Equally important, if you elect to use the character instead

of the ASCII code and CHR$ then the character MUST be placed inside

quotations.

You only need to use the LPRINT statement with two occurrences of CHR$ to

take the computer out of superscript mode.  As in the previous example,

however, the first one must be the ESCAPE code - CHR$(27).  Here's the command

you type to return the printer to normal print.

LPRINT CHR$(27);CHR$(84)

or

LPRINT CHR$(27);"T"

Either form of the command is acceptable since 84 (argument in second CHR$)

corresponds to the capital letter T.

Somewhere in your printer manual (perhaps in an appendix) you should find

a complete list of the combinations of CHR$ you would use to activate any of

the printer's special print capabilities.  For the most popular Dot-Matrix

printers you'll discover that the manufacturer's explanation will use printer

control codes conforming to the ones we've discussed here.

With the knowledge you've acquired reading this tutorial and with some

BASIC programming skills under your belt, it would be a simple matter to write

a program that lets you select any of your printer's special print features

from a master menu.  One thing to remember about selecting special print

functions is that a given print feature will remain in effect until you either

select another feature or turn the printer off.  So developing a program would

allow you to quickly select a feature, then another, and another, etc.

We won't bore you by explaining how to write a program to select

special features.  If you already program in BASIC then you shouldn't have any

difficulty.  What we have done is write a special menu driven program that

allows you to select various dot-matrix print features most common to certain

popular printers, such as, EPSON, IBM, STAR MICRONIX, and Tandy.

There's a program on this disk entitled PRINTER.EXE.  This program

will offer a list of choices for different printer settings.  For example, you

can select italics, expanded, compressed, subscript, and superscript print

modes.  You can instantly change from one to the other by making a choice at

the menu.  Of course there are many more choices available than those just

listed, such as change line spacing, select Pica or Elite fonts, etc., and

much, much, more.

The PRINTER.EXE menu does not attempt to provide all the functions you

might have available with your printer.  Instead, the menu offers functions

that are common among many different brands of printers (Epson compatible

first) and that represent some of the most used functions.  We've attempted to

avoid functions that might be specific to a particular brand of printer.  The

main idea is to provide a menu driven program that allows MOST users to make

use of, and easily access, popular dot-matrix printer functions.  If you're an

experienced programmer then you'll find it easy to write your own "Printer

driver" that accesses ALL your printers' features.

However, for the benefit of those not expert with BASIC programming,

PRINTER.EXE will allow immediate access to many functions not easily

accessible.  You will be able to select functions from the menu that are

available in your printer and maintain some semblance of sanity.  The choices

you make will cause the printer to be configured accordingly and remain so

until you select another or turn the machine off.  And you won't be constantly

referring to the manual for the appropriate code that configures the printer

the way you want.

To start the program simply insert your textbook disk into drive A or B

and log onto that drive by typing A: or B: and press <enter>. Then type

PRINTER and press the <enter> key. The program will start.

If you make a selection at the menu of PRINTER.EXE you may discover that

the indicated function is not performed by your printer.  This might occur

with the IBM Proprinter XL, for example, because the Proprinter requires codes

for certain functions that are different on the Epson printers.  Don't let

this problem prevent you from making use of this program, however.  As

mentioned above, we've only used codes that are common between the most

popular brands of printers.  About the worst problem that could occur is that

you might select one function and the printer does something else.  Another

possibility is that you might select a function and the printer merely ignores

it.  But, by no means will your printer go up in smoke.

One more very important point.  Don't forget to turn the printer on BEFORE

attempting to select functions from the PRINTER.EXE menu.

END

***