fc - A Public Domain FORTH Cross Compiler for the RTX2000

Microcontroller

Version 1.14

fc is a FORTH compiler that generates machine code for the

Harris RTX2000 microcontroller chip.  fc accepts text files generated

by a text editor and produces optimized object code files for the

RTX2000.  fc has options for generating annotated object code listings

and cross reference files which can be incorporated into other

programs to link variable and word definitions.  fc can also be used

to produce code for the Harris RTX2010 and RTX2010 specific

instructions can be added using the #macro and ucode features.

fc currently runs on an AMIGA or IBM PC compatible computer

with either a hard disk or ram disk.  fc has been tested using

AmigaDos 1.3 and MS-DOS 5.0 but, may work with other versions of

either operating system.

fc was made possible by the Public Domain version of Berkely

yacc available on Fred Fish Disk #419.  Many thanks to Bob Corbett and

those who helped make this public domain version of yacc.

I would also like to thank John Goldsten and Augie Mattheiss

for their help in debugging and testing the compiler.  John, Augie,

and the author have developed several programs using the MS-DOS and

AMIGA versions of the compiler that have been successfully executed on

RTX2000 target systems.  We normally use the -o output option or the -

e output option for EPROMs.  The EPROM output option has been

successfully used with a DATAIO UNISITE programmer.

fc is a Public Domain program so no fee should be charged for

distribution except for a possible media charge.  fc was developed

using Aztec C for AMIGA version 3.6A with the default 16 bit integers.

fc has been ported to MS-DOS and the MS-DOS executable is included in

the distribution.  That version is identical to the AMIGA version

except for some details pertaining to memory allocation and the naming

of some of the source file names.  An experimental Macintosh version

may also be included in the distribution and it will require MPW.

fc is currently being used for software development future

revisions are likely.  We already have had several months experience

producing and executing code generated by fc.  However, fc is offered

"as is" and will not necessarily be supported.  Anyone wishing to

develop large programs for the RTX2000 or RTX2010 may wish to look

into other development systems or cross compilers from one of the

several vendors marketing them.

A test file is included with the distribution to demonstrate

code production of the compiler.  This output has been checked against

the instruction set listed in the Harris RTX2000 Programmer's

Reference Manual.  However, users should check the code production in

a disassembly output should they run into any really puzzling problems

when debugging their programs.  We have developed several programs up

to 4K bytes long and have not run into any code production problems

for the last couple of months.

Background

The RTX2000 microcontroller is a 16 bit microprocessor

architectureally designed for executing the FORTH language.  The

RTX2000 instruction set corresponds to FORTH primitive words (like

SWAP, DROP, @, !, etc) and combinations of primitive words which, in

some cases, allow several FORTH instructions to be encoded into a

single RTX2000 instruction.  All RTX2000 instructions execute in one

or two cycles and are sixteen or thirty-two bits long.  It is common

to run the RTX2000 at rates of 8MHz to 10MHz making it fast for real

time control applications.

The RTX2000 chip includes two onboard stacks (256 deep), one is

used for parameters and the other is used mainly for subroutine

returns.  This allows stack operations like SWAP, DROP, etc, to be

performed without an external memory access.  Likewise subroutine

returns are done using the onboard return stack making subroutine

overhead quite low.  In fact, most RTX2000 instructions can be coded

to perform a return as part of the instruction so, returns often don't

even require any extra code.

The RTX2000 has several onboard peripherals, including three

timer/counters and an interrupt controller. For more information on

the RTX2000, consult the data sheet from Harris semiconductor.

Potential users are strongly advised to investigate Harris' plan for

future support of the RTX2000 family before committing to the chip.

fc should also be compatible with the Harris RTX2010RH which is

sold as an ASIC.  The RTX2010RH features higher radiation tolerance

then the RTX2000, a barrel shifter, and a multiply accumulate circuit.

Support for RTX2010 specific instructions can be added by using the

macro capability of fc.

Running fc

fc is invoked from the CLI (AMIGA) or MS-DOS by typing:

fc {-lreoxsd} {iaaa} {-tbbb} filename

fc recognizes nine command line options - l, x, e, o, r, s, d, i and

t.  The l, x, s, d, i, and t options can be used in any combination

along with one of the other three options.  All options must proceed

the source file name.  The source file name is used as the base name

for generating output files.  If the source file has an extension, the

extension is removed when forming the output file names.

The l option writes a disassembled listing of the program into

a file named filename.lst.  The e option produces two EPROM output

files named filename.low and filename.hi instead of the default object

code output.  The r option also generates EPROM files, but the data is

in a nonstandard bit shuffled format.  The o option produces an

ascii/hex output file named filename.ols.  The x option generates a

definition file named filename.x which can be used by other programs

to access words and variables in the compiled program.  The s option

prints a symbol table into filename.sym.  The d option turns on the

conditional compilation.  The i option specifies a directory for

include files.  Finally, the t option specifies a directory for

temporary files.

The l option generates an object code listing along with the

corresponding FORTH source code.  This allows the programmer to check

how well the code was optimized.  Often a fc listing file will include

a single line with thirty-two bits of code.  This is done for long

literal instructions (which are 32 bits long) and when optimizations

bail-out due to a missing key instruction (often an alu operation,

fetch, or store).  The listing file also includes any error messages.

The e option generates object code for hi and low byte EPROMs.

The output format is ASCII/HEX compatible with DATAIO format 51 (or

56) and is produced only if no errors occurred during compilation.

This format consists of a start code, one or more address declarations

and data blocks, followed by a  stop code and checksum.  After

compilation, fc will query the user for the EPROM base address.  This

address is subtracted from the code address to produce an EPROM

relative address for output files. (If EPROMs are at 0x8000 hex, and

code starts at 0x8a00, the EPROM files would have address 0xa00 hex)

The r option generates the same type of files except the data for each

byte is bit reversed.

The o option generates object code in an ascii/hex format that

is compatible with the load block format described later.  Each 16 bit

entity in the output file is represented by a four digit hexadecimal

number.  The format consists of a configuration number (equal to the

compiler version), start address, data word count, control checksum,

object code, and block checksum.  As with the other output formats,

code is only produced if there are no compilation errors.

The x option produces a file with information about the symbols

used in the program.  This information includes the address of all

defined words and variables that are preceeded with the "xlink" label.

The "xcode" and "xheap" labels will cause the next available address

for code and/or variable allocation to be written to the file.  The

information is provided in a form that is compatible with fc.  That

is, another fc program can use a #include directive and include this

file and reference all the words and variables qualified with "xlink"

in the original program.  If the xcode and xheap qualifiers are used

in the original program, any new variables and code produced will fit

continuously with those produced by the first program.  This allows

programs to be compiled a piece at a time.  For example, some code

could be developed for EPROM and some for RAM.  The code developed for

RAM could use this feature to call routines in EPROM.

The s option produces a file containing the symbol table.  Each

symbol is listed along with its hexadecimal value.  A '*' is used to

indicate words and variables that are qualified with the "xlink"

label.

The d option compiles all lines in the input that start with a

'#' followed by a space.  Normally these lines are ignored if the d

option is not specified.  This allows debug code to be inserted into a

program  conditionally depending on if the d option is specified at

compile time.

The i option specifies a directory to be searched for include

files.  The directory name should immediately follow the i without any

spaces.

The t option specifies a directory for temporary files.  The

directory name should immediately follow the t without any spaces.

Environment

fc supports two environment variables, FC_TEMP and FC_INCLUDE.

These environment variables define the path names for temporary fils

and include files respectively. Unfortunately with the AMIGA version,

the variables must be set using the Aztec "set" command supplied with

the compiler instead of the environment commands provided with

AmigaDos 1.3.

Setting the environment variables is not required but, can make

development more convenient and faster.  The FC_TEMP variable should

be set to a directory on either the hard disk or ram disk.  If this

variable is not set, temporary files will be written to the current

working directory.  The FC_INCLUDE directory can be set to a directory

containing a collection of include files.  This directory will be

searched for an include file if it can not be found in the current

working directory.

If an include search directory is specified with the i option,

it will override the directory specified by the FC_INCLUDE environment

variable.  Also if a temporary file directory is specified with the t

option, it will override the directory specified by the FC_TEMP

environment variable.

fc Preprocessor

When fc is invoked, it processes the input file through a

preprocessor that expands all include files and macros and produces a

temporary file used for input to the compiler.  The preprocessor also

supports the conditional compilation and removes comments and inserts

filename/line number stamps used as reference for producing error

messages.  The preprocessor reserves the '#' character for specifying

commands and conditional compilation.

The preprocessor has two commands -- one for including files

and another for macro definitions.  Either command must start in

column one and the keyword (include or macro) must be in lower case

letters. The entire command must fit on a single line.

The include file inserts the contents of another file into the

program.  The filename follows "#include " and must not have any

spaces and may be qualified to access files outside the current

directory.  The current directory is searched for the file.  If it is

not found, the directory specified by the i option or the FC_INCLUDE

environment variable is searched.

The macro command takes the first text following #macro as the

macro name and the remainder of the line as the replacement text.

From then on, whenever the macro name is encountered, the replacement

text is substituted.  Macro substitutions are case sensitive. Text

enclosed in double quotes and macro replacement text is not is not

searched for macros.

Examples of the include and macro commands:

#include filename

#macro  name  replacement text

#include monitor.x

#macro nip swap drop

Comments start with a '(' character and end with the

corresponding ')'.  Any number of matched pairs of '(' and ')' may

appear within a comment.  Comments may not begin or end on a line with

a #macro command.

Macros and include files are used to extend the capabilities of

the fc compiler.  For example, fc does not directly recognize the

RTX2000 register instructions such as pc@, mr!, r>, yet these

instruction are easily added through the use of macros. Code libraries

and files filled with standard macros can be developed and accessed

easily with the include command.  The i option or the FC_INCLUDE

environment variable allows a single directory to be set up as a

repository of macro definitions and code libraries.

In addition to supporting include and macro, the '#' character

is also used to support conditional compilation.  A line in a source

file can be made conditional by inserting a '#' as the first character

of the line.  At least one space or tab should follow the '#' before

any of the source code.  If the program is compiled with the d option,

the conditional lines are included in the program.  If it is compiled

without the d option, the conditional lines are not included in the

program.

fc Memory Allocation

The RTX2000 uses absolute addressing for branches and variable

reference and the compiler fixes the code and variable locations at

compile time.  Code is compiled starting at a user defined address

(defaults to zero) and grows into higher memory.  Variables are stored

in an area referred to as the heap which starts in high memory and

grows downward into low memory.  The heap starting point can also be

defined (defaults to 10000 hex).  When a variable is defined, the

current heap value is decremented (once for characters, twice for

words) to provide the variable address.  (Variable allocation also

assures that all 16 bit variables are assigned even addresses.)

Typically the program starting address is assigned to the

lowest available memory address and the initial heap value is set to

one plus the highest memory address.  fc does not support paged memory

so all variables and code are limited to a single 64K byte page.

Reserved Words and Characters

The words listed below are recognized by the compiler as being

special and can not be used as word or variables names.  These words

also correspond to the operations supported by the compiler.

; : ) code heap constant variable cvariable array carray

xvariable xword nop ucode again begin drop dup else if dup?_if

next over repeat swap then until while nop exit not of( 0< 2*

2*c cU2/ c2/ U2/ 2/ N2* N2*c D2* D2*c cUD2/ cD2/ UD2* D2/ + -

and or xor nor nand +c -c xnor g@ g! u@ u! c@ c! @ ! @+

@- c@+ c@- !+ c!+ !- c!- ['] , { } byte word xcode xheap xlink

The '#' character is reserved for use by preprocessor commands,

conditional compilation, and as a character inside a string

definition.  The '"' character is reserved for strings and should not

be used in word and variable definitions.

Numbers and Strings

fc recognizes base 10 and base 16 (hex) numbers.  Numbers

preceeded by 0x are assumed to be hex and all other numbers are

assumed to be base ten.

fc also recognizes strings.  Strings may be included in word

definitions and must be enclosed by double quotes ("  ").  Within the

source file, strings may not extend over more then one line of the

source file text.  Special characters may be imbedded into strings by

using the following codes:

\n        Carriage Return, Linefeed

\t        Tab character

\b        Backspace character

\r        Carriage Return

\f        Form Feed

\\        Backslash

\"        Double quote

Word, Variable and Constant names

fc supports user defined names as long as 32 characters. In

general, all names should start with a non numeric character and be

kept to 20 characters or less.  As a special case, names can start

with number provided that the second character is not a number or an

'x' or 'X'.

fc Syntax

An fc program consists of memory definitions, variable

definitions, constant definitions, data definitions, x list control,

and word definitions.

Memory definitions tell the compiler where to place code and

variables.  They have the form:

number CODE  ( sets next code output to address = number )

number HEAP  ( sets heap = number )

The program may have any number of code and heap definitions,

but typically there will be only one heap definition and code

definitions will be used at the start of the program and when words

have to be compiled to specific addresses (like interrupt routines).

fc is very stupid and will allow you to overwrite code and variables

without the slightest warning so, be careful.  Note that for each code

definition, fc will start a new output section.

Variable definitions tell the compiler to allocate memory for

variables and arrays.  They have the form:

VARIABLE name  ( declares name as a variable and allocates 1 word )

CVARIABLE name  ( same as above but, allocates 1 byte )

number XVARIABLE name (assumes a variable name is at address = number)

number ARRAY name ( declares and allocates number words for name )

number CARRAY name ( declares and allocates number bytes for name )

The only difference between these definitions is the amount of

memory allocated.  Once a variable is declared and allocated, fc knows

no difference between a cvariable, variable, array, carray, or

xvariable.  The programmer is responsible for insuring that each

variable has the proper allocation for any operations performed in the

program.

Constant definitions tell the compiler about different kinds of

constants that can be used.  They have the form:

number CONSTANT name ( declare name as a constant = number )

number XWORD name ( declare name as a word at address = number )

number UCODE name { declare name as a machine instruction = number )

Constant definitions allow the user to use symbolic names in

place of actual numbers.  Whenever a constant is encountered in a

FORTH word definition, fc will generate code to push its declared

value onto the parameter stack.  Whenever a xword is encountered, fc

will generate code for a subroutine call to the declared address.

When a ucode is encountered, the declared value is substituted

directly as a machine instruction.

Data definitions tell the compiler to place tables of words or

bytes into memory for use by programs.  They have the form:

WORD name { datalist }  ( place datalist into memory as words )

BYTE name { datalist }  ( place datalist into memory as bytes )

where datalist is a list of constants separated by one or more spaces

and enclosed in braces.  In the case of a WORD table, the datalist may

also include variable names or word names.  (Variables must be

declared before they are included in the list)  In these cases, the

address of the variable or word is placed in the list.  For both word

and byte tables, the table name may be used by a program to push the

address of the table onto the stack (just like a variable name).

X list control words control generation of the link file that

is generated when fc is invoked with the -x option.  The link file is

refered to as a .x file (its filename is sourcefilename".x").  The x

list control words appear below:

XCODE  ( causes a CODE statement to be printed in the .x file )

XHEAP  ( causes a HEAP statement to be printed in the .x file )

XLINK  ( causes the next defined symbol* to be printed in the .x file)

*symbol should be a word, variable, or data definition

When XCODE appears in the source file, the .x file will contain

a CODE statement that defines code start at the next available

location when compilation finished.  When XHEAP appears in the source

file, the .x file will contain a HEAP statement that defines the heap

as the next available memory location when compilation finished.  When

XLINK appears before a word, variable, or data definition, it causes

the address of the following symbol to be included into the .x file as

an XWORD or XVARIABLE definition.

The x list control words should be used outside word and data

definition.  The XCODE and XHEAP words need only appear once in a file

to generate the CODE and HEAP statements in the .x file.  The XLINK

word will typically appear immediately before the definition of the

word, variable, or data definition to be installed in the .x file.

Word definitions have the same format as standard FORTH words.

They start with a colon followed by the word name, a body of

statements, and end with a semicolon.  Word definitions do not have to

be ordered in a source file since words may be called before they are

defined.

fc recognizes only a small number of basic FORTH words (called

statements here) summarized below:

if ..statements.. then

if ..statements.. else ..statements.. then

?dup_if ..statements.. then

?dup_if ..statements.. else ..statements.. then

for ..statements.. next

begin ..statements.. again

begin ..statements.. until

begin ..statements.. while ..statements.. repeat

exit      ( return to calling word )

drop

swap

dup

over

nop                            ( no operation )

not                            (ones complement )

+ - +c -c xnor nand xor or     ( alu operations )

g@                             ( fetch from RTX2000 ASIC bus )

g!                             ( store to RTX2000 ASIC bus )

u@                     ( fetch from RTX2000 user memory space )

u!                     ( store to RTX2000 user memory space )

c@ c! @ !                ( character and word fetch and store )

@+ c@+ @- c@-  ( character/word fetch with auto increment/dec )

!+ c!+ !- c!- ( character/word store with auto inc/decrement )

0<  2*  2*c       ( shift instructions )

cU2/ c2/ U2/

2/ N2* N2*c D2*

D2*c cUD2/ cD2/

UD2* D2/

of(            ( indicates RTX2000 streamed instruction )

[']            ( pushes address of following word onto stack )

"text"   ( defines string, pushes string address onto stack )

,          ( causes immediate code production )

Many statements are compatible with the simple words in the

FORTH-83 standard.  The compiler also supports RTX2000 specific

instructions, such as streamed instructions using OF( (a closing right

parenthesis must be included to define the end of the streamed

instruction) and u!, u@, g!, and g@ memory and ASIC bus access

statements. fc also supports all the RTX2000 shift instructions.  (See

RTX2000 data sheet or Programmer's Reference Manual from Harris)  Also

supported is the comma operator which causes the compiler to start a

new RTX machine instruction with the statement that follows.  This

feature is sometimes useful for writing optimized programs.  Many

simple FORTH words are easily generated using macros.  See

accompanying programs for examples.

Most RTX2000 compilers and interpreters directly support a

number of RTX2000 specific words for manipulating registers on the

processor's internal ASIC bus.  These words are not directly supported

by fc, however, the macro feature allows programmers to define macros

for these instructions.  Typically a file filled with these

definitions can be referenced using an include command in the program

source file.  Likewise, support for RTX2010 features can be added

using macro commands.

The basic statements (words) supported by the compiler are

described below.  Stack diagrams are also provided to show the state

of the parameter stack before and after execution of the statement.

if ..statements.. then

The top value of the parameter stack is popped and evaluated.

If the value is nonzero ..statements.. are executed.

if ..statementsA.. else ..statementsB.. then

The top value of the parameter stack is popped and evaluated.

If the value if nonzero, ..statementsA.. are executed, otherwise

..statementsB.. are executed.

?dup_if ..statements.. then

If the top value of the parameter stack is nonzero, then

..statements.. are executed.  Otherwise, the stack is popped and

discarded.

?dup_if ..statementsA.. else ..statementsB.. then

If the top value of the parameter stack is nonzero, then

..statementsA.. are executed.  Otherwise, the stack is popped, the

zero is discarded, and ..statementsB.. are executed.

begin ..statements.. again

Executes ..statements.. again and again in an endless loop.

begin ..statements.. until

..statements.. are executed.  The top of the parameter stack is

popped by "until" and evaluated.  If the value is nonzero, execution

continues with the statement following "until", otherwise

..statements.. are executed again.

begin ..statementsA.. while ..statementsB.. repeat

..statementsA.. are executed, then "while" pops the top of the

parameter stack.  If the value is zero, statements after repeat are

executed, otherwise ..statementsB.. are executed then..statementsA..

are executed again and the "while" test is performed again.

exit ( -- )

Execution is returned to the calling word.

drop  ( x -- )

The top of the parameter stack is popped and discarded.

swap  ( x y -- y x )

The top two items on the parameter stack are exchanged.

dup  ( x -- x x )

The top value of the parameter stack is copied and pushed onto

the parameter stack.

over  ( x y -- x y x )

The second value of the parameter stack is copied and pushed

onto the parameter stack.

nop  ( -- )

No operation is performed.

not  ( x -- y )

The top of the parameter stack is replaced by its ones

complement, y = not(x).

+  ( a b -- c )

The top two items on the parameter stack are replaced by their

sum, c = a + b.

-   ( a b -- c )

The top two items on the parameter stack are replaced by their

difference, c = a - b.

+c  ( a b -- c )

The top two items on the parameter stack are replaced by their

sum plus the value of the carry bit, c = a + b + carry bit.

-c  ( a b -- c )

The top two items on the parameter stack are replaced by their

difference minus the ones complement of the carry bit, c = a - b -

not(carry).

xnor ( a b -- c )

The top two items on the parameter stack are replaced by the

result of a logical exclusive nor operation, c = a xnor b.

nand ( a b -- c )

The top two items on the parameter stack are replaced by the

result of a logical nand operation, c = a nand b.

xor ( a b -- c )

The top two items on the parameter stack are replaced by the

result of a logical exclusive or operation, c = a xor b.

or ( a b -- c )

The top two items on the parameter stack are replaced by the

result of a logical or operation, c = a or b.

g g@  ( -- x )

Fetches word x from address g on the processor's ASIC bus and

pushes onto parameter stack.  g is a constant between zero and thirty-

one.  See RTX2000/RTX2010 data sheet for register assignments.

g g!  ( x -- )

Pops top of parameter stack and stores it into address g of the

processor's ASIC bus.  g is a constant between zero and thirty-one.

See RTX2000/RTX2010 data sheet for register assignments.

u u@   ( -- x )

Fetches word x from user space address u and pushes onto the

parameter stack.  u is a constant between zero and thirty-one.

u u!   ( x -- )

Pops top of parameter stack and stores it into address u of the

user space.  u is a constant between zero and thirty-one.

@  ( a -- d )

Replaces the top of the parameter stack with a word fetched

from the address a.

c@  ( a -- d )

Replaces the top of the parameter stack with the byte fetched

from memory address a.

!   ( d a -- )

Pops two items from the top of the parameter stack and stores

word d into memory address a.

c!  ( d a -- )

Pops two items from the top of the parameter stack and stores

data d as a byte into memory address a.

@+ ( a -- d a+2 )

Pop address a from top of parameter stack and fetch word d from

memory address a.  Push d and a+2 onto parameter stack.

c@+  ( a -- d a+1 )

Pop address a from top of parameter stack and fetch byte d from

memory address a.  Push d and a+1 onto the parameter stack.

@- (a -- d a-2 )

Pop address a from the top of the parameter stack and fetch

word d from memory address a.  Push d and a-2 onto the parameter

stack.

c@-  ( a -- d a-1 )

Pop address a from the top of the parameter stack and fetch

byte d from memory address a.  Push d and a-1 onto the parameter

stack.

!+  ( d a -- a+2 )

Pop top two items off the parameter stack.  Store word d into

memory address a then push value a+2 onto parameter stack.

c!+  ( d a -- a+1 )

Pop top two items off the parameter stack.  Store d as a byte

into memory address a then push value a+1 onto the parameter stack.

!-  ( d a -- a-2 )

Pop top two items off the parameter stack.  Store word d into

memory address a then push value a-2 onto the parameter stack.

c!-  ( d a -- a-1 )

Pop top two items off the parameter stack.  Store d into memory

address a as a byte, then push value a-1 onto the parameter stack.

0<  ( a -- b )

The top of the parameter stack is replaced by a value obtained

by extending the most significant bit to every bit in the word.

2*  ( a -- b )

The top of the parameter stack is shifted left by one bit.

Zero is shifted into the lsb and the msb is shifted into the carry

bit.

2*c  ( a -- b )

The top of the parameter stack is shifted left by one bit.  The

carry bit is shifted into the lsb and the msb is shifted into the

carry bit.

cU2/  ( a -- b )

The top of the parameter stack is shifted right by one bit.

The carry bit is shifted into the msb and the lsb is discarded.  Carry

bit is set to zero.

c2/  ( a -- b )

The top of the parameter stack is shifted right by one bit.

The carry bit is shifted into the msb and the lsb is shifted into the

carry bit.

U2/  ( a -- b )

The top of the parameter stack is shifted right by one bit.

Zero is shifted into the msb and the lsb is discarded.  The carry bit

is set to zero.

2/   ( a -- b )

The top of the parameter stack is shifted right by one bit.

The msb remains unchanged and the lsb is discarded.  The carry bit is

set to the value of the msb.

N2*   ( a x -- b x )

The second value from the top of the parameter stack is shifted

left by one bit. Zero is shifted into the lsb.  The carry bit is not

changed.

N2*c    ( a x -- b x )

The second value from the top of the parameter stack is shifted

left by one bit.  The carry bit is shifted into the lsb.  The carry

bit is not changed.

D2*   ( a b -- c d )

The top two items on the parameter stack are shifted left one

bit together as a thirty-two bit word. The msb of the top is shifted

into the carry bit. Zero is shifted into the lsb of the second item.

D2*c   ( a b -- c d )

The top two items on the parameter stack are shifted left one

bit together as a thirty-two bit word. The carry bit is shifted into

the lsb of the second item and the msb of the top is shifted into the

carry bit.

cUD2/   ( a b -- c d )

The top two items on the parameter stack are shifted right one

bit together as a thirty-two bit word. The carry bit is shifted into

the msb of the top and the lsb of the second item is discarded.  The

carry bit is set to zero.

cD2/   ( a b -- c d )

The top two items on the parameter stack are shifted right one

bit together as a thirty-two bit word. The carry bit is shifted into

msb of the top and the lsb of the second item is shifted into the

carry.

UD2*  ( a b -- c d )

The top two items on the parameter stack are shifted right one

bit together as a thirty-two bit word. Zero is shifted into the msb of

the top and the lsb of the second item is discarded.  The carry bit is

set to zero.

D2/   ( a b -- c d )

The top two items on the parameter stack are shifted right one

bit together as a thirty-two bit word. The msb of the top remains

unchanged and the lsb of the second item is discarded. The carry bit

is set to the state of the msb of the top item.

for ...statements... next

"for" causes a loop count (n) to be popped off the parameter

stack and ...statements... are executed n times.  The iteration count

minus one can be obtained by reading the top value of the return stack

(using r@).  ( Note: return instructions should not be executed inside

for...next loops since the iteration count is stored on the return

stack)

of(   ( n -- )

Indicates an instruction, or instructions should be repeated.

A closing right parenthesis is required to indicate the range of

instructions to be repeated.  The instruction, or instructions must

compile into a single 16 bit RTX2000 machine instruction or an error

will occur.  The repetition count, n, is popped off the parameter

stack and the indicated instruction(s) are executed n + 1 times.

['] user_defined_word

When ['] proceeds a user defined word, the address of the word

is pushed onto the parameter stack instead of word being called as a

subroutine.

"String Text"     ( -- a )

Pushes the address of the defined string onto the parameter

stack.  The first byte of the string will be a count of the number of

characters that follow.  (Assumes that the RTX2000 is configured for

Motorola type byte addressing)

,

Comma causes the compiler to start a new RTX2000 machine

instruction with the statement that follows.  Commas allow programmers

to explicitly show how a program or portion of a program should be

partitioned into RTX2000 instructions.  This allows the user to

directly control which statements are combined into RTX2000

instructions.  Commas are used to force a desired optimization that

the compiler may not be able to achieve without the help of the

programmer.  This feature is useful when writing super optimized code

when the programmer has a good understanding of the RTX2000

instruction set.

In addition to the statements (words) defined above, the

program may refer to user defined variables and words.  All variables

must be defined before they are used, fc assumes all undefined

references to be subroutine calls (user defined words). Words do not

have to be defined before they are used but must be defined somewhere

in the program being compiled.

Comments may be placed anywhere in the program file and begin

with a space or newline followed by a left parenthesis.  They may

extend over several lines and matching pairs of parenthesis may be

included inside comments.  A right parenthesis matching the left

parenthesis that started the comment is used to end a comment.

fc Code Optimization

fc performs optimization at the source code level by collecting

the greatest number of FORTH statements that fit into one of its

syntax templates (coded using yacc).  Many of these templates

correspond directly to single RTX2000 instructions while others

provide a default path in case single instruction optimization can't

be achieved.  As the compiler collects FORTH statements one at a time

to fit to a given template, other templates must be available so that

code can be produced at any point (such as if a call statement is

encountered, which can not be combined with other statements).  The

yacc generated parser insures that the longest template will be

chosen, incorporating as many FORTH instruction as feasible (given a

particular template list) into a single RTX2000 instruction. This

approach does not necessarily produce the shortest RTX2000 program

but, it is easy to code and allows easy incorporation of new

optimizations.

The comma operator can be used to force the compiler to abandon

looking for additional statements to pack into the machine

instruction.  The statements following the comma will always start a

new RTX2000 machine instruction.

fc incorporates limited single token look ahead in its lexical

analyzer.  Whenever a short constant or a "swap" is encountered, it

looks ahead to see what the next token is.  If a "u@", "u!", "g@", or

"g!" follows a short constant, then the compiler must start a new

instruction beginning with the short constant.  When a "swap" is

encountered that proceeds an alu operation, it can be incorporated

into the same instruction as the alu operation.  Also if a "swap"

follows a "swap", both are ignored.  This processing allows the

compiler to aggressively optimize out any number of swaps that might

precede an alu operation.

Default Object Code Format

Whenever fc is invoked without the e, r, or o option, and no

errors have occurred during compilation, fc will produce an object

file in the default format.  This format consists of a sequence of one

or more sections of code.  Each section consists of a start address, a

byte count, and RTX2000 instructions in binary format.  The compiler

will produce a new section each time the code address is set using a

"code" definition, and whenever a current section becomes longer then

1024 bytes.  The format of the default object code file appears below.

Each data entity is stored as a sixteen bit word.

Number of Sections

Starting address of section #1

Number of bytes in section #1

Data for section #1

{ Starting address of section #2 (if needed) }

{ Number of bytes in section #2 (if needed) }

{ Data for section #2 (if needed) }

....

DMSP Load Block Code Format

Whenever fc is invoked with the o option and no compilation

errors occur, code is produced in the DMSP Load Block Format.  This

format consists of a series of 16 bit numbers represented in ascii/hex

format.  Each code section produced by the compiler is packed into an

independent load block and all load blocks are concatenated into a

single file.  The output file format appears below.

Configuration Number

Start Address for section #1

Data Word Count for section #1

Control Checksum for section #1 header

Data for section #1

Block checksum for section #1

{ Configuration Number for section #2 (if needed)}

{ Start Address for section #2 (if needed) }

{ Data Word Count for section #2 (if needed) }

{ Control Checksum for section #2 (if needed) }

{ Data for section #2 (if needed) }

{ Block checksum for section #2 (if needed) }

....

The configuration number for each load block will equal the

compiler version  (for version 1.13, the configuration number = 000D

(hex)).  The checksum algorithm is add and rotate 1 bit right.  The

header checksum consists of the first three words in a block - the

configuration number, the start address, and the word count.  The

block checksum covers the configuration number, the start address, the

word count, the control checksum, and all the data.

Error Messages

fc produces a limited number of rather general error messages.

All error messages, except for undefined word, are accompanied with

file name, line number, and text from the vicinity of the error.

After an error has occurred, the compiler continues until it completes

the file.  When an error occurs within a word definition, the compiler

may skip over nearby errors and not report them.  All errors will be

included in the list file if the l option is specified.

A missing colon in a word definition may cause the compiler to

print out a lot of "invalid statement" errors, one for each label

encountered in the word definition.

Limitations

fc stores generated code internally in a fixed size buffer.

Because of the defined length of the buffer, all programs should be

limited to 20K - 25K bytes or shorter.

Revision History

fc version 1.3 is the first version with intentional

distribution, however a short history of revision is given below:

1.2   First revision with all EPROM output options

1.2a  Bug in l option fixed, l option no longer hangs up the

program for outputs that start with indented lines.

1.3   DMSP output option added.

1.4   var keyword changed to variable

cvar keyword changed to cvariable

xvar keyword changed to xvariable

inline keyword changed to ucode

return keyword changed to exit

To increase compatibility with Harris TFORTH compiler

Bug fix in parser to support heap definition of 0x10000

1.5   ['] word added

String support added

1.5a  IBM only - bug in string support fixed

1.5b  IBM only - bug in ['] fixed

1.6   DMSP option changed so that there are no spaces in file

Configuration number for this option is set to zero

instead of being prompted from the user

1.7   Fixed disassembly output for DUP d g! instructions

Added comma feature

Added word and byte data definitions

1.8   Corrected bug with instructions of the form DUP d alu-op

Changed configuration word to equal the complier version

1.9   Corrected bug with DMSP block load checksum

Printout size of each output section

Modified preprocessor so that strings are ignored

in macro substitutions

1.10  Fixed bug with constants used in "word" statements

Fixed preprocessor so that #, (, and ) may be used in

strings

Added xheap, xcode, and xlink statements

Added -s option

1.11  Fixed bug with @- and c@- instructions

Filename extensions allowed for source files

1.12  Added optimization for longlit OVER alu-op instruction

Added environment variables

Added conditional compilation

Fixed bug with disassembly of d g@ OVER alu-op

Changed command line print-out for code production

1.13  Speeded up preprocessor and symbol table

Added optimization for DUP d u@ aluop instruction

Allows use for larger programs

Added -i -t options

1.14  MS Windows Version (wfc)

Trialing '\' character no longer needed for specifing

path using envirnment variables

No longer prompts user to stop after set number of errors

References

The second reference is an excellent general reference for

scientific computing which also contains some search and sorting

algorithms.  The other three references provide a good practical

introduction to compiler writing using coded examples.

Aho, A, V,, Sethi, R., and Ullman, J., D., Compilers

Principles, Techniques, and Tools, Addison-Wesley Publishing,

Massachusetts, 1988.

Flannery, B. P., Press, W. H., Teukolsky, S. A., and

Vetterling, W. T., Numerical Recipes in C The Art of Scientific

Computing, Cambridge University Press, New York, 1988.

Friedman, H. G. Jr., and Schreiner, A. T., Introduction to

Compiler Construction with UNIX, Prentice-Hall, New Jersey, 1985.

Kernighan, B. W., and Pike, R., The Unix Programming

Environment, Prentice-Hall, New Jersey, 1984.

Lloyd Linstrom

October 1992

#