COPYRIGHT (c) 1994 WESTERN DIGITAL CORPORATION

WDCDRV.DOC

READ ME FILE FOR WDCDRV.386

WDCDRV.386

Version:    2.5

Date:       6/17/94

Size:       16,475 BYTES

CAVIAR.EXE

Version:    1.0

Date:       2/16/94

Size:       87,024

1.1 Overview

WDCDRV.386 is a "Fastdisk" hard disk driver for use under Microsoft's  Windows

3.1 on 386 or higher based systems. Its main benefit is its ability  to transfer

data from the hard disk in either a Block Mode PIO fashion, EISA type 'B' DMA

fashion, or PCI type 'F' fashion.

CAVIAR.EXE is a configuration program that allows the user to change the option

settings for WDCDRV.386 through a Windows program. This program makes it

unnecessary for the user to edit SYSTEM.INI directly to change the options that

WDCDRV.386 reads from the [WDCDRV] section of that file. It also will reduce the

possibility of the user making invalid edits or damaging his SYSTEM.INI while

manually editing it. During installation, the setup program creates a program

group and icon for executing this configuration program. The program includes

on-line help for operating the program as well as the purpose of each option

setting.  Reviewing this help information is highly recommended.

1.2 WDCDRV Features

WDCDRV implements several enhancements to the basic *WDCTRL Fastdisk driver

shipped with Windows 3.1. The subsections that follow contain summaries of

these features. For more detailed descriptions of the related options for each

feature see section 3.x.

1.2.1 Block Mode PIO

Block Mode PIO is a means of transferring multiple sectors at a time between

the host and hard disk as opposed to transferring one sector at a time.

Normally in an AT compatible machine a hardware interrupt occurs for each

sector transferred to/from the hard disk. Under Block Mode PIO one interrupt

will occur for a block of sectors. The number of sectors that are to be

transferred per interrupt is programmable by the host and is referred to as

the blocking factor. The driver will determine if block mode is enabled for

the drive when you start Windows. If block mode is detected then the blocking

factor programmed is used as the default value for the Fastdisk driver. If

block mode was not enabled for the drive when Windows was started the default

blocking factor used equals the maximum blocking factor supported that is

reported by the drive on an Identify Drive command.

Transferring data via Block Mode PIO increases throughput by eliminating the

inter-sector overhead associated with each sector. This is most beneficial when

the hard disk is transferring data out of its cache.  Block Mode data transfers

are enabled by default.

1.2.2 DMA Transfers

Transferring drive data via DMA allows the host CPU to work on other tasks

while hard disk data transfers are taking place. This increases the overall CPU

bandwidth which is critical in multi-tasking environments.

This driver only runs Block Mode PIO and DMA on Western Digital drives. To use

the DMA feature the drive must be installed in an EISA or PCI  system and the

DMARQ and DMACK- signals on the the 40 pin cable must be  connected to the

proper DMA request and acknowledge signals on the EISA  or ISA bus.

This revision of WDCDRV drive automatically detects DMA channel and system bus

type being EISA, PCI/ISA or PCI/EISA.

1.2.2.1 EISA Type B DMA

This driver also supports EISA type 'B' DMA transfers for drives that support

this feature and are installed in an EISA system. Data is transferred at a rate

of 4 MB/sec which is about double that of a 6 MHz AT. The standard Read DMA and

Write DMA commands as described in the CAM ATA specification are used.

1.2.2.2 PCI Type F DMA

Furthermore, type 'F' DMA is supported in PCI based systems. Data is

transferred at a rate of between 6 and 8 MB/sec using type 'F' DMA. The same

Read DMA and Write DMA op codes are used, the only difference is in the

programming of the DMA controller on the motherboard.

1.2.3 Translating BIOS

Translating BIOS allows use of IDE drives larger than 528 megabytes. When a

translating BIOS is present the standard *WDCTRL will not load and operate. The

WDCDRV.386 program recognizes and functions correctly with translating BIOS. In

addition many translating BIOS use logical block addressing (LBA) which is also

incompatible with *WDCTRL. If a drive is capable of LBA operation then LBA mode

will always be used.

1.2.4 Disk Manager Compatibility

Use of Ontrack Disk Manager prevents operation of *WDCTRL since Disk Manager

performs translation and other activities which cause *WDCTRL to be become

non-functional. Versions of WDCDRV later than 2.3 will detect and operate with

Disk Manager version 6.03.

2.1 Notes on Operation

There are several issues related to operation of the driver that the user may

need to be aware of. These are detailed in this section and may cover potential

incompatibilities or may simply be more detailed information on driver operation

or installation.  Not all information will be needed by every user and may often

be skipped or ignored.

2.2 Known Problems

2.2.1 MS-DOS 5.0 EMM386.EXE

This driver will not operate with the version of EMM386.EXE supplied with

MS-DOS 5.0.  This version should not be in use since Windows 3.1 setup will

replace the lines in CONFIG.SYS to load HIMEM.SYS and EMM386.EXE with lines to

load these files from the Windows directory.  New versions of these files

supplied with Windows 3.1 operate correctly with WDCDRV.386. The versions of

EMM386.EXE supplied with MS-DOS 6.0 and MS-DOS 6.2 have also been verified to

operate correctly with WDCDRV.386.

2.3 Technical Information

2.3.1 LBA Mode Operation

The driver will always operate in LBA mode if the drive is capable of using LBA

mode commands. This reduces overhead in the driver and the drive firmware

slightly and will not impact drive operation in any other way.

2.3.1 Temporary Swap Files

When Windows is configured for a temporary swap file it performs accesses to

that swap file through the PC BIOS. A potential conflicts between one blocking

factor being used for swap file accesses by the BIOS and another blocking factor

being used by all other Windows disk accesses the driver requires special

processing by the driver.  Thus the driver will not allow the blocking factor to

be changed from that selected by the BIOS when a temporary swap file is in use

by Windows.  Any attempt to change the blocking factor in the SYSTEM.INI file

will be ignored.  The driver will continue to use block mode transfers using the

blocking factor selected by the system BIOS at power-on.

3.1 Installation Instructions

You must have Windows 3.1 installed on your hard disk prior to performing any

installation of the WDCDRV.386 driver. You then have two methods for

installation of the driver.  The first, and recommended, method is the

automatic installation procedure which only requires you to run the SETUP.EXE

utility from Windows 3.1. This SETUP.EXE and the associated files are located

on the distribution floppy containing the WDCDRV.386 driver. Information on

running the automated installation procedure can be found in the README.DOC

file on the WDCDRV.386 distribution floppy. The second method is manual

installation of the driver program only. The CAVIAR.EXE configuration program

and its associated help files are not installed by this procedure. You must

use the automated procedure to install these files if you wish to use them.

3.1.2 Manual Installation Procedure

1. Copy WDCDRV.386 to the Windows directory of your hard disk. This

directory is normally C:\WINDOWS but may be different for your system

configuration.

2. Edit the SYSTEM.INI file as follows:

OLD:                            NEW:

[386Enh]                        [386Enh]

device=*wdctrl                  REM device=*wdctrl

device=c:\windows\wdcdrv.386

Putting a 'REM' in front of "device=*wdctrl" will cause Windows to  ignore

this line. The second statement defines the path to the  Western Digital

driver WDCDRV.386 which is replacing the wdctrl  FastDisk driver from

Microsoft. If WDCDRV.386 is not in the c:\windows  directory then simply

type in the proper path to its location in the  device statement.

3. Run Windows and select the "Control Panel" icon. Then select the

"386 Enhanced" icon. Next select the "Virtual Memory..." option.

Select "Change>>" and then check the "Use 32-bit Access" option.

The "Use 32-bit Access" option must be on in order for the

WDCDRV.386 FastDisk driver to be loaded. Windows will reboot and

WDCDRV.386 will become active.

>>>>>>  IF you do not see 32 bit Disk access flag displayed at above window

Please enter following lines in [386Enh] section of system.ini file:

[386Enh]

32BitDiskAccess=ON / Off

device=*int13

device=C:\WDCDRV.386

>>>>>>>

OPTIONAL SWITCHES:

==================

A new section called [wdcdrv] can be added in the SYSTEM.INI for optional

command line switches as follows:

[wdcdrv]

DMA=7

DMATYPE=B

In this example, the DMA channel is set to 7 with type B DMA transfers. Since

DMA channel and transfer type are automatically detected during run time, these

switches are either redundant or provide an overide to what the driver would

otherwise use.

Please note that the [wdcdrv] section must come after the [386Enh] section in

SYSTEM.INI.

[WDCDRV] OPTIONAL COMMAND LINE SWITCHES

DMA=3, 5, 6 OR 7        Default is automatically detected by the driver.

This switch allows an overide of the DMA channel

that was automatically detected by the driver.

NOTE:

Please do not use this switch if you are not sure

what DMA channel is used for the IDE connector in

your system. It is possible that there is no

DMA Channel dedicated to the IDE connector.

DMAFLAG=ON / OFF        Default is ON. Can select OFF to turn off DMA.

DMATYPE=F / B           Default is automatically determined by driver. This

switch allows an overide of the DMA type that was

automatically detected by the driver.

BLOCKINGFACTOR=4,8 or 16 This default value for this setting is automatically

determined by the driver. If the BIOS has enabled

block mode the driver will automatically use the

blocking factor programmed by the BIOS as the default.

If the BIOS did not program a blocking factor the

driver uses the value returned by the Identify Drive

command.  This switch allows an overide of the default

value chosen by the driver.

SCATTERGATHER=ON / OFF  Default is OFF. Can be enabled via this switch.

CSL=ON / OFF            Default is OFF. Required for Intel Express and

Corsair systems when DMA channel 3 is being used.

Configures DMA channel 3 properly.