C O M M U N I Q U �

Version 1.11

March 1993

SOME of the changes made

to version 1.10

�������������

For a complete list of the features of Communiqu� v1.11 please

read the FEATURES.DOC document supplied within this archive.

�������������

o Fixed a bug in the hardware flow control that in some situations

caused CU to stay permanently in terminal not ready state. This

is an important bug fix, as it was causing large problems when

people were using hardware flow control with high-speed modems

such as the new ZyXELs and HST modems.

o I have changed the length of all macro strings to 220 characters.

If you have any macro files (*.CMF) then you should run the

upgrade program (UPGRADE.EXE) supplied in this release.

o Added the Area Code Prefix and Country Code into the Modem and

Dialling Setup menu. If the telephone number (must be preceded by

a '+') contains the Country or Area code as set in the Setup/Modem

menu then it strips that part out, if it strips out the country

code but not the area code it will place the Area Code Prefix into

the number string. The Country Code and Area Code should be in the

form of "+61-2" or just "+352".

o I have managed to wrangle quite a bit more speed out of the main

processing loop. You should see a noticable difference if your

modems DTE rate is locked at 19200 and above.

o Added a new option in the SETUP/MODEM/ADVANCED menu to set the

16550 FIFO inturrupt trigger level. This should normally be set

to 14 bytes.

o Fixed a bug that disabled the 16550 Uart FIFO buffer after

returning from a DOS shell.

o The estimated transfer time used in all file transfer protocols

now uses the DCE (Modem to Modem) baudrate as opposed to the

DTE (Computer to Modem) baudrate.

o Fixed the Zmodem bad header problem when using auto-zmodem

downloads on high speed modems.

o Communiqu� will now change the baudrate to the setting in the

dialling directory entry before dialling that entry. The baudrate

will be returned to the previous setting if the dial was

unsuccessful. The baudrate will not be changed if you have the

com port locked.

o Made changes to the way CU handles user customised com port

setups. If the ports interrupt number is changed then CU will

now look up a table for IRQ vectors instead of relying on the

user to set the correct vector for the interrupt.

o Fixed a bug where CU was not using 43/50 line when the setup

screen size said to use the DOS default mode.

o Fixed a bug that did not restore the bottom line if you entered

Full Screen Chat mode without a status bar.

o You are now told if capture is open when you enter chat mode if

you have Auto-ChatMode Capture on.

o You are now asked to confirm changes to the baudrate if you

press ALT-B and change the baudrate when you have the com port

locked.

o Fixed the wildcard bug in the FILE menu load and save functions.

o Fixed a spelling bug on the ALT status bar ALT [5] should have

been ALT [F5].

o Many other cosmetic changes have been made, plus areas of the

code have been changed to lower memory requirments and increase

speed.