Technical Note : FAST SERIAL PORTS

Author : Michael Spalter, S/E/G Communications

Email : segcom@cix.compulink.co.uk

Origin : United Kingdom, EC

Last Modified : 13th April 1993

Status : (c) 1993 Michael Spalter

NOTE: This document may be distributed  freely  providing  that  all

credits  remain  intact  and that it is not modified in any way. Any

suggestions for improving this document are welcome. Please  contact

the  author  as  shown  at  the foot of the document. Bulletin board

sysops are welcome to make this file available as a 'Bulletin'. This

document is provided for information only and no support or warranty

is offered by the author or publisher.

��������������������������������Ŀ

� DO I NEED A FAST SERIAL PORT ? �

����������������������������������

���������������������������������������������������Ŀ

�                 C O N T E N T S                   �

�                                                   �

�  0) INTRODUCTION                                  �

�  1) What is a UART ?                              �

�  2) Why the sudden craze for high speed cards ?   �

�  3) Why aren't normal PC ports good enough ?      �

�  4) How would I know if I needs a faster port ?   �

�  5) Okay, I want one ! What should I buy ?        �

�  6) Is there just one type of 16550 UART ?        �

�  7) Replacing a soldered 16450/8250 UART          �

�  8) Windows and buffered UARTS                    �

�  9) Supplier information                          �

�����������������������������������������������������

INTRODUCTION

------------

Fast  serial  cards  have  become  very  popular  lately  due to the

proliferation of high speed (V32/V32bis) modems. Whilst the computer

press have been keen to cover modems in general, little coverage has

been given to  serial  cards  (they're  just  not  sexy  enough  for

mainstream  articles).  This  means that, whilst many people want or

need fast serial cards, there is much  confusion  about  them.  This

document attempts to answer the most common questions asked.

1) What is a UART ?

----------------

Throughout this document, we will refer to something called a 'UART'

(pronounced 'you-art').  As this is a very important  component,  it

is important to understand what it is.

A  UART  is  an IC (chip!) which takes parallel data from the PC and

outputs it to the serial port as a timed serial stream of data.   It

is  the  main component of the serial card; all other components are

there to support the UART.  Here is a simple functional diagram:

PC's BUS Slot                         Serial port

1 ------------> ��������ͻ

1 ------------> �  UART  �

0 ------------> �        �

1 ------------> �        � ---------> 11010001

0 ------------> �        �

0 ------------> �        �

0 ------------> �        �

1 ------------> ��������ͼ

The diagram shows conversion of data from a  parallel  to  a  serial

stream. Of course, serial ports can receive data as well as transmit

it, so the UART also converts serial data back into parallel data.

2) Why the sudden craze for fast serial ports ?

--------------------------------------------

Now that high speed modems (V32, V32bis) have  become  very  common,

many PC serial ports are having problems running fast enough to keep

up  with the modems which can be driven at speeds of up to 57600 bps

(bits per second).  A standard '8250 UART'  based  serial  port  (or

one  using a multi i/o chip) cannot possibly run at that speed.  The

'16450' UART is only slightly better.

3) Why aren't normal PC ports good enough ?

----------------------------------------

Standard serial ports were designed  to  drive  mice,  plotters  and

modems.  At the time, modems ran at speeds of up to 4800 bps, and we

thought that was fast - anyone who has been using  modems  for  more

than  5  years  will  remember how lucky we thought we were !

Problems with serial ports arise for two reasons:

a) The 8250/16450 or equivalent UARTS are simply not fast enough  to

run  at  the  high  speeds  required  by  fast modems. Whilst the

BIOS/UART  will  allow  you  to select these speeds, they may not

reliably work at these speeds. The chips are rated for high speed

use, but that assumes a simple working enviroment (not a PC!).

b) PC's are busy things, with lots of interrupts to service. If  the

port is running  at  high  speed,  the  PC  may  miss  interrupts

generated by incoming  data  because  it  is  'busy  doing  other

things'  -  this  then means that the UART has nowhere to put the

incoming  byte  and  so  it  gets  overwritten  by  the next byte

received. This usually causes CRC errors.  The faster  you  drive

the port, the more CRC  errors  you  will  get.  This  is  almost

always a problem if you multitask with MS-Windows or Desqview, as

the rapid task switching keeps the processor very busy.

The '16550' UART gets around both of these problems:

a) It is designed to run at  high  speed;  a  16550  chip  will  run

happily  at 57600 bps or faster which is fast enough for even the

fastest modem.

b) The 16550 UART incorporates a 16 byte buffer;  if  the  PC  isn't

ready to received the current byte then the UART can retain it in

the  buffer  until  the  PC is ready for it.  16 bytes provides a

comfortatble time margin to ensure that none of the received data

is lost.  Many communications packages will empty the FIFO buffer

at every interrupt possible, rather than only taking one byte per

interrupt.   The  buffer  on  the  UART is a 'FIFO' buffer.  This

simply  means  that the first byte in will be the first byte out.

The alternative to this would be a LIFO buffer, but that would be

useless for a UART as all the data would become scrambled !

4) How would I know if I needed a faster serial card ?

---------------------------------------------------

If you are multitasking using MS-Windows or Desqview, then you  will

almost  certainly need a fast buffered serial card. Typical symptoms

will be CRC errors or block retries reported when downloading files.

As you will probably be using error correction,  then  these  errors

are unlikely to be caused by anything else.

Another  symptom  of an inadequate serial port is that you're simply

not getting the performance you'd expect from your modem. This could

be caused by you not having your modem configured correctly,  so  in

the  first instance, check with your suppliers or colleagues/friends

for any special setup strings particular to your modem (perhaps  you

have  forgotten  to enable RTS/CTS handshaking or data compression).

If  you  have your modem set up correctly and your performance still

isn't what it ought to be, then a faster  serial  card  may  be  the

solution.  On  a  V32bis/V42bis modem, you should be able to achieve

download speeds of up to 3000 cps on ASCII  text,  or  1500  cps  on

compressed  (e.g.  ZIP)  files.   Remember that poor phone lines can

also affect modem performance.

Even if you are not using Desqview or MS-Windows, you may still  get

the  symptoms details above, especially if you are driving the modem

at 38400 bps or faster, in which case you would be wise to invest in

a faster serial card. Having said that, there are some people  using

high  speed  modems  whose  existing serial port may be adequate, in

which case there is no point investing in a faster one !

5) Okay, I want one, what should I buy ?

-------------------------------------

The  important part about a fast serial card is its UART; this means

that you may be able to simply swap the UART of your existing serial

card, rather than having to fit a whole new card.  You can  only  do

this  if your current serial card uses a 40 pin UART (8250, 16450 or

equivalent). This needs to be  socketed  for  easy  removal,  unless

you're  particularly  experienced  with  a  soldering iron and fancy

desoldering the chip yourself - this is detailed in section (7).

Unfortunately,  most PC's nowadays seem to come with multi i/o cards

with one multi-purpose chip which performs serial, parallel and disk

operations; if you have a multi i/o card, it will normally be a PLCC

package in the centre of the board ( a flat 'surface  mounted'  chip

with pins on all four sides). This obviously cannot be replaced with

a   16550.   Some  PC's  have  the  i/o  circuitry  built  into  the

motherboard; it is unlikely that you can replace the UARTs there.

Thankfully, and more easily, you can simply buy  a  new  card  based

around a 16550 UART. This can either replace or add to your existing

serial  ports and your supplier will advise you on the best type.

6) Is there just one type of 16550 UART ?

--------------------------------------

No! As with all  semiconductors, many manufacturers have  jumped  on

the 16550  bandwaggon. This is good inasmuch that there is a choice,

but bad in that some manufacturers have got it  wrong  -  some  have

FIFO  buffers which don't work and some aren't  recognised correctly

by applications (this can sometimes be worked around but ideally get

one which you know will work).

The 16550 chip 'by which others are  gauged'  is  made  by  National

Semiconductor, and the part no. is NS16550AFN.  There is an older NS

version,  but  make  sure you order the 'AFN' type. If your supplier

stocks a different 16550 version, ensure that it is 100%  compatible

with the NS16550AFN, and ideally get a guarantee that you can return

it if you find that it isn't ! The NS type is the most expensive, so

you will probably save money by buying a clone, but take note of the

above guidelines.

The  FIFO  buffer  on  a  16550 chip can be enabled and disabled. By

default it will be DISABLED. Many comms programs will  automatically

detect  and  enable the FIFO buffer, but if yours doesn't, there are

several programs to enable them,  which  you  can  include  in  your

AUTOEXEC.BAT file or another batch file. Ask your supplier.

There are several programs around to test 16550 chips, once they are

installed  so you'll soon know if your new chip/card is okay. If you

can't get hold of one, ask around on BBS's or ask your supplier.

Both  US  Robotics and Hayes make high speed serial cards  which are

very flexible; ask your supplier for details. If your local supplier

cannot get hold of a fast serial card, see section (8).

7) Replacing a soldered 8250 or 16450 UART

---------------------------------------

If your existing UART is a soldered 16450 or 8250 type  (40 pin DIP)

and you are competant with a soldering iron, then you  may  want  to

remove  the  chip yourself and replace it with a 40 pin socket (into

which you plug your new 16550).  It is not  too  complicated  -  the

most important thing is to work slowly and carefully.  Here are some

useful tips from Bob Kellock:

>>>> ALWAYS OBSERVE STATIC PRECAUTIONS WHEN HANDLING CHIPS <<<<

Get  hold  of a scrap double sided printed circuit board to practice

on; it's no use practicing on a single sided board as they're easy !

>>>>>> DO NOT ATTEMPT TO REMOVE THE CHIP WHOLE ! <<<<<<

Get hold of some fine nosed wire cutters and chop off  each  of  the

legs of the UART as close to the body as possible (therefore leaving

as much of the leg on the board as possible). Discard the main body.

Then,  remove  each  of  the legs in turn using a soldering iron and

tweezers. (Editors note: Bob says "real men use their fingers",  but

I'd  still recommend tweezers). Don't 'wiggle' the iron about on the

surface of the board as you will probably damage the tracks/pads  by

doing so. The time between applying the iron and pulling the leg  is

fairly critical - just keep practicing.

Make  sure  you  pull  the  leg out square from the board or you may

damage the tracks/pads on the component side.

You may find that it helps to tin the iron with new solder  now  and

then,  but don't use too much otherwise you may end up with a pubble

of solder, which will be a read problem.

Get hold of some solder wick or a solder sucker to remove the solder

from the vias (holes in the board where the legs came from).   As  a

last  resort  you  can  use  stranded wire and Fluxite but it's very

tedious. You may find that some holes just don't want to  clean  out

- resolder them with fresh solder and try again.

Check that the holes really are clean - they can be deceptive.  Hold

the board up to the light and check that you  can  see through  each

clearly.  Clean  off  at  least the component side of the board with

PCB cleaner or meths; use it in a well ventilated room using a stiff

brush - an old toothbrush is ideal.

Carefully  inspect the component side of the board - preferably with

a magnifying glass.  You can get minute  traces  of  solder  between

pads  which  completely  messed  it  up.  Remove any odd bits with a

scalpel. Check that there is plating  through  the  holes  -  it  is

possible  to  pull  it  out with the leg but it's not too terrible.

Insert  the  socket  in  the  board  with  the  notch  in  the right

direction. This is when you may find out that one (or more!) of  the

holes isn't cleaned out properly. Do it again.

Solder  socket  in position, clean, inspect and rectify as above. If

you lost any of the through hole platings then solder those pins  on

the top side of the board too.

The board is now ready to take your new 16550 chip. Remember to take

static  precautions  (earth  yourself and don't rub your slippers on

the nylon carpet). Ensure that all pins on the 16550  are  straight;

if  they  are  not,  carefully  press the chip on a table at a right

angle to straighten them. Pins tend of break if they  are  bent  too

much, so be careful.

Place  one  side of the chip just into the socket; do not push it in

yet. Then, place the other side's pins just into the  socket.  Check

that all pins are in the right holes and that they are all straight.

Slowly  press  the  chip into the socket using even pressure, making

sure no pins snag on their way in. Once it's in. Give it a firm push

against the table to make sure everything  is  home  tightly.  Check

that  the  chip  is  inserted properly, the right way round and that

none of the pins have curled up under the body instead of  being  in

its correct hole.

8) MS-WINDOWS and high speed UARTS

-------------------------------

Microsoft  Windows  is  a  prime  cause of lost interrupts.  This is

mostly due to the fact that MS-Windows  is  processor  intensive  so

with  high  speed modems, it is even more likely that data will come

in quicker than the PC can service  the  interrupts.  This  is  made

worse by the fact the Windows Multi-tasks.

A  properly  configured  16550 UART should solve these problems, but

there is a slight complication with Windows 3.1; if you  enable  the

UART's  FIFO  buffers  (using a 16550.EXE or whatever), then Windows

won't recognise the COM port any more; this is strange  because  the

MSD  program supplied with 3.1 will happily report the presence of a

16550 UART. If your Windows software is '16550 aware' then  it  will

happily get around this, but if you Windows software doesn't, or you

are  using  DOS  software  under Windows, then you may have to use a

replacement COM driver, or use different software.

DOS Applications Under Windows

------------------------------

A  company  called  Cherry Hill Software have produced a replacement

comm driver for Windows 3.1 called CHCOMB.386 which will enable  DOS

programs running under windows to recognise 16550's properly.  It is

added  to  your  WIN.INI  file.   It is almost freeware - the author

expects either  a  US$10  donation  or  a  tacky  postcard  for  his

collection.  They can reached by email at 76050.2204@compuserve.com.

The following lines should also be amended in WIN.INI:

device=*combuff      (delete this line - its in your WIN.INI)

device=chcomb.386

[386Enh]

COM3FIFO=true

COM3Buffer=10000

COMBoostTime=4        (normally 2)

COMIrqSharing=true

Experimentation  may be required and doubtless there are people more

experienced than me on Windows and 16550's so if you have  something

to add, please email me and I'll amend this document appropriately.

9) Supplier information

--------------------

S/E/G  Communications,  based  in  London, England, stock a range of

16550 UARTs and 16550 based serial cards at very competitive prices.

For full details and pricing information contact:

����������������������������������������Ŀ

�                                        �

�    S/E/G Communications                �

�    137 Hale Lane                       �

�    Edgware                             �

�    Middlesex HA8 9QP                   �

�    Tel: (081) 959 3377                 �

�    Fax: (081) 959 2137                 �

�    Email: segcom@cix.compulink.co.uk   �

�                                        �

������������������������������������������

Credit Card orders are welcome. Trade enquiries welcome.

------------------ End of Document ----------------

(c) 1993 Michael Spalter, S/E/G Communications

This  document may only be distributed un-modified.  Reproduction as

part  of  any  commercial  transaction  or  distribution   via   any

non-electronic  medium is prohibited in part of full without written

permission from the author.

----- end of file -----