WHATPORT.COM

Version 4.0

October, 1990

For detecting installed serial ports,

type of mouse, IRQs used by ports and mouse,

and potential IRQ conflicts

Copyright 1989-1991 Life Sciences Software

8925 271st St., N.W., Suite 112

Box 1560

Stanwood, Washington 98292

AUTHOR: Pete Petrakis (CIS 76555,1175)

IMPROVEMENTS IN THIS VERSION:

Version 4.0 of WhatPort is a substantial improvement over all previous

versions.  For the first time the program is able to detect the IRQ

line actually being used by the serial ports, instead of assuming that

the standard IRQs are being used.  This greatly increases the program's

usefulness in situations where unconventional IRQs have been assigned

to serial ports.  The new version also detects and reports the base

address of each installed serial port.

DESCRIPTION:

This program detects what serial ports (COM1-COM4) are installed

in a standard (PC, XT, AT) or PS/2 system.  The type of mouse, if

present, is also shown (serial, bus, InPort, PS/2, HP) together with

the interrupt request (IRQ) line it's using (unless it's a PS/2).

The program warns of potential IRQ line conflicts between the

mouse and serial ports.  In the case of PS/2 computers, where

there should be no possibility of conflict between a PS/2 mouse

and a serial port, WHATPORT only reports the presence of the

PS/2 mouse and the characteristics of the serial ports it finds

installed (up to COM4).

-1-

PRINCIPLE:

Because standard PCs have only two IRQ lines for serial ports,

COM3 must share COM1's IRQ line and COM4 must share COM2's

IRQ line.  Unfortunately, two devices using the same IRQ

line can't be active at the same time.  COM3 can't be used for

a printer or modem if a serial mouse is active on COM1, for

example.  Likewise, COM4 can't be used if a serial mouse is

active on COM2.  In other words, if you have four COM

ports and a mouse is attached to one of them, you do not

have three COM ports available for other devices, you only

have two.  The COM port that shares the IRQ line being

used by the mouse is not usable.  WHATPORT detects and

reports such conflicts.

The following table shows what ports are available for other

devices in a conventionally configured PC, XT, or AT when a

serial port mouse is present on a multiple-port system:

Serial port mouse on     Ports available for other devices

_____________________    _________________________________

COM1 or COM3 (IRQ4)          COM2 or COM4 (IRQ3)

COM2 or COM4 (IRQ3)          COM1 or COM3 (IRQ4)

Conflicts with serial ports are also possible in the case of a

bus mouse.  For example, if a bus mouse is configured to use

IRQ3, there will be a conflict with both COM2 and COM4.  If it

is configured to use IRQ4, there will be a conflict with both

COM1 and COM3.  WHATPORT also detects and reports these conditions

so a different IRQ can be selected for the bus mouse.

The PS/2 has a different arrangement.  COM1 in a PS/2 uses IRQ4,

and COM2, COM3, and COM4 share IRQ3.  There should not be any IRQ

conflicts with a PS/2 mouse in a PS/2 system since the PS/2 mouse

has its own port and does not use either IRQ3 or IRQ4.  However, it

seems theoretically possible that someone could attach an ordinary

(non-PS/2) serial or bus mouse to a PS/2 machine, in which case

there could be IRQ conflicts.  With that kind of setup, a bus mouse

using IRQ3 could make three serial ports (COM2, COM3, and COM4)

unusable on a standard PS/2.

As noted above, Version 4.0 does not assume that serial ports 1 to

4 are using the standard IRQ lines for a conventional or PS/2 computer.

Instead, it performs tests to determine which IRQ line each installed

serial port is actually using.  Furthermore, it determines the base

address for each installed serial port.  Thus Version 4.0 should

be useful with virtually any system configuration.

-2-

HOW TO USE THE PROGRAM:

Just run it and read the results.  Adjustment for color or

monochrome display is automatic, but monochrome can be forced

by putting M on the command line.

Some special considerations apply in the case of PS/2 systems,

because the port addresses they use for COM3 and COM4 are different

from those in PCs, XTs, and ATs.  Version 4.0 of WHATPORT will

automatically detect the presence of an IBM PS/2 system and check

the right port addresses.  The option to declare the presence of a

PS/2 system with a command line switch has been eliminated from version

4.00 since experience shows that automatic detection of the system type

is reliable.

IRQ conflicts between the mouse and a port are reported with the message,

"POTENTIAL IRQ CONFLICT -- COMx and the mouse share IRQy"

where x is the number of the serial port and y is the number of the

IRQ line.  The message will be printed for each installed serial port

that is using the same IRQ line as the mouse.  If you see this message,

it means the specified serial port is probably useless for communications

or any other purpose.  With a bus mouse, the conflict can be resolved

by assigning a different IRQ line either to the serial port (in a few

systems) or, most commonly, to the mouse.  Options with a serial port

mouse are more limited.  For example, a serial port mouse on COM1 will

not only tie up that port, it will also tie up COM3 unless there is

some way to give COM3 a different IRQ.  (Note: The user assumes any

risk involved in IRQ assignments.  It is advisable to consult the

mouse or serial port installation manual for guidance in picking the

right IRQs.)

SHAREWARE NOTICE

WhatPort is widely used by computer consultants and others involved in

equipment installation and development, which is most gratifying to any

program author.  Since the effort to continuously improve the program

is not without expense, it has become necessary to make the program

shareware and ask people to register and pay if they use it regularly.

Obviously, many people will use it only once, when they install a new

mouse, for example, and it seems pointless to expect them to register.

However, it does seem appropriate to ask those who use it regularly for

business, professional, or institutional purposes to support its

-3-

continued development.  Beginning with Version 4.00, WhatPort is a

shareware program.  The registration fee is modest, only $15 U.S.,

and your support will be very much appreciated.  Please mail your

registration payment to the above address.

DISTRIBUTION AND DISCLAIMER:

WHATPORT.COM and this document are provided by Life Sciences Software,

Stanwood, Washington.  They may be copied and distributed promiscuously

provided they are not sold, modified, or separated.  Companies involved

in the distribution of diskettes containing shareware or free programs

are permitted to distribute these files and charge their normal fees for

that service.  Although WhatPort has been extensively tested, Life

Science Software and Pete Petrakis make no warranty concerning its

usefulness in all systems and accept no responsibility or liability for

any harm resulting directly or indirectly from its use.

Pete Petrakis

Life Sciences Software

Stanwood, Washington

-4-#