MHSPING: send pings to specified hosts and waits for an echo

ver 0.30 - (c) Grouper Technologies 1994

***FREEWARE*** you may copy and share this program as long as no money is

involved (except for media and shipping expenses)

Object:

- PING is an MHS gateway that sends dummy messages (pings) to

a list of target hosts and waits for an echo.

- PING holds statistical data for each target host: average

delay between sending the ping and receiving the echo, and number

of hops between the pinging host and the target host.

- PING warns the MHS administrator if no echo is received from a

target host after four times the average echo delay for this host.

PING was developped to enhance MHS host communication reliability

by detecting when a distant host is down.

Syntax:

Install the PING gateway with the PGINST batch file, and run the

MHSPING program manually to display statistical information.

To the PING gateway, invoke the PGINST batch with the name(s) of

the MHS host(s) you want to test automatically. For example

"PGINST FARHOST" will install the PING gateway and test host

FARHOST.

Alternatively, PING can accept the name of a file containing a

list of hosts as a parameter. In this case the file should be

located in the gateway's PUBLIC directory, and its name should

be preceeded by a '@' sign in the "Gateway commands" field

(extension should be supplied if necessary).

For example, if you create a host-file "HOSTS.TXT" in the

MHS\MAIL\GATES\PING\PUBLIC directory, invoke the command

"PGINST @HOSTS.TXT" to re-install the PING gateway with the correct

parameter. Hosts names and host-file name(s) can be mixed in the

gateway parameters. The host-file format is pure ASCII, one host

name per line.

To add or delete hosts to be tested by PING after installation,

run the MHS directory manager and add or delete host names in the

"Gateway commands" field for the PING gateway. If you use an

alternate host-file, add or delete host names in this file.

Here is an example of a statistical output displayed when manually

invoking the MHSPING program (after the gateway has been installed

and run at least once):

Host name        : FARHOST

Last ping sent   : Wed Jan 19 16:38:53 1994

Average delay    : 36 minutes

Hops (one way)   : 2

Warning threshold: 3 hours

The PING gateway can be used in conjunction with the MHS Scheduler

from Infinite Technologies or other equivalent utilities, to be

invoked for example once a day.

Files:

MHSPING.EXE copied in MHS/MAIL/GATES/PING/PUBLIC as INPOST.EXE

MHSPING.EXE copied in MHS/EXE

PING.DAT created in MHS/MAIL/PUBLIC by the gateway

Internals:

A dummy user routing is created to route echoes to the gateway.

PING has only been tested with MHS 1.5 hosts.

Host CSERVE does not echo any ping (stealth hub?)...

Changes:

Version 0.30: added support for host files

changed executable name to MHSPING.EXE

Version 0.20: added support for FS/VOL in MV variable

Version 0.10: initial release

Author:

Francois Bergeon

Grouper Technologies

MHS: FBERGEON @ GROUPMNT

Internet: fbergeon@groupmnt.mhs.compuserve.com

CIS: 73377,3170

Thanks to:

Brett Warthen (Infinite Technologies) for the idea...