Packet Send Utility

FTP Emulation Software, and Demo Code.

Author:        Patrick Whittle

Date:          November 4, 1994

Revisions:     February 24, 1995

Purpose:       Provide interface to network packet driver, and MAC level

communication for ethernet networks.  See documentation in

PKTSEND.C for more information.

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

Overview:

This program may be freely distributed as long as the code is not changed

in any way, and the author's name is intact.  The files in this archive

are ready to use; no 'install' needed.  This is freeware!

Use this software for demonstration in a TCP/IP network environment.

Its purpose is to provide an interface to an FTP packet driver and enable

access to ethernet addresses at the physical layer of the ISO/OSI network

model.

Using an FTP packet driver, PKTSEND.COM transmits raw packets

directly to the ethernet via DOS interrupt calls.  The text mode interface

accepts addresses and data from the user, and passes the data to FTP through

interrupt 60h (interrupts 60h through 80h are used by FTP drivers).

Use PKTSEND.COM along with a network utility such as SNOOPER to observe

network activity.  Use this utility in a teaching environment to demonstrate

packet level communication on ethernet networks.

Usage:

Addresses entered in PKTSEND are sent to an FTP packet driver as broadcasts

with specific source and destination addresses (broadcast here does not infer

network broadcast 255.255.255.255, but a lower level direct transmission).

One can easily observe the results of their keystrokes using a second

network terminal.

Install FTP network drivers on two terminals, run PKTSEND.COM on one of

them, and SNOOPER on a second.  Data sent over the ethernet via PKTSEND.COM

will be intercepted by SNOOPER to be recorded.

When no network is installed, load DUMMY.COM into memory first to confirm

that PKTSEND software is working.  This is a TSR program loaded at DOS that

accepts packets via FTP interrupts and discards them (i.e. a null device

driver).  Using PKTSEND.COM and DUMMY.COM together demonstrates the intended

use of this software.

Files in this archive:

PKTSEND.COM        Packet Send Utility, copyright (c) Patrick Whittle, 1995-6

PKTSEND.DOC        This documentation file

PKTSEND.C          Source code and main program

8086.H             Include file used by PKTSEND.C

DUMMY.COM          FTP Packet Driver Emulator v1.01 Copyright (c) Patrick Whittle, 1995

DUMMY.ASM          Source code for dummy packet driver (null device driver)

TRACE.COM          Packet driver tracer version 9.0 copyright 1988-89, Russell Nelson.

TRACE.ASM          Source code for packet driver tracer program

DEFS.ASM           Required by TRACE.ASM

DUMP.EXE           Dump program to view output from TRACE program

DUMP.C             Source code for DUMP program

PKTDRVR.C          Turbo C Driver for FTP Software's packet driver interface.

* Graciously donated to the public domain by Phil Karn.

Using PKTSEND when no network is present:

Load DUMMY.COM

Load TRACE.COM

Run PKTSEND.COM and transmit some data

Exit the Packet Send Utility, and type 'exit' at DOS to remove the TRACE TSR

Run DUMP.EXE to view resulting packet data

Using PKTSEND when a network IS present:

Load an FTP Software FTP packet driver

Load TRACE.COM

Run PKTSEND.COM and transmit some data

Exit the Packet Send Utility, and type 'exit' at DOS to remove the TRACE TSR

Run DUMP.EXE to view resulting packet data

Contact the author via e-mail:

Internet:    patrick@netaccess.on.ca        OR

patrick@xiris.com