ORC2APR   Version 1.4

This converter will convert from a Orcad .TOL aperture file to a GraphiCode

.APR aperture file.  The converter can be used to convert single files or

groups of files.

usage: ORC2APR <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.TOL" for use with GraphiCode

products, type:

ORC2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.TOL" to "NEWALIST.APR" type:

ORC2APR ALIST1 NEWALIST

To convert several aperture files type:

ORC2APR *.TOL

This is the format for the ORCAD aperture file:

({ Time Stamp -  14-OCT-1991    10:49:20 }

Ver    Hor Type Tool

0.008, 0.008,  L,  D10

0.055, 0.055,  R,  D11

0.055, 0.055,  C,  D12

0.375, 0.060,  R,  D13

0.060, 0.060,  C,  D14

0.156, 0.156,  C,  D15

0.060, 0.012,  R,  D16

0.012, 0.060,  R,  D17

Also supported is the format for OrRCAD PCB 386+:

# Format, Version

%IMPERIAL, V3.1

#

# Author:   OrCAD PCB 386+     Version   V1.00   13-Jul-93

#           (C) Copyright 1985-1993  OrCAD, Inc.  ALL RIGHTS RESERVED.

#           OrCAD is a Registered Trademark of OrCAD, Inc.

#

# Written:  Tue Sep 28 21:33:48 1993

#

# D  Shape     Width  Height   Type  Tool   Tool Size   Legend   R90

#

D10  Round     0.0080 0.0080    TH    0     0.0000      D10      0

D11  Round     0.0100 0.0100    TH    0     0.0000      D11      0

D12  Round     0.0500 0.0500    TH    0     0.0000      D12      0

or same as above, but no D preceding the Shape field

#    Shape     Width  Height   Type  Tool   Tool Size   Legend   R90

ORC2APR Version 1.4 also support the %METRIC format.

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.