PAGE 1    WALDEE-WOOD Telescope Parameter Program -- Version 2.1

SHRSCOP.DOC -  Document File for the Shareware Edition

Copyright (c) 1993 - Stephen R. Waldee; All Rights Reserved.

INSTRUCTIONS FOR USE:

(a) Computer/Monitor & Installation

Version 2.1 of this program, optimized for color monitors but also

usable on monochrome monitors, should work on any DOS-compatible personal

computer with at least 384K ram (the program has been tested and found to

run with less than 300 kbytes of available memory) on DOS 2.0 and later.

The user is provided with information for selecting magnification

ranges of an optical telescope for visual astronomical observing.

The program is applicable to any kind of visual telescope, from a

terrestrial "spotter" to an observatory instrument, as long as it employs

interchangeable eyepieces of standard focal lengths and apparent fields,

and has an aperture up to 17 inches (26 inches in the registered version.)

The program may be run directly from the floppy disk by typing

SHRSCOPE at the DOS prompt.  The diskette should contain the files:

1.  SHRSCOP.DOC --  information ASCII text file of the

program manual

2.  DEF.DOC -- information ASCII text file of definitions

3.  REG.DOC -- information ASCII text file about registering

the program

4.  ORDER.DOC -- ASCII text file with registration order form

5.  SHRSCOPE.EXE -- executable program file

6.  README.DOC -- text file describing the program.

This program must not be transmitted as shareware in any form

unless ALL of the above files are included.  You must register this

program if you wish to continue to use it after the 30-day evaluation

period.  You may license either this Shareware edition, or obtain a copy

of the commercial expanded version of "SCOPE - The Waldee-Wood Telescope

Parameter Program" (c) which offers many more features.  Please read the

"Registration Info" file reg.doc, which is accessible from the help menu

by selecting option <r>.

Before using the program disk, make a backup copy.  You will need

the program disk and a fresh blank floppy of the SAME type as the

original program disk: either a 5-1/4" double-sided, double-density

360K floppy, or a 3-1/2" double-sided, double-density 720K floppy

(also called "2S-2D" or "DS-DD".)  Read carefully all the way through

the following paragraphs BEFORE attempting to copy or install the

program!

If you have only one floppy drive, put the original program disk

into the drive and get the A> prompt.  Then type

DISKCOPY A: A:

and press the <ENTER> key.  DOS will prompt you when to take out the

original disk and when to put in the blank floppy.

PAGE 2

If you have two floppy drives, then put the original program disk

into Drive A and the blank floppy into Drive B.  Get A> and type

DISKCOPY A: B:

and press the <ENTER> key.  DOS will not only copy all the files, but

also will copy the formatting of the diskette, so that the blank floppy

need not be formatted in advance.

BE SURE YOU ARE USING THE SAME CAPACITY BLANK DISKETTE!  If you are

not, then the copying process will be halted by DOS.  If you do not

have a double-density blank disk for your copy, then you must copy all

of the files onto a formatted diskette.  Either copy the files one at

a time, or use the *.* command; refer to your DOS instruction book for

all of the details of making copies of files between disks.

To run the program directly from the floppy disk, type SHRSCOPE at

the floppy disk A> DOS prompt.  Once the program and help files are

loaded, SCOPE works just as fast from the floppy as from a hard disk.

All files MUST be present on the disk or the subdirectory for

running the program.  To install SHARESCOPE on a hard disk, copy all

the files into the same hard disk subdirectory; we suggest assigning

it the name SCOPE.

If you are unfamiliar with procedures for making subdirectories,

consult your DOS manual for details; in brief:

(1) Go to the root directory of the desired drive.  If

drive is called C, then at the DOS prompt, type

C:

and press the ENTER key.  At the C> prompt, type

cd\

and press the ENTER key.

(2) Type the following if you wish to name the subdirectory

"SCOPE" -

mkdir SCOPE

and then press the ENTER key.

(3) Enter the directory SCOPE by typing

cd\SCOPE

and pressing the ENTER key.

(4) Place the floppy disk containing the program in the

floppy drive (in this example, called A drive.)

Type the following

copy a:*.*

and press the ENTER key.

(5) After the files have been copied, run the program by

typing

SCOPE

and pressing the ENTER key.

PAGE 3

If difficulties are encountered, refer to your DOS manual or other

appropriate reference book; you may run the program directly from the

floppy if desired.

(b) Entering Data Into the Program

When prompted, you must type in the information requested.  If you

make a typing mistake, the request will be repeated or an error "beep"

will be audible.  Note carefully what you type: the program will lock

out errors and will not proceed until usable information is provided.

Thus, if you are asked for a numerical value, do not type any characters

other than numbers.

Following standard software convention, the program uses brackets

(<...>) to indicate certain numbers or letters that you must type:

do NOT type the brackets themselves, but type only the letters or

numbers indicated inside them in the examples.

Hit the <ENTER> key (sometimes called the <RETURN> key on older

keyboards) only when requested, such as at the end of a numerical value

for your scope aperture.  At other times, if a single letter is required

like "s" for "similar magnitude," the <ENTER> key is not necessary, and

will not be requested.   You may type either upper or lower case letters.

In general, the authors have tried to be understanding and

compassionate to users who are not Grand Wizards of the computer!

We hope that the program's prompts and procedures require little or

no explanations beyond the information presented on the screen.

For users who are not experienced with telescope optics, we

suggest you read the Definition file and look up the entries for

aperture, focal length, focal ratio, eyepieces, and apparent field.

Typical values for each of these subjects will be detailed in the

individual definition.

If you would like to "try out" a sample telescope system, you

might enter the data for the following telescope and eyepieces,

following the on-screen prompts after you start the program:

TELESCOPE APERTURE:                   8 inches (or 203 millimeters)

TELESCOPE FOCAL LENGTH:               2032 mm

or

TELESCOPE FOCAL RATIO:                f/10

1.25" BARREL EYEPIECE FOCAL LENGTH:   26 mm

1.25" BARREL EYEPIECE APPARENT FIELD: 50 Degrees.

This is a very typical amateur telescope combination representing

an 8-inch aperture Schmidt-Cassegrain telescope and an eyepiece that

offers moderate powers for deep-sky viewing.  Or you may request a

power of about 77X, which requires nearly the same eyepiece focal length.

PAGE 4

(c) Features of the Current Version of the Program

The unregistered trial version of the program included on this disk

will calculate amateur visual telescopes up to 17 inches aperture.  The

optical formulas used are in large part the "textbook" values, determined

by the laws of optics and the characteristics of light wave behavior.

In a few cases the formulas are "rule of thumb" subjective values that are

accepted by knowledgeable and experienced visual astronomers.  Licensed

and registered versions of this shareware edition support apertures up to

26 inches, and will print charts of the system screen of optical perfor-

mance; unregistered copies do NOT support any kind of printing.

This current Shareware Version 2.1 corrects some small errors in

the original shareware demo edition, and provides some of the upgraded

menus in the registered and commercial versions.  Two other versions of

this program are available: the full expanded commercial program "SCOPE"

with many more features, and the special upgrade "Observatory Red"

color edition for registered users of the commercial version, offering

red text on a black background for night use at the telescope.  Consult

the "Registration Info" file reg.doc, readable from the Help menu by

selecting option <r>.

Before running the program, the user has the option of reading

the help files: this program manual, the definitions of astronomical

terms relevant to the program and to observing, and the registration

information file.  If the program is used beyond the 30-day evaluation

period, it must be registered to obtain a permanent license for use.

Over one thousand hours of program development have gone into the

editions of this program, which will continue to be upgraded as long

as user support is provided.

The user must supply the following information in order to

run the program:

(1) The telescope light-gathering aperture in inches or millimeters;

(2) The telescope's focal length in either inches or millimeters,

or the focal ratio;

(3) The focal length in millimeters of the eyepiece selected;

(4) And finally, the eyepiece's apparent field of view in degrees,

or the desired eyepiece type.

The apparent field specification is almost always provided

by the eyepiece manufacturer.  If it is not known, a database of

information representative of general characteristics of over 60

standard eyepieces is searched to find a close match to the desires

of the user.  Since different manufacturers use varying methods for

specifying this parameter, the actual visual eyepiece field, sometimes

called "true field" or "real field," may not always be accurately

calculated from the apparent field parameter.  Thus, this program does

NOT intend to calculate the "True Field" as will be described below.

PAGE 5

If a commercial eyepiece of the type specified is available at

the focal length specified, the options list will include that type of

eyepiece for calculations with the telescope.  A complete menu of

configuration options for selecting the size of eyepiece barrel/scope

focuser is presented at the beginning of the program.

After initial data input, the program will calculate the following

system parameters:

-LIGHT GRASP VERSUS THE UNAIDED EYE: the amount of light gathered

across the area of the telescope objective versus the light gathered

across the area of the dark-adapted eye's entrance pupil of 7 mm.

-FOCAL RATIO: the ratio of the aperture to the focal length, which

may be effectively increased with the introduction of a Barlow lens.

-TELESCOPE POWER OF MAGNIFICATION: shown with warnings if the

magnification exceeds typical observing conditions.

-POWER PER INCH OF APERTURE: the magnification per inch of the

diameter of the objective, a good indication of appropriate

magnification range for a given aperture.

-EXIT PUPIL: the measure of the diameter of the image of the

objective presented at the output of the eyepiece, intended to enter

the "entrance pupil" of the observer's eye.  The maximum entrance

pupil of the dark-adapted eye is assumed to be about 7 mm, so a

telescopic exit pupil much in excess of this will cause a loss of

light, in effect reducing the effective aperture of the telescope.

-ESTIMATED WIDEST FIELD OF VIEW: the program checks the eyepiece

barrel size and then scans the ocular database for the lowest

practical magnification available that will provide an exit pupil

as close to 7 mm as possible.  When the eyepiece is selected, the

visual field of view is estimated for that eyepiece; thus the value

changes if the focuser barrel size is switched from 1.25" to 2.00"

eyepiece barrel size, permitting a wider potential field of view.

For example, the popular 8" Schmidt-Cassegrain will achieve its low-

est potential power, widest field, and largest exit pupil with 2.00"

eyepieces, not the smaller 1.25" barrel sizes.  The estimated calcula-

tion of wide field must be verified by actual scope star-field tests!

-APPROXIMATE VISUAL FIELD: in arcminutes, of the telescope/eyepiece

combination at the power specified; due to differences in the eyepiece

makers' methodology in measurements, this specification may not always

agree with the precise, flat "true field" of the eyepiece.  The true

field is always best determined by actual measurements of a calibrated

chart or by measuring the drift of a known star field through the

eyepiece field.  However, the actual differences (if any) between

the approximate visual field and the true field will probably be

minimal if the eyepiece specifications are accurate.

PAGE 6

-VISUAL STELLAR MAGNITUDE LIMIT: of a point-source star image,

with compensations for skyglow, assuming that the observer's eye

is dark-adapted.  The default value is 6.5 magnitude naked-eye limit

for a dark sky offering good opportunities for perceiving faint

stars and nebulae.

The visual limiting magnitude will be displayed during any

power chosen for the optical system, but it may only be achieved

visually by employing enough magnification to be equal to the

diameter of the telescope aperture in millimeters.  Then the star's

"Airy disk" is sufficiently large to register on the retina, and the

magnification provides contrast by dimming the background light.

This specification does not directly indicate the visibility of

non-stellar "extended" objects like nebulae or galaxies, but is useful

in comparing scope/sky efficiencies.

Your scope may perform worse than the prediction if the optics are

dirty or poorly coated, or if the actual sky darkness differs from your

chosen naked-eye limiting magnitude.  Conversely, your scope may actually

outperform the prediction if you use a high-efficiency small refractor

without a diagonal, if your reflector's mirrors have "enhanced" coatings,

and if you use the best quality low-loss coated eyepieces, or if your

sky is darker than 6.5 naked-eye magnitude.

The commercial edition of "SCOPE" provides options for customizing

the user's actual sky conditions to the program's magnitude calculation.

-RESOLUTION [Dawes' Limit]: taken from the famous 19th century British

amateur astronomer Reverend William Rutter Dawes' measurement of percep-

tions of the minimum separations in arcseconds of close similarly-bright

double stars in the 5-7 magnitude range, determined with small refractor

telescopes.

This criterion may not represent the potential performance of short-

focus reflectors or telescopes with misaligned or poor optics.  However,

in some cases the Dawes limit may be exceeded with excellent optics on

very good observing nights.

-CLOSEST SIMILAR DOUBLE STAR OBSERVABLE AT A GIVEN POWER WITHOUT

ATMOSPHERIC LIMITS: a calculation of the closest detectable separation

at the system's operating magnification of a typical Dawes double, based

on the assumption that the naked eye is just able to detect a separation

of 4 arcminutes (or 240 arcseconds.)  This is a theoretical calculation,

assuming excellent optics and no atmospheric distortion.  The formula

employed was confirmed by the experience of observers with excellent

telescopes, viewing under steady skies which occasionally permitted

splitting double stars close to 0.5 arcsecond separation.

-LIGHT POLLUTION REDUCTION and NEBULAR LINE BANDPASS FILTER

SPECIFICATIONS: if chosen during the program start-up, the System Screen

report will list the options of employing light-pollution-reduction (LPR)

PAGE 7

or various kinds of nebula bandpass filters if the exit pupil is within

range, helpful for learning the conditions for using each filter.

(d) Helpful Operating Guides and Error Reports

(1) Magnification Limits

Some operating conditions will flag a warning that certain limits

have been exceeded.  For example, it is assumed for a small scope of,

say, 3" aperture, that 50 power per inch of aperture is the practical

limit for a resolved image of an extended object such as a planetary

or lunar surface detail, and that 60 power per inch of aperture is the

maximum magnification needed to see the closest double stars.

Beyond these values of magnification, the wave nature of light

limits the resolution of the image, which will become larger and larger,

but fuzzier and fuzzier as the power is increased.  However, very large

aperture telescopes will almost never be used at or above 50 power per

inch of aperture, due to atmospheric limitations.  A sliding scale is

provided for judging the appropriate power per inch of aperture as the

aperture increases, utilizing the maximum power that the best atmos-

pheric conditions will permit.

The observer may be enticed to use high powers in excess of 1,000

that William Herschel or S. W. Burnham occasionally utilized for special

observations, but most amateur astronomers with telescopes of, say, 8"

of aperture can never hope to employ more than 300 to 400 power; on some

nights of unsteady air, only the lowest powers provide clean images.

(2) Resolution

In larger scopes it is generally asserted that the best nights of

viewing will permit a resolution of no more than about 0.5 arcseconds,

although there may indeed be a few nights per year when the "seeing"

exceeds this value at high viewing altitudes, if the aperture of the

instrument is within range.  Users of very large observatory scopes have

sometimes been able to split doubles of as little as 1/4 arcsecond

separation, but this is not a common experience with amateur scopes!

(3) Exit Pupil

When the telescope magnification is too low, generally below 3 to 4

power per inch of aperture, the "exit pupil" of the optics will be too

large for the dark-adapted eye, causing some of the image of the full

telescopic aperture to spread out beyond the area of the eye's "entrance

pupil," with a loss of effective aperture.  In a scope with obstructed

aperture (a Newtonian reflector or Schmidt-Cassegrain, for example) an

overly-large exit pupil will often force the user to dodge the dark blob

in the center of the field caused by the secondary mirror.  However,

refractors or other non-obstructed telescopes may be used with over-

large exit pupils in order to gain very wide fields at the expense

of image brightness and contrast.  The program will display a warning if

the exit pupil exceeds the typical maximum of about 7.5 mm.

PAGE 8

(4) Focal Ratios

Typical real-world visual telescopes are usually constructed to be

within a focal ratio range from as low as f/3 (rich-field types) to as

high as f/30 (solar telescopes) although these values are exceeded in

eyepiece projection photography and in the construction of certain types

of telescopes such as schiefspieglers.  This program assumes during scope

scope data input that anything below f/1 or over f/30 represents mistaken

parameters, and will suggest checking for errors.

(5) Viewing Suitability

Based on the telescope/eyepiece combination chosen by the user,

the program makes some brief suggestions about appropriate astronomical

subjects in general detail, as a guide to beginners, in order to help

dispel incorrect expectations about telescope performance.  Of course,

good dark skies are necessary for "richfield" starcloud sweeping or

for discerning galaxies and nebulae.  If the magnification and exit

pupil are within range, the program alerts the user when light-

pollution and nebular filters most effectively may be employed.

(e) Menu Options

At the end of the performance data screen, the option is available

to enter new eyepiece parameters, to change the telescope, to add a

Barlow lens to the system or to change or remove it, to view double

stars, to turn on or off the filter information, to get a specific power

of magnification, or to exit to DOS.  The commercial version "SCOPE"

offers many other options, detailed in the "Registration Info" file

which may be accessed from the help menu by choosing option <r>.

(1) View Double Stars

A handy option is the calculation of power needed for the

splitting of a defined double star separation.  If the double is

closer than 0.5 arcseconds an alert is reported, as most users

may never encounter sky conditions and optical perfection that

will permit splitting this separation.  The function defaults to

the "no Barlow" mode at startup, and the nebular filter data are

turned off during the double star mode.

When the double star separation is entered, the program will

round off values under 1/10th arcsecond, since visual resolution of

such fine detail is virtually impossible.  Next, options are given

for choosing magnifications for detecting different kinds of doubles.

The classic Dawes double -- a similarly colored pair of medium

magnitude stars -- may be split by most observers when the magnifi-

cation employed boosts the double's separation to about 4 arcminutes

(or 240 arcseconds) in the eyepiece field.  For doubles of widely

differing magnitudes, the program offers higher magnifications to

further open up the doubles' separation.

PAGE 9

If the double is below the Dawes limit of the scope aperture

another error is noted, allowing the user to define a larger double.

If the double can be split with the lowest power eyepiece, the program

will install an eyepiece in the barrel size defined that will give

the largest usable dark-adapted exit pupil size equal to or less

than 7.5 mm.  If the double is very wide but within 2/3rds of the

widest possible visual field, the program will use the lowest power

range of the scope consistent with an exit pupil less than 7.5 mm,

and choose the widest-field eyepiece.

If the magnification needed requires the use of Barlow lenses,

the program will automatically install and test a 2X or even a 3X

Barlow to see if the power will be high enough to split the double.

Thus, the novice double star observer will be able to predict

fairly accurately what magnification will be needed if the double

is in range of the telescope defined.

For the expert user with acute vision and a superb instrument,

the program offers the option of calculating the magnification

required to boost the double to just 2.5 arcminutes apparent

separation at the eyepiece; the authors and their associates have

consistently split the famous "Double Double" in Lyra with only about

60 power, though most books recommend using at least 100 power or

more.

After choosing the double star and range needed for its separa-

tion, the program checks the barrel size and the ocular database,

employing a standard commercial eyepiece as close to the required

focal length as possible.  During the search, the program generally

defers to the widest possible eyepiece apparent fields, although

these are not necessarily essential for any but the most generously-

separated "binocular doubles" or naked-eye doubles like Mizar and

Alcor.

(2) Get Power

If the user specifies the power of magnification desired,

the program will calculate what eyepiece is needed and will select

one from the ocular database as close as possible to the power

specified.  The full system performance will then be displayed,

including any warnings if magnification limits have been exceeded.

If the eyepiece required is not available (in case the power is

very low or otherwise out of range) the user may choose other

options: changing the telescope, barrel size, or adding a Barlow

to increase the focal ratio.

(3) Add Barlow

A Barlow lens may be introduced into the system, and Barlow

magnification up to nine times is permitted in the case of users

who wish to stack Barlows for extremely high magnification from

PAGE 10

short-focal-length guidescopes.  The "GET POWER" function will track

the inclusion of the Barlow for obtaining magnifications beyond the

range of standard eyepiece focal lengths.

If the user wishes to calculate the results of the change in

effective focal ratio by the inclusion of a magnifying coma corrector

lens, the magnification factor may be entered by using the Barlow

function.  Thus, a coma corrector that increases the focal ratio

by 15% would be entered as a "1.15X Barlow."  Indeed, a Barlow and a coma

corrector may thus be stacked by specifying their total magnification

factor: a 2X Barlow and A 1.15 coma corrector would then result in an

entry of (2) X (1.15) = 2.3 power.

(4) Other Options Not In Demo Version

The registered Shareware V/2.5 permits searching the actual database

values, which are extended even further in Commercial V/3.7x/3.8x to

over 120 values.  The Commercial editions also provide extremely handy

functions to specify directly the field of view and exit pupil desired.

Registered Shareware and Commercial versions permit the printing of

useful telescope/eyepiece charts.  The Commercial editions have extensive

instructions on how to best observe solar system/deep sky objects, with

extra in-depth definition files and lists of resources for developing

skills in amateur astronomy.  Further details are available in this

Help menu by reading the file "Upgrade."

(f) Formulas and Algorithms Employed for Calculations

This program is intended to be a SEEKER of truth, not an absolute

arbiter of unalterable facts!  An astronomer should soon learn that

visual observation is not an absolutely exact science.

Many formulas for optics and astronomical viewing are, at best,

only approximations, or are based on subjective experiences.  An

experienced astronomer with nearly perfect optics may sometimes exceed

the program's calculations.  If you can do better than this program

predicts, you may compliment yourself and your equipment!

(g) A Few Words About the Eyepiece Database and Accuracy of Specs

In order to aid the user who has only a general idea of the

different types of eyepiece designs and how their apparent fields

will affect the ultimate eyepiece view, the authors provide a

database of eyepieces, representing the GENERAL characteristics of

oculars that may have been manufactured in the past by several

different companies.  This database is not intended as a tool to

recommend the purchase of any type or brand, and cannot possibly

encompass every eyepiece ever made or sold.  There are over 60

oculars in the shareware database; the commercial program "SCOPE"

contains a database with twice as many eyepieces, including the very

latest long eye-relief types.  If your eyepieces are not in the

database, the program will permit you to define their focal lengths

and apparent fields and calculate the system performance.

PAGE 11

It is difficult to cross-reference and compare with absolute

precision the types of eyepiece designs of competing manufacturers,

whose trade-names for specific designs may differ from the classic

astronomers' definitions of Kellners, Ploessls, Erfles, and so forth.

Our solution was to set up general guidelines for eyepiece designs

within certain ranges of apparent fields.  These generic descriptions

may differ from specific manufacturers' trade names; and different

manufacturers use diverse methods for measuring the apparent fields,

so even those data are not precisely comparable.

Thus, we make no claims that, for example, an "Erfle" of a given

focal length and apparent field -- though it may be listed in the

database -- is actually available under that precise description from

a specific company.  Their version of that "Erfle" may differ in

number of elements and performance, and may be an enhanced design.

Use our generic eyepiece-type descriptions, plus the specs for focal

length and apparent field, to help find a match to the commercial

eyepiece you wish to purchase.

The calculations performed by this program are dependent for

accuracy on the quality of the data input by the user, and the

correctness of the published specs for telescopes and eyepieces.

Formulas used for magnification and fields follow the standard methods

published in any good book on telescope design and performance.  We

cannot make claims for the accuracy of manufacturers' data, nor do we

accept responsibility for them.  Our database merely "crunches the

numbers" that are close to values that may represent real eyepieces.

Some published optical specifications can be general indicators of

typical performance in a given model of telescope, rather than precise

and unvarying values.  In hand-figuring the optics for some telescopes,

the exact focal length of the system may differ by as much as 15% from

the nominal value, since surface accuracy, not exact focal length, may be

the criterion selected in optimizing each unique set of lenses or mirrors.

Our database has been written with some proprietary techniques that

take much less disk space and work faster than conventional database

managers, optimizing the performance of the program on laptops and

machines with slow CPU clock speed.  Therefore the database structure is

not editable by the user, but many representative values of focal length

and field are included.  The commercial program "SCOPE" contains a much

more complete database, which may be upgraded as new versions of the

program are prepared.  The commercial version also permits examination

of the contents of the entire database in order to find desired oculars.

Most practiced astronomers will agree: the numbers are NOT the most

important factor in good visual astronomy.  Image sharpness, the flatness

of the field, efficiency of light transmission, and reliability of the

performance of an optical product cannot be represented by mere numbers

describing focal length, apparent field, and so forth.

PAGE 12

(h) Acknowledgments

The primary author, Stephen R. Waldee, wishes to thank the highly-

experienced amateur astronomer Mr. Richard C. Page for his research in

establishing viewing parameters and testing the criteria used for the

formulas employed in this program.  Valuable suggestions were made by

Dr. Douglas Hudgins, Research Scientist, Astrophysics Division, NASA-

Ames Research Center, Moffet Field, CA., and members of the Tri-Valley

Star-Stargazers Astronomy Club, Livermore, CA and its president, Alan

Gorski.

Invaluable assistance in perfecting details of program construction

and syntax, and the creation of the eyepiece database algorithms, have

been provided by co-author Mr. Ron Wood, a gifted astrophotographer and

computer engineer.  We also appreciate the information about filters

provided by Dr. Jack Marling of Lumicon, Livermore, California.

(i) LICENSING INFORMATION AND LIABILITY DISCLAIMER FOR

WALDEE/WOOD PROGRAM "SHARESCOPE"

The program SHARESCOPE is offered with no warranty for absolute

accuracy.  The authors make and you receive no warranties or conditions.

We do not warrant that this program will operate perfectly on all

computers, or that it will be free of all error.  We will not be held

liable for any loss of data, profits, or other direct or indirect damages

arising from the use of this computer program on your equipment.

You do not own this copyrighted program, but are permitted to use

this program for personal and private entertainment and information for

an evaluation period of 30 days.  If you wish to continue to use

SHARESCOPE, you must register your copy and receive a license for its

continued use.  Full information is given in the file reg.doc, which

may be read from the Help menu by choosing option <r>, "Registration

Info."

Copyright (c) February 1993  Stephen R. Waldee  - All Rights Reserved