PAGE 1    BEGINNER'S DEFINITION FILE            (c) Copyright Oct. 1992

"SHARESCOPE" Version 2.1                    Stephen R. Waldee

All Rights Reserved

(1) Using This Help File.

The beginning amateur astronomer will find brief

definitions of topics and terms you need to know to

fully understand how to use the program SHARESCOPE.

If you would like advanced topics and definitions,

consult the commercial edition of the program entitled,

"SCOPE - The Waldee-Wood Telescope Parameter Program" (c).

(2) BEGINNERS' DEFINITIONS

*  *  *  *  *  *

NOTE: If you see a word typed in all capital

letters in the descriptions of a topic, you will

find that word defined in this file.

*  *  *  *  *  *

APERTURE

The diameter of the main lens system (or MIRROR) of a telescope

or binocular, where the light enters from the scene you wish

to observe.  The lens or mirror is also called the "objective."

If you have a beginner's REFRACTOR telescope, your main

lens aperture may have a diameter of, for example, 2.4 inches,

which is 60 millimeters in the metric system.

The larger the aperture, the more light your telescope gathers

compared to the light gathered by your eye.

"Sharescope" will accept apertures from 1 to 17 inches in

diameter, measured either in inches or millimeters.  The commercial

version of the program will accept apertures up to 36 inches.

APPARENT FIELD

The actual FIELD OF VIEW of a telescope EYEPIECE, measured in

degrees of the angle from the eyepiece's focal point out through

the eyepiece BARREL.

The issues may seem confusing, but don't get the eyepiece's

apparent field of view mixed up with the telescope's visual

FIELD OF VIEW.  The telescope's visual field of view will be

dependent upon the apparent field of the eyepiece, and the

telescope's POWER of magnification.

PAGE 2

EYEPIECE apparent fields are determined by their optical

design, and are limited by the barrel size.  Huygenian types have

apparent fields of around 30 degrees; the expensive computer-

designed 1.25" and 2.0" barrel eyepieces may have apparent fields

as wide as nearly 85 degrees!

In order to clarify this issue, remember that a narrow apparent

field and a wide apparent field eyepiece of the same FOCAL LENGTH

will provide the same POWER in a given telescope, but the eyepiece

with the wider apparent field will also provide the wider visual

FIELD OF VIEW.

Eyepiece manufacturers do not always use exactly the same

criteria for specifying the apparent field measurements of their

eyepieces.  Therefore, the best way to judge the apparent field

is by comparing eyepieces of the same focal length while looking

through the telescope at a known star field or test pattern.

ARC (degrees, minutes, and seconds)

The measure of an angle, used in determining the apparent

diameter of celestial objects.  There are 60 arcseconds in an

arcminute, and 60 arcminutes in a degree, which is 1/360th of the

circumference of a sphere.

ARCMINUTE

The average observer can see the narrow angle of just 4 minutes

of arc, or to put it another way, 4 arcminutes.

The Double Star calculations in the program SCOPE are based on

the ability of most people to be able to distinguish this narrow

angle in the eyepiece image.

One of the most useful aspects of a telescope is to be able to

magnify the narrow celestial angles which may be measured in tiny

arcseconds, until they are visible angles measured in arcminutes.

This is done not by changing the actual angles, but instead by

narrowing the FIELD OF VIEW so that the angles seem to be greater,

since they are measured on the narrower field of view seen through

the telescope, and not across the 180-degree field of view of the

entire sky.

BARLOW LENS

A special magnifying lens system which is placed between the

EYEPIECE and the telescope, to increase the magnification.

Barlow lenses are usually rated in 2 to 3 times power increase.

A good Barlow lens makes high-POWER observing through a telescope

easier for a user who wears eyeglasses.

PAGE 3

BARREL (of eyepiece):

The metal barrel of your telescope's eyepiece that fits into

the telescope focuser.

Small imported REFRACTOR telescopes generally use eyepieces of

0.965 inch barrel diameter; larger telescopes can use eyepieces

with barrels of 1.25 inches or even 2.0 inches.

The larger barrel eyepieces are designed to have a wider

APPARENT FIELD so that you can see a wider FIELD OF VIEW through

your telescope.

BINARY STARS - See DOUBLE OR MULTIPLE STARS

BINOCULARS

Binoculars are two matched telescopes attached to each other,

at just the right position to fit over each of the user's eyes.

Binoculars show an image which is upright and corrected, so that

your view of a distant scene is oriented in a normal manner,

but magnified to a higher POWER.

Binoculars are usually used by most people for TERRESTRIAL

viewing at typical powers of 5 to 10 times magnification.

However, certain types are very well suited to studying

celestial objects.

If the binocular has an EXIT PUPIL of at least 5 millimeters,

it can be used to look at the night sky.  To calculate the

exit pupil, divide the APERTURE by the POWER.

EXAMPLE:

A " 7 by 50" (sometimes called " 7 X 50")

binocular has an aperture of 50 millimeters and a

power of 7 times magnification.

50 divided by 7 = 7.1 millimeters EXIT PUPIL,

which is excellent for seeing faint objects in the

night sky.

To use binoculars for viewing celestial objects, you should

try to mount them on a camera tripod for the steadiest images.

PAGE 4

CLUSTERS OF STARS

Groupings of stars that were first chosen for special study by

astronomers because they looked like they must be related or were

relatively close together.  By the 20th century, scientists

discovered that many of these clusters were indeed moving together

in space in groups that were bound together by gravity.

Some clusters are very sparse and consist of only a few

stars which seem to be loosely arranged: these are generally

called "open clusters."  A good example is the beautiful

cluster called "The Pleiades" in the CONSTELLATION of Taurus.

Other clusters look like tiny puff-balls in a small telescope,

and in a larger telescope appear to be roundish, tight groupings

of countless stars: these may in some instances be "globular

clusters," and are found in our own GALAXY and in other distant

galaxies like "Andromeda," or MESSIER 31.

Many star clusters are featured in the MESSIER CATALOG of

DEEP-SKY objects, and are often studied by astronomers, including

amateurs with small telescopes or binoculars.

CONSTELLATIONS

Apparent groupings of bright stars visible to the naked eye.

Ancient astronomers looked at certain chance arrangements of

stars and interpreted them as representing objects and animals

they were familiar with, such as the bull of "Taurus," the

hunter of "Orion," or the swan of "Cygnus."

In recent centuries, other constellations were added to the

ancient group, including a number of southern hemisphere

constellations, such as one that resembled a house fly ("Musca")

and even a telescope ("Telescopium")!  Now there are 88 "official"

constellations.

Most of the constellations are spread out over wide areas of

the sky and occupy many degrees of ARC; thus, they are best

observed by the naked eye, as a telescope -- or even binoculars --

will have a narrow FIELD OF VIEW that will be too small.

DARK ADAPTATION

The process of adapting the observer's eyes to a sensitivity

to the faintest levels of light, critical to successful observing

of the DEEP-SKY objects.  It takes over 30 minutes time to allow the

pupils of the eyes to relax in a dark environment to attain dark

adaptation, permitting the eye's retina to regenerate its "visual

purple" that helps detect the faintest of light sources.

PAGE 5

DEEP-SKY OBJECTS

Celestial objects beyond our own solar system, including

CLUSTERS OF STARS, GALAXIES, and NEBULAE.  Many interesting

and relatively easy to find deep-sky objects are contained

in the MESSIER CATALOG.

However, few of these objects can be seen in the night

sky in the midst of a city.  The observer should travel to

a location where there is less LIGHT POLLUTION, so that

the sky is sprinkled with thousands of visible stars.  Then

the best deep-sky viewing is possible, even with a small

telescope.

DIAGONAL

A telescope attachment which can be placed between the

optical tube assembly and the eyepiece.  An astronomical

REFRACTOR telescope will have an upside down, reversed

image unless a DIAGONAL is attached.

A "Star Diagonal" will provide an upright but reversed

image.  A "Porro Prism" or "Correcting Diagonal" will provide

an upright image, corrected for normal left-right orientation.

DOUBLE OR MULTIPLE STARS

Some isolated star images that are very close together in

a telescope or binocular field of view may be termed double

stars, binary stars, or multiple stars.

A double star is called an "optical double" if the two

stars merely appear close together because they are positioned

that way along our line of sight out into space; in reality,

one star may be billions of miles further away than the other.

A true binary or multiple star system consists of two or

more stars that are relatively close together, in an orbit

around a mutual center of gravity.  The distance that separates

these stars may change as the stars revolve around each other.

Observing double stars and multiple stars can be a very

rewarding activity of amateur astronomy, testing the skills of

the viewer and the quality of the optics.

Be sure, however, that you have a reliable source of

information about the stars you are observing, for binary and

multiple star system separations may vary greatly over time.

PAGE 6

EXIT PUPIL (of Telescope or Binoculars)

The output of the EYEPIECE of the telescope is an image of

the objective lens or mirror of the telescope, and it is called

the telescope's "exit pupil."  The program Sharescope calculates

the exit pupil for each eyepiece you plug into your telescope.

The exit pupil of the telescope must fit into the pupil of

your eye, which opens up to a maximum diameter of around 7

millimeters.  In general, an exit pupil for viewing NEBULAE

should be as large as possibe, while an exit pupil for viewing

stars or planets may be as small as one-half millimeter or

even less.

The larger the exit pupil, the brighter will be the images

of non-stellar (non-star) objects.  As the magnification POWER

of the telescope goes up, the exit pupil goes down and becomes

smaller.

EYEPIECE

The eyepiece is a lens system that is plugged into the telescope

to magnify the telescope's own focused image, since most telescopes

for amateur astronomy are made to use interchangeable external

eyepieces, and contain no fixed eyepieces of their own like

binoculars.

The eyepiece's own internal focal length, measured in

millimeters (abbreviated "mm"), is used to calculate the telescope's

POWER of magnification.  The FOCAL LENGTH of the telescope is divided

by the focal length of the eyepiece, using the same units, to derive

the POWER of magnification.

Thus, a telescope with a 700-millimeter focal length and a

7-millimeter eyepiece has a magnifying POWER of:

700 / 7 = 100 times magnification ( or "100 X". )

The eyepiece is designed with its own intrinsic APPARENT FIELD,

which is based on the optical system of its lenses and the size

of its BARREL.

Over the past several centuries a number of useful eyepiece

types have been developed: Huygenian, Kellner, Orthoscopic,

Ploessl, Erfle, and modern extremely wide-field designs.

FIELD OF VIEW (Telescopes)

The eyepiece/telescope combination has a distinct field of

view, as distinguished from the EYEPIECE's own APPARENT FIELD.

PAGE 7

The telescope's "true" visual field is measured by looking

through the eyepiece at a known field angle.  The approximate

visual field may be calculated by dividing the eyepiece's

APPARENT FIELD by the telescope POWER of magnification.

Thus, if the eyepiece had a 60-degree apparent field, and

the telescope had a power of 60X, the calculated visual field

of view would be 60 / 60 = 1 degree, or 60 arcminutes.

This information is useful in knowing what eyepiece to use

in order to view an object of a known dimension, such as a

GALAXY with an angular dimension of 10 arcminutes, or the

full Moon's sphere, which has a 30-arcminute diameter.

FILTERS - Light Pollution, Nebular, or Colored Types

Special glass filters are available for astronomers to

enhance the images of DEEP-SKY objects or PLANETS.  Colored

filters are useful for viewing planets even with the smallest

of telescopes.  Deep-sky, light pollution reduction, or nebular

filters are helpful in telescopes of about 80 millimeters or

larger aperture.

LPR and nebular filters work properly when the telescope EXIT

PUPIL is in the correct range of size, as shown by the program

Sharescope for each eyepiece that is plugged into a telescope

system.

FOCAL LENGTH (of Telescope)

The distance from the light-gathering input of the

telescope objective's APERTURE to the place where the light

is focused and presented to the EYEPIECE of the telescope.

This dimension is necessary for calculating the POWER of

magnification of the telescope, and is used by the program SCOPE.

FOCAL LENGTH (of Eyepiece) - See EYEPIECE

FOCAL RATIO

The focal ratio of the telescope may be found by dividing the

diameter of the APERTURE by the FOCAL LENGTH, using the same

value of units.

The focal ratio is designed into the telescope, and certain

values are more suited to some applications than others.  However,

by changing EYEPIECES or BARLOWS, most amateur telescopes can

provide a satisfactory range of viewing options for high POWERS

(PLANETS, STARS) and wide fields (for CLUSTERS and NEBULAE.)

PAGE 8

A telescope optimized for planets may have a focal ratio of,

for example, 10: it is generally written, as in this example,

as  "f/10."

A "richest field" telescope for the widest possible fields of

view, usually beyond 2 or 3 degrees, would be designed in a

focal ratio of around f/4 to 4/5.

Some observers want to own telescopes of different focal

ratios to get the best optical performance on the full range

of POWERS of magnification necessary for all celestial objects.

GALAXY

A grouping of many more stars than a cluster, in fact

millions or billions of stars.  Gravity holds these stars

together, and some galaxies may possibly contain dense

"black holes" at their centers, though this has not been

conclusively proved.

The MILKY WAY was the ancient name for our own galaxy,

which was not then known by the ancient astronomers to be

the view of our galaxy from inside!  The MILKY WAY is

a "spiral galaxy," and from a distant universe the arms of

our galaxy would appear to be wound in a loose spiral coil.

The bright spiral galaxy "Andromeda" (number 31 in the MESSIER

CATALOG) may be seen in binoculars or even by naked eye in

a dark night sky, and since it is seen on edge, it has a

bulging shape but no distinct spiral form.

Other prominent kinds of galaxies are called "elliptical

galaxies" because they appear to have an oval or elliptical

shape.

In an amateur telescope, the images of most visible

galaxies are faint fuzzy smudges of light.  It helps to try to

observe galaxies in the very darkest of night skies.

LIGHT-GRASP

Light-grasp is one of a number of ways to judge the

efficiency of a telescope.

The "System Screen" of our program Sharescope will show

your telescope's light grasp as compared to the naked eye.

The calculation measures how much light is gathered across

the diameter of the aperture of your telescope, compared to

the diameter of the pupil of your own eyeball.

PAGE 9

LIGHT POLLUTION

Random scattered man-made light from streetlights, signs,

car headlights, and so forth.  Any haze, smoke, or smog in

the air will help reflect this light until the entire

sky is glowing with a yellowish cast.  DEEP-SKY objects will

be lost in this fog of light, but the brightest stars and

planets will still be visible to the naked eye or telescope.

LIMITING MAGNITUDE - see MAGNITUDE OF STARS

MAGNITUDE (of stars)

The brightness rating for stars, measured in both

positive and negative numbers.  For some beginners this

is confusing, because astronomers changed the rating of

stars some centuries ago to include numbers that are

negative, or less than zero.

Most of the stars in the sky are rated in positive numbers

greater than zero: however, the larger the number, the dimmer the

star.  The brightest star in the northern hemisphere is Sirius

in the CONSTELLATION of Canis Major, which is rated at a magnitude

of -1.6.  The Sun, which is much brighter, is rated at -27!

The very faintest star that can be seen by the naked eye

or through the telescope is said to represent the "stellar

(star-like) limiting magnitude" of the eye or telescope.  Our

program SCOPE will indicate the limiting magnitude of your

scope, depending upon the naked-eye limiting magnitude of your

sky.

A difference of one star magnitude equals a difference of 2.5

times the light intensity.

MESSIER CATALOG

A collection of over 100 famous DEEP-SKY objects, gathered by

an 18th-century French comet-hunter named Charles Messier.  In his

rather poor small REFRACTOR telescope, most of these objects

appeared to look like comets, though we know them now as GALAXIES,

CLUSTERS, AND NEBULAE.

Using today's better optics in a dark sky, we can see actual

detail in many of these Messier objects.  You can even spot many

of them in binoculars!  One of the benefits of Sharescope is to

know in advance what the eyepiece field will be, so that the

observer can judge the expected size of an unknown deep-sky

object.  By judiciously applying the appropriate filter (if

needed), almost all of the Messier objects may be seen in

PAGE 10

moderately-dark skies in a telescope of no more than 3-4 inches

of aperture.

MILKY WAY

Ancient name for our own GALAXY.  If you can see the Milky

Way with your naked eyes, the night sky is dark enough for very

good DEEP-SKY observing.

MIRROR (Telescopes)

Telescopes which gather light by means of a mirror, not a

REFRACTOR-type lens system.  Mirrors are cheaper and easier to

make than high-quality telescope objective lenses, and thus

a mirror telescope can offer more light-gathering power than

a lens-type or REFRACTOR telescope of the same price.

Most mirror-type telescopes are of the "Newtonian" or

"Cassegrain" design, and can offer more light-grasp per dollar

cost than refractor telescopes.

MULTIPLE STARS - See DOUBLE OR MULTIPLE STARS

NEBULAE

Clouds of gases in deep interstellar ("between the stars")

space.  These faint clouds were first called "nebulae" because

they appeared indistinct or nebulous in early telescopes.

By our own century, astronomers discovered that many of

these nebulae were not just clouds of gas but were instead

mighty GALAXIES like our own MILKY WAY.

Other nebulae are indeed gas clouds, like the spectacular

"Great Nebula of Orion," MESSIER No. 42, which some astronomers

consider the finest DEEP-SKY object.  This nebula is even visible

to the naked eye in some city skies!

"Planetary Nebulae" are gas clouds surrounding stars

that may have exploded.  The best-known example is the famous

"Ring Nebula,"  Messier 57, in the CONSTELLATION of Lyra.

Nebulae will appear faint and hard to see in the smallest

telescopes; refer to the "Observer" Help file for instructions

to learn the best viewing techniques.  A dark sky site is

necessary for seeing most DEEP-SKY nebulae, though special

FILTERS may help the view in telescopes larger than about

70 - 80 millimeters aperture (with the correct EXIT PUPIL.)

PAGE 11

PLANETS

At least 6 of the 9 major Solar System planets may be

visible in a city sky with a small telescope of 60-millimeter

aperture.  These include Mercury, Venus, Mars, Saturn, Jupiter,

and sometimes Uranus, not to mention our own Earth!

The fainter Neptune may require a darker sky or larger scope

aperture.  Distant Pluto may be spotted with an 8-inch aperture

telescope, but will look just like a very faint star, and must

be confirmed by plotting its slow motion over several days against

the "fixed" stars in its region of the sky.

Satisfactory planetary astronomy requires high POWERS of

magnification and a good steady telescope mount, preferably of

the equatorial design.

POWER (of Magnification)

One of the most useful measurements of a telescope or binocular

optical performance is the power of magnification: how many times

larger than a naked-eye image is the image through the optics.

Telescope power is not the only useful aspect of its performance.

In some cases, the LIGHT-GRASP of the telescope is more important,

especially if one wishes to view faint NEBULAE.

The power of magnification must be suited to the angular

dimension and brightness of the object you wish to view.  Stars

occupy an EXTREMELY small angular dimension: to view a single star

or a double star, use high powers, up to about 60 power per inch of

aperture of a small amateur telescope.

Planets may benefit from as much as 50 power per inch of aperture

in a small telescope, as long as the sky is very calm and steady.

However, to view a relatively large GALAXY or NEBULA, you must

use a lower power and larger EXIT PUPIL to be able to see faint

contrasts in the image.

REFLECTOR (Telescopes) - See MIRROR

REFRACTOR (Telescopes)

Telescopes that use a "refracting" lens system to gather light,

which is brought to a sharp focus by means of the bending, or

refracting, properties of lenses.

PAGE 12

An astronomical refractor telescope uses a double or multiple

"objective lens system," such as an "achromatic" objective, to

sharply focus the light of different colors.  An ordinary single

lens will not focus all of the light wave colors at the same

precise point.

Achromatic objectives are more difficult and more expensive to

make than telescope MIRRORS, so refractor telescopes generally

cost more in dollars per inch of aperture than reflector or

MIRROR telescopes.

STAR

One of the basic components of the entire universe, stars are

glowing balls of ultra-hot gases, powered in most cases by internal

nuclear explosions.  Our own Sun is the closest star, and is of

course the brightest star in our own sky.

The distant stars are so far off that their apparent angular

dimensions are too small to be resolved by the naked eye, or even

by amateur telescopes.  Stars can be magnified with more telescope

POWER than planets or other objects; that is why the program

Sharescope give a maximum rating of 60 power per inch of aperture

for viewing stars, but only 50 for viewing planets (in scopes of

apertures up to about 10 inches.)

STAR DIAGONAL - See Diagonal

TERRESTRIAL OBSERVING

Looking at objects here on earth ("terra") with your telescope,

as opposed to celestial observing.  You should use a telescope with

an upright image, such as a refractor or a corrected reflector.

Since most Newtonians have upside-down images, they are better

suited for sky-watching.

The unsteadiness of the air near the surface of the earth

limits the POWER of magnification that may be employed for

terrestrial observing.  You may seldom be able to use more than

40 or 50 X magnification for clear images along the horizon.

-- end --