#### This article refers to the 3 switch version without the AB-BC jumper ####

MODIFYING THE HAYES 1200B MODEM TO IGNORE

THE STATUS OF THE DATA TERMINAL READY (DTR) LINE

by Kim Redlack, Capital PC User Group

3/15/84

INTRODUCTION

In the IBM Personal Computer whenever the ASCII communications adapter

is opened the PC provides a signal to the modem indicating that it is

ready to receive data.  This signal is present on pin 20 of the

connecting cable and is referred to as the DTR line.  The 300 and 1200

Hayes freestanding Smartmodems have a switch which permits the user to

set the modem to monitor or ignore the DTR line.  If the DTR line is

monitored and the communications adapter is closed the DTR signal

disappears and the modem then hangs up.  Presence of the DTR signal is

indicated by the TR indicator light being lit.

Because some communications programs open and close the communications

adapter during execution, setting this switch to ignore the DTR line

maintains the connection to the telephone line and the remote computer

throughout program execution.

The Hayes 1200B board modem which mounts in an expansion slot inside

the IBM PC does not contain such a switch.  As a result, some programs

will not work with this modem.  The modification described below may be

used to cause the 1200B to ignore the DTR line thus permitting these

programs to function properly.

MODIFICATION

Modification of the 1200B modem involves the very simple procedure of

soldering a small wire jumper between two pins of one of the integrated

circuits on the modem board.  Modification should not, however, be

attempted unless you are familiar with printed circuit board soldering

techniques because damage to the modem could result.  Be sure and use a

low heat pencil tipped soldering iron.

First place the 1200B circuit board component side down on your work

table so that the edge connectors are located at the top right. In this

position the 25 pin DB-25 connector should be on your right.  Next

locate integrated circuit #5.  This is a 16 pin IC on the bottom of the

board about one-third of the way in from the right edge of the board.

It can be identified by a small upside down "5" on the circuit board.

Although the markings may vary with the manufacturer, the integrated

circuit on our board was a Motorola IC marked "MC14051B" on top and "CP

FF8320" below.

When you have located this IC you should see a heavy foil connecting

pins 2, 4 and 15. This foil can be seen between the two rows of

connecting pins.  The pins of the IC when viewed from this side are

numbered clockwise from the lower left corner of the IC.  Pins 1-8 are

from bottom to top on the left side and pins 9-16 are from top to

bottom on the right side.

--------------

| 8o     o9  |   IC #5 (bottom view)

| 7o     o10 |

| 6o     o11 |

| 5o     o12 |

| 4o--|  o13 |   <-----|  Jumper here to

| 3o  |  o14 |         |  connect these

| 2o--|--o15 |   <-----|  two pins.

| 1o     o16 |

|_____�______|

Having identified the proper IC you should now be able to locate pins

13 and 15.  You will find them on the right side where the heavy foil

should be seen connected to pin 15. Prepare a small jumper wire about

one inch long.  A small solid conductor wire is recommended.  A small

amount of insulation should be stripped from each end and the wire

should then be bent into a "U" shape so that its ends can easily be

soldered to pins 13 and 15.  Once you are satisfied that the wire is

properly shaped you are ready to solder it in place.  A very small

amount of solder should be used to prevent a solder bridge from forming

an unwanted connection between other adjacent pins.  Once your

soldering is complete, inspect the results carefully to make sure that

no unwanted connections have been made.  If all appears well then

carefully bend the wire flush with the board and replace it into your

PC.  If all is well you will find that the modem now ignores the status

of the DTR line and your programs will operate properly.  Good luck.