WhatLine v 2.80              Introduction

WhatLine is a utility designed to squeeze the most out of a

single telephone line.   Equipment that required dedicated

lines can now share a single line and save money.  Use up to

three phone numbers on one telephone line.

WhatLine with One phone number:

Your answering machine or FAX machine can share the line with

a modem.  The answering machine will pick up messages in the

normal manner.   When you call in with your remote PC, WhatLine

will have your Modem answer the line or execute any program you

prefer.

WhatLine with Multiple phone numbers:

People, Fax, Modem, answering machine, voice mail can share a

single phone line.  Dedicate  phone numbers not phone lines.

WhatLine identifies the three different ring patterns used by

the Identa*Ring service offered by the Telephone Company.

WhatLine will execute a program based on which phone number

was dialed.  Identa*Ring is a service mark of Bell Atlantic

and may have a different name in your area.

WhatLine's Time Event feature:

The Time Event feature will execute any program at the time

specified with the  /T=hh:mm  start-up  option.   WhatLine

monitors calls as described above until a Time Event occurs,

then your computer will execute a program(s) of your choice.

Call remote computer systems and pick up or send Email when

telephone cost are lowest,  do backups or run your aquarium

program that puts the little fish on the screen.  When the

Time Event is complete, WhatLine will wait for more incoming

calls.

Miscellaneous:

Delay WhatLine from passing control to a device / program so

you can answer the phone yourself, then use Release when you

want WhatLine to handle the calls.   Delay only the phone

line(s) you chose, other dialed phone numbers will be handled

by WhatLine.

I use WhatLine to manage one phone line with three telephone

numbers.  I run two different configurations of Watson Voice

Information Systems by Natural Micro Systems,  one system for

my use and one for a client.   Each has voice mail, file

transfers  and  interactive  Voice Information Systems.  Daily

reports and disk `housekeeping' are run as a Time Event.  The

third number handles a FAX Card.

WhatLine v 2.80               Start up Options

Syntax    C>WHATLINE /p /RS=x /RL=x /Dn=r /A /B /T=hh:mm

Replace lower case letters with a decimal number.

/p        The number of your Com Port  [1 to 4].  WhatLine

defaults to Com1: if omitted.

/RS=x     Used to fine tune the Ring Short period response.  When

the ring is longer than this value and shorter than  /RL

below, it identifies Line 2.  If the ring is shorter than

this value it identifies Line3. The default is 0.686

seconds.  This option is not normally needed and is

provided for timing variations that may occur with some

telephone systems.

/RL=x     Used  to fine tune the Ring Long period response.  When

the ring is longer than this value it identifies Line1.

The default is 1.8 seconds.  This option is not normally

needed and is provided for timing variations that may occur

with some telephone systems.

/B        Bypass the opening screen.  Use this option if you

start WhatLine from a batch  file.

/A        Use to share phone Line1 with a Answering or

Fax machine and Modem or other program.

/T=hh:mm  Time Event executes WLT.BAT at a set time.  Time

hh:mm must be in 24 hr. format.  A Time Event will

repeat daily until reset.

/Dn=r     Delays passing control for r number of rings when

phone number n is called.  May be used more than

once on the command line to delay more than one

phone number.  Has no effect if used with /A.

/U        User test mode.  Is WhatLine compatible with your modem.

/DTR      Do not set DTR at start up.

WhatLine v 2.80      Files used by WhatLine

WHATLINE.EXE    The executable program file

WLT.Bat         Holds commands to run with Time Event

LINE1.BAT       Holds commands to run for phone number 1

LINE2.BAT       Holds commands to run for phone number 2

LINE3.BAT       Holds commands to run for phone number 3

WHATLINE.DOC    This file

Using WhatLine

Commands to be executed when each phone number is dialed are

placed in corresponding batch files.  When phone number 1 is

dialed, LINE1.BAT executes.   When all commands and or

programs in LINE1.Bat are completed,  control returns to WhatLine.

LINE2.BAT  and  LINE3.BAT hold the commands and or programs to

be executed for phone numbers 2 or 3, respectively.   WLT.BAT

contains the commands and or programs to execute as a Time Event

when set with the /T=hh:mm start-up option.   Programs must

start-up and terminate unattended if you want control to return

to WhatLine.  I use BitCom from Bit Software for modem

communications, the script language is easy to use and allows

for unattended operation.

When WhatLine is loaded the display window shows the current

time and the com port being monitored.  As calls are

processed the display window is updated with the time and

date of the last call, and the total calls for each of three

possible phone  numbers.  The display window relocates every

minute to prevent screen burn-in.

F1:Help      Show a list of options available to you, press

F1 again to clear the help line.

F2:Hold      Use this function to hold WhatLine from passing

control to the LINEn.BAT file when one or more

phone number is called.  This is good for those

times  you may want to answer the phone manually.

The display window will indicate which line(s)

is on-hold.

F3:Release   Use this function to Release a line placed on

hold with F2.  WhatLine will then pass control

according to the value set with the /Dn=r start-

up option.

F4:DOS       Use this to execute DOS commands or programs.

Type Exit to return to WhatLine.

ESC:Quit     Terminates WhatLine and returns to DOS.

WhatLine v 2.80

To share one phone number with an answering or Fax machine

and a modem, you must use the /A start-up option.  Place the

commands and/or programs to be executed in LINE1.BAT.  Set

your answering machine to answer on the fourth ring,  when

the phone rings the answering machine will pick up messages

in the normal manner.  When you want to connect to your

modem, dial your telephone number, let the phone ring twice,

then hang up.  Wait 8 seconds and call back, WhatLine will

load  your modem software.    When you finish The

telecommunication session, or if you do not call back within

60 seconds, WhatLine will have the answering machine answer

the phone.

This program works with Hayes compatible modems. WhatLine

does not control the modem or change any settings.  WhatLine

monitors the UART chip for telephone line status.

If you are using a PC with a very slow access disk drive,

using a RAM drive like VDISK.SYS will speed up overall

response time.  If you use a RAM drive, be sure to backup any

files that are changed.  If power is lost or your system

crashes, all data on the RAM drive will be lost.  Use the

Time Event feature to automatically backup these files to a

physical disk drive.

The examples in this document illustrate some of the ways

that WhatLine can be used.  WhatLine is not limited to these

suggested applications.  Please let me know how you are using

WhatLine, or what other features would be useful in future

releases.

This  program is distributed as Shareware.  You may copy and

distribute this program freely.  Since I have been using

WhatLine, my monthly telephone savings has been more than

the $25.00 Registration fee.

Users of WhatLine are required to register their copy.  Upon

registration you will receive a disk with the latest version

of WhatLine and some other Utilities you may find useful.

If you have trouble

If the address of your Modem Status Register is 3FE hex for COM1

or 2FE hex for COM2 and Bit 6 is the Ring Indicator Bit Whatline

2.80 should work on your system.

Try WhatLine /U and ring the phone line WhatLine is monitoring.

If the words RING and QUIET appear at the proper times WhatLine

2.80 will work on your system.

WhatLine 2.80

With the /U start up switch active and the Last Call display on

screen, ring the phone.  A RingTime value will be displayed.

This is the duration of the ring period in seconds.  This time

period may not be a constant value.  Repeat this a few times for

Line1 and Line2 to come up with a average value.  Use these average

values to help determine the RingLong and RingShort settings below.

A LINEn.BAT file with a pause command in it will make this process

very easy.

The value of x for /RL=x must be less than Long-Ring time for Line1

and greater than the Short-Ring time for Line2.

The value of x for /RS=x must be greater than the Ring-Short time for

Line3 and less than the Ring-Short time for Line2.

Try changing these values in small steps until you get some sort of

reaction.

Send Your $25.00 Registration fee to:

WhatLine 2.80

Advance Systems

215 W Amosland Rd.

Norwood, PA  19074-1502

WhatView is now included with the registered WhatLine Package!

WhatView is the same as WhatLine execpt it includes a built in

interface for LapView.

LapView is a File Transfer and Directory Management Utility for two

IBM compatible computers connected by the Serial Port using a

standard null modem cable.  LapView displays two directories side by

side, one from each computer.

Scroll through the Directory listings and select files to transfer

between computers.  Copy, Move, Delete, View and Print files.  Sort,

Switch, Create and Remove Directories or Run an .EXE, .COM, or .BAT

file on either machine, all with a few key strokes.

AUTOMATIC LINE SWITCHES

Advance Systems is an authorized vendor for the LYNX Line Switches.

RingDirector:  Uses the `Distinctive Ring Pattern' to automatically

route incoming calls to attached telephone devices.

One Line IN with either TWO or FOUR Ports OUT.  Plug

RingDector into the telephone wall jack then plug in

the FAX, Phone, Modem or other Telephone device.

Just Plug it in and it works!

___             /FAX - Phone Number 1

|   | __________/

1 Phone Line    | R | ___________/ANSWERING MACHINE - Phone Number 2

------------->>| D | ___________

4 Phone Numbers |___| _________  \TELEPHONE - Phone Number 3

\

\MODEM - Phone Number 4

4 PORT $149.00        2 PORT  $89.00          PA orders add 6% Sales Tax.

Prices on 2/11/92, subject to availability and change.         215 532-0950

TECHNICAL SUPPORT

Telephone

Help and Tech Support are available to Registered Users Mon-Fri 9am

to 5pm EST.

Voice Mail is accepted from All Users to report any problems or comments

and suggestions.  Call Anytime 24 hrs. 215 532-0950

FAX      215 532-5649

EMail

Compuserve    74425,257

Delphi        AWARE

Genie         J.SAVOIA

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.